MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

MT6261D GSM GPRS SOC Processor Data Sheet

Version:	0.5

Release date:	2014-03-13

© 2014 MediaTek Inc.

This document contains information that is proprietary to MediaTek Inc.

Unauthorized reproduction or disclosure of this information in whole or in part is strictly prohibited. Specifications are subject to change without notice.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

# Document Revision History

| Revision | Date | Author | Description |
| --- | --- | --- | --- |
| 0.1 | 2014-02-18 | CP Dai | 1st release |
| 0.2 | 2014-02-25 | CP Dai | Reviewed by software |
| 0.3 | 2014-03-04 | CP Dai | Detailed PMU part |
| 0.4 | 2014-03-06 | CP Dai | Delete Redundancy and correct contant |
| 0.5 | 2014-03-13 | CP Dai | Reviewed by software |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

# Table of Contents

Document Revision History	2

Table of Contents	3

Preface	7

- System Overview	8

  - Platform Features	12

  - MODEM Features	13

  - GSM/GPRS RF Features	14

  - Multimedia Features	15

  - Bluetooth Features	17

  - FM Features	18

  - General Descriptions	19

- Product Descriptions	21

  - Pin Description	21

  - Electrical Characteristics	39

  - System Configuration	47

  - Power-on Sequence and Protection Logic	48

  - Analog Baseband	52

  - Power Management Unit Blocks	56

  - GSM/GPRS RF	101

  - Bluetooth	107

  - FM RF	111

  - Package Information	113

  - Ordering Information	116

- Micro-Controller Unit Peripherals	117

  - Pulse-Width Modulation Outputs (2 Channel)	117

  - SIM Interface	122

  - Keypad Scanner	142

  - General Purpose Inputs/Outputs	152

  - General-purpose Timer	289

  - MCU OSTIMER	293

  - UART1	305

  - UART2	320

  - UART3	337

  - I2C/SCCB Controller	351

  - I2C/SCCB Controller (I2C_SCCB_Controller_V18)	363

  - Real Time Clock	376

  - Auxiliary ADC Unit	391

  - USB Device Controller	404

  - Accessory Detector	422

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

SD Memory Card Controller (MSDC0)	433

  - SD Memory Card Controller (MSDC1)	460

  - BTIF	486

# Lists of Tables and Figures

Table 1. Pin coordinates	21

Table 2. Acronym for pin types	23

Table 3. PIN function description and power domain	23

Table 4. Acronym for state of pins	27

Table 5. State of pins	27

Table 6. Acronym for pull-up and pull-down types	30

Table 7. Capability of PU/PD, driving and Schmitt trigger	31

Table 8. Absolute maximum ratings for power supply	39

Table 9. Absolute maximum ratings for voltage input	39

Table 10. Absolute maximum ratings for storage temperature	39

Table 11. Recommended operating conditions for power supply	40

Table 12. Recommended operating conditions for voltage input	40

Table 13. Recommended operating conditions for operating temperature	40

Table 14. Electrical characteristics	40

Table 15. Strapping table	47

Table 16. Mode selection of chip	47

Table 17. APC-DAC specifications	52

Table 18. Functional specifications of auxiliary ADC	53

Table 19. Functional specifications of analog voice blocks	55

Table 20. Functional specifications of analog audio blocks	56

Table 21. LDO types and brief specifications	59

Table 22. Analog LDO specifications	59

Table 23. Digital LDO specifications	60

Table 24. RTC LDO specification	61

Table 25. RTC LDO specification	61

Table 26. ISINKs and KPLED Switches Specification.	62

Table 27. Charger detection specifications	67

Table 28. Pre-charge specifications	67

Table 29. Constant current specifications	67

Table 30. Constant voltage and over-voltage protection specifications	68

Table 31. BC1.1 specifications	68

Table 32. DC characteristics (TA = 25oC, VDD = 2.8V unless otherwise stated)	103

Table 33. Rx AC characteristics (TA = 25oC, VDD = 2.8V unless otherwise stated)	103

Table 34. Tx GMSK AC characteristics (TA = 25oC, VDD = 2.8V unless otherwise stated)	105

Table 35. SX AC characteristics (TA = 25oC, VDD = 2.8V unless otherwise stated)	106

Table 36. DCXO AC characteristics (TA = 25oC, VDD = 2.8V unless otherwise stated)	106

Table 37. Basic data rate – receiver specifications	108

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Table 38. Basic data rate – transmitter specification	109

Table 39. Enhanced data rate – receiver specifications	109

Table 40. Enhanced data rate – transmitter specifications	110

Table 41. FM receiver DC characteristics (TA=25°C, VDD=2.8V unless otherwise stated)	111

Table 42. FM receiver AC characteristics	112

Table 43. Time-out condition for answering to reset sequence	139

Table 44. 5*5 double KEY’s order number in COL/ROW matrix	151

Table 45. 5*5 triple KEY’s order number in COL/ROW matrix	151

Table 46. Abbreviations	293

Table 47. Wakeup sources	294

Table 48. Characteristics of wakeup sources	295

Table 49. Relationship between commands and touch panel control signals	394

Table 50. Sharing of pins for SD memory card controller	434

Table 51. Sharing of pins for SD memory card controller	460

Table 52. IIR[5:0] codes associated with the possible interrupts	488

Figure 1. Typical application of MT6261D	11

Figure 2. MT6261D block diagram	20

Figure 3. Ball diagram and top view	21

Figure 4. IO types in state of pins	30

Figure 5. Power-on/off control sequence by pressing PWRKEY and XOSC32_ENB = 0	49

Figure 6. Power-on/off control sequence by pressing PWRKEY and XOSC32_ENB = 1	50

Figure 7. Block diagram of audio mixed-signal blocks	55

Figure 8. PMU system block diagram	57

Figure 9. Power domain	58

Figure 10. LDO block diagram	59

Figure 11. ISINKs and KPLED switches bock diagram	62

Figure 12. PCHR block diagram	64

Figure 13. Charging states diagram	65

Figure 14. Diagram of MT6261D 2G RFSYS	103

Figure 15. System diagram of Bluetooth RF transceiver	108

Figure 16. Block diagram of hardware top-level architecture	111

Figure 17. Outlines and dimension of TFBGA 8.1mm*7.6mm, 145-ball, 0.5 mm pitch package	115

Figure 18. Mass production top marking of MT6261D	116

Figure 19. PWM waveform	117

Figure 20. PWM waveform with register values	118

Figure 21. Block diagram of SIM interface	122

Figure 22. Timing diagram of SIM interface	124

Figure 23. Answering to reset sequence	139

Figure 24. 5x5 double keypad matrix (50 keys)	143

Figure 25. 5x5 triple keypad matrix (75 keys)	144

Figure 26. 5*5 double keypad scan waveform	144

Figure 27. One key pressed with de-bounce mechanism denoted	145

Figure 28. (a) Two keys pressed, case 1; (b) Two keys pressed, case 2	145

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Figure 29. kp timing register	150

Figure 30. GPIO block diagram	152

Figure 31. OS timer system view	293

Figure 32. Wakeup event and Irq_b integration diagram	295

Figure 33. Pause command complete and pause request state	299

Figure 34. Debug wakeup events	305

Figure 35. Block Diagram of UART1	306

Figure 36. Block Diagram of UART2	321

Figure 37. Block Diagram of UART3	337

Figure 38. AUXADC architecture	392

Figure 39. Touch panel circuit structure	393

Figure 40. Touch panel sampling waveform	394

Figure 41. Suggested Accessory Detection Circuit.	423

Figure 42. The Sate machine between Microphone and Hook-Switch plug-in/out change.	424

Figure 43. PWM waveform	424

Figure 44. PWM waveform with register value present	427

Figure 45. Card detection for SD memory card	435

Figure 46. Card detection for SD memory card	461

Figure 47. Card detection for SD memory card (Scheme 2)	462

Figure 48. Interface connection between BT and baseband system	487

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

# Preface

Acronyms for register types

| R/W | For both read and write access |
| --- | --- |
| RO | Read only |
| RC | Read only. After the register bank is read, every bit that is HIGH(1) will be cleared to LOW(0) automatically. |
| WO | Write only |
| W1S | Write only. When data bits are written to the register bank, every bit that is HIGH(1) will cause the corresponding bit to be set to 1. Data bits that are LOW(0) have no effects on the corresponding bit. |
| W1C | Write only. When data bits are written to the register bank, every bit that is HIGH(1) will cause the corresponding bit to be cleared to 0. Data bits that are LOW(0) have no effects on the corresponding bit. |

# System Overview

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

MT6261D is a monolithic chip integrating leading edge power management unit, analog baseband and radio circuitry based on the low-power CMOS process.

MT6261D is a feature-rich and extremely powerful single-chip solution for high-end GSM/GPRS capability. Based on the 32-bit ARM7EJ-STM RISC processor, MT6261D’s superb processing power, along with high bandwidth architecture and dedicated hardware support, provides a platform for

high-performance GPRS Class 12 MODEM application and leading-edge multimedia applications.

MT6261D also features:

- A highly integrated Bluetooth transceiver which is fully compliant with Bluetooth specification v3.0.

- A FM receiver supporting both audio broadcast de-modulation and RDS/RBDS data decoding.

Typical application diagram is shown in Figure 1.

Platform

MT6261D is capable of running the

ARM7EJ-STM RISC processor, which provides the best trade-off between system performance and power consumption.

For large amounts of data transfer,

high-performance DMA (Direct Memory Access) with hardware flow control is implemented, which greatly enhances the data movement speed while reducing the MCU processing load.

Targeted as a media-rich platform for mobile applications, MT6261D also provides hardware

security digital rights management for copyright protection. For further safeguard and to protect the manufacturer’s development investment, hardware flash content protection is provided to prevent unauthorized porting of the software load.

Memory

MT6261D supports serial flash interface with various operating frequencies.

Multimedia

The MT6261D multimedia subsystem provides serial interface for CMOS sensors. The camera resolution is up to VGA size. The software-based codec can be used to process various video types. To take advantage of the high MCU performance, GIF and PNG decoders are implemented by the software.

In addition, MT6261D is implemented with a high-performance audio synthesis technology, as well as a high-quality audio amplifier to provide superior audio experiences.

Connectivity and storage

MT6261D supports UART, USB 1.1 FS/LS ,

SDIO and SD storage systems. These interfaces provide MT6261D users with the highest level of flexibility in implementing high-end solutions.

To achieve a complete user interface, MT6261D also brings together all the necessary peripheral blocks for a multimedia GSM/GPRS phone. The peripheral blocks include the keypad scanner with the capability to detect multiple key presses, SIM controller, real-time clock, PWM, serial LCD controller and general-purpose programmable I/Os.

Audio

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Using a highly integrated mixed-signal audio front-end, the MT6261D architecture provides easy audio interfacing with direct connection to the audio transducers. The audio interface integrates A/D converters for voice band, as well as high-resolution stereo D/A converters for both audio and voice band.

MT6261D supports AMR codec to adaptively optimize the quality of speech and audio.

Moreover, HE-AAC codec is implemented to deliver CD-quality audio at low bit rates.

In addition, an 1.2W audio amplifier is also embedded to save the BOM cost of adopting external amplifiers.

GSM/GPRS radio

MT6261D integrates a mixed-signal baseband front-end in order to provide a well-organized radio interface with flexibility for efficient customization. The front-end contains gain and offset calibration mechanisms and filters with programmable coefficients for comprehensive compatibility control on RF modules. MT6261D achieves outstanding MODEM performance by utilizing a highly dynamic range ADC in the RF downlink path.

MT6261D embeds a high-performance and completely integrated single-ended SAW-less RF transceiver for multi-band GSM cellular system. In this RF transceiver, a quad-band receiving feature with high sensitivity is supported utilizing one RF receiver and a fully integrated channel filter. With ultra-high dynamic range, the off-chip balun and SAW filters on the receiving path can be removed for BOM cost reduction. In addition, the minimum component count is guaranteed by realizing a highly integrated transmitter, low-spur frequency synthesizer and a Digitally-Controlled Crystal Oscillator (DCXO).

Bluetooth radio

MT6261D offers a highly integrated Bluetooth radio and baseband processor. Only a minimum of external components are required. MT6261D provides superior sensitivity and class 1 output power and thus ensures the quality of the connection with a wide range of Bluetooth devices.

MT6261D is fully compliant with Bluetooth v3.0 and offers enhanced data rates of up to 3Mbps. It also provides the coexistence protocol with

802.11 system.

MT6261D supports rich Bluetooth profiles, enabling diversified applications that are widely used on the handset with excellent interoperability.

FM radio

The FM radio subsystem provides a completely integrated FM Rx receiver supporting 65 ~ 108MHz FM bands with 50kHz tuning step. In addition to receiving FM audio broadcasting, the digital RDS/RBDS data system is supported as well. The integrated FM transceiver utilizes state-of-the-art digital demodulation/modulation techniques to achieve excellent performance.

In order to achieve high SINAD, good sensitivity and excellent noise suppression, the FM receiver adopts adaptive demodulation scheme to optimize Rx system performance in all ranges of signal quality by reference of a very sophisticated channel quality index (CQI). When the received signal quality is poor, the design not only enhances the ACI rejection capability but also uses a very ingenious skill to soft mute annoying noise so as to provide good perception quality.

The FM radio subsystem supports both long antenna, which is usually an earphone, and

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

auto-calibrated short antenna, which is usually a FPC short antenna or shared antenna with GSM for different application scenarios.

Debugging function

The JTAG interface enables in-circuit debugging of the software program with the ARM7EJ-STM core. With this standardized debugging interface, MT6261D provides developers with a wide set of options in choosing ARM development kits from different third party vendors.

Power management

A power management is embedded in MT6261D to provide rich features a high-end feature phone supports, including Li-ion battery charger, high performance and low quiescent current LDOs, and drivers for LED and backlight.

MT6261D offers various low-power features to help reduce the system power consumption. MT6261D is also fabricated in an advanced low-power CMOS process, hence providing an overall ultra-low leakage solution.

Package

The MT6261D device is offered in a 8.1mm×7.6mm, 145-ball, 0.5mm pitch, TFBGA package.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

UART

SD SDIO

SIM2  SIM1 USB 1.1 F/S

USIM  USIM

DEVICE

1

4

7

*

2

5

8

0

3

6

9 #

KEYPAD

RF

LCD INTERFACE

APC

PA	SW

SPEECH/AUDIO OUTPUT (PCM INTERFACE)

MT6261D

BPI

SUPPLY VOLTAGES

POWER MANAGEMENT CIRCUITRY

CHARGER

CHIP RID

LCD

PWM

AUXADC

SPEECH/AUDIO INPUT

SDIO INTERFACE	FM STEREO

RADIO INPUT

I2S

I2C

CAM

INTERFACE  INTERFACE

UART	JTAG

HIFI STEREO OUTPUT

4-SIM	CMOS

6306	SENSOR

ATV

WI FI 5931

LCD

DEBUGGER

GPS 3336

AUDIO DAC

Figure 1. Typical application of MT6261D

### Platform Features

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

General

- Integrated	voice-band,	audio-band	and base-band analog front-end

- Integrated full-featured power management unit

MCU subsystem

- ARM7EJ-STM 32-bit RISC processor

- Java	hardware	acceleration	for	fast Java-based games and applets

- High-performance multi-layer AHB bus

- Dedicated DMA bus with 16 DMA channels

- On-chip	boot	ROM	for	factory	flash programming

- Watchdog timer for system crash recovery

- 3 sets of general-purpose timers

- Circuit switch data coprocessor

- Division coprocessor

User interfaces

- 5-row x 5-column keypad controller with hardware scanner

- Supports multiple key presses for gaming

- Dual SIM/USIM controller with hardware T = 0/T = 1 protocol control

- Real-time clock (RTC) operating with a low-quiescent-current power supply

- General-purpose I/Os (GPIOs) available for auxiliary applications

- 2 sets of Pulse Width Modulation (PWM) output

- 24 external interrupt lines

- 1 external channel auxiliary 10-bit A/D converter

Security

- Supports security key and chip random ID

Connectivity

- 3 UARTs with hardware flow control and supports baud rate up to 921,600 bps

- FS/LS USB 1.1 device controller

- Multimedia card, secure digital Memory Card, host controller with flexible I/O voltage power

- Supports 4-bit SDIO interface for SDIO peripherals as well as WIFI connectivity

- DAI/PCM and I2S interface for	audio applications

- I2C	master	interface	for	peripheral management including image sensors

- SPI master/slave interface for peripheral management.

Power management

- Li-ion battery charger

- 13 LDOs for the power supply of memory card, camera, Bluetooth, RF, SIM card and other diversified usage

- 1 open-drain output switches to supply/control the LED

- LDO type vibrator

- One NMOS switch to control keypad LED

- Thermal overload protection

- Under-voltage lock-out protection

- Over-voltage protection

- Different levels of power-down modes with sophisticated software control enables excellent power saving performance.

Test and debugging

- Built-in digital and analog loop back modes for both audio and baseband front-end

- DAI port complies with GSM Rec.11.10.

- JTAG port for debugging embedded MCU

### MODEM Features

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Radio interface and baseband front-end

- Digital PM data path with baseband front-end

- High dynamic range delta-sigma ADC converts the downlink analog I and Q signals to digital baseband.

- 10-bit D/A converter for Automatic Power Control (APC)

- Programmable radio Rx filter with adaptive gain control

- Dedicated Rx filter for FB acquisition

- 6-pin Baseband Parallel Interface (BPI) with programmable driving strength

- Supports multi-band

Voice and modem CODEC

- Dial tone generation

- Voice memo

- Noise reduction

- Echo suppression

- Advanced sidetone oscillation reduction

- Digital	sidetone	generator	with programmable gain

- Two programmable acoustic compensation filters

- Supports GSM/GPRS modem

- GSM quad vocoders for adaptive multirate (AMR), enhanced full rate (EFR), full rate (FR) and half rate (HR)

- GSM channel coding, equalization and A5/1, A5/2 and A5/3 ciphering

- GPRS GEA1, GEA2 and GEA3 ciphering

- GPRS	packet	switched	data	with CS1/CS2/CS3/CS4 coding schemes

- GPRS Class 12

- Supports SAIC (single antenna interference cancellation) technology

- Supports VAMOS(Voice services over Adaptive Multi-user channels on One Slot) technology in R9 spec.

Voice interface and voice front-end

- Microphone input has one low-noise amplifier with programmable gain and Automatic Gain Control (AGC) mechanisms

- Voice power amplifier with programmable gain

- 2nd order Sigma-Delta A/D converter for voice uplink path

- Shares D/A converter with audio playback path

- Supports full-duplex hands-free operation

- Compliant with GSM 03.50

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

### GSM/GPRS RF Features

Receiver

- Dual single-ended LNAs support Quad bandQuadrature RF mixer

- Fully integrated channel filter

- High dynamic range ADC

- 12dB PGA gain with 6dB gain step

Transmitter

- Transmitter outputs support quad bands.

- Highly precise and low noise RF transmitter for GSM/GPRS applications

Frequency synthesizer

- Programmable fractional-N synthesizer

- Integrated wide range RFVCO

- Integrated loop filter

- Fast settling time suitable for multi-slot GPRS applications

Digitally-Controlled Crystal Oscillator (DCXO)

- Two-pin 26MHz crystal oscillator

- On-chip programmable capacitor array for coarse-tuning

- On-chip programmable capacitor array for fine-tuning

- Low power mode supports 32K crystal removal

### Multimedia Features

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

LCD controller

- Supports simultaneous connection to seral 2 lane LCD modules

- LCM formats supported: RGB565, RGB666, RGB888

- Supports	LCD	module	with	maximum resolution up to 320x240

- Per pixel alpha channel

- True color engine

- Supports hardware display rotation

- Capable of combining display memories with up to 4 blending layers

Camera interface

- YUV422 format image input

- Capable of processing image of size up to VGA (Mediatek serial interface)

JPEG decoder

- Baseline JPEG decoding

- Supports various YUV formats, DC/AC Huffman tables and quantization tables

JPEG encoder

- ISO/IEC 10918-1 JPEG baseline mode

- ISO/IEC 10918-2 compliance

- Supports YUV420 and grayscale formats

- Supports EXIF/JFIF

- Standard DC and AC Huffman tables

- Provides 5 levels of encode quality

- Supports zeros shutter delay

MJPEG

- Decode spec: CIF@30fps

- Encode spec: QVGA@15fps

Image data processing

- Supports 4x digital zoom

- High throughput hardware scaler. Capable of

tailoring an image to an arbitrary size.

- Horizontal scaling with bilinear interpolation

- Vertical scaling with bilinear interpolation

- YUV and RGB color space conversion

- RGB/YCbCr format thumbnail output

MPEG-4/H.263 CODEC

- Software-based MPEG4 encoder

- Software-based MPEG4 decoder

- ISO/IEC 14496-2 simple profile:

  - Decode spec: 480x320@25fps

  - Encode spec: QVGA@15fps

- ISO/IEC 14496-2 advanced simple profile:

  - Decode @ level 0/1/2/3

  - ITU-T H.263 profile 0 @ level 40

- Supports visual tools for decoder: I-VOP, P-VOP, B-VOP, AC/DC prediction, 4-MV, unrestricted MV, error resilience, short header, global motion compensation, method 1/2 quantization, quarter-pel motion compensation.

- Error resilience for decoder: Slice resynchronization, data partitioning, reversible VLC

- Supports visual tools for encoder: I-VOP, P-VOP, Half-Pel, DC prediction, unrestricted MV, short header

H.264

- ISO/IEC 14496-10 baseline profile

  - Decode spec: QCIF@30fps

2D accelerator

- Supports 32-bpp ARGB8888, 24-bpp RGB888, 16-bpp RGB565, 24-bpp ARGB6666.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

4 layers overlay with individual color format, window size, source key, constant alpha and rotation

- Rectangle fill with constant

- BitBlt: Capable with 7 rotation types

- Alpha blending with 7 rotation types, per-pixel alpha and pre-multiplied alpha

- Font drawing: Normal font and anti-aliasing font

Audio CODEC

- Supports AAC codec decoding

- Wavetable synthesis with up to 64 tones

- Advanced wavetable synthesizer capable of generating simulated stereo

- Wavetable including GM full set of 128 instruments and 47 sets of percussions

- PCM playback and record

- Digital audio playback

Audio interface and audio front-end

- Supports I2S interface

- High-resolution D/A converters for stereo audio playback

- Voice band A/D converter support

- Stereo to mono conversion

### Bluetooth Features

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Radio features

- Fully compliant with Bluetooth specification

3.0 + EDR

- Low out-of-band spurious emissions support simultaneous operation with GPS and GSM/GPRS worldwide radio systems

- Low-IF architecture with high degree of linearity and high order channel filter

- Integrated T/R switch and Balun

- Fully integrated PA provides 7.5dBm output power

- -95dBm	sensitivity	with	excellent interference rejection performance

- Hardware AGC dynamically adjusts receiver performance in changing environments

Baseband features

- Up to 4 simultaneous active ACL links

- Up to 1 simultaneous SCO or eSCO link with CVSD coding

- Supports eSCO

- Scatternet support: Up to 4 piconets simultaneously	with	background inquiry/page scan

- Supports sniff mode

- AFH and PTA collaborative support for WLAN/BT coexistence

- Idle mode and sleep mode enables ultra-low power consumption.

- Supports PCM interface and built-in programmable transcoders for linear voice with re-transmission

- Built-in hardware modem engine for access code correlation, header error correction, forward error correction, CRC, whitening and encryption

- Channel quality driven data rate adaptation

- Channel assessment for AFH

Platform features

- Embedded processor for Bluetooth protocol stack with built-in memory system

- Fully verified ROM based system with code patch for feature enhancement

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

### FM Features

- 65-108MHz worldwide FM bands with 50KHz tuning step

- Supports RDS/RBDS radio data system

- Digital stereo demodulator

- Adaptive FM demodulator for both high- and low-quality scenarios

- Low	sensitivity	level	with	superior interference rejection

- Programmable	de-emphasis (bypass/50S/75S)

- Stereophonic multiplex signal (MPX) signal detection and demodulation

- Superior stereo noise reduction and soft mute volume control

- Audio dynamic range control

- Mono/stereo blending

- Audio sensitivity3dBµVemf (SINAD=26dB)

- Audio SINAD≥60dB

- Supports Anti-jamming algorithm

- Supports short antenna

### General Descriptions

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Figure 2 is the block diagram of MT6261D. Based on a multi-processor architecture, MT6261D integrates an ARM7EJ-STM core, the main processor running high-level GSM protocol software as well as multimedia applications, single digital signal processor core, which manages the low-level MODEM and advanced audio functions, an embedded processor running Bluetooth baseband and link control protocol and the Bluetooth radio control.

MT6261D consists of the following subsystems:

- Microcontroller Unit (MCU) subsystem: Includes an ARM7EJ-STM RISC processor and its accompanying memory management and interrupt handling logics

- Digital Signal Processor (DSP) subsystem: Includes a DSP and its accompanying memory, memory controller and interrupt controller

- MCU/DSP interface: Junction at which the MCU and the DSP exchange hardware and software information

- Microcontroller peripherals: Include all user interface modules and RF control interface modules

- Microcontroller coprocessors: Run computing-intensive processes in place of the microcontroller

- DSP peripherals: Hardware accelerators for GSM/GPRS channel codec

- Multimedia subsystem: Integrates several advanced accelerators to support multimedia applications

- Voice front-end: Data path for converting analog speech to and from digital speech

- Audio front-end: Data path for converting stereo audio from an audio source

- Baseband front-end: Data path for converting a digital signal to and from an analog signal from the RF modules

- Timing generator: Generates the control signals related to the TDMA frame timing

- Power, reset and clock subsystem: Manage the power, reset and clock distribution inside MT6261D.

- Bluetooth subsystem: Includes an embedded processor with embedded ROM/RAM system, baseband processor, and a high-performance radio block

- Power management unit: Self-contained power supply source which also controls the charging and system startup circuitry.

Details of the individual subsystems and blocks are described in the following chapters.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| GPTimer | Interrupt Controller | Boot ROM |
| --- | --- | --- |

GPS

UART

USB1.1	I2C

5x5 Qwerty  SD/SDIO

Keypad	Card

MT6261D

Peripherals

UART	UART  USB11	I2C

Keypad  SD/SDIO

SIM1

SIM Card

MCU

SIM2

SIM Card

DSP

JTAG

ICE

Internal

Memory

ARM7EJS

DMA

Speaker

Internal

Memory

DMA

GPIO

GPIO

Voice in

Touch Panel

Analog Baseband

AFE	Interrupt

VFE	Controller

BFE

DSP

Multimedia

JPEG	LCD

PA/ASW

RF

CODEC Control

HVGA

LCD

Scaler

BT RF

BLUESYS

Image

DMA

FM RF

FMSYS

Camera

INT

2D

VGA

Camera

Internal memory

PMU

Figure 2. MT6261D block diagram

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

# Product Descriptions

### Pin Description

##### Ball Diagram

For MT6261D, an TFBGA 8.1mm*7.6mm, 145-ball, 0.5mm pitch package is offered. Pin-outs and the top view are illustrated in Figure 3 for this package.

Figure 3. Ball diagram and top view

##### Pin Coordination

Table 1. Pin coordinates

| Pin# | Net name | Pin# | Net name | Pin# | Net name |
| --- | --- | --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Pin# | Net name | Pin# | Net name | Pin# | Net name |
| --- | --- | --- | --- | --- | --- |
| A1 | RXLB | D16 | UTXD1 | J1 | VCDT |
| A10 | CMCSD0 | D2 | TX_HB | J12 | GND |
| A12 | CMRST | D4 | GPIO_10 | J13 | RESETB |
| A13 | KCOL1 | D5 | EXT_CLK_SEL | J14 | VSF |
| A15 | KROW1 | D8 | VIO28 | J15 | LSCE_B |
| A16 | KROW0 | D9 | SDA28 | J16 | LSRSTB |
| A2 | AVSS_2G | E12 | GND | J2 | BATON |
| A3 | XTAL1 | E13 | GND | J9 | GND |
| A4 | XTAL2 | E14 | GPIO_0 | K1 | DRV |
| A6 | AVSS_BT | E15 | GPIO_1 | K10 | GND |
| A7 | BT_LNA | E2 | AVSS44_ALDO | K15 | LSDA |
| A9 | BPI_BUS2 | E5 | VRF | K16 | LSCK |
| B10 | CMMCLK | E8 | SCL28 | K2 | KPLED |
| B11 | CMCSK | E9 | GND | K3 | ISINK |
| B12 | CMPDN | F1 | AVSS_FM | L10 | AVSS44_DLDO |
| B13 | KCOL0 | F10 | GND | L15 | LSA0 |
| B15 | KROW2 | F15 | GPIO_2 | L2 | AVSS44_PMU |
| B2 | RXHB | F16 | GPIO_3 | L4 | AVDD25_V2P5 |
| B3 | EINT | F2 | FM_ANT_P | L7 | AU_MICBIAS0 |
| B4 | AVSS_2G | G1 | VREF | L8 | ACCDET |
| B5 | GPIO_11 | G15 | GPIO_5 | M1 | SPK_OUTN |
| B6 | FREF | G16 | GPIO_4 | M11 | SIM1_SIO |
| B7 | AVSS_BT | G2 | PWRKEY | M12 | VIO18 |
| B8 | BPI_BUS1 | G3 | VBAT_VA | M13 | SIM2_SIO |
| B9 | BPI_BUS0 | G4 | VCAMA | M14 | VSIM2 |
| C1 | TX_LB | G5 | TESTMODE | M15 | LPTE |
| C12 | VDDK | G6 | AVSS44_ALDO | M16 | SIM2_SRST |
| C13 | KCOL2 | G7 | GND | M2 | AVSS_SPK |
| C14 | KCOL3 | G8 | GND | M4 | VSBST |
| C15 | KROW3 | G9 | GND | M5 | VSBST |
| C16 | KROW4 | H12 | GPIO_9 | M7 | AU_VIN0_P |
| C2 | AVSS_2G | H13 | GPIO_8 | M8 | APC |
| C5 | SRCLKENAI | H14 | GPIO_7 | N1 | SPK_OUTP |
| C8 | AVDD15_BTRF | H15 | GPIO_6 | N11 | VUSB |
| C9 | CMCSD1 | H2 | CHRLDO | N13 | MCDA3 |
| D1 | AVSS_2G | H3 | BATSNS | N14 | MCCM0 |
| D12 | GND | H4 | ISENSE | N15 | MCDA1 |
| D14 | KCOL4 | H8 | GND | N16 | SIM2_SCLK |
| D15 | URXD1 | H9 | GND | N2 | AVSS_SPK |
| N4 | AVSS44_BOOST | P4 | VSBST_OUT | R15 | MCCK |
| N5 | AVSS44_BOOST | P5 | AVSS28_ABB | R16 | VMC |
| N7 | AU_VIN0_N | P6 | AU_VIN1_P | R2 | AU_HPL |
| N8 | AUXIN4 | P7 | AU_VIN1_N | R3 | AU_HSP |
| P10 | USB11_DM | P8 | VCORE | R4 | AU_HSN |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Pin# | Net name | Pin# | Net name | Pin# | Net name |
| --- | --- | --- | --- | --- | --- |
| P11 | USB11_DP | P9 | VRTC | R5 | VA |
| P12 | SIM1_SCLK | R1 | AU_HPR | R7 | AVSS28_ABB |
| P13 | MCDA2 | R10 | VIBR | R9 | VBAT_DIGITAL |
| P15 | MCDA0 | R12 | SIM1_SRST |  |  |
| P2 | AVDD_SPK | R13 | VSIM1 |  |  |

##### Detailed Pin Description

Table 2. Acronym for pin types

| Abbreviation | Description |
| --- | --- |
| AI | Analog input |
| AO | Analog output |
| AIO | Analog bi-direction |
| DI | Digital input |
| DO | Digital output |
| DIO | Digital bi-direction |
| P | Power |
| G | Ground |

Table 3. PIN function description and power domain

| Pin name | Type | Description | Power domain |
| --- | --- | --- | --- |
| System |  |  |  |
| RESETB | DIO | System reset | DVDD18_EMI |
| SRCLKENAI | DIO | 26MHz clock request by external devices | VRF |
| EINT | DIO | External Interrupt | VRF |
| GPIO_0 | DIO | General purpose input /output 0 | DVDD28 |
| GPIO_1 | DIO | General purpose input /output 1 | DVDD28 |
| GPIO_2 | DIO | General purpose input /output 2 | DVDD28 |
| GPIO_3 | DIO | General purpose input /output 3 | DVDD28 |
| GPIO_4 | DIO | General purpose input /output 4 | DVDD28 |
| GPIO_5 | DIO | General purpose input /output 5 | DVDD28 |
| GPIO_6 | DIO | General purpose input /output 6 | DVDD28 |
| GPIO_7 | DIO | General purpose input /output 7 | DVDD28 |
| GPIO_8 | DIO | General purpose input /output 8 | DVDD28 |
| GPIO_9 | DIO | General purpose input /output 9 | DVDD28 |
| GPIO_10 | DIO | General purpose input /output 10 | VRF |
| GPIO_11 | DIO | General purpose input /output 11 | VRF |
| RF control circuitro |  |  |  |
| BPI_BUS0 | DIO | RF hard-wire control bus bit 0 | DVDD28 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Pin name | Type | Description | Power domain |
| --- | --- | --- | --- |
| BPI_BUS1 | DIO | RF hard-wire control bus bit 1 | DVDD28 |
| BPI_BUS2 | DIO | RF hard-wire control bus bit 2 | DVDD28 |
| UART interface |  |  |  |
| URXD1 | DIO | UART1 receive data | DVDD28 |
| UTXD1 | DIO | UART1 transmit data | DVDD28 |
| Keypad interface |  |  |  |
| KCOL0 | DIO | Keypad column 0 | DVDD28 |
| KCOL1 | DIO | Keypad column 1 | DVDD28 |
| KCOL2 | DIO | Keypad column 2 | DVDD28 |
| KCOL3 | DIO | Keypad column 3 | DVDD28 |
| KCOL4 | DIO | Keypad column 4 | DVDD28 |
| KROW0 | DIO | Keypad row 0 | DVDD28 |
| KROW1 | DIO | Keypad row 1 | DVDD28 |
| KROW2 | DIO | Keypad row 2 | DVDD28 |
| KROW3 | DIO | Keypad row 3 | DVDD28 |
| KROW4 | DIO | Keypad row 4 | DVDD28 |
| Camera interface |  |  |  |
| CMRST | DIO | CMOS sensor reset signal output | DVDD28 |
| CMPDN | DIO | CMOS sensor power down control | DVDD28 |
| CMCSD0 | DIO | CMOS sensor data input 0 | DVDD28 |
| CMCSD1 | DIO | CMOS sensor data input 1 | DVDD28 |
| CMMCLK | DIO | CMOS sensor pixel clock input | DVDD28 |
| CMCSK | DIO | CMOS sensor pixel clock output | DVDD28 |
| MS/SD card interface |  |  |  |
| MCDA0 | DIO | SD serial data IO 0/memory stick serial data IO | DVDD33_MSDC |
| MCDA1 | DIO | SD serial data IO 1/memory stick serial data IO | DVDD33_MSDC |
| MCDA2 | DIO | SD serial data IO 2/memory stick serial data IO | DVDD33_MSDC |
| MCDA3 | DIO | SD serial data IO 3/memory stick serial data IO | DVDD33_MSDC |
| MCCK | DIO | SD serial clock/memory stick serial clock | DVDD33_MSDC |
| MCCM0 | DIO | SD command output/memory stick bus state output | DVDD33_MSDC |
| SIM card interface |  |  |  |
| SIM1_SIO | DIO | SIM1 data input/outputs | VSIM1 |
| SIM1_SRST | DIO | SIM1 card reset output | VSIM1 |
| SIM1_SCLK | DIO | SIM1 card clock output | VSIM1 |
| SIM2_SIO | DIO | SIM2 data input/outputs | VSIM2 |
| SIM2_SRST | DIO | SIM2 card reset output | VSIM2 |
| SIM2_SCLK | DIO | SIM2 card clock output | VSIM2 |
| I2C interface |  |  |  |
| SCL28 | DIO | I2C clock 2.8v power domain | DVDD28 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Pin name | Type | Description | Power domain |
| --- | --- | --- | --- |
| SDA28 | DIO | I2C data 2.8v power domain | DVDD28 |
| LCD interface |  |  |  |
| LSRSTB | DIO | Serial display interface reset signal | DVDD18_EMI |
| LSCE_B | DIO | Serial display interface chip select output | DVDD18_EMI |
| LSCK | DIO | Serial display interface clock | DVDD18_EMI |
| LSDA | DIO | Serial display interface data | DVDD18_EMI |
| LSA0 | DIO | Serial display interface address | DVDD18_EMI |
| LPTE | DIO | Serial display tearing signal | DVDD18_EMI |
| FM |  |  |  |
| FM_ANT_P | AI | FM input from antenna | VCAMA |
| Bluetooth |  |  |  |
| BT_LNA | AIO | Bluetooth RF single-ended input | DVDD28 |
| 2G RF |  |  |  |
| RXHB | AI | RF input for highband Rx (DCS/PCS) | VRF |
| RXLB | AI | RF input for lowband Rx (GSM900/GSM850) | VRF |
| TX_HB | AO | RF output for highband Tx (DCS/PCS) | VRF |
| TX_LB | AO | RF output pin for lowband Tx (GSM900/GSM850) | VRF |
| FREF | AO | DCXO reference clock output | VRF |
| XTAL1 | AIO | Input 1 for DCXO crystal | VRF |
| XTAL2 | AIO | Input 2 for DCXO crystal | VRF |
| EXT_CLK_SEL | AIO | DCXO mode selection | VRF |
| USB |  |  |  |
| USB11_DM | AIO | D- data input/output | - |
| USB11_DP | AIO | D+ data input/output | - |
| Analog baseband |  |  |  |
| AU_HPR | AIO | Audio head phone output (R channel) | AVDD28_ABB |
| AU_HPL | AIO | Audio head phone output (L channel) | AVDD28_ABB |
| AU_HSP | AIO | Voice handset output (positive) | AVDD28_ABB |
| AU_HSN | AIO | Voice handset output (negative) | AVDD28_ABB |
| AU_VIN0_P | AIO | Microphone 0 input (positive) | AVDD28_ABB |
| AU_VIN0_N | AIO | Microphone 0 input (negative) | AVDD28_ABB |
| AU_VIN1_P | AIO | Microphone 1 input (positive) | AVDD28_ABB |
| AU_VIN1_N | AIO | Microphone 1 input (negative) | AVDD28_ABB |
| AUX_IN4 | AIO | Auxiliary ADC input | AVDD28_ABB |
| SPK_OUTP | AIO | Speaker positive output | VBAT_SPK |
| SPK_OUTN | AIO | Speaker negative output | VBAT_SPK |
| APC | AIO | Automatic power control DAC output | AVDD28_ABB |
| AU_MICBIAS0 | AIO | Microphone bias source 0 | AVDD28_ABB |
| ACCDET | AIO | Accessory detection | AVDD28_ABB |
| Power management unit |  |  |  |
| VA | AIO | LDO output for ABB - VA | VBAT_ANALOG |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Pin name | Type | Description | Power domain |
| --- | --- | --- | --- |
| VCAMA | AIO | LDO output for sensor – VCAMA | VBAT_VA |
| VIBR | AIO | LDO output for vibrator - VIBR | VBAT_DIGITAL |
| VIO18 | AIO | LDO output for 1.8V power - VIO18 | VBAT_DIGITAL |
| VIO28 | AIO | LDO output for 2.8V power - VIO28 | VBAT_DIGITAL |
| VMC | AIO | LDO output for memory card - VMC | VBAT_DIGITAL |
| VSF | AIO | LDO output - VSF | VBAT_DIGITAL |
| VRF | AIO | LDO output for GSMRF - VRF | VBAT_VA |
| VRTC | AIO | LDO output for RTC - VRTC | VBAT_DIGITAL |
| VSIM1 | AIO | LDO output for 1st SIM - VSIM | VBAT_DIGITAL |
| VSIM2 | AIO | LDO output for 2nd SIM - VSIM2 | VBAT_DIGITAL |
| VUSB | AIO | LDO output for USB - VUSB | VBAT_DIGITAL |
| VCORE | AIO | LDO output for core circuit - Vcore | VBAT_DIGITAL |
| VREF | AIO | Band gap reference | BATSNS |
| VCDT | AIO | Charger-In level sense pin | BATSNS |
| DRV | AIO | IDAC current output open-drain pin | BATSNS |
| BATON | AIO | Battery Pack, NTC connected pin | BATSNS |
| ISENSE | AIO | Top node of current sensing 0.2ohm Rsense resistor | BATSNS |
| CHRLDO | AIO | 2.8V shunt-regulator output | BATSNS |
| BATDET | AIO | Battery detection pin | BATSNS |
| ISINK0 | AIO | Backlight driver channel 0 | VBAT_VA |
| KPLED | AIO | Keypad led driver | VBAT_VA |
| VSBST_OUT | AIO | Audio boost output | VSBST |
| TESTMODE | AIO | Test mode | BATSNS |
| PWRKEY | AIO | PWR key | BATSNS |
| AVDD25_V2P5 | AIO | Reference voltage for ABT | - |
| Analog power |  |  |  |
| VSBST | P | VBAT input for audio boost |  |
| AVDD15_BTRF | P | BTRF power input | - |
| VBAT_DIGITAL | P | Digital LDOs used battery voltage input | - |
| VBAT_VA | P | Analog LDOs used battery voltage input | - |
| AVDD_SPK | P | Input for loud speaker driver | - |
| BATSNS | P | Battery node of battery pack | - |
| AVDD15_BTRF | P | BTRF power input | - |
| Analog ground |  |  |  |
| AVSS28_ABB | G | ABB 2.8V ground | - |
| AVSS_BT | G | BT ground | - |
| AVSS_2G | G | 2G RF ground | - |
| AVSS_FM | G | FM ground | - |
| AVSS44_PMU | G | PMU ground | - |
| AVSS44_ALDO | G | ALDO ground | - |
| AVSS44_DLDO | G | DLDO ground | - |
| AVSS_SPK | G | SPK ground | - |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Pin name | Type | Description | Power domain |
| --- | --- | --- | --- |
| AGND | G | GND for VREF | - |
| AVSS44_BOOST | G | Audio boost GND | - |
| Digital power |  |  |  |
| VDDK | P | Core power | - |
| Digital ground |  |  |  |
| GND | G | Ground | - |

Table 4. Acronym for state of pins

| Abbreviation | Description |
| --- | --- |
| I | Input |
| LO | Low output |
| HO | High output |
| LO | Low output |
| PU | Pull-up |
| PD | Pull-down |
| - | No PU/PD |
| 0~N | Aux. function number |
| X | Delicate function pin |

Table 5. State of pins

| Name | Reset |  |  | Output drivability | Termination when not used | IO type |
| --- | --- | --- | --- | --- | --- | --- |
|  | State1 | Aux2 | PU/PD3 |  |  |  |
| System |  |  |  |  |  |  |
| RESETB | HO | 1 | - | DIOH3/DIOL3 | No need | IO Type 3 |
| SRCLKENAI | I | 0 | PD | DIOH1/DIOL1 | No need | IO Type 1 |
| EINT | I | 0 | PD | DIOH1/DIOL1 | No need | IO Type 1 |
| GPIO_0 | I | 0 | PD | DIOH1/DIOL1 | No need | IO Type 1 |
| GPIO_1 | I | 0 | PD | DIOH1/DIOL1 | No need | IO Type 1 |
| GPIO_2 | I | 0 | PD | DIOH1/DIOL1 | No need | IO Type 1 |
| GPIO_3 | I | 0 | PD | DIOH1/DIOL1 | No need | IO Type 1 |
| GPIO_4 | I | 0 | PD | DIOH2/DIOL2 | No need | IO Type 2 |

1 The column “State” of “Reset” shows the pin state during reset. (Input, High Output, Low Output, etc)

2 The column “Aux” for “Reset” means the default aux function number, shown in the table “Pin Multiplexing, Capability and Settings”.

3 The column “PU/PD” for “Reset” means if there is internal pull-up or pull-down when the pin is input in the reset state.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Name | Reset |  |  | Output drivability | Termination when not used | IO type |
| --- | --- | --- | --- | --- | --- | --- |
|  | State1 | Aux2 | PU/PD3 |  |  |  |
| GPIO_5 | LO | 0 | PD | DIOH2/DIOL2 | No need | IO Type 2 |
| GPIO_6 | LO | 0 | PD | DIOH2/DIOL2 | No need | IO Type 2 |
| GPIO_7 | LO | 0 | PD | DIOH2/DIOL2 | No need | IO Type 2 |
| GPIO_8 | I | 0 | PD | DIOH2/DIOL2 | No need | IO Type 2 |
| GPIO_9 | I | 0 | PD | DIOH2/DIOL2 | No need | IO Type 2 |
| GPIO_10 | I | 0 | PD | DIOH1/DIOL1 | No need | IO Type 1 |
| GPIO_11 | I | 0 | PD | DIOH1/DIOL1 | No need | IO Type 1 |
| RF control circuitry |  |  |  |  |  |  |
| BPI_BUS0 | LO | 1 | PD | DIOH2/DIOL2 | No need | IO Type 2 |
| BPI_BUS1 | I | 1 | PD | DIOH2/DIOL2 | No need | IO Type 2 |
| BPI_BUS2 | I | 1 | PD | DIOH2/DIOL2 | No need | IO Type 2 |
| UART interface |  |  |  |  |  |  |
| URXD1 | I | 1 | PU | DIOH3/DIOL3 | No need | IO Type 3 |
| UTXD1 | HO | 1 | PU | DIOH2/DIOL2 | No need | IO Type 2 |
| Keypad Interface |  |  |  |  |  |  |
| KCOL0 | I | 0 | PU | DIOH4/DIOL4 | No need | IO Type 4 |
| KCOL1 | I | 0 | PU | DIOH4/DIOL4 | No need | IO Type 4 |
| KCOL2 | I | 0 | PU | DIOH4/DIOL4 | No need | IO Type 4 |
| KCOL3 | I | 0 | PU | DIOH4/DIOL4 | No need | IO Type 4 |
| KCOL4 | I | 0 | PU | DIOH4/DIOL4 | No need | IO Type 4 |
| KROW0 | I | 0 | PD | DIOH5/DIOL5 | No need | IO Type 5 |
| KROW1 | I | 0 | PD | DIOH5/DIOL5 | No need | IO Type 5 |
| KROW2 | I | 0 | PD | DIOH6/DIOL6 | No need | IO Type 6 |
| KROW3 | I | 0 | PD | DIOH6/DIOL6 | No need | IO Type 6 |
| KROW4 | I | 0 | PD | DIOH6/DIOL6 | No need | IO Type 6 |
| Camera interface |  |  |  |  |  |  |
| CMRST | I | 0 | PD | DIOH2/DIOL2 | No need | IO Type 2 |
| CMPDN | HO | 0 | - | DIOH3/DIOL3 | No need | IO Type 3 |
| CMCSD0 | I | 0 | PU | DIOH3/DIOL3 | No need | IO Type 3 |
| CMCSD1 | I | 0 | PD | DIOH3/DIOL3 | No need | IO Type 3 |
| CMMCLK | I | 0 | PD | DIOH3/DIOL3 | No need | IO Type 3 |
| CMCSK | I | 0 | PD | DIOH2/DIOL2 | No need | IO Type 2 |
| MS/SD card interface |  |  |  |  |  |  |
| MCDA0 | I | 0 | PD | DIOH3/DIOL3 | No need | IO Type 3 |
| MCDA1 | I | 0 | PD | DIOH3/DIOL3 | No need | IO Type 3 |
| MCDA2 | I | 0 | PD | DIOH3/DIOL3 | No need | IO Type 3 |
| MCDA3 | I | 0 | PD | DIOH3/DIOL3 | No need | IO Type 3 |
| MCCK | I | 0 | PU | DIOH3/DIOL3 | No need | IO Type 3 |
| MCCM0 | I | 0 | PU | DIOH3/DIOL3 | No need | IO Type 3 |
| SIM card interface |  |  |  |  |  |  |
| SIM1_SIO | I | 1 | PD | DIOH6/DIOL6 | No need | IO Type 6 |
| SIM1_SRST | I | 1 | PD | DIOH6/DIOL6 | No need | IO Type 6 |

| Name | Reset |  |  | Output drivability | Termination when not used | IO type |
| --- | --- | --- | --- | --- | --- | --- |
|  | State1 | Aux2 | PU/PD3 |  |  |  |
| SIM1_SCLK | I | 1 | PD | DIOH6/DIOL6 | No need | IO Type 6 |
| SIM2_SIO | I | 1 | PD | DIOH6/DIOL6 | No need | IO Type 6 |
| SIM2_SRST | I | 1 | PD | DIOH6/DIOL6 | No need | IO Type 6 |
| SIM2_SCLK | I | 1 | PD | DIOH6/DIOL6 | No need | IO Type 6 |
| I2C interface |  |  |  |  |  |  |
| SCL28 | I | 0 | PD | DIOH2/DIOL2 | No need | IO Type 2 |
| SDA28 | I | 0 | PD | DIOH2/DIOL2 | No need | IO Type 2 |
| LCD interface |  |  |  |  |  |  |
| LSRSTB | I | 0 | PD | DIOH3/DIOL3 | No need | IO Type 3 |
| LSCE_B | HO | 1 | - | DIOH3/DIOL3 | No need | IO Type 3 |
| LSCK | I | 0 | PD | DIOH3/DIOL3 | No need | IO Type 3 |
| LSDA | I | 0 | PD | DIOH3/DIOL3 | No need | IO Type 3 |
| LSA0 | I | 0 | PD | DIOH3/DIOL3 | No need | IO Type 3 |
| LPTE | I | 0 | PD | DIOH3/DIOL3 | No need | IO Type 3 |

DVDIO

DVO

PAD

Output

Enable

Input Enable

DVDIO GNDIO

PU

VIN

PD

GNDIO

DVDIO

DVO

PAD

Output

Enable

Input Enable

DVDIO GNDIO

PU

VIN

PD

GNDIO

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

IO type1

IO type2

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

DVDIO

DVO

Output Enable

DVDIO GNDIO

PU

VIN

PD

GNDIO

PAD

DVDIO

DVO

PAD

Output

Enable

DVDIO GNDIO

PU

VIN

PD

GNDIO

IO type3	IO type4

DVDIO

DVO

Output Enable

DVDIO GNDIO

PU

VIN

PD

GNDIO

PAD

DVDIO

DVO

PAD

Output

Enable

DVDIO GNDIO

PU

VIN

PD

GNDIO

IO type5	IO type6

Figure 4. IO types in state of pins

##### Pin Multiplexing, Capability and Settings

Table 6. Acronym for pull-up and pull-down types

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Abbreviation | Description |
| --- | --- |
| PU | Pull-up, not controllable |
| PD | Pull-down, not controllable |
| CU | Pull-up, controllable |
| CD | Pull-down, controllable |
| X | Cannot pull-up or pull-down |

Table 7. Capability of PU/PD, driving and Schmitt trigger

| Name | Aux. function | Aux. name | Aux. type | PU/PD/ CU/CD | Driving | SMT |
| --- | --- | --- | --- | --- | --- | --- |
| GPIO_0 | 0 | GPIO0 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | EINT0 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 2 | XP | AIO | - | 4, 8, 12, 16mA | 0 |
|  | 3 | U3RXD | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 4 | CMCSD2 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 5 | CMCSK | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 6 | EDIDO | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 7 | JTDI | I | PU | 4, 8, 12, 16mA | 0 |
|  | 8 | BTJTDI | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 9 | FMJTDI | I | CU, CD | 4, 8, 12, 16mA | 0 |
| GPIO_1 | 0 | GPIO1 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | EINT1 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 2 | XM | AIO | - | 4, 8, 12, 16mA | 0 |
|  | 3 | U3TXD | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 4 | U1CTS | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 5 | CMMCLK | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 6 | EDIDI | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 7 | JTMS | I | PU | 4, 8, 12, 16mA | 0 |
|  | 8 | BTJTMS | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 9 | FMJTMS | I | CU, CD | 4, 8, 12, 16mA | 0 |
| GPIO_2 | 0 | GPIO2 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | EINT2 | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 2 | YP | AIO | - | 4, 8, 12, 16mA | 0 |
|  | 3 | GPSFSYNC | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 4 | PWM0 | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 5 | CMCSD0 | I | CU, CD | 4, 8, 12, 16mA | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Name | Aux. function | Aux. name | Aux. type | PU/PD/ CU/CD | Driving | SMT |
| --- | --- | --- | --- | --- | --- | --- |
|  | 6 | EDIWS | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 7 | JTRST_B | I | PD | 4, 8, 12, 16mA | 0 |
|  | 8 | BTJTRSTB | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 9 | FMJTRSTB | I | CU, CD | 4, 8, 12, 16mA | 0 |
| GPIO_3 | 0 | GPIO3 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | MCINS | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 2 | YM | AIO | - | 4, 8, 12, 16mA | 0 |
|  | 4 | PWM1 | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 5 | CMCSD1 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 6 | EDICK | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 7 | JTDO | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 8 | BTJTDO | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 9 | FMJTDO | O | CU, CD | 4, 8, 12, 16mA | 0 |
| GPIO_4 | 0 | GPIO4 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | EINT3 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 4 | U1RTS | O | CU, CD | 4, 8, 12, 16mA | 0 |
| GPIO_5 | 0 | GPIO5 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | EINT4 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 3 | BPI_BUS3 | O | CU, CD | 4, 8, 12, 16mA | 0 |
| GPIO_6 | 0 | GPIO6 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | EINT5 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 2 | MCINS | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 3 | BPI_BUS4 | O | CU, CD | 4, 8, 12, 16mA | 0 |
| GPIO_7 | 0 | GPIO7 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | EINT6 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 3 | BPI_BUS5 | O | CU, CD | 4, 8, 12, 16mA | 0 |
| GPIO_8 | 0 | GPIO8 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | EINT7 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 2 | SCL | IO | CU, CD | 4, 8, 12, 16mA | 0 |
| GPIO_9 | 0 | GPIO9 | IO | CU, CD | 4, 8, 12, 16mA | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Name | Aux. function | Aux. name | Aux. type | PU/PD/ CU/CD | Driving | SMT |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | EINT8 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 2 | SDA | IO | CU, CD | 4, 8, 12, 16mA | 0 |
| URXD1 | 0 | GPIO10 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | U1RXD | I | PU | 4, 8, 12, 16mA | 0 |
|  | 2 | CMRST | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 3 | EINT9 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 4 | MCINS | I | CU, CD | 4, 8, 12, 16mA | 0 |
| UTXD1 | 0 | GPIO11 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | U1TXD | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 2 | CMPDN | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 3 | EINT10 | I | CU, CD | 4, 8, 12, 16mA | 0 |
| KCOL4 | 0 | GPIO12 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | KCOL4 | IO | - | 4, 8, 12, 16mA | 0 |
|  | 2 | U2RXD | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 3 | EDIDI | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 4 | FMJTDI | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 5 | JTDI | I | PU | 4, 8, 12, 16mA | 0 |
|  | 6 | BTJTDI | I | CU, CD | 4, 8, 12, 16mA | 0 |
| KCOL3 | 0 | GPIO13 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | KCOL3 | IO | - | 4, 8, 12, 16mA | 0 |
|  | 2 | EINT11 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 3 | PWM0 | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 4 | FMJTMS | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 5 | JTMS | I | PU | 4, 8, 12, 16mA | 0 |
|  | 6 | BTJTMS | I | CU, CD | 4, 8, 12, 16mA | 0 |
| KCOL2 | 0 | GPIO14 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | KCOL2 | IO | - | 4, 8, 12, 16mA | 0 |
|  | 2 | EINT12 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 3 | U1RTS | I | CU, CD | 4, 8, 12, 16mA | 0 |
| KCOL1 | 0 | GPIO15 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | KCOL1 | IO | - | 4, 8, 12, 16mA | 0 |
|  | 2 | GPSFSYNC | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 3 | U1CTS | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 4 | FMJTCK | I | CU, CD | 4, 8, 12, 16mA | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Name | Aux. function | Aux. name | Aux. type | PU/PD/ CU/CD | Driving | SMT |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5 | JTCK | I | PU | 4, 8, 12, 16mA | 0 |
|  | 6 | BTJTCK | I | CU, CD | 4, 8, 12, 16mA | 0 |
| KCOL0 | 0 | GPIO16 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | KCOL0 | IO | - | 4, 8, 12, 16mA | 0 |
| KROW4 | 0 | GPIO17 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | KROW4 | IO | - | 4, 8, 12, 16mA | 0 |
|  | 2 | U2TXD | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 3 | EDICK | O | CU, CD | 4, 8, 12, 16mA | 0 |
| KROW3 | 0 | GPIO18 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | KROW3 | IO | - | 4, 8, 12, 16mA | 0 |
|  | 2 | EINT13 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 3 | CLKO0 | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 4 | FMJTRSTB | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 5 | JTRST_B | I | PD | 4, 8, 12, 16mA | 0 |
|  | 6 | BTJTRSTB | I | CU, CD | 4, 8, 12, 16mA | 0 |
| KROW2 | 0 | GPIO19 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | KROW2 | IO | - | 4, 8, 12, 16mA | 0 |
|  | 2 | PWM1 | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 3 | EDIWS | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 4 | FMJTDO | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 5 | JTDO | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 6 | BTJTDO | O | CU, CD | 4, 8, 12, 16mA | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Name | Aux. function | Aux. name | Aux. type | PU/PD/ CU/CD | Driving | SMT |
| --- | --- | --- | --- | --- | --- | --- |
| KROW1 | 0 | GPIO20 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | KROW1 | IO | - | 4, 8, 12, 16mA | 0 |
|  | 2 | EINT14 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 3 | EDIDO | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 4 | BTPRI | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 5 | JTRCK | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 6 | BTDBGACKN | O | CU, CD | 4, 8, 12, 16mA | 0 |
| KROW0 | 0 | GPIO21 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | KROW0 | IO | - | 4, 8, 12, 16mA | 0 |
|  | 5 | MCINS | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 6 | BTDBGIN | I | CU, CD | 4, 8, 12, 16mA | 0 |
| BPI_BUS2 | 0 | GPIO22 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | BPI_BUS2 | O | CU, CD | 4, 8, 12, 16mA | 0 |
| BPI_BUS1 | 0 | GPIO23 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | BPI_BUS1 | O | CU, CD | 4, 8, 12, 16mA | 0 |
| BPI_BUS0 | 0 | GPIO24 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | BPI_BUS0 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
| CMRST | 0 | GPIO25 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | CMRST | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 2 | LSRSTB | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 3 | CLKO1 | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 4 | EINT15 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 5 | FMJTDI | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 6 | JTDI | I | PU | 4, 8, 12, 16mA | 0 |
| CMPDN | 0 | GPIO26 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | CMPDN | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 2 | LSCK1 | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 3 | DAICLK | O | CU, CD | 4, 8, 12, 16mA | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Name | Aux. function | Aux. name | Aux. type | PU/PD/ CU/CD | Driving | SMT |
| --- | --- | --- | --- | --- | --- | --- |
|  | 4 | SPICS | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 5 | FMJTMS | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 6 | JTMS | I | PU | 4, 8, 12, 16mA | 0 |
| CMCSD0 | 0 | GPIO27 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | CMCSD0 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 2 | LSCE_B1 | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 3 | DAIPCMIN | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 4 | SPISCK | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 5 | FMJTCK | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 6 | JTCK | I | PU | 4, 8, 12, 16mA | 0 |
|  | 8 | MC2CM0 | O | - | 4, 8, 12, 16mA | 0 |
| CMCSD1 | 0 | GPIO28 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | CMCSD1 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 2 | LSDA1 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 3 | DAIPCMOUT | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 4 | SPIMOSI | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 5 | FMJTRSTB | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 6 | JTRST_B | I | PD | 4, 8, 12, 16mA | 0 |
|  | 8 | MC2CK | O | - | 4, 8, 12, 16mA | 0 |
| CMMCLK | 0 | GPIO29 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | CMMCLK | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 2 | LSA0DA1 | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 3 | DAISYNC | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 4 | SPIMISO | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 5 | FMJTDO | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 6 | JTDO | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 8 | MC2DA0 | IO | - | 4, 8, 12, 16mA | 0 |
| CMCSK | 0 | GPIO30 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | CMCSK | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 2 | LPTE | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 3 | CMCSD2 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 4 | EINT16 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 6 | JTRCK | O | CU, CD | 4, 8, 12, 16mA | 0 |
| MCCK | 0 | GPIO31 | IO | CU, CD | 4, 8, 12, 16mA | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Name | Aux. function | Aux. name | Aux. type | PU/PD/ CU/CD | Driving | SMT |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | MCCK | O | - | 4, 8, 12, 16mA | 0 |
|  | 4 | U2RXD | I | CU, CD | 4, 8, 12, 16mA | 0 |
| MCCM0 | 0 | GPIO32 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | MCCM0 | IO | - | 4, 8, 12, 16mA | 0 |
|  | 4 | U2TXD | O | CU, CD | 4, 8, 12, 16mA | 0 |
| MCDA0 | 0 | GPIO33 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | MCDA0 | IO | - | 4, 8, 12, 16mA | 0 |
|  | 4 | DAISYNC | O | CU, CD | 4, 8, 12, 16mA | 0 |
| MCDA1 | 0 | GPIO34 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | MCDA1 | IO | - | 4, 8, 12, 16mA | 0 |
|  | 2 | EINT17 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 4 | DAIPCMIN | I | CU, CD | 4, 8, 12, 16mA | 0 |
| MCDA2 | 0 | GPIO35 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | MCDA2 | IO | - | 4, 8, 12, 16mA | 0 |
|  | 2 | EINT18 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 4 | DAICLK | O | CU, CD | 4, 8, 12, 16mA | 0 |
| MCDA3 | 0 | GPIO36 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | MCDA3 | IO | - | 4, 8, 12, 16mA | 0 |
|  | 2 | EINT19 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 3 | CLKO2 | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 4 | DAIPCMOUT | O | CU, CD | 4, 8, 12, 16mA | 0 |
| SIM1_SIO | 0 | GPIO37 | IO | CU, CD | 2, 4, 6, 8mA | 0 |
|  | 1 | SIM1_SIO | IO | - | 2, 4, 6, 8mA | 0 |
| SIM1_SRST | 0 | GPIO38 | IO | CU, CD | 2, 4, 6, 8mA | 0 |
|  | 1 | SIM1_SRST | IO | - | 2, 4, 6, 8mA | 0 |
| SIM1_SCLK | 0 | GPIO39 | IO | CU, CD | 2, 4, 6, 8mA | 0 |
|  | 1 | SIM1_SCLK | IO | - | 2, 4, 6, 8mA | 0 |
| SIM2_SIO | 0 | GPIO40 | IO | CU, CD | 2, 4, 6, 8mA | 0 |
|  | 1 | SIM2_SIO | IO | - | 2, 4, 6, 8mA | 0 |
|  | 3 | U2RTS | O | CU, CD | 2, 4, 6, 8mA | 0 |
| SIM2_SRST | 0 | GPIO41 | IO | CU, CD | 2, 4, 6, 8mA | 0 |
|  | 1 | SIM2_SRST | IO | - | 2, 4, 6, 8mA | 0 |
|  | 2 | CLKO3 | O | CU, CD | 2, 4, 6, 8mA | 0 |
|  | 3 | U2CTS | I | CU, CD | 2, 4, 6, 8mA | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Name | Aux. function | Aux. name | Aux. type | PU/PD/ CU/CD | Driving | SMT |
| --- | --- | --- | --- | --- | --- | --- |
| SIM2_SCLK | 0 | GPIO42 | IO | CU, CD | 2, 4, 6, 8mA | 0 |
|  | 1 | SIM2_SCLK | IO | - | 2, 4, 6, 8mA | 0 |
|  | 2 | LSCE1_B1 | O | CU, CD | 2, 4, 6, 8mA | 0 |
| SCL | 0 | GPIO43 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | SCL | IO | CU, CD | 4, 8, 12, 16mA | 0 |
| SDA | 0 | GPIO44 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | SDA | IO | CU, CD | 4, 8, 12, 16mA | 0 |
| LSRSTB | 0 | GPIO45 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | LSRSTB | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 3 | CMRST | O | CU, CD | 4, 8, 12, 16mA | 0 |
| LSCE_B0 | 0 | GPIO46 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | LSCE_B0 | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 2 | EINT20 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 3 | CMCSD0 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 4 | CLKO4 | O | CU, CD | 4, 8, 12, 16mA | 0 |
| LSCK0 | 0 | GPIO47 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | LSCK0 | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 3 | CMPDN | O | CU, CD | 4, 8, 12, 16mA | 0 |
| LSDA0 | 0 | GPIO48 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | LSDA0 | IO | - | 4, 8, 12, 16mA | 0 |
|  | 2 | EINT21 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 3 | CMCSD1 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 4 | WIFITOBT | I | CU, CD | 4, 8, 12, 16mA | 0 |
| LSA0 | 0 | GPIO49 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | LSA0DA0 | O | - | 4, 8, 12, 16mA | 0 |
|  | 2 | LSCE1_B0 | O | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 3 | CMMCLK | O | CU, CD | 4, 8, 12, 16mA | 0 |
| LPTE | 0 | GPIO50 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | LPTE | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 2 | EINT22 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 3 | CMCSK | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 4 | CMCSD2 | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 6 | MCINS | I | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 9 | CLKO5 | O | CU, CD | 4, 8, 12, 16mA | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Name | Aux. function | Aux. name | Aux. type | PU/PD/ CU/CD | Driving | SMT |
| --- | --- | --- | --- | --- | --- | --- |
| RESETB | 0 | GPIO51 | IO | CU, CD | 4, 8, 12, 16mA | 0 |
|  | 1 | RESETB | IO | CU, CD | 4, 8, 12, 16mA | 0 |
| EINT | 0 | AGPI52 | I | CU, CD | 8mA | 0 |
|  | 2 | EINT23 | I | CU, CD | 8mA | 0 |
| SRCLKENAI | 0 | AGPI53 | I | CU, CD | 8mA | 0 |
|  | 1 | SRCLKENAI | I | CU, CD | 8mA | 0 |
|  | 2 | EINT24 | I | - | 8mA | 0 |
| GPIO_10 | 0 | AGPIO54 | IO | CU, CD | 8mA | 0 |
| GPIO_11 | 0 | AGPIO55 | IO | CU, CD | 8mA | 0 |

### Electrical Characteristics

##### Absolute Maximum Ratings

Table 8. Absolute maximum ratings for power supply

| Symbol or pin name | Description | Min. | Max. | Unit |
| --- | --- | --- | --- | --- |
| VBAT_DIGITAL | Digital used battery voltage input | -0.3 | +4.4 | V |
| VBAT_VA | Analog used battery voltage input | -0.3 | +4.4 | V |
| AVDD_SPK | VBAT input for loud speaker driver | -0.3 | +5.5 | V |
| VSBST | Boost used battery voltage input | -0.3 | +4.4 | V |
| VDDK | 1.3v core power | -0.3 | +1.43 | V |

Table 9. Absolute maximum ratings for voltage input

| Symbol or pin name | Description | Min. | Max. | Unit |
| --- | --- | --- | --- | --- |
| VIN1 | Digital input voltage for IO Type 1 | -0.3 | 3.08 | V |
| VIN2 | Digital input voltage for IO Type 2 | -0.3 | 3.08 | V |
| VIN3 | Digital input voltage for IO Type 3 | -0.3 | 3.63 | V |
| VIN4 | Digital input voltage for IO Type 4 | -0.3 | 3.08 | V |
| VIN5 | Digital input voltage for IO Type 5 | -0.3 | 3.08 | V |
| VIN6 | Digital input voltage for IO Type 6 | -0.3 | 3.08 | V |
| VIN7 | Digital input voltage for IO Type 7 | -0.3 | 3.63 | V |

Table 10. Absolute maximum ratings for storage temperature

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Symbol or pin name | Description | Min. | Max. | Unit |
| --- | --- | --- | --- | --- |
| Tstg | Storage temperature | -55 | 125 | oC |

##### Recommended Operating Conditions

Table 11. Recommended operating conditions for power supply

| Symbol or pin name | Description | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- |
| VBAT_DIGITAL | Digital used battery voltage input | 3.4 | 3.8 | 4.2 | V |
| VBAT_VA | Analog used battery voltage input | 3.4 | 3.8 | 4.2 | V |
| AVDD_SPK | VBAT input for loud speaker driver | 3.4 | 3.8 | 4.2 | V |
| VSBST | Boost used battery voltage input | 3.4 | 3.8 | 4.2 | V |
| VDDK | 1.2v core power | 1.17 | 1.3 | 1.43 | V |

Table 12. Recommended operating conditions for voltage input

| Symbol or pin name | Description | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- |
| VIN1 | Digital input voltage for IO Type 1 | -0.3 | - | DVDIO+0.3 | V |
| VIN2 | Digital input voltage for IO Type 2 | -0.3 | - | DVDIO+0.3 | V |
| VIN3 | Digital input voltage for IO Type 3 | -0.3 | - | DVDIO+0.3 | V |
| VIN4 | Digital input voltage for IO Type 4 | -0.3 | - | DVDIO+0.3 | V |
| VIN5 | Digital input voltage for IO Type 5 | -0.3 | - | DVDIO+0.3 | V |
| VIN6 | Digital input voltage for IO Type 6 | -0.3 | - | DVDIO+0.3 | V |
| VIN7 | Digital input voltage for IO Type 7 | -0.3 | - | DVDIO+0.3 | V |

Table 13. Recommended operating conditions for operating temperature

| Symbol or pin name | Description | Min. | Typ | Max. | Unit |
| --- | --- | --- | --- | --- | --- |
| Tc | Operating temperature | -20 | - | 85 | oC |

##### Electrical Characteristics under Recommended Operating Conditions

Table 14. Electrical characteristics

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| DIIH1 | Digital high input current for IO Type 1 | PU/PD disabled, DVDIO = 2.8V, 2.1 < VIN1 < 3.1 | -5 | - | 5 | μA |
|  |  | PU enabled, DVDIO = 2.8V, 2.1 < VIN1 < 3.1 | -22.5 | - | 12.5 |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | PD enabled, DVDIO = 2.8V, 2.1<VIN1<3.1 | 6.1 | - | 82.5 |  |
| DIIL1 | Digital low input current for IO Type 1 | PU/PD disabled, DVDIO = 2.8V, -0.3 < VIN1 < 0.7 | -5 | - | 5 | μA |
|  |  | PU enabled, DVDIO = 2.8V, -0.3 < VIN1 < 0.7 | -82.5 | - | -6.1 |  |
|  |  | PD enabled, DVDIO = 2.8V, -0.3 < VIN1 < 0.7 | -12.5 | - | 22.5 |  |
| DIOH1 | Digital high output current for IO Type 1 | DVOH > 2.38V, DVDIO = 2.8V | -16 | - | - | mA |
| DIOL1 | Digital low output current for IO Type 1 | DVOL < 0.42V, DVDIO = 2.8V | - | - | 16 | mA |
| DRPU1 | Digital I/O pull-up resistance for IO Type 1 | DVDIO = 2.8V | 40 | 85 | 190 | kΩ |
| DRPD1 | Digital I/O pull-down resistance for IO Type 1 | DVDIO = 2.8V | 40 | 85 | 190 | kΩ |
| DVOH1 | Digital output high voltage for IO Type 1 | DVDIO = 2.8V | 2.38 |  |  | V |
| DVOL1 | Digital output low voltage for IO Type 1 | DVDIO = 2.8V |  |  | 0.42 | V |
| DIIH2 | Digital high input current for IO Type 2 | PU/PD disabled, DVDIO = 2.8V, 2.1 < VIN1 < 3.1 | -5 | - | 5 | μA |
|  |  | PU enabled, DVDIO = 2.8V, 2.1 < VIN1 < 3.1 | -22.5 | - | 12.5 |  |
|  |  | PD enabled, DVDIO = 2.8V, 2.1<VIN1<3.1 | 6.1 | - | 82.5 |  |
|  |  | PU/PD disabled, DVDIO = 1.8V, 1.35 < VIN1 < 2.1 | -5 | - | 5 | μA |
|  |  | PU enabled, DVDIO = 1.8V, 1.35 < VIN1 < 2.1 | -11.4 | - | 9.3 |  |
|  |  | PD enabled, DVDIO = 1.8V, 1.35 < VIN1 < 2.1 | -0.8 | - | 35 |  |
| DIIL2 | Digital low input current for IO Type 2 | PU/PD disabled, DVDIO = 2.8V, -0.3 < VIN2 < 0.7 | -5 | - | 5 | μA |
|  |  | PU enabled, DVDIO = 2.8V, -0.3 < VIN2 < 0.7 | -82.5 | - | -6.1 |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | PD enabled, DVDIO = 2.8V, -0.3 < VIN2 < 0.7 | -12.5 | - | 22.5 |  |
|  |  | PU/PD disabled, DVDIO = 1.8V, -0.3 < VIN1 < 0.45 | -5 | - | 5 | μA |
|  |  | PU enabled, DVDIO = 1.8V, -0.3 < VIN1 < 0.45 | -35 | - | 0.8 |  |
|  |  | PD enabled, DVDIO = 1.8V, -0.3 < VIN1 < 0.45 | -9.3 | - | 11.4 |  |
| DIOH2 | Digital high output current for IO Type 2 | DVOH > 2.38V, DVDIO = 2.8V | -16 | - | - | mA |
|  |  | DVOH > 1.53V, DVDIO = 1.8V | -12 | - | - | mA |
| DIOL2 | Digital low output current for IO Type 2 | DVOL < 0.42V, DVDIO = 2.8V | - | - | 16 | mA |
|  |  | DVOL < 0.27V, DVDIO = 1.8V | - | - | 12 | mA |
| DRPU2 | Digital I/O pull-up resistance for IO Type 2 | DVDIO = 2.8V | 40 | 85 | 190 | kΩ |
|  |  | DVDIO = 1.8V | 70 | 150 | 320 | kΩ |
| DRPD2 | Digital I/O pull-down resistance for IO Type 2 | DVDIO = 2.8V | 40 | 85 | 190 | kΩ |
|  |  | DVDIO = 1.8V | 70 | 150 | 320 | kΩ |
| DVOH2 | Digital output high voltage for IO Type 2 | DVDIO = 2.8V | 2.38 |  |  | V |
|  |  | DVDIO = 1.8V | 1.53 |  |  | V |
| DVOL2 | Digital output low voltage for IO Type 2 | DVDIO = 2.8V |  |  | 0.42 | V |
|  |  | DVDIO = 1.8V |  |  | 0.27 | V |
| DIIH3 | Digital high input current for IO Type 3 | PU/PD disabled, DVDIO = 2.8V, 2.1 < VIN3 < 3.1 | -5 | - | 5 | μA |
|  |  | PU enabled, DVDIO = 2.8V, 2.1 < VIN3 < 3.1 | -22.5 | - | 12.5 |  |
|  |  | PD enabled, DVDIO = 2.8V, 2.1 < VIN3 < 3.1 | 6.1 | - | 82.5 |  |
|  |  | PU/PD disabled, DVDIO = 1.8V, 1.35 < VIN3 < 2.1 | -5 | - | 5 | μA |
|  |  | PU enabled, DVDIO = 1.8V, 1.35 < VIN3 < 2.1 | -11.4 | - | 9.3 |  |
|  |  | PD enabled, DVDIO = 1.8V, 1.35 < VIN3 < 2.1 | -0.8 | - | 35 |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| DIIL3 | Digital low input current for IO Type 3 | PU/PD disabled, DVDIO = 2.8V, -0.3 < VIN3 < 0.7 | -5 | - | 5 | μA |
|  |  | PU enabled, DVDIO = 2.8V, -0.3 < VIN3 < 0.7 | -82.5 | - | -6.1 |  |
|  |  | PD enabled, DVDIO = 2.8V, -0.3 < VIN3 < 0.7 | -12.5 | - | 22.5 |  |
|  |  | PU/PD disabled, DVDIO = 1.8V, -0.3 < VIN3 < 0.45 | -5 | - | 5 | μA |
|  |  | PU enabled, DVDIO = 1.8V, -0.3 < VIN3 < 0.45 | -35 | - | 0.8 |  |
|  |  | PD enabled, DVDIO = 1.8V, -0.3 < VIN3 < 0.45 | -9.3 | - | 11.4 |  |
| DIOH3 | Digital high output current for IO Type 3 | DVOH > 2.38V, DVDIO = 2.8V | -16 | - | - | mA |
|  |  | DVOH > 1.53V, DVDIO = 1.8V | -12 | - | - | mA |
| DIOL3 | Digital low output current for IO Type 3 | DVOL < 0.42V, DVDIO = 2.8V | - | - | 16 | mA |
|  |  | DVOL < 0.27V, DVDIO = 1.8V | - | - | 12 | mA |
| DRPU3 | Digital I/O pull-up resistance for IO Type 3 | DVDIO = 2.8V | 10 | 47 | 100 | kΩ |
|  |  | DVDIO = 1.8V | 10 | 47 | 100 | kΩ |
| DRPD3 | Digital I/O pull-down resistance for IO Type 3 | DVDIO = 1.8V | 10 | 47 | 100 | kΩ |
|  |  | DVDIO = 2.8V | 10 | 47 | 100 | kΩ |
| DVOH3 | Digital output high voltage for IO Type 3 | DVDIO = 2.8V | 2.38 |  |  | V |
|  |  | DVDIO = 1.8V | 1.53 |  |  | V |
| DVOL3 | Digital output low voltage for IO Type 3 | DVDIO = 2.8V |  |  | 0.42 | V |
|  |  | DVDIO = 1.8V |  |  | 0.27 | V |
| DIIH4 | Digital high input current for IO Type 4 | PU/PD disabled, DVDIO = 2.8V, 2.1 < VIN4 < 3.1 | -5 | - | 5 | μA |
|  |  | PU enabled, DVDIO = 2.8V, 2.1 < VIN4 < 3.1 | -22.5 | - | 12.5 |  |
|  |  | PD enabled, DVDIO = 2.8V, 2.1 < VIN4 < 3.1 | 6.1 | - | 82.5 |  |
| DIIL4 | Digital low input current for IO Type 4 | PU/PD disabled, DVDIO = 2.8V, -0.3 < VIN4 < 0.7 | -5 | - | 5 | μA |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | PU enabled, DVDIO = 2.8V, -0.3 < VIN4 < 0.7 | -82.5 | - | -6.1 |  |
|  |  | PD enabled, DVDIO = 2.8V, -0.3 < VIN4 < 0.7 | -12.5 | - | 22.5 |  |
| DIOH4 | Digital high output current for IO Type 4 | DVOH > 2.38V, DVDIO = 2.8V | -16 | - | - | mA |
| DIOL4 | Digital low output current for IO Type 4 | DVOL < 0.42V, DVDIO = 2.8V | - | - | 16 | mA |
| DRPU4 | Digital I/O pull-up resistance for IO Type 4 (GPIO mode) | DVDIO = 2.8V | 15 | 36 | 55 | kΩ |
| DRPD4 | Digital I/O pull-down resistance for IO Type 4 (GPIO mode) | DVDIO = 2.8V | 15 | 36 | 55 | kΩ |
| DRPU4 1200K | Digital I/O pull-up resistance for IO Type 4 (Key PAD mode) | DVDIO = 2.8V | 1200 | - | - | kΩ |
| DRPD4 1200K | Digital I/O pull-down resistance for IO Type 4 (Key PAD mode) | DVDIO = 2.8V | 1200 | - | - | kΩ |
| DVOH4 | Digital output high voltage for IO Type 4 | DVDIO = 2.8V | 2.38 |  |  | V |
| DVOL4 | Digital output low voltage for IO Type 4 | DVDIO = 2.8V |  |  | 0.42 | V |
| DIIH5 | Digital high input current for IO Type 5 | PU/PD disabled, DVDIO = 2.8V, 2.1 < VIN5 < 3.1 | -5 | - | 5 | μA |
|  |  | PU enabled, DVDIO = 2.8V, 2.1 < VIN5 < 3.1 | -22.5 | - | 12.5 |  |
|  |  | PD enabled, DVDIO = 2.8V, 2.1 < VIN5 < 3.1 | 6.1 | - | 82.5 |  |
| DIIL5 | Digital low input current for IO Type 5 | PU/PD disabled, DVDIO = 2.8V, -0.3 < VIN5 < 0.7 | -5 | - | 5 | μA |
|  |  | PU enabled, DVDIO = 2.8V, -0.3 < VIN5 < 0.7 | -82.5 | - | -6.1 |  |
|  |  | PD enabled, DVDIO = 2.8V, -0.3 < VIN5 < 0.7 | -12.5 | - | 22.5 |  |
| DIOH5 | Digital high output current for IO Type 5 | DVOH > 2.38V, DVDIO = 2.8V | -16 | - | - | mA |
| DIOL5 | Digital low output current for IO Type 5 | DVOL < 0.42V, DVDIO = 2.8V | - | - | 16 | mA |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| DRPU5 | Digital I/O pull-up resistance for IO Type 5 (GPIO mode) | DVDIO = 2.8V | 15 | 36 | 55 | kΩ |
| DRPD5 | Digital I/O pull-down resistance for IO Type 5 (GPIO mode) | DVDIO = 2.8V | 15 | 36 | 55 | kΩ |
| DRPU5 1K | Digital I/O pull-up resistance for IO Type 4 (Key PAD mode) | DVDIO = 2.8V | 1 | - | - | kΩ |
| DRPD5 1K | Digital I/O pull-down resistance for IO Type 4 (Key PAD mode) | DVDIO = 2.8V | 1 | - | - | kΩ |
| DVOH5 | Digital output high voltage for IO Type 5 | DVDIO = 2.8V | 2.38 |  |  | V |
| DVOL5 | Digital output low voltage for IO Type 5 | DVDIO = 2.8V |  |  | 0.42 | V |
| DIIH6 | Digital high input current for IO Type 6 | PU/PD disabled, DVDIO = 2.8V, 2.1 < VIN6 < 3.1 | -5 | - | 5 | μA |
|  |  | PU enabled, DVDIO = 2.8V, 2.1 < VIN6 < 3.1 | -22.5 | - | 12.5 |  |
|  |  | PD enabled, DVDIO = 2.8V, 2.1 < VIN6 < 3.1 | 6.1 | - | 82.5 |  |
|  |  | PU/PD disabled, DVDIO = 1.8V, 1.35 < VIN6 < 2.1 | -5 | - | 5 | μA |
|  |  | PU enabled, DVDIO = 1.8V, 1.35 < VIN6 < 2.1 | -11.4 | - | 9.3 |  |
|  |  | PD enabled, DVDIO = 1.8V, 1.35 < VIN6 < 2.1 | -0.8 | - | 35 |  |
| DIIL6 | Digital low input current for IO Type 6 | PU/PD disabled, DVDIO = 2.8V, -0.3 < VIN6 < 0.7 | -5 | - | 5 | μA |
|  |  | PU enabled, DVDIO = 2.8V, -0.3 < VIN6 < 0.7 | -82.5 | - | -6.1 |  |
|  |  | PD enabled, DVDIO = 2.8V, -0.3 < VIN6 < 0.7 | -12.5 | - | 22.5 |  |
|  |  | PU/PD disabled, DVDIO = 1.8V, -0.3 < VIN6 < 0.45 | -5 | - | 5 | μA |
|  |  | PU enabled, DVDIO = 1.8V, -0.3 < VIN6 < 0.45 | -35 | - | 0.8 |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | PD enabled, DVDIO = 1.8V, -0.3 < VIN6 < 0.45 | -9.3 | - | 11.4 |  |
| DIOH6 | Digital high output current for IO Type 6 | DVOH > 2.38V, DVDIO = 2.8V | -8 | - | - | mA |
|  |  | DVOH > 1.53V, DVDIO = 1.8V | -6 | - | - | mA |
| DIOL6 | Digital low output current for IO Type 6 | DVOL < 0.42V, DVDIO = 2.8V | - | - | 8 | mA |
|  |  | DVOL < 0.27V, DVDIO = 1.8V | - | - | 6 | mA |
| DRPU6 | Digital I/O pull-up resistance for IO Type 6 | DVDIO = 2.8V | 40 | 85 | 190 | kΩ |
|  |  | DVDIO = 1.8V | 70 | 150 | 320 | kΩ |
| DRPD6 | Digital I/O pull-down resistance for IO Type 6 | DVDIO = 2.8V | 40 | 85 | 190 | kΩ |
|  |  | DVDIO = 1.8V | 70 | 150 | 320 | kΩ |
| DVOH6 | Digital output high voltage for IO Type 6 | DVDIO = 2.8V | 2.38 |  |  | V |
|  |  | DVDIO = 1.8V | 1.53 |  |  | V |
| DVOL6 | Digital output low voltage for IO Type 6 | DVDIO = 2.8V |  |  | 0.42 | V |
|  |  | DVDIO = 1.8V |  |  | 0.27 | V |
| DIIH7 | Digital high input current for IO Type 7 | PU/PD disabled, DVDIO = 2.8V, 2.1 < VIN7 < 3.1 | -5 | - | 5 | μA |
|  |  | PU enabled, DVDIO = 2.8V, 2.1 < VIN7 < 3.1 | -22.5 | - | 12.5 |  |
|  |  | PD enabled, DVDIO = 2.8V, 2.1<VIN7<3.1 | 6.1 | - | 82.5 |  |
|  |  | PU/PD disabled, DVDIO = 1.8V, 1.35 < VIN7 < 2.1 | -5 | - | 5 | μA |
|  |  | PU enabled, DVDIO = 1.8V, 1.35 < VIN7 < 2.1 | -11.4 | - | 9.3 |  |
|  |  | PD enabled, DVDIO = 1.8V, 1.35 < VIN7 < 2.1 | -0.8 | - | 35 |  |
| DIIL7 | Digital low input current for IO Type 7 | PU/PD disabled, DVDIO = 2.8V, -0.3 < VIN7 < 0.7 | -5 | - | 5 | μA |
|  |  | PU enabled, DVDIO = 2.8V, -0.3 < VIN7 < 0.7 | -82.5 | - | -6.1 |  |
|  |  | PD enabled, DVDIO = 2.8V, -0.3 < VIN7 < 0.7 | -12.5 | - | 22.5 |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | PU/PD disabled, DVDIO = 1.8V, -0.3 < VIN7 < 0.45 | -5 | - | 5 | μA |
|  |  | PU enabled, DVDIO = 1.8V, -0.3 < VIN7 < 0.45 | -35 | - | 0.8 |  |
|  |  | PD enabled, DVDIO = 1.8V, -0.3 < VIN7 < 0.45 | -5 | - | 5 |  |
| DIOH7 | Digital high output current for IO Type 7 | DVOH > 2.38V, DVDIO = 2.8V | -16 | - | - | mA |
|  |  | DVOH > 1.53V, DVDIO = 1.8V | -12 | - | - | mA |
| DIOL7 | Digital low output current for IO Type 7 | DVOL < 0.42V, DVDIO = 2.8V | - | - | 16 | mA |
|  |  | DVOL < 0.27V, DVDIO = 1.8V | - | - | 12 | mA |
| DRPU7 | Digital I/O pull-up resistance for IO Type 7 | DVDIO = 2.8V | 40 | 85 | 190 | kΩ |
|  |  | DVDIO = 1.8V | 70 | 150 | 320 | kΩ |
| DRPD7 | Digital I/O pull-down resistance for IO Type 7 | DVDIO = 2.8V | 40 | 85 | 190 | kΩ |
|  |  | DVDIO = 1.8V | 70 | 150 | 320 | kΩ |
| DVOH7 | Digital output high voltage for IO Type 7 | DVDIO = 2.8V | 2.38 |  |  | V |
|  |  | DVDIO = 1.8V | 1.53 |  |  | V |
| DVOL7 | Digital output low voltage for IO Type 7 | DVDIO = 2.8V |  |  | 0.42 | V |
|  |  | DVDIO = 1.8V |  |  | 0.27 | V |

### System Configuration

##### Strapping Resistors

Table 15. Strapping table

| Pin name | Description | Trapping condition |
| --- | --- | --- |
| LSA0 | Pull-up with 10K resister(Default internal pull-down with 47K resister) | Power-on reset |
| BPI_BUS1 | Pull-up with 10K resister (Default internal pull-down with 75K resister) | Power-on reset |
| BPI_BUS2 | Pull-up with 10K resister (Default internal pull-down with 75K resister) | Power-on reset |

##### Mode Selection

Table 16. Mode selection of chip

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Pin name | Description |
| --- | --- |
| EXT_CLK_SEL | GND: Uses DCXO as 26M clock source VRF: Uses external clock as 26M clock source |
| LSA0 | GND: Uses 1.8V serial flash device DVDD18_EMI: Uses 3.3V serial flash device |
| KCOL0 | GND: Boots ROM to enter USB download mode DVDD28: Normal boot-up mode |
| {BPI_BUS1,BPI_BU S2} | {GND, GND}: No JTAG {GND, DVDD28}: JTAG at keypad pins {DVDD28, GND}: JTAG at GPIO pins {DVDD28, DVDD28}: JTAG at camera pins |

### Power-on Sequence and Protection Logic

MT6261D provides 32K crystal removal feature. The XOSC32_ENB state tells if MT6261D provides this feature or not. VRF will be turned on at the same time with VRTC when XOSC32_ENB = 1. The power-on/off sequence controlled by “Control” and “Reset Generator” is shown as the figure below.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

<=100ms

VBAT

DDLO

XOSC32

_ENB=0

VRTC

De	ce

tim	ms

1.7ms

1.7ms

1.7ms

1.7ms

1.7mms

1.7ms

1.7ms

1.7ms

200ms

-boun e = 50

UVLO

PWRKEY

PWRBB

VCORE

VIO18

VIO28

VSF

VA

VUSB

VRF

RESETB

Figure 5. Power-on/off control sequence by pressing PWRKEY and XOSC32_ENB = 0

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

<=100ms

VBAT

DDLO

XOSC32

_ENB=1

VRTC/ VRF

UVLO

De	ce

tim	ms

1.7ms

1.7ms

1.7ms

1.7ms

1.7mms

1.7ms

1.7ms

1.7ms

200ms

-boun e = 50

PWRKEY

PWRBB

VCORE

VIO18

VIO28

VSF

VA

VUSB

RESETB

Figure 6. Power-on/off control sequence by pressing PWRKEY and XOSC32_ENB = 1

Note that each of the above figures only shows one power-on/off condition when XOSC32_ENB = 0 or XOSC32_ENB = 1. MT6261D handles the power-on and off of the handset. The following three methods can switch on the handset (when leaving UVLO): XOSC32_ENB = 0

- Push PWRKEY (Pull the PWRKEY pin to the low level.)

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Pulling PWRKEY low is the typical way to turn on the handset. The turn-on sequence is VCORE  VIO18  VIO28  VSF  VA  VUSB  VRF.

The supplies for the baseband are ready, and the system reset ends at the moment when the above LDOs are fully turned on to ensure correct timing and function. After that, the baseband will send the PWRBB signal back to the PMU for acknowledgement. To successfully power on the handset, PWRKEY should be kept low until PMU receives PWRBB from the baseband.

- RTC module generates PWRBB to wake up the system.

If the RTC module is scheduled to wake up the handset at a certain time, the PWRBB signal will be directly sent to the PMU. In this case, PWRBB will become high at specific moment and allow the PMU to be powered on as the sequence described above. This is called the RTC alarm.

- Valid charger plug-in (CHRIN voltage is within the valid range.)

The charger plug-in will also turn on the handset if the charger is valid (no OVP takes place). However, if the battery voltage is too low to power on the handset (UVLO state), the system will not be turned on by any of the three methods. In this case, the charger will charge the battery first and the handset will be powered on automatically as long as the battery voltage is high enough.

Under-voltage lockout (UVLO)

The UVLO state in the PMU prevents startup if the initial voltage of the main battery is below the 3.2V threshold. It ensures that the handset is powered on with the battery in good condition. The UVLO function is performed by a hysteretic comparator which ensures a smooth power-on sequence. In addition, when the battery voltage is getting lower, it will enter the UVLO state, and the PMU will be turned off by itself, except for VRTC LDO, to prevent further discharging. Once the PMU enters the UVLO state, it will draw low quiescent current. The RTC LDO will still be working until the DDLO disables it.

Deep discharge lockout (DDLO)

The PMU will enter the deep discharge lockout (DDLO) state when the battery voltage drops below

  - V. In this state, the VRTC LDO will be shut down. Otherwise, it will draw very low quiescent current to prevent further discharging or damage to the cells.

Reset

The PMU contains a reset control circuit which takes effect at both power-up and power-down. The RESETB pin is held low in the beginning of power-up and returns to high after the pre-determined delay time. The delay time is controlled by a large counter which uses the clock from internal

ring-oscillator. At power-off, the RESETB pin will return to low immediately without any delay.

Over-temperature protection

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

If the die temperature of PMU exceeds 150°C, the PMU will automatically disable all the LDOs except for VRTC. Once the over-temperature state is resolved, a new power-on sequence will be required to enable the LDOs.

### Analog Baseband

To communicate with analog blocks, a common control interface for all analog blocks is implemented. In addition, there are some dedicated interfaces for data transfer. The common control interface translates the APB bus write and read cycle for specific addresses related to analog front-end control. During the writing or reading of any of these control registers, there is a latency associated with the transfer of data to or from the analog front-end. Dedicated data interface of each analog block is implemented in the corresponding digital block. An analog block includes the following analog functions for the complete GSM/GPRS baseband signal processing:

- RF control: DAC for automatic power control (APC) is included, and its output is provided to external RF power amplifier respectively.

- Auxiliary ADC: Provides an ADC for the battery and other auxiliary analog functions monitoring

- Audio mixed-signal block: Provides complete analog voice signal processing including microphone amplification, A/D conversion, D/A conversion, earphone driver, etc. Dedicated stereo D/A conversion and amplification for audio signals are also included.

- Clock generation: Includes a clock squarer for shaping the system clock, and PLL providing clock signals to DSP, MCU and USB unit

##### APC-DAC

###### Block Description

APC-DAC is a 10-bit DAC with output buffer aiming at automatic power control. See the tables below for its analog pin assignment and functional specifications. It is an event-driven scheme for power saving purpose.

###### Functional Specifications

Table 17. APC-DAC specifications

| Symbol | Parameter | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- |
| N | Resolution |  | 10 |  | Bit |
| FS | Sampling rate |  |  | 1.0833 | MSPS |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Symbol | Parameter | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- |
| SINAD | Signal to noise and distortion ratio (10-kHz sine with 1.0V swing & 100-kHz BW) | 47 |  |  | dB |
|  | 99% settling time (full swing on maximal capacitance) |  |  | 5 | μS |
|  | Output swing | 0 |  | AVDD | V |
|  | Output capacitance |  | 200 | 2200 | pF |
|  | Output resistance | 0.47 | 10 |  | KΩ |
| DNL | Differential nonlinearity for code 20 to 970 |  | ± 1 |  | LSB |
| INL | Integral nonlinearity for code 20 to 970 |  | ± 1 |  | LSB |
| DVDD | Digital power supply | 1.1 | 1.2 | 1.3 | V |
| AVDD | Analog power supply | 2.6 | 2.8 | 3.0 | V |
| T | Operating temperature | -20 |  | 80 | °C |
|  | Current consumption Power-up Power-down |  | 400 1 |  | μA μA |

##### Auxiliary ADC

###### Block Description

The auxiliary ADC includes the following functional blocks:

- Analog multiplexer: Selects signal from one of the seven auxiliary input pins. Real-world messages to be monitored, e.g. temperature, should be transferred to the voltage domain.

- 10-bit A/D converter: Converts the multiplexed input signal to 10-bit digital data.

| Channel | Application | Input range [V] |
| --- | --- | --- |
| 0 | BATSNS | 3.2 ~ 4.2 |
| 1 | ISENSE | 3.2 ~ 4.2 |
| 2 | VCDT | Decided by application circuit |
| 3 | BATON | 0 ~ AVDD28 |
| 4 | AUXIN4 | 0 ~ AVDD28 |
| others | Internal use | N/A |

###### Functional Specifications

The functional specifications of the auxiliary ADC are listed in the following table.

Table 18. Functional specifications of auxiliary ADC

| Symbol | Parameter | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Symbol | Parameter | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- |
| N | Resolution |  | 10 |  | Bit |
| FC | Clock rate |  | 1.08 |  | MHz |
| FS | Sampling rate @ N-Bit |  | 1.08/(N+1) |  | MSPS |
|  | Input swing | 0 |  | AVDD | V |
| CIN | Input capacitance Unselected channel Selected channel |  |  | 50 4 | fF pF |
| RIN | Input resistance Unselected channel Selected channel | 400 1 |  |  | MΩ MΩ |
|  | Clock latency |  | N+1 |  | 1/FC |
| DNL | Differential nonlinearity |  | ± 1 |  | LSB |
| INL | Integral nonlinearity |  | ± 1 |  | LSB |
| OE | Offset error |  | ± 10 |  | mV |
| FSE | Full swing error |  | ± 10 |  | mV |
| SINAD | Signal to noise and distortion ratio (10-kHz full swing input & 1.0833-MHz clock rate) |  | 50 |  | dB |
| DVDD | Digital power supply | 1.1 | 1.2 | 1.3 | V |
| AVDD | Analog power supply | 2.6 | 2.8 | 3.0 | V |
| T | Operating temperature | -20 |  | 80 | °C |
|  | Current consumption Power-up Power-down |  | 280 1 |  | μA μA |

##### Audio Mixed-Signal Blocks

###### Block Description

Audio mixed-signal blocks (AMB) integrate complete voice uplink/downlink and audio playback functions. As shown in the figure below, it includes three parts. The first consists of stereo audio DACs and audio amplifiers for audio playback. The second part is the voice downlink path, including

voice-band DACs (left channel audio DAC) and voice amplifier, which produces voice signals to earphones or other auxiliary output devices. Moreover, a ClassK amplifier is embedded to support continuous >1W output power with an on-chip boost. The last part is the voice uplink path, which is the interface between the microphone (or other auxiliary input device) input and MT6261D DSP. A set of bias voltage is provided for the external electric microphone.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

PGA	Class-K	AU_SPKP

AU_SPKN

Audio Amp-R

Voice Signal Stereo or Mono Audio Signal

AU_HPR

Audio Amp-L

Voice Signal Stereo or Mono Audio Signal

Audio LCHDAC

AU_HPL

AU_HSP

AU_HSN

Voice Amp

AU_VIN1_P

Voice Signal

PGA

AU_VIN1_N

AU_VIN0_P

AU_VIN0_N

Audio RCHDAC

Voice ADC

M U X

Figure 7. Block diagram of audio mixed-signal blocks

###### Functional Specifications

See the table below for the functional specifications of voice-band uplink/downlink blocks.

Table 19. Functional specifications of analog voice blocks

| Symbol | Parameter | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- |
| FS | Sampling rate |  | 6,500 |  | kHz |
| DVDD | Digital power supply | 1.1 | 1.2 | 1.3 | V |
| AVDD | Analog power supply | 2.6 | 2.8 | 3.0 | V |
| T | Operating temperature | -20 |  | 80 | °C |
| VMIC | Microphone biasing voltage |  | 1.9 | 2.2 | V |
| IMIC | Current draw from microphone bias pins |  |  | 2 | mA |
| Uplink path4 |  |  |  |  |  |
| IDC | Current consumption for one channel |  | 1.5 |  | mA |
| SINAD | Signal to noise and distortion ratio Input level: -40 dbm0 Input level: 0 dbm0 | 29 | 69 |  | dB dB |
| RIN | Input impedance (differential) | 13 | 20 | 27 | KΩ |

4 For uplink-path, not all gain settings of VUPG meet the specifications listed in the table, especially for several the lowest gains. The minimum gain that meets the specifications is to be determined.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Symbol | Parameter | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- |
| ICN | Idle channel noise |  |  | -67 | dBm0 |
| Downlink path |  |  |  |  |  |
| IDC | Current consumption |  | 4 |  | mA |
| SINAD | Signal to noise and distortion ratio Input level: -40 dBm0 Input level: 0 dBm0 | 29 | 69 |  | dB dB |
| RLOAD | Output resistor load (differential) | 16 | 32 |  | Ω |
| CLOAD | Output capacitor load |  |  | 250 | pF |
| ICN | Idle channel noise of transmit path |  |  | -64 | dBPa |
| XT | Crosstalk level on transmit path |  |  | -66 | dBm0 |

See the table below for the functional specifications of audio blocks.

Table 20. Functional specifications of analog audio blocks

| Symbol | Parameter | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- |
| FCK | Clock frequency |  | 6.5 |  | MHz |
| Fs | Sampling rate | 32 | 44.1 | 48 | kHz |
| AVDD | Power supply | 2.6 | 2.8 | 3.0 | V |
| T | Operating temperature | -20 |  | 80 | °C |
| IDC | Current consumption |  | 4 |  | mA |
| PSNR | Peak signal to noise ratio |  | 88 |  | dB |
| DR | Dynamic range |  | 88 |  | dB |
| VOUT | Output swing for 0dBFS input level @ -1dB headphone gain |  | 0.707 |  | Vrms |
| VOUTMAX | Maximum output swing |  | 2.0 |  | Vpp |
| THD | Total harmonic distortion 10mW at 64Ω load |  |  | -70 | dB |
| RLOAD | Output resistor load (single-ended) | 64 |  |  | Ω |
| CLOAD | Output capacitor load |  |  | 250 | pF |
| XT | L-R channel cross talk | 70 |  |  | dB |

### Power Management Unit Blocks

The power management unit (PMU) manages the power supply of the entire chip, such as baseband, processor, memory, SIM cards, camera, vibrator, etc. The digital part of PMU is integrated into the analog part (see the figure below). PMU includes the following analog functions for signal processing:

- LDO: Regulates battery voltage to lower voltage level

- BOOST: Boosts battery voltage to target voltage for Class-AB audio amplifier

- Keypad LED driver (KPLED) and current sink (ISINK) switches: Sink current for keypad LED and LCM module

- Start-up (STRUP): Generates power-on/off control sequence of start-up circuits

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Pulse charger (PCHR): Controls battery charging

| 1 | 2 | 3 |
| --- | --- | --- |
| 4 | 5 | 6 |
| 7 | 8 | 9 |
|  | 0 | # |

2G RF Transceiver (on chip)

Blue Tooth (on chip)

Camera sensor

(AF)

SDIO Device

SIM1

Memory card

LCM

Module

Backup Battery

2G Transceiver LDOs

VRF

Pulse-Charger Controller

Extra LDOs Vibrator

Reset Generator

Control

KP LED Driver (Open Drain)

Charger In

BJT

+Rsense

VIBR

VCORE

VRTC VA VIO18

VIO28

Base Band Processor

VSF

Serial Flash

VMC

VSIM1

VSIM2

SIM2

VUSB

M

Current Sink

MT6261D PMU

VRF

SIM

Level Shifter

VCAMA

Basic Feature

LDOs VUSB VCAMA

BOOST

Class-AB Audio Amplifier

Essential LDOs VCORE VRTC

VA VIO18 VIO28 VMC VSIM1 VSIM2 VSF

Figure 8. PMU system block diagram

##### LDO

PMU integrates 13 general low dropout regulators (LDO) optimized for their given functions by balancing the quiescent current, dropout voltage, line/load regulation, ripple rejection and output noise.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

(max 4.3V)

CHR CTRL

V33 LIMT

(2.5~3.3V) AVDD33/XVDD33/DVDD33

BGR	PNP

BIAS	BGR

BGR OSC

DDLO

UVLO

CHRLDO DET

STARTUP

BATSNS

LDO	IGEN

V12	VGEN

STARTU P

LDO-Group2

VBAT2

VIO28

VCAMA

VRF

VIO18

VRTC

VSIM2

7.5k

VIBR

BATSNS

CHRIN

PCHR

LDO-Group1

VBAT1

VSIM1

VMC

PWR_SEL

VCORE

VA

VIO18

VUSB

AVDD28_CHRLDO

C=1uF

CHR LDO28

CHR	1.2V LDO12

Class AB

VBAT_BOOST	and BOOST

BOOST	BOOST

Cotroller	Driver

VBOOST

ClassAB	ClassAB

Cotroller	Driver

Backlight and KPLED

ISINK, KPLED

controller

PWRKEY

CHR ANACKT

Figure 9. Power domain

###### LDO

A low-dropout regulator (LDO) is capable of maintaining its specified output voltage over a wide range of load current and input voltage, down to a very small difference between input and output voltages.

There are several features in the design of LDO, including discharge control, soft start and current limit. Before LDO is enabled, the output pin of LDO should be discharged first to avoid voltage accumulation on the capacitance. The soft-start limits inrush current and controls output-voltage rising time during the power-up. The current limit is the current protection to limit the LDO’s output current and power dissipation.

There are three types of LDOs in PMU of MT6261D PMU. The analog LDO is optimized for

low-frequency ripple rejection in order to reject the ripples coming from the burst of RF power amplifier. The digital IO LDO is a linear regulator optimized for very low quiescent current. The single-step RTC LDO is a linear regulator that can charge up a capacitor-type backup coin cell, which also supplies the RTC module even at the absence of the main battery. The single-step LDO features reverse current protection and is optimized for ultra-low quiescent current while sustaining the RTC function as long as possible.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Block Description

VREF

VOUT

R1

VOUT

R2

Discharge Control

AVDD43_VXX

VOUT

GND

Current Limit

Figure 10. LDO block diagram

        - LDO Types

Table 21. LDO types and brief specifications

| Type | LDO name | Vout (Volt) | Imax(mA) | Description |
| --- | --- | --- | --- | --- |
| ALDO | VRF | 2.8 | 150 | RF chip and 26MHz reference clock |
| ALDO | VA | 2.8 | 150 | Analog baseband |
| ALDO | VCAMA | 2.8 | 70 | Camera sensor |
| DLDO | VIO28 | 2.8 | 100 | Digital IO and Blue tooth |
| DLDO | VSIM1 | 1.8/3.0 | 30 | SIM card |
| DLDO | VSIM2 | 1.8/3.0 | 30 | SIM card |
| DLDO | VUSB | 3.3 | 50 | USB |
| DLDO | VIO18 | 1.8 | 100 | Digital IO |
| DLDO | VCORE | 0.75~1.35 | 150 | Digital baseband |
| DLDO | VIBR | 1.8/2.8/3.0 | 100 | Vibrator |
| DLDO | VMC | 1.8/2.8/3.0/3.3 | 100 | Memory card |
| DLDO | VSF | 1.86/2.8/3.0/3.3 | 50 | Serial flash |
| RTCLDO | VRTC | 2.8/3.3 | 2 | Real-time clock |

        - Functional Specifications

Table 22. Analog LDO specifications

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  | Load capacitor |  |  | 1 |  | μF |
|  | Current limit |  | 1.2*Imax |  | 5*Imax | mA |
|  | Vout | Includes load regulation, line regulation, and temperature coefficient | Max. (-5%, -0.1V) |  | Max. (+5%, +0.1V) | V |
|  | Transient response | Slew: 15mA/us | Max. (-5%, -0.1V) |  | Max. (+5%, +0.1V) | V |
|  | Temperature coefficient |  |  |  | 100 | ppm/C |
|  | PSRR | Iout < 0.5*Imax 10 < f < 3 kHz | 65 |  |  | dB |
|  |  | Iout < 0.5*Imax 3K < f< 30 kHz | 45 |  |  | dB |
|  | Output noise | With A-weighted filter |  |  | 90 | uVrms |
|  | Quiescent current | Iout = 0 |  | 55 |  | μA |
|  | Turn-on overshoot | Iout = 0 |  |  | Max. (+10%, +0.1V) | V |
|  | Turn-on settling time | Iout = 0 |  |  | 240 | μsec |

Table 23. Digital LDO specifications

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  | Load capacitor |  |  | 15 |  | μF |
|  | Current limit |  | 1.2*Imax |  | 5*Imax | mA |
|  | Vout | Includes load regulation, line regulation, and temperature coefficient | Max. (-5%, -0.1V) |  | Max. (+5%, +0.1V) | V |
|  | Transient response | Slew: 15mA/us | Max. (-5%, -0.1V) |  | Max. (+5%, +0.1V) | V |
|  | Temperature coefficient |  |  |  | 100 | ppm/C |
|  | Quiescent current | Iout = 0 |  | 30 |  | μA |
|  | Turn-on overshoot | Iout = 0 |  |  | Max. (+10%, +0.1V) | V |
|  | Turn-on settling | Iout = 0 |  |  | 240 | μs |

5 VCORE loading capacitor typical value is 2.2uF. Other LDOs are 1uF.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  | time |  |  |  |  |  |

Table 24. RTC LDO specification

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  | Load capacitor |  |  | 1 |  | μF |
|  | Vout | Includes load regulation, line regulation, and temperature coefficient | 2 | 2.8 | 3 | V |
|  | Temperature coefficient |  |  |  | 100 | ppm/C |
|  | Quiescent current | Iout = 0 |  | 15 |  | μA |

##### BOOST

###### Functional Specifications

Table 25. RTC LDO specification.

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  | Cin Cout |  |  | 2.2uF 4.7uF |  | μF |
|  | L | Rdcr,max<80mOhm |  | 0.68 |  | uH |
|  | Vout |  |  | 5.3 |  | V |
|  | Ripple | Vin=3.4V/3.8V/4.2V, Cin=2.2uF & Cout=4.7uF, L= 0.68uH (Rdcr,max<80mOhm) 650mA , switching Freq 2MHz |  |  | 100 | mV |
|  | Switching frequency |  |  | 2 |  | MHz |
|  | Quiescent current | Iout = 0 |  | 4 | 6 | mA |

##### ISINK and KPLED Switches

One built-in open-drain output switch drives the keypad LED (KPLED) in the handset. The switch is controlled by the baseband with enabling registers. The switch of keypad LED can sink as much as 60mA current, and the output is high impedance when disabled. The value of the sink current decides the brightness of the LED.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

The current controlled open drain drivers	are also implemented to drive the LCM backlight module, and	provides currentfrom 4mA to 96mA.

###### Block Description

AVSS43_DRV

AVDD43

ISINK

NI_ISINKS_CH_EN RG_ISINKS_CH0_STEP[2:0]

ISINK_EN

Step[2:0]

PAD_KPLED

NI_KPLED_EN

A2D signals

NI_ISINK0_STATUS NI_ISINK1_STATUS NI_ISINK2_STATUS NI_ISINK3_STATUS NI_KPLED_STATUS

KPLED_EN

Figure 11. ISINKs and KPLED switches bock diagram.

###### Functional Specifications

Table 26. ISINKs and KPLED Switches Specification.

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  | Sink current of keypad LED driver | Von > 0.5V, 100% dimming duty | 60 |  |  | mA |
|  | 1 ch Sink current of ISINK without current double option | Von > 0.15V, 100% dimming duty, ISINKS_CHx_STEP = 000 |  | 4 |  | mA |
|  | 1 ch Sink current of ISINK without current double option | Von > 0.15V, 100% dimming duty, ISINKS_CHx_STEP = 001 |  | 8 |  | mA |
|  | 1 ch Sink current of ISINK without current double option | Von > 0.15V, 100% dimming duty, ISINKS_CHx_STEP = 010 |  | 12 |  | mA |
|  | 1 ch Sink current of ISINK without current | Von > 0.15V, 100% dimming duty, ISINKS_CHx_STEP = |  | 16 |  | mA |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  | double option | 011 |  |  |  |  |
|  | 1 ch Sink current of ISINK without current double option | Von > 0.15V, 100% dimming duty, ISINKS_CHx_STEP = 100 |  | 20 |  | mA |
|  | 1 ch Sink current of ISINK without current double option | Von > 0.15V, 100% dimming duty, ISINKS_CHx_STEP = 101 |  | 24 |  | mA |
|  | Current mismatch | Von > 0.15V, 100% dimming duty | -5 |  | 5 | % |

##### STRUP

PMU handles the power-on and off of the handset. If the battery voltage is neither in the UVLO state (VBAT ≥ 3.4V) nor in the thermal condition, there are three methods to power on the handset system: pulling PWRKEY low (the user pushes PWRKEY), pulling PWRBB high (baseband BB_WakeUp) or valid charger plug-in.

According to different battery voltage (VBAT) and phone states, control signals and regulators will have different responses.

##### PCHR

The charger controller senses the charger input voltage from either a standard AC-DC adaptor or an USB charger. When the charger input voltage is within a pre-determined range, the charging process will be activated. This detector can resist higher input voltage than other parts of the PMU.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Block Description

Figure 12. PCHR block diagram.

        - Charger Detection

Whenever an invalid charging source is detected (> 7.0 V), the charger detector stops the charging process immediately to avoid burning out the chip or even the phone. In addition, if the charger-in level is not high enough (< 4.3V), the charger will also be disabled to avoid improper charging behavior.

        - Charging Control

When the charger is active, the charger controller manages the charging phase according to the battery status. During the charging period, the battery voltage is constantly monitored. The battery charger supports pre-charge mode (VBAT < 3.2V, PMU power-off state), CC mode (constant current mode or fast charging mode at the range 3.2V < VBAT < 4.2V) and CV mode (constant voltage mode) to optimize the charging procedure for Li-ion battery. The charging states diagram is shown in the figure below.

NON- CHARGING

Dead Battery VBAT<2.2V

5 min timeout

NO

YES

CHRIN>4. 3V CHRIN DET

YES

VBAT>3.3V

YES

CC Mode

NO

Charger OFF

YES

VBAT< 3 .3V

35 min timeout

NO

VBAT>4.1V	NO

PreCC

NO

NO

CV Mode

FULL

YES

VBAT= 4.2V

Reduce ICHG by step

YES

ICHG <ITERM

YES

NO

Charger OFF

YES

VBAT<4. 05V

Re- Charge

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Figure 13. Charging states diagram

Pre-charge mode

When the battery voltage is in the UVLO state, the charger operates in the pre-charge mode. There are two steps in this mode. When the battery voltage is deeply discharged below 2.2V, PRECC1 trickle charging current will be applied to the battery.

The PRECC1 trickle charging current is about 550ms pulse 70mA current when VBAT is under 2.2V.

When the battery voltage exceeds 2.2V, called the PRECC2 stage, the closed-loop pre-charge is enabled. The voltage drop across the external RSENSE is kept around 40mV (AC charger) or 14mV (USB host). The closed-loop pre-charge current can be calculated:

IPRECC2, AC adapter

 VSENSE 

Rsense

40mV

Rsense

IPRECC2,USBHOST

 VSENSE 

Rsense

14mV

Rsense

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Constant current mode

As the battery is charged up and over 3.4V, it can switch to the CC mode. (CHR_EN should be high) In the CC mode, several charging currents can be set by programming registers or the external RSENSE resistor. The charging current can be determined by CS_VTH/RSENSE, where CS_VTH is programmed by registers. For example, if RSENSE is selected as 0.2ohm, the CC mode charging current can be set from 70 to 800mA. It can accommodate the battery charger to various charger inputs with different current capabilities.

Constant voltage mode and over-voltage protection (OV)

While the battery voltage reaches about 4.2V, a constant voltage is used for charging. This is called the full-voltage charging mode or constant-voltage charging mode in correspondence to a linear charger.

While the battery voltage actually reaches 4.2V, the charging current is gradually decreased

step-by-step, the end-of-charging process starts. It may prolong the charging and detecting period for acquiring optimized full charging volume. The charging process is completed once the current reaches zero automatically and this mechanism is optimized for different battery

BC1.1 Dead-Battery Support of China Standard

MT6261D supports dead-battery condition from China standard (called BC1.1). The specification protects dead-battery charging by timer and trickle current. Once the battery voltage is below 2.2V, a period (TUNIT) of trickle current (IUNIT) will be applied to the battery.

If the battery voltage is still below 2.2V after applying trickle current, the charger will be disabled. On the other hand, if the battery voltage is raised to above 2.2V, the charger will enter the PRECC2 stage, and the charging current will be 70mA or 200mA depending on the type of charging port.

Under the condition of battery voltage from 2.2V to 3.3V, the charger will charge the battery with the PRECC2 current.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A dedicated 5mins (T1) timer will be timed out and disable the charger if the battery voltage is always below 2.7V under charging. Another 35mins (T2) timer will also be timed out and disable the charger if the battery voltage is always kept between 2.7V and 3.3V under charging.

The trickle current (IUNIT) and two dedicated timers protect the charging action if the battery is dead.

###### Functional Specifications

Table 27. Charger detection specifications

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  | Charger detect-on range |  | 4.3 |  | 7 | V |

Table 28. Pre-charge specifications

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  | IUNIT with 500ms pulse | VBAT < 2.2V | 20 | 48 | 100 | mA |
|  | Pre-charging current | VBAT < 2.2V (500ms pulse) | 20 | 48 | 100 | mA |
|  |  | VBAT ≥ 2.2V (USB host) | 7/Rsense | 14/Rsense | 20/Rsense | mA |
|  |  | VBAT ≥ 2.2V (AC adapter < 7V) | 30/Rsense | 40/Rsense | 50/Rsense | mA |
|  |  | VBAT ≥ 2.2V (AC adapter > 7V) | 7/Rsense | 14/Rsense | 20/Rsense | mA |
|  | Pre-charging off threshold | CHR_EN = L |  | 3.3 |  | V |
|  | Pre-charging off hysteresis |  |  | 0.4 |  | V |

Table 29. Constant current specifications

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  | CC mode charging current ( CS_VTH ) | CS_VTH [3:0] = 0000 |  | 320/Rsense |  | mA |
|  |  | CS_VTH [3:0] = 0001 |  | 300/Rsense |  | mA |
|  |  | CS_VTH [3:0] = 0010 |  | 280/Rsense |  | mA |
|  |  | CS_VTH [3:0] = 0011 |  | 260/Rsense |  | mA |
|  |  | CS_VTH [3:0] = 0100 |  | 240/Rsense |  | mA |
|  |  | CS_VTH [3:0] = 0101 |  | 220/Rsense |  | mA |
|  |  | CS_VTH [3:0] = 0110 |  | 200/Rsense |  | mA |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | CS_VTH [3:0] = 0111 |  | 180/Rsense |  | mA |
|  |  | CS_VTH [3:0] = 1000 |  | 160/Rsense |  |  |
|  |  | CS_VTH [3:0] = 1001 |  | 140/Rsense |  |  |
|  |  | CS_VTH [3:0] = 1010 |  | 130/Rsense |  |  |
|  |  | CS_VTH [3:0] = 1011 |  | 110/Rsense |  |  |
|  |  | CS_VTH [3:0] = 1100 |  | 90/Rsense |  |  |
|  |  | CS_VTH [3:0] = 1101 |  | 60/Rsense |  |  |
|  |  | CS_VTH [3:0] = 1110 |  | 40/Rsense |  |  |
|  |  | CS_VTH [3:0] = 1111 |  | 14/Rsense |  |  |
|  | Current sensing resistor | RSENSE |  | 0.2 |  | ohm |

Table 30. Constant voltage and over-voltage protection specifications

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  | Charging complete threshold |  | 4.15 | 4.2 | 4.25 | V |
|  | Battery over-voltage protection threshold (OV) |  |  | 4.3 |  | V |

Table 31. BC1.1 specifications

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| IUNIT | BC1.1 trickle current | VBAT < 2.2V |  | 50 | 100 | mA |
| IPRECC2 (USB host) | PRECC2 current | 2.2 < VBAT < 3.3V |  | 70 | 100 | mA |
| IPRECC2 (AC adapter) | PRECC2 current | 2.2 < VBAT < 3.3V |  | 200 | 250 | mA |
| T1 | 5 minute dedicated timer | 2.2 < VBAT < 2.7V |  | 5 | 6.5 | min. |
| T2 | 35 minute dedicated timer | 2.7 < VBAT < 3.3V |  | 35 | 38.5 | min. |
| TUNIT | BC1.1 trickle current period |  |  | 550 | 770 | ms. |

##### PMU Register Definitions

Module name: PMUSYS Base address: (+A0700000h)

| Address | Name | Width | Register Function |
| --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| A0700050 | APB_CON0 | 16 | APB Control Register 0 Set this register for APB configuration controls |
| --- | --- | --- | --- |
| A0700060 | ACCDET_CON0 | 16 | Accdet Control Register 0 Set this register for PMU accdet circuit configuration controls |
| A0700080 | AUX_CON0 | 16 | Auxadc Control Register 0 Set this register for PMU charger circuit configuration controls |
| A0700088 | AUX_CON2 | 16 | Auxadc Control Register 2 Set this register for PMU charger circuit configuration controls |
| A070008C | AUX_CON3 | 16 | Auxadc Control Register 3 Set this register for PMU charger circuit configuration controls |
| A0700090 | AUX_CON4 | 16 | Auxadc Control Register 4 Set this register for PMU charger circuit configuration controls |
| A0700094 | AUX_CON5 | 16 | Auxadc Control Register 5 Set this register for PMU charger circuit configuration controls |
| A0700098 | AUX_CON6 | 16 | Auxadc Control Register 6 Set this register for PMU charger circuit configuration controls |
| A07000C0 | LDO_CON0 | 16 | LDO Control Register 0 Set this register for PMU LDO circuit configuration controls |
| A0700100 | VRF_CON0 | 16 | VRF LDO Control Register 0 Set this register for PMU VRF LDO circuit configuration controls |
| A0700104 | VRF_CON1 | 16 | VRF LDO Control Register 1 Set this register for PMU VRF LDO circuit configuration controls |
| A0700120 | VA_CON0 | 16 | VA LDO Control Register 0 Set this register for PMU VA LDO circuit configuration controls |
| A0700124 | VA_CON1 | 16 | VA LDO Control Register 1 Set this register for PMU VA LDO circuit configuration controls |
| A0700130 | VCAMA_CON0 | 16 | VCAMA LDO Control Register 0 Set this register for PMU VCAMA LDO circuit configuration controls |
| A0700134 | VCAMA_CON1 | 16 | VCAMA LDO Control Register 1 Set this register for PMU VCAMA LDO circuit configuration controls |
| A0700150 | VIO28_CON0 | 16 | VIO28 LDO Control Register 0 Set this register for PMU VIO28 LDO circuit configuration controls |
| A0700154 | VIO28_CON1 | 16 | VIO28 LDO Control Register 1 Set this register for PMU VIO28 LDO circuit configuration controls |
| A0700160 | VUSB_CON0 | 16 | VUSB LDO Control Register 0 Set this register for PMU VUSB LDO circuit configuration controls |
| A0700164 | VUSB_CON1 | 16 | VUSB LDO Control Register 1 Set this register for PMU VUSB LDO circuit configuration controls |
| A0700180 | VSIM1_CON0 | 16 | VSIM1 LDO Control Register 0 Set this register for PMU VSIM1 LDO circuit configuration controls |
| A0700184 | VSIM1_CON1 | 16 | VSIM1 LDO Control Register 1 Set this register for PMU VSIM1 LDO circuit configuration controls |
| A0700188 | VSIM1_CON2 | 16 | VSIM1 LDO Control Register 2 Set this register for PMU VSIM1 LDO circuit configuration controls |
| A0700190 | VSIM2_CON0 | 16 | VSIM2 LDO Control Register 0 Set this register for PMU VSIM2 LDO circuit configuration controls |
| A0700194 | VSIM2_CON1 | 16 | VSIM2 LDO Control Register 1 Set this register for PMU VSIM2 LDO circuit configuration controls |
| A0700198 | VSIM2_CON2 | 16 | VSIM2 LDO Control Register 2 Set this register for PMU VSIM2 LDO circuit configuration controls |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| A07001A0 | VRTC_CON0 | 16 | VRTC LDO Control Register 0 Set this register for PMU VRTC LDO circuit configuration controls |
| --- | --- | --- | --- |
| A07001B0 | VIBR_CON0 | 16 | VIBR LDO Control Register 0 Set this register for PMU VIBR LDO circuit configuration controls |
| A07001B4 | VIBR_CON1 | 16 | VIBR LDO Control Register 1 Set this register for PMU VIBR LDO circuit configuration controls |
| A07001C0 | VMC_CON0 | 16 | VMC LDO Control Register 0 Set this register for PMU VMC LDO circuit configuration controls |
| A07001C4 | VMC_CON1 | 16 | VMC LDO Control Register 1 Set this register for PMU VMC LDO circuit configuration controls |
| A07001D0 | VSF_CON0 | 16 | VSF LDO Control Register 0 Set this register for PMU VSF LDO circuit configuration controls |
| A07001D4 | VSF_CON1 | 16 | VSF LDO Control Register 1 Set this register for PMU VSF LDO circuit configuration controls |
| A0700200 | VIO18_CON0 | 16 | VIO18 LDO Control Register 0 Set this register for PMU VIO18 LDO circuit configuration controls |
| A0700204 | VIO18_CON1 | 16 | VIO18 LDO Control Register 1 Set this register for PMU VIO18 LDO circuit configuration controls |
| A0700230 | VCORE_CON0 | 16 | VCORE LDO Control Register 0 Set this register for PMU VCORE LDO circuit configuration controls |
| A0700234 | VCORE_CON1 | 16 | VCORE LDO Control Register 1 Set this register for PMU VCORE LDO circuit configuration controls |
| A070023C | VCORE_CON3 | 16 | VCORE LDO Control Register 3 Set this register for PMU VCORE LDO circuit configuration controls |
| A0700A00 | CHR_CON0 | 16 | Charger Control Register 0 Set this register for PMU charger circuit configuration controls |
| A0700A04 | CHR_CON1 | 16 | Charger Control Register 1 Set this register for PMU charger circuit configuration controls |
| A0700A08 | CHR_CON2 | 16 | Charger Control Register 2 Set this register for PMU charger circuit configuration controls |
| A0700A10 | CHR_CON4 | 16 | Charger Control Register 4 Set this register for PMU charger circuit configuration controls |
| A0700A14 | CHR_CON5 | 16 | Charger Control Register 5 Set this register for PMU charger circuit configuration controls |
| A0700A20 | CHR_CON8 | 16 | Charger Control Register 8 Set this register for PMU charger circuit configuration controls |
| A0700A24 | CHR_CON9 | 16 | Charger Control Register 9 Set this register for PMU charger circuit configuration controls |
| A0700A28 | CHR_CON10 | 16 | Charger Control Register 10 Set this register for PMU charger circuit configuration controls |
| A0700A30 | CHR_CON12 | 16 | Charger Control Register 12 Set this register for PMU charger circuit configuration controls |
| A0700A38 | CHR_CON14 | 16 | Charger Control Register 14 Set this register for PMU charger circuit configuration controls |
| A0700C00 | ISINKS_CON0 | 16 | ISINKS Control Register 0 Set this register for PMU ISINKS circuit configuration controls |
| A0700C10 | ISINK0_CON0 | 16 | ISINK0 Control Register 0 Set this register for PMU ISINK0 circuit configuration controls |
| A0700C20 | ISINK1_CON0 | 16 | ISINK1 Control Register 0 Set this register for PMU ISINK1 circuit configuration controls |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| A0700C80 | KPLED_CON0 | 16 | KPLED Control Register 0 Set this register for PMU KPLED circuit configuration controls |
| --- | --- | --- | --- |
| A0700D00 | SPK_CON0 | 16 | SPK Control Register 0 Set this register for PMU SPK circuit configuration controls |
| A0700D0C | SPK_CON3 | 16 | SPK Control Register 3 Set this register for PMU SPK circuit configuration controls |
| A0700D1C | SPK_CON7 | 16 | SPK Control Register 7 Set this register for PMU SPK circuit configuration controls |
| A0700D20 | SPK_CON8 | 16 | SPK Control Register 8 Set this register for PMU SPK circuit configuration controls |
| A0700D28 | VSBST_CON0 | 16 | Boost Control Register 0 |
| A0700D30 | VSBST_CON2 | 16 | Boost Control Register 2 Set this register for PMU boost circuit configuration controls |
| A0700D34 | VSBST_CON3 | 16 | Boost Control Register 3 Set this register for PMU boost circuit configuration controls |
| A0700D38 | VSBST_CON4 | 16 | Boost Control Register 4 Set this register for PMU boost circuit configuration controls |
| A0700D40 | VSBST_CON6 | 16 | Boost Control Register 6 Set this register for PMU boost circuit configuration controls |
| A0700E20 | OC_CON8 | 16 | Over-Current Control Register 8 Set this register for PMU over-current circuit configuration controls |
| A0700E24 | OC_CON9 | 16 | Over-Current Control Register 9 Set this register for PMU over-current circuit configuration controls |
| A0700E2C | OC_CON11 | 16 | Over-Current Control Register 11 Set this register for PMU over-current circuit configuration controls |
| A0700F04 | TEST_CON1 | 16 | PMU TEST Control Register 1 Set this register for PMU TEST circuit configuration controls |

###### LDO

A07000C0	LDO_CON0	LDO Control Register 0	0801

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CCI_S RCLK EN |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 0 | CCI_SRCLK EN | CCI_SRCLKEN | Sleep-mode control in A_FUNC_MODE. Please program it to 0 after power-up when chip is not in A_FUNC_MODE to enable hardware sleep-mode control, otherwise it will be forced as normal mode. 0: Analog sleep-mode is controlled by digital hardware sleep-mode control 1: Force analog not go to sleep-mode |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A0700100	VRF_CON0	VRF LDO Control Register 0	8403

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | QI_VR F_ST ATUS |  |  |  |  | RG_V RF_N DIS_E N |  |  |  |  |  |  |  |  | VRF_ ON_S EL | RG_V RF_E N |
| Type | RO |  |  |  |  | RW |  |  |  |  |  |  |  |  | RW | RW |
| Reset | 1 |  |  |  |  | 1 |  |  |  |  |  |  |  |  | 1 | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | QI_VRF_ST ATUS | QI_VRF_STATUS | LDO on/off status 0: off 1: on |
| 10 | RG_VRF_N DIS_EN | RG_VRF_NDIS_EN | Vrf output power down Enable 1'b0: disable output powerdown 1'b1: enable output powerdown |
| 1 | VRF_ON_SE L | VRF_ON_SEL | Selects VRF LDO enable control path 0: Enable with EXT_SRCLKENA or BT_SRCLKENA or TOPSM_SRCLKENA 1: Enable with RG_VMC_EN |
| 0 | RG_VRF_EN | RG_VRF_EN | Vrf Enable 1'b0: disable 1'b1: enable |

A0700104	VRF_CON1	VRF LDO Control Register 1	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | RG_VRF_CAL |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7:4 | RG_VRF_C AL | RG_VRF_CAL | VRF Voltage Calibration (4'b0000: 0 mV) 4'b0000: 0 mV 4'b0001: +25 mV 4'b0010: +50 mV 4'b0011: +75 mV 4'b0100: +100 mV 4'b0101: +125 mV 4'b0110: +150 mV 4'b0111: +175 mV 4'b1000: -200 mV 4'b1001: -175 mV 4'b1010: -150 mV 4'b1011: -125 mV 4'b1100: -100 mV 4'b1101: -75 mV 4'b1110: -50 mV 4'b1111: -25 mV |

A0700120	VA_CON0	VA LDO Control Register 0	8401

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | QI_VA _STA |  |  |  |  | RG_V A_NDI |  |  |  |  |  |  |  |  |  | RG_V A_EN |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

|  | TUS |  |  |  |  | S_EN |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type | RO |  |  |  |  | RW |  |  |  |  |  |  |  |  |  | RW |
| Reset | 1 |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | QI_VA_STA TUS | QI_VA_STATUS | LDO on/off status 0: off 1: on |
| 10 | RG_VA_NDI S_EN | RG_VA_NDIS_EN | VA output power down Enable 1'b0: disable output powerdown 1'b1: enable output powerdown (analog LDO) |
| 0 | RG_VA_EN | RG_VA_EN | VA enable for testing 1'b0: disable 1'b1: enable(analog LDO) |

A0700124	VA_CON1	VA LDO Control Register 1	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | RG_VA_CAL |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7:4 | RG_VA_CA L | RG_VA_CAL | VA Voltage Calibration (4'b0000: 0 mV) 4'b0000: 0 mV 4'b0001: +25 mV 4'b0010: +50 mV 4'b0011: +75 mV 4'b0100: +100 mV 4'b0101: +125 mV 4'b0110: +150 mV 4'b0111: +175 mV 4'b1000: -200 mV 4'b1001: -175 mV 4'b1010: -150 mV 4'b1011: -125 mV 4'b1100: -100 mV 4'b1101: -75 mV 4'b1110: -50 mV 4'b1111: -25 mV |

A0700130	VCAMA_CON0 VCAMA LDO Control Register 0	0412

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | QI_VC AMA_ STAT US |  |  |  |  | RG_V CAMA _NDIS _EN |  |  |  |  |  |  |  |  | VCAM A_ON _SEL | RG_V CAMA _EN |
| Type | RO |  |  |  |  | RW |  |  |  |  |  |  |  |  | RW | RW |
| Reset | 0 |  |  |  |  | 1 |  |  |  |  |  |  |  |  | 1 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | QI_VCAMA_ STATUS | QI_VCAMA_STATU S | LDO on/off status 0: off |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: on |
| 10 | RG_VCAMA _NDIS_EN | RG_VCAMA_NDIS _EN | Vcama output power down Enable 1'b0: disable output powerdown 1'b1: enable output powerdown |
| 1 | VCAMA_ON _SEL | VCAMA_ON_SEL | Selects VCAMA LDO enable control path 0: Enable with EXT_SRCLKENA or BT_SRCLKENA or TOPSM_SRCLKENA 1: Enable with RG_VCAMA_EN |
| 0 | RG_VCAMA _EN | RG_VCAMA_EN | Vcama Enable 1'b0: disable 1'b1: enable |

A0700134	VCAMA_CON1 VCAMA LDO Control Register 1	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | RG_VCAMA_CAL |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7:4 | RG_VCAMA _CAL | RG_VCAMA_CAL | VCAMA Voltage Calibration (4'b0000: 0 mV) 4'b0000: 0 mV 4'b0001: +25 mV 4'b0010: +50 mV 4'b0011: +75 mV 4'b0100: +100 mV 4'b0101: +125 mV 4'b0110: +150 mV 4'b0111: +175 mV 4'b1000: -200 mV 4'b1001: -175 mV 4'b1010: -150 mV 4'b1011: -125 mV 4'b1100: -100 mV 4'b1101: -75 mV 4'b1110: -50 mV 4'b1111: -25 mV |

A0700150	VIO28_CON0	VIO28 LDO Control Register 0	8401

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | QI_VI O28_S TATU S |  |  |  |  | RG_VI O28_N DIS_E N |  |  |  |  |  |  |  |  |  | RG_VI O28_E N |
| Type | RO |  |  |  |  | RW |  |  |  |  |  |  |  |  |  | RW |
| Reset | 1 |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | QI_VIO28_S TATUS | QI_VIO28_STATUS | LDO on/off status 0: off 1: on |
| 10 | RG_VIO28_ | RG_VIO28_NDIS_ | VIO28 POWER DOWN NMOS Enable |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  | NDIS_EN | EN | 1'b0: disable 1'b1: enable |
| 0 | RG_VIO28_ EN | RG_VIO28_EN | VIO28 enable for testing 1'b0: disable 1'b1: enable(analog LDO) |

A0700154	VIO28_CON1	VIO28 LDO Control Register 1	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | RG_VIO28_CAL |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7:4 | RG_VIO28_ CAL | RG_VIO28_CAL | VIO28 Voltage Calibration 4'b0000: 0 mV 4'b0001: +20 mV 4'b0010: +40 mV 4'b0011: +60 mV 4'b0100: +80 mV 4'b0101: +100 mV 4'b0110: +120 mV 4'b0111: +140 mV 4'b1000: -160 mV 4'b1001: -140 mV 4'b1010: -120 mV 4'b1011: -100 mV 4'b1100: -80 mV 4'b1101: -60 mV 4'b1110: -40 mV 4'b1111: -20 mV |

A0700160	VUSB_CON0	VUSB LDO Control Register 0	8401

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | QI_VU SB_S TATU S |  |  |  |  | RG_V USB_ NDIS_ EN |  |  |  |  |  |  |  |  |  | RG_V USB_ EN |
| Type | RO |  |  |  |  | RW |  |  |  |  |  |  |  |  |  | RW |
| Reset | 1 |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | QI_VUSB_S TATUS | QI_VUSB_STATUS | LDO on/off status 0: off 1: on |
| 10 | RG_VUSB_ NDIS_EN | RG_VUSB_NDIS_E N | VUSB POWER DOWN NMOS Enable 1'b0: disable 1'b1: enable |
| 0 | RG_VUSB_ EN | RG_VUSB_EN | Vusb enable for testing 1'b0: disable 1'b1: enable |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A0700164	VUSB_CON1	VUSB LDO Control Register 1	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | RG_VUSB_CAL |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7:4 | RG_VUSB_ CAL | RG_VUSB_CAL | VUSB Voltage Calibration 4'b0000: 0 mV 4'b0001: +20 mV 4'b0010: +40 mV 4'b0011: +60 mV 4'b0100: +80 mV 4'b0101: +100 mV 4'b0110: +120 mV 4'b0111: +140 mV 4'b1000: -160 mV 4'b1001: -140 mV 4'b1010: -120 mV 4'b1011: -100 mV 4'b1100: -80 mV 4'b1101: -60 mV 4'b1110: -40 mV 4'b1111: -20 mV |

A0700050	APB_CON0	APB Control Register 0	2200

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  | CCI_C 2A_SI M2_V OSEL |  |  |  | CCI_C 2A_SI M1_V OSEL |  |  |  |  |  |  |  |  |
| Type |  |  |  | RW |  |  |  | RW |  |  |  |  |  |  |  |  |
| Reset |  |  |  | 0 |  |  |  | 0 |  |  |  |  |  |  |  |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 12 | CCI_C2A_SI M2_VOSEL | CCI_C2A_SIM2_V OSEL | VSIM2 voltage selection. If SIM2 is enable by SIMLS in normal mode, then this bit can select VSIM2 voltage. 1'b0: 1.8V (VSIM2_VOSEL = 3'b010) 1'b1: 3.0V (VSIM2_VOSEL = 3'b101) NOTE: this bit works at VSIM2_GPLDO_EN (@VSIM2_CON2) = 0 |
| 8 | CCI_C2A_SI M1_VOSEL | CCI_C2A_SIM1_V OSEL | VSIM1 voltage selection. If SIM1 is enable by SIMLS in normal mode, then this bit can select VSIM1 voltage. 1'b0: 1.8V (VSIM1_VOSEL = 1'b0) 1'b1: 3.0V (VSIM1_VOSEL = 1'b1) NOTE: this bit works at VSIM_GPLDO_EN (@VSIM_CON2) = 0 |

A0700180	VSIM1_CON0	VSIM1 LDO Control Register 0	0400

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | QI_VS IM1_S TATU S |  |  |  |  | RG_V SIM1_ NDIS_ EN |  |  |  |  |  | RG_V SIM1_ VOSE L |  |  |  | RG_V SIM1_ EN |
| Type | RO |  |  |  |  | RW |  |  |  |  |  | RW |  |  |  | RW |
| Reset | 0 |  |  |  |  | 1 |  |  |  |  |  | 0 |  |  |  | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | QI_VSIM1_S TATUS | QI_VSIM1_STATU S | LDO on/off status 0: off 1: on |
| 10 | RG_VSIM1_ NDIS_EN | RG_VSIM1_NDIS_ EN | VSIM1 POWER DOWN NMOS Enable 1'b0: disable 1'b1: enable |
| 4 | RG_VSIM1_ VOSEL | RG_VSIM1_VOSEL | VSIM1 Output Voltage Selection Signal 1'b0: 1.8 1'b1: 3.0 |
| 0 | RG_VSIM1_ EN | RG_VSIM1_EN | VSIM1 Enable 1'b0: disable 1'b1: enable |

NOTE: RG_VSIM1_VOSEL works at VSIM1_GPLDO_EN (@VSIM1_CON2) = 1 or a_func_mode or acd_mode

A0700184	VSIM1_CON1	VSIM1 LDO Control Register 1	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | RG_VSIM1_CAL |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7:4 | RG_VSIM1_ CAL | RG_VSIM1_CAL | VSIM1 Voltage Calibration 4'b0000: 0 mV 4'b0001: +20 mV 4'b0010: +40 mV 4'b0011: +60 mV 4'b0100: +80 mV 4'b0101: +100 mV 4'b0110: +120 mV 4'b0111: +140 mV 4'b1000: -160 mV 4'b1001: -140 mV 4'b1010: -120 mV 4'b1011: -100 mV 4'b1100: -80 mV 4'b1101: -60 mV 4'b1110: -40 mV 4'b1111: -20 mV |

A0700188	VSIM1_CON2	VSIM1 LDO Control Register 2	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  | VSIM1 _GPL DO_E N |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 1 | VSIM1_GPL DO_EN | VSIM1_GPLDO_EN | The control selection of SIM1 enable & voltage, which is controlled by SIM1 controller circuit or SIM1_CON0 0: SIM1 is controlled by SIM1 controller 1: SIM1 is controlled by VSIM1_CON0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | NOTE: voltage selection register of SIM1 controller is CCI_C2A_SIM1_VOSEL (@APB_CON0) |

A0700190	VSIM2_CON0	VSIM2 LDO Control Register 0	0400

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | QI_VS IM2_S TATU S |  |  |  |  | RG_V SIM2_ NDIS_ EN |  |  |  |  |  | RG_V SIM2_ VOSE L |  |  |  | RG_V SIM2_ EN |
| Type | RO |  |  |  |  | RW |  |  |  |  |  | RW |  |  |  | RW |
| Reset | 0 |  |  |  |  | 1 |  |  |  |  |  | 0 |  |  |  | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | QI_VSIM2_S TATUS | QI_VSIM2_STATU S | LDO on/off status 0: off 1: on |
| 10 | RG_VSIM2_ NDIS_EN | RG_VSIM2_NDIS_ EN | VSIM2 POWER DOWN NMOS Enable 1'b0: disable 1'b1: enable |
| 4 | RG_VSIM2_ VOSEL | RG_VSIM2_VOSEL | VSIM2 Output Voltage Selection Signal 1'b0: 1.8V 1'b1: 3.0V |
| 0 | RG_VSIM2_ EN | RG_VSIM2_EN | VSIM2 Enable 1'b0: disable 1'b1: enable |

NOTE: RG_VSIM2_VOSEL works at VSIM2_GPLDO_EN (@VSIM2_CON2) = 1 or a_func_mode or acd_mode

A0700194	VSIM2_CON1	VSIM2 LDO Control Register 1	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | RG_VSIM2_CAL |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7:4 | RG_VSIM2_ CAL | RG_VSIM2_CAL | VSIM2 Voltage Calibration 4'b0000: 0 mV 4'b0001: +20 mV 4'b0010: +40 mV 4'b0011: +60 mV 4'b0100: +80 mV 4'b0101: +100 mV 4'b0110: +120 mV 4'b0111: +140 mV 4'b1000: -160 mV 4'b1001: -140 mV 4'b1010: -120 mV 4'b1011: -100 mV 4'b1100: -80 mV 4'b1101: -60 mV 4'b1110: -40 mV 4'b1111: -20 mV |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A0700198	VSIM2_CON2	VSIM2 LDO Control Register 2	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  | VSIM2 _GPL DO_E N |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 1 | VSIM2_GPL DO_EN | VSIM2_GPLDO_EN | The control selection of SIM2 enable & voltage, which is controlled by SIM2 controller circuit or SIM2_CON0 0: SIM2 is controlled by SIM2 controller 1: SIM2 is controlled by VSIM2_CON0 NOTE: voltage selection register of SIM2 controller is CCI_C2A_SIM2_VOSEL (@APB_CON0) |

A07001A0	VRTC_CON0	VRTC LDO Control Register 0	0001

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  | RG_V RTC_ VOSE L |  |  |  | RG_V RTC_ EN |
| Type |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |  | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  | 0 |  |  |  | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 4 | RG_VRTC_V OSEL | RG_VRTC_VOSEL | VRTC Output Voltage Selection Signal 1'b0: 2.8V 1'b1: 3.3V |
| 0 | RG_VRTC_E N | RG_VRTC_EN | Vrtc enable for testing 1'b0: disable 1'b1: enable |

A07001B0	VIBR_CON0	VIBR LDO Control Register 0	0422

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | QI_VI BR_S TATU S |  |  |  |  | RG_VI BR_N DIS_E N |  |  |  |  | RG_VIBR_V OSEL |  |  |  | VIBR_ ON_S EL | RG_VI BR_E N |
| Type | RO |  |  |  |  | RW |  |  |  |  | RW |  |  |  | RW | RW |
| Reset | 0 |  |  |  |  | 1 |  |  |  |  | 1 | 0 |  |  | 1 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | QI_VIBR_ST ATUS | QI_VIBR_STATUS | LDO on/off status 0: off 1: on |
| 10 | RG_VIBR_N DIS_EN | RG_VIBR_NDIS_E N | VIBR POWER DOWN NMOS Enable 1'b0: disable 1'b1: enable |
| 5:4 | RG_VIBR_V OSEL | RG_VIBR_VOSEL | VIBR output selection signal 2'b00: 1.8 V |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 2'b01: no use 2'b10: 2.8 V 2'b11: 3.0 V |
| 1 | VIBR_ON_S EL | VIBR_ON_SEL | Selects VIBR LDO enable control path 0: Enable with EXT_SRCLKENA or BT_SRCLKENA or TOPSM_SRCLKENA 1: Enable with RG_VIBR_EN |
| 0 | RG_VIBR_E N | RG_VIBR_EN | VIBR Enable 1'b0: disable 1'b1: enable |

A07001B4	VIBR_CON1	VIBR LDO Control Register 1	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | RG_VIBR_CAL |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7:4 | RG_VIBR_C AL | RG_VIBR_CAL | VIBR Voltage Calibration 4'b0000: 0 mV 4'b0001: +20 mV 4'b0010: +40 mV 4'b0011: +60 mV 4'b0100: +80 mV 4'b0101: +100 mV 4'b0110: +120 mV 4'b0111: +140 mV 4'b1000: -160 mV 4'b1001: -140 mV 4'b1010: -120 mV 4'b1011: -100 mV 4'b1100: -80 mV 4'b1101: -60 mV 4'b1110: -40 mV 4'b1111: -20 mV |

A07001C0	VMC_CON0	VMC LDO Control Register 0	8442

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | QI_VM C_ST ATUS |  |  |  |  | RG_V MC_N DIS_E N |  |  |  | RG_VMC_VOSEL |  |  |  |  | VMC_ ON_S EL | RG_V MC_E N |
| Type | RO |  |  |  |  | RW |  |  |  | RW |  |  |  |  | RW | RW |
| Reset | 1 |  |  |  |  | 1 |  |  |  | 1 | 0 | 0 |  |  | 1 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | QI_VMC_ST ATUS | QI_VMC_STATUS | LDO on/off status 0: off 1: on |
| 10 | RG_VMC_N DIS_EN | RG_VMC_NDIS_E N | VMC POWER DOWN NMOS Enable 1'b0: disable 1'b1: enable |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 6:4 | RG_VMC_V OSEL | RG_VMC_VOSEL | VMC output selection signal 3'b000: 1.8V 3'b001: no use 3'b010: 2.8V 3'b011: 3.0V 3'b100: 3.3V 3'b101: no use 3'b110: no use 3'b111: no use |
| 1 | VMC_ON_S EL | VMC_ON_SEL | Selects VMC LDO enable control path 0: Enable with EXT_SRCLKENA or BT_SRCLKENA or TOPSM_SRCLKENA 1: Enable with RG_VMC_EN |
| 0 | RG_VMC_E N | RG_VMC_EN | VMC Enable 1'b0: disable 1'b1: enable |

A07001C4	VMC_CON1	VMC LDO Control Register 1	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | RG_VMC_CAL |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7:4 | RG_VMC_C AL | RG_VMC_CAL | VMC Voltage Calibration 4'b0000: 0 mV 4'b0001: +20 mV 4'b0010: +40 mV 4'b0011: +60 mV 4'b0100: +80 mV 4'b0101: +100 mV 4'b0110: +120 mV 4'b0111: +140 mV 4'b1000: -160 mV 4'b1001: -140 mV 4'b1010: -120 mV 4'b1011: -100 mV 4'b1100: -80 mV 4'b1101: -60 mV 4'b1110: -40 mV 4'b1111: -20 mV |

A07001D0	VSF_CON0	VSF LDO Control Register 0	8431

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | QI_VS F_ST ATUS |  |  |  |  | RG_V SF_N DIS_E N |  |  |  | RG_VSF_VOSEL |  |  |  |  |  | RG_V SF_E N |
| Type | RO |  |  |  |  | RW |  |  |  | RW |  |  |  |  |  | RW |
| Reset | 1 |  |  |  |  | 1 |  |  |  | 0 | 1 | 1 |  |  |  | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | QI_VSF_ST ATUS | QI_VSF_STATUS | LDO on/off status 0: off |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: on |
| 10 | RG_VSF_ND IS_EN | RG_VSF_NDIS_EN | VSF POWER DOWN NMOS Enable 1'b0: disable 1'b1: enable |
| 6:4 | RG_VSF_VO SEL | RG_VSF_VOSEL | VSF output selection signal 3'b000: no use 3'b001: 1.86V 3'b010: 2.8V 3'b011: 3.0V 3'b100: 3.3V 3'b101: no use 3'b110: no use 3'b111: no use |
| 0 | RG_VSF_EN | RG_VSF_EN | VSF enable for testing 1'b0: disable 1'b1: enable |

NOTE: RG_VSF_VOSEL works at VSF_VOSEL_SEL (@VSF_CON3) = 1

A07001D4	VSF_CON1	VSF LDO Control Register 1	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | RG_VSF_CAL |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7:4 | RG_VSF_CA L | RG_VSF_CAL | VSF Voltage Calibration 4'b0000: 0 mV 4'b0001: +20 mV 4'b0010: +40 mV 4'b0011: +60 mV 4'b0100: +80 mV 4'b0101: +100 mV 4'b0110: +120 mV 4'b0111: +140 mV 4'b1000: -160 mV 4'b1001: -140 mV 4'b1010: -120 mV 4'b1011: -100 mV 4'b1100: -80 mV 4'b1101: -60 mV 4'b1110: -40 mV 4'b1111: -20 mV |

A0700200	VIO18_CON0	VIO18 LDO Control Register 0	8401

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | QI_VI O18_S TATU S |  |  |  |  | RG_VI O18_N DIS_E N |  |  |  |  |  |  |  |  |  | RG_VI O18_E N |
| Type | RO |  |  |  |  | RW |  |  |  |  |  |  |  |  |  | RW |
| Reset | 1 |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | QI_VIO18_S TATUS | QI_VIO18_STATUS | LDO on/off status 0: off 1: on |
| 10 | RG_VIO18_ NDIS_EN | RG_VIO18_NDIS_ EN | VIO18 output power down Enable 1'b0: disable output powerdown 1'b1: enable output powerdown (analog LDO) |
| 0 | RG_VIO18_ EN | RG_VIO18_EN | VIO18 enable for testing 1'b0: disable 1'b1: enable(analog LDO) |

A0700204	VIO18_CON1	VIO18 LDO Control Register 1	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | RG_VIO18_CAL |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7:4 | RG_VIO18_ CAL | RG_VIO18_CAL | VIO18 Voltage Calibration 4'b0000: 0 mV 4'b0001: +20 mV 4'b0010: +40 mV 4'b0011: +60 mV 4'b0100: +80 mV 4'b0101: +100 mV 4'b0110: +120 mV 4'b0111: +140 mV 4'b1000: -160 mV 4'b1001: -140 mV 4'b1010: -120 mV 4'b1011: -100 mV 4'b1100: -80 mV 4'b1101: -60 mV 4'b1110: -40 mV 4'b1111: -20 mV |

A0700230	VCORE_CON0  VCORE LDO Control Register 0	8481

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | QI_VC ORE_ STAT US | VCOR E_OC _FLA G |  |  |  | RG_V CORE _NDIS _EN |  | RG_VCORE_VOSEL |  |  |  |  |  |  |  | RG_V CORE _EN |
| Type | RO | RO |  |  |  | RW |  | RW |  |  |  |  |  |  |  | RW |
| Reset | 1 | 0 |  |  |  | 1 |  | 0 | 1 | 0 | 0 | 0 |  |  |  | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | QI_VCORE_ STATUS | QI_VCORE_STATU S | LDO on/off status 0: off 1: on |
| 14 | VCORE_OC _FLAG | VCORE_OC_FLAG | Informs some VCORE over current statuses have been asserted. 0: No over current status 1: Some over current status |
| 10 | RG_VCORE | RG_VCORE_NDIS | VCORE POWER DOWN NMOS Enable |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  | _NDIS_EN | _EN | 1'b0: disable 1'b1: enable |
| 8:4 | RG_VCORE _VOSEL | RG_VCORE_VOSE L | VCORE output selection signal (5'b01000:1.3V) 5'b00000: 1.1 V 5'b00001: 1.125 V 5'b00010: 1.15 V 5'b00011: 1.175 V 5'b00100: 1.2 V 5'b00101: 1.225 V 5'b00110: 1.25 V 5'b00111: 1.275 V 5'b01000: 1.3 V 5'b01001: 1.325 V 5'b01010: 1.35 V 5'b01011: 1.375 V 5'b01100: 1.4 V 5'b01101: 1.425 V 5'b01110: 1.45 V 5'b01111: 1.475 V 5'b10000: 0.7 V 5'b10001: 0.725 V 5'b10010: 0.75 V 5'b10011: 0.775 V 5'b10100: 0.8 V 5'b10101: 0.825 V 5'b10110: 0.85 V 5'b10111: 0.875 V 5'b11000: 0.9 V 5'b11001: 0.925 V 5'b11010: 0.95 V 5'b11011: 0.975 V 5'b11100: 1 V 5'b11101: 1.025 V 5'b11110: 1.05 V 5'b11111: 1.075 V |
| 0 | RG_VCORE _EN | RG_VCORE_EN | Vcore enable for testing 1'b0: disable 1'b1: enable |

A0700234	VCORE_CON1  VCORE LDO Control Register 1	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | RG_VCORE_CAL |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7:4 | RG_VCORE _CAL | RG_VCORE_CAL | VCORE Voltage Calibration |

A070023C	VCORE_CON3  VCORE LDO Control Register 3	0080

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  | VCORE_VOSEL_SLEEP |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  | 0 | 1 | 0 | 0 | 0 |  |  |  |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 8:4 | VCORE_VO SEL_SLEEP | VCORE_VOSEL_S LEEP | VCORE output selection signal in sleep mode (5'b01000:1.3V) 5'b00000: 1.1 V 5'b00001: 1.125 V 5'b00010: 1.15 V 5'b00011: 1.175 V 5'b00100: 1.2 V 5'b00101: 1.225 V 5'b00110: 1.25 V 5'b00111: 1.275 V 5'b01000: 1.3 V 5'b01001: 1.325 V 5'b01010: 1.35 V 5'b01011: 1.375 V 5'b01100: 1.4 V 5'b01101: 1.425 V 5'b01110: 1.45 V 5'b01111: 1.475 V 5'b10000: 0.7 V 5'b10001: 0.725 V 5'b10010: 0.75 V 5'b10011: 0.775 V 5'b10100: 0.8 V 5'b10101: 0.825 V 5'b10110: 0.85 V 5'b10111: 0.875 V 5'b11000: 0.9 V 5'b11001: 0.925 V 5'b11010: 0.95 V 5'b11011: 0.975 V 5'b11100: 1 V 5'b11101: 1.025 V 5'b11110: 1.05 V 5'b11111: 1.075 V |

###### Voltage Boost

A0700D28	VSBST_CON0	Boost Control Register 0	C640

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  | VSBS T_VIO 28_PG _STAT US |  |  |  |  | NI_VS BST_ SSBP |  |  |  | RG_V SBST _BP | QI_VS BST_ EN |
| Type |  |  |  |  |  | RO |  |  |  |  | RW |  |  |  | RW | RW |
| Reset |  |  |  |  |  | 1 |  |  |  |  | 0 |  |  |  | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 10 | VSBST_VIO 28_PG_STA TUS | VSBST_VIO28_PG _STATUS | Monitor flag of QI_VIO28_PG_STATUS |
| 5 | NI_VSBST_ | NI_VSBST_SSBP | Bypass enable |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  | SSBP |  | 0: disable 1: enable |
| 1 | RG_VSBST_ BP | RG_VSBST_BP | Bypass mode pre-set 0: disable 1: enable |
| 0 | QI_VSBST_ EN | QI_VSBST_EN | VSBST enable signal, digital register default should be 1 0: disable 1: enable |

A0700D30	VSBST_CON2	Boost Control Register 2	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  | RG_VSBST_ SLEW |  | RG_VSBST_ SLEW_NMO S |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  | RW |  | RW |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  | 0 | 0 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 13:12 | RG_VSBST_ SLEW | RG_VSBST_SLEW | ugate slew rate control (default:00) 00: 1.0X 01: 3/4X 10: 2/4X 11: 1/4x |
| 11:10 | RG_VSBST_ SLEW_NMO S | RG_VSBST_SLEW _NMOS | lgate slew rate control (default:00) 00: 1.0X 01: 3/4X 10: 2/4X 11: 1/4x |

A0700D34	VSBST_CON3	Boost Control Register 3	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | RG_VSBST_ VOSEL[1:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:14 | RG_VSBST_ VOSEL[1:0] | RG_VSBST_VOSE L | output selection 00: 5.3V 01: 5.25V 10: 5.2V 11: 5.35V |

A0700D38	VSBST_CON4	Boost Control Register 4	0012

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | QI_VS BST_ OC_S TATU S |  |  |  |  | QI_VS BST_ PG_S TATU S |  |  |  |  |  |  |  |  | RG_V SBST _OVP _ENB |
| Type |  | RO |  |  |  |  | RO |  |  |  |  |  |  |  |  | RW |
| Reset |  | 0 |  |  |  |  | 0 |  |  |  |  |  |  |  |  | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 14 | QI_VSBST_ OC_STATU S | QI_VSBST_OC_ST ATUS | OC STATUS H: OC L: normal operation |
| 9 | QI_VSBST_ PG_STATUS | QI_VSBST_PG_ST ATUS | PG H: power good (VBST ready) L: power not ready (VBST not ready) |
| 0 | RG_VSBST_ OVP_ENB | RG_VSBST_OVP_ ENB | OVP function enable: 0: enable 1: disable |

A0700D40	VSBST_CON6	Boost Control Register 6	400D

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  | VSBS T_OC _FLA G |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  | RO |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 11 | VSBST_OC_ FLAG | VSBST_OC_FLAG | Informs some BOOST over current statuses have been asserted. 0: No over current status 1: Some over current status |

###### ISINK and KPLED Switches

A0700C00	ISINKS_CON0	ISINKS Control Register 0	00A0

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ISINK S_MO DE |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 0 | ISINKS_MO DE | ISINKS_MODE | ISINK0 and ISINK1 control mode select 0: PWM mode, controlled by hardware PWM3 output signal and NI_ISINKS_CH0_EN and NI_ISINKS_CH1_EN 1: Register control mode, controlled by NI_ISINKS_CH0_EN and NI_ISINKS_CH1_EN |

A0700C10	ISINK0_CON0	ISINK0 Control Register 0	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | NI_ISI NK0_ STAT US |  |  |  |  |  |  | RG_IS INK0_ DOUB LE |  | RG_ISINKS_CH0_S TEP |  |  | NI_ISI NKS_ BIAS0 _EN |  |  | NI_ISI NKS_ CH0_ EN |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Type | RO |  |  |  |  |  |  | RW |  | RW |  |  | RW |  |  | RW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reset | 0 |  |  |  |  |  |  | 0 |  | 0 | 0 | 0 | 0 |  |  | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | NI_ISINK0_S TATUS | NI_ISINK0_STATU S | ISINK0 status flag 1'b0: off 1'b1: on |
| 8 | RG_ISINK0_ DOUBLE | RG_ISINK0_DOUB LE | ISINK0 current double selection 1'b0: normal 1'b1: current double |
| 6:4 | RG_ISINKS_ CH0_STEP | RG_ISINKS_CH0_ STEP | Coarse 6 step current level for ISINK CH0, Gary code 3'b000: 4 mA 3'b001: 8 mA 3'b011: 12 mA 3'b010: 16 mA 3'b110: 20 mA 3'b111: 24 mA 3'b101: reserved 3'b100: reserved |
| 3 | NI_ISINKS_ BIAS0_EN | NI_ISINKS_BIAS0_ EN | ISINK bias 0 enable 1'b0: disable ISINK channel 0 1'b1: enable ISINK channel 0 |
| 0 | NI_ISINKS_ CH0_EN | NI_ISINKS_CH0_E N | ISINK channel 0 enable 1'b0: disable ISINK channel 0 1'b1: enable ISINK channel 0 |

NOTE: RG_ISINK0_CLKSEL works at ISINK0_CLKSEL_MODE (@ISINK0_CON0) = 1

A0700C20	ISINK1_CON0	ISINK1 Control Register 0	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | NI_ISI NK1_ STAT US |  |  |  |  |  |  | RG_IS INK1_ DOUB LE |  | RG_ISINKS_CH1_S TEP |  |  | NI_ISI NKS_ BIAS1 _EN |  |  | NI_ISI NKS_ CH1_ EN |
| Type | RO |  |  |  |  |  |  | RW |  | RW |  |  | RW |  |  | RW |
| Reset | 0 |  |  |  |  |  |  | 0 |  | 0 | 0 | 0 | 0 |  |  | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | NI_ISINK1_S TATUS | NI_ISINK1_STATU S | ISINK1 status flag 1'b0: off 1'b1: on |
| 8 | RG_ISINK1_ DOUBLE | RG_ISINK1_DOUB LE | ISINK1 current double selection 1'b0: normal 1'b1: current double |
| 6:4 | RG_ISINKS_ CH1_STEP | RG_ISINKS_CH1_ STEP | Coarse 6 step current level for ISINK CH1, Gary code 3'b000: 4 mA 3'b001: 8 mA 3'b011: 12 mA 3'b010: 16 mA 3'b110: 20 mA 3'b111: 24 mA 3'b101: reserved 3'b100: reserved |
| 3 | NI_ISINKS_ BIAS1_EN | NI_ISINKS_BIAS1_ EN | ISINK bias 1 enable 1'b0: disable ISINK channel 0 1'b1: enable ISINK channel 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 0 | NI_ISINKS_ CH1_EN | NI_ISINKS_CH1_E N | ISINK channel 1 enable 1'b0: disable ISINK channel 1 1'b1: enable ISINK channel 1 |

NOTE: RG_ISINK1_CLKSEL works at ISINK1_CLKSEL_MODE (@ISINK1_CON0) = 1

A0700C80	KPLED_CON0	KPLED Control Register 0	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | NI_KP LED_ STAT US |  |  |  |  |  |  |  |  |  |  |  |  |  | KPLE D_MO DE | NI_KP LED_ EN |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  | RW | RW |
| Reset | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | NI_KPLED_ STATUS | NI_KPLED_STATU S | KeyPad LED status flag 1'b0: off 1'b1: on |
| 1 | KPLED_MO DE | KPLED_MODE | KPLED control mode select 0: PWM mode, controlled by hardware PWM2 output signal and NI_KPLED_EN 1: Register control mode, controlled by NI_KPLED_EN |
| 0 | NI_KPLED_ EN | NI_KPLED_EN | KeyPad LED enable 1'b0: disable KeyPad LED 1'b1: enable KeyPad LED |

###### PCHR

A0700A00	CHR_CON0	Charger Control Register 0	F200

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | RG_VCDT_HV_VTH |  |  |  | RG_VCDT_LV_VTH |  |  |  |  |  |  | CHR_ EN | RG_C SDAC _EN |  |  |  |
| Type | RW |  |  |  | RW |  |  |  |  |  |  | RW | RW |  |  |  |
| Reset | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 |  |  |  | 0 | 0 |  |  |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:12 | RG_VCDT_ HV_VTH | RG_VCDT_HV_VT H | ChargerIn detection high threshold 4b'0000:4.20V 4b'0001:4.25V 4b'0010:4.30V 4b'0011:4.35V 4b'0100:4.40V 4b'0101:4.45V 4b'0110:4.50V 4b'0111:4.55V 4b'1000:4.60V 4b'1001:6.00V 4b'1010:6.50V 4b'1011:7.00V |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 4b'1100:7.50V 4b'1101:8.50V 4b'1110:9.50V 4b'1111:10.5V |
| 11:8 | RG_VCDT_L V_VTH | RG_VCDT_LV_VT H | ChargerIn detection low threshold 4b'0000:4.20V 4b'0001:4.25V 4b'0010:4.30V 4b'0011:4.35V 4b'0100:4.40V 4b'0101:4.45V 4b'0110:4.50V 4b'0111:4.55V 4b'1000:4.60V 4b'1001:6.00V 4b'1010:6.50V 4b'1011:7.00V 4b'1100:7.50V 4b'1101:8.50V 4b'1110:9.50V 4b'1111:10.5V |
| 4 | CHR_EN | CHR_EN | Charger enable setting, which will gate CSDAC_EN & PCHR_AUTO & charger watchdog timer control registers. 0: Disable 1: Enable NOTE: After charger out, this register would reset when charger LDO drops or charger watch dog reset. |
| 3 | RG_CSDAC _EN | RG_CSDAC_EN | Current DAC driver enable. |

NOTE1: In HW_CV mode, RG_VCDT_HV_VTH and RG_VCDT_LV_VTH function will be gated.

NOTE2: After charger out, RG_CSDAC_EN and RG_PCHR_AUTOMODE would reset when charger LDO drops.

A0700A04	CHR_CON1	Charger Control Register 1	1032

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  | RG_VBAT_CV_VTH |  |  |  |  |  |  | RG_VBAT_C C_VTH |  |  |  | RG_V BAT_ CC_E N | RG_V BAT_ CV_E N |
| Type |  |  |  | RW |  |  |  |  |  |  | RW |  |  |  | RW | RW |
| Reset |  |  |  | 1 | 0 | 0 | 0 | 0 |  |  | 1 | 1 |  |  | 1 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 12:8 | RG_VBAT_ CV_VTH | RG_VBAT_CV_VT H | Battery CV dection threshold 00000:4.200 00011:3.775 00100:3.8 00101:3.85 00110:3.9 00111:4.0 01000:4.05 01001:4.10 01010:4.125 01011:4.1375 01100:4.150 01101:4.1625 01110:4.175 01111:4.1875 10000:4.2000 (default) |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 10001:4.2125 10010:4.2250 10011:4.2375 10100:4.2500 10101:4.2625 10110:4.2750 10111:4.3000 11000:4.3250 11010:4.3750 11011:4.4000 11100:4.4250 11111:2.2000 |
| 5:4 | RG_VBAT_ CC_VTH | RG_VBAT_CC_VT H | Battery CC dection threshold |
| 1 | RG_VBAT_ CC_EN | RG_VBAT_CC_EN | Battery CC detection enable |
| 0 | RG_VBAT_ CV_EN | RG_VBAT_CV_EN | Battery CV detection enable |

NOTE: After charger out, RG_VBAT_CV_VTH, RG_VBAT_CC_VTH, RG_VBAT_CC_EN and RG_VBAT_CV_EN would reset when charger LDO drops.

A0700A08	CHR_CON2	Charger Control Register 2	000F

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  | RG_C S_EN |  |  |  |  | RG_CS_VTH |  |  |  |
| Type |  |  |  |  |  |  |  | RW |  |  |  |  | RW |  |  |  |
| Reset |  |  |  |  |  |  |  | 0 |  |  |  |  | 1 | 1 | 1 | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 8 | RG_CS_EN | RG_CS_EN | Current sense enable |
| 3:0 | RG_CS_VTH | RG_CS_VTH | Current sense threshold Bit[3] is used to sync w/ 6323 Bit[2:0] are used to set charging current. 3b'111:70mA 3b'110:200mA 3b'101:300mA 3b'100:400mA 3b'011:500mA 3b'010:600mA 3b'001:700mA 3b'000:800mA |

NOTE: After charger out, RG_CS_EN and RG_CS_VTH would reset when charger LDO drops.

A0700A10	CHR_CON4	Charger Control Register 4	4200

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | RG_CSDAC_DLY |  |  |  | RG_CSDAC_STP |  |  |  |  |  |  |  | RG_CSDAC_STP_I NC |  |  |
| Type |  | RW |  |  |  | RW |  |  |  |  |  |  |  | RW |  |  |
| Reset |  | 1 | 0 | 0 |  | 0 | 1 | 0 |  |  |  |  |  | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 14:12 | RG_CSDAC _DLY | RG_CSDAC_DLY | Current DAC output detection delay control. 3'b000:16us 3'b001:32us 3'b010:64us |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 3'b011:128us 3'b100:256us 3'b101:512us 3'b110:1024us 3'b111:2048us (256us default, reuse in HW CV Mode) |
| 10:8 | RG_CSDAC _STP | RG_CSDAC_STP | Current DAC output step timer. 3'b000:1 code/ per-step 3'b001:1 code/ per-step 3'b010:2 code/ per-step 3'b011:3 code/ per-step 3'b100:4 code/ per-step 3'b101:5 code/ per-step 3'b110:6 code/ per-step 3'b111:7 code/ per-step 3'b010 is the defaultvalue |
| 2:0 | RG_CSDAC _STP_INC | RG_CSDAC_STP_I NC | CV HW Mode increased code per step. 3'b000:1 code/ per-step 3'b001:1 code/ per-step 3'b010:2 code/ per-step 3'b011:3 code/ per-step 3'b100:4 code/ per-step 3'b101:5 code/ per-step 3'b110:6 code/ per-step 3'b111:7 code/ per-step 3'b010 is the defaultvalue |

NOTE: After charger out, RG_CSDAC_DLY, RG_CSDAC_STP, RG_CSDAC_STP_DEC and RG_CSDAC_STP_INC would reset when charger LDO drops.

A0700A14	CHR_CON5	Charger Control Register 5	2111

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  | RG_B ATON _TDET _EN |  |  | RG_B ATON _HT_E N | RG_B ATON _EN |  | RG_VBAT_OV_VTH |  |  |  |  |  |  |
| Type |  |  |  | RW |  |  | RW | RW |  | RW |  |  |  |  |  |  |
| Reset |  |  |  | 0 |  |  | 0 | 1 |  | 0 | 0 | 1 |  |  |  |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 12 | RG_BATON _TDET_EN | RG_BATON_TDET _EN | BATON temperature detection enable via AUXADC 0: disable temperature detection related TREF power switch 1: enable temperature detection related TREF power switch (SW should enable AUXADC reference buffer before enable this switch) |
| 9 | RG_BATON _HT_EN | RG_BATON_HT_E N | Battery-On high temperature detection (1: enable, 0: disable) |
| 8 | RG_BATON _EN | RG_BATON_EN | Battery-On detection for drving protection (1: enable, 0: disable) |
| 6:4 | RG_VBAT_ OV_VTH | RG_VBAT_OV_VT H | Battery over-voltag detection threshold VTHH VTHL 000: 4.2V 4.15V 001: 4.3V 4.25V 010: 4.4V 4.35V 011: 4.45V 4.40V 100: 3.825V 3.825V 101: 4.3V 4.25V 110: 4.3V 4.25V 111: 4.3V 4.25V |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

NOTE: After charger out, RG_BATON_HT_EN, RG_BATON_EN, RG_VBAT_OV_DEG and RG_VBAT_OV_EN would reset when charger LDO drops.

A0700A20	CHR_CON8	Charger Control Register 8	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  | RG_PCHR_FLAG_SEL |  |  |  |  |  | RG_P CHR_ FLAG _EN |  |  |  |  |  |  |  |
| Type |  |  | RW |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |  |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 13:8 | RG_PCHR_ FLAG_SEL | RG_PCHR_FLAG_ SEL | Charger control debug flag selection |
| 7 | RG_PCHR_ FLAG_EN | RG_PCHR_FLAG_ EN | Charger control debug_flag_en |

| RG_PCHR_FLA G_SEL | RGS_PCHR_FLAG_OU T[3] | RGS_PCHR_FLAG_O UT[2] | RGS_PCHR_FLAG_O UT[1] | RGS_PCHR_FLAG_O UT[0] |
| --- | --- | --- | --- | --- |
| ‘h00 | QI_CS_DET | QI_VBAT_CV_DET | QI_VBAT_CC_DET | QI_VCDT_DET |
| ‘h01 | QI_DDLO_DET | QI_UVLO_DET | QI_BATON_UNDET | QI_VBAT_OV_DET |
| ‘h02 | CHRWDT_OUT | PCHR_STATE[2] | PCHR_STATE[1] | PCHR_STATE[0] |
| ‘h03 | QI_DRV_EN | SFSTR_EN | SFSTR_STATE[1] | SFSTR_STATE[0] |
| ‘h04 | BBCTL_FLAG | VBAT_CC_FLAG | VCDT_HV_FLAG | VCDT_LV_FLAG |
| ‘h05 | QI_CS_EN | QI_VCDT_EN | QI_VBAT_CV_EN | QI_VBAT_CC_EN |
| ‘h06 | DDLO_DEB | UVLO_DEB | BATON_UNDET_DEB | VBAT_OV_DEB |
| ‘h07 | 1'B0 | RGS_USBDL_MODE | QI_USBDL_MODE | USBDL_8SCNT_OUT |
| ‘h08 | PCHR_RSTB | PCHR_CK1MS | PCHR_CK1US | PCHR_CK16US |
| ‘h09 | DDLO_CALI_DAT[3] | DDLO_CALI_DAT[2] | DDLO_CALI_DAT[1] | DDLO_CALI_DAT[0] |
| ‘h0A | QI_DRV_D[7] | QI_DRV_D[6] | QI_DRV_D[5] | QI_DRV_D[4] |
| ‘h0B | QI_DRV_D[3] | QI_DRV_D[2] | QI_DRV_D[1] | QI_DRV_D[0] |
| ‘h0C | QI_VBAT_CV_VTH[4] | QI_CS_VTH[2] | QI_CS_VTH[1] | QI_CS_VTH[0] |
| ‘h0D | QI_VBAT_CV_VTH[3] | QI_DRV_D[9] | QI_DRV_D[8] | QI_DRV_EN |
| ‘h0E | DDLO_CALI_OK | VBAT_CV_FLAG | QI_VBAT_CC_VTH[1] | QI_VBAT_CC_VTH[0] |
| ‘h0F | QI_VCDT_VTH[3] | QI_VCDT_VTH[2] | QI_VCDT_VTH[1] | QI_VCDT_VTH[0] |
| ‘h10 | MON_PCHR_CKRTC | MON_BC11_RTC_CK1 SEC | BC11_RTC_RPEN | BC11_RTC_TIMEOUT |
| ‘h11 | BC11_500MS_RPEN | BC11_500MS_TIMEOU | BC11_1M_RPEN | BC11_1M_TIMEOUT |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

|  |  | T |  |  |
| --- | --- | --- | --- | --- |
| ‘h12 | V22_DEB | BC11_RPEN | BC11_CHARGER_DET | BC11_DET_FINISH |
| ‘h13 | PCHR_BC11_BIAS_EN | PCHR_BC11_VREF_V TH | USBLDO_FORCE_EN | CSDAC_ISUSP |
| ‘h14 | PCHR_BC11_IPD_EN[1 ] | PCHR_BC11_IPD_EN[ 0] | PCHR_BC11_IPU_EN[ 1] | PCHR_BC11_IPU_EN[ 0] |
| ‘h15 | PCHR_BC11_VSRC_E N[1] | PCHR_BC11_VSRC_E N[0] | PCHR_BC11_CMP_EN [1] | PCHR_BC11_CMP_EN [0] |
| ‘h16 | BC11_RSTB | MON_BC11_CK1US | DEAD_LAT | QI_PCHR_BC11_CMP _OUT |
| ‘h17 | BC11_CNT[10] | BC11_CNT[9] | BC11_CNT[3] | BC11_CNT[2] |
| ‘h18 | BC11_500MS_RPCNT[9 ] | BC11_500MS_RPCNT[ 8] | BC11_500MS_RPCNT[ 1] | BC11_500MS_RPCNT[ 0] |
| ‘h19 | BC11_1024MS_RPCNT[ 9] | BC11_1024MS_RPCN T[8] | BC11_1024MS_RPCN T[1] | BC11_1024MS_RPCN T[0] |
| ‘h1A | BC11_300SEC_RPCNT[ 9] | BC11_300SEC_RPCN T[8] | BC11_300SEC_RPCN T[1] | BC11_300SEC_RPCN T[0] |
| ‘h1B | BC11_RTC_32768_RPC NT[14] | BC11_RTC_32768_RP CNT[13] | BC11_RTC_32768_RP CNT[1] | BC11_RTC_32768_RP CNT[0] |
| ‘h1C | BC11_RTC_2100SEC_ RPCNT[10] | BC11_RTC_2100SEC_ RPCNT[9] | BC11_RTC_2100SEC_ RPCNT[1] | BC11_RTC_2100SEC_ RPCNT[0] |
| ‘h1D | BC11_STATE[7] | BC11_STATE[6] | BC11_STATE[5] | BC11_STATE[4] |
| ‘h1E | BC11_STATE[3] | BC11_STATE[2] | BC11_STATE[1] | BC11_STATE[0] |
| ‘h1F | BC11_512MS_TIMEOU T | DETECT_STOP | CSDAC_HV_FLAG | CSDAC_LV_FLAG |
| ‘h20 | QI_CS_VTH[3] | QI_CS_VTH[2] | QI_CS_VTH[1] | QI_CS_VTH[0] |
| ‘h21 | QI_VBAT_OV_VTH[3] | QI_VBAT_OV_VTH[1] | QI_VBAT_OV_VTH[0] | QI_CS_LTH |
| ‘h22 | NI_DRV_ISUP_EN | QI_CHRDET | 0 | 0 |
| ‘h23 | QI_BATON_TDET_EN | QI_BATON_BDET_EN | QI_BATON_EN | QI_CHIND_ON |
| ‘h24 | MASK_32MS_RPEN | ULC_DET_EN_SYNC | ULC_DET_32MS_TIM EOUT | ULC_DET_RPEN |
| ‘h25 | CSCMP_HTH_LE | CSCMP_70MA_LE | CSCMP_LTH_LE | 0 |
| ‘h26 | CSCMP_LTH_PU | CSCMP_HTH_PU | CSCMP_70MA_PU | 0 |
| ‘h27 | CSDAC_HV_FLAG | CSDAC_70MA_FLAG | CSDAC_LV_FLAG | CSDAC_EN_TRACK |
| ‘h28 | CHRIND_DIMMING_ON | QI_CHRIND_ON | 0 | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| ‘h29 | 0 | USBDL_DEB | QI_AUTO_POWERON _EN | PRE_BATON_UNDET |
| --- | --- | --- | --- | --- |
| ‘h2A | 0 | 0 | 0 | 0 |
| ‘h2B | 0 | 0 | 0 | 0 |
| ‘h2C | 0 | 0 | 0 | 0 |
| ‘h2D | 0 | 0 | 0 | 0 |
| ‘h2E | 0 | 0 | 0 | 0 |
| ‘h2F | PWON_RSTB_SYNC | PWON_RSTB_SYNC_ D1 | 0 | 0 |

A0700A24	CHR_CON9	Charger Control Register 9	0010

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  | CHRW DT_FL AG_W R | CHRW DT_IN T_EN |  |  | RG_C HRWD T_WR | RG_C HRWD T_EN | RG_CHRWDT_TD |  |  |  |
| Type |  |  |  |  |  |  | RW | RW |  |  | RW | RW | RW |  |  |  |
| Reset |  |  |  |  |  |  | 0 | 0 |  |  | 0 | 1 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 9 | CHRWDT_F LAG_WR | CHRWDT_FLAG_ WR | Time-out flag for charger watchdog timer Read: 0: No time-out status 1: Has time-out status Write: 1: Clears time-out status flag to 0 without delay |
| 8 | CHRWDT_IN T_EN | CHRWDT_INT_EN | Interrupt enable setting for charger watchdog timer. 0: Disable 1: Enable |
| 5 | RG_CHRWD T_WR | RG_CHRWDT_WR | Charger control watchdog write enable |
| 4 | RG_CHRWD T_EN | RG_CHRWDT_EN | Charger control watchdog enable |
| 3:0 | RG_CHRWD T_TD | RG_CHRWDT_TD | Charger control watchdog delay. |

NOTE1: UVLO does not care RG_CHRWDT_EN and will timeout after 3000s.

NOTE2: RG_PCHR_TESTMODE can force to control watchdog enable by using RG_CHRWDT_EN. NOTE3: After charger out, RG_CHRWDT_EN would reset when charger LDO drops.

A0700A28	CHR_CON10	Charger Control Register 10	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  | RG_A DCIN_ CHR_ EN | RG_A DCIN_ VSEN _EN | RG_A DCIN_ VBAT _EN |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  | RW | RW | RW |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 |  |  |  |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 6 | RG_ADCIN_ CHR_EN | RG_ADCIN_CHR_ EN | AUXADC input source enable for CHR (1: enable, 0: disable) |
| 5 | RG_ADCIN_ VSEN_EN | RG_ADCIN_VSEN_ EN | AUXADC input source enable for VSEN (1: enable, 0: disable) |
| 4 | RG_ADCIN_ VBAT_EN | RG_ADCIN_VBAT_ EN | AUXADC input source enable for VBAT (1: enable, 0: disable) |

A0700A30	CHR_CON12	Charger Control Register 12	0410

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | RG_U LC_D ET_E N |  |  | RG_T RACKI NG_E N |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  | RW |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 |  |  | 1 |  |  |  |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7 | RG_ULC_D ET_EN | RG_ULC_DET_EN | Plug out HW detection enable. 1b'0: detection disable. 1b'1: detection enable. |
| 4 | RG_TRACKI NG_EN | RG_TRACKING_E N | 1b'0 : Current calibration use CS_VTH[n] and CS_VTH[n-1] as high/low threshold. 1b'1 : Current calibration use CS_VTH[n] and LTH as high/low threshold. |

A0700A38	CHR_CON14	Charger Control Register 14	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | RG_PCHR_RV2 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7:0 | RG_PCHR_ RV2 | RG_PCHR_RV2 | PCHR reserved 8 bits register [0]: QI_AUTO_PWR_SWOFF is removed in MT6261D |

###### OTHERS

        - OC

A0700E20	OC_CON8	Over-Current Control Register 8	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  | QI_VS F_OC _STAT US | QI_VM C_OC _STAT US | QI_VI BR_O C_ST ATUS |  | QI_VS IM2_O C_ST ATUS | QI_VS IM1_O C_ST ATUS |  | QI_VU SB_O C_ST ATUS | QI_VI O28_ OC_S TATU S |  | QI_VC AMA_ OC_S TATU S | QI_VA _OC_ STAT US |  | QI_VR F_OC _STA TUS |
| Type |  |  | RO | RO | RO |  | RO | RO |  | RO | RO |  | RO | RO |  | RO |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Reset |  |  | 0 | 0 | 0 |  | 0 | 0 |  | 0 | 0 |  | 0 | 0 |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 13 | QI_VSF_OC _STATUS | QI_VSF_OC_STAT US | VSF CURRENT LIMIT STATUS 1'b0: CURRENT LIMIT NOT WORK 1'b1: CURRENT LIMIT WORK |
| 12 | QI_VMC_OC _STATUS | QI_VMC_OC_STAT US | VMC CURRENT LIMIT STATUS 1'b0: CURRENT LIMIT NOT WORK 1'b1: CURRENT LIMIT WORK |
| 11 | QI_VIBR_O C_STATUS | QI_VIBR_OC_STA TUS | VIBR CURRENT LIMIT STATUS 1'b0: CURRENT LIMIT NOT WORK; 1'b1: CURRENT LIMIT WORK |
| 9 | QI_VSIM2_O C_STATUS | QI_VSIM2_OC_ST ATUS | VSIM2 current limit status 1'b0: CURRENT LIMIT NOT WORK 1'b1: CURRENT LIMIT WORK |
| 8 | QI_VSIM1_O C_STATUS | QI_VSIM1_OC_ST ATUS | VSIM1 current limit status 1'b0: CURRENT LIMIT NOT WORK 1'b1: CURRENT LIMIT WORK |
| 6 | QI_VUSB_O C_STATUS | QI_VUSB_OC_STA TUS | VUSB current limit status 1'b0: CURRENT LIMIT NOT WORK 1'b1: CURRENT LIMIT WORK |
| 5 | QI_VIO28_O C_STATUS | QI_VIO28_OC_STA TUS | VIO28 CURRENT LIMIT STATUS 1'b0: CURRENT LIMIT NOT WORK 1'b1: CURRENT LIMIT WORK |
| 3 | QI_VCAMA_ OC_STATU S | QI_VCAMA_OC_S TATUS | Vcama current limit status (1'b1: current limit work; 1'b0: current limit not work) |
| 2 | QI_VA_OC_ STATUS | QI_VA_OC_STATU S | Va current limit status 1'b0: current limit not work 1'b1: current limit work(analog LDO) |
| 0 | QI_VRF_OC _STATUS | QI_VRF_OC_STAT US | Vrf current limit status (1'b1: current limit work; 1'b0: current limit not work) |

A0700E24	OC_CON9	Over-Current Control Register 9	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QI_VI O18_ OC_S TATU S | QI_VC ORE_ OC_S TATU S |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RO | RO |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 1 | QI_VIO18_O C_STATUS | QI_VIO18_OC_STA TUS | VIO18 CURRENT LIMIT STATUS 1'b0: CURRENT LIMIT NOT WORK 1'b1: CURRENT LIMIT WORK |
| 0 | QI_VCORE_ OC_STATU S | QI_VCORE_OC_S TATUS | VCORE CURRENT LIMIT STATUS 1'b0: CURRENT LIMIT NOT WORK 1'b1: CURRENT LIMIT WORK |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

        - TEST MODE

| A_FUNC_MODE (for FT debug) |  |
| --- | --- |
| Function | GPIO |
| CCI_SCLK | GPIO_1 |
| CCI_SFSI | GPIO_0 |
| CCI_SDI | GPIO_2 |
| CCI_SDO | GPIO_3 |
| A_FUNC_DCK | UTXD1 |
| A_FUNC_RSTB | BPI_BUS1 |
| BBWAKEUP_EXT | LSCK |
| A_FUNC_DOUT[0] | SDA28 |
| A_FUNC_DOUT[1] | LSCE_B |
| A_FUNC_DOUT[2] | LSDA |
| A_FUNC_DOUT[3] | LPTE |
| A_FUNC_DIN[0] | URXD1 |
| A_FUNC_DIN[1] | MCCM0 |
| A_FUNC_DIN[2] | MCDA0 |
| A_FUNC_DIN[3] | SCL28 |
| SYS_RSTB | KCOL1 |

A0700000	WR_PATH_CON Switch Configuring Path Control Register 0	0004

0

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  | AUX_ PWDB |  |  | CCI_S EL_26 M |  |  | CCI_P RST_ MODE | CCI_A BIST_ MODE | CCI_A CD_M ODE |
| Type |  |  |  |  |  |  |  | RW |  |  | RW |  |  | RW | RW | RW |
| Reset |  |  |  |  |  |  |  | 0 |  |  | 0 |  |  | 1 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 8 | AUX_PWDB | AUX_PWDB | AUXADC power down bit 0: Power down 1: Normal mode |
| 5 | CCI_SEL_26M | CCI_SEL_26M | The selection bit for speed up 32k clock to 26M clock 0: 32k clock keeps original clock 1: 32k clock speed up to 26M |
| 2 | CCI_PRST_M | CCI_PRST_MODE | The selection bit for PMIC control registers would reset by watchdog |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

|  | ODE |  | signal or not 0: Not reset 1: Reset |
| --- | --- | --- | --- |
| 1 | CCI_ABIST_M ODE | CCI_ABIST_MODE | Reserved for testing |
| 0 | CCI_ACD_MO DE | CCI_ACD_MODE | The register bit decides the input/output path of the mixedsys module. For SIMLS, the input selection interface is divided at either FSM or GPIO (also shared with A_FUNC_MODE). The bit is for convenient debug-usage in normal mode, such that the data pattern can be observed or be feed-in by external device, while control register setting still comes from the chip internally(By use of JTAG). It should be notice that this special debug mode should be accompanied by proper setting of GPIO, which decides the PAD OE when in normal function. 0: data pattern comes from chip internally, and the output data cannot be bypassed to GPIO. 1: analog debug mode in normal function. |

A0700F04	TEST_CON1	PMU TEST Control Register 1	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  | RG_IBIAS_TRIM |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  | RW |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |  |  |  |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 11:8 | RG_IBIAS_T RIM | RG_IBIAS_TRIM | V/I source bias triming 4'b0000: +0 % 4'b0001: +6.25 % 4'b0010: +12.5 % 4'b0011: +18.75 % 4'b0100: +25 % 4'b0101: +31.25 % 4'b0110: +37.5 % 4'b0111: +43.75 % 4'b1000: -50 % 4'b1001: -43.75 % 4'b1010: -37.5 % 4'b1011: -31.25 % 4'b1100: -25 % 4'b1101: -18.75 % 4'b1110: -12.5 % 4'b1111: -6.25 % |

##### Programming Guide

###### LDOs

        - Power Control of LDO

VCAMA, VMC and VIBR is the power that can be controlled by either hardware or software. The default value of vx_on_sel is 1. If vx_on_sel is set to 0, VCAMA, VMC and VIBR can be configured as hardware control by SRCLKENA in sleep mode. Otherwise, if vx_on_sel is 1, LDOs are controlled by software by using rg_vx_en. The simplified model of control logics are illustrated as below:

MUX

cci_srclkena

rg_vx_en

0

MUX

1

vx_srclkena_sel

vx_on_sel

EXT_SRCLKENA BT_SRCLKENA TOPSM_SRCLKENA

BT_SRCLKENA TOPSM_SRCLKENA

RG_VX_EN

MUX

cci_srclkena

rg_vx_lp_en

0

MUX

1

VRF, VA, VSF, VIO18, VIO28, VSIM1, VSIM2 and VUSB can be configured as hardware-controlled or software-controlled low power mode. The default value of vx_mode_sel is 0. LDO low power mode selection	is controlled by software by using qi_vx_lp_en. Otherwise, if vx_mode_sel is 1, LDO low power mode selection is configured as hardware control by SRCLKENA in sleep mode. That is, these LDOs are set to low power configuration during sleep mode. The simplified model of control logics are illustrated as below:

EXT_SRCLKENA BT_SRCLKENA TOPSM_SRCLKENA

BT_SRCLKENA TOPSM_SRCLKENA

RG_VX_LP_EN

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

vx_srclkena_sel	vx_mode_sel

Other LDOs are turned on/off by software only. The programming guideline of on/off control of all LDOs is listed as below:

| Symbol | Control Register | Control (VX_ON_SEL) |  | Software Control (VX_EN) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Hardware | Software | Enable | Disable |
| VRF LDO | *PMU_VRF_CON0 | - | - | ¦ 0x0001 | & ~0x0001 |
| VCAMA LDO | *PMU_VCAMA_CON0 | & ~0x0002 | ¦ 0x0002 | ¦ 0x0001 | & ~0x0001 |
| VIO28 LDO | * PMU_VIO28_CON0 | - | - | ¦ 0x0001 | & ~0x0001 |
| VIO18 LDO | * PMU_VIO18_CON0 | - | - | ¦ 0x0001 | & ~0x0001 |
| VCORE LDO | * PMU_VCORE_CON0 | - | - | ¦ 0x0001 | & ~0x0001 |
| VUSB LDO | * PMU_VUSB_CON0 | - | - | ¦ 0x0001 | & ~0x0001 |
| VSIM1 LDO | Set to GPIO mode first: * PMU_VSIM1_CON2 ¦ 0x0002 |  |  |  |  |
|  | * PMU_VSIM1_CON0 | - | - | ¦ 0x0001 | & ~0x0001 |
| VSIM2 LDO | Set to GPIO mode first: * PMU_VSIM2_CON2 ¦ 0x0002 |  |  |  |  |
|  | * PMU_VSIM2_CON0 | - | - | ¦ 0x0001 | & ~0x0001 |
| VIBR LDO | * PMU_VIBR_CON0 | & ~0x0002 | ¦ 0x0002 | ¦ 0x0001 | & ~0x0001 |
| VSF LDO | * PMU_VSF_CON0 | - | - | ¦ 0x0001 | & ~0x0001 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| VMC LDO | * PMU_VMC_CON0 | & ~0x0002 | ¦ 0x0002 | ¦ 0x0001 | & ~0x0001 |
| --- | --- | --- | --- | --- | --- |
| VA LDO | * PMU_VA_CON0 | - | - | ¦ 0x0001 | & ~0x0001 |
| RTC LDO | No Test Requirement |  |  |  |  |

        - Voltage Control of LDO

Voltage of VSF and VCORE can be controlled by either hardware or software.

The hardware control of VSF voltage is either by bonding type or trapping value. The default value of vx_vosel_sel is 0 and is hardware control mode. That is, for VSF with external 3.3V serial flash (BOND_EXTSF_SEL = 1’b1, LSA0 = 1’b1), RG_VSF_VOSEL = 3’b011 (3.0V). For VSF with internal

1.8V serial flash (BOND_EXTSF_SEL = 1’b0, BOND_SIPSF_18V = 1’b1), RG_VSF_VOSEL = 1’b001

(1.86V). If vsf_vosel_sel is set to 1, the voltage of LDOs are controlled by software by using rg_vsf_vosel.

VCORE voltage selection can be configured as hardware-controlled mode by SRCLKENA. In sleep mode, VCORE volate would decide by register vcore_vosel_sleep. The simplified model of control logics are illustrated as below:

rg_vcore_vosel

vcore_vosel_sleep

0

MUX

1

MUX

cci_srclkena

vcore_srclkena_sel

RG_VCORE_VOSEL

EXT_SRCLKENA BT_SRCLKENA TOPSM_SRCLKENA

BT_SRCLKENA TOPSM_SRCLKENA

## GSM/GPRS RF

##### General Description

2G RFSYS which is built in MT6261D SOC is a highly integrated RF transceiver for multi-band GSM and GPRS cellular systems.

The features include:

Receiver

- Single-end saw-less Rx

- Quadrature RF mixer

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

- Fully integrated channel filter

- High dynamic range ADC

- 12dB PGA gain with 6dB gain step

Transmitter

- High accurate transmitter modulator for GSM/GPRS application

- Built-in calibration of SX loop filter and loop gain

Frequency synthesizer

- Programmable fractional-N synthesizer

- Integrated wide range RFVCO

- Integrated loop filter

- Fast settling time suitable for multi-slot GSM/GPRS applications

Digitally-Controlled Crystal Oscillator (DCXO)

- Two-pin 26 MHz crystal oscillator

- On-chip programmable capacitor array for coarse-tuning

- On-chip programmable capacitor array for fine-tuning

- Supports 32K XTAL-less operation

##### Functional Block Diagram

2GRXIP

2GRXIN

2GRXQP

2GRXQN

DMD

SDM MASH111

RFVCO

VCO Sub Band Calibration

CP

PFD

DCXO

Loop Filter

DIV4

DIV2

DIV2/ DIV4

2G RFDIG

RXHB

| Predistortion |  |
| --- | --- |
| Gaussian Filter |  |

RXLB

TXO_HB

TXO_LB

TP1 TP2 TP3 TP4

XTAL1 XTAL2

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

VRF

FREF

OUT_32K OUT_32KB

DBB_CLK

GND_RF

Figure 14. Diagram of MT6261D 2G RFSYS

##### Electrical Characteristics

Table 32. DC characteristics (TA = 25oC, VDD = 2.8V unless otherwise stated)

| RFSYS mode | VRF | AVDD28_2GAFE | RFSYS total | Unit |
| --- | --- | --- | --- | --- |
| BCM_Deep sleep (DCXO is off) | 17 | 1 | 18 | uA |
| BCM_Sleep (DCXO is on) | 1.2 | 0.26 | 1.5 | mA |
| Low power mode | 60 | 1 | 61 | uA |
| Full power mode | 1.2 | 0.26 | 1.5 | mA |
| RX (GSM850/EGSM) | 62 | 5 | 67 | mA |
| RX (DCS/PCS) | 66 | 5 | 71 | mA |
| TX (GSM850/EGSM) | 41 | 2 | 43 | mA |
| TX (DCS/PCS) | 36 | 2 | 38 | mA |

Table 33. Rx AC characteristics (TA = 25oC, VDD = 2.8V unless otherwise stated)

| Item | Symbol | Band | Test condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Item | Symbol | Band | Test condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Input frequency | FRX | GSM850 |  | 869 |  | 894 | MHz |
|  |  | GSM900 |  | 925 |  | 960 | MHz |
|  |  | DCS1800 |  | 1,805 |  | 1,880 | MHz |
|  |  | PCS1900 |  | 1,930 |  | 1,990 | MHz |
| Voltage gain 1 | G1 | GSM850 | LNA = High gain PGA = High gain | 521 | 55 |  | dB |
|  |  | GSM900 |  | 522 | 55 |  | dB |
|  |  | DCS1800 | LNA = High gain PGA = High gain | 523 | 55 |  | dB |
|  |  | PCS1900 |  | 524 | 55 |  | dB |
| Voltage gain 2 | G2 | GSM850 | LNA = Middle gain PGA = High gain |  | 46 |  | dB |
|  |  | GSM900 |  |  | 46 |  | dB |
|  |  | DCS1800 | LNA = Middle gain PGA = High gain |  | 45 |  | dB |
|  |  | PCS1900 |  |  | 45 |  | dB |
| Voltage gain 3 | G3 | GSM850 | LNA = Low gain PGA = High gain |  | 26 |  | dB |
|  |  | GSM900 |  |  | 26 |  | dB |
|  |  | DCS1800 | LNA = Low gain PGA = High gain |  | 26 |  | dB |
|  |  | PCS1900 |  |  | 26 |  | dB |
| Noise figure at 25°C | NF25 | GSM850 | G1 |  | 3 | 51 | dB |
|  |  | GSM900 |  |  | 3 | 52 | dB |
|  |  | DCS1800 |  |  | 3 | 53 | dB |
|  |  | PCS1900 |  |  | 3 | 54 | dB |
| Noise figure at 85°C | NF85 | GSM850 | G1 |  | 4.5 |  | dB |
|  |  | GSM900 |  |  | 4.5 |  | dB |
|  |  | DCS1800 |  |  | 4.5 |  | dB |
|  |  | PCS1900 |  |  | 4.5 |  | dB |
| 2nd-order input intercept point | IIP2 | GSM850 | G2 | 311 | 43 |  | dBm |
|  |  | GSM900 |  | 312 | 43 |  | dBm |
|  |  | DCS1800 |  | 313 | 43 |  | dBm |
|  |  | PCS1900 |  | 314 | 43 |  | dBm |
| 3rd-order input intercept point | IIP3 | GSM850 | G2 | -141 | -3 |  | dBm |
|  |  | GSM900 |  | -142 | -3 |  | dBm |
|  |  | DCS1800 |  | -143 | -3 |  | dBm |
|  |  | PCS1900 |  | -144 | -3 |  | dBm |
| 3rd-order input intercept point @ -20°C | IIP3-20 | GSM850 | G2 |  | -5 |  | dBm |
|  |  | GSM900 |  |  | -5 |  | dBm |
|  |  | DCS1800 |  |  | -5 |  | dBm |
|  |  | PCS1900 |  |  | -5 |  | dBm |
| Receiver S/N with 3MHz blocker | SN3M | GSM850 | G2 Blocker = -23dBm | 81 | 12 |  | dB |
|  |  | GSM900 |  | 82 | 12 |  | dB |
|  |  | DCS1800 | G2 | 83 | 12 |  | dB |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Item | Symbol | Band | Test condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCS1900 | Blocker = -26dBm | 84 | 12 |  | dB |
| Receiver S/N with OBB | SNOOB | GSM850 | G2 Blocker = 2dBm, offset +/-20MHz | 65 | 8 |  | dB |
|  |  | GSM900 |  | 65 | 8 |  | dB |
|  |  | DCS1800 | G2 Blocker = -10/2dBm, offset +/-80/-100MHz | 65 | 8 |  | dB |
|  |  | PCS1900 |  | 65 | 8 |  | dB |
| Image rejection ratio | IRR | ALL | G2 | 321,2,3,4 | 40 |  | dB |
| Receiver channel response attenuation |  | ALL | @3MHz offset |  | 20 |  | dB |
|  |  |  | @6MHz offset |  | 35 |  | dB |
| Receiver filtering 3-dB bandwidth |  | ALL | For all gain settings |  | 900 |  | kHz |
| PGA gain linearity |  | ALL | INL |  | 0.2 | 15 | dBΩ |
|  |  |  | DNL |  | 0.1 | 0.55 | dBΩ |
| PGA gain step |  | ALL |  |  | 6 |  | dBΩ |
| PGA dynamic range |  | ALL |  |  | 12 |  | dBΩ |
| I/Q common-mode output voltage |  | ALL | G1 | 1.15 | 1.2 | 1.35 | V |
| Output static dc offset |  | ALL | G1 |  | 100 | 200 | mV |

Table 34. Tx GMSK AC characteristics (TA = 25oC, VDD = 2.8V unless otherwise stated)

| Item | Symbol | Band | Test condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency | FTX | GSM850 |  | 824 |  | 849 | MHz |
|  |  | GSM900 |  | 880 |  | 915 | MHz |
|  |  | DCS1800 |  | 1,710 |  | 1,785 | MHz |
|  |  | PCS1900 |  | 1,850 |  | 1,910 | MHz |
| RMS phase error | PErms | GSM850 GSM900 |  |  | 1.5 | 2.51,2 | degree |
|  |  | DCS1800 PCS1900 |  |  | 1.5 | 2.53,4 | degree |
| Output modulation spectrum | ORFS | GSM850 GSM900 | 400kHz offset (RBW = 30kHz bandwidth) |  | -66 | -641,2 | dBc |
|  |  | DCS1800 PCS1900 |  |  | -66 | -643,4 | dBc |
|  |  | GSM850 GSM900 | 1.8MHz offset |  |  | -755 | dBc |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Item | Symbol | Band | Test condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DCS1800 PCS1900 | (RBW = 30kHz bandwidth) |  |  | -755 | dBc |
| Tx noise in Rx band |  | GSM850 | 20MHz offset |  | -165 | -1635 | dBc/Hz |
|  |  |  | 35MHz offset |  | -166 | -1645 | dBc/Hz |
|  |  | GSM900 | 20MHz offset |  | -165 | -1635 | dBc/Hz |
|  |  |  | 35MHz offset |  | -166 | -1645 | dBc/Hz |
|  |  | DCS1800 | 20MHz offset |  | -160 | -1565 | dBc/Hz |
|  |  | PCS1900 | 20MHz offset |  | -160 | -1565 | dBc/Hz |
| Output power level | Pout | GSM850 GSM900 | PA driver amplifier Rload = 50Ω | 11,2 | 3 | 61,2 | dBm |
|  |  | DCS1800 PCS1900 |  | 13,4 | 3 | 63,4 | dBm |
| Output 3rd harmonics |  | ALL | PA driver amplifier |  | -10 |  | dBc |

Table 35. SX AC characteristics (TA = 25oC, VDD = 2.8V unless otherwise stated)

| Item | Symbol | Test condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency range | Frange |  | 3,296 |  | 3,980 | MHz |
| Reference frequency | Fref |  |  | 26 |  | MHz |
| Frequency step resolution | Fres |  |  | 3 |  | Hz |
| Phase noise | PN10k | @ 10kHz offset |  | -83 |  | dBc/Hz |
|  | PN400k | @ 400kHz offset |  | -114 |  | dBc/Hz |
|  | PN3M | @ 3MHz offset |  | -136 |  | dBc/Hz |
| Lock time of Rx burst | Tlock_rx | Frequency error < ± 0.1ppm |  | 150 | 2005 | us |
| Lock time of Tx burst | Tlock_tx | Frequency error < ± 0.1ppm |  | 200 | 3005 | us |
| Pushing figure |  | With internal RFVCO LDO |  | 400 |  | kHz/V |

Table 36. DCXO AC characteristics (TA = 25oC, VDD = 2.8V unless otherwise stated)

| Item | Symbol | Test condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Operating frequency | Fref |  |  | 26 |  | MHz |
| Crystal C load | CL |  |  | 7.5 |  | pF |
| Crystal tuning sensitivity | TS |  | 27.5 | 32.3 |  | ppm/pF |
| Static range | SR | CDAC from 0 to 255 | ± 22 | ± 50 |  | ppm |
| Dynamic range | DR | CAFC from 0 to 8191 | 36 | 50 |  | ppm |
| AFC tuning step | Fres-AFC |  |  | 0.008 |  | ppm/DAC |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Item | Symbol | Test condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| AFC settling time | TAFC | CAFC from 0 to 8191 CAFC from 8191 to 0 Frequency error < 0.1ppm |  | 100 | 2005 | us |
| Start-up time | TDCXO | Frequency error < 1ppm Amplitude > 90 % |  |  | 45 | ms |
| Pushing figure |  |  |  | 0.2 |  | ppm/V |
| Fref buffer output level | VFref | Max. loading = 19pF | 0.85 |  |  | Vp-p |
| Fref buffer output phase noise |  | 10kHz offset Jitter noise |  | -135 |  | dBc/Hz |

1, 2: Tested at E-GSM Tx channel 0 and GSM850 Rx channel 190.

3, 4: Tested at PCS Tx channel 601 and DCS Rx channel 636.

5: Not subject to production test – verified by characterization and design.

### Bluetooth

##### Block Description

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Figure 15. System diagram of Bluetooth RF transceiver

The Bluetooth RF subsystem contains a fully integrated transceiver.

For TX path, the baseband transmit data are digitally modulated in the baseband processor then

up-converted to 2.4GHz RF channels through DA converter, filter, IQ up-converter and power amplifier. The power amplifier is capable of transmitting 7.5dBm power for class-1 operation.

For RX path, MT6261D is a low IF receiver architecture. An image-reject mixer down-converts the RF signal to the IF with LO from the synthesizer, which supports different clock frequencies as the reference clock. The mixer output is then converted to digital signal and down-converted to baseband for demodulation. A fast AGC enables effective discovery of device within dynamic range of the receiver.

MT6261D features self calibration schemes to compensate the process and temperature variation to maintain high performance. Those calibrations are performed automatically right after system boot-up.

##### Functional Specifications

###### Basic Data Rate – Receiver Specifications

Table 37. Basic data rate – receiver specifications

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  | Frequency range |  | 2,402 | - | 2,480 | MHz |
|  | Receiver sensitivity | BER < 0.1% | - | -95 | - | dBm |
|  | Max. detectable input power | BER < 0.1% | - | 0 | - | dBm |
|  | C/I co-channel selectivity | BER < 0.1% | - | 4 | - | dB |
|  | C/I 1 MHz adj. channel selectivity | BER < 0.1% | - | -12 | - | dB |
|  | C/I 2 MHz adj. channel selectivity | BER < 0.1% | - | -42.5 | - | dB |
|  | C/I  3 MHz adj. channel selectivity | BER < 0.1% | - | -46 | - | dB |
|  | C/I image channel selectivity | BER < 0.1% | - | -24 | - | dB |
|  | C/I image 1 MHz adj. channel selectivity | BER < 0.1% | - | -45 | - | dB |
|  | Out-of-band blocking | 30 to 2,000 MHz | - | -4 | - | dBm |
|  |  | 2,000 to 2,399 MHz | - | -18 | - | dBm |
|  |  | 2,498 to 3,000 MHz | - | -18 | - | dBm |
|  |  | 3,000 MHz to 12.75 GHz | - | 1 | - | dBm |
|  | Intermodulation |  | - | -22 | - | dBm |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Basic Data Rate – Transmitter Specification

Table 38. Basic data rate – transmitter specification

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  | Frequency range |  | 2,402 | - | 2,480 | MHz |
|  | Maximum transmit power |  | - | 7.5 | - | dBm |
|  | Gain step |  | - | 4 | - | dB |
|  | Δf1avg (00001111) |  | 140 | 158 | 175 | kHz |
|  | Δf2max (10101010) |  | 115 | 130 | - | kHz |
|  | Δf1avg/Δf2avg |  | 0.8 | 0.9 | - | kHz |
|  | Initial carrier frequency drift |  | -75 | 5 | 75 | kHz |
|  | Frequency drift | DH1 | -25 | 9 | 25 | kHz |
|  |  | DH3 | -40 | 10 | 40 | kHz |
|  |  | DH5 | -40 | 10 | 40 | kHz |
|  | Max. drift rate |  | - | 100 | 400 | Hz/μs |
|  | BW20dB of Tx output spectrum |  | - | 920 | 1,000 | kHz |
|  | In-band spurious emission | ±2 MHz offset | - | -38 | - | dBm |
|  |  | ±3 MHz offset | - | -43 | - | dBm |
|  |  | > ±3 MHz offset | - | -43 | - | dBm |
|  | Out-of-band spurious emission | 30 MHz to 1 GHz | - | -36 | - | dBm |
|  |  | 1 to 12.75 GHz | - | -30 | - | dBm |
|  |  | 1.8 to 1.9 GHz | - | -47 | - | dBm |
|  |  | 5.15 to 5.3 GHz | - | -47 | - | dBm |

###### Enhanced Data Rate – Receiver Specifications

Table 39. Enhanced data rate – receiver specifications

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  | Frequency range |  | 2,402 | - | 2,480 | MHz |
|  | Receiver sensitivity | π/4 DQPSK, BER < 0.01% | - | -95 | - | dBm |
|  |  | 8PSK, BER < 0.01% | - | -88 | - | dBm |
|  | Max. detectable input power | π/4 DQPSK, BER < 0.01% | - | -4.5 | - | dBm |
|  |  | 8PSK, BER < 0.01% | - | -4.5 | - | dBm |
|  | C/I co-channel selectivity | π/4 DQPSK, BER < 0.01% | - | 8 | - | dB |
|  |  | 8PSK, BER < 0.01% | - | 14.5 | - | dB |
|  | C/I 1MHz adj. channel selectivity | π/4 DQPSK, BER < 0.01% | - | -13 | - | dB |
|  |  | 8PSK, BER < 0.01% | - | -7 | - | dB |
|  | C/I 2MHz adj. channel | π/4 DQPSK, BER < 0.01% | - | -42 | - | dB |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  | selectivity | 8PSK, BER < 0.01% | - | --41.5 | - | dB |
|  | C/I  3MHz adj. channel selectivity | π/4 DQPSK, BER < 0.01% | - | -48 | - | dB |
|  |  | 8PSK, BER < 0.01% | - | -44.5 | - | dB |
|  | C/I image channel selectivity | π/4 DQPSK, BER < 0.01% | - | -30 | - | dB |
|  |  | 8PSK, BER < 0.01% | - | -23 | - | dB |
|  | C/I image 1 MHz adj. channel selectivity | π/4 DQPSK, BER < 0.01% | - | -47.5 | - | dB |
|  |  | 8PSK, BER < 0.01% | - | -44.5 | - | dB |

###### Enhanced Data Rate – Transmitter Specifications

Table 40. Enhanced data rate – transmitter specifications

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  | Frequency range |  | 2,402 | - | 2,480 | MHz |
|  | Max. transmit power | π/4 DQPSK | - | 4.5 | - | dBm |
|  |  | 8PSK | - | 4.5 | - | dBm |
|  | Relative transmit power | π/4 DQPSK | - | -1.7 | - | dB |
|  |  | 8PSK | - | -1.7 | - | dB |
|  | Freq. stability ω0 | π/4 DQPSK | - | 1.5 | - | kHz |
|  |  | 8PSK | - | 1.5 | - | kHz |
|  | Freq. stability ω1 | π/4 DQPSK | - | 3 | - | kHz |
|  |  | 8PSK | - | 3 | - | kHz |
|  | ¦ ω0+ω1¦ | π/4 DQPSK | - | 2.8 | - | kHz |
|  |  | 8PSK | - | 2.8 | - | kHz |
|  | RMS DEVM | π/4 DQPSK | - | 7 | - | % |
|  |  | 8PSK | - | 6 | - | % |
|  | 99% DEVM | π/4 DQPSK | - | 11 | - | % |
|  |  | 8PSK | - | 11 | - | % |
|  | Peak DEVM | π/4 DQPSK | - | 18 | - | % |
|  |  | 8PSK | - | 18 | - | % |
|  | In-band spurious emission | π/4 DQPSK, ±1 MHz offset | - | -28 | - | dBm |
|  |  | 8PSK, ±1 MHz offset | - | -28 | - | dBm |
|  |  | π/4 DQPSK, ±2 MHz offset | - | -25 | - | dBm |
|  |  | 8PSK, ±2 MHz offset | - | -25 | - | dBm |
|  |  | π/4 DQPSK, ±3 MHz offset | - | -40.5 | - | dBm |
|  |  | 8PSK, ±3 MHz offset | - | -40.5 | - | dBm |

Note: To meet the specifications, use a front-end band-pass filter.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

FM RF

##### Block Description

The connection between internal modules, as well as external interfaces, are as shown in Figure 16. The FM receiver section incorporates the complete receiving path with wide tuning range. The FM baseband signal processor incorporates the digital demodulator and audio processing function which provides superior audio quality.

FM contains completely integrated FM audio receiver functions (RDS/RBDS may also be supported depending on the model number). The integrated receiver enables superior sensitivity, ACI performance and FM audio performances with minimum external BOM.

The FM subsystem supports either high performance stereo analog line out or digital audio output (I2S).

For models supporting RDS/RBDS, large dedicated internal data buffers are allocated to reduce the frequency of the interrupt to the host, so that the receiving host can enter low power states efficiently.

| sysrst_b fspi_core_rstb fspi_rgf_rstb I2S Interface interrupts FSPI host Interface | FM RX FM_CLKCON FM_MAINCON FM FMX FM Modem Front End FM RDS FM SCRG FM HCI FM RGF (SPI slave) | FM_RF FM RX AFE RF MACRO Interface FM RX RF |  |
| --- | --- | --- | --- |
|  |  | RF FM_RGF Control _RF |  |

Figure 16. Block diagram of hardware top-level architecture

##### Functional Specifications

Table 41. FM receiver DC characteristics (TA=25°C, VDD=2.8V unless otherwise stated)

| Operating mode | Current consumption | Unit |
| --- | --- | --- |
| Idle | 5 | μA |
| FM receiver | 12 | mA |

Unless otherwise stated, all receiver characteristics are applicable to both long and short antenna ports when operated under the recommended operating conditions. Typical specifications are for channel 98.7MHz, default register settings and under recommended operating conditions. The

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

minimum and maximum specifications are for extreme operating voltage and temperature conditions, unless otherwise stated.

Table 42. FM receiver AC characteristics

| Symbol | Description | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  | Input frequency range |  | 65 |  | 108 | MHz |
|  | Sensitivity (long antenna)1,3 | SINAD = 26dB, unmatched |  | 3 |  | dBVemf |
|  |  | SINAD = 26dB, matched |  | 2 |  | dBVemf |
|  | RDS sensitivity (long antenna) | f=2kHz, BLER < 5%, unmatched |  | 18 |  | dBVemf |
|  | Sensitivity (short antenna)1,3 | SINAD = 26dB, unmatched |  | 3 |  | dBVemf |
|  | RDS sensitivity (short antenna) | f = 2kHz, BLER < 5%, unmatched |  | 18 |  | dBVemf |
|  | LNA input resistance4 | Antenna port |  | 2.4k |  | Ohm |
|  | LNA input capacitance4 | Antenna port |  | 8 |  | pF |
|  | AM suppression1,4 | M = 0.3 |  | 58 |  | dB |
|  | Adjacent channel selectivity1,4 | 200kHz |  | 53 |  | dB |
|  | Alternate channel selectivity1,4 | 400kHz |  | 65 |  | dB |
|  | Spurious response rejection4 | In-band |  | 55 |  | dB |
|  | Maximum input level |  |  | 117 |  | dBVemf |
|  | Audio mono (S+N+D)/(N+D)1,3,4 |  |  | 60 |  | dB |
|  | Audio stereo (S+N+D)/(N+D)2,3,4 |  |  | 52 |  | dB |
|  | Audio stereo separation4 | f = 75kHz |  | 45 |  | dB |
|  | Audio output load resistance | Single-ended at AFR/AFL outputs |  | 10k |  | Ohm |
|  | Audio output load capacitance | Single-ended at AFR/AFL outputs |  | 12.5 |  | pF |
|  | Audio output voltage1,4 | At AFR/AFL outputs |  | 80 |  | mVrms |
|  | Audio output THD1,4 |  |  | 0.05 | 0.1 | % |
|  | Audio output frequency range | 3dB corner frequency | 30 |  | 15k | Hz |
| 1 f = 22.5kHz, fm = 1kHz, 50s de-emphasis, mono, L = R 2 f = 22.5kHz, fm = 1kHz, 50us de-emphasis, stereo 3 A-weighting, BW = 300 Hz to 15 kHz 4 Vin = 60dBVemf 5 Reference clock accuracy assumes ideal FM source. If the input FM source has less frequency error, it is recommended to use a reference clock of accuracy within 100ppm so as not to affect the channel scan quality. |  |  |  |  |  |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

### Package Information

##### Package Outlines

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Figure 17. Outlines and dimension of TFBGA 8.1mm*7.6mm, 145-ball, 0.5 mm pitch package

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Thermal Operating Specifications

| Symbol | Description | Value | Unit | Notes |
| --- | --- | --- | --- | --- |
|  | Thermal resistance from device junction to package case | 48 | C/W |  |
|  | Maximum package temperature | 65 | Deg C |  |
|  | Maximum power dissipation | 1.28 | W |  |

    - Lead-free Packaging

MT6261D is provided in a lead-free package and meets RoHS requirements

### Ordering Information

##### Top Marking Definition

MEDIATEK

ARM

MT6XXXX DDDD - #### LLLLLL

MTXXXXXX	Product No.

DDDD:	Date Code

####:	Subcontractor Code

LLLLLL:	Die Lot No.

Figure 18. Mass production top marking of MT6261D

| Part number | Package | Description |
| --- | --- | --- |
| MT6261DA/A | TFBGA | 8.1mm*7.6mm, 145-ball, 0.5 mm pitch package, non-security version |

# Micro-Controller Unit Peripherals

### Pulse-Width Modulation Outputs (2 Channel)

##### General Description

2 generic pulse-width modulators are implemented to generate pulse sequences with programmable frequency and duty cycles for LCD backlight. As long as the internal counter value is bigger than or equal to the threshold value, the duration of the PWM output signal is LOW. The waveform is shown in Figure 19.

Internal counter Threshold

PWM Signal

Figure 19. PWM waveform

The frequency and volume of the PWM output signal are determined by PWM1_COUNT, PWM1_THRES and PWM1_CON. The POWERDOWN (pdn1_pwm) signal is applied to power-down the PWM_1ch module. When PWM_1ch is deactivated (pwm1_pdn=1), the output is in the LOW state.

The output PWM frequency is determined by:

CLK

CLOCK _ DIV (PWM _ COUNT 1)

CLK  13000000 when CLKSEL  0,CLK  32000whenCLKSEL  1

CLOCK_DIV = 1, when CLK[1:0] = 00b CLOCK_DIV = 2, when CLK[1:0] = 01b CLOCK_DIV = 4, when CLK[1:0] = 10b CLOCK_DIV = 8, when CLK[1:0] = 11b

The output PWM duty cycle is determined by:

PWM _ THRES PWM _ COUNT  1

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Note: PWM_THRES should be less than the PWM_COUNT. If this condition is not satisfied, the output pulse of the PWM will always be HIGH.

Figure 20 shows the PWM waveform with the indicated register values.

13MHz

PWM_COUNT = 5

PWM_THRES = 1 PWM_CON= 0b

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Figure 20. PWM waveform with register values

##### Register Definition

Module name: Pulse Width Modulation base address: (+A00E0000h)

| Address | Name | Width | Register function |
| --- | --- | --- | --- |
| A00E0000 | PWM1_CTRL_ADDR | 16 | PWM1 control register |
| A00E0004 | PWM1_COUNT_ADDR | 16 | PWM1 max counter value register |
| A00E0008 | PWM1_THRESH_ADDR | 16 | PWM1 threshold value register |

A00E0000	PWM1_CTRL_ADDR	PWM1 Control Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  | PWM1 _CLK_ SEL | PWM1_CLK_ DIV |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | RW | RW |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 2 | PWM1_CLK _SEL | CLK_SEL | Selects source clock frequency of PWM1 0: CLK = 13MHz 1: CLK = 32kHz |
| 1:0 | PWM1_CLK _DIV | CLK_DIV | Selects clock prescaler scale of PWM1 2'b00: f = fclk 2'b01: f = fclk/2 2'b10: f = fclk/4 2'b11: f = fclk/8 |

A00E0004	PWM1_COUNT_ADDR	PWM1 Max. Counter Value Register	1FFF

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  | PWM1_COUNT |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  | RW |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 12:0 | PWM1_COU NT | PWM1_COUNT | PWM1 maximum counter value This value is the initial value of the internal counter. Regardless of the operation mode, if PWM1_COUNT is written while the internal counter is counting backwards, the new initial value will not take effect until the internal counter counts down to 0, i.e. a complete period. |

A00E0008	PWM1_THRESH_ADDR	PWM1 Threshold Value Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  | PWM1_THRES |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  | RW |  |  |  |  |  |  |  |  |  |  |  |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Reset |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 12:0 | PWM1_THR ES | PWM1_THRES | PWM1 threshold value When the internal counter value is bigger than or equal to PWM1_THRES, the PWM1 output signal will be "0". When the internal counter is less than PWM1_THRES, the PWM1 output signal will be "1". |

Module name: Pulse Width Modulation base address: (+A0280000h)

| Address | Name | Width | Register function |
| --- | --- | --- | --- |
| A0280000 | PMW4_CTRL_ADDR | 16 | PMW4 control register |
| A0280004 | PMW4_COUNT_ADDR | 16 | PMW4 max counter value register |
| A0280008 | PMW4_THRESH_ADDR | 16 | PMW4 threshold value register |

A0280000	PMW4_CTRL_ADDR	PMW4 Control Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  | PMW4 _CLK_ SEL | PMW4_CLK_ DIV |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | RW | RW |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 2 | PMW4_CLK _SEL | CLK_SEL | Selects source clock frequency of PMW4 0: CLK = 13MHz 1: CLK = 32kHz |
| 1:0 | PMW4_CLK _DIV | CLK_DIV | Selects clock prescaler scale of PMW4 2'b00: f = fclk 2'b01: f = fclk/2 2'b10: f = fclk/4 2'b11: f = fclk/8 |

A0280004	PMW4_COUNT_ADDR	PMW4 Max. Counter Value Register	1FFF

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  | PMW4_COUNT |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  | RW |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 12:0 | PMW4_COU NT | PMW4_COUNT | PMW4 maximum counter value This value is the initial value of the internal counter. Regardless of the operation mode, if PMW4_COUNT is written while the internal counter is counting backwards, the new initial value will not take effect until the internal counter counts down to 0, i.e. a complete period. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A0280008	PMW4_THRESH_ADDR	PMW4 Threshold Value Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  | PMW4_THRES |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  | RW |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 12:0 | PMW4_THR ES | PMW4_THRES | PMW4 threshold value When the internal counter value is bigger than or equal to PMW4_THRES, the PMW4 output signal will be "0". When the internal counter is less than PMW4_THRES, the PMW4 output signal will be "1". |

Module name: PWM_2CH base address: (+A0740000h)

| Address | Name | Width | Register Function |
| --- | --- | --- | --- |
| A074000C | PWM2_CTRL | 16 | PWM2 control register Selects CLK SRC and prescaler scale. |
| A0740014 | PWM2_THRES | 16 | PWM2 threshold value register Controls the duty of waveform |
| A0740018 | PWM3_CTRL | 16 | PWM3 control register Select CLK SRC and prescaler scale. |
| A074001C | PWM3_COUNT | 16 | PWM3 max counter value register Configures internal counter's max. value |
| A0740020 | PWM3_THRES | 16 | PWM3 threshold value register |

A074000C	PWM2_CTRL	PWM2 Control Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  | PWM2 _CLK_ SEL |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 2 | PWM2_CLK_SEL | Selects PWM2 CLK 0: CLK = 13M CLK 1: CLK = 32k CLK |

A0740014	PWM2_THRES PWM2 Threshold Value Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PWM2_THRE S |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Name | Description |
| --- | --- | --- |
| 1:0 | PWM2_THRES | PWM2 threshold value 0: Duty = 0% 1: Duty = 50% 2: Duty = 100% |

A0740018	PWM3_CTRL	PWM3 Control Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | PWM3 _ALW AYS_ HIGH | PWM3 _CLK_ SEL | PWM3_CLK_ DIV |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW | RW | RW |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 3 | PWM3_ALWAYS_HIG H | When pwm3_thresh is set to be bigger than pwm3_width, which means the PWM output is always high, the driver should set this register to 1. It is specially used by ISINK. 0: Duty! = 100% 1: Duty = 100% |
| 2 | PWM3_CLK_SEL | Selects PWM3 CLK 0: CLK = 13M CLK 1: CLK = 32k CLK |
| 1:0 | PWM3_CLK_DIV | PWM3 CLK division 2'b0: f = fclk 2'b1: f = fclk/2 2'b2: f = fclk/4 2'b3: f = fclk/8 |

A074001C	PWM3_COUNT	PWM3 Max Counter Value Register	1FFF

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  | PWM3_COUNT |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  | RW |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 12:0 | PWM3_COUNT | PWM3 maximum counter value This value is the initial value of the internal counter. Regardless of the operation mode, if PWM3_COUNT is written while the internal counter is counting backwards, the new initial value will not take effect until the internal counter counts down to 0, i.e. a complete period. |

A0740020	PWM3_THRES	PWM3 Threshold Value Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  | PWM3_THRES |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  | RW |  |  |  |  |  |  |  |  |  |  |  |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Reset |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 12:0 | PWM3_THRES | PWM3 threshold value When the internal counter value is bigger than or equal to PWM3_THRES, the PWM3 output signal will be 0. When the internal counter is less than PWM3_THRES, the PWM3 output signal will be 1. |

### SIM Interface

SIMVCC SIMSEL SIMRST SIMCLK

SIMDATA

SIM2VCC SIM2SEL SIM2RST SIM2CLK

SIM2DATA

13MHz

13MHz

1st SIM I/F

2nd SIM I/F

2nd SIM CARD

VCC1

RST1 CLK1 DATA1

IRQ Ctrl

APB

Bridge

VCC2

RST2 CLK2 DATA2

1st SIM CARD

SIMIRQ

SIM2IRQ

MT6261D contains two dedicated smart card interfaces to allow the MCU to access two SIM cards. Each interface can operate via 5 terminals. See Figure 21, SIMVCC, SIMSEL, SIMRST, SIMCLK and SIMDATA are for one SIM interface, and SIM2VCC, SIM2SEL, SIM2RST, SIM2CLK and SIM2DATA are for the other SIM interface.

|  | Level Shift |
| --- | --- |

Figure 21. Block diagram of SIM interface

The functions of the two SIM interfaces are identical; therefore, only the first SIM interface will be described in this document. SIMVCC is used to control the external voltage supply to the SIM card, and SIMSEL determines the regulated smart card supply voltage. SIMRST is used as the SIM card reset signal. Besides, SIMDATA and SIMCLK are used for data exchange.

The SIM interface is a half duplex asynchronous communication port, and its data format is composed of ten consecutive bits: a start bit in state “low”, eight information bits and a tenth bit used for parity checking. The data format can be divided into two modes as follows:

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

- Direct convention mode (ODD = SDIR = SINV = 0)

SB D0 D1 D2 D3 D4 D5 D6 D7 PB

SB: Start bit (in state “low”)

Dx: Data byte (LSB is the first and logic level ONE is in state “high”)

PB: Even parity check bit

- Inverse convention mode (ODD = SDIR = SINV = 1)

SB N7 N6 N5 N4 N3 N2 N1 N0 PB

SB: Start bit (in state “low”)

Nx: Data byte (MSB is the first and logic level ONE is in state “low”)

PB: Odd parity check bit

If the receiver obtains a wrong parity bit, it will respond by pulling the SIMDATA “low” to inform the transmitter, and the transmitter will retransmit the character.

If the receiver is an SIM card, the error response will start 0.5 bit after the PB and may last for 1 ~ 2-bit period. If the receiver is an SIM interface, the error response will start 0.5 bit after the PB and last for 1.5-bit period.

If the SIM interface is a transmitter, it will take total 14 bits guard period wherever the error response appears. If the receiver shows the error response, the SIM interface will retransmit the previous character again, or it will transmit the next character.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Figure 22. Timing diagram of SIM interface

##### Register Definition

When the MCU controls two SIM card interfaces, all registers will be duplicated to two copies but with different base address. n = “0” is for the 1st SIM card interface; n=1 is for the 2nd SIM card interface. For example, address SIMIF0+0000h is mapped to the SIMIF0_SIM_CTRL register while address SIMIF1+0000h is mapped to the SIMIF1_SIM_CTRL register.

###### Register Overview

| MCU register address (hex) | Acronym | Description |
| --- | --- | --- |
| 1st SIM card interface |  |  |
| SIMIF0+0000h | SIMIF0_SIM_CTRL | Control register |
| SIMIF0+0004h | SIMIF0_SIM_CONF | Configuration register |
| SIMIF0+0008h | SIMIF0_SIM_BRR | Baudrate register |
| SIMIF0+0010h | SIMIF0_SIM_IRQEN | Interrupt enabling register |
| SIMIF0+0014h | SIMIF0_SIM_STS | Status register |
| SIMIF0+0018h | SIMIF0_SIM_CLR_STA | SIM clear status |
| SIMIF0+0020h | SIMIF0_SIM_RETRY | Retry limit register |
| SIMIF0+0024h | SIMIF0_SIM_TIDE | FIFO tide mark register |
| SIMIF0+0030h | SIMIF0_SIM_DATA | TX/RX data register |
| SIMIF0+0034h | SIMIF0_SIM_COUNT | FIFO count register |
| SIMIF0+0040h | SIMIF0_SIM_ATIME | Activation time register |
| SIMIF0+0044h | SIMIF0_SIM_DTIME | Deactivation time register |
| SIMIF0+0048h | SIMIF0_SIM_TOUT | Character to character waiting time register |
| SIMIF0+004Ch | SIMIF0_SIM_GTIME | Block to block guard time register |
| SIMIF0+0050h | SIMIF0_SIM_ETIME | Block to error signal time register |
| SIMIF0+0054h | SIMIF0_SIM_EXT_TIME | Extend data I/O state switch time register |
| SIMIF0+0058h | SIMIF0_SIM_CGTIME | Character to character guard time register |
| SIMIF0+0060h | SIMIF0_SIM_INS | Command header register : INS |
| SIMIF0+0064h | SIMIF0_SIM_IMP3 | Command header register : P3 |
| SIMIF0+0068h | SIMIF0_SIM_SW1 | Procedure byte register : SW1 |
| SIMIF0+006Ch | SIMIF0_SIM_SW2 | Procedure byte register : SW2 |
| SIMIF0+0070h | SIMIF0_SIM_ATRSTA | ATR state register |
| SIMIF0+0074h | SIMIF0_SIM_STATUS | Protocol state register |
| SIMIF0+0080h | SIMIF0_SIM_DMADATA | TX/RX data register for DMA |
| SIMIF0+0090h | SIMIF0_SIM_DBG | Debug register |
| SIMIF0+0094h | SIMIF0_SIM_DBGDATA | FIFO data debug register |
| SIMIF0+00A0h | SIMIF0_SIM_SCLK | SCLK PAD control register |
| SIMIF0+00A4h | SIMIF0_SIM_SRST | SRST PAD control register |
| SIMIF0+00A8h | SIMIF0_SIM_SIO | SIO PAD control register |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| MCU register address (hex) | Acronym | Description |
| --- | --- | --- |
| SIMIF0+00ACh | SIMIF0_SIM_MON | PAD monitor register |
| SIMIF0+00B0h | SIMIF0_SIM_SEL | Testing output select |
| 2nd SIM card interface |  |  |
| SIMIF1+0000h | SIMIF1_SIM_CTRL | Control register |
| SIMIF1+0004h | SIMIF1_SIM_CONF | Configuration register |
| SIMIF1+0008h | SIMIF1_SIM_BRR | Baudrate register |
| SIMIF1+0010h | SIMIF1_SIM_IRQEN | Interrupt enabling register |
| SIMIF1+0014h | SIMIF1_SIM_STS | Status register |
| SIMIF1+0018h | SIMIF1_SIM_CLR_STA | Sim clear status |
| SIMIF1+0020h | SIMIF1_SIM_RETRY | Retry limit register |
| SIMIF1+0024h | SIMIF1_SIM_TIDE | FIFO tide mark register |
| SIMIF1+0030h | SIMIF1_SIM_DATA | TX/RX data register |
| SIMIF1+0034h | SIMIF1_SIM_COUNT | FIFO count register |
| SIMIF1+0040h | SIMIF1_SIM_ATIME | Activation time register |
| SIMIF1+0044h | SIMIF1_SIM_DTIME | Deactivation time register |
| SIMIF1+0048h | SIMIF1_SIM_TOUT | Character to character waiting time register |
| SIMIF1+004Ch | SIMIF1_SIM_GTIME | Block to block guard time register |
| SIMIF1+0050h | SIMIF1_SIM_ETIME | Block to error signal time register |
| SIMIF1+0054h | SIMIF1_SIM_EXT_TIME | Extend data I/O state switch time register |
| SIMIF1+0058h | SIMIF1_SIM_CGTIME | Character to character guard time register |
| SIMIF1+0060h | SIMIF1_SIM_INS | Command header register : INS |
| SIMIF1+0064h | SIMIF1_SIM_IMP3 | Command header register : P3 |
| SIMIF1+0068h | SIMIF1_SIM_SW1 | Procedure byte register : SW1 |
| SIMIF1+006Ch | SIMIF1_SIM_SW2 | Procedure byte register : SW2 |
| SIMIF1+0070h | SIMIF1_SIM_ATRSTA | ATR state register |
| SIMIF1+0074h | SIMIF1_SIM_STATUS | Protocol state register |
| SIMIF1+0080h | SIMIF1_SIM_DMADATA | TX/RX data register for DMA |
| SIMIF1+0090h | SIMIF1_SIM_DBGD | Debug register |
| SIMIF1+0094h | SIMIF1_SIM_DBGDATA | FIFO data debug register |
| SIMIF1+00A0h | SIMIF1_SIM_SCLK | SCLK PAD control register |
| SIMIF1+00A4h | SIMIF1_SIM_SRST | SRST PAD control register |
| SIMIF1+00A8h | SIMIF1_SIM_SIO | SIO PAD control register |
| SIMIF1+00ACh | SIMIF1_SIM_MON | PAD monitor register |
| SIMIF1+00B0h | SIMIF1_SIM_SEL | Testing output select |

      - Register Description

SIMn+0000h	SIM Module Control Register	SIMIFN_SIM_CTRL

| bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  | VCCC TRL | VCCL V | RSTC TRL | RSTL V | WRST | CSTO P | SIMO N |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Type |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | W | R/W | R/W |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reset |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

SIMON	Conrols SIM card power-up/power-down

        - 1-to-0 change will start the card deactivation sequence.

        - 0-to-1 change will start the card activation sequence.

CSTOP	Enables clock stop mode. Together with CPOL in the SIM_CONF register, it determines the polarity of SIMCLK in this mode.

- Enable SIMCLK output

- Disable SIMCLK outpu

WRST	Controls SIM card warm reset

RSTLV	Controls SIMRST parking level in SIMRST direct control mode

RSTCTRL  Enables SIMRST direct control mode

VCCLV	Controls SIMVCC parking level in SIMVCC direct control mode

VCCCTRL Enables SIMVCC direct control mode

SIMn+0004h	SIM Module Configuration Register	SIMIFN_SIM_CONF

| bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  | T1TX2 RXEN | TXRDI S | RXRDI S | HFEN | T0EN | T1EN | TOUT | SIMSE L | ODD | SDIR | SINV | CPOL | TXAC K | RXAC K |
| Type |  |  | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |
| Reset |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

RXACK	Handshaking control of SIM card reception error

- Disable character receipt handshaking

- Enable character receipt handshaking

TXACK	Handshaking control of SIM card transmission error

- Disable character transmission handshaking

- Enable character transmission handshaking

CPOL	SIMCLK polarity control in clock stop mode

- Make SIMCLK stop in “low” level

- Make SIMCLK stop in “high” level

SINV	Data inversion mode

- Does not invert the transmitted and received data; data logic ONE is in “high” state

- Invert the transmitted and received data; data logic ONE is in “low” state

SDIR	Direction of data transfer

- LSB is transmitted and received first.

- MSB is transmitted and received first.

ODD	Selecting odd or even parity

- Even parity

- Odd parity

SIMSEL	Selects SIM card supply voltage (also configure SIMSEL in PMU register)

- SIMSEL pin is set to “low” level, 1.8V

- SIMSEL pin is set to “high” level, 3V

TOUT	Controls SIM work waiting time counter

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Disable time-out counter

- Enable time-out counter

T1EN	Controls T = 1 protocol controller

- Disable T = 1 protocol controller

- Enable T = 1 protocol controller

T0EN	Controls T = 0 protocol controller

- Disable T = 0 protocol controller

- Enable T = 0 protocol controller

HFEN	Controls hardware flow

- Disable hardware flow control

- Enable hardware flow control

RXRDIS	Disables RX DMA request

- Enable RX DMA request (default)

RXRDIS must be set to 0 for protocol T = 1

- Disable RX DMA request

During TX transmission and not protocol T = 1, the recommended setting of RXRDIS

is 1

TXRDIS	Disables TX DMA request disable

- Enable TX DMA request (default)

TXRDIS must be set to 0 for protocol T = 1

- Disable TX DMA request

During RX transmission and not protocol T = 1, the recommended setting of TXRDIS is 1.

T1TX2RXEN	Enables DMA type auto switch for protocol T = 1 (this function is not supported in MT6260)

  - Disable DMA type auto switch function

If the current block is TX transmission and the next block is also TX transmission, disabling this bit is recommended

  - Enable DMA type auto switch function

If the current block is TX transmission and the next block is RX transmission, enabling this bit is recommended to improve transmission quality

SIMn +0008h	SIM Baudrate Register	SIMIFN_SIM_BRR

| bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  | ETU[8:0] |  |  |  |  |  |  |  |  | SIMCLK[1:0] |  |
| Type |  |  |  |  |  | R/W |  |  |  |  |  |  |  |  | R/W |  |
| Reset |  |  |  |  |  | 372d |  |  |  |  |  |  |  |  | 01 |  |

SIMCLK	Sets up SIMCLK frequency

- Reserved

- 13/4 MHz

10	13/8 MHz

11	13/12 MHz

ETU	Determines duration of elementary time unit in SIMCLK unit

The minimum valid setting of ETU is 8

SIMn +0010h	SIM Interrupt Enable Register	SIMIFN_SIM_IRQEN

| bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  | UDRU N | EDCE RR | T1EN D | RXER R | T0EN D | SIMO FF | ATRER R | TXER R | TOU T | OVRU N | RXTID E | TXTID E |
| Type |  |  |  |  | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |
| Reset |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

For all the bits

- Disable interrupt

- Enable interrupt

SIMn +0014h	SIM Module Status Register	SIMIFN_SIM_STS

| bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  | UDRU N | EDCE RR | T1EN D | RXER R | T0EN D | SIMO FF | ATRER R | TXER R | TOU T | OVRU N | RXTID E | TXTID E |
| Type |  |  |  |  | R/C | R/C | R/C | R/C | R/C | R/C | R/C | R/C | R/C | R/C | R | R |
| Reset |  |  |  | - | - | - | - | - | - | - | - | - | - | - | - | - |

TXTIDE	The interrupt occurs when the number of transmitted data in the FIFO is less than the transmitted tide.

RXTIDE	The interrupt occurs when the number of received data in the FIFO is less than the received tide.

OVRUN	Receiving FIFO overflow interrupt occurrs. TOUT	Between characters time-out interrupt occurs. TXERR	 Character transmission error interrupt occurs. ATRERR	ATR start time-out interrupt occurrs.

SIMOFF	Card deactivation completed interrupt occurs.

T0END	Data transfer handled by T = 0 controller completed interrupt occurs.

RXERR	Character reception error interrupt occurrs.

T1END	Data transfer handled by T = 1 controller completed interrupt occurs.

EDCERR	T = 1 controller CRC error occurs.

UDRUN	FIFO underflow interrupt occurs (still reading FIFO when FIFO is empty).

SIMn +0018h	SIM Clear Status Register

SIMIFN_SIM_CLR_ST

A

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CLR_ STA |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RO |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

CLA_STA	1: Clear SIMIF. Do not write to SIMIF; 0: SIMIF clear finished or not in clear status, you can write data to SIMIF.

SIMn +0020h	SIM Retry Limit Register	SIMIFN_SIM_RETRY

| bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  | TXRETRY[2:0] |  |  |  |  |  |  |  | RXRETRY[2:0] |  |  |
| Type |  |  |  |  |  | R/W |  |  |  |  |  |  |  | R/W |  |  |
| Reset |  |  |  |  |  | 3h |  |  |  |  |  |  |  | 3h |  |  |

RXRETRY Specifies maximum number of receive retries allowed when parity error occurrs.

TXRETRY Specifies maximum number of transmit retries allowed when parity error occurrs.

SIMn +0024h	SIM FIFO Tide Mark Register	SIMIFN_SIM_TIDE

| bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  | TXTIDE[3:0] |  |  |  |  |  |  |  | RXTIDE[3:0] |  |  |  |
| Type |  |  |  |  | R/W |  |  |  |  |  |  |  | R/W |  |  |  |
| Reset |  |  |  |  | 0h |  |  |  |  |  |  |  | 0h |  |  |  |

RXTIDE	Trigger point of RXTIDE interrupt

TXTIDE	Trigger point of TXTIDE interrupt

SIMn +0030h	Data Register Used As Tx/Rx Data Register	SIMIFN_SIM_DATA

| bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | DATA[7:0] |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  |

DATA  Eight data digits, corresponding to the character being read or written

SIMn +0034h	SIM FIFO Count Register	SIMIFN_SIM_COUNT

| bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  | COUNT[4:0] |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  | 0h |  |  |  |  |

COUNT	Number of characters in the SIM FIFO when read and flushes when written.

SIMn +0040h	SIM Activation Time Register	SIMIFN_SIM_ATIME

| bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  | ATIME[9:0] |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  | R/W |  |  |  |  |  |  |  |  |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Reset |  |  |  |  |  |  | 2BEh |
| --- | --- | --- | --- | --- | --- | --- | --- |

ATIME	Defines the duration, in 64 SIM clock cycles, of the time taken for each of the three stages of the card activation process, from SIMON transiting to “high” to turning on VCC, from turning on VCC to pull data “high” and then from pulling data “high” to turning on CLK.

SIMn +0044h	SIM Deactivation Time Register	SIMIFN_SIM_DTIME

| bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  | DTIME[5:0] |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  | Fh |  |  |  |  |  |

DTIME	Defines the duration, in 64 13 MHz clock cycles, of the time taken for each of the three stages of the card deactivation sequence, from pulling RST “low” to turning off CLK, from turning off CLK to pulling data “low”, from pulling data “low” to turning off VCC.

SIMn +0048h	Character to Character Waiting Time Register	SIMIFN_SIM_TOUT

| bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  | WTIME[21:0] |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  | 260h |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | WTIME[21:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 260h |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

WTIME	Maximum interval between the leading edge of two consecutive characters in 16 ETU units

SIMn +004Ch	Block to Block Guard Time Register	SIMIFN_SIM_GTIME

| bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | GTIME[3:0] |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 10d |  |  |  |

GTIME	Minimum interval between the leading edge of two consecutive characters sent in opposite directions in ETU unit

SIMn +0050h	Block to Error Signal Time Register	SIMN_SIM_ETIME

| bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  | ETIME[5:0] |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  | 15d |  |  |  |  |  |

ETIME	Defines the interval, in 1/16 ETU unit, between the end of the transmitted parity bit and the time to check the parity error signal sent from SIM card.

SIMn +0054h	Active High Period Control Register

SIMIFN_SIM_EXT_TIM

E

| bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | EXT_TIME[3:0] |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 1d |  |  |  |

EXT_TIME Defines the interval, in 1/16 ETU unit, between the end of the transmitted parity bit and the time to switch SIO to input mode. This value should be smaller than ETIME.

SIMn +0058h	Character to Character Guard Time Register	SIMIFN_SIM_CGTIME

| bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | CGTIME[7:0] |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 2h |  |  |  |  |  |  |  |

CGTIME		Defines the minimum interval between the leading edges of two consecutive characters in ETU unit.

In the same transmission direction, the minimum interval is (12 + CGTIME) ETU. In opposite transmission direction, the minimum interval is (12 + CGTIME + GTIME) ETU.

SIMn +0060h	SIM Command Header Register: INS	SIMIFN_SIM_INS

| bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  | INSD | SIMINS[7:0] |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  | R/W | R/W |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  | 0h | 0h |  |  |  |  |  |  |  |

SIMINS	This field should be identical to the INS instruction code. When writing to this register, the T = 0 controller will be activated and data transfer initiated.

INSD	Instruction direction

- T = 0 controller receives data from the SIM card.

- T = 0 controller sends data to the SIM card.

SIMn +0064h	SIM Command Header Register: P3

SIMIFN_SIM_IMP3

(ICC_LEN)

| bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  | SIMP3 [8] | SIMP3[7:0] |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  | R | R/W |  |  |  |  |  |  |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Reset |  |  |  |  |  |  |  | 0h | 0h |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

SIMP3	This field should be identical to the P3 instruction code. It should be written prior to the SIM_INS register. When the data transfer is being conducted, this field will show the number of the remaining data to be sent or to be received.

SIMn +0068h	SIM Procedure Byte Register: SW1

SIMIFN_SIM_SW1

(ICC_LEN)

| bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | SIMSW1[7:0] |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | R |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0h |  |  |  |  |  |  |  |

SIMSW1	This field holds the last received procedure byte for debugging. When the T0END interrupt occurrs, it will keep the SW1 procedure byte.

SIMn +006Ch	SIM Procedure Byte Register: SW2

SIMIFN_SIM_SW2

(ICC_EDC)

| bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | SIMSW2[7:0] |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | R |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0h |  |  |  |  |  |  |  |

SIMSW2	This field holds the SW2 procedure byte

SIMn +0070h	SIM ATR State Register

SIMIFN_SIM_ATRST

A

| bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  | AL | IR |  |  |  |  |  |  | OFF |
| Type |  |  |  |  |  |  |  | R | R |  |  |  |  |  |  | R |
| Reset |  |  |  |  |  |  |  | 0h | 0h |  |  |  |  |  |  | 1h |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

The SIM card is initially turned off. After configuring SIMON of SIMn_SIM_CTRL and ATR procedure, SIMn_SIM_ATRSTA will set IR or AL to 1 to indicate the card’s feature.

OFF	Indicates On/Off of the SIM card

IR	SIM card is IR (internal reset) card

AL	SIM card is AL (active low reset) card

SIMn +0074h	SIM Protocol State Register	SIMIFN_SIM_STATUS

| bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  | ALL | ONE |  |  |  | IDLE |
| Type |  |  |  |  |  |  |  |  |  |  | R | R |  |  |  | R |
| Reset |  |  |  |  |  |  |  |  |  |  | 0h | 0h |  |  |  | 1h |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

T0 or T1 protocol of the SIM card is initially turned off. When T0 or T1 protocol is turned on, SIMn_SIM_T0STA will transit between ONE or ALL according to the procedure byte of the SIM card.

IDLE	SIM card’s T0 or T1 protocol is active or idle.

ONE	SIM card will send the next byte

ALL	SIM card will send all the remaining bytes.

SIMn +0080h	Data Register Used As Tx/Rx Data Register	SIMIFN_SIM_DMADATA

| bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | DATA[7:0] |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  |

DATA	Eight data digits, corresponding to the character being read or written

SIMn +0090h	SIM Module Debug Register	SIMIFN_SIM_DBG

| bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  | DBG7 | DBG6 | DBG5 | DBG4 |  |  |  | DBG3[4:0] |  |  |  |  |
| Type |  |  |  |  | R | R | R | R |  |  |  | R |  |  |  |  |
| Reset |  |  |  |  | 0h | 0h | 0h | 0h |  |  |  | 0h |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  | DBG2[4:0] |  |  |  |  |  |  |  | DBG1[4:0] |  |  |  |  |
| Type |  |  |  | R |  |  |  |  |  |  |  | R |  |  |  |  |
| Reset |  |  |  | 0h |  |  |  |  |  |  |  | 0h |  |  |  |  |

DBG1 Debugging register 1 DBG2 Debugging register 2 DBG3 Debugging register 3 DBG4 Debugging register 4 DBG5 Debugging register 5 DBG6 Debugging register 6 DBG7  Debugging register 7

SIMn +0094h	SIM FIFO Data Debug Register	SIMIFN_SIM_DBGDATA

| bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  | DBGRPTR[3:0] |  |  |  | DBGDATA[7:0] |  |  |  |  |  |  |  |
| Type |  |  |  |  | R |  |  |  | R |  |  |  |  |  |  |  |
| Reset |  |  |  |  | 0h |  |  |  | - |  |  |  |  |  |  |  |

DBGDATA	FIFO data debugging register

There is no impact on data transmission when this register is read.

DBGRPTR	FIFO read pointer related to DBGDATA

Automatically increases by 1 after this register is read.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

SIMn +00A0h  SIM SCLK PAD Control Register	SIMIFN_SIM_SCLK

| bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DEBU G | ACD_ FUN C |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0h | 0h |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | IES_ CTRL | IES_L V |  |  | TDSEL[1:0] |  | RDSEL[1:0] |  | R1 | R0 | PUPD | SMT | E4 | E2 | SR[1:0] |  |
| Type | R/W | R/W |  |  | R/W |  | R/W |  | R/W | R/W | R/W | R/W | R/W | R/W | R/W |  |
| Reset | 0h | 0h |  |  | 0h |  | 0h |  | 0h | 0h | 0h | 1h | 0h | 1h | 3h |  |

SR	Output slew rate control

High asserted. SR = 1, slower slew. SR = 0, no slew rate control.

For SIM card mode, SR[1:0] = [1 1] is the recommended setting to eliminate overshooting/undershooting. For non-SIM card mode, SR[1:0] = [0 0] is set for best speed.

E2	TX driving strength control

For SIM card mode, E2 = [1] is the recommended setting for SCLK/SRST/SIO. (SIO/SRST can use [0])

E4	TX driving strength control

For SIM card mode, E4 = [0] is the recommended setting for SCLK/SRST/SIO.

SMT	RX input buffer schmit trigger hysteresis control enable High asserted. SMT = 1, schmit trigger enable.

For SIM card mode, SMT = [1] is the recommended setting.

PUPD Weak pull-up/pull-down control

- Pull-up

- Pull-down

R0	Weak pull-up/pull-down resistance select Check the table in register “R1”.

R1	Weak pull-up/pull-down resistance select Check the following table.

| E | PUPD | R1 | R0 | R Value |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | High – Z |
| 0 | 0 | 0 | 1 | PU – 20k |
| 0 | 0 | 1 | 0 | PU – 5k |
| 0 | 0 | 1 | 1 | PU – 4k |
| 0 | 1 | 0 | 0 | High – Z |
| 0 | 1 | 0 | 1 | PD – 75k |
| 0 | 1 | 1 | 0 | PD – 75k |
| 0 | 1 | 1 | 1 | PD – 37.5k |
| 1 | x | x | x | High - Z |

RDSEL	Selects RX duty

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

RDSEL[0]: Input buffer duty high when asserted. (high pulse width adjustment) RDSEL[1]: Input buffer duty low when asserted. (low pulse width adjustment) For SIM card mode, RDSEL = [0 0] is the recommended setting.

TDSEL	Selects TX duty

TDSEL[0]: Output level shifter duty high when asserted. (high pulse width adjustment) TDSEL[1]: Output level shifter duty low when asserted. (low pulse width adjustment) For SIM card mode, TDSEL = [0 0] is the recommended setting.

IES_LV	Controls IES (RX input buffer enable) parking level in IES direct control mode High asserted. Datapath: From IO to O. IES = 0, O = 0.

In quiescent mode, IES = 0 is suggested for power saving.

IES_CTRL		Enables IES direct control mode ACD_FUNC		ACD function mode for analog designer DEBUG	Output PAD related signals for monitoring

  - Disable

  - Enable

SIMn +00A4h  SIM SRST PAD Control Register	SIMIFN_SIM_SRST

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | IES_ CTRL | IES_L V |  |  | TDSEL[1:0] |  | RDSEL[1:0] |  | R1 | R0 | PUPD | SMT | E4 | E2 | SR[1:0] |  |
| Type | R/W | R/W |  |  | R/W |  | R/W |  | R/W | R/W | R/W | R/W | R/W | R/W | R/W |  |
| Reset | 0h | 0h |  |  | 0h |  | 0h |  | 0h | 0h | 0h | 1h | 0h | 1h | 3h |  |

SR	Output slew rate control

High asserted. SR = 1, slower slew. SR = 0, no slew rate control.

For SIM card mode, SR[1:0] = [1 1] is the recommended setting to eliminate overshooting/undershooting. For non-SIM card mode, SR[1:0] = [0 0] is set for best speed.

E2	TX driving strength control

For SIM card mode, E2 = [1] is the recommended setting for SCLK/SRST/SIO. (SIO/SRST can use [0])

E4	TX driving strength control

For SIM card mode, E4 = [0] is the recommended setting for SCLK/SRST/SIO.

SMT	RX input buffer schmit trigger hysteresis control enable High asserted. SMT = 1, schmit trigger enable.

For SIM card mode, SMT = [1] is the recommended setting.

PUPD Weak pull-up/pull-down control

- Pull-up

- Pull-down

R0	Weak pull-up/pull-down resistance select Check the table in register “R1”.

R1	Weak pull-up/pull-down resistance select Check the following table.

| E | PUPD | R1 | R0 | R Value |
| --- | --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| E | PUPD | R1 | R0 | R Value |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | High – Z |
| 0 | 0 | 0 | 1 | PU – 20k |
| 0 | 0 | 1 | 0 | PU – 5k |
| 0 | 0 | 1 | 1 | PU – 4k |
| 0 | 1 | 0 | 0 | High – Z |
| 0 | 1 | 0 | 1 | PD – 75k |
| 0 | 1 | 1 | 0 | PD – 75k |
| 0 | 1 | 1 | 1 | PD – 37.5k |
| 1 | x | x | x | High - Z |

RDSEL	Selects RX duty

RDSEL[0]: Input buffer duty high when asserted. (high pulse width adjustment) RDSEL[1]: Input buffer duty low when asserted. (low pulse width adjustment) For SIM card mode, RDSEL = [0 0] is the recommended setting.

TDSEL	Selects TX duty

TDSEL[0]: Output level shifter duty high when asserted. (high pulse width adjustment) TDSEL[1]: Output level shifter duty low when asserted. (low pulse width adjustment) For SIM card mode, TDSEL = [0 0] is the recommended setting.

IES_LV	Controls IES (RX input buffer enable) parking level in IES direct control mode High asserted. Datapath: From IO to O. IES = 0, O = 0.

In quiescent mode, IES = 0 is suggested for power saving.

IES_CTRL	Enables IES direct control mode

SIMn +00A8h  SIM SIO PAD Control Register	SIMIFN_SIM_SIO

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | IES_ CTRL | IES_L V |  |  | TDSEL[1:0] |  | RDSEL[1:0] |  | R1 | R0 | PUPD | SMT | E4 | E2 | SR[1:0] |  |
| Type | R/W | R/W |  |  | R/W |  | R/W |  | R/W | R/W | R/W | R/W | R/W | R/W | R/W |  |
| Reset | 0h | 0h |  |  | 0h |  | 0h |  | 1h | 0h | 0h | 1h | 0h | 1h | 3h |  |

SR	Output slew rate control

High asserted. SR = 1, slower slew. SR = 0, no slew rate control.

For SIM card mode, SR[1:0] = [1 1] is the recommended setting to eliminate overshooting/undershooting. For non-SIM card mode, SR[1:0] = [0 0] is set for best speed.

E2	TX driving strength control

For SIM card mode, E2 = [1] is the recommended setting for SCLK/SRST/SIO. (SIO/SRST can use [0])

E4	TX driving strength control

For SIM card mode, E4 = [0] is the recommended setting for SCLK/SRST/SIO.

SMT	RX input buffer schmit trigger hysteresis control enable High asserted. SMT = 1, schmit trigger enable.

For SIM card mode, SMT = [1] is the recommended setting.

PUPD Weak pull-up/pull-down control

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Pull-up

- Pull-down

R0 Weak pull-up/pull-down resistance select Check the table in register “R1”.

R1 Weak pull-up/pull-down resistance select Check the following table.

For SIO, [R1 R0] = [1 0] is the recommended setting for 5k weak pull-up. In 4 SIM application and SIO is connected to external SIM switch, please disable pull-up resistance. ([R1 R0] = [0 0])

| E | PUPD | R1 | R0 | R Value |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | High – Z |
| 0 | 0 | 0 | 1 | PU – 20k |
| 0 | 0 | 1 | 0 | PU – 5k |
| 0 | 0 | 1 | 1 | PU – 4k |
| 0 | 1 | 0 | 0 | High – Z |
| 0 | 1 | 0 | 1 | PD – 75k |
| 0 | 1 | 1 | 0 | PD – 75k |
| 0 | 1 | 1 | 1 | PD – 37.5k |
| 1 | x | x | x | High - Z |

RDSEL	Selects RX duty

RDSEL[0]: Input buffer duty high when asserted. (high pulse width adjustment) RDSEL[1]: Input buffer duty low when asserted. (low pulse width adjustment) For SIM card mode, RDSEL = [0 0] is the recommended setting.

TDSEL	Selects TX duty

TDSEL[0]: Output level shifter duty high when asserted. (high pulse width adjustment) TDSEL[1]: Output level shifter duty low when asserted. (low pulse width adjustment) For SIM card mode, TDSEL = [0 0] is the recommended setting.

IES_LV	Controlling IES (RX input buffer enable) parking level in IES direct control mode High asserted. Datapath: From IO to O. IES = 0, O = 0.

In quiescent mode, IES = 0 is suggested for power saving.

IES_CTRL	Enables IES direct control mode

SIMn +00ACh SIM Monitor Register	SIMIFN_SIM_MON

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | MON1 2 | MON1 1 | MON1 0 |  | MON9 | MON8 | MON7 |  | MON6 | MON5 | MON4 |  | MON3 | MON2 | MON1 |
| Type |  | R | R | R |  | R | R | R |  | R | R | R |  | R | R | R |
| Reset |  | 0h | 0h | 0h |  | 0h | 0h | 0h |  | 0h | 0h | 0h |  | 0h | 0h | 0h |

MON1	Monitor signal 1

MON2	Monitor signal 2

MON3	Monitor signal 3

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

MON4	Monitor signal 4

MON5	Monitor signal 5

MON6	Monitor signal 6

MON7	Monitor signal 7

MON8	Monitor signal 8

MON9	Monitor signal 9 MON10	Monitor signal 10 MON11	Monitor signal 11 MON12	Monitor signal 12

SIMn+00B0h	SIM Test Select	SIMIFN_SIM_SEL

| bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  | SIMSEL |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 3’b001 |  |  |

| SIMSEL | Selects | monitor SIMRST, SIMCLK, SIMIO input signal |
| --- | --- | --- |
|  | 001 | SIMRST input is monitored. |
|  | 010 | SIMCLK input is monitored. |
|  | 100 | SIMSIO input is monitored. |

Others No meaning

##### SIM Card Insertion and Removal

The detection of physical connection to the SIM card and card removal can be done by the external interrupt controller or by GPIO.

##### Card activation and Deactivation

The card activation and deactivation sequence are both controlled by H/W. The MCU initiates the activation sequence by writing “1” to bit 0 of the SIM_CTRL register, and then the interface performs the following activation sequence:

- Assert SIMRST ”low”

- Set SIMVCC at “high” level and SIMDATA in the reception mode

- Enable SIMCLK clock

- De-assert SIMRST “high” (required if it belongs to active low reset SIM card)

The final step in a typical card session is contacting deactivation in case the card will be electrically damaged. The deactivation sequence is initiated by writing “0” to bit 0 of the SIM_CTRL register, and the interface will perform the following deactivation sequence:

- Assert SIMRST ”low”

- Set SCIMCLK at “low” level

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Set SIMDATA at “low” level

- Set SIMVCC at “low” level

##### Answering to Reset Sequence

After the card is activated, a reset operation will result in an answer from the card consisting of the initial character TS, followed by maximum 32 characters. The initial character TS provides a bit synchronization sequence and defines the conventions to interpret data bytes in all subsequent characters.

On reception of the first character, TS, the MCU should read this character, establish the respective required convention and re-program the related registers. These processes should be completed prior to the completion of reception of the next character. Next, the remainder of the ATR sequence will be received, read via the SIM_DATA in the selected convention and interpreted by the S/W.

The timing requirement and procedures for ATR sequence are handled by H/W and shall meet the requirement of ISO 7816-3, as shown in Figure 23.

Figure 23. Answering to reset sequence

Table 43. Time-out condition for answering to reset sequence

| Time | Value | Comment |
| --- | --- | --- |
| T1 | > 400 SIMCLK | SIMCLK start to ATR appears |
| T2 | < 200 SIMCLK | SIMCLK start to SIMDATA in reception mode |
| T3 | > 40,000 SIMCLK | SIMCLK start to SIMRST ”high” |
| T4 | - | SIMVCC “high” to SIMCLK start |
| T5 | - | SIMRST “low” to SIMCLK stop |
| T6 | - | SIMCLK stop to SIMDATA ”low” |
| T7 | - | SIMDATA “low” to SIMVCC ”low” |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

SIM Data Transfer

There are two transfer modes provided, in software controlled byte by byte fashion or in a block fashion using T=0 controller and DMA controller. In both modes, the time-out counter can be enabled to monitor the elapsed time between two consecutive bytes.

###### Byte Transfer Mode

This mode is used during ATR and PPS procedure. In this mode, the SIM interface only ensures error free character transmission and reception.

Receiving characters

Upon detection of the start-bit sent by SIM card, the interface transforms into reception mode and the following bits are shifted into an internal register. If no parity error is detected or the character-received handshaking is disabled, the received-character will be written into the SIM FIFO and the SIM_COUNT register increased by one. Otherwise, the SIMDATA line will be held “low” at 0.5 etu after detecting the parity error for 1.5 etus, and the character will be re-received. If a character fails to be received correctly for the RXRETRY times, the receive-handshaking will be aborted, the last-received character written into the SIM FIFO, the SIM_COUNT increased by one and the RXERR interrupt generated.

When the number of characters held in the received FIFO exceeds the level defined in the SIM_TIDE register, a RXTIDE interrupt will be generated. The number of characters held in the SIM FIFO can be determined by reading the SIM_COUNT register, and writing to this register will flush the SIM FIFO.

Sending characters

Characters that are to be sent to the card are first written into the SIM FIFO and then automatically transmitted to the card at timed intervals. If character-transmitted handshaking is enabled, the SIMDATA line will be sampled at 1 etu after the parity bit. If the card indicates that it does not receive the character correctly, the character will be re-transmitted for maximum of TXRETRY times before a TXERR interrupt is generated and the transmission is aborted. Otherwise, the succeeding byte in the SIM FIFO will be transmitted.

If a character fails to be transmitted and a TXERR interrupt is generated, the interface will need to be reset by flushing the SIM FIFO before any subsequent transmission or reception operation.

When the number of characters held in the SIM FIFO falls below the level defined in the SIM_TIDE register, a TXTIDE interrupt will be generated. The number of characters held in the SIM FIFO can be determined by reading the SIM_COUNT register, and writing to this register will flush the SIM FIFO.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Block Transfer Mode

Basically the SIM interface is designed to work in conjunction with the T=0 protocol controller and the DMA controller during non-ATR and non-PPS phase, though it is still possible for software to service the data transfer manually as in the byte transfer mode if necessary. Thus the T=0 protocol should be controlled by software.

The T=0 controller can be accessed via four registers representing the instruction header bytes INS and P3, and the procedure bytes SW1 and SW2. The registers are:

- SIM_INS, SIM_P3

- SIM_SW1, SIM_SW2

During the character transfer, SIM_P3 holds the number of characters to be sent or to be received, and SIM_SW1 holds the last received procedure byte including NULL, ACK, NACK and SW1 for debugging.

Data receiving instruction

Data receiving instructions receive data from the SIM card. See the following instantiated procedure.

- Enable the T=0 protocol controller by setting the T0EN bit to 1 in the SIM_CONF register.

- Program the SIM_TIDE register to 0x0000 (TXTIDE = 0, RXTIDE = 0).

- Program the SIM_IRQEN to 0x019C (enable RXERR, TXERR, T0END, TOUT and OVRUN interrupts).

- Write CLA, INS, P1, P2 and P3 into SIM FIFO.

- Program the DMA controller:

  - DMAn_MSBSRC and DMAn_LSBSRC: Address of the SIM_DATA register

  - DMAn_MSBDST and DMAn_LSBDST: Memory address reserved to store the received characters

  - DMAn_COUNT: Identical to P3 or 256 (if P3 = 0)

  - DMAn_CON: 0x0078

- Write P3 into the SIM_P3 register and then INS into SIM_INS register. (Data transfer is initiated now.)

- Enable the time-out counter by setting the TOUT bit to 1 in the SIM_CONF register.

- Start the DMA controller by writing 0x8000 into the DMAn_START register.

Upon completion of the data receiving instruction, T0END interrupt will be generated and the time-out counter should be disabled by setting the TOUT bit to 0 in the SIM_CONF register.

If error occurs during the data transfer (RXERR, TXERR, OVRUN or TOUT interrupt is generated), the SIM card should be deactivated first and then activate the prior subsequent operations.

Data sending instruction

Data sending instructions send data to the SIM card. See the following instantiated procedure.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Enable the T=0 protocol controller by setting the T0EN bit to 1 in the SIM_CONF register.

- Program the SIM_TIDE register to 0x0100 (TXTIDE = 1, RXTIDE = 0)

- Program the SIM_IRQEN to 0x019C (enable RXERR, TXERR, T0END, TOUT and OVRUN interrupts)

- Write CLA, INS, P1, P2 and P3 into SIM FIFO

- Program the DMA controller:

  - DMAn_MSBSRC and DMAn_LSBSRC: Memory address reserved to store the transmitted characters

  - DMAn_MSBDST and DMAn_LSBDST: Address of the SIM_DATA register

  - DMAn_COUNT: Identical to P3

  - DMAn_CON: 0x0074

- Write P3 into the SIM_P3 register and then (0x0100 | INS) into SIM_INS register. (Data transfer is initiated now.)

- Enable the time-out counter by setting the TOUT bit to 1 in the SIM_CONF register.

- Start the DMA controller by writing 0x8000 into the DMAn_START register.

Upon completion of the data sending instruction, T0END interrupt will be generated and the time-out counter should be disabled by setting the TOUT bit back to 0 in the SIM_CONF register.

If error occurs during the data transfer (RXERR, TXERR, OVRUN or TOUT interrupt is generated), the SIM card should be deactivated first and then activate the prior subsequent operations.

### Keypad Scanner

##### General Description

The keypad supports two types of keypads: 5*5 double keypad and 5*5 triple keypad.

The 5*5 keypad can be divided into two parts: 1) The keypad interface including 5 columns and 5 rows (see Figure 24 and Figure 25); 2) The key detection block providing key pressed, key released and

de-bounce mechanisms.

Each time the key is pressed or released, i.e. something different in the 5*5 matrix, the key detection block senses the change and recognizes if a key has been pressed or released. Whenever the key status changes and is stable, a KEYPAD IRQ will be issued. The MCU can then read the key(s) pressed directly in the KP_MEM1, KP_MEM2, KP_MEM3, KP_MEM4 and KP_MEM5 registers. To ensure the key pressed information is not missed, the status register in keypad is not read-cleared by the APB read command. The status register can only be changed by the key-pressed detection FSM.

This keypad can detect one or two keys pressed simultaneously. Figure 27 shows the one key pressed condition. Figure 28(a) and Figure 28(b) illustrate the two keys pressed cases. Since the key pressed detection depends on the HIGH or LOW level of the external keypad interface, if the keys are pressed at the same time and there exists a key that is on the same column and the same row with other keys, the pressed key cannot be correctly decoded. For example, if there are three key pressed: key1 = (x1,

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

y1), key2 = (x2, y2), and key3 = (x1, y2), then both key3 and key4 = (x2, y1) will be detected, and therefore they cannot be distinguished correctly. Hence, the keypad can detect only one or two keys pressed simultaneously at any combination. More than two keys pressed simultaneously in a specific pattern will retrieve the wrong information.

The 5*5 double keypad (Figure 24) supports a 5*5*2 = 50 keys matrix. The 50 keys are divided into 25 sub groups and each group consists of 2 keys and a off-chip resistor. 5*5 double keypad has another limitation, which is it cannot detect two keys pressed simultaneously when the two keys are in one group, i.e. the 5*5 keypad cannot detect key0 and key1 pressed simultaneously or key15 and key16 pressed simultaneously.

The 5*5 triple keypad (Figure 25) supports a 5*5*3 = 75 keys matrix. The 75 keys are divided into 25 sub groups and each group consists of 3 keys and two off-chip resistors. 5*5 triple keypad has another limitation, which is it cannot detect three keys pressed simultaneously when the three keys are in one group, i.e. 5*5 keypad cannot detect key0, key1 and key2 pressed simultaneously or key15, key16 and key17 pressed simultaneously.

KCOL0	KCOL1	KCOL2	KCOL3	KCOL4

KROW0

0

3

6

9

12

1

4

7

10

13

KROW1

15

18

21

24

27

16

19

22

25

28

KROW2

30

33

36

39

42

31

34

37

40

43

KROW3

45

48

51

54

57

46

49

52

55

58

KROW4

60

63

66

69

72

61

64

67

70

73

Baseband

Figure 24. 5x5 double keypad matrix (50 keys)

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

KCOL0	KCOL1

KCOL2

KCOL3

KCOL4

KROW0

0

3

6

9

12

1

4

7

10

13

2

5

8

11

14

KROW1

15

18

21

24

27

16

19

22

25

28

17

20

23

26

29

KROW2

30

33

36

39

42

31

34

37

40

43

32

35

38

41

44

KROW3

45

48

51

54

57

46

49

52

55

58

47

50

53

56

59

KROW4

60

63

66

69

72

61

64

67

70

73

62

65

68

71

74

Baseband

Figure 25. 5x5 triple keypad matrix (75 keys)

Figure 26. 5*5 double keypad scan waveform

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Key Pressed

De-bounce time	De-bounce time

Key-pressedStatus

KP_IRQ

KEY_PRESS_IRQ	KEY_RELEASE_IRQ

Figure 27. One key pressed with de-bounce mechanism denoted

Key1 pressed

Key2 pressed

Status IRQ

Key1 pressed

Key2 pressed

Key1 released

Key2 released

(a)

Key1 pressed

Key2 pressed

Status IRQ

Key1 pressed

Key2 pressed

Key2 released

Key1 released

(b)

Figure 28. (a) Two keys pressed, case 1; (b) Two keys pressed, case 2

##### Register Definitions

Module name: KP Base address: (+A00D0000)

| Address | Name | Width | Register Function |
| --- | --- | --- | --- |
| A00D0000 | KP_STA | 16 | Keypad Status |
| A00D0004 | KP_MEM1 | 16 | Keypad Scanning Output Register Shows the key-pressed status of key 0 (LSB) ~ key 15. Refer to Table 1 and Table 2. |
| A00D0008 | KP_MEM2 | 16 | Keypad Scanning Output Register Shows the key-pressed status of key 16 (LSB) ~ key 31. Refer to Table 1 and Table 2. |
| A00D000C | KP_MEM3 | 16 | Keypad Scanning Output Register Shows the key-pressed status of key 32 (LSB) ~ key 47. Refer to Table 1 and Table 2. |
| A00D0010 | KP_MEM4 | 16 | Keypad Scanning Output Register Shows the key-pressed status of key 48 (LSB) ~ key 63. Refer to Table 1 and Table 2. |
| A00D0014 | KP_MEM5 | 16 | Keypad Scanning Output Register Shows the key-pressed status of key 64 (LSB) ~ key 77. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Address | Name | Width | Register Function |
| --- | --- | --- | --- |
|  |  |  | Refer to Table 1 and Table 2. |
| A00D0018 | KP_DEBOUNCE | 16 | De-bounce Period Setting Defines the waiting period before key pressing or release events are considered stable. If the de-bounce setting is too small, the keypad will be too sensitive and detect too many unexpected key presses. The suitable de-bounce time setting must be adjusted according to the user's habit. |
| A00D001C | KP_SCAN_TIMING | 16 | Keypad Scan Timing Adjustment Register Sets up the keypad scan timing. Note: ROW_SCAN_DIV > ROW_INTERVAL_DIV and COL_SCAN_DIV > COL_INTERVAL_DIV. ROW_INTERVAL_DIV/COL_INTERVAL_DIV are used to lower the power consumption for it decreases the actual scan number during the de-bounce time. |
| A00D0020 | KP_SEL | 16 | Keypad Selection Register Selects: 1: Use the double keypad or triple keypad 2: Which cols and rows are used |
| A00D0024 | KP_EN | 16 | Keypad Enable Register Enables/Disables keypad. |

A00D0000	KP_STA	Keypad Status	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | STA |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RO |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 0 | STA | STA | Indicates the keypad status The register is not cleared by the read operation. 0: No key pressed 1: Key pressed |

A00D0004	KP_MEM1	Keypad Scanning Output Register	FFFF

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | KEY1 5 | KEY14 | KEY13 | KEY12 | KEY11 | KEY10 | KEY9 | KEY8 | KEY7 | KEY6 | KEY5 | KEY4 | KEY3 | KEY2 | KEY1 | KEY0 |
| Type | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO |
| Reset | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Overview: Shows the key-pressed status of key 0 (LSB) ~ key 15. Refer to Table 1 and Table 2.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | KEY15 | KEY15 |  |
| 14 | KEY14 | KEY14 |  |
| 13 | KEY13 | KEY13 |  |
| 12 | KEY12 | KEY12 |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 11 | KEY11 | KEY11 |  |
| 10 | KEY10 | KEY10 |  |
| 9 | KEY9 | KEY9 |  |
| 8 | KEY8 | KEY8 |  |
| 7 | KEY7 | KEY7 |  |
| 6 | KEY6 | KEY6 |  |
| 5 | KEY5 | KEY5 |  |
| 4 | KEY4 | KEY4 |  |
| 3 | KEY3 | KEY3 |  |
| 2 | KEY2 | KEY2 |  |
| 1 | KEY1 | KEY1 |  |
| 0 | KEY0 | KEY0 |  |

A00D0008	KP_MEM2	Keypad Scanning Output Register	FFFF

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | KEY3 1 | KEY30 | KEY29 | KEY28 | KEY27 | KEY26 | KEY25 | KEY24 | KEY23 | KEY22 | KEY21 | KEY20 | KEY19 | KEY18 | KEY17 | KEY1 6 |
| Type | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO |
| Reset | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Overview: Shows the key-pressed status of key 16 (LSB) ~ key 31. Refer to Table 1 and Table 2.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | KEY31 | KEY31 |  |
| 14 | KEY30 | KEY30 |  |
| 13 | KEY29 | KEY29 |  |
| 12 | KEY28 | KEY28 |  |
| 11 | KEY27 | KEY27 |  |
| 10 | KEY26 | KEY26 |  |
| 9 | KEY25 | KEY25 |  |
| 8 | KEY24 | KEY24 |  |
| 7 | KEY23 | KEY23 |  |
| 6 | KEY22 | KEY22 |  |
| 5 | KEY21 | KEY21 |  |
| 4 | KEY20 | KEY20 |  |
| 3 | KEY19 | KEY19 |  |
| 2 | KEY18 | KEY18 |  |
| 1 | KEY17 | KEY17 |  |
| 0 | KEY16 | KEY16 |  |

A00D000C	KP_MEM3	Keypad Scanning Output Register	FFFF

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Name | KEY4 7 | KEY46 | KEY45 | KEY44 | KEY43 | KEY42 | KEY41 | KEY40 | KEY39 | KEY38 | KEY37 | KEY36 | KEY35 | KEY34 | KEY33 | KEY3 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO |
| Reset | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Overview: Shows the key-pressed status of key 32 (LSB) ~ key 47. Refer to Table 1 and Table 2.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | KEY47 | KEY47 |  |
| 14 | KEY46 | KEY46 |  |
| 13 | KEY45 | KEY45 |  |
| 12 | KEY44 | KEY44 |  |
| 11 | KEY43 | KEY43 |  |
| 10 | KEY42 | KEY42 |  |
| 9 | KEY41 | KEY41 |  |
| 8 | KEY40 | KEY40 |  |
| 7 | KEY39 | KEY39 |  |
| 6 | KEY38 | KEY38 |  |
| 5 | KEY37 | KEY37 |  |
| 4 | KEY36 | KEY36 |  |
| 3 | KEY35 | KEY35 |  |
| 2 | KEY34 | KEY34 |  |
| 1 | KEY33 | KEY33 |  |
| 0 | KEY32 | KEY32 |  |

A00D0010	KP_MEM4	Keypad Scanning Output Register	FFFF

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | KEY6 3 | KEY62 | KEY61 | KEY60 | KEY59 | KEY58 | KEY57 | KEY56 | KEY55 | KEY54 | KEY53 | KEY52 | KEY51 | KEY50 | KEY49 | KEY4 8 |
| Type | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO |
| Reset | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Overview: Shows the key-pressed status of key 48 (LSB) ~ key 63. Refer to Table 1 and Table 2.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | KEY63 | KEY63 |  |
| 14 | KEY62 | KEY62 |  |
| 13 | KEY61 | KEY61 |  |
| 12 | KEY60 | KEY60 |  |
| 11 | KEY59 | KEY59 |  |
| 10 | KEY58 | KEY58 |  |
| 9 | KEY57 | KEY57 |  |
| 8 | KEY56 | KEY56 |  |
| 7 | KEY55 | KEY55 |  |
| 6 | KEY54 | KEY54 |  |
| 5 | KEY53 | KEY53 |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 4 | KEY52 | KEY52 |  |
| 3 | KEY51 | KEY51 |  |
| 2 | KEY50 | KEY50 |  |
| 1 | KEY49 | KEY49 |  |
| 0 | KEY48 | KEY48 |  |

A00D0014	KP_MEM5	Keypad Scanning Output Register	07FF

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  | KEY74 | KEY73 | KEY72 | KEY71 | KEY70 | KEY69 | KEY68 | KEY67 | KEY66 | KEY65 | KEY6 4 |
| Type |  |  |  |  |  | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO |
| Reset |  |  |  |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Overview: Shows the key-pressed status of key 64 (LSB) ~ key 77. Refer to Table 1 and Table 2.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 10 | KEY74 | KEY74 |  |
| 9 | KEY73 | KEY73 |  |
| 8 | KEY72 | KEY72 |  |
| 7 | KEY71 | KEY71 |  |
| 6 | KEY70 | KEY70 |  |
| 5 | KEY69 | KEY69 |  |
| 4 | KEY68 | KEY68 |  |
| 3 | KEY67 | KEY67 |  |
| 2 | KEY66 | KEY66 |  |
| 1 | KEY65 | KEY65 |  |
| 0 | KEY64 | KEY64 |  |

The five registers list the status of 75 keys on the keypad. For 5*5 keypad, KP_MEM1~4 registers list the status of 75 keys on the keypad. When the MCU receives KEYPAD IRQ, both two registers must be read. If any key is pressed, the relative bit will be set to 0.

In order to work normally, the corresponding pull-up/down setting must be programmed correctly. If some keys can be used because their COL or ROW is used as GPIO, these corresponding enabling bit should be set.

KEYS	Status list of the 75 keys.

A00D0018	KP_DEBOUNCE De-bounce Period Setting	0400

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  | DEBOUNCE |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: Defines the waiting period before key pressing or release events are considered stable. If the

de-bounce setting is too small, the keypad will be too sensitive and detect too many unexpected key presses. The suitable de-bounce time setting must be adjusted according to the user's habit.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 13:0 | DEBOUNCE | DEBOUNCE | De-bounce time = KP_DEBOUNCE/32ms. |

A00D001C	KP_SCAN_TIMI Keypad Scan Timing Adjustment Register	0011

NG

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | COL_INTERVAL_DIV |  |  |  | ROW_INTERVAL_DIV |  |  |  | COL_SCAN_DIV |  |  |  | ROW_SCAN_DIV |  |  |  |
| Type | RW |  |  |  | RW |  |  |  | RW |  |  |  | RW |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |

Overview: Sets up the keypad scan timing. Note: ROW_SCAN_DIV > ROW_INTERVAL_DIV and COL_SCAN_DIV > COL_INTERVAL_DIV. ROW_INTERVAL_DIV/COL_INTERVAL_DIV are used to lower the power consumption for it decreases the actual scan number during the de-bounce time.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:12 | COL_INTER VAL_DIV | COL_INTERVAL_D IV | Sets up the COL SCAN interval cycle, i.e. cycles between two scans Default 0 means there is 1 cycle between two high scan pulses. |
| 11:8 | ROW_INTER VAL_DIV | ROW_INTERVAL_ DIV | Sets up the ROW SCAN interval cycle, i.e. cycles between two scans Default 0 means there is 1 cycle between two high scan pulses. |
| 7:4 | COL_SCAN _DIV | COL_SCAN_DIV | Sets up the COL SCAN cycle which includes COL_INTERVAL_DIV and the high pulse period Default 1 means there are 2 cycles for each scan, including 1 cycle high pulse and 1 cycle interval. |
| 3:0 | ROW_SCAN _DIV | ROW_SCAN_DIV | Sets up the ROW SCAN cycle which includes ROW_INTERVAL_DIV and the high pulse period Default 1 means there are 2 cycles for each scan, including 1 cycle high pulse and 1 cycle interval. |

f32k_ck

Row0_scan

Row1_scan

row_interval_div

= 4'h0

row_scan_div

= 4'h1

Figure 29. kp timing register

| A00D0020 | KP_SEL | Keypad Selection Register | FFC0 |
| --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | KP1_COL_SEL |  |  |  |  |  |  |  |  |  | SAMPLE_DELAY |  |  |  |  | KP_S EL |
| Type | RW |  |  |  |  |  |  |  |  |  | RW |  |  |  |  | RW |
| Reset | 1 | 1 | 1 | 1 | 1 |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: Selects 1: Use the double keypad or triple keypad; 2: Which cols and rows are used.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:11 | KP1_COL_S EL | KP1_COL_SEL | Selects to use which col 0: Disable corresponding column 1: Enable corresponding column |
| 5:1 | SAMPLE_D ELAY | SAMPLE_DELAY | Sets up delay cycles to sample col 0: No delay n: n*31.25ns delay to sample col |
| 0 | KP_SEL | KP_SEL | Selects to use double keypad or triple keypad 0: Use triple keypad 1: Use double keypad |

A00D0024	KP_EN	Keypad Enable Register	0001

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | KP_E N |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |

Overview: Enables/Disables keypad.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 0 | KP_EN | KP_EN | 0: Disable keypad (Both double and triple keypads will not work.) 1: Enable keypad (Only either of double or triple keypads can work.) |

Table 44. 5*5 double KEY’s order number in COL/ROW matrix

|  | COL0 | COL1 | COL2 | COL3 | COL4 |
| --- | --- | --- | --- | --- | --- |
| ROW4 | 60/61 | 63/64 | 66/67 | 69/70 | 72/73 |
| ROW3 | 45/46 | 48/49 | 51/52 | 54/55 | 57/58 |
| ROW2 | 30/31 | 33/34 | 36/37 | 39/40 | 42/43 |
| ROW1 | 15/16 | 18/19 | 21/22 | 24/25 | 27/28 |
| ROW0 | 0/1 | 3/4 | 6/7 | 9/10 | 12/13 |

Table 45. 5*5 triple KEY’s order number in COL/ROW matrix

|  | COL0 | COL1 | COL2 | COL3 | COL4 |
| --- | --- | --- | --- | --- | --- |
| ROW4 | 60/61/62 | 63/64/65 | 66/67/68 | 69/70/71 | 72/73/74 |
| ROW3 | 45/46/47 | 48/49/50 | 51/52/53 | 54/55/56 | 57/58/59 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| ROW2 | 30/31/32 | 33/34/35 | 36/37/38 | 39/40/41 | 42/43/44 |
| --- | --- | --- | --- | --- | --- |
| ROW1 | 15/16/17 | 18/19/20 | 21/22/23 | 24/25/26 | 27/28/29 |
| ROW0 | 0/1/2 | 3/4/5 | 6/7/8 | 9/10/11 | 12/13/14 |

### General Purpose Inputs/Outputs

##### General Description

MT6261D offers 56 general purpose I/O pins. By setting up the control registers, the MCU software can control the direction, the output value, and read the input values on these pins. These GPIOs and GPOs are multiplexed with other functions to reduce the pin count. In addition, all GPO pins are removed. To facilitate application use, the software can configure which clock to send outside the chip. There are 6 clock-out ports embedded in 56 GPIO pins, and each clock-out can be programmed to output appropriate clock source. Besides, when 2 GPIO function for the same peripheral IP, the smaller GPIO serial numbers have higher priority than larger numbers.

Figure 30. GPIO block diagram

##### Register Definitions

Module name: gpio_reg Base address: (+A0020000h)

| Address | Name | Width | Register Function |
| --- | --- | --- | --- |
| A0020000 | GPIO_DIR0 | 32 | GPIO Direction Control Configures GPIO direction |
| A0020004 | GPIO_DIR0_SET | 32 | GPIO Direction Control For bitwise access of GPIO_DIR0 |
| A0020008 | GPIO_DIR0_CLR | 32 | GPIO Direction Control For bitwise access of GPIO_DIR0 |
| A0020010 | GPIO_DIR1 | 32 | GPIO Direction Control Configures GPIO direction |
| A0020014 | GPIO_DIR1_SET | 32 | GPIO Direction Control For bitwise access of GPIO_DIR1 |
| A0020018 | GPIO_DIR1_CLR | 32 | GPIO Direction Control For bitwise access of GPIO_DIR1 |
| A0020100 | GPIO_PULLEN0 | 32 | GPIO Pull-up/down Enable Control Configures GPIO pull enabling |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Address | Name | Width | Register Function |
| --- | --- | --- | --- |
| A0020104 | GPIO_PULLEN0_ SET | 32 | GPIO Pull-up/down Enable Control For bitwise access of GPIO_PULLEN0 |
| A0020108 | GPIO_PULLEN0_ CLR | 32 | GPIO Pull-up/down Enable Control For bitwise access of GPIO_PULLEN0 |
| A0020110 | GPIO_PULLEN1 | 32 | GPIO Pull-up/down Enable Control Configures GPIO pull enabling |
| A0020114 | GPIO_PULLEN1_ SET | 32 | GPIO Pull-up/down Enable Control For bitwise access of GPIO_PULLEN1 |
| A0020118 | GPIO_PULLEN1_ CLR | 32 | GPIO Pull-up/down Enable Control For bitwise access of GPIO_PULLEN1 |
| A0020200 | GPIO_DINV0 | 32 | GPIO Data Inversion Control Configures GPIO inversion enabling |
| A0020204 | GPIO_DINV0_SE T | 32 | GPIO Data Inversion Control For bitwise access of GPIO_DINV0 |
| A0020208 | GPIO_DINV0_CL R | 32 | GPIO Data Inversion Control For bitwise access of GPIO_DINV0 |
| A0020210 | GPIO_DINV1 | 32 | GPIO Data Inversion Control Configures GPIO inversion enabling |
| A0020214 | GPIO_DINV1_SE T | 32 | GPIO Data Inversion Control For bitwise access of GPIO_DINV1 |
| A0020218 | GPIO_DINV1_CL R | 32 | GPIO Data Inversion Control For bitwise access of GPIO_DINV1 |
| A0020300 | GPIO_DOUT0 | 32 | GPIO Output Data Control Configures GPIO output value |
| A0020304 | GPIO_DOUT0_SE T | 32 | GPIO Output Data Control For bitwise access of GPIO_DIR0 |
| A0020308 | GPIO_DOUT0_CL R | 32 | GPIO Output Data Control For bitwise access of GPIO_DIR0 |
| A0020310 | GPIO_DOUT1 | 32 | GPIO Output Data Control Configures GPIO output value |
| A0020314 | GPIO_DOUT1_SE T | 32 | GPIO Output Data Control For bitwise access of GPIO_DIR1 |
| A0020318 | GPIO_DOUT1_CL R | 32 | GPIO Output Data Control For bitwise access of GPIO_DIR1 |
| A0020400 | GPIO_DIN0 | 32 | GPIO Input Data Value Reads GPIO input value |
| A0020410 | GPIO_DIN1 | 32 | GPIO Input Data Value Reads GPIO input value |
| A0020500 | GPIO_PULLSEL0 | 32 | GPIO Pullsel Control Configures GPIO PUPD selection |
| A0020504 | GPIO_PULLSEL0 _SET | 32 | GPIO Pullsel Control For bitwise access of GPIO_PULLSEL0 |
| A0020508 | GPIO_PULLSEL0 _CLR | 32 | GPIO Pullsel Control For bitwise access of GPIO_PULLSEL0 |
| A0020510 | GPIO_PULLSEL1 | 32 | GPIO Pullsel Control Configures GPIO PUPD selection |
| A0020514 | GPIO_PULLSEL1 _SET | 32 | GPIO Pullsel Control For bitwise access of GPIO_PULLSEL1 |
| A0020518 | GPIO_PULLSEL1 _CLR | 32 | GPIO Pullsel Control For bitwise access of GPIO_PULLSEL1 |
| A0020600 | GPIO_SMT0 | 32 | GPIO SMT Control |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Address | Name | Width | Register Function |
| --- | --- | --- | --- |
|  |  |  | Configures GPIO Schmit trigger control |
| A0020604 | GPIO_SMT0_SET | 32 | GPIO SMT Control For bitwise access of GPIO_SMT0 |
| A0020608 | GPIO_SMT0_CLR | 32 | GPIO SMT Control For bitwise access of GPIO_SMT0 |
| A0020610 | GPIO_SMT1 | 32 | GPIO SMT Control Configures GPIO Schmit trigger control |
| A0020614 | GPIO_SMT1_SET | 32 | GPIO SMT Control For bitwise access of GPIO_SMT1 |
| A0020618 | GPIO_SMT1_CLR | 32 | GPIO SMT Control For bitwise access of GPIO_SMT1 |
| A0020700 | GPIO_SR0 | 32 | GPIO SR Control Configures GPIO slew rate control |
| A0020704 | GPIO_SR0_SET | 32 | GPIO SR Control For bitwise access of GPIO_SR0 |
| A0020708 | GPIO_SR0_CLR | 32 | GPIO SR Control For bitwise access of GPIO_SR0 |
| A0020710 | GPIO_SR1 | 32 | GPIO SR Control Configures GPIO slew rate control |
| A0020714 | GPIO_SR1_SET | 32 | GPIO SR Control For bitwise access of GPIO_SR1 |
| A0020718 | GPIO_SR1_CLR | 32 | GPIO SR Control For bitwise access of GPIO_SR1 |
| A0020720 | GPIO_SIM_SR | 32 | GPIO SIM SR Control Configures GPIO slew rate control for SIM IO |
| A0020724 | GPIO_SIM_SR_S ET | 32 | GPIO SIM SR Control For bitwise access of GPIO_SIM_SR |
| A0020728 | GPIO_SIM_SR_C LR | 32 | GPIO SIM SR Control For bitwise access of GPIO_SIM_SR |
| A0020800 | GPIO_DRV0 | 32 | GPIO DRV Control Configures GPIO driving control |
| A0020804 | GPIO_DRV0_SET | 32 | GPIO DRV Control For bitwise access of GPIO_DRV0 |
| A0020808 | GPIO_DRV0_CLR | 32 | GPIO DRV Control For bitwise access of GPIO_DRV0 |
| A0020810 | GPIO_DRV1 | 32 | GPIO DRV Control Configures GPIO driving control |
| A0020814 | GPIO_DRV1_SET | 32 | GPIO DRV Control For bitwise access of GPIO_DRV1 |
| A0020818 | GPIO_DRV1_CLR | 32 | GPIO DRV Control For bitwise access of GPIO_DRV1 |
| A0020900 | GPIO_IES0 | 32 | GPIO IES Control Configures GPIO input enabling control |
| A0020904 | GPIO_IES0_SET | 32 | GPIO IES Control For bitwise access of GPIO_IES0 |
| A0020908 | GPIO_IES0_CLR | 32 | GPIO IES Control For bitwise access of GPIO_IES0 |
| A0020910 | GPIO_IES1 | 32 | GPIO IES Control Configures GPIO input enabling control |
| A0020914 | GPIO_IES1_SET | 32 | GPIO IES Control For bitwise access of GPIO_IES1 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Address | Name | Width | Register Function |
| --- | --- | --- | --- |
| A0020918 | GPIO_IES1_CLR | 32 | GPIO IES Control For bitwise access of GPIO_IES1 |
| A0020A00 | GPIO_PUPD0 | 32 | GPIO PUPD Control Configures GPIO PUPD control |
| A0020A04 | GPIO_PUPD0_SE T | 32 | GPIO PUPD Control For bitwise access of GPIO_PUPD0 |
| A0020A08 | GPIO_PUPD0_CL R | 32 | GPIO PUPD Control For bitwise access of GPIO_PUPD0 |
| A0020A10 | GPIO_PUPD1 | 32 | GPIO PUPD Control Configures GPIO PUPD control |
| A0020A14 | GPIO_PUPD1_SE T | 32 | GPIO PUPD Control For bitwise access of GPIO_PUPD1 |
| A0020A18 | GPIO_PUPD1_CL R | 32 | GPIO PUPD Control For bitwise access of GPIO_PUPD1 |
| A0020B00 | GPIO_RESEN0_0 | 32 | GPIO R0 Control Configures GPIO R0 control |
| A0020B04 | GPIO_RESEN0_0 _SET | 32 | GPIO R0 Control For bitwise access of GPIO_RESEN0_0 |
| A0020B08 | GPIO_RESEN0_0 _CLR | 32 | GPIO R0 Control For bitwise access of GPIO_RESEN0_0 |
| A0020B10 | GPIO_RESEN0_1 | 32 | GPIO R0 Control Configures GPIO R0 control |
| A0020B14 | GPIO_RESEN0_1 _SET | 32 | GPIO R0 Control For bitwise access of GPIO_RESEN0_1 |
| A0020B18 | GPIO_RESEN0_1 _CLR | 32 | GPIO R0 Control For bitwise access of GPIO_RESEN0_1 |
| A0020B20 | GPIO_RESEN1_0 | 32 | GPIO R1 Control Configures GPIO R1 control |
| A0020B24 | GPIO_RESEN1_0 _SET | 32 | GPIO R1 Control For bitwise access of GPIO_RESEN1_0 |
| A0020B28 | GPIO_RESEN1_0 _CLR | 32 | GPIO R1 Control For bitwise access of GPIO_RESEN1_0 |
| A0020B30 | GPIO_RESEN1_1 | 32 | GPIO R1 Control Configures GPIO R1 control |
| A0020B34 | GPIO_RESEN1_1 _SET | 32 | GPIO R1 Control For bitwise access of GPIO_RESEN1_1 |
| A0020B38 | GPIO_RESEN1_1 _CLR | 32 | GPIO R1 Control For bitwise access of GPIO_RESEN1_1 |
| A0020C00 | GPIO_MODE0 | 32 | GPIO Mode Control Configures GPIO aux. mode |
| A0020C04 | GPIO_MODE0_S ET | 32 | GPIO Mode Control For bitwise access of GPIO_MODE0 |
| A0020C08 | GPIO_MODE0_C LR | 32 | GPIO Mode Control For bitwise access of GPIO_MODE0 |
| A0020C10 | GPIO_MODE1 | 32 | GPIO Mode Control Configures GPIO aux. mode |
| A0020C14 | GPIO_MODE1_S ET | 32 | GPIO Mode Control For bitwise access of GPIO_MODE1 |
| A0020C18 | GPIO_MODE1_C LR | 32 | GPIO Mode Control For bitwise access of GPIO_MODE1 |
| A0020C20 | GPIO_MODE2 | 32 | GPIO Mode Control |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Address | Name | Width | Register Function |
| --- | --- | --- | --- |
|  |  |  | Configures GPIO aux. mode |
| A0020C24 | GPIO_MODE2_S ET | 32 | GPIO Mode Control For bitwise access of GPIO_MODE2 |
| A0020C28 | GPIO_MODE2_C LR | 32 | GPIO Mode Control For bitwise access of GPIO_MODE2 |
| A0020C30 | GPIO_MODE3 | 32 | GPIO Mode Control Configures GPIO aux. mode |
| A0020C34 | GPIO_MODE3_S ET | 32 | GPIO Mode Control For bitwise access of GPIO_MODE3 |
| A0020C38 | GPIO_MODE3_C LR | 32 | GPIO Mode Control For bitwise access of GPIO_MODE3 |
| A0020C40 | GPIO_MODE4 | 32 | GPIO Mode Control Configures GPIO aux. mode |
| A0020C44 | GPIO_MODE4_S ET | 32 | GPIO Mode Control For bitwise access of GPIO_MODE4 |
| A0020C48 | GPIO_MODE4_C LR | 32 | GPIO Mode Control For bitwise access of GPIO_MODE4 |
| A0020C50 | GPIO_MODE5 | 32 | GPIO Mode Control Configures GPIO aux. mode |
| A0020C54 | GPIO_MODE5_S ET | 32 | GPIO Mode Control For bitwise access of GPIO_MODE5 |
| A0020C58 | GPIO_MODE5_C LR | 32 | GPIO Mode Control For bitwise access of GPIO_MODE5 |
| A0020C60 | GPIO_MODE6 | 32 | GPIO Mode Control Configures GPIO aux. mode |
| A0020C64 | GPIO_MODE6_S ET | 32 | GPIO Mode Control For bitwise access of GPIO_MODE6 |
| A0020C68 | GPIO_MODE6_C LR | 32 | GPIO Mode Control For bitwise access of GPIO_MODE6 |
| A0020D10 | GPIO_TDSEL | 32 | GPIO TDSEL Control GPIO TX duty control register |
| A0020D14 | GPIO_TDSEL_SE T | 32 | GPIO TDSEL Control For bitwise access of GPIO_TDSEL |
| A0020D18 | GPIO_TDSEL_CL R | 32 | GPIO TDSEL Control For bitwise access of GPIO_TDSEL |
| A0020E00 | CLK_OUT0 | 32 | CLK Out Selection Control CLK OUT0 Setting |
| A0020E10 | CLK_OUT1 | 32 | CLK Out Selection Control CLK OUT1 Setting |
| A0020E20 | CLK_OUT2 | 32 | CLK Out Selection Control CLK OUT2 Setting |
| A0020E30 | CLK_OUT3 | 32 | CLK Out Selection Control CLK OUT3 Setting |
| A0020E40 | CLK_OUT4 | 32 | CLK Out Selection Control CLK OUT4 Setting |
| A0020E50 | CLK_OUT5 | 32 | CLK Out Selection Control CLK OUT5 Setting |

| A0020000 | GPIO_DIR0 | GPIO Direction Control | 040008E0 |
| --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne | GPIO3 1 | GPIO3 0 | GPIO2 9 | GPIO2 8 | GPIO2 7 | GPIO2 6 | GPIO2 5 | GPIO2 4 | GPIO2 3 | GPIO2 2 | GPIO2 1 | GPIO2 0 | GPIO1 9 | GPIO1 8 | GPIO1 7 | GPIO1 6 |
| Type | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW |
| Reset | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Mne | GPIO1 5 | GPIO1 4 | GPIO1 3 | GPIO1 2 | GPIO1 1 | GPIO1 0 | GPIO9 | GPIO8 | GPIO7 | GPIO6 | GPIO5 | GPIO4 | GPIO3 | GPIO2 | GPIO1 | GPIO0 |
| Type | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW |
| Reset | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |

Overview: Configures GPIO direction

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31 | GPIO31 | GPIO31_DIR | GPIO31 direction control 0: GPIO as input 1: GPIO as output |
| 30 | GPIO30 | GPIO30_DIR | GPIO30 direction control 0: GPIO as input 1: GPIO as output |
| 29 | GPIO29 | GPIO29_DIR | GPIO29 direction control 0: GPIO as input 1: GPIO as output |
| 28 | GPIO28 | GPIO28_DIR | GPIO28 direction control 0: GPIO as input 1: GPIO as output |
| 27 | GPIO27 | GPIO27_DIR | GPIO27 direction control 0: GPIO as input 1: GPIO as output |
| 26 | GPIO26 | GPIO26_DIR | GPIO26 direction control 0: GPIO as input 1: GPIO as output |
| 25 | GPIO25 | GPIO25_DIR | GPIO25 direction control 0: GPIO as input 1: GPIO as output |
| 24 | GPIO24 | GPIO24_DIR | GPIO24 direction control 0: GPIO as input 1: GPIO as output |
| 23 | GPIO23 | GPIO23_DIR | GPIO23 direction control 0: GPIO as input 1: GPIO as output |
| 22 | GPIO22 | GPIO22_DIR | GPIO22 direction control 0: GPIO as input 1: GPIO as output |
| 21 | GPIO21 | GPIO21_DIR | GPIO21 direction control 0: GPIO as input 1: GPIO as output |
| 20 | GPIO20 | GPIO20_DIR | GPIO20 direction control 0: GPIO as input 1: GPIO as output |
| 19 | GPIO19 | GPIO19_DIR | GPIO19 direction control 0: GPIO as input 1: GPIO as output |
| 18 | GPIO18 | GPIO18_DIR | GPIO18 direction control |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: GPIO as input 1: GPIO as output |
| 17 | GPIO17 | GPIO17_DIR | GPIO17 direction control 0: GPIO as input 1: GPIO as output |
| 16 | GPIO16 | GPIO16_DIR | GPIO16 direction control 0: GPIO as input 1: GPIO as output |
| 15 | GPIO15 | GPIO15_DIR | GPIO15 direction control 0: GPIO as input 1: GPIO as output |
| 14 | GPIO14 | GPIO14_DIR | GPIO14 direction control 0: GPIO as input 1: GPIO as output |
| 13 | GPIO13 | GPIO13_DIR | GPIO13 direction control 0: GPIO as input 1: GPIO as output |
| 12 | GPIO12 | GPIO12_DIR | GPIO12 direction control 0: GPIO as input 1: GPIO as output |
| 11 | GPIO11 | GPIO11_DIR | GPIO11 direction control 0: GPIO as input 1: GPIO as output |
| 10 | GPIO10 | GPIO10_DIR | GPIO10 direction control 0: GPIO as input 1: GPIO as output |
| 9 | GPIO9 | GPIO9_DIR | GPIO9 direction control 0: GPIO as input 1: GPIO as output |
| 8 | GPIO8 | GPIO8_DIR | GPIO8 direction control 0: GPIO as input 1: GPIO as output |
| 7 | GPIO7 | GPIO7_DIR | GPIO7 direction control 0: GPIO as input 1: GPIO as output |
| 6 | GPIO6 | GPIO6_DIR | GPIO6 direction control 0: GPIO as input 1: GPIO as output |
| 5 | GPIO5 | GPIO5_DIR | GPIO5 direction control 0: GPIO as input 1: GPIO as output |
| 4 | GPIO4 | GPIO4_DIR | GPIO4 direction control 0: GPIO as input 1: GPIO as output |
| 3 | GPIO3 | GPIO3_DIR | GPIO3 direction control 0: GPIO as input 1: GPIO as output |
| 2 | GPIO2 | GPIO2_DIR | GPIO2 direction control 0: GPIO as input 1: GPIO as output |
| 1 | GPIO1 | GPIO1_DIR | GPIO1 direction control 0: GPIO as input |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: GPIO as output |
| 0 | GPIO0 | GPIO0_DIR | GPIO0 direction control 0: GPIO as input 1: GPIO as output |

A0020004	GPIO_DIR0_SE T

GPIO Direction Control	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne | GPIO3 1 | GPIO3 0 | GPIO2 9 | GPIO2 8 | GPIO2 7 | GPIO2 6 | GPIO2 5 | GPIO2 4 | GPIO2 3 | GPIO2 2 | GPIO2 1 | GPIO2 0 | GPIO1 9 | GPIO1 8 | GPIO1 7 | GPIO1 6 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Mne | GPIO1 5 | GPIO1 4 | GPIO1 3 | GPIO1 2 | GPIO1 1 | GPIO1 0 | GPIO9 | GPIO8 | GPIO7 | GPIO6 | GPIO5 | GPIO4 | GPIO3 | GPIO2 | GPIO1 | GPIO0 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_DIR0

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31 | GPIO31 | GPIO31_DIR | Bitwise SET operation of GPIO31 direction 0: Keep 1: SET bits |
| 30 | GPIO30 | GPIO30_DIR | Bitwise SET operation of GPIO30 direction 0: Keep 1: SET bits |
| 29 | GPIO29 | GPIO29_DIR | Bitwise SET operation of GPIO29 direction 0: Keep 1: SET bits |
| 28 | GPIO28 | GPIO28_DIR | Bitwise SET operation of GPIO28 direction 0: Keep 1: SET bits |
| 27 | GPIO27 | GPIO27_DIR | Bitwise SET operation of GPIO27 direction 0: Keep 1: SET bits |
| 26 | GPIO26 | GPIO26_DIR | Bitwise SET operation of GPIO26 direction 0: Keep 1: SET bits |
| 25 | GPIO25 | GPIO25_DIR | Bitwise SET operation of GPIO25 direction 0: Keep 1: SET bits |
| 24 | GPIO24 | GPIO24_DIR | Bitwise SET operation of GPIO24 direction 0: Keep 1: SET bits |
| 23 | GPIO23 | GPIO23_DIR | Bitwise SET operation of GPIO23 direction 0: Keep 1: SET bits |
| 22 | GPIO22 | GPIO22_DIR | Bitwise SET operation of GPIO22 direction 0: Keep 1: SET bits |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 21 | GPIO21 | GPIO21_DIR | Bitwise SET operation of GPIO21 direction 0: Keep 1: SET bits |
| 20 | GPIO20 | GPIO20_DIR | Bitwise SET operation of GPIO20 direction 0: Keep 1: SET bits |
| 19 | GPIO19 | GPIO19_DIR | Bitwise SET operation of GPIO19 direction 0: Keep 1: SET bits |
| 18 | GPIO18 | GPIO18_DIR | Bitwise SET operation of GPIO18 direction 0: Keep 1: SET bits |
| 17 | GPIO17 | GPIO17_DIR | Bitwise SET operation of GPIO17 direction 0: Keep 1: SET bits |
| 16 | GPIO16 | GPIO16_DIR | Bitwise SET operation of GPIO16 direction 0: Keep 1: SET bits |
| 15 | GPIO15 | GPIO15_DIR | Bitwise SET operation of GPIO15 direction 0: Keep 1: SET bits |
| 14 | GPIO14 | GPIO14_DIR | Bitwise SET operation of GPIO14 direction 0: Keep 1: SET bits |
| 13 | GPIO13 | GPIO13_DIR | Bitwise SET operation of GPIO13 direction 0: Keep 1: SET bits |
| 12 | GPIO12 | GPIO12_DIR | Bitwise SET operation of GPIO12 direction 0: Keep 1: SET bits |
| 11 | GPIO11 | GPIO11_DIR | Bitwise SET operation of GPIO11 direction 0: Keep 1: SET bits |
| 10 | GPIO10 | GPIO10_DIR | Bitwise SET operation of GPIO10 direction 0: Keep 1: SET bits |
| 9 | GPIO9 | GPIO9_DIR | Bitwise SET operation of GPIO9 direction 0: Keep 1: SET bits |
| 8 | GPIO8 | GPIO8_DIR | Bitwise SET operation of GPIO8 direction 0: Keep 1: SET bits |
| 7 | GPIO7 | GPIO7_DIR | Bitwise SET operation of GPIO7 direction 0: Keep 1: SET bits |
| 6 | GPIO6 | GPIO6_DIR | Bitwise SET operation of GPIO6 direction 0: Keep 1: SET bits |
| 5 | GPIO5 | GPIO5_DIR | Bitwise SET operation of GPIO5 direction 0: Keep 1: SET bits |
| 4 | GPIO4 | GPIO4_DIR | Bitwise SET operation of GPIO4 direction |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Keep 1: SET bits |
| 3 | GPIO3 | GPIO3_DIR | Bitwise SET operation of GPIO3 direction 0: Keep 1: SET bits |
| 2 | GPIO2 | GPIO2_DIR | Bitwise SET operation of GPIO2 direction 0: Keep 1: SET bits |
| 1 | GPIO1 | GPIO1_DIR | Bitwise SET operation of GPIO1 direction 0: Keep 1: SET bits |
| 0 | GPIO0 | GPIO0_DIR | Bitwise SET operation of GPIO0 direction 0: Keep 1: SET bits |

A0020008	GPIO_DIR0_CL R

GPIO Direction Control	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne | GPIO3 1 | GPIO3 0 | GPIO2 9 | GPIO2 8 | GPIO2 7 | GPIO2 6 | GPIO2 5 | GPIO2 4 | GPIO2 3 | GPIO2 2 | GPIO2 1 | GPIO2 0 | GPIO1 9 | GPIO1 8 | GPIO1 7 | GPIO1 6 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Mne | GPIO1 5 | GPIO1 4 | GPIO1 3 | GPIO1 2 | GPIO1 1 | GPIO1 0 | GPIO9 | GPIO8 | GPIO7 | GPIO6 | GPIO5 | GPIO4 | GPIO3 | GPIO2 | GPIO1 | GPIO0 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_DIR0

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31 | GPIO31 | GPIO31_DIR | Bitwise CLR operation of GPIO31 direction 0: Keep 1: CLR bits |
| 30 | GPIO30 | GPIO30_DIR | Bitwise CLR operation of GPIO30 direction 0: Keep 1: CLR bits |
| 29 | GPIO29 | GPIO29_DIR | Bitwise CLR operation of GPIO29 direction 0: Keep 1: CLR bits |
| 28 | GPIO28 | GPIO28_DIR | Bitwise CLR operation of GPIO28 direction 0: Keep 1: CLR bits |
| 27 | GPIO27 | GPIO27_DIR | Bitwise CLR operation of GPIO27 direction 0: Keep 1: CLR bits |
| 26 | GPIO26 | GPIO26_DIR | Bitwise CLR operation of GPIO26 direction 0: Keep 1: CLR bits |
| 25 | GPIO25 | GPIO25_DIR | Bitwise CLR operation of GPIO25 direction 0: Keep |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: CLR bits |
| 24 | GPIO24 | GPIO24_DIR | Bitwise CLR operation of GPIO24 direction 0: Keep 1: CLR bits |
| 23 | GPIO23 | GPIO23_DIR | Bitwise CLR operation of GPIO23 direction 0: Keep 1: CLR bits |
| 22 | GPIO22 | GPIO22_DIR | Bitwise CLR operation of GPIO22 direction 0: Keep 1: CLR bits |
| 21 | GPIO21 | GPIO21_DIR | Bitwise CLR operation of GPIO21 direction 0: Keep 1: CLR bits |
| 20 | GPIO20 | GPIO20_DIR | Bitwise CLR operation of GPIO20 direction 0: Keep 1: CLR bits |
| 19 | GPIO19 | GPIO19_DIR | Bitwise CLR operation of GPIO19 direction 0: Keep 1: CLR bits |
| 18 | GPIO18 | GPIO18_DIR | Bitwise CLR operation of GPIO18 direction 0: Keep 1: CLR bits |
| 17 | GPIO17 | GPIO17_DIR | Bitwise CLR operation of GPIO17 direction 0: Keep 1: CLR bits |
| 16 | GPIO16 | GPIO16_DIR | Bitwise CLR operation of GPIO16 direction 0: Keep 1: CLR bits |
| 15 | GPIO15 | GPIO15_DIR | Bitwise CLR operation of GPIO15 direction 0: Keep 1: CLR bits |
| 14 | GPIO14 | GPIO14_DIR | Bitwise CLR operation of GPIO14 direction 0: Keep 1: CLR bits |
| 13 | GPIO13 | GPIO13_DIR | Bitwise CLR operation of GPIO13 direction 0: Keep 1: CLR bits |
| 12 | GPIO12 | GPIO12_DIR | Bitwise CLR operation of GPIO12 direction 0: Keep 1: CLR bits |
| 11 | GPIO11 | GPIO11_DIR | Bitwise CLR operation of GPIO11 direction 0: Keep 1: CLR bits |
| 10 | GPIO10 | GPIO10_DIR | Bitwise CLR operation of GPIO10 direction 0: Keep 1: CLR bits |
| 9 | GPIO9 | GPIO9_DIR | Bitwise CLR operation of GPIO9 direction 0: Keep 1: CLR bits |
| 8 | GPIO8 | GPIO8_DIR | Bitwise CLR operation of GPIO8 direction 0: Keep 1: CLR bits |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7 | GPIO7 | GPIO7_DIR | Bitwise CLR operation of GPIO7 direction 0: Keep 1: CLR bits |
| 6 | GPIO6 | GPIO6_DIR | Bitwise CLR operation of GPIO6 direction 0: Keep 1: CLR bits |
| 5 | GPIO5 | GPIO5_DIR | Bitwise CLR operation of GPIO5 direction 0: Keep 1: CLR bits |
| 4 | GPIO4 | GPIO4_DIR | Bitwise CLR operation of GPIO4 direction 0: Keep 1: CLR bits |
| 3 | GPIO3 | GPIO3_DIR | Bitwise CLR operation of GPIO3 direction 0: Keep 1: CLR bits |
| 2 | GPIO2 | GPIO2_DIR | Bitwise CLR operation of GPIO2 direction 0: Keep 1: CLR bits |
| 1 | GPIO1 | GPIO1_DIR | Bitwise CLR operation of GPIO1 direction 0: Keep 1: CLR bits |
| 0 | GPIO0 | GPIO0_DIR | Bitwise CLR operation of GPIO0 direction 0: Keep 1: CLR bits |

A0020010	GPIO_DIR1	GPIO Direction Control	00004000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | GPIO5 5 | GPIO5 4 |  |  | GPIO5 1 | GPIO5 0 | GPIO4 9 | GPIO4 8 |
| Type |  |  |  |  |  |  |  |  | RW | RW |  |  | RW | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 |  |  | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO4 7 | GPIO4 6 | GPIO4 5 | GPIO4 4 | GPIO4 3 | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 | GPIO3 6 | GPIO3 5 | GPIO3 4 | GPIO3 3 | GPIO3 2 |
| Type | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW |
| Reset | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: Configures GPIO direction

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 23 | GPIO55 | GPIO55_DIR | GPIO55 direction control 0: GPIO as input 1: GPIO as output |
| 22 | GPIO54 | GPIO54_DIR | GPIO54 direction control 0: GPIO as input 1: GPIO as output |
| 19 | GPIO51 | GPIO51_DIR | GPIO51 direction control 0: GPIO as input 1: GPIO as output |
| 18 | GPIO50 | GPIO50_DIR | GPIO50 direction control 0: GPIO as input |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: GPIO as output |
| 17 | GPIO49 | GPIO49_DIR | GPIO49 direction control 0: GPIO as input 1: GPIO as output |
| 16 | GPIO48 | GPIO48_DIR | GPIO48 direction control 0: GPIO as input 1: GPIO as output |
| 15 | GPIO47 | GPIO47_DIR | GPIO47 direction control 0: GPIO as input 1: GPIO as output |
| 14 | GPIO46 | GPIO46_DIR | GPIO46 direction control 0: GPIO as input 1: GPIO as output |
| 13 | GPIO45 | GPIO45_DIR | GPIO45 direction control 0: GPIO as input 1: GPIO as output |
| 12 | GPIO44 | GPIO44_DIR | GPIO44 direction control 0: GPIO as input 1: GPIO as output |
| 11 | GPIO43 | GPIO43_DIR | GPIO43 direction control 0: GPIO as input 1: GPIO as output |
| 10 | GPIO42 | GPIO42_DIR | GPIO42 direction control 0: GPIO as input 1: GPIO as output |
| 9 | GPIO41 | GPIO41_DIR | GPIO41 direction control 0: GPIO as input 1: GPIO as output |
| 8 | GPIO40 | GPIO40_DIR | GPIO40 direction control 0: GPIO as input 1: GPIO as output |
| 7 | GPIO39 | GPIO39_DIR | GPIO39 direction control 0: GPIO as input 1: GPIO as output |
| 6 | GPIO38 | GPIO38_DIR | GPIO38 direction control 0: GPIO as input 1: GPIO as output |
| 5 | GPIO37 | GPIO37_DIR | GPIO37 direction control 0: GPIO as input 1: GPIO as output |
| 4 | GPIO36 | GPIO36_DIR | GPIO36 direction control 0: GPIO as input 1: GPIO as output |
| 3 | GPIO35 | GPIO35_DIR | GPIO35 direction control 0: GPIO as input 1: GPIO as output |
| 2 | GPIO34 | GPIO34_DIR | GPIO34 direction control 0: GPIO as input 1: GPIO as output |
| 1 | GPIO33 | GPIO33_DIR | GPIO33 direction control 0: GPIO as input 1: GPIO as output |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 0 | GPIO32 | GPIO32_DIR | GPIO32 direction control 0: GPIO as input 1: GPIO as output |

A0020014	GPIO_DIR1_SE T

GPIO Direction Control	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | GPIO5 5 | GPIO5 4 |  |  | GPIO5 1 | GPIO5 0 | GPIO4 9 | GPIO4 8 |
| Type |  |  |  |  |  |  |  |  | WO | WO |  |  | WO | WO | WO | WO |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 |  |  | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO4 7 | GPIO4 6 | GPIO4 5 | GPIO4 4 | GPIO4 3 | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 | GPIO3 6 | GPIO3 5 | GPIO3 4 | GPIO3 3 | GPIO3 2 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_DIR1

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 23 | GPIO55 | GPIO55_DIR | Bitwise SET operation of GPIO55 direction 0: Keep 1: SET bits |
| 22 | GPIO54 | GPIO54_DIR | Bitwise SET operation of GPIO54 direction 0: Keep 1: SET bits |
| 19 | GPIO51 | GPIO51_DIR | Bitwise SET operation of GPIO51 direction 0: Keep 1: SET bits |
| 18 | GPIO50 | GPIO50_DIR | Bitwise SET operation of GPIO50 direction 0: Keep 1: SET bits |
| 17 | GPIO49 | GPIO49_DIR | Bitwise SET operation of GPIO49 direction 0: Keep 1: SET bits |
| 16 | GPIO48 | GPIO48_DIR | Bitwise SET operation of GPIO48 direction 0: Keep 1: SET bits |
| 15 | GPIO47 | GPIO47_DIR | Bitwise SET operation of GPIO47 direction 0: Keep 1: SET bits |
| 14 | GPIO46 | GPIO46_DIR | Bitwise SET operation of GPIO46 direction 0: Keep 1: SET bits |
| 13 | GPIO45 | GPIO45_DIR | Bitwise SET operation of GPIO45 direction 0: Keep 1: SET bits |
| 12 | GPIO44 | GPIO44_DIR | Bitwise SET operation of GPIO44 direction 0: Keep 1: SET bits |
| 11 | GPIO43 | GPIO43_DIR | Bitwise SET operation of GPIO43 direction |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Keep 1: SET bits |
| 10 | GPIO42 | GPIO42_DIR | Bitwise SET operation of GPIO42 direction 0: Keep 1: SET bits |
| 9 | GPIO41 | GPIO41_DIR | Bitwise SET operation of GPIO41 direction 0: Keep 1: SET bits |
| 8 | GPIO40 | GPIO40_DIR | Bitwise SET operation of GPIO40 direction 0: Keep 1: SET bits |
| 7 | GPIO39 | GPIO39_DIR | Bitwise SET operation of GPIO39 direction 0: Keep 1: SET bits |
| 6 | GPIO38 | GPIO38_DIR | Bitwise SET operation of GPIO38 direction 0: Keep 1: SET bits |
| 5 | GPIO37 | GPIO37_DIR | Bitwise SET operation of GPIO37 direction 0: Keep 1: SET bits |
| 4 | GPIO36 | GPIO36_DIR | Bitwise SET operation of GPIO36 direction 0: Keep 1: SET bits |
| 3 | GPIO35 | GPIO35_DIR | Bitwise SET operation of GPIO35 direction 0: Keep 1: SET bits |
| 2 | GPIO34 | GPIO34_DIR | Bitwise SET operation of GPIO34 direction 0: Keep 1: SET bits |
| 1 | GPIO33 | GPIO33_DIR | Bitwise SET operation of GPIO33 direction 0: Keep 1: SET bits |
| 0 | GPIO32 | GPIO32_DIR | Bitwise SET operation of GPIO32 direction 0: Keep 1: SET bits |

A0020018	GPIO_DIR1_CL R

GPIO Direction Control	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | GPIO5 5 | GPIO5 4 |  |  | GPIO5 1 | GPIO5 0 | GPIO4 9 | GPIO4 8 |
| Type |  |  |  |  |  |  |  |  | WO | WO |  |  | WO | WO | WO | WO |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 |  |  | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO4 7 | GPIO4 6 | GPIO4 5 | GPIO4 4 | GPIO4 3 | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 | GPIO3 6 | GPIO3 5 | GPIO3 4 | GPIO3 3 | GPIO3 2 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_DIR1

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 23 | GPIO55 | GPIO55_DIR | Bitwise CLR operation of GPIO55 direction 0: Keep 1: CLR bits |
| 22 | GPIO54 | GPIO54_DIR | Bitwise CLR operation of GPIO54 direction 0: Keep 1: CLR bits |
| 19 | GPIO51 | GPIO51_DIR | Bitwise CLR operation of GPIO51 direction 0: Keep 1: CLR bits |
| 18 | GPIO50 | GPIO50_DIR | Bitwise CLR operation of GPIO50 direction 0: Keep 1: CLR bits |
| 17 | GPIO49 | GPIO49_DIR | Bitwise CLR operation of GPIO49 direction 0: Keep 1: CLR bits |
| 16 | GPIO48 | GPIO48_DIR | Bitwise CLR operation of GPIO48 direction 0: Keep 1: CLR bits |
| 15 | GPIO47 | GPIO47_DIR | Bitwise CLR operation of GPIO47 direction 0: Keep 1: CLR bits |
| 14 | GPIO46 | GPIO46_DIR | Bitwise CLR operation of GPIO46 direction 0: Keep 1: CLR bits |
| 13 | GPIO45 | GPIO45_DIR | Bitwise CLR operation of GPIO45 direction 0: Keep 1: CLR bits |
| 12 | GPIO44 | GPIO44_DIR | Bitwise CLR operation of GPIO44 direction 0: Keep 1: CLR bits |
| 11 | GPIO43 | GPIO43_DIR | Bitwise CLR operation of GPIO43 direction 0: Keep 1: CLR bits |
| 10 | GPIO42 | GPIO42_DIR | Bitwise CLR operation of GPIO42 direction 0: Keep 1: CLR bits |
| 9 | GPIO41 | GPIO41_DIR | Bitwise CLR operation of GPIO41 direction 0: Keep 1: CLR bits |
| 8 | GPIO40 | GPIO40_DIR | Bitwise CLR operation of GPIO40 direction 0: Keep 1: CLR bits |
| 7 | GPIO39 | GPIO39_DIR | Bitwise CLR operation of GPIO39 direction 0: Keep 1: CLR bits |
| 6 | GPIO38 | GPIO38_DIR | Bitwise CLR operation of GPIO38 direction 0: Keep 1: CLR bits |
| 5 | GPIO37 | GPIO37_DIR | Bitwise CLR operation of GPIO37 direction 0: Keep 1: CLR bits |
| 4 | GPIO36 | GPIO36_DIR | Bitwise CLR operation of GPIO36 direction |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Keep 1: CLR bits |
| 3 | GPIO35 | GPIO35_DIR | Bitwise CLR operation of GPIO35 direction 0: Keep 1: CLR bits |
| 2 | GPIO34 | GPIO34_DIR | Bitwise CLR operation of GPIO34 direction 0: Keep 1: CLR bits |
| 1 | GPIO33 | GPIO33_DIR | Bitwise CLR operation of GPIO33 direction 0: Keep 1: CLR bits |
| 0 | GPIO32 | GPIO32_DIR | Bitwise CLR operation of GPIO32 direction 0: Keep 1: CLR bits |

A0020100	GPIO_PULLEN0 GPIO Pull-up/down Enable Control	43C00BFF

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO3 0 |  |  |  |  | GPIO2 5 | GPIO2 4 | GPIO2 3 | GPIO2 2 |  |  |  |  |  |  |
| Type |  | RW |  |  |  |  | RW | RW | RW | RW |  |  |  |  |  |  |
| Reset |  | 1 |  |  |  |  | 1 | 1 | 1 | 1 |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  | GPIO1 1 |  | GPIO9 | GPIO8 | GPIO7 | GPIO6 | GPIO5 | GPIO4 | GPIO3 | GPIO2 | GPIO1 | GPIO0 |
| Type |  |  |  |  | RW |  | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW |
| Reset |  |  |  |  | 1 |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Overview: Configures GPIO pull enabling

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30 | GPIO30 | GPIO30_PULLEN | GPIO30 PULLEN 0: Disable 1: Enable |
| 25 | GPIO25 | GPIO25_PULLEN | GPIO25 PULLEN 0: Disable 1: Enable |
| 24 | GPIO24 | GPIO24_PULLEN | GPIO24 PULLEN 0: Disable 1: Enable |
| 23 | GPIO23 | GPIO23_PULLEN | GPIO23 PULLEN 0: Disable 1: Enable |
| 22 | GPIO22 | GPIO22_PULLEN | GPIO22 PULLEN 0: Disable 1: Enable |
| 11 | GPIO11 | GPIO11_PULLEN | GPIO11 PULLEN 0: Disable 1: Enable |
| 9 | GPIO9 | GPIO9_PULLEN | GPIO9 PULLEN 0: Disable 1: Enable |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 8 | GPIO8 | GPIO8_PULLEN | GPIO8 PULLEN 0: Disable 1: Enable |
| 7 | GPIO7 | GPIO7_PULLEN | GPIO7 PULLEN 0: Disable 1: Enable |
| 6 | GPIO6 | GPIO6_PULLEN | GPIO6 PULLEN 0: Disable 1: Enable |
| 5 | GPIO5 | GPIO5_PULLEN | GPIO5 PULLEN 0: Disable 1: Enable |
| 4 | GPIO4 | GPIO4_PULLEN | GPIO4 PULLEN 0: Disable 1: Enable |
| 3 | GPIO3 | GPIO3_PULLEN | GPIO3 PULLEN 0: Disable 1: Enable |
| 2 | GPIO2 | GPIO2_PULLEN | GPIO2 PULLEN 0: Disable 1: Enable |
| 1 | GPIO1 | GPIO1_PULLEN | GPIO1 PULLEN 0: Disable 1: Enable |
| 0 | GPIO0 | GPIO0_PULLEN | GPIO0 PULLEN 0: Disable 1: Enable |

A0020104	GPIO_PULLEN0 GPIO Pull-up/down Enable Control	00000000

_SET

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO3 0 |  |  |  |  | GPIO2 5 | GPIO2 4 | GPIO2 3 | GPIO2 2 |  |  |  |  |  |  |
| Type |  | WO |  |  |  |  | WO | WO | WO | WO |  |  |  |  |  |  |
| Reset |  | 0 |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  | GPIO1 1 |  | GPIO9 | GPIO8 | GPIO7 | GPIO6 | GPIO5 | GPIO4 | GPIO3 | GPIO2 | GPIO1 | GPIO0 |
| Type |  |  |  |  | WO |  | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset |  |  |  |  | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_PULLEN0

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30 | GPIO30 | GPIO30_PULLEN | Bitwise SET operation of GPIO30 PULLEN_SET 0: Keep 1: SET bits |
| 25 | GPIO25 | GPIO25_PULLEN | Bitwise SET operation of GPIO25 PULLEN_SET 0: Keep 1: SET bits |
| 24 | GPIO24 | GPIO24_PULLEN | Bitwise SET operation of GPIO24 PULLEN_SET |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Keep 1: SET bits |
| 23 | GPIO23 | GPIO23_PULLEN | Bitwise SET operation of GPIO23 PULLEN_SET 0: Keep 1: SET bits |
| 22 | GPIO22 | GPIO22_PULLEN | Bitwise SET operation of GPIO22 PULLEN_SET 0: Keep 1: SET bits |
| 11 | GPIO11 | GPIO11_PULLEN | Bitwise SET operation of GPIO11 PULLEN_SET 0: Keep 1: SET bits |
| 9 | GPIO9 | GPIO9_PULLEN | Bitwise SET operation of GPIO9 PULLEN_SET 0: Keep 1: SET bits |
| 8 | GPIO8 | GPIO8_PULLEN | Bitwise SET operation of GPIO8 PULLEN_SET 0: Keep 1: SET bits |
| 7 | GPIO7 | GPIO7_PULLEN | Bitwise SET operation of GPIO7 PULLEN_SET 0: Keep 1: SET bits |
| 6 | GPIO6 | GPIO6_PULLEN | Bitwise SET operation of GPIO6 PULLEN_SET 0: Keep 1: SET bits |
| 5 | GPIO5 | GPIO5_PULLEN | Bitwise SET operation of GPIO5 PULLEN_SET 0: Keep 1: SET bits |
| 4 | GPIO4 | GPIO4_PULLEN | Bitwise SET operation of GPIO4 PULLEN_SET 0: Keep 1: SET bits |
| 3 | GPIO3 | GPIO3_PULLEN | Bitwise SET operation of GPIO3 PULLEN_SET 0: Keep 1: SET bits |
| 2 | GPIO2 | GPIO2_PULLEN | Bitwise SET operation of GPIO2 PULLEN_SET 0: Keep 1: SET bits |
| 1 | GPIO1 | GPIO1_PULLEN | Bitwise SET operation of GPIO1 PULLEN_SET 0: Keep 1: SET bits |
| 0 | GPIO0 | GPIO0_PULLEN | Bitwise SET operation of GPIO0 PULLEN_SET 0: Keep 1: SET bits |

A0020108	GPIO_PULLEN0 GPIO Pull-up/down Enable Control	00000000

_CLR

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO3 0 |  |  |  |  | GPIO2 5 | GPIO2 4 | GPIO2 3 | GPIO2 2 |  |  |  |  |  |  |
| Type |  | WO |  |  |  |  | WO | WO | WO | WO |  |  |  |  |  |  |
| Reset |  | 0 |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  | GPIO1 |  | GPIO9 | GPIO8 | GPIO7 | GPIO6 | GPIO5 | GPIO4 | GPIO3 | GPIO2 | GPIO1 | GPIO0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

|  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type |  |  |  |  | WO |  | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset |  |  |  |  | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_PULLEN0

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30 | GPIO30 | GPIO30_PULLEN | Bitwise CLR operation of GPIO30 PULLEN_CLR 0: Keep 1: CLR bits |
| 25 | GPIO25 | GPIO25_PULLEN | Bitwise CLR operation of GPIO25 PULLEN_CLR 0: Keep 1: CLR bits |
| 24 | GPIO24 | GPIO24_PULLEN | Bitwise CLR operation of GPIO24 PULLEN_CLR 0: Keep 1: CLR bits |
| 23 | GPIO23 | GPIO23_PULLEN | Bitwise CLR operation of GPIO23 PULLEN_CLR 0: Keep 1: CLR bits |
| 22 | GPIO22 | GPIO22_PULLEN | Bitwise CLR operation of GPIO22 PULLEN_CLR 0: Keep 1: CLR bits |
| 11 | GPIO11 | GPIO11_PULLEN | Bitwise CLR operation of GPIO11 PULLEN_CLR 0: Keep 1: CLR bits |
| 9 | GPIO9 | GPIO9_PULLEN | Bitwise CLR operation of GPIO9 PULLEN_CLR 0: Keep 1: CLR bits |
| 8 | GPIO8 | GPIO8_PULLEN | Bitwise CLR operation of GPIO8 PULLEN_CLR 0: Keep 1: CLR bits |
| 7 | GPIO7 | GPIO7_PULLEN | Bitwise CLR operation of GPIO7 PULLEN_CLR 0: Keep 1: CLR bits |
| 6 | GPIO6 | GPIO6_PULLEN | Bitwise CLR operation of GPIO6 PULLEN_CLR 0: Keep 1: CLR bits |
| 5 | GPIO5 | GPIO5_PULLEN | Bitwise CLR operation of GPIO5 PULLEN_CLR 0: Keep 1: CLR bits |
| 4 | GPIO4 | GPIO4_PULLEN | Bitwise CLR operation of GPIO4 PULLEN_CLR 0: Keep 1: CLR bits |
| 3 | GPIO3 | GPIO3_PULLEN | Bitwise CLR operation of GPIO3 PULLEN_CLR 0: Keep 1: CLR bits |
| 2 | GPIO2 | GPIO2_PULLEN | Bitwise CLR operation of GPIO2 PULLEN_CLR 0: Keep 1: CLR bits |
| 1 | GPIO1 | GPIO1_PULLEN | Bitwise CLR operation of GPIO1 PULLEN_CLR 0: Keep 1: CLR bits |
| 0 | GPIO0 | GPIO0_PULLEN | Bitwise CLR operation of GPIO0 PULLEN_CLR |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Keep 1: CLR bits |

A0020110	GPIO_PULLEN1 GPIO Pull-up/down Enable Control	00F01800

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | GPIO5 5 | GPIO5 4 | GPIO5 3 | GPIO5 2 |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW | RW | RW | RW |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 1 | 1 | 1 | 1 |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  | GPIO4 4 | GPIO4 3 |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  | RW | RW |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |

Overview: Configures GPIO pull enabling

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 23 | GPIO55 | GPIO55_PULLEN | GPIO55 PULLEN 0: Disable 1: Enable |
| 22 | GPIO54 | GPIO54_PULLEN | GPIO54 PULLEN 0: Disable 1: Enable |
| 21 | GPIO53 | GPIO53_PULLEN | GPIO53 PULLEN 0: Disable 1: Enable |
| 20 | GPIO52 | GPIO52_PULLEN | GPIO52 PULLEN 0: Disable 1: Enable |
| 12 | GPIO44 | GPIO44_PULLEN | GPIO44 PULLEN 0: Disable 1: Enable |
| 11 | GPIO43 | GPIO43_PULLEN | GPIO43 PULLEN 0: Disable 1: Enable |

A0020114	GPIO_PULLEN1 GPIO Pull-up/down Enable Control	00000000

_SET

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | GPIO5 5 | GPIO5 4 | GPIO5 3 | GPIO5 2 |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | WO | WO | WO | WO |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  | GPIO4 4 | GPIO4 3 |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  | WO | WO |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Overview: For bitwise access of GPIO_PULLEN1

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 23 | GPIO55 | GPIO55_PULLEN | Bitwise SET operation of GPIO51 PULLEN_SET 0: Keep 1: SET bits |
| 22 | GPIO54 | GPIO54_PULLEN | Bitwise SET operation of GPIO44 PULLEN_SET 0: Keep 1: SET bits |
| 21 | GPIO53 | GPIO53_PULLEN | Bitwise SET operation of GPIO43 PULLEN_SET 0: Keep 1: SET bits |
| 20 | GPIO52 | GPIO52_PULLEN | Bitwise SET operation of GPIO30 PULLEN_SET 0: Keep 1: SET bits |
| 12 | GPIO44 | GPIO44_PULLEN | Bitwise SET operation of GPIO44 PULLEN_SET 0: Keep 1: SET bits |
| 11 | GPIO43 | GPIO43_PULLEN | Bitwise SET operation of GPIO43 PULLEN_SET 0: Keep 1: SET bits |

A0020118	GPIO_PULLEN1 GPIO Pull-up/down Enable Control	00000000

_CLR

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | GPIO5 5 | GPIO5 4 | GPIO5 3 | GPIO5 2 |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | WO | WO | WO | WO |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  | GPIO4 4 | GPIO4 3 |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  | WO | WO |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |

Overview: For bitwise access of GPIO_PULLEN1

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 23 | GPIO55 | GPIO55_PULLEN | Bitwise CLR operation of GPIO51 PULLEN_CLR 0: Keep 1: CLR bits |
| 22 | GPIO54 | GPIO54_PULLEN | Bitwise CLR operation of GPIO44 PULLEN_CLR 0: Keep 1: CLR bits |
| 21 | GPIO53 | GPIO53_PULLEN | Bitwise CLR operation of GPIO43 PULLEN_CLR 0: Keep 1: CLR bits |
| 20 | GPIO52 | GPIO52_PULLEN | Bitwise CLR operation of GPIO30 PULLEN_CLR 0: Keep 1: CLR bits |
| 12 | GPIO44 | GPIO44_PULLEN | Bitwise CLR operation of GPIO44 PULLEN_CLR |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Keep 1: CLR bits |
| 11 | GPIO43 | GPIO43_PULLEN | Bitwise CLR operation of GPIO43 PULLEN_CLR 0: Keep 1: CLR bits |

A0020200	GPIO_DINV0	GPIO Data Inversion Control	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | INV31 | INV30 | INV29 | INV28 | INV27 | INV26 | INV25 | INV24 | INV23 | INV22 | INV21 | INV20 | INV19 | INV18 | INV17 | INV16 |
| Type | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | INV15 | INV14 | INV13 | INV12 | INV11 | INV10 | INV9 | INV8 | INV7 | INV6 | INV5 | INV4 | INV3 | INV2 | INV1 | INV0 |
| Type | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: Configures GPIO inversion enabling

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31 | INV31 | GPIO31_DINV | GPIO31 inversion control 0: Keep input value 1: Invert input value |
| 30 | INV30 | GPIO30_DINV | GPIO30 inversion control 0: Keep input value 1: Invert input value |
| 29 | INV29 | GPIO29_DINV | GPIO29 inversion control 0: Keep input value 1: Invert input value |
| 28 | INV28 | GPIO28_DINV | GPIO28 inversion control 0: Keep input value 1: Invert input value |
| 27 | INV27 | GPIO27_DINV | GPIO27 inversion control 0: Keep input value 1: Invert input value |
| 26 | INV26 | GPIO26_DINV | GPIO26 inversion control 0: Keep input value 1: Invert input value |
| 25 | INV25 | GPIO25_DINV | GPIO25 inversion control 0: Keep input value 1: Invert input value |
| 24 | INV24 | GPIO24_DINV | GPIO24 inversion control 0: Keep input value 1: Invert input value |
| 23 | INV23 | GPIO23_DINV | GPIO23 inversion control 0: Keep input value 1: Invert input value |
| 22 | INV22 | GPIO22_DINV | GPIO22 inversion control 0: Keep input value 1: Invert input value |
| 21 | INV21 | GPIO21_DINV | GPIO21 inversion control |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Keep input value 1: Invert input value |
| 20 | INV20 | GPIO20_DINV | GPIO20 inversion control 0: Keep input value 1: Invert input value |
| 19 | INV19 | GPIO19_DINV | GPIO19 inversion control 0: Keep input value 1: Invert input value |
| 18 | INV18 | GPIO18_DINV | GPIO18 inversion control 0: Keep input value 1: Invert input value |
| 17 | INV17 | GPIO17_DINV | GPIO17 inversion control 0: Keep input value 1: Invert input value |
| 16 | INV16 | GPIO16_DINV | GPIO16 inversion control 0: Keep input value 1: Invert input value |
| 15 | INV15 | GPIO15_DINV | GPIO15 inversion control 0: Keep input value 1: Invert input value |
| 14 | INV14 | GPIO14_DINV | GPIO14 inversion control 0: Keep input value 1: Invert input value |
| 13 | INV13 | GPIO13_DINV | GPIO13 inversion control 0: Keep input value 1: Invert input value |
| 12 | INV12 | GPIO12_DINV | GPIO12 inversion control 0: Keep input value 1: Invert input value |
| 11 | INV11 | GPIO11_DINV | GPIO11 inversion control 0: Keep input value 1: Invert input value |
| 10 | INV10 | GPIO10_DINV | GPIO10 inversion control 0: Keep input value 1: Invert input value |
| 9 | INV9 | GPIO9_DINV | GPIO9 inversion control 0: Keep input value 1: Invert input value |
| 8 | INV8 | GPIO8_DINV | GPIO8 inversion control 0: Keep input value 1: Invert input value |
| 7 | INV7 | GPIO7_DINV | GPIO7 inversion control 0: Keep input value 1: Invert input value |
| 6 | INV6 | GPIO6_DINV | GPIO6 inversion control 0: Keep input value 1: Invert input value |
| 5 | INV5 | GPIO5_DINV | GPIO5 inversion control 0: Keep input value 1: Invert input value |
| 4 | INV4 | GPIO4_DINV | GPIO4 inversion control 0: Keep input value |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: Invert input value |
| 3 | INV3 | GPIO3_DINV | GPIO3 inversion control 0: Keep input value 1: Invert input value |
| 2 | INV2 | GPIO2_DINV | GPIO2 inversion control 0: Keep input value 1: Invert input value |
| 1 | INV1 | GPIO1_DINV | GPIO1 inversion control 0: Keep input value 1: Invert input value |
| 0 | INV0 | GPIO0_DINV | GPIO0 inversion control 0: Keep input value 1: Invert input value |

A0020204	GPIO_DINV0_S ET

GPIO Data Inversion Control	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | INV31 | INV30 | INV29 | INV28 | INV27 | INV26 | INV25 | INV24 | INV23 | INV22 | INV21 | INV20 | INV19 | INV18 | INV17 | INV16 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | INV15 | INV14 | INV13 | INV12 | INV11 | INV10 | INV9 | INV8 | INV7 | INV6 | INV5 | INV4 | INV3 | INV2 | INV1 | INV0 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_DINV0

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31 | INV31 | GPIO31_DINV | Bitwise SET operation of GPIO31 inversion control 0: Keep 1: SET bits |
| 30 | INV30 | GPIO30_DINV | Bitwise SET operation of GPIO30 inversion control 0: Keep 1: SET bits |
| 29 | INV29 | GPIO29_DINV | Bitwise SET operation of GPIO29 inversion control 0: Keep 1: SET bits |
| 28 | INV28 | GPIO28_DINV | Bitwise SET operation of GPIO28 inversion control 0: Keep 1: SET bits |
| 27 | INV27 | GPIO27_DINV | Bitwise SET operation of GPIO27 inversion control 0: Keep 1: SET bits |
| 26 | INV26 | GPIO26_DINV | Bitwise SET operation of GPIO26 inversion control 0: Keep 1: SET bits |
| 25 | INV25 | GPIO25_DINV | Bitwise SET operation of GPIO25 inversion control 0: Keep 1: SET bits |
| 24 | INV24 | GPIO24_DINV | Bitwise SET operation of GPIO24 inversion control |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Keep 1: SET bits |
| 23 | INV23 | GPIO23_DINV | Bitwise SET operation of GPIO23 inversion control 0: Keep 1: SET bits |
| 22 | INV22 | GPIO22_DINV | Bitwise SET operation of GPIO22 inversion control 0: Keep 1: SET bits |
| 21 | INV21 | GPIO21_DINV | Bitwise SET operation of GPIO21 inversion control 0: Keep 1: SET bits |
| 20 | INV20 | GPIO20_DINV | Bitwise SET operation of GPIO20 inversion control 0: Keep 1: SET bits |
| 19 | INV19 | GPIO19_DINV | Bitwise SET operation of GPIO19 inversion control 0: Keep 1: SET bits |
| 18 | INV18 | GPIO18_DINV | Bitwise SET operation of GPIO18 inversion control 0: Keep 1: SET bits |
| 17 | INV17 | GPIO17_DINV | Bitwise SET operation of GPIO17 inversion control 0: Keep 1: SET bits |
| 16 | INV16 | GPIO16_DINV | Bitwise SET operation of GPIO16 inversion control 0: Keep 1: SET bits |
| 15 | INV15 | GPIO15_DINV | Bitwise SET operation of GPIO15 inversion control 0: Keep 1: SET bits |
| 14 | INV14 | GPIO14_DINV | Bitwise SET operation of GPIO14 inversion control 0: Keep 1: SET bits |
| 13 | INV13 | GPIO13_DINV | Bitwise SET operation of GPIO13 inversion control 0: Keep 1: SET bits |
| 12 | INV12 | GPIO12_DINV | Bitwise SET operation of GPIO12 inversion control 0: Keep 1: SET bits |
| 11 | INV11 | GPIO11_DINV | Bitwise SET operation of GPIO11 inversion control 0: Keep 1: SET bits |
| 10 | INV10 | GPIO10_DINV | Bitwise SET operation of GPIO10 inversion control 0: Keep 1: SET bits |
| 9 | INV9 | GPIO9_DINV | Bitwise SET operation of GPIO9 inversion control 0: Keep 1: SET bits |
| 8 | INV8 | GPIO8_DINV | Bitwise SET operation of GPIO8 inversion control 0: Keep 1: SET bits |
| 7 | INV7 | GPIO7_DINV | Bitwise SET operation of GPIO7 inversion control 0: Keep |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: SET bits |
| 6 | INV6 | GPIO6_DINV | Bitwise SET operation of GPIO6 inversion control 0: Keep 1: SET bits |
| 5 | INV5 | GPIO5_DINV | Bitwise SET operation of GPIO5 inversion control 0: Keep 1: SET bits |
| 4 | INV4 | GPIO4_DINV | Bitwise SET operation of GPIO4 inversion control 0: Keep 1: SET bits |
| 3 | INV3 | GPIO3_DINV | Bitwise SET operation of GPIO3 inversion control 0: Keep 1: SET bits |
| 2 | INV2 | GPIO2_DINV | Bitwise SET operation of GPIO2 inversion control 0: Keep 1: SET bits |
| 1 | INV1 | GPIO1_DINV | Bitwise SET operation of GPIO1 inversion control 0: Keep 1: SET bits |
| 0 | INV0 | GPIO0_DINV | Bitwise SET operation of GPIO0 inversion control 0: Keep 1: SET bits |

A0020208	GPIO_DINV0_C LR

GPIO Data Inversion Control	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | INV31 | INV30 | INV29 | INV28 | INV27 | INV26 | INV25 | INV24 | INV23 | INV22 | INV21 | INV20 | INV19 | INV18 | INV17 | INV16 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | INV15 | INV14 | INV13 | INV12 | INV11 | INV10 | INV9 | INV8 | INV7 | INV6 | INV5 | INV4 | INV3 | INV2 | INV1 | INV0 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_DINV0

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31 | INV31 | GPIO31_DINV | Bitwise CLR operation of GPIO31 inversion control 0: Keep 1: CLR bits |
| 30 | INV30 | GPIO30_DINV | Bitwise CLR operation of GPIO30 inversion control 0: Keep 1: CLR bits |
| 29 | INV29 | GPIO29_DINV | Bitwise CLR operation of GPIO29 inversion control 0: Keep 1: CLR bits |
| 28 | INV28 | GPIO28_DINV | Bitwise CLR operation of GPIO28 inversion control 0: Keep 1: CLR bits |
| 27 | INV27 | GPIO27_DINV | Bitwise CLR operation of GPIO27 inversion control |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Keep 1: CLR bits |
| 26 | INV26 | GPIO26_DINV | Bitwise CLR operation of GPIO26 inversion control 0: Keep 1: CLR bits |
| 25 | INV25 | GPIO25_DINV | Bitwise CLR operation of GPIO25 inversion control 0: Keep 1: CLR bits |
| 24 | INV24 | GPIO24_DINV | Bitwise CLR operation of GPIO24 inversion control 0: Keep 1: CLR bits |
| 23 | INV23 | GPIO23_DINV | Bitwise CLR operation of GPIO23 inversion control 0: Keep 1: CLR bits |
| 22 | INV22 | GPIO22_DINV | Bitwise CLR operation of GPIO22 inversion control 0: Keep 1: CLR bits |
| 21 | INV21 | GPIO21_DINV | Bitwise CLR operation of GPIO21 inversion control 0: Keep 1: CLR bits |
| 20 | INV20 | GPIO20_DINV | Bitwise CLR operation of GPIO20 inversion control 0: Keep 1: CLR bits |
| 19 | INV19 | GPIO19_DINV | Bitwise CLR operation of GPIO19 inversion control 0: Keep 1: CLR bits |
| 18 | INV18 | GPIO18_DINV | Bitwise CLR operation of GPIO18 inversion control 0: Keep 1: CLR bits |
| 17 | INV17 | GPIO17_DINV | Bitwise CLR operation of GPIO17 inversion control 0: Keep 1: CLR bits |
| 16 | INV16 | GPIO16_DINV | Bitwise CLR operation of GPIO16 inversion control 0: Keep 1: CLR bits |
| 15 | INV15 | GPIO15_DINV | Bitwise CLR operation of GPIO15 inversion control 0: Keep 1: CLR bits |
| 14 | INV14 | GPIO14_DINV | Bitwise CLR operation of GPIO14 inversion control 0: Keep 1: CLR bits |
| 13 | INV13 | GPIO13_DINV | Bitwise CLR operation of GPIO13 inversion control 0: Keep 1: CLR bits |
| 12 | INV12 | GPIO12_DINV | Bitwise CLR operation of GPIO12 inversion control 0: Keep 1: CLR bits |
| 11 | INV11 | GPIO11_DINV | Bitwise CLR operation of GPIO11 inversion control 0: Keep 1: CLR bits |
| 10 | INV10 | GPIO10_DINV | Bitwise CLR operation of GPIO10 inversion control 0: Keep |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: CLR bits |
| 9 | INV9 | GPIO9_DINV | Bitwise CLR operation of GPIO9 inversion control 0: Keep 1: CLR bits |
| 8 | INV8 | GPIO8_DINV | Bitwise CLR operation of GPIO8 inversion control 0: Keep 1: CLR bits |
| 7 | INV7 | GPIO7_DINV | Bitwise CLR operation of GPIO7 inversion control 0: Keep 1: CLR bits |
| 6 | INV6 | GPIO6_DINV | Bitwise CLR operation of GPIO6 inversion control 0: Keep 1: CLR bits |
| 5 | INV5 | GPIO5_DINV | Bitwise CLR operation of GPIO5 inversion control 0: Keep 1: CLR bits |
| 4 | INV4 | GPIO4_DINV | Bitwise CLR operation of GPIO4 inversion control 0: Keep 1: CLR bits |
| 3 | INV3 | GPIO3_DINV | Bitwise CLR operation of GPIO3 inversion control 0: Keep 1: CLR bits |
| 2 | INV2 | GPIO2_DINV | Bitwise CLR operation of GPIO2 inversion control 0: Keep 1: CLR bits |
| 1 | INV1 | GPIO1_DINV | Bitwise CLR operation of GPIO1 inversion control 0: Keep 1: CLR bits |
| 0 | INV0 | GPIO0_DINV | Bitwise CLR operation of GPIO0 inversion control 0: Keep 1: CLR bits |

A0020210	GPIO_DINV1	GPIO Data Inversion Control	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | INV55 | INV54 | INV53 | INV52 | INV51 | INV50 | INV49 | INV48 |
| Type |  |  |  |  |  |  |  |  | RW | RW | RW | RW | RW | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | INV47 | INV46 | INV45 | INV44 | INV43 | INV42 | INV41 | INV40 | INV39 | INV38 | INV37 | INV36 | INV35 | INV34 | INV33 | INV32 |
| Type | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: Configures GPIO inversion enabling

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 23 | INV55 | GPIO55_DINV | GPIO55 inversion control 0: Keep input value 1: Invert input value |
| 22 | INV54 | GPIO54_DINV | GPIO54 inversion control 0: Keep input value |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: Invert input value |
| 21 | INV53 | GPIO53_DINV | GPIO53 inversion control 0: Keep input value 1: Invert input value |
| 20 | INV52 | GPIO52_DINV | GPIO52 inversion control 0: Keep input value 1: Invert input value |
| 19 | INV51 | GPIO51_DINV | GPIO51 inversion control 0: Keep input value 1: Invert input value |
| 18 | INV50 | GPIO50_DINV | GPIO50 inversion control 0: Keep input value 1: Invert input value |
| 17 | INV49 | GPIO49_DINV | GPIO49 inversion control 0: Keep input value 1: Invert input value |
| 16 | INV48 | GPIO48_DINV | GPIO48 inversion control 0: Keep input value 1: Invert input value |
| 15 | INV47 | GPIO47_DINV | GPIO47 inversion control 0: Keep input value 1: Invert input value |
| 14 | INV46 | GPIO46_DINV | GPIO46 inversion control 0: Keep input value 1: Invert input value |
| 13 | INV45 | GPIO45_DINV | GPIO45 inversion control 0: Keep input value 1: Invert input value |
| 12 | INV44 | GPIO44_DINV | GPIO44 inversion control 0: Keep input value 1: Invert input value |
| 11 | INV43 | GPIO43_DINV | GPIO43 inversion control 0: Keep input value 1: Invert input value |
| 10 | INV42 | GPIO42_DINV | GPIO42 inversion control 0: Keep input value 1: Invert input value |
| 9 | INV41 | GPIO41_DINV | GPIO41 inversion control 0: Keep input value 1: Invert input value |
| 8 | INV40 | GPIO40_DINV | GPIO40 inversion control 0: Keep input value 1: Invert input value |
| 7 | INV39 | GPIO39_DINV | GPIO39 inversion control 0: Keep input value 1: Invert input value |
| 6 | INV38 | GPIO38_DINV | GPIO38 inversion control 0: Keep input value 1: Invert input value |
| 5 | INV37 | GPIO37_DINV | GPIO37 inversion control 0: Keep input value 1: Invert input value |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 4 | INV36 | GPIO36_DINV | GPIO36 inversion control 0: Keep input value 1: Invert input value |
| 3 | INV35 | GPIO35_DINV | GPIO35 inversion control 0: Keep input value 1: Invert input value |
| 2 | INV34 | GPIO34_DINV | GPIO34 inversion control 0: Keep input value 1: Invert input value |
| 1 | INV33 | GPIO33_DINV | GPIO33 inversion control 0: Keep input value 1: Invert input value |
| 0 | INV32 | GPIO32_DINV | GPIO32 inversion control 0: Keep input value 1: Invert input value |

A0020214	GPIO_DINV1_S ET

GPIO Data Inversion Control	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | INV55 | INV54 | INV53 | INV52 | INV51 | INV50 | INV49 | INV48 |
| Type |  |  |  |  |  |  |  |  | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | INV47 | INV46 | INV45 | INV44 | INV43 | INV42 | INV41 | INV40 | INV39 | INV38 | INV37 | INV36 | INV35 | INV34 | INV33 | INV32 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_DINV1

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 23 | INV55 | GPIO55_DINV | Bitwise SET operation of GPIO55 inversion control 0: Keep 1: SET bits |
| 22 | INV54 | GPIO54_DINV | Bitwise SET operation of GPIO54 inversion control 0: Keep 1: SET bits |
| 21 | INV53 | GPIO53_DINV | Bitwise SET operation of GPIO53 inversion control 0: Keep 1: SET bits |
| 20 | INV52 | GPIO52_DINV | Bitwise SET operation of GPIO52 inversion control 0: Keep 1: SET bits |
| 19 | INV51 | GPIO51_DINV | Bitwise SET operation of GPIO51 inversion control 0: Keep 1: SET bits |
| 18 | INV50 | GPIO50_DINV | Bitwise SET operation of GPIO50 inversion control 0: Keep 1: SET bits |
| 17 | INV49 | GPIO49_DINV | Bitwise SET operation of GPIO49 inversion control 0: Keep |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: SET bits |
| 16 | INV48 | GPIO48_DINV | Bitwise SET operation of GPIO48 inversion control 0: Keep 1: SET bits |
| 15 | INV47 | GPIO47_DINV | Bitwise SET operation of GPIO47 inversion control 0: Keep 1: SET bits |
| 14 | INV46 | GPIO46_DINV | Bitwise SET operation of GPIO46 inversion control 0: Keep 1: SET bits |
| 13 | INV45 | GPIO45_DINV | Bitwise SET operation of GPIO45 inversion control 0: Keep 1: SET bits |
| 12 | INV44 | GPIO44_DINV | Bitwise SET operation of GPIO44 inversion control 0: Keep 1: SET bits |
| 11 | INV43 | GPIO43_DINV | Bitwise SET operation of GPIO43 inversion control 0: Keep 1: SET bits |
| 10 | INV42 | GPIO42_DINV | Bitwise SET operation of GPIO42 inversion control 0: Keep 1: SET bits |
| 9 | INV41 | GPIO41_DINV | Bitwise SET operation of GPIO41 inversion control 0: Keep 1: SET bits |
| 8 | INV40 | GPIO40_DINV | Bitwise SET operation of GPIO40 inversion control 0: Keep 1: SET bits |
| 7 | INV39 | GPIO39_DINV | Bitwise SET operation of GPIO39 inversion control 0: Keep 1: SET bits |
| 6 | INV38 | GPIO38_DINV | Bitwise SET operation of GPIO38 inversion control 0: Keep 1: SET bits |
| 5 | INV37 | GPIO37_DINV | Bitwise SET operation of GPIO37 inversion control 0: Keep 1: SET bits |
| 4 | INV36 | GPIO36_DINV | Bitwise SET operation of GPIO36 inversion control 0: Keep 1: SET bits |
| 3 | INV35 | GPIO35_DINV | Bitwise SET operation of GPIO35 inversion control 0: Keep 1: SET bits |
| 2 | INV34 | GPIO34_DINV | Bitwise SET operation of GPIO34 inversion control 0: Keep 1: SET bits |
| 1 | INV33 | GPIO33_DINV | Bitwise SET operation of GPIO33 inversion control 0: Keep 1: SET bits |
| 0 | INV32 | GPIO32_DINV | Bitwise SET operation of GPIO32 inversion control 0: Keep 1: SET bits |

A0020218	GPIO_DINV1_C LR

GPIO Data Inversion Control	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | INV55 | INV54 | INV53 | INV52 | INV51 | INV50 | INV49 | INV48 |
| Type |  |  |  |  |  |  |  |  | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | INV47 | INV46 | INV45 | INV44 | INV43 | INV42 | INV41 | INV40 | INV39 | INV38 | INV37 | INV36 | INV35 | INV34 | INV33 | INV32 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_DINV1

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 23 | INV55 | GPIO55_DINV | Bitwise CLR operation of GPIO55 inversion control 0: Keep 1: CLR bits |
| 22 | INV54 | GPIO54_DINV | Bitwise CLR operation of GPIO54 inversion control 0: Keep 1: CLR bits |
| 21 | INV53 | GPIO53_DINV | Bitwise CLR operation of GPIO53 inversion control 0: Keep 1: CLR bits |
| 20 | INV52 | GPIO52_DINV | Bitwise CLR operation of GPIO52 inversion control 0: Keep 1: CLR bits |
| 19 | INV51 | GPIO51_DINV | Bitwise CLR operation of GPIO51 inversion control 0: Keep 1: CLR bits |
| 18 | INV50 | GPIO50_DINV | Bitwise CLR operation of GPIO50 inversion control 0: Keep 1: CLR bits |
| 17 | INV49 | GPIO49_DINV | Bitwise CLR operation of GPIO49 inversion control 0: Keep 1: CLR bits |
| 16 | INV48 | GPIO48_DINV | Bitwise CLR operation of GPIO48 inversion control 0: Keep 1: CLR bits |
| 15 | INV47 | GPIO47_DINV | Bitwise CLR operation of GPIO47 inversion control 0: Keep 1: CLR bits |
| 14 | INV46 | GPIO46_DINV | Bitwise CLR operation of GPIO46 inversion control 0: Keep 1: CLR bits |
| 13 | INV45 | GPIO45_DINV | Bitwise CLR operation of GPIO45 inversion control 0: Keep 1: CLR bits |
| 12 | INV44 | GPIO44_DINV | Bitwise CLR operation of GPIO44 inversion control 0: Keep 1: CLR bits |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 11 | INV43 | GPIO43_DINV | Bitwise CLR operation of GPIO43 inversion control 0: Keep 1: CLR bits |
| 10 | INV42 | GPIO42_DINV | Bitwise CLR operation of GPIO42 inversion control 0: Keep 1: CLR bits |
| 9 | INV41 | GPIO41_DINV | Bitwise CLR operation of GPIO41 inversion control 0: Keep 1: CLR bits |
| 8 | INV40 | GPIO40_DINV | Bitwise CLR operation of GPIO40 inversion control 0: Keep 1: CLR bits |
| 7 | INV39 | GPIO39_DINV | Bitwise CLR operation of GPIO39 inversion control 0: Keep 1: CLR bits |
| 6 | INV38 | GPIO38_DINV | Bitwise CLR operation of GPIO38 inversion control 0: Keep 1: CLR bits |
| 5 | INV37 | GPIO37_DINV | Bitwise CLR operation of GPIO37 inversion control 0: Keep 1: CLR bits |
| 4 | INV36 | GPIO36_DINV | Bitwise CLR operation of GPIO36 inversion control 0: Keep 1: CLR bits |
| 3 | INV35 | GPIO35_DINV | Bitwise CLR operation of GPIO35 inversion control 0: Keep 1: CLR bits |
| 2 | INV34 | GPIO34_DINV | Bitwise CLR operation of GPIO34 inversion control 0: Keep 1: CLR bits |
| 1 | INV33 | GPIO33_DINV | Bitwise CLR operation of GPIO33 inversion control 0: Keep 1: CLR bits |
| 0 | INV32 | GPIO32_DINV | Bitwise CLR operation of GPIO32 inversion control 0: Keep 1: CLR bits |

A0020300	GPIO_DOUT0	GPIO Output Data Control	04000800

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | GPIO3 1 | GPIO3 0 | GPIO2 9 | GPIO2 8 | GPIO2 7 | GPIO2 6 | GPIO2 5 | GPIO2 4 | GPIO2 3 | GPIO2 2 | GPIO2 1 | GPIO2 0 | GPIO1 9 | GPIO1 8 | GPIO1 7 | GPIO1 6 |
| Type | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW |
| Reset | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO1 5 | GPIO1 4 | GPIO1 3 | GPIO1 2 | GPIO1 1 | GPIO1 0 | GPIO9 | GPIO8 | GPIO7 | GPIO6 | GPIO5 | GPIO4 | GPIO3 | GPIO2 | GPIO1 | GPIO0 |
| Type | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW |
| Reset | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: Configures GPIO output value

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31 | GPIO31 | GPIO31_OUT | GPIO31 data output value 0: GPIO output LO 1: GPIO output HI |
| 30 | GPIO30 | GPIO30_OUT | GPIO30 data output value 0: GPIO output LO 1: GPIO output HI |
| 29 | GPIO29 | GPIO29_OUT | GPIO29 data output value 0: GPIO output LO 1: GPIO output HI |
| 28 | GPIO28 | GPIO28_OUT | GPIO28 data output value 0: GPIO output LO 1: GPIO output HI |
| 27 | GPIO27 | GPIO27_OUT | GPIO27 data output value 0: GPIO output LO 1: GPIO output HI |
| 26 | GPIO26 | GPIO26_OUT | GPIO26 data output value 0: GPIO output LO 1: GPIO output HI |
| 25 | GPIO25 | GPIO25_OUT | GPIO25 data output value 0: GPIO output LO 1: GPIO output HI |
| 24 | GPIO24 | GPIO24_OUT | GPIO24 data output value 0: GPIO output LO 1: GPIO output HI |
| 23 | GPIO23 | GPIO23_OUT | GPIO23 data output value 0: GPIO output LO 1: GPIO output HI |
| 22 | GPIO22 | GPIO22_OUT | GPIO22 data output value 0: GPIO output LO 1: GPIO output HI |
| 21 | GPIO21 | GPIO21_OUT | GPIO21 data output value 0: GPIO output LO 1: GPIO output HI |
| 20 | GPIO20 | GPIO20_OUT | GPIO20 data output value 0: GPIO output LO 1: GPIO output HI |
| 19 | GPIO19 | GPIO19_OUT | GPIO19 data output value 0: GPIO output LO 1: GPIO output HI |
| 18 | GPIO18 | GPIO18_OUT | GPIO18 data output value 0: GPIO output LO 1: GPIO output HI |
| 17 | GPIO17 | GPIO17_OUT | GPIO17 data output value 0: GPIO output LO 1: GPIO output HI |
| 16 | GPIO16 | GPIO16_OUT | GPIO16 data output value 0: GPIO output LO 1: GPIO output HI |
| 15 | GPIO15 | GPIO15_OUT | GPIO15 data output value 0: GPIO output LO 1: GPIO output HI |
| 14 | GPIO14 | GPIO14_OUT | GPIO14 data output value |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: GPIO output LO 1: GPIO output HI |
| 13 | GPIO13 | GPIO13_OUT | GPIO13 data output value 0: GPIO output LO 1: GPIO output HI |
| 12 | GPIO12 | GPIO12_OUT | GPIO12 data output value 0: GPIO output LO 1: GPIO output HI |
| 11 | GPIO11 | GPIO11_OUT | GPIO11 data output value 0: GPIO output LO 1: GPIO output HI |
| 10 | GPIO10 | GPIO10_OUT | GPIO10 data output value 0: GPIO output LO 1: GPIO output HI |
| 9 | GPIO9 | GPIO9_OUT | GPIO9 data output value 0: GPIO output LO 1: GPIO output HI |
| 8 | GPIO8 | GPIO8_OUT | GPIO8 data output value 0: GPIO output LO 1: GPIO output HI |
| 7 | GPIO7 | GPIO7_OUT | GPIO7 data output value 0: GPIO output LO 1: GPIO output HI |
| 6 | GPIO6 | GPIO6_OUT | GPIO6 data output value 0: GPIO output LO 1: GPIO output HI |
| 5 | GPIO5 | GPIO5_OUT | GPIO5 data output value 0: GPIO output LO 1: GPIO output HI |
| 4 | GPIO4 | GPIO4_OUT | GPIO4 data output value 0: GPIO output LO 1: GPIO output HI |
| 3 | GPIO3 | GPIO3_OUT | GPIO3 data output value 0: GPIO output LO 1: GPIO output HI |
| 2 | GPIO2 | GPIO2_OUT | GPIO2 data output value 0: GPIO output LO 1: GPIO output HI |
| 1 | GPIO1 | GPIO1_OUT | GPIO1 data output value 0: GPIO output LO 1: GPIO output HI |
| 0 | GPIO0 | GPIO0_OUT | GPIO0 data output value 0: GPIO output LO 1: GPIO output HI |

A0020304	GPIO_DOUT0_S GPIO Output Data Control	00000000

ET

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | GPIO3 1 | GPIO3 0 | GPIO2 9 | GPIO2 8 | GPIO2 7 | GPIO2 6 | GPIO2 5 | GPIO2 4 | GPIO2 3 | GPIO2 2 | GPIO2 1 | GPIO2 0 | GPIO1 9 | GPIO1 8 | GPIO1 7 | GPIO1 6 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO1 5 | GPIO1 4 | GPIO1 3 | GPIO1 2 | GPIO1 1 | GPIO1 0 | GPIO9 | GPIO8 | GPIO7 | GPIO6 | GPIO5 | GPIO4 | GPIO3 | GPIO2 | GPIO1 | GPIO0 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_DIR0

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31 | GPIO31 | GPIO31_OUT | Bitwise SET operation of GPIO31 data output value 0: Keep 1: SET bits |
| 30 | GPIO30 | GPIO30_OUT | Bitwise SET operation of GPIO30 data output value 0: Keep 1: SET bits |
| 29 | GPIO29 | GPIO29_OUT | Bitwise SET operation of GPIO29 data output value 0: Keep 1: SET bits |
| 28 | GPIO28 | GPIO28_OUT | Bitwise SET operation of GPIO28 data output value 0: Keep 1: SET bits |
| 27 | GPIO27 | GPIO27_OUT | Bitwise SET operation of GPIO27 data output value 0: Keep 1: SET bits |
| 26 | GPIO26 | GPIO26_OUT | Bitwise SET operation of GPIO26 data output value 0: Keep 1: SET bits |
| 25 | GPIO25 | GPIO25_OUT | Bitwise SET operation of GPIO25 data output value 0: Keep 1: SET bits |
| 24 | GPIO24 | GPIO24_OUT | Bitwise SET operation of GPIO24 data output value 0: Keep 1: SET bits |
| 23 | GPIO23 | GPIO23_OUT | Bitwise SET operation of GPIO23 data output value 0: Keep 1: SET bits |
| 22 | GPIO22 | GPIO22_OUT | Bitwise SET operation of GPIO22 data output value 0: Keep 1: SET bits |
| 21 | GPIO21 | GPIO21_OUT | Bitwise SET operation of GPIO21 data output value 0: Keep 1: SET bits |
| 20 | GPIO20 | GPIO20_OUT | Bitwise SET operation of GPIO20 data output value 0: Keep 1: SET bits |
| 19 | GPIO19 | GPIO19_OUT | Bitwise SET operation of GPIO19 data output value 0: Keep 1: SET bits |
| 18 | GPIO18 | GPIO18_OUT | Bitwise SET operation of GPIO18 data output value 0: Keep 1: SET bits |
| 17 | GPIO17 | GPIO17_OUT | Bitwise SET operation of GPIO17 data output value |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Keep 1: SET bits |
| 16 | GPIO16 | GPIO16_OUT | Bitwise SET operation of GPIO16 data output value 0: Keep 1: SET bits |
| 15 | GPIO15 | GPIO15_OUT | Bitwise SET operation of GPIO15 data output value 0: Keep 1: SET bits |
| 14 | GPIO14 | GPIO14_OUT | Bitwise SET operation of GPIO14 data output value 0: Keep 1: SET bits |
| 13 | GPIO13 | GPIO13_OUT | Bitwise SET operation of GPIO13 data output value 0: Keep 1: SET bits |
| 12 | GPIO12 | GPIO12_OUT | Bitwise SET operation of GPIO12 data output value 0: Keep 1: SET bits |
| 11 | GPIO11 | GPIO11_OUT | Bitwise SET operation of GPIO11 data output value 0: Keep 1: SET bits |
| 10 | GPIO10 | GPIO10_OUT | Bitwise SET operation of GPIO10 data output value 0: Keep 1: SET bits |
| 9 | GPIO9 | GPIO9_OUT | Bitwise SET operation of GPIO9 data output value 0: Keep 1: SET bits |
| 8 | GPIO8 | GPIO8_OUT | Bitwise SET operation of GPIO8 data output value 0: Keep 1: SET bits |
| 7 | GPIO7 | GPIO7_OUT | Bitwise SET operation of GPIO7 data output value 0: Keep 1: SET bits |
| 6 | GPIO6 | GPIO6_OUT | Bitwise SET operation of GPIO6 data output value 0: Keep 1: SET bits |
| 5 | GPIO5 | GPIO5_OUT | Bitwise SET operation of GPIO5 data output value 0: Keep 1: SET bits |
| 4 | GPIO4 | GPIO4_OUT | Bitwise SET operation of GPIO4 data output value 0: Keep 1: SET bits |
| 3 | GPIO3 | GPIO3_OUT | Bitwise SET operation of GPIO3 data output value 0: Keep 1: SET bits |
| 2 | GPIO2 | GPIO2_OUT | Bitwise SET operation of GPIO2 data output value 0: Keep 1: SET bits |
| 1 | GPIO1 | GPIO1_OUT | Bitwise SET operation of GPIO1 data output value 0: Keep 1: SET bits |
| 0 | GPIO0 | GPIO0_OUT | Bitwise SET operation of GPIO0 data output value 0: Keep |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: SET bits |

A0020308	GPIO_DOUT0_CGPIO Output Data Control	00000000

LR

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | GPIO3 1 | GPIO3 0 | GPIO2 9 | GPIO2 8 | GPIO2 7 | GPIO2 6 | GPIO2 5 | GPIO2 4 | GPIO2 3 | GPIO2 2 | GPIO2 1 | GPIO2 0 | GPIO1 9 | GPIO1 8 | GPIO1 7 | GPIO1 6 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO1 5 | GPIO1 4 | GPIO1 3 | GPIO1 2 | GPIO1 1 | GPIO1 0 | GPIO9 | GPIO8 | GPIO7 | GPIO6 | GPIO5 | GPIO4 | GPIO3 | GPIO2 | GPIO1 | GPIO0 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_DIR0

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31 | GPIO31 | GPIO31_OUT | Bitwise CLR operation of GPIO31 data output value 0: Keep 1: CLR bits |
| 30 | GPIO30 | GPIO30_OUT | Bitwise CLR operation of GPIO30 data output value 0: Keep 1: CLR bits |
| 29 | GPIO29 | GPIO29_OUT | Bitwise CLR operation of GPIO29 data output value 0: Keep 1: CLR bits |
| 28 | GPIO28 | GPIO28_OUT | Bitwise CLR operation of GPIO28 data output value 0: Keep 1: CLR bits |
| 27 | GPIO27 | GPIO27_OUT | Bitwise CLR operation of GPIO27 data output value 0: Keep 1: CLR bits |
| 26 | GPIO26 | GPIO26_OUT | Bitwise CLR operation of GPIO26 data output value 0: Keep 1: CLR bits |
| 25 | GPIO25 | GPIO25_OUT | Bitwise CLR operation of GPIO25 data output value 0: Keep 1: CLR bits |
| 24 | GPIO24 | GPIO24_OUT | Bitwise CLR operation of GPIO24 data output value 0: Keep 1: CLR bits |
| 23 | GPIO23 | GPIO23_OUT | Bitwise CLR operation of GPIO23 data output value 0: Keep 1: CLR bits |
| 22 | GPIO22 | GPIO22_OUT | Bitwise CLR operation of GPIO22 data output value 0: Keep 1: CLR bits |
| 21 | GPIO21 | GPIO21_OUT | Bitwise CLR operation of GPIO21 data output value 0: Keep 1: CLR bits |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 20 | GPIO20 | GPIO20_OUT | Bitwise CLR operation of GPIO20 data output value 0: Keep 1: CLR bits |
| 19 | GPIO19 | GPIO19_OUT | Bitwise CLR operation of GPIO19 data output value 0: Keep 1: CLR bits |
| 18 | GPIO18 | GPIO18_OUT | Bitwise CLR operation of GPIO18 data output value 0: Keep 1: CLR bits |
| 17 | GPIO17 | GPIO17_OUT | Bitwise CLR operation of GPIO17 data output value 0: Keep 1: CLR bits |
| 16 | GPIO16 | GPIO16_OUT | Bitwise CLR operation of GPIO16 data output value 0: Keep 1: CLR bits |
| 15 | GPIO15 | GPIO15_OUT | Bitwise CLR operation of GPIO15 data output value 0: Keep 1: CLR bits |
| 14 | GPIO14 | GPIO14_OUT | Bitwise CLR operation of GPIO14 data output value 0: Keep 1: CLR bits |
| 13 | GPIO13 | GPIO13_OUT | Bitwise CLR operation of GPIO13 data output value 0: Keep 1: CLR bits |
| 12 | GPIO12 | GPIO12_OUT | Bitwise CLR operation of GPIO12 data output value 0: Keep 1: CLR bits |
| 11 | GPIO11 | GPIO11_OUT | Bitwise CLR operation of GPIO11 data output value 0: Keep 1: CLR bits |
| 10 | GPIO10 | GPIO10_OUT | Bitwise CLR operation of GPIO10 data output value 0: Keep 1: CLR bits |
| 9 | GPIO9 | GPIO9_OUT | Bitwise CLR operation of GPIO9 data output value 0: Keep 1: CLR bits |
| 8 | GPIO8 | GPIO8_OUT | Bitwise CLR operation of GPIO8 data output value 0: Keep 1: CLR bits |
| 7 | GPIO7 | GPIO7_OUT | Bitwise CLR operation of GPIO7 data output value 0: Keep 1: CLR bits |
| 6 | GPIO6 | GPIO6_OUT | Bitwise CLR operation of GPIO6 data output value 0: Keep 1: CLR bits |
| 5 | GPIO5 | GPIO5_OUT | Bitwise CLR operation of GPIO5 data output value 0: Keep 1: CLR bits |
| 4 | GPIO4 | GPIO4_OUT | Bitwise CLR operation of GPIO4 data output value 0: Keep 1: CLR bits |
| 3 | GPIO3 | GPIO3_OUT | Bitwise CLR operation of GPIO3 data output value |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Keep 1: CLR bits |
| 2 | GPIO2 | GPIO2_OUT | Bitwise CLR operation of GPIO2 data output value 0: Keep 1: CLR bits |
| 1 | GPIO1 | GPIO1_OUT | Bitwise CLR operation of GPIO1 data output value 0: Keep 1: CLR bits |
| 0 | GPIO0 | GPIO0_OUT | Bitwise CLR operation of GPIO0 data output value 0: Keep 1: CLR bits |

A0020310	GPIO_DOUT1	GPIO Output Data Control	00004000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | GPIO5 5 | GPIO5 4 |  |  | GPIO5 1 | GPIO5 0 | GPIO4 9 | GPIO4 8 |
| Type |  |  |  |  |  |  |  |  | RW | RW |  |  | RW | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 |  |  | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO4 7 | GPIO4 6 | GPIO4 5 | GPIO4 4 | GPIO4 3 | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 | GPIO3 6 | GPIO3 5 | GPIO3 4 | GPIO3 3 | GPIO3 2 |
| Type | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW |
| Reset | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: Configures GPIO output value

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 23 | GPIO55 | GPIO55_OUT | GPIO55 data output value 0: GPIO output LO 1: GPIO output HI |
| 22 | GPIO54 | GPIO54_OUT | GPIO54 data output value 0: GPIO output LO 1: GPIO output HI |
| 19 | GPIO51 | GPIO51_OUT | GPIO51 data output value 0: GPIO output LO 1: GPIO output HI |
| 18 | GPIO50 | GPIO50_OUT | GPIO50 data output value 0: GPIO output LO 1: GPIO output HI |
| 17 | GPIO49 | GPIO49_OUT | GPIO49 data output value 0: GPIO output LO 1: GPIO output HI |
| 16 | GPIO48 | GPIO48_OUT | GPIO48 data output value 0: GPIO output LO 1: GPIO output HI |
| 15 | GPIO47 | GPIO47_OUT | GPIO47 data output value 0: GPIO output LO 1: GPIO output HI |
| 14 | GPIO46 | GPIO46_OUT | GPIO46 data output value 0: GPIO output LO 1: GPIO output HI |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 13 | GPIO45 | GPIO45_OUT | GPIO45 data output value 0: GPIO output LO 1: GPIO output HI |
| 12 | GPIO44 | GPIO44_OUT | GPIO44 data output value 0: GPIO output LO 1: GPIO output HI |
| 11 | GPIO43 | GPIO43_OUT | GPIO43 data output value 0: GPIO output LO 1: GPIO output HI |
| 10 | GPIO42 | GPIO42_OUT | GPIO42 data output value 0: GPIO output LO 1: GPIO output HI |
| 9 | GPIO41 | GPIO41_OUT | GPIO41 data output value 0: GPIO output LO 1: GPIO output HI |
| 8 | GPIO40 | GPIO40_OUT | GPIO40 data output value 0: GPIO output LO 1: GPIO output HI |
| 7 | GPIO39 | GPIO39_OUT | GPIO39 data output value 0: GPIO output LO 1: GPIO output HI |
| 6 | GPIO38 | GPIO38_OUT | GPIO38 data output value 0: GPIO output LO 1: GPIO output HI |
| 5 | GPIO37 | GPIO37_OUT | GPIO37 data output value 0: GPIO output LO 1: GPIO output HI |
| 4 | GPIO36 | GPIO36_OUT | GPIO36 data output value 0: GPIO output LO 1: GPIO output HI |
| 3 | GPIO35 | GPIO35_OUT | GPIO35 data output value 0: GPIO output LO 1: GPIO output HI |
| 2 | GPIO34 | GPIO34_OUT | GPIO34 data output value 0: GPIO output LO 1: GPIO output HI |
| 1 | GPIO33 | GPIO33_OUT | GPIO33 data output value 0: GPIO output LO 1: GPIO output HI |
| 0 | GPIO32 | GPIO32_OUT | GPIO32 data output value 0: GPIO output LO 1: GPIO output HI |

A0020314	GPIO_DOUT1_S GPIO Output Data Control	00000000

ET

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | GPIO5 5 | GPIO5 4 |  |  | GPIO5 1 | GPIO5 0 | GPIO4 9 | GPIO4 8 |
| Type |  |  |  |  |  |  |  |  | WO | WO |  |  | WO | WO | WO | WO |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 |  |  | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Name | GPIO4 7 | GPIO4 6 | GPIO4 5 | GPIO4 4 | GPIO4 3 | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 | GPIO3 6 | GPIO3 5 | GPIO3 4 | GPIO3 3 | GPIO3 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_DIR1

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 23 | GPIO55 | GPIO55_OUT | Bitwise SET operation of GPIO55 data output value 0: Keep 1: SET bits |
| 22 | GPIO54 | GPIO54_OUT | Bitwise SET operation of GPIO54 data output value 0: Keep 1: SET bits |
| 19 | GPIO51 | GPIO51_OUT | Bitwise SET operation of GPIO51 data output value 0: Keep 1: SET bits |
| 18 | GPIO50 | GPIO50_OUT | Bitwise SET operation of GPIO50 data output value 0: Keep 1: SET bits |
| 17 | GPIO49 | GPIO49_OUT | Bitwise SET operation of GPIO49 data output value 0: Keep 1: SET bits |
| 16 | GPIO48 | GPIO48_OUT | Bitwise SET operation of GPIO48 data output value 0: Keep 1: SET bits |
| 15 | GPIO47 | GPIO47_OUT | Bitwise SET operation of GPIO47 data output value 0: Keep 1: SET bits |
| 14 | GPIO46 | GPIO46_OUT | Bitwise SET operation of GPIO46 data output value 0: Keep 1: SET bits |
| 13 | GPIO45 | GPIO45_OUT | Bitwise SET operation of GPIO45 data output value 0: Keep 1: SET bits |
| 12 | GPIO44 | GPIO44_OUT | Bitwise SET operation of GPIO44 data output value 0: Keep 1: SET bits |
| 11 | GPIO43 | GPIO43_OUT | Bitwise SET operation of GPIO43 data output value 0: Keep 1: SET bits |
| 10 | GPIO42 | GPIO42_OUT | Bitwise SET operation of GPIO42 data output value 0: Keep 1: SET bits |
| 9 | GPIO41 | GPIO41_OUT | Bitwise SET operation of GPIO41 data output value 0: Keep 1: SET bits |
| 8 | GPIO40 | GPIO40_OUT | Bitwise SET operation of GPIO40 data output value 0: Keep 1: SET bits |
| 7 | GPIO39 | GPIO39_OUT | Bitwise SET operation of GPIO39 data output value 0: Keep 1: SET bits |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 6 | GPIO38 | GPIO38_OUT | Bitwise SET operation of GPIO38 data output value 0: Keep 1: SET bits |
| 5 | GPIO37 | GPIO37_OUT | Bitwise SET operation of GPIO37 data output value 0: Keep 1: SET bits |
| 4 | GPIO36 | GPIO36_OUT | Bitwise SET operation of GPIO36 data output value 0: Keep 1: SET bits |
| 3 | GPIO35 | GPIO35_OUT | Bitwise SET operation of GPIO35 data output value 0: Keep 1: SET bits |
| 2 | GPIO34 | GPIO34_OUT | Bitwise SET operation of GPIO34 data output value 0: Keep 1: SET bits |
| 1 | GPIO33 | GPIO33_OUT | Bitwise SET operation of GPIO33 data output value 0: Keep 1: SET bits |
| 0 | GPIO32 | GPIO32_OUT | Bitwise SET operation of GPIO32 data output value 0: Keep 1: SET bits |

A0020318	GPIO_DOUT1_CGPIO Output Data Control	00000000

LR

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | GPIO5 5 | GPIO5 4 |  |  | GPIO5 1 | GPIO5 0 | GPIO4 9 | GPIO4 8 |
| Type |  |  |  |  |  |  |  |  | WO | WO |  |  | WO | WO | WO | WO |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 |  |  | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO4 7 | GPIO4 6 | GPIO4 5 | GPIO4 4 | GPIO4 3 | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 | GPIO3 6 | GPIO3 5 | GPIO3 4 | GPIO3 3 | GPIO3 2 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_DIR1

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 23 | GPIO55 | GPIO55_OUT | Bitwise CLR operation of GPIO55 data output value 0: Keep 1: CLR bits |
| 22 | GPIO54 | GPIO54_OUT | Bitwise CLR operation of GPIO54 data output value 0: Keep 1: CLR bits |
| 19 | GPIO51 | GPIO51_OUT | Bitwise CLR operation of GPIO51 data output value 0: Keep 1: CLR bits |
| 18 | GPIO50 | GPIO50_OUT | Bitwise CLR operation of GPIO50 data output value 0: Keep 1: CLR bits |
| 17 | GPIO49 | GPIO49_OUT | Bitwise CLR operation of GPIO49 data output value |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Keep 1: CLR bits |
| 16 | GPIO48 | GPIO48_OUT | Bitwise CLR operation of GPIO48 data output value 0: Keep 1: CLR bits |
| 15 | GPIO47 | GPIO47_OUT | Bitwise CLR operation of GPIO47 data output value 0: Keep 1: CLR bits |
| 14 | GPIO46 | GPIO46_OUT | Bitwise CLR operation of GPIO46 data output value 0: Keep 1: CLR bits |
| 13 | GPIO45 | GPIO45_OUT | Bitwise CLR operation of GPIO45 data output value 0: Keep 1: CLR bits |
| 12 | GPIO44 | GPIO44_OUT | Bitwise CLR operation of GPIO44 data output value 0: Keep 1: CLR bits |
| 11 | GPIO43 | GPIO43_OUT | Bitwise CLR operation of GPIO43 data output value 0: Keep 1: CLR bits |
| 10 | GPIO42 | GPIO42_OUT | Bitwise CLR operation of GPIO42 data output value 0: Keep 1: CLR bits |
| 9 | GPIO41 | GPIO41_OUT | Bitwise CLR operation of GPIO41 data output value 0: Keep 1: CLR bits |
| 8 | GPIO40 | GPIO40_OUT | Bitwise CLR operation of GPIO40 data output value 0: Keep 1: CLR bits |
| 7 | GPIO39 | GPIO39_OUT | Bitwise CLR operation of GPIO39 data output value 0: Keep 1: CLR bits |
| 6 | GPIO38 | GPIO38_OUT | Bitwise CLR operation of GPIO38 data output value 0: Keep 1: CLR bits |
| 5 | GPIO37 | GPIO37_OUT | Bitwise CLR operation of GPIO37 data output value 0: Keep 1: CLR bits |
| 4 | GPIO36 | GPIO36_OUT | Bitwise CLR operation of GPIO36 data output value 0: Keep 1: CLR bits |
| 3 | GPIO35 | GPIO35_OUT | Bitwise CLR operation of GPIO35 data output value 0: Keep 1: CLR bits |
| 2 | GPIO34 | GPIO34_OUT | Bitwise CLR operation of GPIO34 data output value 0: Keep 1: CLR bits |
| 1 | GPIO33 | GPIO33_OUT | Bitwise CLR operation of GPIO33 data output value 0: Keep 1: CLR bits |
| 0 | GPIO32 | GPIO32_OUT | Bitwise CLR operation of GPIO32 data output value 0: Keep |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: CLR bits |

A0020400	GPIO_DIN0	GPIO Input Data Value	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | GPIO3 1 | GPIO3 0 | GPIO2 9 | GPIO2 8 | GPIO2 7 | GPIO2 6 | GPIO2 5 | GPIO2 4 | GPIO2 3 | GPIO2 2 | GPIO2 1 | GPIO2 0 | GPIO1 9 | GPIO1 8 | GPIO1 7 | GPIO1 6 |
| Type | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO1 5 | GPIO1 4 | GPIO1 3 | GPIO1 2 | GPIO1 1 | GPIO1 0 | GPIO9 | GPIO8 | GPIO7 | GPIO6 | GPIO5 | GPIO4 | GPIO3 | GPIO2 | GPIO1 | GPIO0 |
| Type | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: Reads GPIO input value

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31 | GPIO31 | GPIO31_DIN | GPIO31 data input value |
| 30 | GPIO30 | GPIO30_DIN | GPIO30 data input value |
| 29 | GPIO29 | GPIO29_DIN | GPIO29 data input value |
| 28 | GPIO28 | GPIO28_DIN | GPIO28 data input value |
| 27 | GPIO27 | GPIO27_DIN | GPIO27 data input value |
| 26 | GPIO26 | GPIO26_DIN | GPIO26 data input value |
| 25 | GPIO25 | GPIO25_DIN | GPIO25 data input value |
| 24 | GPIO24 | GPIO24_DIN | GPIO24 data input value |
| 23 | GPIO23 | GPIO23_DIN | GPIO23 data input value |
| 22 | GPIO22 | GPIO22_DIN | GPIO22 data input value |
| 21 | GPIO21 | GPIO21_DIN | GPIO21 data input value |
| 20 | GPIO20 | GPIO20_DIN | GPIO20 data input value |
| 19 | GPIO19 | GPIO19_DIN | GPIO19 data input value |
| 18 | GPIO18 | GPIO18_DIN | GPIO18 data input value |
| 17 | GPIO17 | GPIO17_DIN | GPIO17 data input value |
| 16 | GPIO16 | GPIO16_DIN | GPIO16 data input value |
| 15 | GPIO15 | GPIO15_DIN | GPIO15 data input value |
| 14 | GPIO14 | GPIO14_DIN | GPIO14 data input value |
| 13 | GPIO13 | GPIO13_DIN | GPIO13 data input value |
| 12 | GPIO12 | GPIO12_DIN | GPIO12 data input value |
| 11 | GPIO11 | GPIO11_DIN | GPIO11 data input value |
| 10 | GPIO10 | GPIO10_DIN | GPIO10 data input value |
| 9 | GPIO9 | GPIO9_DIN | GPIO9 data input value |
| 8 | GPIO8 | GPIO8_DIN | GPIO8 data input value |
| 7 | GPIO7 | GPIO7_DIN | GPIO7 data input value |
| 6 | GPIO6 | GPIO6_DIN | GPIO6 data input value |
| 5 | GPIO5 | GPIO5_DIN | GPIO5 data input value |
| 4 | GPIO4 | GPIO4_DIN | GPIO4 data input value |
| 3 | GPIO3 | GPIO3_DIN | GPIO3 data input value |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 2 | GPIO2 | GPIO2_DIN | GPIO2 data input value |
| 1 | GPIO1 | GPIO1_DIN | GPIO1 data input value |
| 0 | GPIO0 | GPIO0_DIN | GPIO0 data input value |

A0020410	GPIO_DIN1	GPIO Input Data Value	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | GPIO5 5 | GPIO5 4 | GPIO5 3 | GPIO5 2 | GPIO5 1 | GPIO5 0 | GPIO4 9 | GPIO4 8 |
| Type |  |  |  |  |  |  |  |  | RO | RO | RO | RO | RO | RO | RO | RO |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO4 7 | GPIO4 6 | GPIO4 5 | GPIO4 4 | GPIO4 3 | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 | GPIO3 6 | GPIO3 5 | GPIO3 4 | GPIO3 3 | GPIO3 2 |
| Type | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO | RO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: Reads GPIO input value

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 23 | GPIO55 | GPIO55_DIN | GPIO55 data input value |
| 22 | GPIO54 | GPIO54_DIN | GPIO54 data input value |
| 21 | GPIO53 | GPIO53_DIN | GPIO53 data input value |
| 20 | GPIO52 | GPIO52_DIN | GPIO52 data input value |
| 19 | GPIO51 | GPIO51_DIN | GPIO51 data input value |
| 18 | GPIO50 | GPIO50_DIN | GPIO50 data input value |
| 17 | GPIO49 | GPIO49_DIN | GPIO49 data input value |
| 16 | GPIO48 | GPIO48_DIN | GPIO48 data input value |
| 15 | GPIO47 | GPIO47_DIN | GPIO47 data input value |
| 14 | GPIO46 | GPIO46_DIN | GPIO46 data input value |
| 13 | GPIO45 | GPIO45_DIN | GPIO45 data input value |
| 12 | GPIO44 | GPIO44_DIN | GPIO44 data input value |
| 11 | GPIO43 | GPIO43_DIN | GPIO43 data input value |
| 10 | GPIO42 | GPIO42_DIN | GPIO42 data input value |
| 9 | GPIO41 | GPIO41_DIN | GPIO41 data input value |
| 8 | GPIO40 | GPIO40_DIN | GPIO40 data input value |
| 7 | GPIO39 | GPIO39_DIN | GPIO39 data input value |
| 6 | GPIO38 | GPIO38_DIN | GPIO38 data input value |
| 5 | GPIO37 | GPIO37_DIN | GPIO37 data input value |
| 4 | GPIO36 | GPIO36_DIN | GPIO36 data input value |
| 3 | GPIO35 | GPIO35_DIN | GPIO35 data input value |
| 2 | GPIO34 | GPIO34_DIN | GPIO34 data input value |
| 1 | GPIO33 | GPIO33_DIN | GPIO33 data input value |
| 0 | GPIO32 | GPIO32_DIN | GPIO32 data input value |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A0020500	GPIO_PULLSEL GPIO Pullsel Control	00000000

0

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO3 0 |  |  |  |  | GPIO2 5 | GPIO2 4 | GPIO2 3 | GPIO2 2 |  |  |  |  |  |  |
| Type |  | RW |  |  |  |  | RW | RW | RW | RW |  |  |  |  |  |  |
| Reset |  | 0 |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  | GPIO1 1 |  | GPIO9 | GPIO8 | GPIO7 | GPIO6 | GPIO5 | GPIO4 | GPIO3 | GPIO2 | GPIO1 | GPIO0 |
| Type |  |  |  |  | RW |  | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW |
| Reset |  |  |  |  | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: Configures GPIO PUPD selection

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30 | GPIO30 | GPIO30_PULLSEL | GPIO30 PULLSEL 0: Pull down 1: Pull up |
| 25 | GPIO25 | GPIO25_PULLSEL | GPIO25 PULLSEL 0: Pull down 1: Pull up |
| 24 | GPIO24 | GPIO24_PULLSEL | GPIO24 PULLSEL 0: Pull down 1: Pull up |
| 23 | GPIO23 | GPIO23_PULLSEL | GPIO23 PULLSEL 0: Pull down 1: Pull up |
| 22 | GPIO22 | GPIO22_PULLSEL | GPIO22 PULLSEL 0: Pull down 1: Pull up |
| 11 | GPIO11 | GPIO11_PULLSEL | GPIO11 PULLSEL 0: Pull down 1: Pull up |
| 9 | GPIO9 | GPIO9_PULLSEL | GPIO9 PULLSEL 0: Pull down 1: Pull up |
| 8 | GPIO8 | GPIO8_PULLSEL | GPIO8 PULLSEL 0: Pull down 1: Pull up |
| 7 | GPIO7 | GPIO7_PULLSEL | GPIO7 PULLSEL 0: Pull down 1: Pull up |
| 6 | GPIO6 | GPIO6_PULLSEL | GPIO6 PULLSEL 0: Pull down 1: Pull up |
| 5 | GPIO5 | GPIO5_PULLSEL | GPIO5 PULLSEL 0: Pull down 1: Pull up |
| 4 | GPIO4 | GPIO4_PULLSEL | GPIO4 PULLSEL 0: Pull down 1: Pull up |
| 3 | GPIO3 | GPIO3_PULLSEL | GPIO3 PULLSEL |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Pull down 1: Pull up |
| 2 | GPIO2 | GPIO2_PULLSEL | GPIO2 PULLSEL 0: Pull down 1: Pull up |
| 1 | GPIO1 | GPIO1_PULLSEL | GPIO1 PULLSEL 0: Pull down 1: Pull up |
| 0 | GPIO0 | GPIO0_PULLSEL | GPIO0 PULLSEL 0: Pull down 1: Pull up |

A0020504	GPIO_PULLSEL GPIO Pullsel Control	00000000

0_SET

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO3 0 |  |  |  |  | GPIO2 5 | GPIO2 4 | GPIO2 3 | GPIO2 2 |  |  |  |  |  |  |
| Type |  | WO |  |  |  |  | WO | WO | WO | WO |  |  |  |  |  |  |
| Reset |  | 0 |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  | GPIO1 1 |  | GPIO9 | GPIO8 | GPIO7 | GPIO6 | GPIO5 | GPIO4 | GPIO3 | GPIO2 | GPIO1 | GPIO0 |
| Type |  |  |  |  | WO |  | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset |  |  |  |  | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_PULLSEL0

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30 | GPIO30 | GPIO30_PULLSEL | Bitwise SET operation of GPIO30 PULLSEL_SET 0: Keep 1: SET bits |
| 25 | GPIO25 | GPIO25_PULLSEL | Bitwise SET operation of GPIO25 PULLSEL_SET 0: Keep 1: SET bits |
| 24 | GPIO24 | GPIO24_PULLSEL | Bitwise SET operation of GPIO24 PULLSEL_SET 0: Keep 1: SET bits |
| 23 | GPIO23 | GPIO23_PULLSEL | Bitwise SET operation of GPIO23 PULLSEL_SET 0: Keep 1: SET bits |
| 22 | GPIO22 | GPIO22_PULLSEL | Bitwise SET operation of GPIO22 PULLSEL_SET 0: Keep 1: SET bits |
| 11 | GPIO11 | GPIO11_PULLSEL | Bitwise SET operation of GPIO11 PULLSEL_SET 0: Keep 1: SET bits |
| 9 | GPIO9 | GPIO9_PULLSEL | Bitwise SET operation of GPIO9 PULLSEL_SET 0: Keep 1: SET bits |
| 8 | GPIO8 | GPIO8_PULLSEL | Bitwise SET operation of GPIO8 PULLSEL_SET 0: Keep |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: SET bits |
| 7 | GPIO7 | GPIO7_PULLSEL | Bitwise SET operation of GPIO7 PULLSEL_SET 0: Keep 1: SET bits |
| 6 | GPIO6 | GPIO6_PULLSEL | Bitwise SET operation of GPIO6 PULLSEL_SET 0: Keep 1: SET bits |
| 5 | GPIO5 | GPIO5_PULLSEL | Bitwise SET operation of GPIO5 PULLSEL_SET 0: Keep 1: SET bits |
| 4 | GPIO4 | GPIO4_PULLSEL | Bitwise SET operation of GPIO4 PULLSEL_SET 0: Keep 1: SET bits |
| 3 | GPIO3 | GPIO3_PULLSEL | Bitwise SET operation of GPIO3 PULLSEL_SET 0: Keep 1: SET bits |
| 2 | GPIO2 | GPIO2_PULLSEL | Bitwise SET operation of GPIO2 PULLSEL_SET 0: Keep 1: SET bits |
| 1 | GPIO1 | GPIO1_PULLSEL | Bitwise SET operation of GPIO1 PULLSEL_SET 0: Keep 1: SET bits |
| 0 | GPIO0 | GPIO0_PULLSEL | Bitwise SET operation of GPIO0 PULLSEL_SET 0: Keep 1: SET bits |

A0020508	GPIO_PULLSEL GPIO Pullsel Control	00000000

0_CLR

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO3 0 |  |  |  |  | GPIO2 5 | GPIO2 4 | GPIO2 3 | GPIO2 2 |  |  |  |  |  |  |
| Type |  | WO |  |  |  |  | WO | WO | WO | WO |  |  |  |  |  |  |
| Reset |  | 0 |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  | GPIO1 1 |  | GPIO9 | GPIO8 | GPIO7 | GPIO6 | GPIO5 | GPIO4 | GPIO3 | GPIO2 | GPIO1 | GPIO0 |
| Type |  |  |  |  | WO |  | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset |  |  |  |  | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_PULLSEL0

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30 | GPIO30 | GPIO30_PULLSEL | Bitwise CKR operation of GPIO30 PULLSEL_CLR 0: Keep 1: CLR bits |
| 25 | GPIO25 | GPIO25_PULLSEL | Bitwise CKR operation of GPIO25 PULLSEL_CLR 0: Keep 1: CLR bits |
| 24 | GPIO24 | GPIO24_PULLSEL | Bitwise CKR operation of GPIO24 PULLSEL_CLR 0: Keep 1: CLR bits |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 23 | GPIO23 | GPIO23_PULLSEL | Bitwise CKR operation of GPIO23 PULLSEL_CLR 0: Keep 1: CLR bits |
| 22 | GPIO22 | GPIO22_PULLSEL | Bitwise CKR operation of GPIO22 PULLSEL_CLR 0: Keep 1: CLR bits |
| 11 | GPIO11 | GPIO11_PULLSEL | Bitwise CKR operation of GPIO11 PULLSEL_CLR 0: Keep 1: CLR bits |
| 9 | GPIO9 | GPIO9_PULLSEL | Bitwise CKR operation of GPIO9 PULLSEL_CLR 0: Keep 1: CLR bits |
| 8 | GPIO8 | GPIO8_PULLSEL | Bitwise CKR operation of GPIO8 PULLSEL_CLR 0: Keep 1: CLR bits |
| 7 | GPIO7 | GPIO7_PULLSEL | Bitwise CKR operation of GPIO7 PULLSEL_CLR 0: Keep 1: CLR bits |
| 6 | GPIO6 | GPIO6_PULLSEL | Bitwise CKR operation of GPIO6 PULLSEL_CLR 0: Keep 1: CLR bits |
| 5 | GPIO5 | GPIO5_PULLSEL | Bitwise CKR operation of GPIO5 PULLSEL_CLR 0: Keep 1: CLR bits |
| 4 | GPIO4 | GPIO4_PULLSEL | Bitwise CKR operation of GPIO4 PULLSEL_CLR 0: Keep 1: CLR bits |
| 3 | GPIO3 | GPIO3_PULLSEL | Bitwise CKR operation of GPIO3 PULLSEL_CLR 0: Keep 1: CLR bits |
| 2 | GPIO2 | GPIO2_PULLSEL | Bitwise CKR operation of GPIO2 PULLSEL_CLR 0: Keep 1: CLR bits |
| 1 | GPIO1 | GPIO1_PULLSEL | Bitwise CKR operation of GPIO1 PULLSEL_CLR 0: Keep 1: CLR bits |
| 0 | GPIO0 | GPIO0_PULLSEL | Bitwise CKR operation of GPIO0 PULLSEL_CLR 0: Keep 1: CLR bits |

A0020510	GPIO_PULLSEL GPIO Pullsel Control	00000000

1

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | GPIO5 5 | GPIO5 4 | GPIO5 3 | GPIO5 2 |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW | RW | RW | RW |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  | GPIO4 4 | GPIO4 3 |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  | RW | RW |  |  |  |  |  |  |  |  |  |  |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Reset |  |  |  | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Overview: Configures GPIO PUPD selection

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 23 | GPIO55 | GPIO55_PULLSEL | GPIO55 PULLSEL 0: Pull down 1: Pull up |
| 22 | GPIO54 | GPIO54_PULLSEL | GPIO54 PULLSEL 0: Pull down 1: Pull up |
| 21 | GPIO53 | GPIO53_PULLSEL | GPIO53 PULLSEL 0: Pull down 1: Pull up |
| 20 | GPIO52 | GPIO52_PULLSEL | GPIO52 PULLSEL 0: Pull down 1: Pull up |
| 12 | GPIO44 | GPIO44_PULLSEL | GPIO44 PULLSEL 0: Pull down 1: Pull up |
| 11 | GPIO43 | GPIO43_PULLSEL | GPIO43 PULLSEL 0: Pull down 1: Pull up |

A0020514	GPIO_PULLSEL GPIO Pullsel Control	00000000

1_SET

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | GPIO5 5 | GPIO5 4 | GPIO5 3 | GPIO5 2 |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | WO | WO | WO | WO |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  | GPIO4 4 | GPIO4 3 |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  | WO | WO |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |

Overview: For bitwise access of GPIO_PULLSEL1

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 23 | GPIO55 | GPIO55_PULLSEL | Bitwise SET operation of GPIO55 PULLSEL_SET 0: Keep 1: SET bits |
| 22 | GPIO54 | GPIO54_PULLSEL | Bitwise SET operation of GPIO54 PULLSEL_SET 0: Keep 1: SET bits |
| 21 | GPIO53 | GPIO53_PULLSEL | Bitwise SET operation of GPIO53 PULLSEL_SET 0: Keep 1: SET bits |
| 20 | GPIO52 | GPIO52_PULLSEL | Bitwise SET operation of GPIO52 PULLSEL_SET 0: Keep |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: SET bits |
| 12 | GPIO44 | GPIO44_PULLSEL | Bitwise SET operation of GPIO44 PULLSEL_SET 0: Keep 1: SET bits |
| 11 | GPIO43 | GPIO43_PULLSEL | Bitwise SET operation of GPIO43 PULLSEL_SET 0: Keep 1: SET bits |

A0020518	GPIO_PULLSEL GPIO Pullsel Control	00000000

1_CLR

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | GPIO5 5 | GPIO5 4 | GPIO5 3 | GPIO5 2 |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | WO | WO | WO | WO |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  | GPIO4 4 | GPIO4 3 |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  | WO | WO |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |

Overview: For bitwise access of GPIO_PULLSEL1

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 23 | GPIO55 | GPIO55_PULLSEL | Bitwise CKR operation of GPIO55 PULLSEL_CLR 0: Keep 1: CLR bits |
| 22 | GPIO54 | GPIO54_PULLSEL | Bitwise CKR operation of GPIO54 PULLSEL_CLR 0: Keep 1: CLR bits |
| 21 | GPIO53 | GPIO53_PULLSEL | Bitwise CKR operation of GPIO53 PULLSEL_CLR 0: Keep 1: CLR bits |
| 20 | GPIO52 | GPIO52_PULLSEL | Bitwise CKR operation of GPIO52 PULLSEL_CLR 0: Keep 1: CLR bits |
| 12 | GPIO44 | GPIO44_PULLSEL | Bitwise CKR operation of GPIO44 PULLSEL_CLR 0: Keep 1: CLR bits |
| 11 | GPIO43 | GPIO43_PULLSEL | Bitwise CKR operation of GPIO43 PULLSEL_CLR 0: Keep 1: CLR bits |

A0020600	GPIO_SMT0	GPIO SMT Control	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | GPIO3 1 | GPIO3 0 | GPIO2 9 | GPIO2 8 | GPIO2 7 | GPIO2 6 | GPIO2 5 | GPIO2 4 | GPIO2 3 | GPIO2 2 | GPIO2 1 | GPIO2 0 | GPIO1 9 | GPIO1 8 | GPIO1 7 | GPIO1 6 |
| Type | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | GPIO1 5 | GPIO1 4 | GPIO1 3 | GPIO1 2 | GPIO1 1 | GPIO1 0 | GPIO9 | GPIO8 | GPIO7 | GPIO6 | GPIO5 | GPIO4 | GPIO3 | GPIO2 | GPIO1 | GPIO0 |
| Type | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: Configures GPIO Schmit trigger control

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31 | GPIO31 | GPIO31_SMT | SMT for GPIO31 0: Disable 1: Enable |
| 30 | GPIO30 | GPIO30_SMT | SMT for GPIO30 0: Disable 1: Enable |
| 29 | GPIO29 | GPIO29_SMT | SMT for GPIO29 0: Disable 1: Enable |
| 28 | GPIO28 | GPIO28_SMT | SMT for GPIO28 0: Disable 1: Enable |
| 27 | GPIO27 | GPIO27_SMT | SMT for GPIO27 0: Disable 1: Enable |
| 26 | GPIO26 | GPIO26_SMT | SMT for GPIO26 0: Disable 1: Enable |
| 25 | GPIO25 | GPIO25_SMT | SMT for GPIO25 0: Disable 1: Enable |
| 24 | GPIO24 | GPIO24_SMT | SMT for GPIO24 0: Disable 1: Enable |
| 23 | GPIO23 | GPIO23_SMT | SMT for GPIO23 0: Disable 1: Enable |
| 22 | GPIO22 | GPIO22_SMT | SMT for GPIO22 0: Disable 1: Enable |
| 21 | GPIO21 | GPIO21_SMT | SMT for GPIO21 0: Disable 1: Enable |
| 20 | GPIO20 | GPIO20_SMT | SMT for GPIO20 0: Disable 1: Enable |
| 19 | GPIO19 | GPIO19_SMT | SMT for GPIO19 0: Disable 1: Enable |
| 18 | GPIO18 | GPIO18_SMT | SMT for GPIO18 0: Disable 1: Enable |
| 17 | GPIO17 | GPIO17_SMT | SMT for GPIO17 0: Disable |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: Enable |
| 16 | GPIO16 | GPIO16_SMT | SMT for GPIO16 0: Disable 1: Enable |
| 15 | GPIO15 | GPIO15_SMT | SMT for GPIO15 0: Disable 1: Enable |
| 14 | GPIO14 | GPIO14_SMT | SMT for GPIO14 0: Disable 1: Enable |
| 13 | GPIO13 | GPIO13_SMT | SMT for GPIO13 0: Disable 1: Enable |
| 12 | GPIO12 | GPIO12_SMT | SMT for GPIO12 0: Disable 1: Enable |
| 11 | GPIO11 | GPIO11_SMT | SMT for GPIO11 0: Disable 1: Enable |
| 10 | GPIO10 | GPIO10_SMT | SMT for GPIO10 0: Disable 1: Enable |
| 9 | GPIO9 | GPIO9_SMT | SMT for GPIO9 0: Disable 1: Enable |
| 8 | GPIO8 | GPIO8_SMT | SMT for GPIO8 0: Disable 1: Enable |
| 7 | GPIO7 | GPIO7_SMT | SMT for GPIO7 0: Disable 1: Enable |
| 6 | GPIO6 | GPIO6_SMT | SMT for GPIO6 0: Disable 1: Enable |
| 5 | GPIO5 | GPIO5_SMT | SMT for GPIO5 0: Disable 1: Enable |
| 4 | GPIO4 | GPIO4_SMT | SMT for GPIO4 0: Disable 1: Enable |
| 3 | GPIO3 | GPIO3_SMT | SMT for GPIO3 0: Disable 1: Enable |
| 2 | GPIO2 | GPIO2_SMT | SMT for GPIO2 0: Disable 1: Enable |
| 1 | GPIO1 | GPIO1_SMT | SMT for GPIO1 0: Disable 1: Enable |
| 0 | GPIO0 | GPIO0_SMT | SMT for GPIO0 0: Disable 1: Enable |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A0020604	GPIO_SMT0_SE GPIO SMT Control	00000000

T

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | GPIO3 1 | GPIO3 0 | GPIO2 9 | GPIO2 8 | GPIO2 7 | GPIO2 6 | GPIO2 5 | GPIO2 4 | GPIO2 3 | GPIO2 2 | GPIO2 1 | GPIO2 0 | GPIO1 9 | GPIO1 8 | GPIO1 7 | GPIO1 6 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO1 5 | GPIO1 4 | GPIO1 3 | GPIO1 2 | GPIO1 1 | GPIO1 0 | GPIO9 | GPIO8 | GPIO7 | GPIO6 | GPIO5 | GPIO4 | GPIO3 | GPIO2 | GPIO1 | GPIO0 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_SMT0

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31 | GPIO31 | GPIO31_SMT | Bitwise SET operation of GPIO31 SMT 0: Keep 1: SET bits |
| 30 | GPIO30 | GPIO30_SMT | Bitwise SET operation of GPIO30 SMT 0: Keep 1: SET bits |
| 29 | GPIO29 | GPIO29_SMT | Bitwise SET operation of GPIO29 SMT 0: Keep 1: SET bits |
| 28 | GPIO28 | GPIO28_SMT | Bitwise SET operation of GPIO28 SMT 0: Keep 1: SET bits |
| 27 | GPIO27 | GPIO27_SMT | Bitwise SET operation of GPIO27 SMT 0: Keep 1: SET bits |
| 26 | GPIO26 | GPIO26_SMT | Bitwise SET operation of GPIO26 SMT 0: Keep 1: SET bits |
| 25 | GPIO25 | GPIO25_SMT | Bitwise SET operation of GPIO25 SMT 0: Keep 1: SET bits |
| 24 | GPIO24 | GPIO24_SMT | Bitwise SET operation of GPIO24 SMT 0: Keep 1: SET bits |
| 23 | GPIO23 | GPIO23_SMT | Bitwise SET operation of GPIO23 SMT 0: Keep 1: SET bits |
| 22 | GPIO22 | GPIO22_SMT | Bitwise SET operation of GPIO22 SMT 0: Keep 1: SET bits |
| 21 | GPIO21 | GPIO21_SMT | Bitwise SET operation of GPIO21 SMT 0: Keep 1: SET bits |
| 20 | GPIO20 | GPIO20_SMT | Bitwise SET operation of GPIO20 SMT 0: Keep |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: SET bits |
| 19 | GPIO19 | GPIO19_SMT | Bitwise SET operation of GPIO19 SMT 0: Keep 1: SET bits |
| 18 | GPIO18 | GPIO18_SMT | Bitwise SET operation of GPIO18 SMT 0: Keep 1: SET bits |
| 17 | GPIO17 | GPIO17_SMT | Bitwise SET operation of GPIO17 SMT 0: Keep 1: SET bits |
| 16 | GPIO16 | GPIO16_SMT | Bitwise SET operation of GPIO16 SMT 0: Keep 1: SET bits |
| 15 | GPIO15 | GPIO15_SMT | Bitwise SET operation of GPIO15 SMT 0: Keep 1: SET bits |
| 14 | GPIO14 | GPIO14_SMT | Bitwise SET operation of GPIO14 SMT 0: Keep 1: SET bits |
| 13 | GPIO13 | GPIO13_SMT | Bitwise SET operation of GPIO13 SMT 0: Keep 1: SET bits |
| 12 | GPIO12 | GPIO12_SMT | Bitwise SET operation of GPIO12 SMT 0: Keep 1: SET bits |
| 11 | GPIO11 | GPIO11_SMT | Bitwise SET operation of GPIO11 SMT 0: Keep 1: SET bits |
| 10 | GPIO10 | GPIO10_SMT | Bitwise SET operation of GPIO10 SMT 0: Keep 1: SET bits |
| 9 | GPIO9 | GPIO9_SMT | Bitwise SET operation of GPIO9 SMT 0: Keep 1: SET bits |
| 8 | GPIO8 | GPIO8_SMT | Bitwise SET operation of GPIO8 SMT 0: Keep 1: SET bits |
| 7 | GPIO7 | GPIO7_SMT | Bitwise SET operation of GPIO7 SMT 0: Keep 1: SET bits |
| 6 | GPIO6 | GPIO6_SMT | Bitwise SET operation of GPIO6 SMT 0: Keep 1: SET bits |
| 5 | GPIO5 | GPIO5_SMT | Bitwise SET operation of GPIO5 SMT 0: Keep 1: SET bits |
| 4 | GPIO4 | GPIO4_SMT | Bitwise SET operation of GPIO4 SMT 0: Keep 1: SET bits |
| 3 | GPIO3 | GPIO3_SMT | Bitwise SET operation of GPIO3 SMT 0: Keep 1: SET bits |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 2 | GPIO2 | GPIO2_SMT | Bitwise SET operation of GPIO2 SMT 0: Keep 1: SET bits |
| 1 | GPIO1 | GPIO1_SMT | Bitwise SET operation of GPIO1 SMT 0: Keep 1: SET bits |
| 0 | GPIO0 | GPIO0_SMT | Bitwise SET operation of GPIO0 SMT 0: Keep 1: SET bits |

A0020608	GPIO_SMT0_CL GPIO SMT Control	00000000

R

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | GPIO3 1 | GPIO3 0 | GPIO2 9 | GPIO2 8 | GPIO2 7 | GPIO2 6 | GPIO2 5 | GPIO2 4 | GPIO2 3 | GPIO2 2 | GPIO2 1 | GPIO2 0 | GPIO1 9 | GPIO1 8 | GPIO1 7 | GPIO1 6 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO1 5 | GPIO1 4 | GPIO1 3 | GPIO1 2 | GPIO1 1 | GPIO1 0 | GPIO9 | GPIO8 | GPIO7 | GPIO6 | GPIO5 | GPIO4 | GPIO3 | GPIO2 | GPIO1 | GPIO0 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_SMT0

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31 | GPIO31 | GPIO31_SMT | Bitwise CLR operation of GPIO31 SMT 0: Keep 1: CLR bits |
| 30 | GPIO30 | GPIO30_SMT | Bitwise CLR operation of GPIO30 SMT 0: Keep 1: CLR bits |
| 29 | GPIO29 | GPIO29_SMT | Bitwise CLR operation of GPIO29 SMT 0: Keep 1: CLR bits |
| 28 | GPIO28 | GPIO28_SMT | Bitwise CLR operation of GPIO28 SMT 0: Keep 1: CLR bits |
| 27 | GPIO27 | GPIO27_SMT | Bitwise CLR operation of GPIO27 SMT 0: Keep 1: CLR bits |
| 26 | GPIO26 | GPIO26_SMT | Bitwise CLR operation of GPIO26 SMT 0: Keep 1: CLR bits |
| 25 | GPIO25 | GPIO25_SMT | Bitwise CLR operation of GPIO25 SMT 0: Keep 1: CLR bits |
| 24 | GPIO24 | GPIO24_SMT | Bitwise CLR operation of GPIO24 SMT 0: Keep 1: CLR bits |
| 23 | GPIO23 | GPIO23_SMT | Bitwise CLR operation of GPIO23 SMT |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Keep 1: CLR bits |
| 22 | GPIO22 | GPIO22_SMT | Bitwise CLR operation of GPIO22 SMT 0: Keep 1: CLR bits |
| 21 | GPIO21 | GPIO21_SMT | Bitwise CLR operation of GPIO21 SMT 0: Keep 1: CLR bits |
| 20 | GPIO20 | GPIO20_SMT | Bitwise CLR operation of GPIO20 SMT 0: Keep 1: CLR bits |
| 19 | GPIO19 | GPIO19_SMT | Bitwise CLR operation of GPIO19 SMT 0: Keep 1: CLR bits |
| 18 | GPIO18 | GPIO18_SMT | Bitwise CLR operation of GPIO18 SMT 0: Keep 1: CLR bits |
| 17 | GPIO17 | GPIO17_SMT | Bitwise CLR operation of GPIO17 SMT 0: Keep 1: CLR bits |
| 16 | GPIO16 | GPIO16_SMT | Bitwise CLR operation of GPIO16 SMT 0: Keep 1: CLR bits |
| 15 | GPIO15 | GPIO15_SMT | Bitwise CLR operation of GPIO15 SMT 0: Keep 1: CLR bits |
| 14 | GPIO14 | GPIO14_SMT | Bitwise CLR operation of GPIO14 SMT 0: Keep 1: CLR bits |
| 13 | GPIO13 | GPIO13_SMT | Bitwise CLR operation of GPIO13 SMT 0: Keep 1: CLR bits |
| 12 | GPIO12 | GPIO12_SMT | Bitwise CLR operation of GPIO12 SMT 0: Keep 1: CLR bits |
| 11 | GPIO11 | GPIO11_SMT | Bitwise CLR operation of GPIO11 SMT 0: Keep 1: CLR bits |
| 10 | GPIO10 | GPIO10_SMT | Bitwise CLR operation of GPIO10 SMT 0: Keep 1: CLR bits |
| 9 | GPIO9 | GPIO9_SMT | Bitwise CLR operation of GPIO9 SMT 0: Keep 1: CLR bits |
| 8 | GPIO8 | GPIO8_SMT | Bitwise CLR operation of GPIO8 SMT 0: Keep 1: CLR bits |
| 7 | GPIO7 | GPIO7_SMT | Bitwise CLR operation of GPIO7 SMT 0: Keep 1: CLR bits |
| 6 | GPIO6 | GPIO6_SMT | Bitwise CLR operation of GPIO6 SMT 0: Keep |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: CLR bits |
| 5 | GPIO5 | GPIO5_SMT | Bitwise CLR operation of GPIO5 SMT 0: Keep 1: CLR bits |
| 4 | GPIO4 | GPIO4_SMT | Bitwise CLR operation of GPIO4 SMT 0: Keep 1: CLR bits |
| 3 | GPIO3 | GPIO3_SMT | Bitwise CLR operation of GPIO3 SMT 0: Keep 1: CLR bits |
| 2 | GPIO2 | GPIO2_SMT | Bitwise CLR operation of GPIO2 SMT 0: Keep 1: CLR bits |
| 1 | GPIO1 | GPIO1_SMT | Bitwise CLR operation of GPIO1 SMT 0: Keep 1: CLR bits |
| 0 | GPIO0 | GPIO0_SMT | Bitwise CLR operation of GPIO0 SMT 0: Keep 1: CLR bits |

A0020610	GPIO_SMT1	GPIO SMT Control	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | GPIO5 1 | GPIO5 0 | GPIO4 9 | GPIO4 8 |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO4 7 | GPIO4 6 | GPIO4 5 | GPIO4 4 | GPIO4 3 | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 | GPIO3 6 | GPIO3 5 | GPIO3 4 | GPIO3 3 | GPIO3 2 |
| Type | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: Configures GPIO Schmit trigger control

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 19 | GPIO51 | GPIO51_SMT | SMT for GPIO51 0: Disable 1: Enable |
| 18 | GPIO50 | GPIO50_SMT | SMT for GPIO50 0: Disable 1: Enable |
| 17 | GPIO49 | GPIO49_SMT | SMT for GPIO49 0: Disable 1: Enable |
| 16 | GPIO48 | GPIO48_SMT | SMT for GPIO48 0: Disable 1: Enable |
| 15 | GPIO47 | GPIO47_SMT | SMT for GPIO47 0: Disable 1: Enable |
| 14 | GPIO46 | GPIO46_SMT | SMT for GPIO46 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Disable 1: Enable |
| 13 | GPIO45 | GPIO45_SMT | SMT for GPIO45 0: Disable 1: Enable |
| 12 | GPIO44 | GPIO44_SMT | SMT for GPIO44 0: Disable 1: Enable |
| 11 | GPIO43 | GPIO43_SMT | SMT for GPIO43 0: Disable 1: Enable |
| 10 | GPIO42 | GPIO42_SMT | SMT for GPIO42 0: Disable 1: Enable |
| 9 | GPIO41 | GPIO41_SMT | SMT for GPIO41 0: Disable 1: Enable |
| 8 | GPIO40 | GPIO40_SMT | SMT for GPIO40 0: Disable 1: Enable |
| 7 | GPIO39 | GPIO39_SMT | SMT for GPIO39 0: Disable 1: Enable |
| 6 | GPIO38 | GPIO38_SMT | SMT for GPIO38 0: Disable 1: Enable |
| 5 | GPIO37 | GPIO37_SMT | SMT for GPIO37 0: Disable 1: Enable |
| 4 | GPIO36 | GPIO36_SMT | SMT for GPIO36 0: Disable 1: Enable |
| 3 | GPIO35 | GPIO35_SMT | SMT for GPIO35 0: Disable 1: Enable |
| 2 | GPIO34 | GPIO34_SMT | SMT for GPIO34 0: Disable 1: Enable |
| 1 | GPIO33 | GPIO33_SMT | SMT for GPIO33 0: Disable 1: Enable |
| 0 | GPIO32 | GPIO32_SMT | SMT for GPIO32 0: Disable 1: Enable |

A0020614	GPIO_SMT1_SE GPIO SMT Control	00000000

T

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | GPIO5 1 | GPIO5 0 | GPIO4 9 | GPIO4 8 |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | WO | WO | WO | WO |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO4 7 | GPIO4 6 | GPIO4 5 | GPIO4 4 | GPIO4 3 | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 | GPIO3 6 | GPIO3 5 | GPIO3 4 | GPIO3 3 | GPIO3 2 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_SMT1

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 19 | GPIO51 | GPIO51_SMT | Bitwise SET operation of GPIO51 SMT 0: Keep 1: SET bits |
| 18 | GPIO50 | GPIO50_SMT | Bitwise SET operation of GPIO50 SMT 0: Keep 1: SET bits |
| 17 | GPIO49 | GPIO49_SMT | Bitwise SET operation of GPIO49 SMT 0: Keep 1: SET bits |
| 16 | GPIO48 | GPIO48_SMT | Bitwise SET operation of GPIO48 SMT 0: Keep 1: SET bits |
| 15 | GPIO47 | GPIO47_SMT | Bitwise SET operation of GPIO47 SMT 0: Keep 1: SET bits |
| 14 | GPIO46 | GPIO46_SMT | Bitwise SET operation of GPIO46 SMT 0: Keep 1: SET bits |
| 13 | GPIO45 | GPIO45_SMT | Bitwise SET operation of GPIO45 SMT 0: Keep 1: SET bits |
| 12 | GPIO44 | GPIO44_SMT | Bitwise SET operation of GPIO44 SMT 0: Keep 1: SET bits |
| 11 | GPIO43 | GPIO43_SMT | Bitwise SET operation of GPIO43 SMT 0: Keep 1: SET bits |
| 10 | GPIO42 | GPIO42_SMT | Bitwise SET operation of GPIO42 SMT 0: Keep 1: SET bits |
| 9 | GPIO41 | GPIO41_SMT | Bitwise SET operation of GPIO41 SMT 0: Keep 1: SET bits |
| 8 | GPIO40 | GPIO40_SMT | Bitwise SET operation of GPIO40 SMT 0: Keep 1: SET bits |
| 7 | GPIO39 | GPIO39_SMT | Bitwise SET operation of GPIO39 SMT 0: Keep 1: SET bits |
| 6 | GPIO38 | GPIO38_SMT | Bitwise SET operation of GPIO38 SMT 0: Keep 1: SET bits |
| 5 | GPIO37 | GPIO37_SMT | Bitwise SET operation of GPIO37 SMT |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Keep 1: SET bits |
| 4 | GPIO36 | GPIO36_SMT | Bitwise SET operation of GPIO36 SMT 0: Keep 1: SET bits |
| 3 | GPIO35 | GPIO35_SMT | Bitwise SET operation of GPIO35 SMT 0: Keep 1: SET bits |
| 2 | GPIO34 | GPIO34_SMT | Bitwise SET operation of GPIO34 SMT 0: Keep 1: SET bits |
| 1 | GPIO33 | GPIO33_SMT | Bitwise SET operation of GPIO33 SMT 0: Keep 1: SET bits |
| 0 | GPIO32 | GPIO32_SMT | Bitwise SET operation of GPIO32 SMT 0: Keep 1: SET bits |

A0020618	GPIO_SMT1_CL GPIO SMT Control	00000000

R

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | GPIO5 1 | GPIO5 0 | GPIO4 9 | GPIO4 8 |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | WO | WO | WO | WO |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO4 7 | GPIO4 6 | GPIO4 5 | GPIO4 4 | GPIO4 3 | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 | GPIO3 6 | GPIO3 5 | GPIO3 4 | GPIO3 3 | GPIO3 2 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_SMT1

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 19 | GPIO51 | GPIO51_SMT | Bitwise CLR operation of GPIO51 SMT 0: Keep 1: CLR bits |
| 18 | GPIO50 | GPIO50_SMT | Bitwise CLR operation of GPIO50 SMT 0: Keep 1: CLR bits |
| 17 | GPIO49 | GPIO49_SMT | Bitwise CLR operation of GPIO49 SMT 0: Keep 1: CLR bits |
| 16 | GPIO48 | GPIO48_SMT | Bitwise CLR operation of GPIO48 SMT 0: Keep 1: CLR bits |
| 15 | GPIO47 | GPIO47_SMT | Bitwise CLR operation of GPIO47 SMT 0: Keep 1: CLR bits |
| 14 | GPIO46 | GPIO46_SMT | Bitwise CLR operation of GPIO46 SMT 0: Keep |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: CLR bits |
| 13 | GPIO45 | GPIO45_SMT | Bitwise CLR operation of GPIO45 SMT 0: Keep 1: CLR bits |
| 12 | GPIO44 | GPIO44_SMT | Bitwise CLR operation of GPIO44 SMT 0: Keep 1: CLR bits |
| 11 | GPIO43 | GPIO43_SMT | Bitwise CLR operation of GPIO43 SMT 0: Keep 1: CLR bits |
| 10 | GPIO42 | GPIO42_SMT | Bitwise CLR operation of GPIO42 SMT 0: Keep 1: CLR bits |
| 9 | GPIO41 | GPIO41_SMT | Bitwise CLR operation of GPIO41 SMT 0: Keep 1: CLR bits |
| 8 | GPIO40 | GPIO40_SMT | Bitwise CLR operation of GPIO40 SMT 0: Keep 1: CLR bits |
| 7 | GPIO39 | GPIO39_SMT | Bitwise CLR operation of GPIO39 SMT 0: Keep 1: CLR bits |
| 6 | GPIO38 | GPIO38_SMT | Bitwise CLR operation of GPIO38 SMT 0: Keep 1: CLR bits |
| 5 | GPIO37 | GPIO37_SMT | Bitwise CLR operation of GPIO37 SMT 0: Keep 1: CLR bits |
| 4 | GPIO36 | GPIO36_SMT | Bitwise CLR operation of GPIO36 SMT 0: Keep 1: CLR bits |
| 3 | GPIO35 | GPIO35_SMT | Bitwise CLR operation of GPIO35 SMT 0: Keep 1: CLR bits |
| 2 | GPIO34 | GPIO34_SMT | Bitwise CLR operation of GPIO34 SMT 0: Keep 1: CLR bits |
| 1 | GPIO33 | GPIO33_SMT | Bitwise CLR operation of GPIO33 SMT 0: Keep 1: CLR bits |
| 0 | GPIO32 | GPIO32_SMT | Bitwise CLR operation of GPIO32 SMT 0: Keep 1: CLR bits |

A0020700	GPIO_SR0	GPIO SR Control	FFFFFFFF

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | GPIO3 1 | GPIO3 0 | GPIO2 9 | GPIO2 8 | GPIO2 7 | GPIO2 6 | GPIO2 5 | GPIO2 4 | GPIO2 3 | GPIO2 2 | GPIO2 1 | GPIO2 0 | GPIO1 9 | GPIO1 8 | GPIO1 7 | GPIO1 6 |
| Type | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW |
| Reset | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Name | GPIO1 5 | GPIO1 4 | GPIO1 3 | GPIO1 2 | GPIO1 1 | GPIO1 0 | GPIO9 | GPIO8 | GPIO7 | GPIO6 | GPIO5 | GPIO4 | GPIO3 | GPIO2 | GPIO1 | GPIO0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW |
| Reset | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Overview: Configures GPIO slew rate control

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31 | GPIO31 | GPIO31_SR | SR for GPIO31 0: Disable 1: Enable |
| 30 | GPIO30 | GPIO30_SR | SR for GPIO30 0: Disable 1: Enable |
| 29 | GPIO29 | GPIO29_SR | SR for GPIO29 0: Disable 1: Enable |
| 28 | GPIO28 | GPIO28_SR | SR for GPIO28 0: Disable 1: Enable |
| 27 | GPIO27 | GPIO27_SR | SR for GPIO27 0: Disable 1: Enable |
| 26 | GPIO26 | GPIO26_SR | SR for GPIO26 0: Disable 1: Enable |
| 25 | GPIO25 | GPIO25_SR | SR for GPIO25 0: Disable 1: Enable |
| 24 | GPIO24 | GPIO24_SR | SR for GPIO24 0: Disable 1: Enable |
| 23 | GPIO23 | GPIO23_SR | SR for GPIO23 0: Disable 1: Enable |
| 22 | GPIO22 | GPIO22_SR | SR for GPIO22 0: Disable 1: Enable |
| 21 | GPIO21 | GPIO21_SR | SR for GPIO21 0: Disable 1: Enable |
| 20 | GPIO20 | GPIO20_SR | SR for GPIO20 0: Disable 1: Enable |
| 19 | GPIO19 | GPIO19_SR | SR for GPIO19 0: Disable 1: Enable |
| 18 | GPIO18 | GPIO18_SR | SR for GPIO18 0: Disable 1: Enable |
| 17 | GPIO17 | GPIO17_SR | SR for GPIO17 0: Disable 1: Enable |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 16 | GPIO16 | GPIO16_SR | SR for GPIO16 0: Disable 1: Enable |
| 15 | GPIO15 | GPIO15_SR | SR for GPIO15 0: Disable 1: Enable |
| 14 | GPIO14 | GPIO14_SR | SR for GPIO14 0: Disable 1: Enable |
| 13 | GPIO13 | GPIO13_SR | SR for GPIO13 0: Disable 1: Enable |
| 12 | GPIO12 | GPIO12_SR | SR for GPIO12 0: Disable 1: Enable |
| 11 | GPIO11 | GPIO11_SR | SR for GPIO11 0: Disable 1: Enable |
| 10 | GPIO10 | GPIO10_SR | SR for GPIO10 0: Disable 1: Enable |
| 9 | GPIO9 | GPIO9_SR | SR for GPIO9 0: Disable 1: Enable |
| 8 | GPIO8 | GPIO8_SR | SR for GPIO8 0: Disable 1: Enable |
| 7 | GPIO7 | GPIO7_SR | SR for GPIO7 0: Disable 1: Enable |
| 6 | GPIO6 | GPIO6_SR | SR for GPIO6 0: Disable 1: Enable |
| 5 | GPIO5 | GPIO5_SR | SR for GPIO5 0: Disable 1: Enable |
| 4 | GPIO4 | GPIO4_SR | SR for GPIO4 0: Disable 1: Enable |
| 3 | GPIO3 | GPIO3_SR | SR for GPIO3 0: Disable 1: Enable |
| 2 | GPIO2 | GPIO2_SR | SR for GPIO2 0: Disable 1: Enable |
| 1 | GPIO1 | GPIO1_SR | SR for GPIO1 0: Disable 1: Enable |
| 0 | GPIO0 | GPIO0_SR | SR for GPIO0 0: Disable 1: Enable |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A0020704	GPIO_SR0_SET GPIO SR Control	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | GPIO3 1 | GPIO3 0 | GPIO2 9 | GPIO2 8 | GPIO2 7 | GPIO2 6 | GPIO2 5 | GPIO2 4 | GPIO2 3 | GPIO2 2 | GPIO2 1 | GPIO2 0 | GPIO1 9 | GPIO1 8 | GPIO1 7 | GPIO1 6 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO1 5 | GPIO1 4 | GPIO1 3 | GPIO1 2 | GPIO1 1 | GPIO1 0 | GPIO9 | GPIO8 | GPIO7 | GPIO6 | GPIO5 | GPIO4 | GPIO3 | GPIO2 | GPIO1 | GPIO0 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_SR0

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31 | GPIO31 | GPIO31_SR | Bitwise SET operation of GPIO31 SR 0: Keep 1: SET bits |
| 30 | GPIO30 | GPIO30_SR | Bitwise SET operation of GPIO30 SR 0: Keep 1: SET bits |
| 29 | GPIO29 | GPIO29_SR | Bitwise SET operation of GPIO29 SR 0: Keep 1: SET bits |
| 28 | GPIO28 | GPIO28_SR | Bitwise SET operation of GPIO28 SR 0: Keep 1: SET bits |
| 27 | GPIO27 | GPIO27_SR | Bitwise SET operation of GPIO27 SR 0: Keep 1: SET bits |
| 26 | GPIO26 | GPIO26_SR | Bitwise SET operation of GPIO26 SR 0: Keep 1: SET bits |
| 25 | GPIO25 | GPIO25_SR | Bitwise SET operation of GPIO25 SR 0: Keep 1: SET bits |
| 24 | GPIO24 | GPIO24_SR | Bitwise SET operation of GPIO24 SR 0: Keep 1: SET bits |
| 23 | GPIO23 | GPIO23_SR | Bitwise SET operation of GPIO23 SR 0: Keep 1: SET bits |
| 22 | GPIO22 | GPIO22_SR | Bitwise SET operation of GPIO22 SR 0: Keep 1: SET bits |
| 21 | GPIO21 | GPIO21_SR | Bitwise SET operation of GPIO21 SR 0: Keep 1: SET bits |
| 20 | GPIO20 | GPIO20_SR | Bitwise SET operation of GPIO20 SR 0: Keep 1: SET bits |
| 19 | GPIO19 | GPIO19_SR | Bitwise SET operation of GPIO19 SR |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Keep 1: SET bits |
| 18 | GPIO18 | GPIO18_SR | Bitwise SET operation of GPIO18 SR 0: Keep 1: SET bits |
| 17 | GPIO17 | GPIO17_SR | Bitwise SET operation of GPIO17 SR 0: Keep 1: SET bits |
| 16 | GPIO16 | GPIO16_SR | Bitwise SET operation of GPIO16 SR 0: Keep 1: SET bits |
| 15 | GPIO15 | GPIO15_SR | Bitwise SET operation of GPIO15 SR 0: Keep 1: SET bits |
| 14 | GPIO14 | GPIO14_SR | Bitwise SET operation of GPIO14 SR 0: Keep 1: SET bits |
| 13 | GPIO13 | GPIO13_SR | Bitwise SET operation of GPIO13 SR 0: Keep 1: SET bits |
| 12 | GPIO12 | GPIO12_SR | Bitwise SET operation of GPIO12 SR 0: Keep 1: SET bits |
| 11 | GPIO11 | GPIO11_SR | Bitwise SET operation of GPIO11 SR 0: Keep 1: SET bits |
| 10 | GPIO10 | GPIO10_SR | Bitwise SET operation of GPIO10 SR 0: Keep 1: SET bits |
| 9 | GPIO9 | GPIO9_SR | Bitwise SET operation of GPIO9 SR 0: Keep 1: SET bits |
| 8 | GPIO8 | GPIO8_SR | Bitwise SET operation of GPIO8 SR 0: Keep 1: SET bits |
| 7 | GPIO7 | GPIO7_SR | Bitwise SET operation of GPIO7 SR 0: Keep 1: SET bits |
| 6 | GPIO6 | GPIO6_SR | Bitwise SET operation of GPIO6 SR 0: Keep 1: SET bits |
| 5 | GPIO5 | GPIO5_SR | Bitwise SET operation of GPIO5 SR 0: Keep 1: SET bits |
| 4 | GPIO4 | GPIO4_SR | Bitwise SET operation of GPIO4 SR 0: Keep 1: SET bits |
| 3 | GPIO3 | GPIO3_SR | Bitwise SET operation of GPIO3 SR 0: Keep 1: SET bits |
| 2 | GPIO2 | GPIO2_SR | Bitwise SET operation of GPIO2 SR 0: Keep |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: SET bits |
| 1 | GPIO1 | GPIO1_SR | Bitwise SET operation of GPIO1 SR 0: Keep 1: SET bits |
| 0 | GPIO0 | GPIO0_SR | Bitwise SET operation of GPIO0 SR 0: Keep 1: SET bits |

A0020708	GPIO_SR0_CLR GPIO SR Control	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | GPIO3 1 | GPIO3 0 | GPIO2 9 | GPIO2 8 | GPIO2 7 | GPIO2 6 | GPIO2 5 | GPIO2 4 | GPIO2 3 | GPIO2 2 | GPIO2 1 | GPIO2 0 | GPIO1 9 | GPIO1 8 | GPIO1 7 | GPIO1 6 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO1 5 | GPIO1 4 | GPIO1 3 | GPIO1 2 | GPIO1 1 | GPIO1 0 | GPIO9 | GPIO8 | GPIO7 | GPIO6 | GPIO5 | GPIO4 | GPIO3 | GPIO2 | GPIO1 | GPIO0 |
| Type | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_SR0

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31 | GPIO31 | GPIO31_SR | Bitwise CLR operation of GPIO31 SR 0: Keep 1: CLR bits |
| 30 | GPIO30 | GPIO30_SR | Bitwise CLR operation of GPIO30 SR 0: Keep 1: CLR bits |
| 29 | GPIO29 | GPIO29_SR | Bitwise CLR operation of GPIO29 SR 0: Keep 1: CLR bits |
| 28 | GPIO28 | GPIO28_SR | Bitwise CLR operation of GPIO28 SR 0: Keep 1: CLR bits |
| 27 | GPIO27 | GPIO27_SR | Bitwise CLR operation of GPIO27 SR 0: Keep 1: CLR bits |
| 26 | GPIO26 | GPIO26_SR | Bitwise CLR operation of GPIO26 SR 0: Keep 1: CLR bits |
| 25 | GPIO25 | GPIO25_SR | Bitwise CLR operation of GPIO25 SR 0: Keep 1: CLR bits |
| 24 | GPIO24 | GPIO24_SR | Bitwise CLR operation of GPIO24 SR 0: Keep 1: CLR bits |
| 23 | GPIO23 | GPIO23_SR | Bitwise CLR operation of GPIO23 SR 0: Keep 1: CLR bits |
| 22 | GPIO22 | GPIO22_SR | Bitwise CLR operation of GPIO22 SR |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Keep 1: CLR bits |
| 21 | GPIO21 | GPIO21_SR | Bitwise CLR operation of GPIO21 SR 0: Keep 1: CLR bits |
| 20 | GPIO20 | GPIO20_SR | Bitwise CLR operation of GPIO20 SR 0: Keep 1: CLR bits |
| 19 | GPIO19 | GPIO19_SR | Bitwise CLR operation of GPIO19 SR 0: Keep 1: CLR bits |
| 18 | GPIO18 | GPIO18_SR | Bitwise CLR operation of GPIO18 SR 0: Keep 1: CLR bits |
| 17 | GPIO17 | GPIO17_SR | Bitwise CLR operation of GPIO17 SR 0: Keep 1: CLR bits |
| 16 | GPIO16 | GPIO16_SR | Bitwise CLR operation of GPIO16 SR 0: Keep 1: CLR bits |
| 15 | GPIO15 | GPIO15_SR | Bitwise CLR operation of GPIO15 SR 0: Keep 1: CLR bits |
| 14 | GPIO14 | GPIO14_SR | Bitwise CLR operation of GPIO14 SR 0: Keep 1: CLR bits |
| 13 | GPIO13 | GPIO13_SR | Bitwise CLR operation of GPIO13 SR 0: Keep 1: CLR bits |
| 12 | GPIO12 | GPIO12_SR | Bitwise CLR operation of GPIO12 SR 0: Keep 1: CLR bits |
| 11 | GPIO11 | GPIO11_SR | Bitwise CLR operation of GPIO11 SR 0: Keep 1: CLR bits |
| 10 | GPIO10 | GPIO10_SR | Bitwise CLR operation of GPIO10 SR 0: Keep 1: CLR bits |
| 9 | GPIO9 | GPIO9_SR | Bitwise CLR operation of GPIO9 SR 0: Keep 1: CLR bits |
| 8 | GPIO8 | GPIO8_SR | Bitwise CLR operation of GPIO8 SR 0: Keep 1: CLR bits |
| 7 | GPIO7 | GPIO7_SR | Bitwise CLR operation of GPIO7 SR 0: Keep 1: CLR bits |
| 6 | GPIO6 | GPIO6_SR | Bitwise CLR operation of GPIO6 SR 0: Keep 1: CLR bits |
| 5 | GPIO5 | GPIO5_SR | Bitwise CLR operation of GPIO5 SR 0: Keep |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: CLR bits |
| 4 | GPIO4 | GPIO4_SR | Bitwise CLR operation of GPIO4 SR 0: Keep 1: CLR bits |
| 3 | GPIO3 | GPIO3_SR | Bitwise CLR operation of GPIO3 SR 0: Keep 1: CLR bits |
| 2 | GPIO2 | GPIO2_SR | Bitwise CLR operation of GPIO2 SR 0: Keep 1: CLR bits |
| 1 | GPIO1 | GPIO1_SR | Bitwise CLR operation of GPIO1 SR 0: Keep 1: CLR bits |
| 0 | GPIO0 | GPIO0_SR | Bitwise CLR operation of GPIO0 SR 0: Keep 1: CLR bits |

A0020710	GPIO_SR1	GPIO SR Control	000FF81F

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | GPIO5 1 | GPIO5 0 | GPIO4 9 | GPIO4 8 |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 | 1 | 1 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO4 7 | GPIO4 6 | GPIO4 5 | GPIO4 4 | GPIO4 3 |  |  |  |  |  |  | GPIO3 6 | GPIO3 5 | GPIO3 4 | GPIO3 3 | GPIO3 2 |
| Type | RW | RW | RW | RW | RW |  |  |  |  |  |  | RW | RW | RW | RW | RW |
| Reset | 1 | 1 | 1 | 1 | 1 |  |  |  |  |  |  | 1 | 1 | 1 | 1 | 1 |

Overview: Configures GPIO slew rate control

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 19 | GPIO51 | GPIO51_SR | SR for GPIO51 0: Disable 1: Enable |
| 18 | GPIO50 | GPIO50_SR | SR for GPIO50 0: Disable 1: Enable |
| 17 | GPIO49 | GPIO49_SR | SR for GPIO49 0: Disable 1: Enable |
| 16 | GPIO48 | GPIO48_SR | SR for GPIO48 0: Disable 1: Enable |
| 15 | GPIO47 | GPIO47_SR | SR for GPIO47 0: Disable 1: Enable |
| 14 | GPIO46 | GPIO46_SR | SR for GPIO46 0: Disable 1: Enable |
| 13 | GPIO45 | GPIO45_SR | SR for GPIO45 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Disable 1: Enable |
| 12 | GPIO44 | GPIO44_SR | SR for GPIO44 0: Disable 1: Enable |
| 11 | GPIO43 | GPIO43_SR | SR for GPIO43 0: Disable 1: Enable |
| 4 | GPIO36 | GPIO36_SR | SR for GPIO36 0: Disable 1: Enable |
| 3 | GPIO35 | GPIO35_SR | SR for GPIO35 0: Disable 1: Enable |
| 2 | GPIO34 | GPIO34_SR | SR for GPIO34 0: Disable 1: Enable |
| 1 | GPIO33 | GPIO33_SR | SR for GPIO33 0: Disable 1: Enable |
| 0 | GPIO32 | GPIO32_SR | SR for GPIO32 0: Disable 1: Enable |

A0020714	GPIO_SR1_SET GPIO SR Control	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | GPIO5 1 | GPIO5 0 | GPIO4 9 | GPIO4 8 |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | WO | WO | WO | WO |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO4 7 | GPIO4 6 | GPIO4 5 | GPIO4 4 | GPIO4 3 |  |  |  |  |  |  | GPIO3 6 | GPIO3 5 | GPIO3 4 | GPIO3 3 | GPIO3 2 |
| Type | WO | WO | WO | WO | WO |  |  |  |  |  |  | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_SR1

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 19 | GPIO51 | GPIO51_SR | Bitwise SET operation of GPIO51 SR 0: Keep 1: SET bits |
| 18 | GPIO50 | GPIO50_SR | Bitwise SET operation of GPIO50 SR 0: Keep 1: SET bits |
| 17 | GPIO49 | GPIO49_SR | Bitwise SET operation of GPIO49 SR 0: Keep 1: SET bits |
| 16 | GPIO48 | GPIO48_SR | Bitwise SET operation of GPIO48 SR 0: Keep 1: SET bits |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | GPIO47 | GPIO47_SR | Bitwise SET operation of GPIO47 SR 0: Keep 1: SET bits |
| 14 | GPIO46 | GPIO46_SR | Bitwise SET operation of GPIO46 SR 0: Keep 1: SET bits |
| 13 | GPIO45 | GPIO45_SR | Bitwise SET operation of GPIO45 SR 0: Keep 1: SET bits |
| 12 | GPIO44 | GPIO44_SR | Bitwise SET operation of GPIO44 SR 0: Keep 1: SET bits |
| 11 | GPIO43 | GPIO43_SR | Bitwise SET operation of GPIO43 SR 0: Keep 1: SET bits |
| 4 | GPIO36 | GPIO36_SR | Bitwise SET operation of GPIO36 SR 0: Keep 1: SET bits |
| 3 | GPIO35 | GPIO35_SR | Bitwise SET operation of GPIO35 SR 0: Keep 1: SET bits |
| 2 | GPIO34 | GPIO34_SR | Bitwise SET operation of GPIO34 SR 0: Keep 1: SET bits |
| 1 | GPIO33 | GPIO33_SR | Bitwise SET operation of GPIO33 SR 0: Keep 1: SET bits |
| 0 | GPIO32 | GPIO32_SR | Bitwise SET operation of GPIO32 SR 0: Keep 1: SET bits |

A0020718	GPIO_SR1_CLR GPIO SR Control	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | GPIO5 1 | GPIO5 0 | GPIO4 9 | GPIO4 8 |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | WO | WO | WO | WO |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO4 7 | GPIO4 6 | GPIO4 5 | GPIO4 4 | GPIO4 3 |  |  |  |  |  |  | GPIO3 6 | GPIO3 5 | GPIO3 4 | GPIO3 3 | GPIO3 2 |
| Type | WO | WO | WO | WO | WO |  |  |  |  |  |  | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_SR1

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 19 | GPIO51 | GPIO51_SR | Bitwise CLR operation of GPIO51 SR 0: Keep 1: CLR bits |
| 18 | GPIO50 | GPIO50_SR | Bitwise CLR operation of GPIO50 SR 0: Keep |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: CLR bits |
| 17 | GPIO49 | GPIO49_SR | Bitwise CLR operation of GPIO49 SR 0: Keep 1: CLR bits |
| 16 | GPIO48 | GPIO48_SR | Bitwise CLR operation of GPIO48 SR 0: Keep 1: CLR bits |
| 15 | GPIO47 | GPIO47_SR | Bitwise CLR operation of GPIO47 SR 0: Keep 1: CLR bits |
| 14 | GPIO46 | GPIO46_SR | Bitwise CLR operation of GPIO46 SR 0: Keep 1: CLR bits |
| 13 | GPIO45 | GPIO45_SR | Bitwise CLR operation of GPIO45 SR 0: Keep 1: CLR bits |
| 12 | GPIO44 | GPIO44_SR | Bitwise CLR operation of GPIO44 SR 0: Keep 1: CLR bits |
| 11 | GPIO43 | GPIO43_SR | Bitwise CLR operation of GPIO43 SR 0: Keep 1: CLR bits |
| 4 | GPIO36 | GPIO36_SR | Bitwise CLR operation of GPIO36 SR 0: Keep 1: CLR bits |
| 3 | GPIO35 | GPIO35_SR | Bitwise CLR operation of GPIO35 SR 0: Keep 1: CLR bits |
| 2 | GPIO34 | GPIO34_SR | Bitwise CLR operation of GPIO34 SR 0: Keep 1: CLR bits |
| 1 | GPIO33 | GPIO33_SR | Bitwise CLR operation of GPIO33 SR 0: Keep 1: CLR bits |
| 0 | GPIO32 | GPIO32_SR | Bitwise CLR operation of GPIO32 SR 0: Keep 1: CLR bits |

A0020720	GPIO_SIM_SR	GPIO SIM SR Control	003F003F

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 |
| Type |  |  |  |  |  |  |  |  |  |  | RW | RW | RW | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  |  |  | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 |
| Type |  |  |  |  |  |  |  |  |  |  | RW | RW | RW | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  | 1 | 1 | 1 | 1 | 1 | 1 |

Overview: Configures GPIO slew rate control for SIM IO

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 21 | GPIO42 | GPIO42_SR1 | SR1 control for GPIO42 0: Disable 1: Enable |
| 20 | GPIO41 | GPIO41_SR1 | SR1 control for GPIO41 0: Disable 1: Enable |
| 19 | GPIO40 | GPIO40_SR1 | SR1 control for GPIO40 0: Disable 1: Enable |
| 18 | GPIO39 | GPIO39_SR1 | SR1 control for GPIO39 0: Disable 1: Enable |
| 17 | GPIO38 | GPIO38_SR1 | SR1 control for GPIO38 0: Disable 1: Enable |
| 16 | GPIO37 | GPIO37_SR1 | SR1 control for GPIO37 0: Disable 1: Enable |
| 5 | GPIO42 | GPIO42_SR0 | SR0 control for GPIO42 0: Disable 1: Enable |
| 4 | GPIO41 | GPIO41_SR0 | SR0 control for GPIO41 0: Disable 1: Enable |
| 3 | GPIO40 | GPIO40_SR0 | SR0 control for GPIO40 0: Disable 1: Enable |
| 2 | GPIO39 | GPIO39_SR0 | SR0 control for GPIO39 0: Disable 1: Enable |
| 1 | GPIO38 | GPIO38_SR0 | SR0 control for GPIO38 0: Disable 1: Enable |
| 0 | GPIO37 | GPIO37_SR0 | SR0 control for GPIO37 0: Disable 1: Enable |

A0020724	GPIO_SIM_SR_ GPIO SIM SR Control	00000000

SET

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 |
| Type |  |  |  |  |  |  |  |  |  |  | WO | WO | WO | WO | WO | WO |
| Reset |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  |  |  | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 |
| Type |  |  |  |  |  |  |  |  |  |  | WO | WO | WO | WO | WO | WO |
| Reset |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Overview: For bitwise access of GPIO_SIM_SR

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 21 | GPIO42 | GPIO42_SR1 | Bitwise SET operation of GPIO42 SR1 control 0: Keep 1: SET bits |
| 20 | GPIO41 | GPIO41_SR1 | Bitwise SET operation of GPIO41 SR1 control 0: Keep 1: SET bits |
| 19 | GPIO40 | GPIO40_SR1 | Bitwise SET operation of GPIO40 SR1 control 0: Keep 1: SET bits |
| 18 | GPIO39 | GPIO39_SR1 | Bitwise SET operation of GPIO39 SR1 control 0: Keep 1: SET bits |
| 17 | GPIO38 | GPIO38_SR1 | Bitwise SET operation of GPIO38 SR1 control 0: Keep 1: SET bits |
| 16 | GPIO37 | GPIO37_SR1 | Bitwise SET operation of GPIO37 SR1 control 0: Keep 1: SET bits |
| 5 | GPIO42 | GPIO42_SR0 | Bitwise SET operation of GPIO42 SR0 control 0: Keep 1: SET bits |
| 4 | GPIO41 | GPIO41_SR0 | Bitwise SET operation of GPIO41 SR0 control 0: Keep 1: SET bits |
| 3 | GPIO40 | GPIO40_SR0 | Bitwise SET operation of GPIO40 SR0 control 0: Keep 1: SET bits |
| 2 | GPIO39 | GPIO39_SR0 | Bitwise SET operation of GPIO39 SR0 control 0: Keep 1: SET bits |
| 1 | GPIO38 | GPIO38_SR0 | Bitwise SET operation of GPIO38 SR0 control 0: Keep 1: SET bits |
| 0 | GPIO37 | GPIO37_SR0 | Bitwise SET operation of GPIO37 SR0 control 0: Keep 1: SET bits |

A0020728	GPIO_SIM_SR_ GPIO SIM SR Control	00000000

CLR

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 |
| Type |  |  |  |  |  |  |  |  |  |  | WO | WO | WO | WO | WO | WO |
| Reset |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  |  |  | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 |
| Type |  |  |  |  |  |  |  |  |  |  | WO | WO | WO | WO | WO | WO |
| Reset |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Overview: For bitwise access of GPIO_SIM_SR

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 21 | GPIO42 | GPIO42_SR1 | Bitwise CLR operation of GPIO42 SR1 control 0: Keep 1: CLR bits |
| 20 | GPIO41 | GPIO41_SR1 | Bitwise CLR operation of GPIO41 SR1 control 0: Keep 1: CLR bits |
| 19 | GPIO40 | GPIO40_SR1 | Bitwise CLR operation of GPIO40 SR1 control 0: Keep 1: CLR bits |
| 18 | GPIO39 | GPIO39_SR1 | Bitwise CLR operation of GPIO39 SR1 control 0: Keep 1: CLR bits |
| 17 | GPIO38 | GPIO38_SR1 | Bitwise CLR operation of GPIO38 SR1 control 0: Keep 1: CLR bits |
| 16 | GPIO37 | GPIO37_SR1 | Bitwise CLR operation of GPIO37 SR1 control 0: Keep 1: CLR bits |
| 5 | GPIO42 | GPIO42_SR0 | Bitwise CLR operation of GPIO42 SR0 control 0: Keep 1: CLR bits |
| 4 | GPIO41 | GPIO41_SR0 | Bitwise CLR operation of GPIO41 SR0 control 0: Keep 1: CLR bits |
| 3 | GPIO40 | GPIO40_SR0 | Bitwise CLR operation of GPIO40 SR0 control 0: Keep 1: CLR bits |
| 2 | GPIO39 | GPIO39_SR0 | Bitwise CLR operation of GPIO39 SR0 control 0: Keep 1: CLR bits |
| 1 | GPIO38 | GPIO38_SR0 | Bitwise CLR operation of GPIO38 SR0 control 0: Keep 1: CLR bits |
| 0 | GPIO37 | GPIO37_SR0 | Bitwise CLR operation of GPIO37 SR0 control 0: Keep 1: CLR bits |

A0020800	GPIO_DRV0	GPIO DRV Control	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | DRV0[31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | DRV0[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Overview: Configures GPIO driving control

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31:0 |  | DRV0 | GPIO_DRV0 [1: 0]: GPIO_0 [3: 2]: GPIO_1 [5: 4]: GPIO_2 [7: 6]: GPIO_3 [9: 8]: GPIO_4 [11: 10]: GPIO_5 [13: 12]: GPIO_6 [15: 14]: GPIO_7 [17: 16]: GPIO_8 [19: 18]: GPIO_9 [21: 20]: URXD1 [21: 20]: UTXD1 [23: 22]: KCOL4 [25: 24]: KCOL3 [27: 26]: KCOL2 [29: 28]: KCOL1 [31: 30]: KCOL0 |

A0020804	GPIO_DRV0_SE GPIO DRV Control	00000000

T

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | DRV0[31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | WO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | DRV0[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | WO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_DRV0

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31:0 |  | DRV0 | Bitwise SET operation of GPIO_DRV0_SET 0: Keep 1: SET bits |

A0020808	GPIO_DRV0_CL GPIO DRV Control	00000000

R

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | DRV0[31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | WO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | DRV0[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | WO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_DRV0

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31:0 |  | DRV0 | Bitwise CLR operation of GPIO_DRV0_CLR 0: Keep 1: CLR bits |

A0020810	GPIO_DRV1	GPIO DRV Control	00C00000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  | DRV1[25:16] |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | DRV1[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: Configures GPIO driving control

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 25:0 |  | DRV1 | GPIO_DRV1 [1: 0]: KROW4 [3: 2]: KROW3 [5: 4]: KROW2 [7: 6]: KROW1 [9: 8]: KROW0 [11: 10]: BPI_BUS2 [11: 10]: BPI_BUS1 [11: 10]: BPI_BUS0 [13: 12]: CMRST [13: 12]: CMPDN [13: 12]: CMCSD0 [13: 12]: CMCSD1 [13: 12]: CMMCLK [13: 12]: CMCSK [15: 14]: MCCK [15: 14]: MCCM0 [15: 14]: MCDA0 [15: 14]: MCDA1 [15: 14]: MCDA2 [15: 14]: MCDA3 [17: 16]: SIM1_SIO [17: 16]: SIM1_SRST [17: 16]: SIM1_SCLK [19: 18]: SIM2_SIO [19: 18]: SIM2_SRST [19: 18]: SIM2_SCLK [21: 20]: SCL28 [21: 20]: SDA28 [23: 22]: LSRSTB [23: 22]: LSCE_B [23: 22]: LSCK [23: 22]: LSDA [23: 22]: LSA0 [23: 22]:LPTE [25: 24]:RESETB |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A0020814	GPIO_DRV1_SE GPIO DRV Control	00000000

T

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  | DRV1[25:16] |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  | WO |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | DRV1[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | WO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_DRV1

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 25:0 |  | DRV1 | Bitwise SET operation of GPIO_DRV1_SET 0: Keep 1: SET bits |

A0020818	GPIO_DRV1_CL GPIO DRV Control	00000000

R

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  | DRV1[25:16] |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  | WO |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | DRV1[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | WO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_DRV1

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 25:0 |  | DRV1 | Bitwise CLR operation of GPIO_DRV1_CLR 0: Keep 1: CLR bits |

A0020900	GPIO_IES0	GPIO IES Control	43C00BFF

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO3 0 |  |  |  |  | GPIO2 5 | GPIO2 4 | GPIO2 3 | GPIO2 2 |  |  |  |  |  |  |
| Type |  | RW |  |  |  |  | RW | RW | RW | RW |  |  |  |  |  |  |
| Reset |  | 1 |  |  |  |  | 1 | 1 | 1 | 1 |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  | GPIO1 1 |  | GPIO9 | GPIO8 | GPIO7 | GPIO6 | GPIO5 | GPIO4 | GPIO3 | GPIO2 | GPIO1 | GPIO0 |
| Type |  |  |  |  | RW |  | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW |
| Reset |  |  |  |  | 1 |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Overview: Configures GPIO input enabling control

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30 | GPIO30 | GPIO30_IES | Input buffer for GPIO30 0: Disable 1: Enable |
| 25 | GPIO25 | GPIO25_IES | Input buffer for GPIO25 0: Disable 1: Enable |
| 24 | GPIO24 | GPIO24_IES | Input buffer for GPIO24 0: Disable 1: Enable |
| 23 | GPIO23 | GPIO23_IES | Input buffer for GPIO23 0: Disable 1: Enable |
| 22 | GPIO22 | GPIO22_IES | Input buffer for GPIO22 0: Disable 1: Enable |
| 11 | GPIO11 | GPIO11_IES | Input buffer for GPIO11 0: Disable 1: Enable |
| 9 | GPIO9 | GPIO9_IES | Input buffer for GPIO9 0: Disable 1: Enable |
| 8 | GPIO8 | GPIO8_IES | Input buffer for GPIO8 0: Disable 1: Enable |
| 7 | GPIO7 | GPIO7_IES | Input buffer for GPIO7 0: Disable 1: Enable |
| 6 | GPIO6 | GPIO6_IES | Input buffer for GPIO6 0: Disable 1: Enable |
| 5 | GPIO5 | GPIO5_IES | Input buffer for GPIO5 0: Disable 1: Enable |
| 4 | GPIO4 | GPIO4_IES | Input buffer for GPIO4 0: Disable 1: Enable |
| 3 | GPIO3 | GPIO3_IES | Input buffer for GPIO3 0: Disable 1: Enable |
| 2 | GPIO2 | GPIO2_IES | Input buffer for GPIO2 0: Disable 1: Enable |
| 1 | GPIO1 | GPIO1_IES | Input buffer for GPIO1 0: Disable 1: Enable |
| 0 | GPIO0 | GPIO0_IES | Input buffer for GPIO0 0: Disable 1: Enable |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A0020904	GPIO_IES0_SETGPIO IES Control	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO3 0 |  |  |  |  | GPIO2 5 | GPIO2 4 | GPIO2 3 | GPIO2 2 |  |  |  |  |  |  |
| Type |  | WO |  |  |  |  | WO | WO | WO | WO |  |  |  |  |  |  |
| Reset |  | 0 |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  | GPIO1 1 |  | GPIO9 | GPIO8 | GPIO7 | GPIO6 | GPIO5 | GPIO4 | GPIO3 | GPIO2 | GPIO1 | GPIO0 |
| Type |  |  |  |  | WO |  | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset |  |  |  |  | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_IES0

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30 | GPIO30 | GPIO30_IES | Bitwise SET operation of GPIO30 input buffer 0: Keep 1: SET bits |
| 25 | GPIO25 | GPIO25_IES | Bitwise SET operation of GPIO25 input buffer 0: Keep 1: SET bits |
| 24 | GPIO24 | GPIO24_IES | Bitwise SET operation of GPIO24 input buffer 0: Keep 1: SET bits |
| 23 | GPIO23 | GPIO23_IES | Bitwise SET operation of GPIO23 input buffer 0: Keep 1: SET bits |
| 22 | GPIO22 | GPIO22_IES | Bitwise SET operation of GPIO22 input buffer 0: Keep 1: SET bits |
| 11 | GPIO11 | GPIO11_IES | Bitwise SET operation of GPIO11 input buffer 0: Keep 1: SET bits |
| 9 | GPIO9 | GPIO9_IES | Bitwise SET operation of GPIO9 input buffer 0: Keep 1: SET bits |
| 8 | GPIO8 | GPIO8_IES | Bitwise SET operation of GPIO8 input buffer 0: Keep 1: SET bits |
| 7 | GPIO7 | GPIO7_IES | Bitwise SET operation of GPIO7 input buffer 0: Keep 1: SET bits |
| 6 | GPIO6 | GPIO6_IES | Bitwise SET operation of GPIO6 input buffer 0: Keep 1: SET bits |
| 5 | GPIO5 | GPIO5_IES | Bitwise SET operation of GPIO5 input buffer 0: Keep 1: SET bits |
| 4 | GPIO4 | GPIO4_IES | Bitwise SET operation of GPIO4 input buffer 0: Keep 1: SET bits |
| 3 | GPIO3 | GPIO3_IES | Bitwise SET operation of GPIO3 input buffer 0: Keep |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: SET bits |
| 2 | GPIO2 | GPIO2_IES | Bitwise SET operation of GPIO2 input buffer 0: Keep 1: SET bits |
| 1 | GPIO1 | GPIO1_IES | Bitwise SET operation of GPIO1 input buffer 0: Keep 1: SET bits |
| 0 | GPIO0 | GPIO0_IES | Bitwise SET operation of GPIO0 input buffer 0: Keep 1: SET bits |

A0020908	GPIO_IES0_CL R

GPIO IES Control	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO3 0 |  |  |  |  | GPIO2 5 | GPIO2 4 | GPIO2 3 | GPIO2 2 |  |  |  |  |  |  |
| Type |  | WO |  |  |  |  | WO | WO | WO | WO |  |  |  |  |  |  |
| Reset |  | 0 |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  | GPIO1 1 |  | GPIO9 | GPIO8 | GPIO7 | GPIO6 | GPIO5 | GPIO4 | GPIO3 | GPIO2 | GPIO1 | GPIO0 |
| Type |  |  |  |  | WO |  | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset |  |  |  |  | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_IES0

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30 | GPIO30 | GPIO30_IES | Bitwise CLR operation of GPIO30 input buffer 0: Keep 1: CLR bits |
| 25 | GPIO25 | GPIO25_IES | Bitwise CLR operation of GPIO25 input buffer 0: Keep 1: CLR bits |
| 24 | GPIO24 | GPIO24_IES | Bitwise CLR operation of GPIO24 input buffer 0: Keep 1: CLR bits |
| 23 | GPIO23 | GPIO23_IES | Bitwise CLR operation of GPIO23 input buffer 0: Keep 1: CLR bits |
| 22 | GPIO22 | GPIO22_IES | Bitwise CLR operation of GPIO22 input buffer 0: Keep 1: CLR bits |
| 11 | GPIO11 | GPIO11_IES | Bitwise CLR operation of GPIO11 input buffer 0: Keep 1: CLR bits |
| 9 | GPIO9 | GPIO9_IES | Bitwise CLR operation of GPIO9 input buffer 0: Keep 1: CLR bits |
| 8 | GPIO8 | GPIO8_IES | Bitwise CLR operation of GPIO8 input buffer 0: Keep 1: CLR bits |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7 | GPIO7 | GPIO7_IES | Bitwise CLR operation of GPIO7 input buffer 0: Keep 1: CLR bits |
| 6 | GPIO6 | GPIO6_IES | Bitwise CLR operation of GPIO6 input buffer 0: Keep 1: CLR bits |
| 5 | GPIO5 | GPIO5_IES | Bitwise CLR operation of GPIO5 input buffer 0: Keep 1: CLR bits |
| 4 | GPIO4 | GPIO4_IES | Bitwise CLR operation of GPIO4 input buffer 0: Keep 1: CLR bits |
| 3 | GPIO3 | GPIO3_IES | Bitwise CLR operation of GPIO3 input buffer 0: Keep 1: CLR bits |
| 2 | GPIO2 | GPIO2_IES | Bitwise CLR operation of GPIO2 input buffer 0: Keep 1: CLR bits |
| 1 | GPIO1 | GPIO1_IES | Bitwise CLR operation of GPIO1 input buffer 0: Keep 1: CLR bits |
| 0 | GPIO0 | GPIO0_IES | Bitwise CLR operation of GPIO0 input buffer 0: Keep 1: CLR bits |

A0020910	GPIO_IES1	GPIO IES Control	00001FE0

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  | GPIO4 4 | GPIO4 3 | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 |  |  |  |  |  |
| Type |  |  |  | RW | RW | RW | RW | RW | RW | RW | RW |  |  |  |  |  |
| Reset |  |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  |  |  |  |  |

Overview: Configures GPIO input enabling control

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 12 | GPIO44 | GPIO44_IES | Input buffer for GPIO44 0: Disable 1: Enable |
| 11 | GPIO43 | GPIO43_IES | Input buffer for GPIO43 0: Disable 1: Enable |
| 10 | GPIO42 | GPIO42_IES | Input buffer for GPIO42 0: Disable 1: Enable |
| 9 | GPIO41 | GPIO41_IES | Input buffer for GPIO41 0: Disable |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: Enable |
| 8 | GPIO40 | GPIO40_IES | Input buffer for GPIO40 0: Disable 1: Enable |
| 7 | GPIO39 | GPIO39_IES | Input buffer for GPIO39 0: Disable 1: Enable |
| 6 | GPIO38 | GPIO38_IES | Input buffer for GPIO38 0: Disable 1: Enable |
| 5 | GPIO37 | GPIO37_IES | Input buffer for GPIO37 0: Disable 1: Enable |

A0020914	GPIO_IES1_SETGPIO IES Control	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  | GPIO4 4 | GPIO4 3 | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 |  |  |  |  |  |
| Type |  |  |  | WO | WO | WO | WO | WO | WO | WO | WO |  |  |  |  |  |
| Reset |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |

Overview: For bitwise access of GPIO_IES1

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 12 | GPIO44 | GPIO44_IES | Bitwise SET operation of GPIO44 input buffer 0: Keep 1: SET bits |
| 11 | GPIO43 | GPIO43_IES | Bitwise SET operation of GPIO43 input buffer 0: Keep 1: SET bits |
| 10 | GPIO42 | GPIO42_IES | Bitwise SET operation of GPIO42 input buffer 0: Keep 1: SET bits |
| 9 | GPIO41 | GPIO41_IES | Bitwise SET operation of GPIO41 input buffer 0: Keep 1: SET bits |
| 8 | GPIO40 | GPIO40_IES | Bitwise SET operation of GPIO40 input buffer 0: Keep 1: SET bits |
| 7 | GPIO39 | GPIO39_IES | Bitwise SET operation of GPIO39 input buffer 0: Keep 1: SET bits |
| 6 | GPIO38 | GPIO38_IES | Bitwise SET operation of GPIO38 input buffer 0: Keep 1: SET bits |
| 5 | GPIO37 | GPIO37_IES | Bitwise SET operation of GPIO37 input buffer |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Keep 1: SET bits |

A0020918	GPIO_IES1_CL R

GPIO IES Control	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  | GPIO4 4 | GPIO4 3 | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 |  |  |  |  |  |
| Type |  |  |  | WO | WO | WO | WO | WO | WO | WO | WO |  |  |  |  |  |
| Reset |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |

Overview: For bitwise access of GPIO_IES1

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 12 | GPIO44 | GPIO44_IES | Bitwise CLR operation of GPIO44 input buffer 0: Keep 1: CLR bits |
| 11 | GPIO43 | GPIO43_IES | Bitwise CLR operation of GPIO43 input buffer 0: Keep 1: CLR bits |
| 10 | GPIO42 | GPIO42_IES | Bitwise CLR operation of GPIO42 input buffer 0: Keep 1: CLR bits |
| 9 | GPIO41 | GPIO41_IES | Bitwise CLR operation of GPIO41 input buffer 0: Keep 1: CLR bits |
| 8 | GPIO40 | GPIO40_IES | Bitwise CLR operation of GPIO40 input buffer 0: Keep 1: CLR bits |
| 7 | GPIO39 | GPIO39_IES | Bitwise CLR operation of GPIO39 input buffer 0: Keep 1: CLR bits |
| 6 | GPIO38 | GPIO38_IES | Bitwise CLR operation of GPIO38 input buffer 0: Keep 1: CLR bits |
| 5 | GPIO37 | GPIO37_IES | Bitwise CLR operation of GPIO37 input buffer 0: Keep 1: CLR bits |

A0020A00	GPIO_PUPD0	GPIO PUPD Control	303E0000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | GPIO3 1 |  | GPIO2 9 | GPIO2 8 | GPIO2 7 | GPIO2 6 |  |  |  |  | GPIO2 1 | GPIO2 0 | GPIO1 9 | GPIO1 8 | GPIO1 7 | GPIO1 6 |
| Type | RW |  | RW | RW | RW | RW |  |  |  |  | RW | RW | RW | RW | RW | RW |
| Reset | 0 |  | 1 | 1 | 0 | 0 |  |  |  |  | 1 | 1 | 1 | 1 | 1 | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | GPIO1 5 | GPIO1 4 | GPIO1 3 | GPIO1 2 |  | GPIO1 0 |  |  |  |  |  |  |  |  |  |  |
| Type | RW | RW | RW | RW |  | RW |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 |  | 0 |  |  |  |  |  |  |  |  |  |  |

Overview: Configures GPIO PUPD control

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31 | GPIO31 | GPIO31_PUPD | PUPD for GPIO31 0: Disable 1: Enable |
| 29 | GPIO29 | GPIO29_PUPD | PUPD for GPIO29 0: Disable 1: Enable |
| 28 | GPIO28 | GPIO28_PUPD | PUPD for GPIO28 0: Disable 1: Enable |
| 27 | GPIO27 | GPIO27_PUPD | PUPD for GPIO27 0: Disable 1: Enable |
| 26 | GPIO26 | GPIO26_PUPD | PUPD for GPIO26 0: Disable 1: Enable |
| 21 | GPIO21 | GPIO21_PUPD | PUPD for GPIO21 0: Disable 1: Enable |
| 20 | GPIO20 | GPIO20_PUPD | PUPD for GPIO20 0: Disable 1: Enable |
| 19 | GPIO19 | GPIO19_PUPD | PUPD for GPIO19 0: Disable 1: Enable |
| 18 | GPIO18 | GPIO18_PUPD | PUPD for GPIO18 0: Disable 1: Enable |
| 17 | GPIO17 | GPIO17_PUPD | PUPD for GPIO17 0: Disable 1: Enable |
| 16 | GPIO16 | GPIO16_PUPD | PUPD for GPIO16 0: Disable 1: Enable |
| 15 | GPIO15 | GPIO15_PUPD | PUPD for GPIO15 0: Disable 1: Enable |
| 14 | GPIO14 | GPIO14_PUPD | PUPD for GPIO14 0: Disable 1: Enable |
| 13 | GPIO13 | GPIO13_PUPD | PUPD for GPIO13 0: Disable 1: Enable |
| 12 | GPIO12 | GPIO12_PUPD | PUPD for GPIO12 0: Disable |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: Enable |
| 10 | GPIO10 | GPIO10_PUPD | PUPD for GPIO10 0: Disable 1: Enable |

A0020A04	GPIO_PUPD0_S GPIO PUPD Control	00000000

ET

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | GPIO3 1 |  | GPIO2 9 | GPIO2 8 | GPIO2 7 | GPIO2 6 |  |  |  |  | GPIO2 1 | GPIO2 0 | GPIO1 9 | GPIO1 8 | GPIO1 7 | GPIO1 6 |
| Type | WO |  | WO | WO | WO | WO |  |  |  |  | WO | WO | WO | WO | WO | WO |
| Reset | 0 |  | 0 | 0 | 0 | 0 |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO1 5 | GPIO1 4 | GPIO1 3 | GPIO1 2 |  | GPIO1 0 |  |  |  |  |  |  |  |  |  |  |
| Type | WO | WO | WO | WO |  | WO |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 |  | 0 |  |  |  |  |  |  |  |  |  |  |

Overview: For bitwise access of GPIO_PUPD0

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31 | GPIO31 | GPIO31_PUPD | Bitwise SET operation of GPIO31 PUPD 0: Keep 1: SET bits |
| 29 | GPIO29 | GPIO29_PUPD | Bitwise SET operation of GPIO29 PUPD 0: Keep 1: SET bits |
| 28 | GPIO28 | GPIO28_PUPD | Bitwise SET operation of GPIO28 PUPD 0: Keep 1: SET bits |
| 27 | GPIO27 | GPIO27_PUPD | Bitwise SET operation of GPIO27 PUPD 0: Keep 1: SET bits |
| 26 | GPIO26 | GPIO26_PUPD | Bitwise SET operation of GPIO26 PUPD 0: Keep 1: SET bits |
| 21 | GPIO21 | GPIO21_PUPD | Bitwise SET operation of GPIO21 PUPD 0: Keep 1: SET bits |
| 20 | GPIO20 | GPIO20_PUPD | Bitwise SET operation of GPIO20 PUPD 0: Keep 1: SET bits |
| 19 | GPIO19 | GPIO19_PUPD | Bitwise SET operation of GPIO19 PUPD 0: Keep 1: SET bits |
| 18 | GPIO18 | GPIO18_PUPD | Bitwise SET operation of GPIO18 PUPD 0: Keep 1: SET bits |
| 17 | GPIO17 | GPIO17_PUPD | Bitwise SET operation of GPIO17 PUPD 0: Keep 1: SET bits |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 16 | GPIO16 | GPIO16_PUPD | Bitwise SET operation of GPIO16 PUPD 0: Keep 1: SET bits |
| 15 | GPIO15 | GPIO15_PUPD | Bitwise SET operation of GPIO15 PUPD 0: Keep 1: SET bits |
| 14 | GPIO14 | GPIO14_PUPD | Bitwise SET operation of GPIO14 PUPD 0: Keep 1: SET bits |
| 13 | GPIO13 | GPIO13_PUPD | Bitwise SET operation of GPIO13 PUPD 0: Keep 1: SET bits |
| 12 | GPIO12 | GPIO12_PUPD | Bitwise SET operation of GPIO12 PUPD 0: Keep 1: SET bits |
| 10 | GPIO10 | GPIO10_PUPD | Bitwise SET operation of GPIO10 PUPD 0: Keep 1: SET bits |

A0020A08	GPIO_PUPD0_C GPIO PUPD Control	00000000

LR

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | GPIO3 1 |  | GPIO2 9 | GPIO2 8 | GPIO2 7 | GPIO2 6 |  |  |  |  | GPIO2 1 | GPIO2 0 | GPIO1 9 | GPIO1 8 | GPIO1 7 | GPIO1 6 |
| Type | WO |  | WO | WO | WO | WO |  |  |  |  | WO | WO | WO | WO | WO | WO |
| Reset | 0 |  | 0 | 0 | 0 | 0 |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO1 5 | GPIO1 4 | GPIO1 3 | GPIO1 2 |  | GPIO1 0 |  |  |  |  |  |  |  |  |  |  |
| Type | WO | WO | WO | WO |  | WO |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 |  | 0 |  |  |  |  |  |  |  |  |  |  |

Overview: For bitwise access of GPIO_PUPD0

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31 | GPIO31 | GPIO31_PUPD | Bitwise CLR operation of GPIO31 PUPD 0: Keep 1: CLR bits |
| 29 | GPIO29 | GPIO29_PUPD | Bitwise CLR operation of GPIO29 PUPD 0: Keep 1: CLR bits |
| 28 | GPIO28 | GPIO28_PUPD | Bitwise CLR operation of GPIO28 PUPD 0: Keep 1: CLR bits |
| 27 | GPIO27 | GPIO27_PUPD | Bitwise CLR operation of GPIO27 PUPD 0: Keep 1: CLR bits |
| 26 | GPIO26 | GPIO26_PUPD | Bitwise CLR operation of GPIO26 PUPD 0: Keep 1: CLR bits |
| 21 | GPIO21 | GPIO21_PUPD | Bitwise CLR operation of GPIO21 PUPD |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Keep 1: CLR bits |
| 20 | GPIO20 | GPIO20_PUPD | Bitwise CLR operation of GPIO20 PUPD 0: Keep 1: CLR bits |
| 19 | GPIO19 | GPIO19_PUPD | Bitwise CLR operation of GPIO19 PUPD 0: Keep 1: CLR bits |
| 18 | GPIO18 | GPIO18_PUPD | Bitwise CLR operation of GPIO18 PUPD 0: Keep 1: CLR bits |
| 17 | GPIO17 | GPIO17_PUPD | Bitwise CLR operation of GPIO17 PUPD 0: Keep 1: CLR bits |
| 16 | GPIO16 | GPIO16_PUPD | Bitwise CLR operation of GPIO16 PUPD 0: Keep 1: CLR bits |
| 15 | GPIO15 | GPIO15_PUPD | Bitwise CLR operation of GPIO15 PUPD 0: Keep 1: CLR bits |
| 14 | GPIO14 | GPIO14_PUPD | Bitwise CLR operation of GPIO14 PUPD 0: Keep 1: CLR bits |
| 13 | GPIO13 | GPIO13_PUPD | Bitwise CLR operation of GPIO13 PUPD 0: Keep 1: CLR bits |
| 12 | GPIO12 | GPIO12_PUPD | Bitwise CLR operation of GPIO12 PUPD 0: Keep 1: CLR bits |
| 10 | GPIO10 | GPIO10_PUPD | Bitwise CLR operation of GPIO10 PUPD 0: Keep 1: CLR bits |

A0020A10	GPIO_PUPD1	GPIO PUPD Control	0007A7FE

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | GPIO5 1 | GPIO5 0 | GPIO4 9 | GPIO4 8 |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 1 | 1 | 1 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO4 7 | GPIO4 6 | GPIO4 5 |  |  | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 | GPIO3 6 | GPIO3 5 | GPIO3 4 | GPIO3 3 | GPIO3 2 |
| Type | RW | RW | RW |  |  | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW |
| Reset | 1 | 0 | 1 |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |

Overview: Configures GPIO PUPD control

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 19 | GPIO51 | GPIO51_PUPD | PUPD for GPIO51 0: Disable 1: Enable |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 18 | GPIO50 | GPIO50_PUPD | PUPD for GPIO50 0: Disable 1: Enable |
| 17 | GPIO49 | GPIO49_PUPD | PUPD for GPIO49 0: Disable 1: Enable |
| 16 | GPIO48 | GPIO48_PUPD | PUPD for GPIO48 0: Disable 1: Enable |
| 15 | GPIO47 | GPIO47_PUPD | PUPD for GPIO47 0: Disable 1: Enable |
| 14 | GPIO46 | GPIO46_PUPD | PUPD for GPIO46 0: Disable 1: Enable |
| 13 | GPIO45 | GPIO45_PUPD | PUPD for GPIO45 0: Disable 1: Enable |
| 10 | GPIO42 | GPIO42_PUPD | PUPD for GPIO42 0: Disable 1: Enable |
| 9 | GPIO41 | GPIO41_PUPD | PUPD for GPIO41 0: Disable 1: Enable |
| 8 | GPIO40 | GPIO40_PUPD | PUPD for GPIO40 0: Disable 1: Enable |
| 7 | GPIO39 | GPIO39_PUPD | PUPD for GPIO39 0: Disable 1: Enable |
| 6 | GPIO38 | GPIO38_PUPD | PUPD for GPIO38 0: Disable 1: Enable |
| 5 | GPIO37 | GPIO37_PUPD | PUPD for GPIO37 0: Disable 1: Enable |
| 4 | GPIO36 | GPIO36_PUPD | PUPD for GPIO36 0: Disable 1: Enable |
| 3 | GPIO35 | GPIO35_PUPD | PUPD for GPIO35 0: Disable 1: Enable |
| 2 | GPIO34 | GPIO34_PUPD | PUPD for GPIO34 0: Disable 1: Enable |
| 1 | GPIO33 | GPIO33_PUPD | PUPD for GPIO33 0: Disable 1: Enable |
| 0 | GPIO32 | GPIO32_PUPD | PUPD for GPIO32 0: Disable 1: Enable |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A0020A14	GPIO_PUPD1_S GPIO PUPD Control	00000000

ET

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | GPIO5 1 | GPIO5 0 | GPIO4 9 | GPIO4 8 |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | WO | WO | WO | WO |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO4 7 | GPIO4 6 | GPIO4 5 |  |  | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 | GPIO3 6 | GPIO3 5 | GPIO3 4 | GPIO3 3 | GPIO3 2 |
| Type | WO | WO | WO |  |  | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_PUPD1

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 19 | GPIO51 | GPIO51_PUPD | Bitwise SET operation of GPIO51 PUPD 0: Keep 1: SET bits |
| 18 | GPIO50 | GPIO50_PUPD | Bitwise SET operation of GPIO50 PUPD 0: Keep 1: SET bits |
| 17 | GPIO49 | GPIO49_PUPD | Bitwise SET operation of GPIO49 PUPD 0: Keep 1: SET bits |
| 16 | GPIO48 | GPIO48_PUPD | Bitwise SET operation of GPIO48 PUPD 0: Keep 1: SET bits |
| 15 | GPIO47 | GPIO47_PUPD | Bitwise SET operation of GPIO47 PUPD 0: Keep 1: SET bits |
| 14 | GPIO46 | GPIO46_PUPD | Bitwise SET operation of GPIO46 PUPD 0: Keep 1: SET bits |
| 13 | GPIO45 | GPIO45_PUPD | Bitwise SET operation of GPIO45 PUPD 0: Keep 1: SET bits |
| 10 | GPIO42 | GPIO42_PUPD | Bitwise SET operation of GPIO42 PUPD 0: Keep 1: SET bits |
| 9 | GPIO41 | GPIO41_PUPD | Bitwise SET operation of GPIO41 PUPD 0: Keep 1: SET bits |
| 8 | GPIO40 | GPIO40_PUPD | Bitwise SET operation of GPIO40 PUPD 0: Keep 1: SET bits |
| 7 | GPIO39 | GPIO39_PUPD | Bitwise SET operation of GPIO39 PUPD 0: Keep 1: SET bits |
| 6 | GPIO38 | GPIO38_PUPD | Bitwise SET operation of GPIO38 PUPD 0: Keep 1: SET bits |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 5 | GPIO37 | GPIO37_PUPD | Bitwise SET operation of GPIO37 PUPD 0: Keep 1: SET bits |
| 4 | GPIO36 | GPIO36_PUPD | Bitwise SET operation of GPIO36 PUPD 0: Keep 1: SET bits |
| 3 | GPIO35 | GPIO35_PUPD | Bitwise SET operation of GPIO35 PUPD 0: Keep 1: SET bits |
| 2 | GPIO34 | GPIO34_PUPD | Bitwise SET operation of GPIO34 PUPD 0: Keep 1: SET bits |
| 1 | GPIO33 | GPIO33_PUPD | Bitwise SET operation of GPIO33 PUPD 0: Keep 1: SET bits |
| 0 | GPIO32 | GPIO32_PUPD | Bitwise SET operation of GPIO32 PUPD 0: Keep 1: SET bits |

A0020A18	GPIO_PUPD1_C GPIO PUPD Control	00000000

LR

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | GPIO5 1 | GPIO5 0 | GPIO4 9 | GPIO4 8 |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | WO | WO | WO | WO |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO4 7 | GPIO4 6 | GPIO4 5 |  |  | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 | GPIO3 6 | GPIO3 5 | GPIO3 4 | GPIO3 3 | GPIO3 2 |
| Type | WO | WO | WO |  |  | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_PUPD1

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 19 | GPIO51 | GPIO51_PUPD | Bitwise CLR operation of GPIO51 PUPD 0: Keep 1: CLR bits |
| 18 | GPIO50 | GPIO50_PUPD | Bitwise CLR operation of GPIO50 PUPD 0: Keep 1: CLR bits |
| 17 | GPIO49 | GPIO49_PUPD | Bitwise CLR operation of GPIO49 PUPD 0: Keep 1: CLR bits |
| 16 | GPIO48 | GPIO48_PUPD | Bitwise CLR operation of GPIO48 PUPD 0: Keep 1: CLR bits |
| 15 | GPIO47 | GPIO47_PUPD | Bitwise CLR operation of GPIO47 PUPD 0: Keep 1: CLR bits |
| 14 | GPIO46 | GPIO46_PUPD | Bitwise CLR operation of GPIO46 PUPD |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Keep 1: CLR bits |
| 13 | GPIO45 | GPIO45_PUPD | Bitwise CLR operation of GPIO45 PUPD 0: Keep 1: CLR bits |
| 10 | GPIO42 | GPIO42_PUPD | Bitwise CLR operation of GPIO42 PUPD 0: Keep 1: CLR bits |
| 9 | GPIO41 | GPIO41_PUPD | Bitwise CLR operation of GPIO41 PUPD 0: Keep 1: CLR bits |
| 8 | GPIO40 | GPIO40_PUPD | Bitwise CLR operation of GPIO40 PUPD 0: Keep 1: CLR bits |
| 7 | GPIO39 | GPIO39_PUPD | Bitwise CLR operation of GPIO39 PUPD 0: Keep 1: CLR bits |
| 6 | GPIO38 | GPIO38_PUPD | Bitwise CLR operation of GPIO38 PUPD 0: Keep 1: CLR bits |
| 5 | GPIO37 | GPIO37_PUPD | Bitwise CLR operation of GPIO37 PUPD 0: Keep 1: CLR bits |
| 4 | GPIO36 | GPIO36_PUPD | Bitwise CLR operation of GPIO36 PUPD 0: Keep 1: CLR bits |
| 3 | GPIO35 | GPIO35_PUPD | Bitwise CLR operation of GPIO35 PUPD 0: Keep 1: CLR bits |
| 2 | GPIO34 | GPIO34_PUPD | Bitwise CLR operation of GPIO34 PUPD 0: Keep 1: CLR bits |
| 1 | GPIO33 | GPIO33_PUPD | Bitwise CLR operation of GPIO33 PUPD 0: Keep 1: CLR bits |
| 0 | GPIO32 | GPIO32_PUPD | Bitwise CLR operation of GPIO32 PUPD 0: Keep 1: CLR bits |

A0020B00	GPIO_RESEN0_ GPIO R0 Control	B83FF400 0

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | GPIO3 1 |  | GPIO2 9 | GPIO2 8 | GPIO2 7 | GPIO2 6 |  |  |  |  | GPIO2 1 | GPIO2 0 | GPIO1 9 | GPIO1 8 | GPIO1 7 | GPIO1 6 |
| Type | RW |  | RW | RW | RW | RW |  |  |  |  | RW | RW | RW | RW | RW | RW |
| Reset | 1 |  | 1 | 1 | 1 | 0 |  |  |  |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO1 5 | GPIO1 4 | GPIO1 3 | GPIO1 2 |  | GPIO1 0 |  |  |  |  |  |  |  |  |  |  |
| Type | RW | RW | RW | RW |  | RW |  |  |  |  |  |  |  |  |  |  |
| Reset | 1 | 1 | 1 | 1 |  | 1 |  |  |  |  |  |  |  |  |  |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Overview: Configures GPIO R0 control

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31 | GPIO31 | GPIO31_R0 | R0 for GPIO31 0: Disable 1: Enable |
| 29 | GPIO29 | GPIO29_R0 | R0 for GPIO29 0: Disable 1: Enable |
| 28 | GPIO28 | GPIO28_R0 | R0 for GPIO28 0: Disable 1: Enable |
| 27 | GPIO27 | GPIO27_R0 | R0 for GPIO27 0: Disable 1: Enable |
| 26 | GPIO26 | GPIO26_R0 | R0 for GPIO26 0: Disable 1: Enable |
| 21 | GPIO21 | GPIO21_R0 | R0 for GPIO21 0: Disable 1: Enable |
| 20 | GPIO20 | GPIO20_R0 | R0 for GPIO20 0: Disable 1: Enable |
| 19 | GPIO19 | GPIO19_R0 | R0 for GPIO19 0: Disable 1: Enable |
| 18 | GPIO18 | GPIO18_R0 | R0 for GPIO18 0: Disable 1: Enable |
| 17 | GPIO17 | GPIO17_R0 | R0 for GPIO17 0: Disable 1: Enable |
| 16 | GPIO16 | GPIO16_R0 | R0 for GPIO16 0: Disable 1: Enable |
| 15 | GPIO15 | GPIO15_R0 | R0 for GPIO15 0: Disable 1: Enable |
| 14 | GPIO14 | GPIO14_R0 | R0 for GPIO14 0: Disable 1: Enable |
| 13 | GPIO13 | GPIO13_R0 | R0 for GPIO13 0: Disable 1: Enable |
| 12 | GPIO12 | GPIO12_R0 | R0 for GPIO12 0: Disable 1: Enable |
| 10 | GPIO10 | GPIO10_R0 | R0 for GPIO10 0: Disable 1: Enable |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A0020B04	GPIO_RESEN0_ GPIO R0 Control	00000000

0_SET

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | GPIO3 1 |  | GPIO2 9 | GPIO2 8 | GPIO2 7 | GPIO2 6 |  |  |  |  | GPIO2 1 | GPIO2 0 | GPIO1 9 | GPIO1 8 | GPIO1 7 | GPIO1 6 |
| Type | WO |  | WO | WO | WO | WO |  |  |  |  | WO | WO | WO | WO | WO | WO |
| Reset | 0 |  | 0 | 0 | 0 | 0 |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO1 5 | GPIO1 4 | GPIO1 3 | GPIO1 2 |  | GPIO1 0 |  |  |  |  |  |  |  |  |  |  |
| Type | WO | WO | WO | WO |  | WO |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 |  | 0 |  |  |  |  |  |  |  |  |  |  |

Overview: For bitwise access of GPIO_RESEN0_0

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31 | GPIO31 | GPIO31_R0 | Bitwise SET operation of GPIO31 R0 0: Keep 1: SET bits |
| 29 | GPIO29 | GPIO29_R0 | Bitwise SET operation of GPIO29 R0 0: Keep 1: SET bits |
| 28 | GPIO28 | GPIO28_R0 | Bitwise SET operation of GPIO28 R0 0: Keep 1: SET bits |
| 27 | GPIO27 | GPIO27_R0 | Bitwise SET operation of GPIO27 R0 0: Keep 1: SET bits |
| 26 | GPIO26 | GPIO26_R0 | Bitwise SET operation of GPIO26 R0 0: Keep 1: SET bits |
| 21 | GPIO21 | GPIO21_R0 | Bitwise SET operation of GPIO21 R0 0: Keep 1: SET bits |
| 20 | GPIO20 | GPIO20_R0 | Bitwise SET operation of GPIO20 R0 0: Keep 1: SET bits |
| 19 | GPIO19 | GPIO19_R0 | Bitwise SET operation of GPIO19 R0 0: Keep 1: SET bits |
| 18 | GPIO18 | GPIO18_R0 | Bitwise SET operation of GPIO18 R0 0: Keep 1: SET bits |
| 17 | GPIO17 | GPIO17_R0 | Bitwise SET operation of GPIO17 R0 0: Keep 1: SET bits |
| 16 | GPIO16 | GPIO16_R0 | Bitwise SET operation of GPIO16 R0 0: Keep 1: SET bits |
| 15 | GPIO15 | GPIO15_R0 | Bitwise SET operation of GPIO15 R0 0: Keep 1: SET bits |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 14 | GPIO14 | GPIO14_R0 | Bitwise SET operation of GPIO14 R0 0: Keep 1: SET bits |
| 13 | GPIO13 | GPIO13_R0 | Bitwise SET operation of GPIO13 R0 0: Keep 1: SET bits |
| 12 | GPIO12 | GPIO12_R0 | Bitwise SET operation of GPIO12 R0 0: Keep 1: SET bits |
| 10 | GPIO10 | GPIO10_R0 | Bitwise SET operation of GPIO10 R0 0: Keep 1: SET bits |

A0020B08	GPIO_RESEN0_ GPIO R0 Control	00000000

0_CLR

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | GPIO3 1 |  | GPIO2 9 | GPIO2 8 | GPIO2 7 | GPIO2 6 |  |  |  |  | GPIO2 1 | GPIO2 0 | GPIO1 9 | GPIO1 8 | GPIO1 7 | GPIO1 6 |
| Type | WO |  | WO | WO | WO | WO |  |  |  |  | WO | WO | WO | WO | WO | WO |
| Reset | 0 |  | 0 | 0 | 0 | 0 |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO1 5 | GPIO1 4 | GPIO1 3 | GPIO1 2 |  | GPIO1 0 |  |  |  |  |  |  |  |  |  |  |
| Type | WO | WO | WO | WO |  | WO |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 |  | 0 |  |  |  |  |  |  |  |  |  |  |

Overview: For bitwise access of GPIO_RESEN0_0

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31 | GPIO31 | GPIO31_R0 | Bitwise CLR operation of GPIO31 R0 0: Keep 1: CLR bits |
| 29 | GPIO29 | GPIO29_R0 | Bitwise CLR operation of GPIO29 R0 0: Keep 1: CLR bits |
| 28 | GPIO28 | GPIO28_R0 | Bitwise CLR operation of GPIO28 R0 0: Keep 1: CLR bits |
| 27 | GPIO27 | GPIO27_R0 | Bitwise CLR operation of GPIO27 R0 0: Keep 1: CLR bits |
| 26 | GPIO26 | GPIO26_R0 | Bitwise CLR operation of GPIO26 R0 0: Keep 1: CLR bits |
| 21 | GPIO21 | GPIO21_R0 | Bitwise CLR operation of GPIO21 R0 0: Keep 1: CLR bits |
| 20 | GPIO20 | GPIO20_R0 | Bitwise CLR operation of GPIO20 R0 0: Keep 1: CLR bits |
| 19 | GPIO19 | GPIO19_R0 | Bitwise CLR operation of GPIO19 R0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Keep 1: CLR bits |
| 18 | GPIO18 | GPIO18_R0 | Bitwise CLR operation of GPIO18 R0 0: Keep 1: CLR bits |
| 17 | GPIO17 | GPIO17_R0 | Bitwise CLR operation of GPIO17 R0 0: Keep 1: CLR bits |
| 16 | GPIO16 | GPIO16_R0 | Bitwise CLR operation of GPIO16 R0 0: Keep 1: CLR bits |
| 15 | GPIO15 | GPIO15_R0 | Bitwise CLR operation of GPIO15 R0 0: Keep 1: CLR bits |
| 14 | GPIO14 | GPIO14_R0 | Bitwise CLR operation of GPIO14 R0 0: Keep 1: CLR bits |
| 13 | GPIO13 | GPIO13_R0 | Bitwise CLR operation of GPIO13 R0 0: Keep 1: CLR bits |
| 12 | GPIO12 | GPIO12_R0 | Bitwise CLR operation of GPIO12 R0 0: Keep 1: CLR bits |
| 10 | GPIO10 | GPIO10_R0 | Bitwise CLR operation of GPIO10 R0 0: Keep 1: CLR bits |

A0020B10	GPIO_RESEN0_ GPIO R0 Control	0007A7FF 1

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | GPIO5 1 | GPIO5 0 | GPIO4 9 | GPIO4 8 |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 1 | 1 | 1 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO4 7 | GPIO4 6 | GPIO4 5 |  |  | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 | GPIO3 6 | GPIO3 5 | GPIO3 4 | GPIO3 3 | GPIO3 2 |
| Type | RW | RW | RW |  |  | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW |
| Reset | 1 | 0 | 1 |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Overview: Configures GPIO R0 control

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 19 | GPIO51 | GPIO51_R0 | R0 for GPIO51 0: Disable 1: Enable |
| 18 | GPIO50 | GPIO50_R0 | R0 for GPIO50 0: Disable 1: Enable |
| 17 | GPIO49 | GPIO49_R0 | R0 for GPIO49 0: Disable |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: Enable |
| 16 | GPIO48 | GPIO48_R0 | R0 for GPIO48 0: Disable 1: Enable |
| 15 | GPIO47 | GPIO47_R0 | R0 for GPIO47 0: Disable 1: Enable |
| 14 | GPIO46 | GPIO46_R0 | R0 for GPIO46 0: Disable 1: Enable |
| 13 | GPIO45 | GPIO45_R0 | R0 for GPIO45 0: Disable 1: Enable |
| 10 | GPIO42 | GPIO42_R0 | R0 for GPIO42 0: Disable 1: Enable |
| 9 | GPIO41 | GPIO41_R0 | R0 for GPIO41 0: Disable 1: Enable |
| 8 | GPIO40 | GPIO40_R0 | R0 for GPIO40 0: Disable 1: Enable |
| 7 | GPIO39 | GPIO39_R0 | R0 for GPIO39 0: Disable 1: Enable |
| 6 | GPIO38 | GPIO38_R0 | R0 for GPIO38 0: Disable 1: Enable |
| 5 | GPIO37 | GPIO37_R0 | R0 for GPIO37 0: Disable 1: Enable |
| 4 | GPIO36 | GPIO36_R0 | R0 for GPIO36 0: Disable 1: Enable |
| 3 | GPIO35 | GPIO35_R0 | R0 for GPIO35 0: Disable 1: Enable |
| 2 | GPIO34 | GPIO34_R0 | R0 for GPIO34 0: Disable 1: Enable |
| 1 | GPIO33 | GPIO33_R0 | R0 for GPIO33 0: Disable 1: Enable |
| 0 | GPIO32 | GPIO32_R0 | R0 for GPIO32 0: Disable 1: Enable |

A0020B14	GPIO_RESEN0_ GPIO R0 Control	00000000

1_SET

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Name |  |  |  |  |  |  |  |  |  |  |  |  | GPIO5 1 | GPIO5 0 | GPIO4 9 | GPIO4 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | WO | WO | WO | WO |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO4 7 | GPIO4 6 | GPIO4 5 |  |  | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 | GPIO3 6 | GPIO3 5 | GPIO3 4 | GPIO3 3 | GPIO3 2 |
| Type | WO | WO | WO |  |  | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_RESEN0_1

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 19 | GPIO51 | GPIO51_R0 | Bitwise SET operation of GPIO51 R0 0: Keep 1: SET bits |
| 18 | GPIO50 | GPIO50_R0 | Bitwise SET operation of GPIO50 R0 0: Keep 1: SET bits |
| 17 | GPIO49 | GPIO49_R0 | Bitwise SET operation of GPIO49 R0 0: Keep 1: SET bits |
| 16 | GPIO48 | GPIO48_R0 | Bitwise SET operation of GPIO48 R0 0: Keep 1: SET bits |
| 15 | GPIO47 | GPIO47_R0 | Bitwise SET operation of GPIO47 R0 0: Keep 1: SET bits |
| 14 | GPIO46 | GPIO46_R0 | Bitwise SET operation of GPIO46 R0 0: Keep 1: SET bits |
| 13 | GPIO45 | GPIO45_R0 | Bitwise SET operation of GPIO45 R0 0: Keep 1: SET bits |
| 10 | GPIO42 | GPIO42_R0 | Bitwise SET operation of GPIO42 R0 0: Keep 1: SET bits |
| 9 | GPIO41 | GPIO41_R0 | Bitwise SET operation of GPIO41 R0 0: Keep 1: SET bits |
| 8 | GPIO40 | GPIO40_R0 | Bitwise SET operation of GPIO40 R0 0: Keep 1: SET bits |
| 7 | GPIO39 | GPIO39_R0 | Bitwise SET operation of GPIO39 R0 0: Keep 1: SET bits |
| 6 | GPIO38 | GPIO38_R0 | Bitwise SET operation of GPIO38 R0 0: Keep 1: SET bits |
| 5 | GPIO37 | GPIO37_R0 | Bitwise SET operation of GPIO37 R0 0: Keep 1: SET bits |
| 4 | GPIO36 | GPIO36_R0 | Bitwise SET operation of GPIO36 R0 0: Keep |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: SET bits |
| 3 | GPIO35 | GPIO35_R0 | Bitwise SET operation of GPIO35 R0 0: Keep 1: SET bits |
| 2 | GPIO34 | GPIO34_R0 | Bitwise SET operation of GPIO34 R0 0: Keep 1: SET bits |
| 1 | GPIO33 | GPIO33_R0 | Bitwise SET operation of GPIO33 R0 0: Keep 1: SET bits |
| 0 | GPIO32 | GPIO32_R0 | Bitwise SET operation of GPIO32 R0 0: Keep 1: SET bits |

A0020B18	GPIO_RESEN0_ GPIO R0 Control	00000000

1_CLR

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | GPIO5 1 | GPIO5 0 | GPIO4 9 | GPIO4 8 |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | WO | WO | WO | WO |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO4 7 | GPIO4 6 | GPIO4 5 |  |  | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 | GPIO3 6 | GPIO3 5 | GPIO3 4 | GPIO3 3 | GPIO3 2 |
| Type | WO | WO | WO |  |  | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_RESEN0_1

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 19 | GPIO51 | GPIO51_R0 | Bitwise CLR operation of GPIO51 R0 0: Keep 1: CLR bits |
| 18 | GPIO50 | GPIO50_R0 | Bitwise CLR operation of GPIO50 R0 0: Keep 1: CLR bits |
| 17 | GPIO49 | GPIO49_R0 | Bitwise CLR operation of GPIO49 R0 0: Keep 1: CLR bits |
| 16 | GPIO48 | GPIO48_R0 | Bitwise CLR operation of GPIO48 R0 0: Keep 1: CLR bits |
| 15 | GPIO47 | GPIO47_R0 | Bitwise CLR operation of GPIO47 R0 0: Keep 1: CLR bits |
| 14 | GPIO46 | GPIO46_R0 | Bitwise CLR operation of GPIO46 R0 0: Keep 1: CLR bits |
| 13 | GPIO45 | GPIO45_R0 | Bitwise CLR operation of GPIO45 R0 0: Keep 1: CLR bits |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 10 | GPIO42 | GPIO42_R0 | Bitwise CLR operation of GPIO42 R0 0: Keep 1: CLR bits |
| 9 | GPIO41 | GPIO41_R0 | Bitwise CLR operation of GPIO41 R0 0: Keep 1: CLR bits |
| 8 | GPIO40 | GPIO40_R0 | Bitwise CLR operation of GPIO40 R0 0: Keep 1: CLR bits |
| 7 | GPIO39 | GPIO39_R0 | Bitwise CLR operation of GPIO39 R0 0: Keep 1: CLR bits |
| 6 | GPIO38 | GPIO38_R0 | Bitwise CLR operation of GPIO38 R0 0: Keep 1: CLR bits |
| 5 | GPIO37 | GPIO37_R0 | Bitwise CLR operation of GPIO37 R0 0: Keep 1: CLR bits |
| 4 | GPIO36 | GPIO36_R0 | Bitwise CLR operation of GPIO36 R0 0: Keep 1: CLR bits |
| 3 | GPIO35 | GPIO35_R0 | Bitwise CLR operation of GPIO35 R0 0: Keep 1: CLR bits |
| 2 | GPIO34 | GPIO34_R0 | Bitwise CLR operation of GPIO34 R0 0: Keep 1: CLR bits |
| 1 | GPIO33 | GPIO33_R0 | Bitwise CLR operation of GPIO33 R0 0: Keep 1: CLR bits |
| 0 | GPIO32 | GPIO32_R0 | Bitwise CLR operation of GPIO32 R0 0: Keep 1: CLR bits |

A0020B20	GPIO_RESEN1_ GPIO R1 Control	00000000

0

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | GPIO3 1 |  | GPIO2 9 | GPIO2 8 | GPIO2 7 | GPIO2 6 |  |  |  |  | GPIO2 1 | GPIO2 0 | GPIO1 9 | GPIO1 8 | GPIO1 7 | GPIO1 6 |
| Type | RW |  | RW | RW | RW | RW |  |  |  |  | RW | RW | RW | RW | RW | RW |
| Reset | 0 |  | 0 | 0 | 0 | 0 |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO1 5 | GPIO1 4 | GPIO1 3 | GPIO1 2 |  | GPIO1 0 |  |  |  |  |  |  |  |  |  |  |
| Type | RW | RW | RW | RW |  | RW |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 |  | 0 |  |  |  |  |  |  |  |  |  |  |

Overview: Configures GPIO R1 control

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31 | GPIO31 | GPIO31_R1 | R1 for GPIO31 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Disable 1: Enable |
| 29 | GPIO29 | GPIO29_R1 | R1 for GPIO29 0: Disable 1: Enable |
| 28 | GPIO28 | GPIO28_R1 | R1 for GPIO28 0: Disable 1: Enable |
| 27 | GPIO27 | GPIO27_R1 | R1 for GPIO27 0: Disable 1: Enable |
| 26 | GPIO26 | GPIO26_R1 | R1 for GPIO26 0: Disable 1: Enable |
| 21 | GPIO21 | GPIO21_R1 | R1 for GPIO21 0: Disable 1: Enable |
| 20 | GPIO20 | GPIO20_R1 | R1 for GPIO20 0: Disable 1: Enable |
| 19 | GPIO19 | GPIO19_R1 | R1 for GPIO19 0: Disable 1: Enable |
| 18 | GPIO18 | GPIO18_R1 | R1 for GPIO18 0: Disable 1: Enable |
| 17 | GPIO17 | GPIO17_R1 | R1 for GPIO17 0: Disable 1: Enable |
| 16 | GPIO16 | GPIO16_R1 | R1 for GPIO16 0: Disable 1: Enable |
| 15 | GPIO15 | GPIO15_R1 | R1 for GPIO15 0: Disable 1: Enable |
| 14 | GPIO14 | GPIO14_R1 | R1 for GPIO14 0: Disable 1: Enable |
| 13 | GPIO13 | GPIO13_R1 | R1 for GPIO13 0: Disable 1: Enable |
| 12 | GPIO12 | GPIO12_R1 | R1 for GPIO12 0: Disable 1: Enable |
| 10 | GPIO10 | GPIO10_R1 | R1 for GPIO10 0: Disable 1: Enable |

| A0020B24 | GPIO_RESEN1_ 0_SET | GPIO R1 Control | 00000000 |
| --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | GPIO3 1 |  | GPIO2 9 | GPIO2 8 | GPIO2 7 | GPIO2 6 |  |  |  |  | GPIO2 1 | GPIO2 0 | GPIO1 9 | GPIO1 8 | GPIO1 7 | GPIO1 6 |
| Type | WO |  | WO | WO | WO | WO |  |  |  |  | WO | WO | WO | WO | WO | WO |
| Reset | 0 |  | 0 | 0 | 0 | 0 |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO1 5 | GPIO1 4 | GPIO1 3 | GPIO1 2 |  | GPIO1 0 |  |  |  |  |  |  |  |  |  |  |
| Type | WO | WO | WO | WO |  | WO |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 |  | 0 |  |  |  |  |  |  |  |  |  |  |

Overview: For bitwise access of GPIO_RESEN1_0

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31 | GPIO31 | GPIO31_R1 | Bitwise SET operation of GPIO31 R1 0: Keep 1: SET bits |
| 29 | GPIO29 | GPIO29_R1 | Bitwise SET operation of GPIO29 R1 0: Keep 1: SET bits |
| 28 | GPIO28 | GPIO28_R1 | Bitwise SET operation of GPIO28 R1 0: Keep 1: SET bits |
| 27 | GPIO27 | GPIO27_R1 | Bitwise SET operation of GPIO27 R1 0: Keep 1: SET bits |
| 26 | GPIO26 | GPIO26_R1 | Bitwise SET operation of GPIO26 R1 0: Keep 1: SET bits |
| 21 | GPIO21 | GPIO21_R1 | Bitwise SET operation of GPIO21 R1 0: Keep 1: SET bits |
| 20 | GPIO20 | GPIO20_R1 | Bitwise SET operation of GPIO20 R1 0: Keep 1: SET bits |
| 19 | GPIO19 | GPIO19_R1 | Bitwise SET operation of GPIO19 R1 0: Keep 1: SET bits |
| 18 | GPIO18 | GPIO18_R1 | Bitwise SET operation of GPIO18 R1 0: Keep 1: SET bits |
| 17 | GPIO17 | GPIO17_R1 | Bitwise SET operation of GPIO17 R1 0: Keep 1: SET bits |
| 16 | GPIO16 | GPIO16_R1 | Bitwise SET operation of GPIO16 R1 0: Keep 1: SET bits |
| 15 | GPIO15 | GPIO15_R1 | Bitwise SET operation of GPIO15 R1 0: Keep 1: SET bits |
| 14 | GPIO14 | GPIO14_R1 | Bitwise SET operation of GPIO14 R1 0: Keep 1: SET bits |
| 13 | GPIO13 | GPIO13_R1 | Bitwise SET operation of GPIO13 R1 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Keep 1: SET bits |
| 12 | GPIO12 | GPIO12_R1 | Bitwise SET operation of GPIO12 R1 0: Keep 1: SET bits |
| 10 | GPIO10 | GPIO10_R1 | Bitwise SET operation of GPIO10 R1 0: Keep 1: SET bits |

A0020B28	GPIO_RESEN1_ GPIO R1 Control	00000000

0_CLR

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | GPIO3 1 |  | GPIO2 9 | GPIO2 8 | GPIO2 7 | GPIO2 6 |  |  |  |  | GPIO2 1 | GPIO2 0 | GPIO1 9 | GPIO1 8 | GPIO1 7 | GPIO1 6 |
| Type | WO |  | WO | WO | WO | WO |  |  |  |  | WO | WO | WO | WO | WO | WO |
| Reset | 0 |  | 0 | 0 | 0 | 0 |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO1 5 | GPIO1 4 | GPIO1 3 | GPIO1 2 |  | GPIO1 0 |  |  |  |  |  |  |  |  |  |  |
| Type | WO | WO | WO | WO |  | WO |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 |  | 0 |  |  |  |  |  |  |  |  |  |  |

Overview: For bitwise access of GPIO_RESEN1_0

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31 | GPIO31 | GPIO31_R1 | Bitwise CLR operation of GPIO31 R1 0: Keep 1: CLR bits |
| 29 | GPIO29 | GPIO29_R1 | Bitwise CLR operation of GPIO29 R1 0: Keep 1: CLR bits |
| 28 | GPIO28 | GPIO28_R1 | Bitwise CLR operation of GPIO28 R1 0: Keep 1: CLR bits |
| 27 | GPIO27 | GPIO27_R1 | Bitwise CLR operation of GPIO27 R1 0: Keep 1: CLR bits |
| 26 | GPIO26 | GPIO26_R1 | Bitwise CLR operation of GPIO26 R1 0: Keep 1: CLR bits |
| 21 | GPIO21 | GPIO21_R1 | Bitwise CLR operation of GPIO21 R1 0: Keep 1: CLR bits |
| 20 | GPIO20 | GPIO20_R1 | Bitwise CLR operation of GPIO20 R1 0: Keep 1: CLR bits |
| 19 | GPIO19 | GPIO19_R1 | Bitwise CLR operation of GPIO19 R1 0: Keep 1: CLR bits |
| 18 | GPIO18 | GPIO18_R1 | Bitwise CLR operation of GPIO18 R1 0: Keep |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: CLR bits |
| 17 | GPIO17 | GPIO17_R1 | Bitwise CLR operation of GPIO17 R1 0: Keep 1: CLR bits |
| 16 | GPIO16 | GPIO16_R1 | Bitwise CLR operation of GPIO16 R1 0: Keep 1: CLR bits |
| 15 | GPIO15 | GPIO15_R1 | Bitwise CLR operation of GPIO15 R1 0: Keep 1: CLR bits |
| 14 | GPIO14 | GPIO14_R1 | Bitwise CLR operation of GPIO14 R1 0: Keep 1: CLR bits |
| 13 | GPIO13 | GPIO13_R1 | Bitwise CLR operation of GPIO13 R1 0: Keep 1: CLR bits |
| 12 | GPIO12 | GPIO12_R1 | Bitwise CLR operation of GPIO12 R1 0: Keep 1: CLR bits |
| 10 | GPIO10 | GPIO10_R1 | Bitwise CLR operation of GPIO10 R1 0: Keep 1: CLR bits |

A0020B30	GPIO_RESEN1_ GPIO R1 Control	00000000

1

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | GPIO5 1 | GPIO5 0 | GPIO4 9 | GPIO4 8 |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO4 7 | GPIO4 6 | GPIO4 5 |  |  | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 | GPIO3 6 | GPIO3 5 | GPIO3 4 | GPIO3 3 | GPIO3 2 |
| Type | RW | RW | RW |  |  | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW |
| Reset | 0 | 0 | 0 |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: Configures GPIO R1 control

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 19 | GPIO51 | GPIO51_R1 | R1 for GPIO51 0: Disable 1: Enable |
| 18 | GPIO50 | GPIO50_R1 | R1 for GPIO50 0: Disable 1: Enable |
| 17 | GPIO49 | GPIO49_R1 | R1 for GPIO49 0: Disable 1: Enable |
| 16 | GPIO48 | GPIO48_R1 | R1 for GPIO48 0: Disable 1: Enable |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | GPIO47 | GPIO47_R1 | R1 for GPIO47 0: Disable 1: Enable |
| 14 | GPIO46 | GPIO46_R1 | R1 for GPIO46 0: Disable 1: Enable |
| 13 | GPIO45 | GPIO45_R1 | R1 for GPIO45 0: Disable 1: Enable |
| 10 | GPIO42 | GPIO42_R1 | R1 for GPIO42 0: Disable 1: Enable |
| 9 | GPIO41 | GPIO41_R1 | R1 for GPIO41 0: Disable 1: Enable |
| 8 | GPIO40 | GPIO40_R1 | R1 for GPIO40 0: Disable 1: Enable |
| 7 | GPIO39 | GPIO39_R1 | R1 for GPIO39 0: Disable 1: Enable |
| 6 | GPIO38 | GPIO38_R1 | R1 for GPIO38 0: Disable 1: Enable |
| 5 | GPIO37 | GPIO37_R1 | R1 for GPIO37 0: Disable 1: Enable |
| 4 | GPIO36 | GPIO36_R1 | R1 for GPIO36 0: Disable 1: Enable |
| 3 | GPIO35 | GPIO35_R1 | R1 for GPIO35 0: Disable 1: Enable |
| 2 | GPIO34 | GPIO34_R1 | R1 for GPIO34 0: Disable 1: Enable |
| 1 | GPIO33 | GPIO33_R1 | R1 for GPIO33 0: Disable 1: Enable |
| 0 | GPIO32 | GPIO32_R1 | R1 for GPIO32 0: Disable 1: Enable |

A0020B34	GPIO_RESEN1_ GPIO R1 Control	00000000

1_SET

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | GPIO5 1 | GPIO5 0 | GPIO4 9 | GPIO4 8 |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | WO | WO | WO | WO |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Name | GPIO4 7 | GPIO4 6 | GPIO4 5 |  |  | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 | GPIO3 6 | GPIO3 5 | GPIO3 4 | GPIO3 3 | GPIO3 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type | WO | WO | WO |  |  | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_RESEN1_1

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 19 | GPIO51 | GPIO51_R1 | Bitwise SET operation of GPIO51 R1 0: Keep 1: SET bits |
| 18 | GPIO50 | GPIO50_R1 | Bitwise SET operation of GPIO50 R1 0: Keep 1: SET bits |
| 17 | GPIO49 | GPIO49_R1 | Bitwise SET operation of GPIO49 R1 0: Keep 1: SET bits |
| 16 | GPIO48 | GPIO48_R1 | Bitwise SET operation of GPIO48 R1 0: Keep 1: SET bits |
| 15 | GPIO47 | GPIO47_R1 | Bitwise SET operation of GPIO47 R1 0: Keep 1: SET bits |
| 14 | GPIO46 | GPIO46_R1 | Bitwise SET operation of GPIO46 R1 0: Keep 1: SET bits |
| 13 | GPIO45 | GPIO45_R1 | Bitwise SET operation of GPIO45 R1 0: Keep 1: SET bits |
| 10 | GPIO42 | GPIO42_R1 | Bitwise SET operation of GPIO42 R1 0: Keep 1: SET bits |
| 9 | GPIO41 | GPIO41_R1 | Bitwise SET operation of GPIO41 R1 0: Keep 1: SET bits |
| 8 | GPIO40 | GPIO40_R1 | Bitwise SET operation of GPIO40 R1 0: Keep 1: SET bits |
| 7 | GPIO39 | GPIO39_R1 | Bitwise SET operation of GPIO39 R1 0: Keep 1: SET bits |
| 6 | GPIO38 | GPIO38_R1 | Bitwise SET operation of GPIO38 R1 0: Keep 1: SET bits |
| 5 | GPIO37 | GPIO37_R1 | Bitwise SET operation of GPIO37 R1 0: Keep 1: SET bits |
| 4 | GPIO36 | GPIO36_R1 | Bitwise SET operation of GPIO36 R1 0: Keep 1: SET bits |
| 3 | GPIO35 | GPIO35_R1 | Bitwise SET operation of GPIO35 R1 0: Keep 1: SET bits |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 2 | GPIO34 | GPIO34_R1 | Bitwise SET operation of GPIO34 R1 0: Keep 1: SET bits |
| 1 | GPIO33 | GPIO33_R1 | Bitwise SET operation of GPIO33 R1 0: Keep 1: SET bits |
| 0 | GPIO32 | GPIO32_R1 | Bitwise SET operation of GPIO32 R1 0: Keep 1: SET bits |

A0020B38	GPIO_RESEN1_ GPIO R1 Control	00000000

1_CLR

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | GPIO5 1 | GPIO5 0 | GPIO4 9 | GPIO4 8 |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | WO | WO | WO | WO |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO4 7 | GPIO4 6 | GPIO4 5 |  |  | GPIO4 2 | GPIO4 1 | GPIO4 0 | GPIO3 9 | GPIO3 8 | GPIO3 7 | GPIO3 6 | GPIO3 5 | GPIO3 4 | GPIO3 3 | GPIO3 2 |
| Type | WO | WO | WO |  |  | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO | WO |
| Reset | 0 | 0 | 0 |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_RESEN1_1

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 19 | GPIO51 | GPIO51_R1 | Bitwise CLR operation of GPIO51 R1 0: Keep 1: CLR bits |
| 18 | GPIO50 | GPIO50_R1 | Bitwise CLR operation of GPIO50 R1 0: Keep 1: CLR bits |
| 17 | GPIO49 | GPIO49_R1 | Bitwise CLR operation of GPIO49 R1 0: Keep 1: CLR bits |
| 16 | GPIO48 | GPIO48_R1 | Bitwise CLR operation of GPIO48 R1 0: Keep 1: CLR bits |
| 15 | GPIO47 | GPIO47_R1 | Bitwise CLR operation of GPIO47 R1 0: Keep 1: CLR bits |
| 14 | GPIO46 | GPIO46_R1 | Bitwise CLR operation of GPIO46 R1 0: Keep 1: CLR bits |
| 13 | GPIO45 | GPIO45_R1 | Bitwise CLR operation of GPIO45 R1 0: Keep 1: CLR bits |
| 10 | GPIO42 | GPIO42_R1 | Bitwise CLR operation of GPIO42 R1 0: Keep 1: CLR bits |
| 9 | GPIO41 | GPIO41_R1 | Bitwise CLR operation of GPIO41 R1 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Keep 1: CLR bits |
| 8 | GPIO40 | GPIO40_R1 | Bitwise CLR operation of GPIO40 R1 0: Keep 1: CLR bits |
| 7 | GPIO39 | GPIO39_R1 | Bitwise CLR operation of GPIO39 R1 0: Keep 1: CLR bits |
| 6 | GPIO38 | GPIO38_R1 | Bitwise CLR operation of GPIO38 R1 0: Keep 1: CLR bits |
| 5 | GPIO37 | GPIO37_R1 | Bitwise CLR operation of GPIO37 R1 0: Keep 1: CLR bits |
| 4 | GPIO36 | GPIO36_R1 | Bitwise CLR operation of GPIO36 R1 0: Keep 1: CLR bits |
| 3 | GPIO35 | GPIO35_R1 | Bitwise CLR operation of GPIO35 R1 0: Keep 1: CLR bits |
| 2 | GPIO34 | GPIO34_R1 | Bitwise CLR operation of GPIO34 R1 0: Keep 1: CLR bits |
| 1 | GPIO33 | GPIO33_R1 | Bitwise CLR operation of GPIO33 R1 0: Keep 1: CLR bits |
| 0 | GPIO32 | GPIO32_R1 | Bitwise CLR operation of GPIO32 R1 0: Keep 1: CLR bits |

KeyPad KCOL0-4

| PUPD | R1 | R0 | WEAK PULL UP/DOWN STATE |
| --- | --- | --- | --- |
| 0 | 0 | 0 | Disable both resistors |
| 0 | 0 | 1 | PU-36K ohms |
| 0 | 1 | 0 | PU-1200K ohms |
| 0 | 1 | 1 | PU- 1200K//36K ohms |
| 1 | 0 | 0 | Disable both resistors |
| 1 | 0 | 1 | PD-36K ohms |
| 1 | 1 | 0 | PD-1200K ohms |
| 1 | 1 | 1 | PD- 1200K//36K ohms |

KROW0-4

| PUPD | R1 | R0 | WEAK PULL UP/DOWN STATE |
| --- | --- | --- | --- |
| 0 | 0 | 0 | Disable both resistors |
| 0 | 0 | 1 | PU-36K ohms |
| 0 | 1 | 0 | PU-1K ohms |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| PUPD | R1 | R0 | WEAK PULL UP/DOWN STATE |
| --- | --- | --- | --- |
| 0 | 1 | 1 | PU- 1K//36K ohms |
| 1 | 0 | 0 | Disable both resistors |
| 1 | 0 | 1 | PD-36K ohms |
| 1 | 1 | 0 | PD-1K ohms |
| 1 | 1 | 1 | PD- 1K//36K ohms |

URXD:

CMPDN/CMCSD0/CMCSD1/CMMCLK MCCK/MCCM0/MCDA0/MCDA1/MCDA2/MCDA3 LSRSTB/LSCE_B/LSCK/LSDA/LSA0/LPTE RESETB:

| PUPD | R1 | R0 | WEAK PULL UP/DOWN STATE |
| --- | --- | --- | --- |
| 0 | 0 | 0 | Disable both resistors |
| 0 | 0 | 1 | PU-47K ohms |
| 0 | 1 | 0 | PU-47K ohms |
| 0 | 1 | 1 | PU- 23.5K ohms |
| 1 | 0 | 0 | Disable both resistors |
| 1 | 0 | 1 | PD-47K ohms |
| 1 | 1 | 0 | PD-47K ohms |
| 1 | 1 | 1 | PD- 23.5K ohms |

A0020C00	GPIO_MODE0	GPIO Mode Control	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO7 |  |  |  | GPIO6 |  |  |  | GPIO5 |  |  |  | GPIO4 |  |  |
| Type |  | RW |  |  |  | RW |  |  |  | RW |  |  |  | RW |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO3 |  |  |  | GPIO2 |  |  |  | GPIO1 |  |  |  | GPIO0 |  |  |  |
| Type | RW |  |  |  | RW |  |  |  | RW |  |  |  | RW |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: Configures GPIO aux. mode

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30:28 |  | GPIO7 | Aux. mode of GPIO_7 0: GPIO7 (IO) 1: EINT6 (I) 2: Reserved 3: BPI_BUS5 (O) 4: Reserved 5: Reserved 6: Reserved 7: Reserved |
| 26:24 |  | GPIO6 | Aux. mode of GPIO_6 0: GPIO6 (IO) 1: EINT5 (I) |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 2: MCINS (I) 3: BPI_BUS4 (O) 4: Reserved 5: Reserved 6: Reserved 7: Reserved |
| 22:20 |  | GPIO5 | Aux. mode of GPIO_5 0: GPIO5 (IO) 1: EINT4 (I) 2: Reserved 3: BPI_BUS3 (O) 4: Reserved 5: Reserved 6: Reserved 7: Reserved |
| 18:16 |  | GPIO4 | Aux. mode of GPIO_4 0: GPIO4 (IO) 1: EINT3 (I) 2: Reserved 3: Reserved 4: U1RTS (O) 5: Reserved 6: Reserved 7: Reserved |
| 15:12 |  | GPIO3 | Aux. mode of GPIO_3 0: GPIO3 (IO) 1: MCINS (I) 2: YM (AIO) 3: Reserved 4: PWM1 (O) 5: CMCSD1 (I) 6: EDICK (O) 7: JTDO (O) 8: BTJTDO (O) 9: FMJTDO (O) |
| 11:8 |  | GPIO2 | Aux. mode of GPIO_2 0: GPIO2 (IO) 1: EINT2 (I) 2: YP (AIO) 3: GPSFSYNC (O) 4: PWM0 (O) 5: CMCSD0 (I) 6: EDIWS (O) 7: JTRST_B (I) 8: BTJTRSTB (I) 9: FMJTRSTB (I) |
| 7:4 |  | GPIO1 | Aux. mode of GPIO_1 0: GPIO1 (IO) 1: EINT1 (I) 2: XM (AIO) 3: U3TXD (O) 4: U1CTS (I) 5: CMMCLK (O) 6: EDIDI (I) 7: JTMS (I) 8: BTJTMS (I) 9: FMJTMS (I) |
| 3:0 |  | GPIO0 | Aux. mode of GPIO_0 0: GPIO0 (IO) |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: EINT0 (I) 2: XP (AIO) 3: U3RXD (I) 4: CMCSD2 (I) 5: CMCSK (I) 6: EDIDO (O) 7: JTDI (I) 8: BTJTDI (I) 9: FMJTDI (I) |

A0020C04	GPIO_MODE0_ SET

GPIO Mode Control	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO7 |  |  |  | GPIO6 |  |  |  | GPIO5 |  |  |  | GPIO4 |  |  |
| Type |  | WO |  |  |  | WO |  |  |  | WO |  |  |  | WO |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO3 |  |  |  | GPIO2 |  |  |  | GPIO1 |  |  |  | GPIO0 |  |  |  |
| Type | WO |  |  |  | WO |  |  |  | WO |  |  |  | WO |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_MODE0

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30:28 |  | GPIO7 | Bitwise SET operation for Aux. mode of GPIO_7 0: Keep 1: SET bits |
| 26:24 |  | GPIO6 | Bitwise SET operation for Aux. mode of GPIO_6 0: Keep 1: SET bits |
| 22:20 |  | GPIO5 | Bitwise SET operation for Aux. mode of GPIO_5 0: Keep 1: SET bits |
| 18:16 |  | GPIO4 | Bitwise SET operation for Aux. mode of GPIO_4 0: Keep 1: SET bits |
| 15:12 |  | GPIO3 | Bitwise SET operation for Aux. mode of GPIO_3 0: Keep 1: SET bits |
| 11:8 |  | GPIO2 | Bitwise SET operation for Aux. mode of GPIO_2 0: Keep 1: SET bits |
| 7:4 |  | GPIO1 | Bitwise SET operation for Aux. mode of GPIO_1 0: Keep 1: SET bits |
| 3:0 |  | GPIO0 | Bitwise SET operation for Aux. mode of GPIO_0 0: Keep 1: SET bits |

A0020C08	GPIO_MODE0_

CLR

GPIO Mode Control	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO7 |  |  |  | GPIO6 |  |  |  | GPIO5 |  |  |  | GPIO4 |  |  |
| Type |  | WO |  |  |  | WO |  |  |  | WO |  |  |  | WO |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO3 |  |  |  | GPIO2 |  |  |  | GPIO1 |  |  |  | GPIO0 |  |  |  |
| Type | WO |  |  |  | WO |  |  |  | WO |  |  |  | WO |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_MODE0

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30:28 |  | GPIO7 | Bitwise CLR operation for Aux. mode of GPIO_7 0: Keep 1: CLR bits |
| 26:24 |  | GPIO6 | Bitwise CLR operation for Aux. mode of GPIO_6 0: Keep 1: CLR bits |
| 22:20 |  | GPIO5 | Bitwise CLR operation for Aux. mode of GPIO_5 0: Keep 1: CLR bits |
| 18:16 |  | GPIO4 | Bitwise CLR operation for Aux. mode of GPIO_4 0: Keep 1: CLR bits |
| 15:12 |  | GPIO3 | Bitwise CLR operation for Aux. mode of GPIO_3 0: Keep 1: CLR bits |
| 11:8 |  | GPIO2 | Bitwise CLR operation for Aux. mode of GPIO_2 0: Keep 1: CLR bits |
| 7:4 |  | GPIO1 | Bitwise CLR operation for Aux. mode of GPIO_1 0: Keep 1: CLR bits |
| 3:0 |  | GPIO0 | Bitwise CLR operation for Aux. mode of GPIO_0 0: Keep 1: CLR bits |

A0020C10	GPIO_MODE1	GPIO Mode Control	00001100

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO15 |  |  |  | GPIO14 |  |  |  | GPIO13 |  |  |  | GPIO12 |  |  |
| Type |  | RW |  |  |  | RW |  |  |  | RW |  |  |  | RW |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  | GPIO11 |  |  |  | GPIO10 |  |  |  | GPIO9 |  |  |  | GPIO8 |  |  |
| Type |  | RW |  |  |  | RW |  |  |  | RW |  |  |  | RW |  |  |
| Reset |  | 0 | 0 | 1 |  | 0 | 0 | 1 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |

Overview: Configures GPIO aux. mode

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30:28 |  | GPIO15 | Aux. mode of GPIO_15 0: GPIO15 (IO) 1: KCOL1 (IO) 2: GPSFSYNC (O) 3: U1CTS (I) 4: FMJTCK (I) 5: JTCK (I) 6: BTJTCK (I) 7: Reserved |
| 26:24 |  | GPIO14 | Aux. mode of GPIO_14 0: GPIO14 (IO) 1: KCOL2 (IO) 2: EINT12 (I) 3: U1RTS (I) 4: Reserved 5: Reserved 6: Reserved 7: Reserved |
| 22:20 |  | GPIO13 | Aux. mode of GPIO_13 0: GPIO13 (IO) 1: KCOL3 (IO) 2: EINT11 (I) 3: PWM0 (O) 4: FMJTMS (I) 5: JTMS (I) 6: BTJTMS (I) 7: Reserved |
| 18:16 |  | GPIO12 | Aux. mode of GPIO_12 0: GPIO12 (IO) 1: KCOL4 (IO) 2: U2RXD (I) 3: EDIDI (I) 4: FMJTDI (I) 5: JTDI (I) 6: BTJTDI (I) 7: Reserved |
| 14:12 |  | GPIO11 | Aux. mode of GPIO_11 0: GPIO11 (IO) 1: U1TXD (O) 2: CMPDN (O) 3: EINT10 (I) 4: Reserved 5: Reserved 6: Reserved 7: Reserved |
| 10:8 |  | GPIO10 | Aux. mode of GPIO_10 0: GPIO10 (IO) 1: U1RXD (I) 2: CMRST (O) 3: EINT9 (I) 4: MCINS (I) 5: Reserved 6: Reserved 7: Reserved |
| 6:4 |  | GPIO9 | Aux. mode of GPIO_9 0: GPIO9 (IO) 1: EINT8 (I) 2: SDA (IO) |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 3: Reserved 4: Reserved 5: Reserved 6: Reserved 7: Reserved |
| 2:0 |  | GPIO8 | Aux. mode of GPIO_8 0: GPIO8 (IO) 1: EINT7 (I) 2: SCL (IO) 3: Reserved 4: Reserved 5: Reserved 6: Reserved 7: Reserved |

A0020C14	GPIO_MODE1_ SET

GPIO Mode Control	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO15 |  |  |  | GPIO14 |  |  |  | GPIO13 |  |  |  | GPIO12 |  |  |
| Type |  | WO |  |  |  | WO |  |  |  | WO |  |  |  | WO |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  | GPIO11 |  |  |  | GPIO10 |  |  |  | GPIO9 |  |  |  | GPIO8 |  |  |
| Type |  | WO |  |  |  | WO |  |  |  | WO |  |  |  | WO |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_MODE1

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30:28 |  | GPIO15 | Bitwise SET operation for Aux. mode of KCOL1 0: Keep 1: SET bits |
| 26:24 |  | GPIO14 | Bitwise SET operation for Aux. mode of KCOL2 0: Keep 1: SET bits |
| 22:20 |  | GPIO13 | Bitwise SET operation for Aux. mode of KCOL3 0: Keep 1: SET bits |
| 18:16 |  | GPIO12 | Bitwise SET operation for Aux. mode of KCOL4 0: Keep 1: SET bits |
| 14:12 |  | GPIO11 | Bitwise SET operation for Aux. mode of UTXD1 0: Keep 1: SET bits |
| 10:8 |  | GPIO10 | Bitwise SET operation for Aux. mode of URXD1 0: Keep 1: SET bits |
| 6:4 |  | GPIO9 | Bitwise SET operation for Aux. mode of GPIO_9 0: Keep 1: SET bits |
| 2:0 |  | GPIO8 | Bitwise SET operation for Aux. mode of GPIO_8 0: Keep |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: SET bits |

A0020C18	GPIO_MODE1_

CLR

GPIO Mode Control	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO15 |  |  |  | GPIO14 |  |  |  | GPIO13 |  |  |  | GPIO12 |  |  |
| Type |  | WO |  |  |  | WO |  |  |  | WO |  |  |  | WO |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  | GPIO11 |  |  |  | GPIO10 |  |  |  | GPIO9 |  |  |  | GPIO8 |  |  |
| Type |  | WO |  |  |  | WO |  |  |  | WO |  |  |  | WO |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_MODE1

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30:28 |  | GPIO15 | Bitwise CLR operation for Aux. mode of KCOL1 0: Keep 1: CLR bits |
| 26:24 |  | GPIO14 | Bitwise CLR operation for Aux. mode of KCOL2 0: Keep 1: CLR bits |
| 22:20 |  | GPIO13 | Bitwise CLR operation for Aux. mode of KCOL3 0: Keep 1: CLR bits |
| 18:16 |  | GPIO12 | Bitwise CLR operation for Aux. mode of KCOL4 0: Keep 1: CLR bits |
| 14:12 |  | GPIO11 | Bitwise CLR operation for Aux. mode of UTXD1 0: Keep 1: CLR bits |
| 10:8 |  | GPIO10 | Bitwise CLR operation for Aux. mode of URXD1 0: Keep 1: CLR bits |
| 6:4 |  | GPIO9 | Bitwise CLR operation for Aux. mode of GPIO_9 0: Keep 1: CLR bits |
| 2:0 |  | GPIO8 | Bitwise CLR operation for Aux. mode of GPIO_8 0: Keep 1: CLR bits |

A0020C20	GPIO_MODE2	GPIO Mode Control	11000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO23 |  |  |  | GPIO22 |  |  |  | GPIO21 |  |  |  | GPIO20 |  |  |
| Type |  | RW |  |  |  | RW |  |  |  | RW |  |  |  | RW |  |  |
| Reset |  | 0 | 0 | 1 |  | 0 | 0 | 1 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  | GPIO19 |  |  |  | GPIO18 |  |  |  | GPIO17 |  |  |  | GPIO16 |  |  |
| Type |  | RW |  |  |  | RW |  |  |  | RW |  |  |  | RW |  |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Overview: Configures GPIO aux. mode

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30:28 |  | GPIO23 | Aux. mode of GPIO_23 0: GPIO23 (IO) 1: BPI_BUS1 (O) 2: Reserved 3: Reserved 4: Reserved 5: Reserved 6: Reserved 7: Reserved |
| 26:24 |  | GPIO22 | Aux. mode of GPIO_22 0: GPIO22 (IO) 1: BPI_BUS2 (O) 2: Reserved 3: Reserved 4: Reserved 5: Reserved 6: Reserved 7: Reserved |
| 22:20 |  | GPIO21 | Aux. mode of GPIO_21 0: GPIO21 (IO) 1: KROW0 (IO) 2: Reserved 3: Reserved 4: Reserved 5: MCINS (I) 6: BTDBGIN (I) 7: Reserved |
| 18:16 |  | GPIO20 | Aux. mode of GPIO_20 0: GPIO20 (IO) 1: KROW1 (IO) 2: EINT14 (I) 3: EDIDO (O) 4: BTPRI (IO) 5: JTRCK (O) 6: BTDBGACKN (O) 7: Reserved |
| 14:12 |  | GPIO19 | Aux. mode of GPIO_19 0: GPIO19 (IO) 1: KROW2 (IO) 2: PWM1 (O) 3: EDIWS (O) 4: FMJTDO (O) 5: JTDO (O) 6: BTJTDO (O) 7: Reserved |
| 10:8 |  | GPIO18 | Aux. mode of GPIO_18 0: GPIO18 (IO) 1: KROW3 (IO) 2: EINT13 (I) 3: CLKO0 (O) 4: FMJTRSTB (I) 5: JTRST_B (I) 6: BTJTRSTB (I) |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 7: Reserved |
| 6:4 |  | GPIO17 | Aux. mode of GPIO_17 0: GPIO17 (IO) 1: KROW4 (IO) 2: U2TXD (O) 3: EDICK (O) 4: Reserved 5: Reserved 6: Reserved 7: Reserved |
| 2:0 |  | GPIO16 | Aux. mode of GPIO_16 0: GPIO16 (IO) 1: KCOL0 (IO) 2: Reserved 3: Reserved 4: Reserved 5: Reserved 6: Reserved 7: Reserved |

A0020C24	GPIO_MODE2_ SET

GPIO Mode Control	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO23 |  |  |  | GPIO22 |  |  |  | GPIO21 |  |  |  | GPIO20 |  |  |
| Type |  | WO |  |  |  | WO |  |  |  | WO |  |  |  | WO |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  | GPIO19 |  |  |  | GPIO18 |  |  |  | GPIO17 |  |  |  | GPIO16 |  |  |
| Type |  | WO |  |  |  | WO |  |  |  | WO |  |  |  | WO |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_MODE2

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30:28 |  | GPIO23 | Bitwise SET operation for Aux. mode of BPI_BUS1 0: Keep 1: SET bits |
| 26:24 |  | GPIO22 | Bitwise SET operation for Aux. mode of BPI_BUS2 0: Keep 1: SET bits |
| 22:20 |  | GPIO21 | Bitwise SET operation for Aux. mode of KROW0 0: Keep 1: SET bits |
| 18:16 |  | GPIO20 | Bitwise SET operation for Aux. mode of KROW1 0: Keep 1: SET bits |
| 14:12 |  | GPIO19 | Bitwise SET operation for Aux. mode of KROW2 0: Keep 1: SET bits |
| 10:8 |  | GPIO18 | Bitwise SET operation for Aux. mode of KROW3 0: Keep 1: SET bits |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 6:4 |  | GPIO17 | Bitwise SET operation for Aux. mode of KROW4 0: Keep 1: SET bits |
| 2:0 |  | GPIO16 | Bitwise SET operation for Aux. mode of KCOL0 0: Keep 1: SET bits |

A0020C28	GPIO_MODE2_

CLR

GPIO Mode Control	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO23 |  |  |  | GPIO22 |  |  |  | GPIO21 |  |  |  | GPIO20 |  |  |
| Type |  | WO |  |  |  | WO |  |  |  | WO |  |  |  | WO |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  | GPIO19 |  |  |  | GPIO18 |  |  |  | GPIO17 |  |  |  | GPIO16 |  |  |
| Type |  | WO |  |  |  | WO |  |  |  | WO |  |  |  | WO |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_MODE2

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30:28 |  | GPIO23 | Bitwise CLR operation for Aux. mode of BPI_BUS1 0: Keep 1: CLR bits |
| 26:24 |  | GPIO22 | Bitwise CLR operation for Aux. mode of BPI_BUS2 0: Keep 1: CLR bits |
| 22:20 |  | GPIO21 | Bitwise CLR operation for Aux. mode of KROW0 0: Keep 1: CLR bits |
| 18:16 |  | GPIO20 | Bitwise CLR operation for Aux. mode of KROW1 0: Keep 1: CLR bits |
| 14:12 |  | GPIO19 | Bitwise CLR operation for Aux. mode of KROW2 0: Keep 1: CLR bits |
| 10:8 |  | GPIO18 | Bitwise CLR operation for Aux. mode of KROW3 0: Keep 1: CLR bits |
| 6:4 |  | GPIO17 | Bitwise CLR operation for Aux. mode of KROW4 0: Keep 1: CLR bits |
| 2:0 |  | GPIO16 | Bitwise CLR operation for Aux. mode of KCOL0 0: Keep 1: CLR bits |

A0020C30	GPIO_MODE3	GPIO Mode Control	00000001

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Name |  | GPIO31 |  |  |  | GPIO30 |  |  | GPIO29 |  |  |  | GPIO28 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type |  | RW |  |  |  | RW |  |  | RW |  |  |  | RW |  |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO27 |  |  |  |  | GPIO26 |  |  |  | GPIO25 |  |  |  | GPIO24 |  |  |
| Type | RW |  |  |  |  | RW |  |  |  | RW |  |  |  | RW |  |  |
| Reset | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 1 |

Overview: Configures GPIO aux. mode

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30:28 |  | GPIO31 | Aux. mode of GPIO_31 0: GPIO31 (IO) 1: MCCK (O) 2: Reserved 3: Reserved 4: U2RXD (I) 5: Reserved 6: Reserved 7: Reserved |
| 26:24 |  | GPIO30 | Aux. mode of GPIO_30 0: GPIO30 (IO) 1: CMCSK (I) 2: LPTE (I) 3: CMCSD2 (I) 4: EINT16 (I) 5: Reserved 6: JTRCK (O) 7: Reserved |
| 23:20 |  | GPIO29 | Aux. mode of GPIO_29 0: GPIO29 (IO) 1: CMMCLK (O) 2: LSA0DA1 (O) 3: DAISYNC (O) 4: SPIMISO (IO) 5: FMJTDO (O) 6: JTDO (O) 7: Reserved 8: MC2DA0 (IO) |
| 19:16 |  | GPIO28 | Aux. mode of GPIO_28 0: GPIO28 (IO) 1: CMCSD1 (I) 2: LSDA1 (IO) 3: DAIPCMOUT (O) 4: SPIMOSI (IO) 5: FMJTRSTB (I) 6: JTRST_B (I) 7: Reserved 8: MC2CK (O) |
| 15:12 |  | GPIO27 | Aux. mode of GPIO_27 0: GPIO27 (IO) 1: CMCSD0 (I) 2: LSCE_B1 (O) 3: DAIPCMIN (I) 4: SPISCK (IO) 5: FMJTCK (I) 6: JTCK (I) 7: Reserved 8: MC2CM0 (O) |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 10:8 |  | GPIO26 | Aux. mode of GPIO_26 0: GPIO26 (IO) 1: CMPDN (O) 2: LSCK1 (O) 3: DAICLK (O) 4: SPICS (IO) 5: FMJTMS (I) 6: JTMS (I) 7: Reserved) |
| 6:4 |  | GPIO25 | Aux. mode of GPIO_25 0: GPIO25 (IO) 1: CMRST (O) 2: LSRSTB (O) 3: CLKO1 (O) 4: EINT15 (I) 5: FMJTDI (I) 6: JTDI (I) 7: Reserved |
| 2:0 |  | GPIO24 | Aux. mode of GPIO_24 0: GPIO24 (IO) 1: BPI_BUS0 (O) 2: Reserved 3: Reserved 4: Reserved 5: Reserved 6: Reserved 7: Reserved |

A0020C34	GPIO_MODE3_ SET

GPIO Mode Control	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO31 |  |  |  | GPIO30 |  |  | GPIO29 |  |  |  | GPIO28 |  |  |  |
| Type |  | WO |  |  |  | WO |  |  | WO |  |  |  | WO |  |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO27 |  |  |  |  | GPIO26 |  |  |  | GPIO25 |  |  |  | GPIO24 |  |  |
| Type | WO |  |  |  |  | WO |  |  |  | WO |  |  |  | WO |  |  |
| Reset | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_MODE3

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30:28 |  | GPIO31 | Bitwise SET operation for Aux. mode of MCCK 0: Keep 1: SET bits |
| 26:24 |  | GPIO30 | Bitwise SET operation for Aux. mode of CMCSK 0: Keep 1: SET bits |
| 23:20 |  | GPIO29 | Bitwise SET operation for Aux. mode of CMMCLK 0: Keep 1: SET bits |
| 19:16 |  | GPIO28 | Bitwise SET operation for Aux. mode of CMCSD1 0: Keep |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: SET bits |
| 15:12 |  | GPIO27 | Bitwise SET operation for Aux. mode of CMCSD0 0: Keep 1: SET bits |
| 10:8 |  | GPIO26 | Bitwise SET operation for Aux. mode of CMPDN 0: Keep 1: SET bits |
| 6:4 |  | GPIO25 | Bitwise SET operation for Aux. mode of CMRST 0: Keep 1: SET bits |
| 2:0 |  | GPIO24 | Bitwise SET operation for Aux. mode of BPI_BUS0 0: Keep 1: SET bits |

A0020C38	GPIO_MODE3_

CLR

GPIO Mode Control	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO31 |  |  |  | GPIO30 |  |  | GPIO29 |  |  |  | GPIO28 |  |  |  |
| Type |  | WO |  |  |  | WO |  |  | WO |  |  |  | WO |  |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO27 |  |  |  |  | GPIO26 |  |  |  | GPIO25 |  |  |  | GPIO24 |  |  |
| Type | WO |  |  |  |  | WO |  |  |  | WO |  |  |  | WO |  |  |
| Reset | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_MODE3

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30:28 |  | GPIO31 | Bitwise CLR operation for Aux. mode of MCCK 0: Keep 1: CLR bits |
| 26:24 |  | GPIO30 | Bitwise CLR operation for Aux. mode of CMCSK 0: Keep 1: CLR bits |
| 23:20 |  | GPIO29 | Bitwise CLR operation for Aux. mode of CMMCLK 0: Keep 1: CLR bits |
| 19:16 |  | GPIO28 | Bitwise CLR operation for Aux. mode of CMCSD1 0: Keep 1: CLR bits |
| 15:12 |  | GPIO27 | Bitwise CLR operation for Aux. mode of CMCSD0 0: Keep 1: CLR bits |
| 10:8 |  | GPIO26 | Bitwise CLR operation for Aux. mode of CMPDN 0: Keep 1: CLR bits |
| 6:4 |  | GPIO25 | Bitwise CLR operation for Aux. mode of CMRST 0: Keep 1: CLR bits |
| 2:0 |  | GPIO24 | Bitwise CLR operation for Aux. mode of BPI_BUS0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Keep 1: CLR bits |

A0020C40	GPIO_MODE4	GPIO Mode Control	11100000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO39 |  |  |  | GPIO38 |  |  |  | GPIO37 |  |  |  | GPIO36 |  |  |
| Type |  | RW |  |  |  | RW |  |  |  | RW |  |  |  | RW |  |  |
| Reset |  | 0 | 0 | 1 |  | 0 | 0 | 1 |  | 0 | 0 | 1 |  | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  | GPIO35 |  |  |  | GPIO34 |  |  |  | GPIO33 |  |  |  | GPIO32 |  |  |
| Type |  | RW |  |  |  | RW |  |  |  | RW |  |  |  | RW |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |

Overview: Configures GPIO aux. mode

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30:28 |  | GPIO39 | Aux. mode of GPIO_39 0: GPIO39 (IO) 1: SIM1_SCLK (IO) 2: Reserved 3: Reserved 4: Reserved 5: Reserved 6: Reserved 7: Reserved |
| 26:24 |  | GPIO38 | Aux. mode of GPIO_38 0: GPIO38 (IO) 1: SIM1_SRST (IO) 2: Reserved 3: Reserved 4: Reserved 5: Reserved 6: Reserved 7: Reserved |
| 22:20 |  | GPIO37 | Aux. mode of GPIO_37 0: GPIO37 (IO) 1: SIM1_SIO (IO) 2: Reserved 3: Reserved 4: Reserved 5: Reserved 6: Reserved 7: Reserved |
| 18:16 |  | GPIO36 | Aux. mode of GPIO_36 0: GPIO36 (IO) 1: MCDA3 (IO) 2: EINT19 (I) 3: CLKO2 (O) 4: DAIPCMOUT (O) 5: Reserved 6: Reserved 7: Reserved |
| 14:12 |  | GPIO35 | Aux. mode of GPIO_35 0: GPIO35 (IO) 1: MCDA2 (IO) |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 2: EINT18 (I) 3: Reserved 4: DAICLK (O) 5: Reserved 6: Reserved 7: Reserved |
| 10:8 |  | GPIO34 | Aux. mode of GPIO_34 0: GPIO34 (IO) 1: MCDA1 (IO) 2: EINT17 (I) 3: Reserved 4: DAIPCMIN (I) 5: Reserved 6: Reserved 7: Reserved |
| 6:4 |  | GPIO33 | Aux. mode of GPIO_33 0: GPIO33 (IO) 1: MCDA0 (IO) 2: Reserved 3: Reserved 4: DAISYNC (O) 5: Reserved 6: Reserved 7: Reserved |
| 2:0 |  | GPIO32 | Aux. mode of GPIO_32 0: GPIO32 (IO) 1: MCCM0 (O) 2: Reserved 3: Reserved 4: U2TXD (O) 5: Reserved 6: Reserved 7: Reserved |

A0020C44	GPIO_MODE4_ SET

GPIO Mode Control	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO39 |  |  |  | GPIO38 |  |  |  | GPIO37 |  |  |  | GPIO36 |  |  |
| Type |  | WO |  |  |  | WO |  |  |  | WO |  |  |  | WO |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  | GPIO35 |  |  |  | GPIO34 |  |  |  | GPIO33 |  |  |  | GPIO32 |  |  |
| Type |  | WO |  |  |  | WO |  |  |  | WO |  |  |  | WO |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_MODE4

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30:28 |  | GPIO39 | Bitwise SET operation for Aux. mode of SIM1_SCLK 0: Keep 1: SET bits |
| 26:24 |  | GPIO38 | Bitwise SET operation for Aux. mode of SIM1_SRST 0: Keep 1: SET bits |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 22:20 |  | GPIO37 | Bitwise SET operation for Aux. mode of SIM1_SIO 0: Keep 1: SET bits |
| 18:16 |  | GPIO36 | Bitwise SET operation for Aux. mode of MCDA3 0: Keep 1: SET bits |
| 14:12 |  | GPIO35 | Bitwise SET operation for Aux. mode of MCDA2 0: Keep 1: SET bits |
| 10:8 |  | GPIO34 | Bitwise SET operation for Aux. mode of MCDA1 0: Keep 1: SET bits |
| 6:4 |  | GPIO33 | Bitwise SET operation for Aux. mode of MCDA0 0: Keep 1: SET bits |
| 2:0 |  | GPIO32 | Bitwise SET operation for Aux. mode of MCCM0 0: Keep 1: SET bits |

A0020C48	GPIO_MODE4_

CLR

GPIO Mode Control	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO39 |  |  |  | GPIO38 |  |  |  | GPIO37 |  |  |  | GPIO36 |  |  |
| Type |  | WO |  |  |  | WO |  |  |  | WO |  |  |  | WO |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  | GPIO35 |  |  |  | GPIO34 |  |  |  | GPIO33 |  |  |  | GPIO32 |  |  |
| Type |  | WO |  |  |  | WO |  |  |  | WO |  |  |  | WO |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_MODE4

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30:28 |  | GPIO39 | Bitwise CLR operation for Aux. mode of SIM1_SCLK 0: Keep 1: CLR bits |
| 26:24 |  | GPIO38 | Bitwise CLR operation for Aux. mode of SIM1_SRST 0: Keep 1: CLR bits |
| 22:20 |  | GPIO37 | Bitwise CLR operation for Aux. mode of SIM1_SIO 0: Keep 1: CLR bits |
| 18:16 |  | GPIO36 | Bitwise CLR operation for Aux. mode of MCDA3 0: Keep 1: CLR bits |
| 14:12 |  | GPIO35 | Bitwise CLR operation for Aux. mode of MCDA2 0: Keep 1: CLR bits |
| 10:8 |  | GPIO34 | Bitwise CLR operation for Aux. mode of MCDA1 0: Keep |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: CLR bits |
| 6:4 |  | GPIO33 | Bitwise CLR operation for Aux. mode of MCDA0 0: Keep 1: CLR bits |
| 2:0 |  | GPIO32 | Bitwise CLR operation for Aux. mode of MCCM0 0: Keep 1: CLR bits |

A0020C50	GPIO_MODE5	GPIO Mode Control	01000111

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO47 |  |  |  | GPIO46 |  |  |  | GPIO45 |  |  |  | GPIO44 |  |  |
| Type |  | RW |  |  |  | RW |  |  |  | RW |  |  |  | RW |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 1 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  | GPIO43 |  |  |  | GPIO42 |  |  |  | GPIO41 |  |  |  | GPIO40 |  |  |
| Type |  | RW |  |  |  | RW |  |  |  | RW |  |  |  | RW |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 1 |  | 0 | 0 | 1 |  | 0 | 0 | 1 |

Overview: Configures GPIO aux. mode

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30:28 |  | GPIO47 | Aux. mode of GPIO_47 0: GPIO47 (IO) 1: LSCK0 (O) 2: Reserved 3: CMPDN (O) 4: Reserved 5: Reserved 6: Reserved 7: Reserved |
| 26:24 |  | GPIO46 | Aux. mode of GPIO_46 0: GPIO46 (IO) 1: LSCE_B0 (O) 2: EINT20 (I) 3: CMCSD0 (I) 4: CLKO4 (O) 5: Reserved 6: Reserved 7: Reserved |
| 22:20 |  | GPIO45 | Aux. mode of GPIO_45 0: GPIO45 (IO) 1: LSRSTB (O) 2: Reserved 3: CMRST (O) 4: Reserved 5: Reserved 6: Reserved 7: Reserved |
| 18:16 |  | GPIO44 | Aux. mode of GPIO_44 0: GPIO44 (IO) 1: SDA (IO) 2: Reserved 3: Reserved 4: Reserved |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 5: Reserved 6: Reserved 7: Reserved |
| 14:12 |  | GPIO43 | Aux. mode of GPIO_43 0: GPIO43 (IO) 1: SCL (IO) 2: Reserved 3: Reserved 4: Reserved 5: Reserved 6: Reserved 7: Reserved |
| 10:8 |  | GPIO42 | Aux. mode of GPIO_42 0: GPIO42 (IO) 1: SIM2_SCLK (IO) 2: LSCE1_B1 (O) 3: Reserved 4: Reserved 5: Reserved 6: Reserved 7: Reserved |
| 6:4 |  | GPIO41 | Aux. mode of GPIO_41 0: GPIO41 (IO) 1: SIM2_SRST (IO) 2: CLKO3 (O) 3: U2CTS (I) 4: Reserved 5: Reserved 6: Reserved 7: Reserved |
| 2:0 |  | GPIO40 | Aux. mode of GPIO_40 0: GPIO40 (IO) 1: SIM2_SIO (IO) 2: Reserved 3: U2RTS (O) 4: Reserved 5: Reserved 6: Reserved 7: Reserved |

A0020C54	GPIO_MODE5_ SET

GPIO Mode Control	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO47 |  |  |  | GPIO46 |  |  |  | GPIO45 |  |  |  | GPIO44 |  |  |
| Type |  | WO |  |  |  | WO |  |  |  | WO |  |  |  | WO |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  | GPIO43 |  |  |  | GPIO42 |  |  |  | GPIO41 |  |  |  | GPIO40 |  |  |
| Type |  | WO |  |  |  | WO |  |  |  | WO |  |  |  | WO |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_MODE5

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30:28 |  | GPIO47 | Bitwise SET operation for Aux. mode of LSCK 0: Keep 1: SET bits |
| 26:24 |  | GPIO46 | Bitwise SET operation for Aux. mode of LSCE_B 0: Keep 1: SET bits |
| 22:20 |  | GPIO45 | Bitwise SET operation for Aux. mode of LSRSTB 0: Keep 1: SET bits |
| 18:16 |  | GPIO44 | Bitwise SET operation for Aux. mode of SDA28 0: Keep 1: SET bits |
| 14:12 |  | GPIO43 | Bitwise SET operation for Aux. mode of SCL28 0: Keep 1: SET bits |
| 10:8 |  | GPIO42 | Bitwise SET operation for Aux. mode of SIM2_SCLK 0: Keep 1: SET bits |
| 6:4 |  | GPIO41 | Bitwise SET operation for Aux. mode of SIM2_SRST 0: Keep 1: SET bits |
| 2:0 |  | GPIO40 | Bitwise SET operation for Aux. mode of SIM2_SIO 0: Keep 1: SET bits |

A0020C58	GPIO_MODE5_

CLR

GPIO Mode Control	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO47 |  |  |  | GPIO46 |  |  |  | GPIO45 |  |  |  | GPIO44 |  |  |
| Type |  | WO |  |  |  | WO |  |  |  | WO |  |  |  | WO |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  | GPIO43 |  |  |  | GPIO42 |  |  |  | GPIO41 |  |  |  | GPIO40 |  |  |
| Type |  | WO |  |  |  | WO |  |  |  | WO |  |  |  | WO |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_MODE5

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30:28 |  | GPIO47 | Bitwise CLR operation for Aux. mode of LSCK 0: Keep 1: CLR bits |
| 26:24 |  | GPIO46 | Bitwise CLR operation for Aux. mode of LSCE_B 0: Keep 1: CLR bits |
| 22:20 |  | GPIO45 | Bitwise CLR operation for Aux. mode of LSRSTB 0: Keep 1: CLR bits |
| 18:16 |  | GPIO44 | Bitwise CLR operation for Aux. mode of SDA28 0: Keep |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: CLR bits |
| 14:12 |  | GPIO43 | Bitwise CLR operation for Aux. mode of SCL28 0: Keep 1: CLR bits |
| 10:8 |  | GPIO42 | Bitwise CLR operation for Aux. mode of SIM2_SCLK 0: Keep 1: CLR bits |
| 6:4 |  | GPIO41 | Bitwise CLR operation for Aux. mode of SIM2_SRST 0: Keep 1: CLR bits |
| 2:0 |  | GPIO40 | Bitwise CLR operation for Aux. mode of SIM2_SIO 0: Keep 1: CLR bits |

A0020C60	GPIO_MODE6	GPIO Mode Control	00001000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO55 |  |  |  | GPIO54 |  |  |  | GPIO53 |  |  |  | GPIO52 |  |  |
| Type |  | RW |  |  |  | RW |  |  |  | RW |  |  |  | RW |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  | GPIO51 |  |  | GPIO50 |  |  |  |  | GPIO49 |  |  |  | GPIO48 |  |  |
| Type |  | RW |  |  | RW |  |  |  |  | RW |  |  |  | RW |  |  |
| Reset |  | 0 | 0 | 1 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |

Overview: Configures GPIO aux. mode

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30:28 |  | GPIO55 | Aux. mode of GPIO_55 0: AGPIO55 (AGIO) 1: Reserved 2: Reserved 3: Reserved 4: Reserved 5: Reserved 6: Reserved 7: Reserved |
| 26:24 |  | GPIO54 | Aux. mode of GPIO_54 0: AGPIO54 (AGIO) 1: Reserved 2: Reserved 3: Reserved 4: Reserved 5: Reserved 6: Reserved 7: Reserved |
| 22:20 |  | GPIO53 | Aux. mode of GPIO_53 0: AGPIO53 (AGI) 1: SRCLKENAI (I) 2: EINT24 (I) 3: Reserved 4: Reserved 5: Reserved 6: Reserved |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 7: Reserved |
| 18:16 |  | GPIO52 | Aux. mode of GPIO_52 0: AGPI52 (AGI) 1: Reserved 2: EINT23 (I) 3: Reserved 4: Reserved 5: Reserved 6: Reserved 7: Reserved |
| 14:12 |  | GPIO51 | Aux. mode of GPIO_51 0: GPIO51 (IO) 1: RESETB (IO) 2: Reserved 3: Reserved 4: Reserved 5: Reserved 6: Reserved 7: Reserved |
| 11:8 |  | GPIO50 | Aux. mode of GPIO_50 0: GPIO50 (IO) 1: LPTE (I) 2: EINT22 (I) 3: CMCSK (I) 4: CMCSD2 (I) 5: Reserved 6: MCINS (I) 7: Reserved 8: Reserved 9: CLKO5 (O) |
| 6:4 |  | GPIO49 | Aux. mode of GPIO_49 0: GPIO49 (IO) 1: LSA0DA0 (O) 2: LSCE1_B0 (O) 3: CMMCLK (O) 4: Reserved 5: Reserved 6: Reserved 7: Reserved |
| 2:0 |  | GPIO48 | Aux. mode of GPIO_48 0: GPIO48 (IO) 1: LSDA0 (IO) 2: EINT21 (I) 3: CMCSD1 (I) 4: WIFITOBT (I) 5: Reserved 6: Reserved 7: Reserved |

A0020C64	GPIO_MODE6_ SET

GPIO Mode Control	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO55 |  |  |  | GPIO54 |  |  |  | GPIO53 |  |  |  | GPIO52 |  |  |
| Type |  | WO |  |  |  | WO |  |  |  | WO |  |  |  | WO |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

| Name |  | GPIO51 |  |  | GPIO50 |  |  |  |  | GPIO49 |  |  |  | GPIO48 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type |  | WO |  |  | WO |  |  |  |  | WO |  |  |  | WO |  |  |
| Reset |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_MODE6

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30:28 |  | GPIO55 | Bitwise SET operation for Aux. mode of SRCLKENAI 0: Keep 1: SET bits |
| 26:24 |  | GPIO54 | Bitwise SET operation for Aux. mode of EINT 0: Keep 1: SET bits |
| 22:20 |  | GPIO53 | Bitwise SET operation for Aux. mode of TP4 0: Keep 1: SET bits |
| 18:16 |  | GPIO52 | Bitwise SET operation for Aux. mode of TP3 0: Keep 1: SET bits |
| 14:12 |  | GPIO51 | Bitwise SET operation for Aux. mode of RESETB 0: Keep 1: SET bits |
| 11:8 |  | GPIO50 | Bitwise SET operation for Aux. mode of LPTE 0: Keep 1: SET bits |
| 6:4 |  | GPIO49 | Bitwise SET operation for Aux. mode of LSA0 0: Keep 1: SET bits |
| 2:0 |  | GPIO48 | Bitwise SET operation for Aux. mode of LSDA 0: Keep 1: SET bits |

A0020C68	GPIO_MODE6_

CLR

GPIO Mode Control	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | GPIO55 |  |  |  | GPIO54 |  |  |  | GPIO53 |  |  |  | GPIO52 |  |  |
| Type |  | WO |  |  |  | WO |  |  |  | WO |  |  |  | WO |  |  |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  | GPIO51 |  |  | GPIO50 |  |  |  |  | GPIO49 |  |  |  | GPIO48 |  |  |
| Type |  | WO |  |  | WO |  |  |  |  | WO |  |  |  | WO |  |  |
| Reset |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_MODE6

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 30:28 |  | GPIO55 | Bitwise CLR operation for Aux. mode of SRCLKENAI 0: Keep 1: CLR bits |
| 26:24 |  | GPIO54 | Bitwise CLR operation for Aux. mode of EINT 0: Keep |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: CLR bits |
| 22:20 |  | GPIO53 | Bitwise CLR operation for Aux. mode of TP4 0: Keep 1: CLR bits |
| 18:16 |  | GPIO52 | Bitwise CLR operation for Aux. mode of TP3 0: Keep 1: CLR bits |
| 14:12 |  | GPIO51 | Bitwise CLR operation for Aux. mode of RECLRB 0: Keep 1: CLR bits |
| 11:8 |  | GPIO50 | Bitwise CLR operation for Aux. mode of LPTE 0: Keep 1: CLR bits |
| 6:4 |  | GPIO49 | Bitwise CLR operation for Aux. mode of LSA0 0: Keep 1: CLR bits |
| 2:0 |  | GPIO48 | Bitwise CLR operation for Aux. mode of LSDA 0: Keep 1: CLR bits |

A0020D10	GPIO_TDSEL	GPIO TDSEL Control	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  | GPIO50 |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO46 |  | GPIO42 |  | GPIO41 |  | GPIO36 |  | GPIO25 |  | GPIO18 |  | GPIO9 |  | GPIO8 |  |
| Type | RW |  | RW |  | RW |  | RW |  | RW |  | RW |  | RW |  | RW |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: GPIO TX duty control register

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 17:16 | GPIO50 | GPIO50_TDSEL | GPIO50 Tx duty control |
| 15:14 | GPIO46 | GPIO46_TDSEL | GPIO46 Tx duty control |
| 13:12 | GPIO42 | GPIO42_TDSEL | GPIO42 Tx duty control |
| 11:10 | GPIO41 | GPIO41_TDSEL | GPIO41 Tx duty control |
| 9:8 | GPIO36 | GPIO36_TDSEL | GPIO36 Tx duty control |
| 7:6 | GPIO25 | GPIO25_TDSEL | GPIO25 Tx duty control |
| 5:4 | GPIO18 | GPIO18_TDSEL | GPIO18 Tx duty control |
| 3:2 | GPIO9 | GPIO9_TDSEL | GPIO9 Tx duty control |
| 1:0 | GPIO8 | GPIO8_TDSEL | GPIO8 Tx duty control |

A0020D14	GPIO_TDSEL_S GPIO TDSEL Control	00000000

ET

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  | GPIO50 |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | WO |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO46 |  | GPIO42 |  | GPIO41 |  | GPIO36 |  | GPIO25 |  | GPIO18 |  | GPIO9 |  | GPIO8 |  |
| Type | WO |  | WO |  | WO |  | WO |  | WO |  | WO |  | WO |  | WO |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_TDSEL

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 17:16 | GPIO50 | GPIO50_TDSEL | Bitwise SET operation of GPIO50_TDSEL Tx duty control 0: Keep 1: SET bits |
| 15:14 | GPIO46 | GPIO46_TDSEL | Bitwise SET operation of GPIO46_TDSEL Tx duty control 0: Keep 1: SET bits |
| 13:12 | GPIO42 | GPIO42_TDSEL | Bitwise SET operation of GPIO42_TDSEL Tx duty control 0: Keep 1: SET bits |
| 11:10 | GPIO41 | GPIO41_TDSEL | Bitwise SET operation of GPIO41_TDSEL Tx duty control 0: Keep 1: SET bits |
| 9:8 | GPIO36 | GPIO36_TDSEL | Bitwise SET operation of GPIO36_TDSEL Tx duty control 0: Keep 1: SET bits |
| 7:6 | GPIO25 | GPIO25_TDSEL | Bitwise SET operation of GPIO25_TDSEL Tx duty control 0: Keep 1: SET bits |
| 5:4 | GPIO18 | GPIO18_TDSEL | Bitwise SET operation of GPIO18_TDSEL Tx duty control 0: Keep 1: SET bits |
| 3:2 | GPIO9 | GPIO9_TDSEL | Bitwise SET operation of GPIO9_TDSEL Tx duty control 0: Keep 1: SET bits |
| 1:0 | GPIO8 | GPIO8_TDSEL | Bitwise SET operation of GPIO8_TDSEL Tx duty control 0: Keep 1: SET bits |

A0020D18	GPIO_TDSEL_C GPIO TDSEL Control	00000000

LR

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  | GPIO50 |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | WO |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | GPIO46 |  | GPIO42 |  | GPIO41 |  | GPIO36 |  | GPIO25 |  | GPIO18 |  | GPIO9 |  | GPIO8 |  |
| Type | WO |  | WO |  | WO |  | WO |  | WO |  | WO |  | WO |  | WO |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: For bitwise access of GPIO_TDSEL

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 17:16 | GPIO50 | GPIO50_TDSEL | Bitwise CLR operation of GPIO50_TDSEL Tx duty control 0: Keep 1: CLR bits |
| 15:14 | GPIO46 | GPIO46_TDSEL | Bitwise CLR operation of GPIO46_TDSEL Tx duty control 0: Keep 1: CLR bits |
| 13:12 | GPIO42 | GPIO42_TDSEL | Bitwise CLR operation of GPIO42_TDSEL Tx duty control 0: Keep 1: CLR bits |
| 11:10 | GPIO41 | GPIO41_TDSEL | Bitwise CLR operation of GPIO41_TDSEL Tx duty control 0: Keep 1: CLR bits |
| 9:8 | GPIO36 | GPIO36_TDSEL | Bitwise CLR operation of GPIO36_TDSEL Tx duty control 0: Keep 1: CLR bits |
| 7:6 | GPIO25 | GPIO25_TDSEL | Bitwise CLR operation of GPIO25_TDSEL Tx duty control 0: Keep 1: CLR bits |
| 5:4 | GPIO18 | GPIO18_TDSEL | Bitwise CLR operation of GPIO18_TDSEL Tx duty control 0: Keep 1: CLR bits |
| 3:2 | GPIO9 | GPIO9_TDSEL | Bitwise CLR operation of GPIO9_TDSEL Tx duty control 0: Keep 1: CLR bits |
| 1:0 | GPIO8 | GPIO8_TDSEL | Bitwise CLR operation of GPIO8_TDSEL Tx duty control 0: Keep 1: CLR bits |

A0020E00	CLK_OUT0	CLK Out Selection Control	00000004

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | CLK_OUT0 |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 1 | 0 | 0 |

Overview: CLK OUT0 Setting

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 3:0 | CLK_OUT0 | CFG0 | Selects clock output for CLKO_0 [1]: 26Mhz clock [4]: 32Khz_clock others: debug clock |

A0020E10	CLK_OUT1	CLK Out Selection Control	00000004

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | CLK_OUT1 |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 1 | 0 | 0 |

Overview: CLK OUT1 setting

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 3:0 | CLK_OUT1 | CFG1 | Selects clock output for CLKO_1 [1]: 26Mhz clock [4]: 32Khz_clock others: debug clock |

A0020E20	CLK_OUT2	CLK Out Selection Control	00000004

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | CLK_OUT2 |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 1 | 0 | 0 |

Overview: CLK OUT2 setting

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 3:0 | CLK_OUT2 | CFG2 | Selects clock output for CLKO_2 [1]: 26Mhz clock [4]: 32Khz_clock others: debug clock |

A0020E30	CLK_OUT3	CLK Out Selection Control	00000004

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | CLK_OUT3 |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 1 | 0 | 0 |

Overview: CLK OUT3 setting

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 3:0 | CLK_OUT3 | CFG3 | Selects clock output for CLKO_3 [1]: 26Mhz clock [4]: 32Khz_clock |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | others: debug clock |

A0020E40	CLK_OUT4	CLK Out Selection Control	00000004

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | CLK_OUT4 |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 1 | 0 | 0 |

Overview: CLK OUT4 setting

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 3:0 | CLK_OUT4 | CFG4 | Selects clock output for CLKO_4 [1]: 26Mhz clock [4]: 32Khz_clock others: debug clock |

A0020E50	CLK_OUT5	CLK Out Selection Control	00000004

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | CLK_OUT5 |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 1 | 0 | 0 |

Overview: CLK OUT5 setting

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 3:0 | CLK_OUT5 | CFG5 | Selects clock output for CLKO_5 [1]: 26Mhz clock [4]: 32Khz_clock others: debug clock |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

General-purpose Timer

### General Descriptions

Three general-purpose timers are provided. Two timers are 16 bits long and one timer is 32 bits long. Each runs independently. GPT1 ~ 2 use 32k clock source to count, whereas GPT4 uses 26M clock source. The 26M clock source can be gated when the system enters the sleep mode, and this will cause GPT4 to stop counting, whereas 32k clock source is always toggling. GPT1 and GPT2 can operate in one of the two modes: one-shot mode and auto-repeat mode.	GPT4 are free running timer. In the one-shot mode, when the timer counts down and reaches 0, it will be halted. In the auto-repeat mode, when the timer reaches 0, it will simply be reset to counting down the initial value and repeating the count-down to 0. This loop keeps repeating until the disabling signal is set to 1. Regardless of the timer’s mode, if the countdown initial value (i.e. GPTIMER1_DAT for GPT1 or GPTIMER_DAT2 for GPT2) is written when the timer is running, the new initial value will not take effect until the next time the timer is restarted. In the auto-repeat mode, the new countdown start value is used on the next countdown iteration. Therefore, before enabling the timer, the desired values for GPTIMER_DAT and the GPTIMER_PRESCALER registers must first be set.

Note that GPTimer’s design is inherited form MT6260, GPT3 is removed for achieving lower cost. For other timers, register addresses are the same with those of MT6260.

##### Register Definition

Module name: GPTimer base address: (+A00C0000h)

| Address | Name | Width | Register function |
| --- | --- | --- | --- |
| A00C0000 | GPTIMER1_CON | 32 | GPT1 control register |
| A00C0004 | GPTIMER1_DAT | 32 | GPT1 time-out interval register |
| A00C0008 | GPTIMER2_CON | 32 | GPT2 control register |
| A00C000C | GPTIMER2_DAT | 32 | GPT2 time-out Interval register |
| A00C0010 | GPTIMER_STA | 32 | GPT status register |
| A00C0014 | GPTIMER1_PRESCALER | 32 | GPT1 prescaler register |
| A00C0018 | GPTIMER2_PRESCALER | 32 | GPT2 prescaler register |
| A00C0028 | GPTIMER4_CON | 32 | GPT4 control register |
| A00C002C | GPTIMER4_DAT | 32 | GPT4 data register |

A00C0000	GPTIMER1_CON	GPT1 Control Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | EN | MODE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | EN | EN | Controls GPT1 to start counting or to stop 0: Disable GPT1 1: Enable GPT1 |
| 14 | MODE | MODE | Controls GPT1 to count repeatedly (in a loop) or just one-shot 0: One-shot mode is selected. 1: Auto-repeat mode is selected. |

A00C0004	GPTIMER1_DAT GPT1 Time-out Interval Register	0000FFFF

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | CNT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:0 | CNT | CNT | Initial counting value GPT1 counts down from GPTIMER1_DAT. When GPT1 counts down to 0, a GPT1 interrupt will be generated. |

A00C0008	GPTIMER2_CON	GPT2 Control Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | EN | MODE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | EN | EN | Controls GPT2 to start counting or to stop 0: Disable GPT2 1: Enable GPT2 |
| 14 | MODE | MODE | Controls GPT2 to count repeatedly (in a loop) or just one-shot 0: One-shot mode is selected. 1: Auto-repeat mode is selected. |

| A00C000C | GPTIMER2_DAT | GPT2 Time-out Interval Register | 0000FFFF |
| --- | --- | --- | --- |

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | CNT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:0 | CNT | CNT | Initial counting value GPT2 counts down from GPTIMER2_DAT. When GPT2 counts down to 0, a GPT2 interrupt will be generated. |

A00C0010	GPTIMER_STA	GPT Status Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  | GPT2 | GPT1 |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RC | RC |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 1 | GPT2 | GPT2 | GP timer time-out status Each flag is set when the corresponding timer count-down is completed. Can be cleared when the CPU reads the status register. |
| 0 | GPT1 | GPT1 | GP timer time-out status Each flag is set when the corresponding timer count-down is completed. Can be cleared when the CPU reads the status register. |

A00C0014	GPTIMER1_PRE

SCALER

GPT1 Prescaler Register	00000004

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  | PRESCALER |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 2:0 | PRESCALE R | PRESCALER | Controls the counting clock for GP timer 1 0: 16,384Hz 1: 8,192Hz 2: 4,096Hz 3: 2,048Hz 4: 1,024Hz 5: 512Hz |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 6: 256Hz 7: 128Hz |

A00C0018	GPTIMER2_PRE

SCALER

GPT2 Prescaler Register	00000004

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  | PRESCALER |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 2:0 | PRESCALE R | PRESCALER | Controls the counting clock for GP timer 2 0: 16,384Hz 1: 8,192Hz 2: 4,096Hz 3: 2,048Hz 4: 1,024Hz 5: 512Hz 6: 256Hz 7: 128Hz |

A00C0028	GPTIMER4_CON	GPT4 Control Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  | LOCK | EN |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 1 | LOCK | LOCK | Controls GPT4 EN bit can be modified or not If LOCK = 0, EN can be modified. If LOCK = 1, the EN value will be fixed, and the LOCK bit will always be 1 and cannot be modified until hardware reset. 0: Unlock 1: Lock |
| 0 | EN | EN | Controls GPT4 to start counting or to stop 0: Disable GPT4 1: Enable GPT4 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A00C002C	GPTIMER4_DAT	GPT4 Data Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | CNT[31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | CNT[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31:0 | CNT | CNT | If EN = 1, GPT4 will be a free running timer. This register records the GPT4 value. If EN = 0, this register will be cleared to 0. This register does not allow continuous read. It requires at least 1 26M clock cycle between 2 APB reads. |

##### Application Note

When the GPT is in running status, GPTIMER_DAT cannot be configured. To start GPT1 or GPT2, SW should make sure the timer has finished counting for at least 3 cycles of the 32kHz clock.

## MCU OSTIMER

##### Overview

Processor

Clock Control

Wakeup Sources

IRQ

Controller

OS Timer

Sleep Manager

APB

Figure 31. OS timer system view

The OS timer is a hardware timer which specifies the OS time frame duration and generates time-out interrupts by programming the frame counter number. The OS timer has the pause mode. The user can specify pause duration period before the pause mode, and the timer will resume from the pause mode by the external wakeup sources or when the pause duration is timed out.

##### Terminology

Table 46. Abbreviations

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Abbreviation |  |
| --- | --- |
| R/W | Read/Write |
| RO | Read only |
| WO | Write only |
| W1C | Write 1 to clear |
| R/W1C | Read/Write 1 to clear |
| FRC | Free running counter in the system |
| OST | OS timer |

##### Introduction to Wakeup Source

The OS timer only accepts level trigger wakeup source. The wakeup sources are all treated as asynchronous input (will be changed) and will be synchronized by OST clock in OST wakeup source controller.

The possible wakeup sources are listed in the table below.

Table 47. Wakeup sources

| No | Wakeup source |
| --- | --- |
| 0 | GPT |
| 1 | EINT |
| 2 | Timer trigger |
| 3 | KP |
| 4 | MSDC |
| 5 | ANALOG |
| 6 | DSP |
| 7 | MSDC2 |
| 8 | SPISLV |
| 9 | Reserved |
| 10 | Reserved |
| 11 | DSP_ASYNC |
| 12 | Reserved |
| 13 | Reserved |
| 14 | Reserved |
| 15 | Reserved |
| 16 | Reserved |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Table 48. Characteristics of wakeup sources

| No | Wakeup source | Edge/Level | SW clear | Clock domain |
| --- | --- | --- | --- | --- |
| 0 | GPT | Edge |  | F32k_CK |
| 1 | EINT | Edge/Level |  | F32k_CK |
| 2 | Timer trigger | Level |  | F32k_CK |
| 3 | KP | Edge |  |  |
| 4 | MSDC | Level |  |  |
| 5 | ANALOG | Edge/Level |  |  |
| 6 | DSP | Level |  | DSP_CK |
| 7 | MSDC2 | Level |  |  |
| 8 | SPISLV | Level |  |  |
| 9 | Reserved |  |  |  |
| 10 | Reserved |  |  |  |
| 11 | DSP_ASYNC | Level |  | DSP_CK |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |
| 16 | Reserved |  |  |  |

OST WAKEUP CONTROL

Wakeup Source

Wakeup_event_b

irq_b

IRQ Controller

Processor

Wakeup_irq_b

Figure 32. Wakeup event and Irq_b integration diagram

wakeup_irq_b may be asserted before wakeup_event_b is asserted from the wakeup source peripheral to ensure the software will enter wakeup ISR after the pause command is set.

Recommended software programming sequence:

- I-BIT is set when a pause command is executed.

- Set up IRQ mask registers to select IRQ wakeup sources

- Pause criteria are met.

- Set up pause command.

- Confirm the pause command is executed at OST, and check if the pause command is pending or not.

- Set up processor in request for interrupt state.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

The processor clock will be off if there is no interrupt.

- Check if the pause request command is completed (after the processor clock is active or resumes).

- Clear I-BIT.

##### Register Definition

| Address | Name | Width | Register function |
| --- | --- | --- | --- |
| 0XA01F0000 | OST_CON | 32 | OS timer control register Only updated when OST_CMD.OST_WR is set and OST_STA.WR_RDY is high. |
| 0XA01F0004 | OST_CMD | 32 | OS timer command Only valid when the write data BIT31 to BIT16 is 0x1153. |
| 0XA01F0008 | OST_STA | 32 | OS timer status |
| 0XA01F000C | OST_FRM | 32 | OS timer frame duration Specifies OS timer frame duration by micron second resolution. The maximum duration is 8ms and minimum duration is 1ms. This register should be set before OS timer is enabled. The hardware will set up OST_ISR[0] periodically at frame duration period when the OS timer is enabled. |
| 0XA01F0010 | OST_FRM_F32K | 32 | OS timer frame duration by 32K clock Specifies OS timer frame duration with 30.5176 micron second (32kHz) resolution. The maximum duration is 8ms and minimum duration is 1ms. This register should be set before the OS timer is enabled. OST_FRAM_F32K*30.5176us should be less than OST_FRM_NUM*OST_FRM*1us - 30.5176us. Set up OST_FRM_NUM if the frame duration is shorter than system settling time. |
| 0XA01F0014 | OST_UFN | 32 | OS timer un-alignment frame number Specifies OS timer un-alignment event frame number count. This register value is updated to OS timer only when OST_CMD.OST_WR is set. The read value of this register may not be the current OS time Un-alignment frame number. The hardware will count down the OST_UFN counter at frame time-out and stop the count-down when the current value is 0. When the OS timer is in the pause mode, this counter is still active and wakes up the OS timer when OST_UFN becomes 0 at frame time-out boundary. The software should enable the OS timer pause mode when OST_UFN is larger than 1, or the pause command will be ignored by the hardware. |
| 0XA01F0018 | OST_AFN | 32 | OS timer alignment frame number Specifies OS timer alignment event frame number count. This register value is updated to the OS timer only when OST_CMD.OST_WR is set. The read value of this register may not be the current OS time alignment frame number. The hardware will count down the OST_AFN counter at frame time-out and stop the count-down when the current value is 0. The hardware will set OST_ISR[1] when OST_AFN is 1 or 0 at frame time-out, and the OS timer is in normal mode. OST_AFN is current hardware AFN down counter value, and the software cannot read this register directly. The software can only read the current frame number through the OST_AFN_R register, and there is synchronization latency |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Address | Name | Width | Register function |
| --- | --- | --- | --- |
|  |  |  | from OST_AFN to OST_AFN_R. |
| 0XA01F001C | OST_AFN_DLY | 32 | OS timer alignment frame delay number count Specifies the OS timer alignment event frame delay count due to OS timer pause mode. The software has to use the OST_CMD.OST_RD command to update this register value, or the value may be out of date. |
| 0XA01F0020 | OST_UFN_R | 32 | Current OS timer un-alignment frame number Specifies the OS timer current un-alignment frame number. The software has to use the OST_CMD.OST_RD command to update this register value, or the value may be out of date. |
| 0XA01F0024 | OST_AFN_R | 32 | Current OS timer alignment frame number Specifies the OS timer current alignment frame number. The software has to use the OST_CMD.OST_RD command to update this register value, or the value may be out of date. |
| 0XA01F0030 | OST_INT_MASK | 32 | OS timer interrupt mask Specifies the OS timer interrupt mask control. |
| 0XA01F0040 | OST_ISR | 32 | OS timer interrupt status Specifies the OS timer interrupt status. The software has to write 1 at the corresponding bit to clear the interrupt status bit. OSR_ISR[2-0] are also cleared when the OSR_WR command is executed and AFN, UFN are updated. |
| 0XA01F0050 | OST_EVENT_MAS K | 32 | OS timer event mask Specifies which wakeup event will be masked to wake up the OS timer in the OS timer pause mode. |
| 0XA01F0054 | OST_WAKEUP_ST A | 32 | OS timer event wakeup status |
| 0XA01F0060 | OST_DBG_WAKE UP | 32 | OS timer debug wakeup |

0XA01F0000	OS Timer Control Register	OST_CON

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  | OST_ DBG | UFN_ DOWN | EN |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 1 | 0 |

Overview: This registers is only updated when OST_CMD.OST_WR is set and OST_STA.WR_RDY is high.

| Bit(s) | Name | Description |
| --- | --- | --- |
| 2 | OST_DBG | Enables OST wakeup debugging function 0: Disable 1: Enable |
| 1 | UFN_DOWN | Enables OST_UFN count-down 0: Disable OST_UFN count-down 1: Enable OST_UFN count-down |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Name | Description |
| --- | --- | --- |
| 0 | EN | Enables OS timer 0: OS timer is disabled. Then all internal timers are stopped. 1: OS Timer is enabled. The software has to ensure OST_AFN, OST_UFN, OST_FRAM are configured before enabling the OS timer. |

0XA01F0004	OS Timer Command	OST_CMD

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | OST_KEY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | WO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | OST_ CON_ WR | OST_ AFN_ WR | OST_ UFN_ WR |  |  |  |  |  |  |  |  |  |  | OST_ WR | OST_ RD | PAUS E_ST R |
| Type | RW | RW | RW |  |  |  |  |  |  |  |  |  |  | WO | WO | WO |
| Reset | 0 | 0 | 0 |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 |

Overview: The command is only valid when the write data BIT31 to BIT16 is 0x1153.

| Bit(s) | Name | Description |
| --- | --- | --- |
| 31:16 | OST_KEY |  |
| 15 | OST_CON_WR | Updates OST_CON when the OST_WR command is active. |
| 14 | OST_AFN_WR | Updates OST_AFN when the OST_WR command is active. |
| 13 | OST_UFN_WR | Updates OST_UFN when the OST_WR command is active. |
| 2 | OST_WR | Write 1 to this bit to update the bus clock domain OS timer configuration into the OST SYSCLK domain |
| 1 | OST_RD | Write 1 to this bit to update the current OS timer status to the bus clock domain |
| 0 | PAUSE_STR | Write 1 to this bit to enable the OS timer pause function. The command will be ignored if the current OST UFN is less than 2. The software has to ensure OST_CMD.OST_WR is completed before the next software pause sequence. |

0XA01F0008	OS Timer Status	OST_STA

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | CPU_ SLEE P |  |  |  |  |  |  |  |  | AFN_ DLY_ OVER |  | PAUSE_REQ |  |  | CMD_ CPL | READ Y |
| Type | RO |  |  |  |  |  |  |  |  | RO |  | RO |  |  | RO | RO |
| Reset | 0 |  |  |  |  |  |  |  |  | 0 |  | 0 | 0 |  | 1 | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Figure 33. Pause command complete and pause request state

| Bit(s) | Name | Description |
| --- | --- | --- |
| 15 | CPU_SLEEP | The processor is in the sleep mode. (for debugging) 0: Active 1: Sleep |
| 6 | AFN_DLY_OVER | AFN_DLY counter overflows. This bit is cleared when AFN is updated. 0: Does not overflow 1: Overflow |
| 4:3 | PAUSE_REQ | An OS timer pause request is pending. A pause command will be completed when the pause command is set. - Processor is in the sleep mode (processor clock is off), UFN ≥ 2 and no wakeup sources - Any wakeup sources are sensed after the pause command is set. - UFN < 2 00: The last pause command request is not completed yet (CP15 is not enable and no wakeup source). 01: The last pause command request is completed with OST pause mode |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Name | Description |
| --- | --- | --- |
|  |  | being active. 10: The last pause command request is completed with wakeup sources. 11: The last pause command request is completed with UFN < 2. |
| 1 | CMD_CPL | OST command is completed. It takes several clocks from OST_CMD being updated to command being active. 0: OST command is not completed. 1: OST command is completed. |
| 0 | READY | OS timer status 0: OST is in pause mode. 1: OST is in normal mode. |

0XA01F000C	OS Timer Frame Duration	OST_FRM

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  | OST_FRM |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  | RW |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |

Overview: This register specifies the OS timer frame duration by micron second resolution. The maximum duration is 8ms and minimum duration is 1ms. This register should be set before the OS timer is enabled. The hardware will set up OST_ISR[0] periodically at frame duration period when the OS timer is enabled.

| Bit(s) | Name | Description |
| --- | --- | --- |
| 12:0 | OST_FRM |  |

0XA01F0010	OS Timer Frame Duration by 32K Clock	OST_FRM_F32K

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | OST_UFN |  |  |  |  |  |  |  | OST_FRM_F32K |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: This register specifies the OS timer frame duration with 30.5176 micron second (32kHz) resolution. The maximum duration is 8ms and minimum duration is 1ms. This register should be set up before the OS timer is enabled. OST_FRAM_F32K*30.5176us should be less than OST_FRM_NUM*OST_FRM*1us - 30.5176us. Set up OST_FRM_NUM if the frame duration is shorter than the system settling time.

| Bit(s) | Name | Description |
| --- | --- | --- |
| 15:12 | OST_UFN |  |
| 11:0 | OST_FRM_F32K |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

0XA01F0014	OS Timer Un-alignment Frame Number	OST_UFN

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | OST_UFN[31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | OST_UFN[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: This register specifies the OS timer un-alignment event frame number count. This register value is updated to the OS timer only when OST_CMD.OST_WR is set. The read value of this register may not be the current OS time un-alignment frame number. The hardware will count down the OST_UFN counter at frame time-out and stop the count-down when the current value is 0. When the OS timer is in the pause mode, this counter is still active and wakes up the OS timer when OST_UFN becomes 0 at frame time-out boundary. The

software should enable the OS timer pause mode when OST_UFN is larger than 1, or the pause command will be ignored by hardware.

| Bit(s) | Name | Description |
| --- | --- | --- |
| 31:0 | OST_UFN |  |

0XA01F0018	OS Timer Alignment Frame Number	OST_AFN

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | OST_AFN[31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | OST_AFN[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: This register specifies the OS timer alignment event frame number count. This register value is updated to the OS timer only when OST_CMD.OST_WR is set. The read value of this register may not be the current OS time alignment frame number. The hardware will count down the OST_AFN counter at frame time-out and stop the count-down when the current value is 0. The hardware will set up OST_ISR[1] when OST_AFN is 1 or 0 at frame time-out and the OS timer is in the normal mode. OST_AFN is the current hardware AFN down counter value, and the software cannot read this register directly. The software can only read the current frame number through the OST_AFN_R register, and there is synchronization latency from OST_AFN to OST_AFN_R.

| Bit(s) | Name | Description |
| --- | --- | --- |
| 31:0 | OST_AFN |  |

0XA01F001C	OS Timer Alignment Frame Delay Number Count	OST_AFN_DLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Name | OST_AFN_DLY[31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | OST_AFN_DLY[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: This register specifies the OS Timer Alignment event frame delay count due to OS timer pause mode. Software has to use OST_CMD.OST_RD command to update this register value, or the value maybe out of date.

| Bit(s) | Name | Description |
| --- | --- | --- |
| 31:0 | OST_AFN_DLY |  |

0XA01F0020	Current OS Timer Un-alignment Frame Number	OST_UFN_R

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | OST_UFN_R[31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | OST_UFN_R[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: This register specifies the OS timer current un-alignment frame number. The software has to use the OST_CMD.OST_RD command to update this register value, or the value may be out of date.

| Bit(s) | Name | Description |
| --- | --- | --- |
| 31:0 | OST_UFN_R |  |

0XA01F0024	Current OS Timer Alignment Frame Number	OST_AFN_R

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | OST_AFN_R[31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | OST_AFN_R[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: This register specifies the OS timer current alignment frame number. The software has to use the OST_CMD.OST_RD command to update this register value, or the value may be out of date.

| Bit(s) | Name | Description |
| --- | --- | --- |
| 31:0 | OST_AFN_R |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

0XA01F0030	OS Timer Interrupt Mask	OST_INT_MASK

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  |  |  |  | OST_INT_MASK |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 | 1 | 1 | 1 |

Overview: This register specifies the OS timer interrupt mask control.

| Bit(s) | Name | Description |
| --- | --- | --- |
| 4:0 | OST_INT_MASK | 0: Mask OS timer frame time-out interrupt 1: Mask OS timer alignment frame time-out interrupt 2: Mask OS timer un-alignment frame time-out interrupt 3: Mask OS timer pause abort interrupt 4: Mask OS timer pause interrupt |

0XA01F0040	OS Timer Interrupt Status	OST_ISR

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  |  |  |  | OST_ISR |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  | W1C |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 |

Overview: This register specifies the OS timer interrupt status. The software has to write 1 to the corresponding bit to clear the interrupt status bit. OSR_ISR[2-0] are also cleared when the OSR_WR command is executed and AFN, UFN are updated.

| Bit(s) | Name | Description |
| --- | --- | --- |
| 4:0 | OST_ISR | 0: OS timer frame time-out interrupt status 1: OS timer alignment frame time-out interrupt status 2: OS timer un-alignment frame time-out interrupt status 3: OS timer pause abort interrupt status 4: OS timer pause interrupt status |

0XA01F0050	OS Timer Event Mask	OST_EVENT_MASK

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  | OST_EVENT_MASK [18:16] |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 1 | 1 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Name | OST_EVENT_MASK[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Overview: This register specifies which wakeup event will be masked to wake up the OS timer in the OS timer pause mode.

| Bit(s) | Name | Description |
| --- | --- | --- |
| 18:0 | OST_EVENT_MASK |  |

0XA01F0054	OS Timer Event Wakeup Status	OST_WAKEUP_STA

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  | OST_WAKEUP_STA [18:16] |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | RO |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | OST_WAKEUP_STA[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 18:0 | OST_WAKEUP_STA |  |

0XA01F0060	OS Timer Debug Wakeup	OST_DBG_WAKEOSUP

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | CIRQ_ MASK _EN |  |  |  |  |  |  |  |  |  |  |  |  | OST_DBG_WAKEU P[18:16] |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | OST_DBG_WAKEUP[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 31 | CIRQ_MASK_EN | 0: Disable cirq mask function 1: Enable cirq mask function |
| 18:0 | OST_DBG_WAKEUP | Controls wakeup status in debug timing event1 |

To OST Wakeup Contol

MUX

OST_DBG

1

DBG_WAKE[0]  DBG_WAKE[1]	DBG_WAKE[n]

OST_DBG_WAKEUP

#### DeMUX

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

GPT_EVENT_B	Wakeup Sources

Figure 34. Debug wakeup events

## UART1

##### General Description

The baseband chipset houses two UARTs. The UARTs provide full duplex serial communication channels between baseband chipset and external dev ices.

The UART has M16C450 and M16550A modes of operation, which are compatible with a range of standard software drivers.	The extensions have been designed to be broadly software compatible with 16550A variants, but certain areas offer no consensus.

In common with the M16550A, the UART supports word lengths from 5 to 8 bits, an optional parity bit and one or two stop bits, and is fully programmable by an 8-bit CPU interface. A 16-bit programmable baud rate generator and an 8-bit scratch register are included, together with separate transmit and receive FIFOs. 8 modem control lines and a diagnostic loop-back mode are provided.

UART also includes two DMA handshake lines, indicating when the FIFOs are ready to transfer data to the CPU. Interrupts can be generated from any of the 10 sources.

Note that UART is designed so that all internal operation is synchronized by the CLK signal. This synchronization results in minor timing differences between the UART and industry standard 16550A device, which means that the core is not clock for clock identical to the original device.

After a hardware reset, UART will be in M16C450 mode. Its FIFOs can then be enabled and	UART can enter M16550A mode. UART has further functions beyond the M16550A mode. Each of the

extended functions can be selected individually under software control.

UART provides more powerful enhancements than the industry-standard 16550:

- Hardware flow control. This feature is very useful when the ISR latency is hard to predict and control in the embedded applications. The MCU is relieved of having to fetch the received data within a fixed amount of time.

Note that in order to enable any of the enhancements, the Enhanced Mode bit, EFR[4], must be set. If EFR[4] is not set, IER[7:5], FCR[5:4], IIR[5:4] and MCR[7:6] cannot be written. The enhanced mode bit ensures that UART is backward compatible with the software that has been written for 16C450 and 16550A devices. Figure 35 is the block diagram of the UART1 device.

Clock

APB Bus

Uart_tx_data

Baud

Divisor

APB BUS

I/F

RX FIFO

RX Machine

Modem Control

TX Machine

Baud Rate Generator

TX FIFO

Uart_rx_data

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Modem Outputs Modem Inputs

Figure 35. Block Diagram of UART1

##### Register Definition

Module name: UART1 Base address: (+A0080000h)

| Address | Name | Width | Register Function |
| --- | --- | --- | --- |
| A0080000 | RBR | 8 | RX Buffer Register Note: Only when LCR[7] = 0. |
| A0080000 | THR | 8 | TX Holding Register Note: Only when LCR[7] = 0 |
| A0080000 | DLL | 8 | Divisor Latch (LS) Divides the bclk frequency Note: Nodified when LCR[7]!=0 |
| A0080004 | IER | 8 | Interrupt Enable Register Note: Only when LCR[7] = 0. By storing 1 to a specific bit position, the interrupt associated with that bit is enabled. Otherwise, the interrupt will be disabled. IER[3:0] are modified when LCR[7] = 0. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Address | Name | Width | Register Function |
| --- | --- | --- | --- |
|  |  |  | IER[7:4] are modified when LCR[7] = 0 & EFR[4] = 1. |
| A0080004 | DLM | 8 | Divisor Latch (MS) uesd to divid the bclk frequency .*NOTE: modified when LCR[7]!=0 |
| A0080008 | IIR | 8 | Interrupt Identification Register Note: Only when LCR!=BF'h. Priority is from high to low as the following. IIR[5:0]=0X1: No interrupt pending IIR[5:0]=0X6: Line status interrupt (under IER[2]=1) IIR[5:0]=0Xc: RX data time-out interrupt (under IER[0]=1) IIR[5:0]=0X4: RX data are placed in the RX bBuffer register or the RX trigger level is reached. (under IER[0]=1). IIR[5:0]=0X2: TX holding register is empty or the contents of the TX FIFO have been reduced to its trigger level (under IER[1]=1). IIR[5:0]=0X10: XOFF character received (under IER[5]=1,EFR[4] = 1). |
| A0080008 | FCR | 8 | FIFO Control Register FCR is used to control the trigger levels of the FIFOs or flush the FIFOs. FCR[7:6] is modified when LCR != BFh FCR[5:4] is modified when LCR != BFh & EFR[4] = 1 FCR[4:0] is modified when LCR != BFh |
| A0080008 | EFR | 8 | Enhanced Feature Register Note: Only when LCR=BF'h |
| A008000C | LCR | 8 | Line Control Register Determines characteristics of serial communication signals. |
| A0080010 | MCR | 8 | Modem Control Register Controls interface signals of the UART. MCR[5:0] are modified when LCR != 8'hBF, MCR[7] can be read when LCR != 8'hBF & EFR[4] = 1. |
| A0080010 | XON1 | 8 | XON1 Char Register Note: XON1 is modified only when LCR=BF'h. |
| A0080014 | LSR | 8 | Line Status Register Modified when LCR != BFh. |
| A0080018 | XOFF1 | 8 | XOFF1 Char Register *Note: XOFF1 is modified only when LCR=BF'h. |
| A008001C | SCR | 8 | Scratch Register A general purpose read/write register. After reset, its value will be un-defined. Modified when LCR != BFh. |
| A0080020 | AUTOBAUD_EN | 8 | Auto Baud Detect Enable Register |
| A0080024 | HIGHSPEED | 8 | High Speed Mode Register |
| A0080028 | SAMPLE_COUNT | 8 | Sample Counter Register When HIGHSPEED=3, sample_count will be the threshold value for UART sample counter (sample_num). Count from 0 to sample_count. |
| A008002C | SAMPLE_POINT | 8 | Sample Point Register When HIGHSPEED=3, UART gets the input data when sample_count=sample_num, e.g. system clock = 13MHz, 921600 = 13000000/14. Therefore, sample_count = 13, and |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Address | Name | Width | Register Function |
| --- | --- | --- | --- |
|  |  |  | sample point = 6 (sampling the central point to decrease the inaccuracy) SAMPLE_POINT is usually (SAMPLE_COUNT-1)/2 without decimal. |
| A0080030 | AUTOBAUD_REG | 8 | Auto Baud Monitor Register the autobaud detection state ,it will not change until enable the autobaud_en again. |
| A0080034 | RATEFIX_AD | 8 | Clock Rate Fix Register |
| A0080038 | AUTOBAUDSAMPL E | 8 | Auto Baud Sample Register Since the system clock may change, autobaud sample duration should change as the system clock changes. When system clock = 13MHz, autobaudsample = 6; when system clock = 26MHz, autobaudsample = 13. When system clock = 52MHz, autobaudsample = 27. |
| A008003C | GUARD | 8 | Guard Time Added Register |
| A0080040 | ESCAPE_DAT | 8 | Escape Character Register |
| A0080044 | ESCAPE_EN | 8 | Escape Enable Register |
| A0080048 | SLEEP_EN | 8 | Sleep Enable Register |
| A008004C | DMA_EN | 8 | DMA Enable Register |
| A0080050 | RXTRI_AD | 8 | Rx Trigger Address |
| A0080054 | FRACDIV_L | 8 | Fractional Divider LSB Address |
| A0080058 | FRACDIV_M | 8 | Fractional Divider MSB Address |
| A008005C | FCR_RD | 8 | FIFO Control Register |

A0080000	RBR	RX Buffer Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | RBR |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RU |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | RBR | Read-only register The received data can be read by accessing this register. Only when LCR[7] = 0. |

A0080000	THR	TX Holding Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | THR |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | WO |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | THR | TX Holding Register Write-only register. The data to be transmitted are written to this register and sent to the PC via serial communication. Only when LCR[7] = 0. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A0080000	DLL	Divisor Latch (LS)	01

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | DLL |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | DLL | Divisor latch low 8-bit data Note: Modified when LCR[7]!=0. |

A0080004	IER	Interrupt Enable Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | CTSI | RTSI | XOFFI |  | EDSSI | ELSI | ETBEI | ERBFI |
| Type |  |  |  |  |  |  |  |  | RW | RW | RW |  | RW | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7 | CTSI | Masks an interrupt that is generated when a rising edge is detected on the CTS modem control line. Note: This interrupt is only enabled when hardware flow control is enabled. 0: Mask an interrupt that is generated when a rising edge is detected on the CTS modem control line. 1: Unmask an interrupt that is generated when a rising edge is detected on the CTS modem control line. |
| 6 | RTSI | Masks an interrupt that is generated when a rising edge is detected on the RTS modem control line. Note: This interrupt is only enabled when hardware flow control is enabled. 0: Mask an interrupt that is generated when a rising edge is detected on the RTS modem control line. 1: Unmask an interrupt that is generated when a rising edge is detected on the RTS modem control line. |
| 5 | XOFFI | Masks an interrupt that is generated when an XOFF character is received. Note: This interrupt is only enabled when software flow control is enabled. 0: Mask an interrupt that is generated when an XOFF character is received. 1: Unmask an interrupt that is generated when an XOFF character is received. |
| 3 | EDSSI | When set to 1, an interrupt will be generated if DCTS (MSR[0]) becomes set. 0: No interrupt is generated if DCTS (MSR[0]) becomes set. 1: An interrupt is generated if DCTS (MSR[0]) becomes set. |
| 2 | ELSI | When set to1, an interrupt will be generated if BI, FE, PE or OE (LSR[4:1]) becomes set. 0: No interrupt is generated if BI, FE, PE or OE (LSR[4:1]) becomes set. 1: An interrupt is generated if BI, FE, PE or OE (LSR[4:1]) becomes set. |
| 1 | ETBEI | When set to 1, an interrupt will be generated if the TX holding register is empty or the contents of the TX FIFO have been reduced to its trigger level. 0: No interrupt will be generated if the TX holding register is empty or the contents of the TX FIFO have been reduced to its trigger level. 1: An interrupt will be generated if the TX folding register is empty or the contents of the TX FIFO have been reduced to its trigger level. |
| 0 | ERBFI | When set to 1, an interrupt will be generated if RX data are placed in RX buffer register or the RX trigger level is reached. 0: No interrupt will be generated if RX data are placed in the RX buffer register or the RX trigger level is reached. 1: An interrupt will be generated if RX Data are placed in the RX buffer register or the RX |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Name | Description |
| --- | --- | --- |
|  |  | trigger level is reached. |

A0080004	DLM	Divisor Latch (MS)	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | DLM |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | DLM | Divisor latch high 8-bit data Note: Modified when LCR[7]!=0. DLL & DLM can only be updated when DLAB(LCR[7]) is set to 1. Division by 1 will generate a BAUD signal that is constantly high. DLL & DLM setting formula is {DLH,DLL}=(system clock frequency/baud_pulse/baud_rate). When RATE_FIX(RATEFIX_AD[0])=0, system clock frequency = 52MHz. When RATE_FIX(RATEFIX_AD[0])=1 and RATE_FIX(RATEFIX_AD[2])=0, system clock frequency = 26MHz. When RATE_FIX(RATEFIX_AD[0])=1 and RATE_FIX(RATEFIX_AD[2])=1, system clock frequency = 13MHz. For baud_pulse value, refer to HIGH_SPEED(offset=24H) register For example, when at 52MHz, default speed mode and 115200 baud rate, {DLH,DLL}=52MHz/16/115200=28. |

A0080008	IIR	Interrupt Identification Register	01

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FIFOE |  | ID |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RO |  | RU |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:6 | FIFOE |  |
| 5:0 | ID | IIR[5:0] Priority level interrupt source 000001 - No interrupt pending 000110 1 Line status interrupt: BI, FE, PE or OE set in LSR. (Under IER[2]=1) 001100 2 RX data time-out: Time-out on character in RX FIFO. (Under IER[0]=1) 000100 3 RX data received: RX data received or RX trigger level reached. (Under IER[0]=1) 000010 4 TX holding register empty: 000000 5 Modem status change: DCTS set in MSR. (Under IER[3]=1) TX Holding Register empty or TX FIFO trigger level reached. (Under IER[1]=1) 010000 6 Software flow control: XOFF Character received. (Under IER[5]=1) 100000 7 Hardware flow control: CTS or RTS Rising Edge. (Under IER[7]=1 or IER[6]=1) Line status interrupt: A RX line status interrupt (IIR[5:0] = 000110b) will be generated if ELSI (IER[2]) is set and any of BI, FE, PE or OE (LSR[4:1]) becomes set. The interrupt is cleared by reading the line status register. RX data time-out interrupt: When the virtual FIFO mode is disabled, RX data time-out interrupt will be generated if all of the following conditions are applied: - FIFO contains at least one character. - The most recent character is received longer than four character periods ago |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Name | Description |
| --- | --- | --- |
|  |  | (including all start, parity and stop bits); - The most recent CPU read of the FIFO is longer than four character periods ago. The timeout timer is restarted upon receipt of a new byte from the RX shift register or upon a CPU read from the RX FIFO. The RX data time-out interrupt is enabled by setting EFRBI (IER[0]) to 1 and is cleared by reading RX FIFO. When the virtual FIFO mode is enabled, RX data time-out interrupt will be generated if all of the following conditions are applied: - FIFO is empty. - The most recent character is received longer than four character periods ago (including all start, parity and stop bits). - The most recent CPU read of the FIFO is longer than four character periods ago. The timeout timer is restarted upon receipt of a new byte from the RX shift register or reading DMA_EN register. The RX Data Timeout Interrupt is enabled by setting EFRBI (IER[0]) to 1 and is cleared by reading DMA_EN register. RX data received interrupt: A RX received interrupt (IER[5:0] = 000100b) is generated if EFRBI (IER[0]) is set and either RX data are placed in the RX buffer register or the RX trigger level is reached. The interrupt is cleared by reading the RX buffer register or the RX FIFO (if enabled). TX holding register empty interrupt: A TX holding register empty interrupt (IIR[5:0] = 000010b) is generated if ETRBI (IER[1]) is set and either the TX holding register is empty or the contents of the TX FIFO are reduced to its trigger level. The interrupt is cleared by writing to the TX holding register or TX FIFO if FIFO is enabled. Modem status change interrupt: A modem status change Interrupt (IIR[5:0] = 000000b) will be generated if EDSSI (IER[3]) is set and e DCTS (MSR[3:0]) becomes set. The interrupt is cleared by reading the modem status register. Software flow control interrupt: A software flow control interrupt (IIR[5:0] = 010000b) will be generated if the software flow control is enabled and XOFFI (IER[5]) becomes set, indicating that an XOFF character has been received. The interrupt is cleared by reading the interrupt identification register. Hardware flow control interrupt: A hardware flow control interrupt (IER[5:0] = 100000b) will be generated if the hardware flow control is enabled and either RTSI (IER[6]) or CTSI (IER[7]) becomes set indicating that a rising edge has been detected on either the RTS/CTS modem control line. The interrupt is cleared by reading the interrupt identification register. |

A0080008	FCR	FIFO Control Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | RFTL1_RFTL0 |  | TFTL1_TFTL0 |  |  | CLRT | CLRR | FIFOE |
| Type |  |  |  |  |  |  |  |  | WO |  | WO |  |  | WO | WO | WO |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:6 | RFTL1_RFTL0 | RX FIFO trigger threshold RX FIFO contains total 32 bytes. 0: 1 1: 6 2: 12 3: RXTRIG |
| 5:4 | TFTL1_TFTL0 | TX FIFO trigger threshold TX FIFO contains total 32 bytes. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Name | Description |
| --- | --- | --- |
|  |  | 0: 1 1: 4 2: 8 3: 14 |
| 2 | CLRT | Control bit to clear TX FIFO 0: No effect 1: Clear TX FIFO |
| 1 | CLRR | Control bit to clear RX FIFO 0: No effect 1: Clear RX FIFO |
| 0 | FIFOE | Enables FIFO This bit must be set to 1 for any of other bits in the registers to have any effect. 0: Disable both RX and TX FIFOs. 1: Enable both RX and TX FIFOs. |

A0080008	EFR	Enhanced Feature Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | AUTO_ CTS | AUTO_ RTS |  | ENABL E_E | SW_FLOW_CONT |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW | RW |  | RW | RW |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7 | AUTO_CTS | Enables hardware transmission flow control 0: Disable 1: Enable |
| 6 | AUTO_RTS | Enables hardware reception flow control 0: Disable 1: Enable |
| 4 | ENABLE_E | Enables enhancement feature 0: Disable 1: Enable |
| 3:0 | SW_FLOW_CONT | Software flow control bits 00xx: No TX flow control 01xx: No TX flow control 10xx: Transmit XON1/XOFF1 as flow control bytes xx00: No RX flow control xx01: No RX flow control xx10: Receive XON1/XOFF1 as flow control bytes |

A008000C	LCR	Line Control Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | DLAB | SB | SP | EPS | PEN | STB | WLS1_WLS0 |  |
| Type |  |  |  |  |  |  |  |  | RW | RW | RW | RW | RW | RW | RW |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7 | DLAB | Divisor latch access bit 0: RX and TX registers are read/written at Address 0 and the IER register is read/written at Address 4. 1: Divisor Latch LS is read/written at Address 0 and the Divisor Latch MS is read/written at Address 4. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Name | Description |
| --- | --- | --- |
| 6 | SB | Set break 0: No effect 1: SOUT signal is forced to the 0 state. |
| 5 | SP | Stick parity 0: No effect. 1: The parity bit is forced to a defined state, depending on the states of EPS and PEN: If EPS=1 & PEN=1, the parity bit will be set and checked = 0. If EPS=0 & PEN=1, the parity bit will be set and checked = 1. |
| 4 | EPS | Selects even parity 0: When EPS=0, an odd number of ones is sent and checked. 1: When EPS=1, an even number of ones is sent and checked. |
| 3 | PEN | Enables parity 0: The parity is neither transmitted nor checked. 1: The parity is transmitted and checked. |
| 2 | STB | Number of STOP bits 0: One STOP bit is always added. 1: Two STOP bits are added after each character is sent; unless the character length is 5 when 1 STOP bit is added. |
| 1:0 | WLS1_WLS0 | Selects word length 0: 5 bits 1: 6 bits 2: 7 bits 3: 8 bits |

A0080010	MCR	Modem Control Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | XOFF_ STATU S |  |  | Loop |  |  | RTS |  |
| Type |  |  |  |  |  |  |  |  | RU |  |  | RW |  |  | RW |  |
| Reset |  |  |  |  |  |  |  |  | 0 |  |  | 0 |  |  | 0 |  |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7 | XOFF_STATUS | Read-only bit 0: When an XON character is received. 1: When an XOFF character is received. |
| 4 | Loop | Loop-back control bit 0: No loop-back is enabled. 1: Loop-back mode is enabled. |
| 1 | RTS | Controls the state of the output NRTS, even in loop mode. 0: RTS will always output 1. 1: RTS's output will be controlled by flow control condition. |

A0080010	XON1	XON1 Char Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | XON1 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | XON1 | XON1 character for software flow control |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Name | Description |
| --- | --- | --- |
|  |  | Modified only when LCR=BF'h. |

A0080014	LSR	Line Status Register	60

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FIFOE RR | TEMT | THRE | BI | FE | PE | OE | DR |
| Type |  |  |  |  |  |  |  |  | RU | RU | RU | RU | RU | RU | RU | RU |
| Reset |  |  |  |  |  |  |  |  | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7 | FIFOERR | RX FIFO error indicator 0: No PE, FE, BI set in the RX FIFO. 1: Set to 1 when there is at least one PE, FE or BI in the RX FIFO. |
| 6 | TEMT | TX holding register (or TX FIFO) and the TX shift register are empty. 0: Empty conditions below are not met. 1: If FIFOs are enabled, the bit will be set whenever the TX FIFO and the TX shift register are empty. If FIFOs are disabled, the bit will be set whenever TX holding register and TX shift register are empty. |
| 5 | THRE | Indicates if there is room for TX holding register or TX FIFO is reduced to its trigger level 0: Reset whenever the contents of the TX FIFO are more than its trigger level (FIFOs are enabled), or whenever TX holding register is not empty (FIFOs are disabled). 1: Set whenever the contents of the TX FIFO are reduced to its trigger level (FIFOs are enabled), or whenever TX holding register is empty and ready to accept new data (FIFOs are disabled). |
| 4 | BI | Break interrupt 0: Reset by the CPU reading this register 1: If the FIFOs are disabled, this bit will be set whenever the SIN is held in the 0 state for more than one transmission time (START bit + DATA bits + PARITY + STOP bits). If the FIFOs are enabled, this error will be associated with a corresponding character in the FIFO and is flagged when this byte is at the top of the FIFO. When a break occurs, only one zero character is loaded into the FIFO: The next character transfer is enabled when SIN goes into the marking state and receives the next valid start bit. |
| 3 | FE | Framing error 0: Reset by the CPU reading this register 1: If the FIFOs are disabled, this bit will be set if the received data do not have a valid STOP bit. If the FIFOs are enabled, the state of this bit will be revealed when the byte it refers to is the next to be read. |
| 2 | PE | Parity error 0: Reset by the CPU reading this register 1: If the FIFOs are disabled, this bit will be set if the received data do not have a valid parity bit. If the FIFOs are enabled, the state of this bit will be revealed when the referred byte is the next to be read. |
| 1 | OE | Overrun error 0: Reset by the CPU reading this register. 1: If the FIFOs are disabled, this bit will be set if the RX buffer is not read by the CPU before the new data from the RX shift register overwrites the previous contents. If the FIFOs are enabled, an overrun error will occur when the RX FIFO is full and the RX shift register becomes full. OE will be set as soon as this happens. The character in the shift register is then overwritten, but not transferred to the FIFO. |
| 0 | DR | Data ready 0: Cleared by the CPU reading the RX buffer or by reading all the FIFO bytes. 1: Set by the RX buffer becoming full or by the FIFO becoming no empty. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A0080018	XOFF1	XOFF1 Char Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | XOFF1 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | XOFF1 | XOFF1 character for software flow control Modified only when LCR=BF'h. |

A008001C	SCR	Scratch Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | SCR |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | SCR | General purpose read/write register After reset, its value will be undefined. Modified when LCR != BFh. |

A0080020	AUTOBAUD_EN Auto Baud Detect Enable Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  | SLEEP _ACK_ SEL | AUTOB AUD_S EL | AUTOB AUD_E N |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 2 | SLEEP_ACK_SEL | Selects sleep ack when autobaud_en 0: Support sleep_ack when autobaud_en is opened . 1: Does not support sleep_ack when autobaud_en is opened . |
| 1 | AUTOBAUD_SEL | Selects auto-baud 0: Support standard baud rate detection 1: Support non_standard baud rate detection (support baud from 110 to 115200; recommended to use 52MHz to auto fix) . |
| 0 | AUTOBAUD_EN | Auto-baud enabling signal 0: Disable auto-baud function 1: Enable auto-baud function (UARTn+0024h SPEED should be set to 0.) Note: When AUTOBAUD_EN is active, there should not be A*/a* char before the auto baud char AT/at. If A*/a* is Inevitable, autobaud will fail and please disable AUTOBAUD_EN to reset the autobaud feature and autobaud_en again. |

A0080024	HIGHSPEED	High Speed Mode Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SPEED |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Name | Description |
| --- | --- | --- |
| 1:0 | SPEED | UART sample counter base 0: Based on 16*baud_pulse, baud_rate = system clock frequency/16/{DLH, DLL} 1: Based on 8*baud_pulse, baud_rate = system clock frequency/8/{DLH, DLL} 2: Based on 4*baud_pulse, baud_rate = system clock frequency/4/{DLH, DLL} 3: Based on sampe_count * baud_pulse, baud_rate = system clock frequency / (sampe_count+1)/{DLM, DLL} |

A0080028	SAMPLE_COUN Sample Counter Register	00

T

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | SAMPLECOUNT |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | SAMPLECOUNT | Only useful when HIGHSPEED mode = 3. |

A008002C	SAMPLE_POINTSample Point Register	FF

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | SAMPLEPOINT |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | SAMPLEPOINT | SAMPLE_POINT is usually (SAMPLE_COUNT-1)/2 without decimal. Effective only when HIGHSPEED=3. |

A0080030	AUTOBAUD_RE Auto Baud Monitor Register	00

G

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | BAUD_STAT |  |  |  | BAUD_RATE |  |  |  |
| Type |  |  |  |  |  |  |  |  | RU |  |  |  | RU |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:4 | BAUD_STAT | Autobaud state (only true value in standard autobaud detection) 0: Autobaud is detecting. 1: AT_7N1 2: AT_7O1 3: AT_7E1 4: AT_8N1 5: AT_8O1 6: AT_8E1 7: at_7N1 8: at_7E1 9: at_7O1 10: at_8N1 11: at_8E1 12: at_8O1 13: Autobaud detection fails |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 3:0 | BAUD_RATE | Autobaud baud rate (only true value in standard autobaud detection) 0: 115,200 1: 57,600 2: 38,400 3: 19,200 4: 9,600 5: 4,800 6: 2,400 7: 1,200 8: 300 9: 110 |

A0080034	RATEFIX_AD	Clock Rate Fix Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  | FREQ_ SEL | AUTOB AUD_R ATE_FI X | RATE_ FIX |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 2 | FREQ_SEL | 0: Select 26MHz as system clock 1: Select 13MHz as system clock |
| 1 | AUTOBAUD_RATE_FIX | 0: Use 52MHz as system clock for UART auto baud detection 1: Use 26MHz/13MHz (depending on FREQ_SEL) as system clock for UART auto baud detection |
| 0 | RATE_FIX | 0: Use 52MHz as system clock for UART TX/RX 1: Use 26MHz/13MHz (depending on FREQ_SEL) as system clock for UART TX/RX |

A0080038	AUTOBAUDSA

MPLE

Auto Baud Sample Register	0D

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  | AUTOBAUDSAMPLE |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 1 | 1 | 0 | 1 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 5:0 | AUTOBAUDSAMPLE | clk diveision for autobaud rate detection For standard baud rate detection. System clk 52m: 'd 27 System clk 26m: 'd 13 System clk 13m: 'd 6 For non-standard baud rate detection. :15. |

A008003C	GUARD	Guard Time Added Register	0F

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  | GUAR D_EN | GUARD_CNT |  |  |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Type |  |  |  |  |  |  |  |  |  |  |  | RW | RW |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reset |  |  |  |  |  |  |  |  |  |  |  | 0 | 1 | 1 | 1 | 1 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 4 | GUARD_EN | Guard interval add enabling signal 0: No guard interval added 1: Add guard interval after stop bit. |
| 3:0 | GUARD_CNT | Guard interval count value Guard interval = [1/(system clock/div_step/div)]*GUARD_CNT. |

A0080040	ESCAPE_DAT	Escape Character Register	FF

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | ESCAPE_DAT |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | ESCAPE_DAT | Escape character added before software flow control data and escape character If TX data are xon (31h), with esc_en =1, UART will transmit data as esc + CEh (~xon). |

A0080044	ESCAPE_EN	Escape Enable Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ESC_E N |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 0 | ESC_EN | Adds escape character in transmitter and removes escape character in receiver by UART 0: Does not deal with the escape character 1: Add escape character in transmitter and remove escape character in receiver |

A0080048	SLEEP_EN	Sleep Enable Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SLEEP _EN |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 0 | SLEEP_EN | For sleep mode issue 0: Does not deal with sleep mode indicate signal 1: Activate hardware flow control or software control according to software initial setting when the chip enters sleep mode. Release hardware flow when the chip wakes up. However, for software control, UART sends xon when awaken and when FIFO does not reach threshold level. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A008004C	DMA_EN	DMA Enable Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | FIFO_l sr_sel | TO_CN T_AUT ORST | TX_DM A_EN | RX_DM A_EN |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 3 | FIFO_lsr_sel | Selects FIFO LSR mode 0: LSR will hold the first line status error state until you read the LSR register. 1: LSR will update automatically. |
| 2 | TO_CNT_AUTORST | Time-out counter auto reset register 0: After RX time-out happens, SW shall reset the interrupt by reading UART 0x4C. 1: The time-out counter will be auto reset. Set this register when Rain's new DMA is used. |
| 1 | TX_DMA_EN | TX_DMA mechanism enabling signal 0: Does not use DMA in TX 1: Use DMA in TX. When this register is enabled, the flow control will be based on the DMA threshold and generate a time-out interrupt for DMA.for DMA. |
| 0 | RX_DMA_EN | RX_DMA mechanism enabling signal 0: Does not use DMA in RX 1: Use DMA in RX. When this register is enabled, the flow control will be based on the DMA threshold and generate a time-out interrupt |

A0080050	RXTRI_AD	Rx Trigger Address	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | RXTRIG |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 3:0 | RXTRIG | When {rtm,rtl}=2'b11, the RX FIFO threshold will be Rxtrig. The value is suggested to be smaller than half of RX FIFO size, which is 32 bytes. |

A0080054	FRACDIV_L	Fractional Divider LSB Address	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FRACDIV_L |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | FRACDIV_L | Adds sampling count (+1) from state data7 to data0 to contribute fractional divisor.only when high_speed=3. |

A0080058	FRACDIV_M	Fractional Divider MSB Address	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FRACDIV_M |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 1:0 | FRACDIV_M | Adds sampling count when in state stop to parity to contribute fractional divisor.only when high_speed=3. |

A008005C	FCR_RD	FIFO Control Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | RFTL1_RFTL0 |  | TFTL1_TFTL0 |  |  | CLRT | CLRR | FIFOE |
| Type |  |  |  |  |  |  |  |  | RO |  | RO |  |  | RO | RO | RO |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:6 | RFTL1_RFTL0 | RX FIFO trigger threshold RX FIFO contains total 32 bytes. 0: 1 1: 6 2: 12 3: RXTRIG |
| 5:4 | TFTL1_TFTL0 | TX FIFO trigger threshold TX FIFO contains total 32 bytes. 0: 1 1: 4 2: 8 3: 14 |
| 2 | CLRT | 0: TX FIFO is not cleared. 1: TX FIFO is cleared. |
| 1 | CLRR | 0: RX FIFO is not cleared. 1: RX FIFO is cleared. |
| 0 | FIFOE | Enables FIFO This bit must be set to 1 for any of other bits in the registers to have any effect. 0: RX and TX FIFOs are not enabled. 1: RX and TX FIFOs are enabled. |

## UART2

##### General Description

The baseband chipset houses two UARTs. The UARTs provide full duplex serial communication channels between baseband chipset and external dev ices.

The UART has M16C450 and M16550A modes of operation, which are compatible with a range of standard software drivers.	The extensions have been designed to be broadly software compatible with 16550A variants, but certain areas offer no consensus.

In common with the M16550A, the UART supports word lengths from 5 to 8 bits, an optional parity bit and one or two stop bits, and is fully programmable by an 8-bit CPU interface. A 16-bit programmable baud rate generator and an 8-bit scratch register are included, together with separate

transmit and receive FIFOs. 8 modem control lines and a diagnostic loop-back mode are provided. UART also includes two DMA handshake lines, indicating when the FIFOs are ready to transfer data to the CPU. Interrupts can be generated from any of the 10 sources.

Note that UART is designed so that all internal operation is synchronized by the CLK signal. This synchronization results in minor timing differences between the UART and industry standard 16550A device, which means that the core is not clock for clock identical to the original device.

After a hardware reset, UART will be in M16C450 mode. Its FIFOs can then be enabled and	UART can enter M16550A mode. UART has further functions beyond the M16550A mode. Each of the extended functions can be selected individually under software control.

UART provides more powerful enhancements than the industry-standard 16550:

- Hardware flow control. This feature is very useful when the ISR latency is hard to predict and control in the embedded applications. The MCU is relieved of having to fetch the received data within a fixed amount of time.

Note that in order to enable any of the enhancements, the Enhanced Mode bit, EFR[4], must be set. If EFR[4] is not set, IER[7:5], FCR[5:4], IIR[5:4] and MCR[7:6] cannot be written. The enhanced mode bit ensures that UART is backward compatible with the software that has been written for 16C450 and 16550A devices. Figure 35 is the block diagram of the UART2 device.

Clock

APB Bus

Uart_tx_data

Baud

Divisor

APB BUS

I/F

RX FIFO

RX Machine

Modem Control

TX Machine

Baud Rate Generator

TX FIFO

Uart_rx_data

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Modem Outputs Modem Inputs

Figure 36. Block Diagram of UART2

##### Register Definition

Module name: UART2 Base address: (+A0090000h)

| Address | Name | Width | Register Function |
| --- | --- | --- | --- |
| A0090000 | RBR | 8 | RX Buffer Register |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Address | Name | Width | Register Function |
| --- | --- | --- | --- |
|  |  |  | Note: Only when LCR[7] = 0. |
| A0090000 | THR | 8 | TX Holding Register Note: Only when LCR[7] = 0 |
| A0090000 | DLL | 8 | Divisor Latch (LS) Divides the bclk frequency Note: Nodified when LCR[7]!=0 |
| A0090004 | IER | 8 | Interrupt Enable Register Note: Only when LCR[7] = 0. By storing 1 to a specific bit position, the interrupt associated with that bit is enabled. Otherwise, the interrupt will be disabled. IER[3:0] are modified when LCR[7] = 0. IER[7:4] are modified when LCR[7] = 0 & EFR[4] = 1. |
| A0090004 | DLM | 8 | Divisor Latch (MS) uesd to divid the bclk frequency .*NOTE: modified when LCR[7]!=0 |
| A0090008 | IIR | 8 | Interrupt Identification Register Note: Only when LCR!=BF'h. Priority is from high to low as the following. IIR[5:0]=0X1: No interrupt pending IIR[5:0]=0X6: Line status interrupt (under IER[2]=1) IIR[5:0]=0Xc: RX data time-out interrupt (under IER[0]=1) IIR[5:0]=0X4: RX data are placed in the RX bBuffer register or the RX trigger level is reached. (under IER[0]=1). IIR[5:0]=0X2: TX holding register is empty or the contents of the TX FIFO have been reduced to its trigger level (under IER[1]=1). IIR[5:0]=0X10: XOFF character received (under IER[5]=1,EFR[4] = 1). |
| A0090008 | FCR | 8 | FIFO Control Register FCR is used to control the trigger levels of the FIFOs or flush the FIFOs. FCR[7:6] is modified when LCR != BFh FCR[5:4] is modified when LCR != BFh & EFR[4] = 1 FCR[4:0] is modified when LCR != BFh |
| A0090008 | EFR | 8 | Enhanced Feature Register Note: Only when LCR=BF'h |
| A009000C | LCR | 8 | Line Control Register Determines characteristics of serial communication signals. |
| A0090010 | MCR | 8 | Modem Control Register Controls interface signals of the UART. MCR[5:0] are modified when LCR != 8'hBF, MCR[7] can be read when LCR != 8'hBF & EFR[4] = 1. |
| A0090010 | XON1 | 8 | XON1 Char Register Note: XON1 is modified only when LCR=BF'h. |
| A0090014 | LSR | 8 | Line Status Register Modified when LCR != BFh. |
| A0090018 | XOFF1 | 8 | XOFF1 Char Register *Note: XOFF1 is modified only when LCR=BF'h. |
| A009001C | SCR | 8 | Scratch Register A general purpose read/write register. After reset, its value |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Address | Name | Width | Register Function |
| --- | --- | --- | --- |
|  |  |  | will be un-defined. Modified when LCR != BFh. |
| A0090020 | AUTOBAUD_EN | 8 | Auto Baud Detect Enable Register |
| A0090024 | HIGHSPEED | 8 | High Speed Mode Register |
| A0090028 | SAMPLE_COUNT | 8 | Sample Counter Register When HIGHSPEED=3, sample_count will be the threshold value for UART sample counter (sample_num). Count from 0 to sample_count. |
| A009002C | SAMPLE_POINT | 8 | Sample Point Register When HIGHSPEED=3, UART gets the input data when sample_count=sample_num, e.g. system clock = 13MHz, 921600 = 13000000/14. Therefore, sample_count = 13, and sample point = 6 (sampling the central point to decrease the inaccuracy) SAMPLE_POINT is usually (SAMPLE_COUNT-1)/2 without decimal. |
| A0090030 | AUTOBAUD_REG | 8 | Auto Baud Monitor Register the autobaud detection state ,it will not change until enable the autobaud_en again. |
| A0090034 | RATEFIX_AD | 8 | Clock Rate Fix Register |
| A0090038 | AUTOBAUDSAMPL E | 8 | Auto Baud Sample Register Since the system clock may change, autobaud sample duration should change as the system clock changes. When system clock = 13MHz, autobaudsample = 6; when system clock = 26MHz, autobaudsample = 13. When system clock = 52MHz, autobaudsample = 27. |
| A009003C | GUARD | 8 | Guard Time Added Register |
| A0090040 | ESCAPE_DAT | 8 | Escape Character Register |
| A0090044 | ESCAPE_EN | 8 | Escape Enable Register |
| A0090048 | SLEEP_EN | 8 | Sleep Enable Register |
| A009004C | DMA_EN | 8 | DMA Enable Register |
| A0090050 | RXTRI_AD | 8 | Rx Trigger Address |
| A0090054 | FRACDIV_L | 8 | Fractional Divider LSB Address |
| A0090058 | FRACDIV_M | 8 | Fractional Divider MSB Address |
| A009005C | FCR_RD | 8 | FIFO Control Register |

A0090000	RBR	RX Buffer Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | RBR |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RU |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | RBR | Read-only register The received data can be read by accessing this register. Only when LCR[7] = 0. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A0090000	THR	TX Holding Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | THR |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | WO |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | THR | TX Holding Register Write-only register. The data to be transmitted are written to this register and sent to the PC via serial communication. Only when LCR[7] = 0. |

A0090000	DLL	Divisor Latch (LS)	01

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | DLL |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | DLL | Divisor latch low 8-bit data Note: Modified when LCR[7]!=0. |

A0090004	IER	Interrupt Enable Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | CTSI | RTSI | XOFFI |  | EDSSI | ELSI | ETBEI | ERBFI |
| Type |  |  |  |  |  |  |  |  | RW | RW | RW |  | RW | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7 | CTSI | Masks an interrupt that is generated when a rising edge is detected on the CTS modem control line. Note: This interrupt is only enabled when hardware flow control is enabled. 0: Mask an interrupt that is generated when a rising edge is detected on the CTS modem control line. 1: Unmask an interrupt that is generated when a rising edge is detected on the CTS modem control line. |
| 6 | RTSI | Masks an interrupt that is generated when a rising edge is detected on the RTS modem control line. Note: This interrupt is only enabled when hardware flow control is enabled. 0: Mask an interrupt that is generated when a rising edge is detected on the RTS modem control line. 1: Unmask an interrupt that is generated when a rising edge is detected on the RTS modem control line. |
| 5 | XOFFI | Masks an interrupt that is generated when an XOFF character is received. Note: This interrupt is only enabled when software flow control is enabled. 0: Mask an interrupt that is generated when an XOFF character is received. 1: Unmask an interrupt that is generated when an XOFF character is received. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Name | Description |
| --- | --- | --- |
| 3 | EDSSI | When set to 1, an interrupt will be generated if DCTS (MSR[0]) becomes set. 0: No interrupt is generated if DCTS (MSR[0]) becomes set. 1: An interrupt is generated if DCTS (MSR[0]) becomes set. |
| 2 | ELSI | When set to1, an interrupt will be generated if BI, FE, PE or OE (LSR[4:1]) becomes set. 0: No interrupt is generated if BI, FE, PE or OE (LSR[4:1]) becomes set. 1: An interrupt is generated if BI, FE, PE or OE (LSR[4:1]) becomes set. |
| 1 | ETBEI | When set to 1, an interrupt will be generated if the TX holding register is empty or the contents of the TX FIFO have been reduced to its trigger level. 0: No interrupt will be generated if the TX holding register is empty or the contents of the TX FIFO have been reduced to its trigger level. 1: An interrupt will be generated if the TX folding register is empty or the contents of the TX FIFO have been reduced to its trigger level. |
| 0 | ERBFI | When set to 1, an interrupt will be generated if RX data are placed in RX buffer register or the RX trigger level is reached. 0: No interrupt will be generated if RX data are placed in the RX buffer register or the RX trigger level is reached. 1: An interrupt will be generated if RX Data are placed in the RX buffer register or the RX trigger level is reached. |

A0090004	DLM	Divisor Latch (MS)	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | DLM |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | DLM | Divisor latch high 8-bit data Note: Modified when LCR[7]!=0. DLL & DLM can only be updated when DLAB(LCR[7]) is set to 1. Division by 1 will generate a BAUD signal that is constantly high. DLL & DLM setting formula is {DLH,DLL}=(system clock frequency/baud_pulse/baud_rate). When RATE_FIX(RATEFIX_AD[0])=0, system clock frequency = 52MHz. When RATE_FIX(RATEFIX_AD[0])=1 and RATE_FIX(RATEFIX_AD[2])=0, system clock frequency = 26MHz. When RATE_FIX(RATEFIX_AD[0])=1 and RATE_FIX(RATEFIX_AD[2])=1, system clock frequency = 13MHz. For baud_pulse value, refer to HIGH_SPEED(offset=24H) register For example, when at 52MHz, default speed mode and 115200 baud rate, {DLH,DLL}=52MHz/16/115200=28. |

A0090008	IIR	Interrupt Identification Register	01

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FIFOE |  | ID |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RO |  | RU |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:6 | FIFOE |  |
| 5:0 | ID | IIR[5:0] Priority level interrupt source 000001 - No interrupt pending 000110 1 Line status interrupt: BI, FE, PE or OE set in LSR. (Under IER[2]=1) 001100 2 RX data time-out: Time-out on character in RX FIFO. (Under IER[0]=1) 000100 3 RX data received: RX data received or RX trigger level reached. (Under IER[0]=1) 000010 4 TX holding register empty: 000000 5 Modem status change: DCTS set in MSR. (Under IER[3]=1) TX Holding Register empty or TX FIFO trigger level reached. (Under IER[1]=1) 010000 6 Software flow control: XOFF Character received. (Under IER[5]=1) 100000 7 Hardware flow control: CTS or RTS Rising Edge. (Under IER[7]=1 or IER[6]=1) Line status interrupt: A RX line status interrupt (IIR[5:0] = 000110b) will be generated if ELSI (IER[2]) is set and any of BI, FE, PE or OE (LSR[4:1]) becomes set. The interrupt is cleared by reading the line status register. RX data time-out interrupt: When the virtual FIFO mode is disabled, RX data time-out interrupt will be generated if all of the following conditions are applied: - FIFO contains at least one character. - The most recent character is received longer than four character periods ago (including all start, parity and stop bits); - The most recent CPU read of the FIFO is longer than four character periods ago. The timeout timer is restarted upon receipt of a new byte from the RX shift register or upon a CPU read from the RX FIFO. The RX data time-out interrupt is enabled by setting EFRBI (IER[0]) to 1 and is cleared by reading RX FIFO. When the virtual FIFO mode is enabled, RX data time-out interrupt will be generated if all of the following conditions are applied: - FIFO is empty. - The most recent character is received longer than four character periods ago (including all start, parity and stop bits). - The most recent CPU read of the FIFO is longer than four character periods ago. The timeout timer is restarted upon receipt of a new byte from the RX shift register or reading DMA_EN register. The RX Data Timeout Interrupt is enabled by setting EFRBI (IER[0]) to 1 and is cleared by reading DMA_EN register. RX data received interrupt: A RX received interrupt (IER[5:0] = 000100b) is generated if EFRBI (IER[0]) is set and either RX data are placed in the RX buffer register or the RX trigger level is reached. The interrupt is cleared by reading the RX buffer register or the RX FIFO (if enabled). TX holding register empty interrupt: A TX holding register empty interrupt (IIR[5:0] = 000010b) is generated if ETRBI (IER[1]) is set and either the TX holding register is empty or the contents of the TX FIFO are reduced to its trigger level. The interrupt is cleared by writing to the TX holding register or TX FIFO if FIFO is enabled. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Name | Description |
| --- | --- | --- |
|  |  | Modem status change interrupt: A modem status change Interrupt (IIR[5:0] = 000000b) will be generated if EDSSI (IER[3]) is set and e DCTS (MSR[3:0]) becomes set. The interrupt is cleared by reading the modem status register. Software flow control interrupt: A software flow control interrupt (IIR[5:0] = 010000b) will be generated if the software flow control is enabled and XOFFI (IER[5]) becomes set, indicating that an XOFF character has been received. The interrupt is cleared by reading the interrupt identification register. Hardware flow control interrupt: A hardware flow control interrupt (IER[5:0] = 100000b) will be generated if the hardware flow control is enabled and either RTSI (IER[6]) or CTSI (IER[7]) becomes set indicating that a rising edge has been detected on either the RTS/CTS modem control line. The interrupt is cleared by reading the interrupt identification register. |

A0090008	FCR	FIFO Control Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | RFTL1_RFTL 0 |  | TFTL1_TFTL 0 |  |  | CLRT | CLRR | FIFOE |
| Type |  |  |  |  |  |  |  |  | WO |  | WO |  |  | WO | WO | WO |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:6 | RFTL1_RFTL0 | RX FIFO trigger threshold RX FIFO contains total 32 bytes. 0: 1 1: 6 2: 12 3: RXTRIG |
| 5:4 | TFTL1_TFTL0 | TX FIFO trigger threshold TX FIFO contains total 32 bytes. 0: 1 1: 4 2: 8 3: 14 |
| 2 | CLRT | Control bit to clear TX FIFO 0: No effect 1: Clear TX FIFO |
| 1 | CLRR | Control bit to clear RX FIFO 0: No effect 1: Clear RX FIFO |
| 0 | FIFOE | Enables FIFO This bit must be set to 1 for any of other bits in the registers to have any effect. 0: Disable both RX and TX FIFOs. 1: Enable both RX and TX FIFOs. |

A0090008	EFR	Enhanced Feature Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | AUTO | AUTO |  | ENAB | SW_FLOW_CONT |  |  |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

|  |  |  |  |  |  |  |  |  | _CTS | _RTS |  | LE_E |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type |  |  |  |  |  |  |  |  | RW | RW |  | RW | RW |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7 | AUTO_CTS | Enables hardware transmission flow control 0: Disable 1: Enable |
| 6 | AUTO_RTS | Enables hardware reception flow control 0: Disable 1: Enable |
| 4 | ENABLE_E | Enables enhancement feature 0: Disable 1: Enable |
| 3:0 | SW_FLOW_CONT | Software flow control bits 00xx: No TX flow control 01xx: No TX flow control 10xx: Transmit XON1/XOFF1 as flow control bytes xx00: No RX flow control xx01: No RX flow control xx10: Receive XON1/XOFF1 as flow control bytes |

A009000C	LCR	Line Control Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | DLAB | SB | SP | EPS | PEN | STB | WLS1_WLS0 |  |
| Type |  |  |  |  |  |  |  |  | RW | RW | RW | RW | RW | RW | RW |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7 | DLAB | Divisor latch access bit 0: RX and TX registers are read/written at Address 0 and the IER register is read/written at Address 4. 1: Divisor Latch LS is read/written at Address 0 and the Divisor Latch MS is read/written at Address 4. |
| 6 | SB | Set break 0: No effect 1: SOUT signal is forced to the 0 state. |
| 5 | SP | Stick parity 0: No effect. 1: The parity bit is forced to a defined state, depending on the states of EPS and PEN: If EPS=1 & PEN=1, the parity bit will be set and checked = 0. If EPS=0 & PEN=1, the parity bit will be set and checked = 1. |
| 4 | EPS | Selects even parity 0: When EPS=0, an odd number of ones is sent and checked. 1: When EPS=1, an even number of ones is sent and checked. |
| 3 | PEN | Enables parity 0: The parity is neither transmitted nor checked. 1: The parity is transmitted and checked. |
| 2 | STB | Number of STOP bits 0: One STOP bit is always added. 1: Two STOP bits are added after each character is sent; unless the character |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Name | Description |
| --- | --- | --- |
|  |  | length is 5 when 1 STOP bit is added. |
| 1:0 | WLS1_WLS0 | Selects word length 0: 5 bits 1: 6 bits 2: 7 bits 3: 8 bits |

A0090010	MCR	Modem Control Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | XOFF _STAT US |  |  | Loop |  |  | RTS |  |
| Type |  |  |  |  |  |  |  |  | RU |  |  | RW |  |  | RW |  |
| Reset |  |  |  |  |  |  |  |  | 0 |  |  | 0 |  |  | 0 |  |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7 | XOFF_STATUS | Read-only bit 0: When an XON character is received. 1: When an XOFF character is received. |
| 4 | Loop | Loop-back control bit 0: No loop-back is enabled. 1: Loop-back mode is enabled. |
| 1 | RTS | Controls the state of the output NRTS, even in loop mode. 0: RTS will always output 1. 1: RTS's output will be controlled by flow control condition. |

A0090010	XON1	XON1 Char Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | XON1 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | XON1 | XON1 character for software flow control Modified only when LCR=BF'h. |

A0090014	LSR	Line Status Register	60

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FIFOE RR | TEMT | THRE | BI | FE | PE | OE | DR |
| Type |  |  |  |  |  |  |  |  | RU | RU | RU | RU | RU | RU | RU | RU |
| Reset |  |  |  |  |  |  |  |  | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7 | FIFOERR | RX FIFO error indicator 0: No PE, FE, BI set in the RX FIFO. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Name | Description |
| --- | --- | --- |
|  |  | 1: Set to 1 when there is at least one PE, FE or BI in the RX FIFO. |
| 6 | TEMT | TX holding register (or TX FIFO) and the TX shift register are empty. 0: Empty conditions below are not met. 1: If FIFOs are enabled, the bit will be set whenever the TX FIFO and the TX shift register are empty. If FIFOs are disabled, the bit will be set whenever TX holding register and TX shift register are empty. |
| 5 | THRE | Indicates if there is room for TX holding register or TX FIFO is reduced to its trigger level 0: Reset whenever the contents of the TX FIFO are more than its trigger level (FIFOs are enabled), or whenever TX holding register is not empty (FIFOs are disabled). 1: Set whenever the contents of the TX FIFO are reduced to its trigger level (FIFOs are enabled), or whenever TX holding register is empty and ready to accept new data (FIFOs are disabled). |
| 4 | BI | Break interrupt 0: Reset by the CPU reading this register 1: If the FIFOs are disabled, this bit will be set whenever the SIN is held in the 0 state for more than one transmission time (START bit + DATA bits + PARITY + STOP bits). If the FIFOs are enabled, this error will be associated with a corresponding character in the FIFO and is flagged when this byte is at the top of the FIFO. When a break occurs, only one zero character is loaded into the FIFO: The next character transfer is enabled when SIN goes into the marking state and receives the next valid start bit. |
| 3 | FE | Framing error 0: Reset by the CPU reading this register 1: If the FIFOs are disabled, this bit will be set if the received data do not have a valid STOP bit. If the FIFOs are enabled, the state of this bit will be revealed when the byte it refers to is the next to be read. |
| 2 | PE | Parity error 0: Reset by the CPU reading this register 1: If the FIFOs are disabled, this bit will be set if the received data do not have a valid parity bit. If the FIFOs are enabled, the state of this bit will be revealed when the referred byte is the next to be read. |
| 1 | OE | Overrun error 0: Reset by the CPU reading this register. 1: If the FIFOs are disabled, this bit will be set if the RX buffer is not read by the CPU before the new data from the RX shift register overwrites the previous contents. If the FIFOs are enabled, an overrun error will occur when the RX FIFO is full and the RX shift register becomes full. OE will be set as soon as this happens. The character in the shift register is then overwritten, but not transferred to the FIFO. |
| 0 | DR | Data ready 0: Cleared by the CPU reading the RX buffer or by reading all the FIFO bytes. 1: Set by the RX buffer becoming full or by the FIFO becoming no empty. |

A0090018	XOFF1	XOFF1 Char Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | XOFF1 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | XOFF1 | XOFF1 character for software flow control Modified only when LCR=BF'h. |

A009001C	SCR	Scratch Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | SCR |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | SCR | General purpose read/write register After reset, its value will be undefined. Modified when LCR != BFh. |

A0090020	AUTOBAUD_EN Auto Baud Detect Enable Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  | SLEE P_AC K_SEL | AUTO BAUD _SEL | AUTO BAUD _EN |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 2 | SLEEP_ACK_SEL | Selects sleep ack when autobaud_en 0: Support sleep_ack when autobaud_en is opened . 1: Does not support sleep_ack when autobaud_en is opened . |
| 1 | AUTOBAUD_SEL | Selects auto-baud 0: Support standard baud rate detection 1: Support non_standard baud rate detection (support baud from 110 to 115200; recommended to use 52MHz to auto fix) . |
| 0 | AUTOBAUD_EN | Auto-baud enabling signal 0: Disable auto-baud function 1: Enable auto-baud function (UARTn+0024h SPEED should be set to 0.) Note: When AUTOBAUD_EN is active, there should not be A*/a* char before the auto baud char AT/at. If A*/a* is Inevitable, autobaud will fail and please disable AUTOBAUD_EN to reset the autobaud feature and autobaud_en again. |

A0090024	HIGHSPEED	High Speed Mode Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SPEED |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 1:0 | SPEED | UART sample counter base 0: Based on 16*baud_pulse, baud_rate = system clock frequency/16/{DLH, |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Name | Description |
| --- | --- | --- |
|  |  | DLL} 1: Based on 8*baud_pulse, baud_rate = system clock frequency/8/{DLH, DLL} 2: Based on 4*baud_pulse, baud_rate = system clock frequency/4/{DLH, DLL} 3: Based on sampe_count * baud_pulse, baud_rate = system clock frequency / (sampe_count+1)/{DLM, DLL} |

A0090028	SAMPLE_COUN Sample Counter Register	00

T

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | SAMPLECOUNT |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | SAMPLECOUNT | Only useful when HIGHSPEED mode = 3. |

A009002C	SAMPLE_POINTSample Point Register	FF

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | SAMPLEPOINT |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | SAMPLEPOINT | SAMPLE_POINT is usually (SAMPLE_COUNT-1)/2 without decimal. Effective only when HIGHSPEED=3. |

A0090030	AUTOBAUD_RE Auto Baud Monitor Register	00

G

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | BAUD_STAT |  |  |  | BAUD_RATE |  |  |  |
| Type |  |  |  |  |  |  |  |  | RU |  |  |  | RU |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:4 | BAUD_STAT | Autobaud state (only true value in standard autobaud detection) 0: Autobaud is detecting. 1: AT_7N1 2: AT_7O1 3: AT_7E1 4: AT_8N1 5: AT_8O1 6: AT_8E1 7: at_7N1 8: at_7E1 9: at_7O1 10: at_8N1 11: at_8E1 |

| Bit(s) | Name | Description |
| --- | --- | --- |
|  |  | 12: at_8O1 13: Autobaud detection fails |
| 3:0 | BAUD_RATE | Autobaud baud rate (only true value in standard autobaud detection) |
|  |  | 0: 115,200 |
|  |  | 1: 57,600 |
|  |  | 2: 38,400 |
|  |  | 3: 19,200 |
|  |  | 4: 9,600 |
|  |  | 5: 4,800 |
|  |  | 6: 2,400 |
|  |  | 7: 1,200 |
|  |  | 8: 300 |
|  |  | 9: 110 |

A0090034	RATEFIX_AD	Clock Rate Fix Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  | FREQ _SEL | AUTO BAUD _RAT E_FIX | RATE _FIX |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 2 | FREQ_SEL | 0: Select 26MHz as system clock 1: Select 13MHz as system clock |
| 1 | AUTOBAUD_RATE_FI X | 0: Use 52MHz as system clock for UART auto baud detection 1: Use 26MHz/13MHz (depending on FREQ_SEL) as system clock for UART auto baud detection |
| 0 | RATE_FIX | 0: Use 52MHz as system clock for UART TX/RX 1: Use 26MHz/13MHz (depending on FREQ_SEL) as system clock for UART TX/RX |

A0090038	AUTOBAUDSA

MPLE

Auto Baud Sample Register	0D

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  | AUTOBAUDSAMPLE |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 1 | 1 | 0 | 1 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 5:0 | AUTOBAUDSAMPLE | clk diveision for autobaud rate detection For standard baud rate detection. System clk 52m: 'd 27 System clk 26m: 'd 13 System clk 13m: 'd 6 For non-standard baud rate detection. :15. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A009003C	GUARD	Guard Time Added Register	0F

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  | GUAR D_EN | GUARD_CNT |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  | RW | RW |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  | 0 | 1 | 1 | 1 | 1 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 4 | GUARD_EN | Guard interval add enabling signal 0: No guard interval added 1: Add guard interval after stop bit. |
| 3:0 | GUARD_CNT | Guard interval count value Guard interval = [1/(system clock/div_step/div)]*GUARD_CNT. |

A0090040	ESCAPE_DAT	Escape Character Register	FF

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | ESCAPE_DAT |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | ESCAPE_DAT | Escape character added before software flow control data and escape character If TX data are xon (31h), with esc_en =1, UART will transmit data as esc + CEh (~xon). |

A0090044	ESCAPE_EN	Escape Enable Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ESC_ EN |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 0 | ESC_EN | Adds escape character in transmitter and removes escape character in receiver by UART 0: Does not deal with the escape character 1: Add escape character in transmitter and remove escape character in receiver |

A0090048	SLEEP_EN	Sleep Enable Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SLEE P_EN |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Name | Description |
| --- | --- | --- |
| 0 | SLEEP_EN | For sleep mode issue 0: Does not deal with sleep mode indicate signal 1: Activate hardware flow control or software control according to software initial setting when the chip enters sleep mode. Release hardware flow when the chip wakes up. However, for software control, UART sends xon when awaken and when FIFO does not reach threshold level. |

A009004C	DMA_EN	DMA Enable Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | FIFO_l sr_sel | TO_C NT_A UTOR ST | TX_D MA_E N | RX_D MA_E N |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 3 | FIFO_lsr_sel | Selects FIFO LSR mode 0: LSR will hold the first line status error state until you read the LSR register. 1: LSR will update automatically. |
| 2 | TO_CNT_AUTORST | Time-out counter auto reset register 0: After RX time-out happens, SW shall reset the interrupt by reading UART 0x4C. 1: The time-out counter will be auto reset. Set this register when Rain's new DMA is used. |
| 1 | TX_DMA_EN | TX_DMA mechanism enabling signal 0: Does not use DMA in TX 1: Use DMA in TX. When this register is enabled, the flow control will be based on the DMA threshold and generate a time-out interrupt for DMA.for DMA. |
| 0 | RX_DMA_EN | RX_DMA mechanism enabling signal 0: Does not use DMA in RX 1: Use DMA in RX. When this register is enabled, the flow control will be based on the DMA threshold and generate a time-out interrupt |

A0090050	RXTRI_AD	Rx Trigger Address	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | RXTRIG |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 3:0 | RXTRIG | When {rtm,rtl}=2'b11, the RX FIFO threshold will be Rxtrig. The value is suggested to be smaller than half of RX FIFO size, which is 32 bytes. |

| A0090054 | FRACDIV_L | Fractional Divider LSB Address | 00 |
| --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FRACDIV_L |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | FRACDIV_L | Adds sampling count (+1) from state data7 to data0 to contribute fractional divisor.only when high_speed=3. |

A0090058	FRACDIV_M	Fractional Divider MSB Address	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FRACDIV_M |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 1:0 | FRACDIV_M | Adds sampling count when in state stop to parity to contribute fractional divisor.only when high_speed=3. |

A009005C	FCR_RD	FIFO Control Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | RFTL1_RFTL 0 |  | TFTL1_TFTL 0 |  |  | CLRT | CLRR | FIFOE |
| Type |  |  |  |  |  |  |  |  | RO |  | RO |  |  | RO | RO | RO |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:6 | RFTL1_RFTL0 | RX FIFO trigger threshold RX FIFO contains total 32 bytes. 0: 1 1: 6 2: 12 3: RXTRIG |
| 5:4 | TFTL1_TFTL0 | TX FIFO trigger threshold TX FIFO contains total 32 bytes. 0: 1 1: 4 2: 8 3: 14 |
| 2 | CLRT | 0: TX FIFO is not cleared. 1: TX FIFO is cleared. |
| 1 | CLRR | 0: RX FIFO is not cleared. 1: RX FIFO is cleared. |
| 0 | FIFOE | Enables FIFO This bit must be set to 1 for any of other bits in the registers to have any effect. 0: RX and TX FIFOs are not enabled. 1: RX and TX FIFOs are enabled. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

UART3

##### General Description

The baseband chipset houses two UARTs. The UARTs provide full duplex serial communication channels between baseband chipset and external dev ices.

The UART has M16C450 and M16550A modes of operation, which are compatible with a range of standard software drivers.	The extensions have been designed to be broadly software compatible with 16550A variants, but certain areas offer no consensus.

In common with the M16550A, the UART supports word lengths from 5 to 8 bits, an optional parity bit and one or two stop bits, and is fully programmable by an 8-bit CPU interface.	A 16-bit programmable baud rate generator and an 8-bit scratch register are included, together with separate transmit and receive FIFOs.	Eight modem control lines and a diagnostic loop-back mode are provided.	The UART also includes two DMA handshake lines, used to indicate when the FIFOs are ready to transfer data to the CPU.	Interrupts can be generated from any of the 10 sources.

Note: The UART has been designed so that all internal operations are synchronized by the CLK signal. This synchronization results in minor timing differences between the UART and the industry standard 16550A device, which means that the core is not clock for clock identical to the original device.

After a hardware reset, the UART is in M16C450 mode.	Its FIFOs can be enabled and the UART can then enter M16550A mode. The UART adds further functionality beyond M16550A mode.	Each of the extended functions can be selected individually under software control.

The UART provides more powerful enhancements than the industry-standard 16550:

Note: In order to enable any of the enhancements, the Enhanced Mode bit, EFR[4], must be set.	If EFR[4] is not set, IER[7:5], FCR[5:4], IIR[5:4] and MCR[7:6] cannot be written.	The Enhanced Mode bit ensures that the UART is backward compatible with software that has been written for 16C450 and 16550A devices.

Figure 35 shows the block diagram of the UART3 device.

Baud Rate Generator

Baud

Divisor

Clock

TX Machine

Uart_tx_data

APB Bus

APB BUS

I/F

RX FIFO

RX Machine

Uart_rx_data

TX FIFO

Figure 37. Block Diagram of UART3

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Register Definitions

Module name: UART3 Base address: (+A00A0000h)

| Address | Name | Width | Register Function |
| --- | --- | --- | --- |
| A00A0000 | RBR | 8 | RX Buffer Register *NOTE:onnly when LCR[7] = 0. |
| A00A0000 | THR | 8 | TX Holding Register *NOTE:only when LCR[7] = 0. |
| A00A0000 | DLL | 8 | Divisor Latch (LS) uesd to divid the bclk frequency .*NOTE: modified when LCR[7]!=0 |
| A00A0004 | IER | 8 | Interrupt Enable Register *NOTE:onnly when LCR[7] = 0. By storing a '1' to a specific bit position, the interrupt associated with that bit is enabled. Otherwise, the interrupt is disabled. IER[3:0] are modified when LCR[7] = 0. IER[7:4] are modified when LCR[7] = 0 & EFR[4] = 1. |
| A00A0004 | DLM | 8 | Divisor Latch (MS) uesd to divid the bclk frequency .*NOTE: modified when LCR[7]!=0 |
| A00A0008 | IIR | 8 | Interrupt Identification Register *NOTE:only when LCR!=BF'h. priority is from high to low as following .IIR[5:0]==0X1: no interrupt pending .IIR[5:0]==0X6:line status interrup(Under IER[2]=1). IIR[5:0]==0Xc:rx data timeout interrup(Under IER[0]=1). IIR[5:0]==0X4:RX Data is placed in the RX Buffer Register or the RX Trigger Level is reached.(Under IER[0]=1) . IIR[5:0]==0X2: TX Holding Register is empty or the contents of the TX FIFO have been reduced to its Trigger Level(Under IER[1]=1). IIR[5:0]==0X10: XOFF character recieved (Under IER[5]=1,EFR[4] = 1). |
| A00A0008 | FCR | 8 | FIFO Control Register FCR is used to control the trigger levels of the FIFOs, or flush the FIFOs. FCR[7:6] is modified when LCR != BFh FCR[5:4] is modified when LCR != BFh & EFR[4] = 1 FCR[4:0] is modified when LCR != BFh |
| A00A0008 | EFR | 8 | Enhanced Feature Register *NOTE: Only when LCR=BF'h |
| A00A000C | LCR | 8 | Line Control Register Line Control Register. Determines characteristics of serial communication signals. |
| A00A0010 | MCR | 8 | Modem Control Register Modem Control Register. Control interface signals of the UART. MCR[5:0] are modified when LCR != 8'hBF, MCR[7] can be read when LCR != 8'hBF & EFR[4] = 1. |
| A00A0010 | XON1 | 8 | XON1 Char Register *Note: XON1modified only when LCR=BF'h. |
| A00A0014 | LSR | 8 | Line Status Register Line Status Register. Modified when LCR != BFh. |
| A00A0018 | XOFF1 | 8 | XOFF1 Char Register *Note: , XOFF1 modified only when LCR=BF'h. |
| A00A001C | SCR | 8 | Scratch Register A general purpose read/write register. After reset, its value is un-defined. Modified when LCR != BFh. |
| A00A0020 | AUTOBAUD_EN | 8 | Auto Baud Detect Enable Register |
| A00A0024 | HIGHSPEED | 8 | High Speed Mode Register |
| A00A0028 | SAMPLE_COUNT | 8 | Sample Counter Register When HIGHSPEED=3, the sample_count is the threshold value for UART sample counter (sample_num). Count from 0 to sample_count. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| A00A002C | SAMPLE_POINT | 8 | Sample Point Register When HIGHSPEED=3, UART gets the input data when sample_count=sample_num. e.g. system clock = 13MHz, 921600 = 13000000 / 14 sample_count = 13 and sample point = 6 (sample the central point to decrease the inaccuracy) The SAMPLE_POINT is usually (SAMPLE_COUNT-1)/2 and remove the decimal. |
| --- | --- | --- | --- |
| A00A0030 | AUTOBAUD_REG | 8 | Auto Baud Monitor Register the autobaud detection state ,it will not change until enable the autobaud_en again. |
| A00A0034 | RATEFIX_AD | 8 | Clock Rate Fix Register |
| A00A0038 | AUTOBAUDSAMPLE | 8 | Auto Baud Sample Register Since the system clock may change, autobaud sample duration should change as system clock changes. When system clock = 13MHz, autobaudsample = 6; when system clock = 26MHz, autobaudsample = 13;When system clock = 52MHz, autobaudsample = 27. |
| A00A003C | GUARD | 8 | Guard time added register |
| A00A0040 | ESCAPE_DAT | 8 | Escape character register |
| A00A0044 | ESCAPE_EN | 8 | Escape enable register |
| A00A0048 | SLEEP_EN | 8 | Sleep enable register |
| A00A004C | DMA_EN | 8 | DMA enable register |
| A00A0050 | RXTRI_AD | 8 | Rx Trigger Address |
| A00A0054 | FRACDIV_L | 8 | Fractional Divider LSB Address |
| A00A0058 | FRACDIV_M | 8 | Fractional Divider MSB Address |
| A00A005C | FCR_RD | 8 | FIFO Control Register |

A00A0000	RBR	RX Buffer Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | RBR |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RU |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | RBR | Read-only register. The received data can be read by accessing this register. Only when LCR[7] = 0. |

A00A0000	THR	TX Holding Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | THR |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | WO |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | THR | TX Holding Register. Write-only register. The data to be transmitted is written to this register, and then sent to the PC via serial communication. only when LCR[7] = 0. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A00A0000	DLL	Divisor Latch (LS)	01

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | DLL |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | DLL | divisor Latch low 8bit data. *NOTE: modified when LCR[7]!=0 |

A00A0004	IER	Interrupt Enable Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  | XOFFI |  |  | ELSI | ETBEI | ERBFI |
| Type |  |  |  |  |  |  |  |  |  |  | RW |  |  | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  | 0 |  |  | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 5 | XOFFI | Masks an interrupt that is generated when an XOFF character is received. Note: This interrupt is only enabled when software flow control is enabled. 0: Mask an interrupt that is generated when an XOFF character is received. 1: Unmask an interrupt that is generated when an XOFF character is received. |
| 2 | ELSI | When set ("1"), an interrupt is generated if BI, FE, PE or OE (LSR[4:1]) becomes set. 0: No interrupt is generated if BI, FE, PE or OE (LSR[4:1]) becomes set. 1: An interrupt is generated if BI, FE, PE or OE (LSR[4:1]) becomes set. |
| 1 | ETBEI | When set ("1"), an interrupt is generated if the TX Holding Register is empty or the contents of the TX FIFO have been reduced to its Trigger Level. 0: No interrupt is generated if the TX Holding Register is empty or the contents of the TX FIFO have been reduced to its Trigger Level. 1: An interrupt is generated if the TX Holding Register is empty or the contents of the TX FIFO have been reduced to its Trigger Level |
| 0 | ERBFI | When set ("1"), an interrupt is generated if RX Data is placed in the RX Buffer Register or the RX Trigger Level is reached. 0: No interrupt is generated if RX Data is placed in the RX Buffer Register or the RX Trigger Level is reached. 1: An interrupt is generated if RX Data is placed in the RX Buffer Register or the RX Trigger Level is reached. |

A00A0004	DLM	Divisor Latch (MS)	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | DLM |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | DLM | divisor Latch high 8bit data.. *NOTE: modified when LCR[7]!=0 Note: DLL & DLM can only be updated when DLAB(LCR[7]) is set to 1, Note to that division by 1 generates a BAUD signal that is constantly high. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Note: DLL & DLM setting formula is {DLH,DLL}=(system clock frequency/baud_pulse/baud_rate).

When RATE_FIX(RATEFIX_AD[0])=0, system clock frequency = 52MHz. When RATE_FIX(RATEFIX_AD[0])=1 and RATE_FIX(RATEFIX_AD[2])=0,

system clock frequency = 26MHz.

When RATE_FIX(RATEFIX_AD[0])=1 and RATE_FIX(RATEFIX_AD[2])=1,

system clock frequency = 13MHz.

For baud_pulse value refer to HIGH_SPEED(offset=24H) register.

e.g. When 52MHz,default speed mode and 115200 baud rate, the

{DLH,DLL}=52MHz/16/115200=28.

A00A0008	IIR	Interrupt Identification Register	01

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FIFOE |  | ID |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RO |  | RU |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:6 | FIFOE |  |
| 5:0 | ID | IIR[5:0] Priority Level Interrupt Source 000001 - No interrupt pending 000110 1 Line Status Interrupt: BI, FE, PE or OE set in LSR. (Under IER[2]=1) 001100 2 RX Data Timeout: Timeout on character in RX FIFO. (Under IER[0]=1) 000100 3 RX Data Received: RX Data received or RX Trigger Level reached. (Under IER[0]=1) 000010 4 TX Holding Register Empty: 000000 5 Modem Status change: DCTS set in MSR. (Under IER[3]=1) TX Holding Register empty or TX FIFO Trigger Level reached. (Under IER[1]=1) 010000 6 Software Flow Control: XOFF Character received. (Under IER[5]=1) 100000 7 Hardware Flow Control: CTS or RTS Rising Edge. (Under IER[7]=1 or IER[6]=1) Line Status Interrupt: A RX Line Status Interrupt (IIR[5:0] == 000110b) is generated if ELSI (IER[2]) is set and any of BI, FE, PE or OE (LSR[4:1]) becomes set. The interrupt is cleared by reading the Line Status Register. RX Data Timeout Interrupt:When virtual FIFO mode is disabled, RX Data Timeout Interrupt is generated if all of the following apply: - FIFO contains at least one character; - The most recent character was received longer than four character periods ago (including all start, parity and stop bits); - The most recent CPU read of the FIFO was longer than four character periods ago. The timeout timer is restarted on receipt of a new byte from the RX Shift Register, or on a CPU read from the RX FIFO. The RX Data Timeout Interrupt is enabled by setting EFRBI (IER[0]) to 1, and is cleared by reading RX FIFO. When virtual FIFO mode is enabled, RX Data Timeout Interrupt is generated if all of the following apply: - FIFO is empty; - The most recent character was received longer than four character periods ago (including all start, parity and stop bits); - The most recent CPU read of the FIFO was longer than four character periods ago. The timeout timer is restarted on receipt of a new byte from the RX Shift Register or reading DMA_EN register. The RX Data Timeout Interrupt is enabled by setting EFRBI (IER[0]) to 1, and is cleared by reading DMA_EN register. RX Data Received Interrupt: A RX Received interrupt (IER[5:0] == 000100b) is generated if EFRBI (IER[0]) is set and either RX Data is placed in the RX Buffer Register or the RX Trigger Level is reached. The interrupt is cleared by reading the RX Buffer Register or the RX FIFO (if enabled). |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

TX Holding Register Empty Interrupt: A TX Holding Register Empty Interrupt (IIR[5:0] = 000010b) is generated if ETRBI (IER[1]) is set and either the TX Holding Register is empty or the contents of the TX FIFO have been reduced to its Trigger Level. The interrupt is cleared by writing to the TX Holding Register or TX FIFO if FIFO enabled.

Modem Status Change Interrupt: A Modem Status Change Interrupt (IIR[5:0] = 000000b) is generated if EDSSI (IER[3]) is set and e DCTS (MSR[3:0]) becomes set. The interrupt is cleared by reading the Modem Status Register.

Software Flow Control Interrupt: A Software Flow Control Interrupt (IIR[5:0] = 010000b) is generated if Software Flow Control is enabled and XOFFI (IER[5]) becomes set, indicating that an XOFF character has been received. The interrupt is cleared by reading the Interrupt Identification Register.

Hardware Flow Control Interrupt: A Hardware Flow Control Interrupt (IER[5:0] = 100000b) is generated if Hardware Flow Control is enabled and either RTSI (IER[6]) or CTSI (IER[7]) becomes set indicating that a rising edge has been detected on either the RTS/CTS Modem Control line. The interrupt is cleared by reading the Interrupt Identification Register.

A00A0008	FCR	FIFO Control Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | RFTL1_RFTL 0 |  | TFTL1_TFTL 0 |  |  | CLRT | CLRR | FIFOE |
| Type |  |  |  |  |  |  |  |  | WO |  | WO |  |  | WO | WO | WO |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:6 | RFTL1_RFTL0 | RX FIFO trigger threshold. (RX FIFO contains total 32 bytes.) 0: 1 1: 6 2: 12 3: RXTRIG |
| 5:4 | TFTL1_TFTL0 | TX FIFO trigger threshold (TX FIFO contains total 32 bytes.) 0: 1 1: 4 2: 8 3: 14 |
| 2 | CLRT | control bit to clear tx fifo 0: no effect 1: clear TX FIFO |
| 1 | CLRR | control bit to clear rx fifo 0: no effect 1: clear RX FIFO |
| 0 | FIFOE | FIFO Enabled. This bit must be set to 1 for any of the other bits in the registers to have any effect. 0: Disable both the RX and TX FIFOs. 1: Enable both the RX and TX FIFOs. |

A00A0008	EFR	Enhanced Feature Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  | ENAB LE_E | SW_FLOW_CONT |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  | RW | RW |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Name | Description |
| --- | --- | --- |
| 4 | ENABLE_E | Enables enhancement feature 0: Disabled. 1: Enabled. |
| 3:0 | SW_FLOW_CONT | Software flow control bits. 00xx: No TX Flow Control 01xx: No TX Flow Control 10xx: Transmit XON1/XOFF1 as flow control bytes xx00: No RX Flow Control xx01: No RX Flow Control xx10: Receive XON1/XOFF1 as flow control bytes |

A00A000C	LCR	Line Control Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | DLAB | SB | SP | EPS | PEN | STB | WLS1_WLS0 |  |
| Type |  |  |  |  |  |  |  |  | RW | RW | RW | RW | RW | RW | RW |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7 | DLAB | Divisor Latch Access Bit. 0: The RX and TX Registers are read/written at Address 0 and the IER register is read/written at Address 4. 1: The Divisor Latch LS is read/written at Address 0 and the Divisor Latch MS is read/written at Address 4. |
| 6 | SB | Set Break 0: No effect 1: SOUT signal is forced into the "0" state. |
| 5 | SP | Stick Parity 0: No effect. 1: The Parity bit is forced into a defined state, depending on the states of EPS and PEN: If EPS=1 & PEN=1, the Parity bit is set and checked = 0. If EPS=0 & PEN=1, the Parity bit is set and checked = 1. |
| 4 | EPS | Even Parity Select 0: When EPS=0, an odd number of ones is sent and checked. 1: When EPS=1, an even number of ones is sent and checked. |
| 3 | PEN | Parity Enable 0: The Parity is neither transmitted nor checked. 1: The Parity is transmitted and checked. |
| 2 | STB | Number of STOP bits 0: One STOP bit is always added. 1: Two STOP bits are added after each character is sent; unless the character length is 5 when 1 STOP bit is added. |
| 1:0 | WLS1_WLS0 | Word Length Select. 0: 5 bits 1: 6 bits 2: 7 bits 3: 8 bits |

A00A0010	MCR	Modem Control Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | XOFF _STAT |  |  | Loop |  |  |  |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

|  |  |  |  |  |  |  |  |  | US |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type |  |  |  |  |  |  |  |  | RU |  |  | RW |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 |  |  | 0 |  |  |  |  |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7 | XOFF_STATUS | This is a read-only bit. 0: When an XON character is received. 1: When an XOFF character is received. |
| 4 | Loop | Loop-back control bit. 0: No loop-back is enabled. 1: Loop-back mode is enabled. |

A00A0010	XON1	XON1 Char Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | XON1 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | XON1 | XON1 character for software flow control. modified only when LCR=BF'h. |

A00A0014	LSR	Line Status Register	60

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FIFOE RR | TEMT | THRE | BI | FE | PE | OE | DR |
| Type |  |  |  |  |  |  |  |  | RU | RU | RU | RU | RU | RU | RU | RU |
| Reset |  |  |  |  |  |  |  |  | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7 | FIFOERR | RX FIFO Error Indicator. 0: No PE, FE, BI set in the RX FIFO. 1: Set to 1 when there is at least one PE, FE or BI in the RX FIFO. |
| 6 | TEMT | TX Holding Register (or TX FIFO) and the TX Shift Register are empty. 0: Empty conditions below are not met. 1: If FIFOs are enabled, the bit is set whenever the TX FIFO and the TX Shift Register are empty. If FIFOs are disabled, the bit is set whenever TX Holding Register and TX Shift Register are empty. |
| 5 | THRE | Indicates if there is room for TX Holding Register or TX FIFO is reduced to its Trigger Level. 0: Reset whenever the contents of the TX FIFO are more than its Trigger Level (FIFOs are enabled), or whenever TX Holding Register is not empty(FIFOs are disabled). 1: Set whenever the contents of the TX FIFO are reduced to its Trigger Level (FIFOs are enabled), or whenever TX Holding Register is empty and ready to accept new data (FIFOs are disabled). |
| 4 | BI | Break Interrupt. 0: Reset by the CPU reading this register 1: If the FIFOs are disabled, this bit is set whenever the SIN is held in the 0 state for more than one transmission time (START bit + DATA bits + PARITY + STOP bits). If the FIFOs are enabled, this error is associated with a corresponding character in the FIFO and is flagged when this byte is at the top of the FIFO. When a break occurs, only one zero character is loaded into the FIFO: the next character transfer is enabled when SIN goes into the marking state and receives the next valid start bit. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| 3 | FE | Framing Error. 0: Reset by the CPU reading this register 1: If the FIFOs are disabled, this bit is set if the received data did not have a valid STOP bit. If the FIFOs are enabled, the state of this bit is revealed when the byte it refers to is the next to be read. |
| --- | --- | --- |
| 2 | PE | Parity Error 0: Reset by the CPU reading this register 1: If the FIFOs are disabled, this bit is set if the received data did not have a valid parity bit. If the FIFOs are enabled, the state of this bit is revealed when the referred byte is the next to be read. |
| 1 | OE | Overrun Error. 0: Reset by the CPU reading this register. 1: If the FIFOs are disabled, this bit is set if the RX Buffer was not read by the CPU before new data from the RX Shift Register overwrote the previous contents. If the FIFOs are enabled, an overrun error occurs when the RX FIFO is full and the RX Shift Register becomes full. OE is set as soon as this happens. The character in the Shift Register is then overwritten, but not transferred to the FIFO. |
| 0 | DR | Data Ready. 0: Cleared by the CPU reading the RX Buffer or by reading all the FIFO bytes. 1: Set by the RX Buffer becoming full or by the FIFO becoming no empty. |

A00A0018	XOFF1	XOFF1 Char Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | XOFF1 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | XOFF1 | XOFF1 character for software flow control. modified only when LCR=BF'h. |

A00A001C	SCR	Scratch Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | SCR |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | SCR | A general purpose read/write register. After reset, its value is un-defined. Modified when LCR != BFh. |

A00A0020	AUTOBAUD_EN Auto Baud Detect Enable Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  | SLEE P_AC K_SEL | AUTO BAUD _SEL | AUTO BAUD _EN |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| 2 | SLEEP_ACK_SEL | Sleep ack select when autobaud_en 0: support sleep_ack when autobaud_en is opened . 1: not support sleep_ack when autobaud_en is opened . |
| --- | --- | --- |
| 1 | AUTOBAUD_SEL | Auto-baud select 0: support standard baud rate detection . 1: support non_standard baud rate detection(support baud from 110 to 115200, it is recommend to use 52MHZ to auto fix) . |
| 0 | AUTOBAUD_EN | Auto-baud enable signal 0: Auto-baud function disable 1: Auto-baud function enable (UARTn+0024h SPEED should be set 0) Note: when AUTOBAUD_EN is active, there should not A*/a* char before the auto baud char AT/at, if the A*/a* is Inevitable, the autobaud will fail and please disable the AUTOBAUD_EN to reset the autobaud feature and autobaud_en again. |

A00A0024	HIGHSPEED	High Speed Mode Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SPEED |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 1:0 | SPEED | UART sample counter base 0: based on 16*baud_pulse, baud_rate = system clock frequency/16/{DLH, DLL} 1: based on 8*baud_pulse, baud_rate = system clock frequency/8/{DLH, DLL} 2: based on 4*baud_pulse, baud_rate = system clock frequency/4/{DLH, DLL} 3: based on sampe_count * baud_pulse, baud_rate = system clock frequency / (sampe_count+1) / {DLM, DLL} |

A00A0028	SAMPLE_COUN Sample Counter Register	00

T

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | SAMPLECOUNT |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | SAMPLECOUNT | Just be useful when HIGHSPEED mode = 3. |

A00A002C	SAMPLE_POINTSample Point Register	FF

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | SAMPLEPOINT |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | SAMPLEPOINT | The SAMPLE_POINT is usually (SAMPLE_COUNT-1)/2 and remove the decimal. sample point , is effective only When HIGHSPEED=3 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A00A0030	AUTOBAUD_RE Auto Baud Monitor Register	00

G

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | BAUD_STAT |  |  |  | BAUD_RATE |  |  |  |
| Type |  |  |  |  |  |  |  |  | RU |  |  |  | RU |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:4 | BAUD_STAT | Autobaud state(only true value in standard autobaud detection) 0: Autobaud is detecting 1: AT_7N1 2: AT_7O1 3: AT_7E1 4: AT_8N1 5: AT_8O1 6: AT_8E1 7: at_7N1 8: at_7E1 9: at_7O1 10: at_8N1 11: at_8E1 12: at_8O1 13: Autobaud detection fails |
| 3:0 | BAUD_RATE | Autobaud baud rate(only true value in standard autobaud detection) 0: 115200 1: 57600 2: 38400 3: 19200 4: 9600 5: 4800 6: 2400 7: 1200 8: 300 9: 110 |

A00A0034	RATEFIX_AD	Clock Rate Fix Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  | FREQ _SEL | AUTO BAUD _RAT E_FIX | RATE _FIX |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 2 | FREQ_SEL | 0: Select 26MHZ as system clock 1: Select 13MHZ as system clock |
| 1 | AUTOBAUD_RATE_FIX | 0: Use 52MHZ as system clock for UART auto baud detect 1: Use 26MHZ/13MHZ(depends on FREQ_SEL) as system clock for UART auto baud detect |
| 0 | RATE_FIX | 0: Use 52MHZ as system clock for UART TX/RX 1: Use 26MHZ/13MHZ(depends on FREQ_SEL) as system clock for UART TX/RX |

| A00A0038 | AUTOBAUDSA MPLE | Auto Baud Sample Register | 0D |
| --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  | AUTOBAUDSAMPLE |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 1 | 1 | 0 | 1 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 5:0 | AUTOBAUDSAMPLE | clk diveision for autobaud rate dectection. for standard baud rate detection. system clk 52m :'d 27 system clk 26m :'d 13 system clk 13m :'d 6 for non-standard baud rate detection. :15. |

A00A003C	GUARD	Guard time added register	0F

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  | GUAR D_EN | GUARD_CNT |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  | RW | RW |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  | 0 | 1 | 1 | 1 | 1 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 4 | GUARD_EN | Guard interval add enable signal. 0: No guard interval added. 1: Add guard interval after stop bit. |
| 3:0 | GUARD_CNT | Guard interval count value. Guard interval = (1/(system clock / div_step / div )) * GUARD_CNT. |

A00A0040	ESCAPE_DAT	Escape character register	FF

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | ESCAPE_DAT |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | ESCAPE_DAT | Escape character added before software flow control data and escape character, i.e. if tx data is xon (31h), with esc_en =1, uart transmits data as esc + CEh (~xon). |

A00A0044	ESCAPE_EN	Escape enable register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ESC_ EN |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 0 | ESC_EN | Add escape character in transmitter and remove escape character in receiver by UART. 0: Do not deal with the escape character. 1: Add escape character in transmitter and remove escape character in receiver. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A00A0048	SLEEP_EN	Sleep enable register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SLEE P_EN |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 0 | SLEEP_EN | For sleep mode issue 0: Do not deal with sleep mode indicate signal 1: To activate hardware flow control or software control according to software initial setting when chip enters sleep mode. Releasing hardware flow when chip wakes up; but for software control, uart sends xon when awaken and when FIFO does not reach threshold level. |

A00A004C	DMA_EN	DMA enable register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | FIFO_l sr_sel | TO_C NT_A UTOR ST | TX_D MA_E N | RX_D MA_E N |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 3 | FIFO_lsr_sel | fifo lsr mode selection 0: lsr will hold the first line status error state until you read the lsr register. 1: lsr will update automatically. |
| 2 | TO_CNT_AUTORST | Timeout counter auto reset register 0: After RX timeout happen, SW shall reset the interrupt by reading UART 0x4C. 1: The timeout counter will be auto reset. Set this register when Rain's new DMA is used. |
| 1 | TX_DMA_EN | TX_DMA mechanism enable signal 0: Do not use DMA in TX. 1: Use DMA in TX. When this reguster is enabled, the flow control is based on the DMA threshold, and generates a timeout interrupt for DMA.for DMA. |
| 0 | RX_DMA_EN | RX_DMA mechanism enable signal 0: Do not use DMA in RX 1: Use DMA in RX. When this reguster is enabled, the flow control is based on the DMA threshold, and generates a timeout interrupt |

A00A0050	RXTRI_AD	Rx Trigger Address	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | RXTRIG |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 3:0 | RXTRIG | When {rtm,rtl}=2'b11, The Rx FIFO threshold will be Rxtrig. The value is suggested to be less than half of RX FIFO size, which is 32 Bytes. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A00A0054	FRACDIV_L	Fractional Divider LSB Address	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FRACDIV_L |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | FRACDIV_L | Add sampling count (+1) from state data7 to data0, in order to contribute fractional divisor.only when high_speed==3. |

A00A0058	FRACDIV_M	Fractional Divider MSB Address	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FRACDIV_M |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 1:0 | FRACDIV_M | Add sampling count when in state stop to parity, in order to contribute fractional divisor.only when high_speed==3. |

A00A005C	FCR_RD	FIFO Control Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | RFTL1_RFTL 0 |  | TFTL1_TFTL 0 |  |  | CLRT | CLRR | FIFOE |
| Type |  |  |  |  |  |  |  |  | RO |  | RO |  |  | RO | RO | RO |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:6 | RFTL1_RFTL0 | RX FIFO trigger threshold. (RX FIFO contains total 32 bytes.) 0: 1 1: 6 2: 12 3: RXTRIG |
| 5:4 | TFTL1_TFTL0 | TX FIFO trigger threshold (TX FIFO contains total 32 bytes.) 0: 1 1: 4 2: 8 3: 14 |
| 2 | CLRT | 0: TX FIFO is not cleared 1: TX FIFO is cleared |
| 1 | CLRR | 0: RX FIFO is not cleared 1: RX FIFOis cleared |
| 0 | FIFOE | FIFO Enabled. This bit must be set to 1 for any of the other bits in the registers to have any effect. 0: the RX and TX FIFOs are not enabled.. 1: RX and TX FIFOs are enabled. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

I2C/SCCB Controller

##### General Description

I2C (Inter-IC)/SCCB (Serial Camera Control Bus) is a two-wire serial interface. The two signals are SCL and SDA. SCL is a clock signal that is driven by the master. SDA is a bi-directional data signal that can be driven by either the master or the slave. This generic controller supports the master role and conforms to the I2C specification.

###### Feature

- I2C compliant master mode operation

- Adjustable clock speed for LS/FS mode operation

- Supports 7-bit/10-bit addressing

- Supports high-speed mode

- Supports slave clock extension

- START/STOP/REPEATED START condition

- Manual transfer mode

- Multi-write per transfer (up to 8 data bytes for non-DMA mode)

- Multi-read per transfer (up to 8 data bytes for non-DMA mode)

- Multi-transfer per transaction

- Combined format transfer with length change capability

- Active drive/wired-and I/O configuration

###### Manual Transfer Mode

The controller offers one transfer mode, the manual mode.

When the manual mode is selected, in addition to the slave address register, the controller has a

built-in 8byte deep FIFO which allows MCU to prepare up to 8 bytes of data for a write transfer, or read up to 8 bytes of data for a read transfer.

###### Transfer Format Support

This controller has been designed to be as generic as possible in order to support a wide range of devices that may utilize different combinations of transfer formats. Here are the transfer format types that can be supported through different software configuration:

Wording convention note

- Transfer = Anything encapsulated within a Start and Stop or Repeated Start.

- Transfer length = Number of bytes within the transfer

- Transaction = This is the top unit. Everything combined equals 1 transaction.

- Transaction length = Number of transfers to be conducted.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Master to slave dir  Slave to master dir

Single-byte access

Single Byte Write

S

A

Slave Address

A

DATA

P

Single Byte Read

S

A

Slave Address

nA

DATA

P

Multi-byte access

S

A

Slave Address

P

Multi Byte Write

N bytes + ack

DATA

A

Multi Byte Read

S

A

Slave Address

A/

nA

DATA

P

N bytes + ack/nak

Multi-byte transfer + multi-transfer (same direction)

Multi Byte Write + Multi Transfer

Slave

Address

A

DATA	A

+ wait time +

N bytes + ack/nak

X transfers

P

S

Multi Byte Read + Multi Transfer

| DATA | A/ |
| --- | --- |
|  | nA |

+ wait time +

N bytes + ack/nak

X transfers

A

Slave

Address

P

S

Multi-byte transfer + multi-transfer w RS (same direction)

Multi Byte Write + Multi Transfer + Repeated Start

Slave

Address

A

DATA	A

N bytes + ack/nak

X transfers

R

P

S

+

N bytes + ack/nak

X transfers

A

Slave Address

R

Multi Byte Read + Multi Transfer + Repeated Start

P

S

| DATA | A/ |
| --- | --- |
|  | nA |

+	+

Combined write/read with Repeated Start (direction change)

Note: Only supports write and then read sequence. Read and then write is not supported.

Combined Multi Byte Write + Multi Byte Read

S

A

Slave Address

A

DATA

R

A

Slave Address

A

DATA

P

N bytes + ack/nak	M bytes + ack/nak

##### Programming Examples

Common transfer programmable parameters

Programmable Parameters

slave_addr

rs_stop

slave_addr + dir change

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

S

Slave

Address

A

P/

RS

transfer_len

P/

RS

transfer_len / aux transfer_len

| DATA | A | DATA | A |
| --- | --- | --- | --- |

| S | Slave Address | A |
| --- | --- | --- |

| DATA | A | DATA | A |
| --- | --- | --- | --- |

delay_len

transac_len

Output waveform timing programmable parameters

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

##### Register Definition

Module name: I2C_SCCB_Controller base address: (+A0120000h)

| Address | Name | Width | Register function |
| --- | --- | --- | --- |
| A0120000 | DATA_PORT | 16 | Data port register |
| A0120004 | SLAVE_ADDR | 16 | Slave address register |
| A0120008 | INTR_MASK | 16 | Interrupt mask register This register provides masks for the corresponding interrupt sources as indicated in the intr_stat register. 1 = Allow interrupt 0 = Disable interrupt Note: While disabled, the corresponding interrupt will not be asserted, however intr_stat will still be updated with the status, i.e. mask does not affect intr_stat register values. |
| A012000C | INTR_STAT | 16 | Interrupt status register When an interrupt is issued by the I2C controller, this register will need to be read by MCU to determine the cause for the interrupt. After this status has been read and appropriate actions are taken, the corresponding interrupt source will need to be written 1 to clear. |
| A0120010 | CONTROL | 16 | Control register |
| A0120014 | TRANSFER_LEN | 16 | Transfer length register (number of bytes per transfer) |
| A0120018 | TRANSAC_LEN | 16 | Transaction length register (number of transfers per transaction) |
| A012001C | DELAY_LEN | 16 | Inter delay length register |
| A0120020 | TIMING | 16 | Timing control register LS/FS only. This register is used to control the output waveform timing. Each half pulse width, i.e. each high or low pulse, is equal to (step_cnt_div+1)*(sample_cnt_div + 1)/13MHz |
| A0120024 | START | 16 | Start register |
| A0120030 | FIFO_STAT | 16 | FIFO status register |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Address | Name | Width | Register function |
| --- | --- | --- | --- |
| A0120038 | FIFO_ADDR_CLR | 16 | FIFO address clear register |
| A0120040 | IO_CONFIG | 16 | IO config register This register is used to configure the I/O for the SDA and SCL lines to select between normal I/O mode, or open-drain mode to support wired-and bus. |
| A0120048 | HS | 16 | High speed mode register This register contains options for supporting high speed operation features. Each HS half pulse width, i.e. each high or low pulse, is equal to (step_cnt_div+1)*(sample_cnt_div + 1)/13MHz |
| A0120050 | SOFTRESET | 16 | Soft reset register |
| A0120060 | SPARE | 16 | SPARE |
| A0120064 | DEBUGSTAT | 16 | Debug status register |
| A0120068 | DEBUGCTRL | 16 | Debug control register |
| A012006C | TRANSFER_LEN_A UX | 16 | Transfer length register (number of bytes per transfer) |
| A0120074 | TIMEOUT | 16 | Timeout timing register |

A0120000	DATA_PORT	Data Port Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | DATA_PORT |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7:0 | DATA_PORT | DATA_PORT | FIFO access port During master write sequences (slave_addr[0] = 0), this port can be written by APB, and during master read sequences (slave_addr[0] = 1), this port can be read by APB. Note: Slave_addr must be set correctly before accessing FIFO. For debugging only: If the fifo_apb_debug bit is set, FIFO can be read and written by the APB. |

A0120004	SLAVE_ADDR	Slave Address Register	00BF

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | SLAVE_ADDR |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7:0 | SLAVE_ADD R | SLAVE_ADDR | Specifies the slave address of the device to be accessed Bit 0 is defined by the I2C protocol as a bit that indicates the direction of transfer. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Master write 1: Master read |

A0120008	INTR_MASK	Interrupt Mask Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  | MASK _TIME OUT |  | MASK _HS_N ACKE RR | MASK _ACK ERR | MASK _TRA NSAC _COM P |
| Type |  |  |  |  |  |  |  |  |  |  |  | RW |  | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  | 0 |  | 0 | 0 | 0 |

Overview: This register provides masks for the corresponding interrupt sources as indicated in the intr_stat register. 1 = allow interrupt; 0 = disable interrupt Note that while disabled, the corresponding interrupt will not be asserted. However, intr_stat will still be updated with the status, i.e. mask does not affect intr_stat register values.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 4 | MASK_TIME OUT | MASK_TIMEOUT | Setting this value to 0 will mask TIMEOUT interrupt signal. |
| 2 | MASK_HS_ NACKERR | MASK_HS_NACKE RR | Setting this value to 0 will mask HS_NACKERR interrupt signal. |
| 1 | MASK_ACK ERR | MASK_ACKERR | Setting this value to 0 will mask ACK_ERR interrupt signal. |
| 0 | MASK_TRA NSAC_COM P | MASK_TRANSAC_ COMP | Setting this value to 0 will mask TRANSAC_COMP interrupt signal. |

A012000C	INTR_STAT	Interrupt Status Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  | TIMEO UT | ARB_ LOST | HS_N ACKE RR | ACKE RR | TRAN SAC_ COMP |
| Type |  |  |  |  |  |  |  |  |  |  |  | W1C | W1C | W1C | W1C | W1C |
| Reset |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 |

Overview: When an interrupt is issued by the I2C controller, this register will need to be read by MCU to determine the cause for the interrupt. After this status has been read and appropriate actions are taken, the corresponding interrupt source will need to be written 1 to clear.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 4 | TIMEOUT | TIMEOUT_IRQ | This status is asserted if time-out is enabled and the timer expires. The internal master state machine will stop, and MCU will need to manually clear the state machine by either issuing software reset by disabling the transact_en bit. Time-out can be used to detect PCB or I2C malfunction. |
| 3 | ARB_LOST | SPARE | Reserved |
| 2 | HS_NACKE | HS_NACKERR | This status is asserted if HS master code NACK error detection is |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  | RR |  | enabled. If enabled, HS master code NACK err will cause transaction to end, and stop will be issued. |
| 1 | ACKERR | ACKERR | This status is asserted if ACK error detection is enabled. If enabled, ACKERR will cause transaction to end, and stop will be issued. |
| 0 | TRANSAC_ COMP | TRANSAC_COMP | This status is asserted when a transaction is completed successfully. |

A0120010	CONTROL	Control Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  | TIMEO UT_E N | RESE T_BU S_BU SY_E N | TRAN SFER _LEN_ CHAN GE | ACKE RR_D ET_E N | DIR_C HANG E | CLK_ EXT_E N |  | RS_S TOP |  |
| Type |  |  |  |  |  |  |  | RW | RW | RW | RW | RW | RW |  | RW |  |
| Reset |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 8 | TIMEOUT_E N | TIMEOUT_EN | Enables time-out mechanism When enabled, if SCL stays at 0 for too long period of time due to I2C slave holds SCL at 0 for too long or SCL stucks at 0 due to PCB issue, the master shall terminate the transaction, stop the internal state machine and assert time-out interrupt. MCU shall handle this case appropriately and reset the master and FIFO address before reissuing the transaction again. If this option is disabled, the HW timer will not count and nor expire forever. 0: Disable 1: Enable |
| 7 | RESET_BUS _BUSY_EN | SPARE | Reserved |
| 6 | TRANSFER_ LEN_CHAN GE | TRANSFER_LEN_ CHANGE | Specifies whether or not to change the transfer length after the fist transfer is completed If enabled, the transfers after the first transfer will use the transfer_len_aux parameter. 0: Disable 1: Enable |
| 5 | ACKERR_D ET_EN | ACKERR_DET_EN | Enables slave ACK error detection When enabled, if slave ACK error is detected, the master shall terminate the transaction by issuing a STOP condition and then assert the ACKERR interrupt. MCU handles this case appropriately then resets the FIFO address before reissuing transaction. If this option is disabled, the controller will ignore slave ACK error and keep on scheduled transaction. 0: Disable 1: Enable |
| 4 | DIR_CHANG E | DIR_CHANGE | Combined transfer format, where the direction of transfer is to be changed from write to read after the FIRST RS condition Note: When set to 1, the transfers after the direction change will be based on the transfer_len_aux parameter. 0: Disable 1: Enable |
| 3 | CLK_EXT_E | CLK_EXT_EN | I2C spec allows slaves to hold the SCL line low if it is not yet |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  | N |  | ready for further processing Therefore, if this bit is set to 1, the master controller will enter a high wait state until the slave releases the SCL line. |
| 1 | RS_STOP | RS_STOP | In LS/FS mode, this bit affects multi-transfer transaction only. It controls whether or not the REPEATED-START condition is used between transfers. The last ending transfer always ends with a STOP. In HS mode, this bit must be set to 1. 0: Use STOP 1: Use REPEATED-START |

A0120014	TRANSFER_LE N

Transfer Length Register (Number of Bytes per Transfer)

0001

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | TRANSFER_LEN_AUX |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 3:0 | TRANSFER_ LEN_AUX | TRANSFER_LEN | Indicates the number of data bytes to be transferred in 1 transfer unit (excluding slave address byte) Note: The value must be set to be bigger than 1; otherwise no transfer will take place. |

A0120018	TRANSAC_LEN Transaction Length Register (Number of Transfers per

Transaction)

0001

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | TRANSAC_LEN |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7:0 | TRANSAC_ LEN | TRANSAC_LEN | Indicates the number of transfers to be transferred in 1 transaction Note: The value must be set to be bigger than 1; otherwise no transfer will take place. |

A012001C	DELAY_LEN	Inter Delay Length Register	0002

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | DELAY_LEN |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7:0 | DELAY_LEN | DELAY_LEN | Sets up wait delay between consecutive transfers when RS_STOP bit is set to 0 Unit: Half the pulse width |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A0120020	TIMING	Timing Control Register	1303

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | DATA _REA D_AD J | DATA_READ_TIME |  |  |  | SAMPLE_CNT_DIV |  |  |  |  | STEP_CNT_DIV |  |  |  |  |  |
| Type | RW | RW |  |  |  | RW |  |  |  |  | RW |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 1 |  | 0 | 1 | 1 |  |  | 0 | 0 | 0 | 0 | 1 | 1 |

Overview: LS/FS only. This register is used to control the output waveform timing. Each half pulse width, i.e. each high or low pulse, is equal to (step_cnt_div+1)*(sample_cnt_div + 1)/13MHz

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | DATA_REA D_ADJ | DATA_READ_ADJ | When set to 1, data latch in sampling time during master reads are adjusted according to the DATA_READ_TIME value. Otherwise, by default, the data are latched in at half of the high pulse width point. This value must be set to be smaller than or equal to half the high pulse width. |
| 14:12 | DATA_REA D_TIME | DATA_READ_TIME | This value is valid only when DATA_READ_ADJ is set to 1. This can be used to adjust so that the data are latched in at earlier sampling points (assuming data are settled by then). |
| 10:8 | SAMPLE_C NT_DIV | SAMPLE_CNT_DIV | Used for LS/FS only. This adjusts the width of each sample. Sample width = (sample_cnt_div +1)/13MHz |
| 5:0 | STEP_CNT_ DIV | STEP_CNT_DIV | Specifies the number of samples per half pulse width, i.e. each high or low pulse |

A0120024	START	Start Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | STAR T |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 0 | START | START | Starts the transaction on the bus It is auto de-asserted at the end of the transaction. |

A0120030	FIFO_STAT	FIFO Status Register	0001

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | RD_ADDR |  |  |  | WR_ADDR |  |  |  | FIFO_OFFSET |  |  |  |  |  | WR_F ULL | RD_E MPTY |
| Type | RO |  |  |  | RU |  |  |  | RU |  |  |  |  |  | RO | RO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |  | 0 | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:12 | RD_ADDR | RD_ADDR | Current RD address pointer |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | Only bit [2:0] have physical meanings. |
| 11:8 | WR_ADDR | WR_ADDR | Current WR address pointer Only bit [2:0] have physical meanings. |
| 7:4 | FIFO_OFFS ET | FIFO_OFFSET | wr_addr[3:0] - rd_addr[3:0] |
| 1 | WR_FULL | WR_FULL | Indicates FIFO is full |
| 0 | RD_EMPTY | RD_EMPTY | Indicates FIFO is empty |

A0120038	FIFO_ADDR_CLR FIFO Address Clear Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FIFO_ ADDR _CLR |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | WO |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 0 | FIFO_ADDR _CLR | FIFO_ADDR_CLR | When written with 1'b1, a 1 pulse fifo_addr_clr is generated to clear the FIFO address back to 0. |

A0120040	IO_CONFIG	IO Config Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | IDLE_ OE_E N |  | SDA_I O_CO NFIG | SCL_I O_CO NFIG |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW |  | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  | 0 | 0 |

Overview: This register is used to configure the I/O for the SDA and SCL lines to select between normal I/O mode or open-drain mode to support wired-and bus.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 3 | IDLE_OE_E N | IDLE_OE_EN | 0: Does not drive bus in idle state 1: Drive bus in idle state |
| 1 | SDA_IO_CO NFIG | SDA_IO_CONFIG | 0: Normal tristate I/O mode 1: Open-drain mode |
| 0 | SCL_IO_CO NFIG | SCL_IO_CONFIG | 0: Normal tristate I/O mode 1: Open-drain mode |

A0120048	HS	High Speed Mode Register	0102

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | HS_SAMPLE_CNT_ DIV |  |  |  | HS_STEP_CNT_DIV |  |  |  | MASTER_CODE |  |  |  |  | HS_N ACKE RR_D ET_E N | HS_E N |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Type |  | RW |  |  |  | RW |  |  |  | RW |  |  |  |  | RW | RW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 1 |  | 0 | 0 | 0 |  |  | 1 | 0 |

Overview: This register contains options for supporting high speed operation features Each HS half pulse width,

i.e. each high or low pulse, is equal to (step_cnt_div+1)*(sample_cnt_div + 1)/13MHz.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 14:12 | HS_SAMPL E_CNT_DIV | HS_SAMPLE_CNT _DIV | When the high-speed mode is entered after the master code transfer is completed, the sample width will become dependent on this parameter. |
| 10:8 | HS_STEP_C NT_DIV | HS_STEP_CNT_DI V | When the high-speed mode is entered after the master code transfer is completed, the number of samples per half pulse width will become dependent on this value. |
| 6:4 | MASTER_C ODE | MASTER_CODE | This is the 3 bit programmable value for the master code to be transmitted. |
| 1 | HS_NACKE RR_DET_EN | HS_NACKERR_DE T_EN | Enables NACKERR detection during the master code transmission When enabled, if NACK is not received after the master code is transmitted, the transaction will be terminated with a STOP condition. |
| 0 | HS_EN | HS_EN | Enables high-speed transaction Note: rs_stop must be set to 1. |

A0120050	SOFTRESET	Soft Reset Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SOFT _RES ET |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | WO |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 0 | SOFT_RESE T | SOFT_RESET | When written with 1'b1, a 1 pulse soft reset is used as synchronous reset to reset the I2C internal hardware circuits. |

A0120060	SPARE	SPARE	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | SPARE |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 3:0 | SPARE | SPARE | Reserved for future use |

A0120064	DEBUGSTAT	Debug Status Register	0020

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | BUS_ BUSY | MAST ER_W | MAST ER_R | MASTER_STATE |  |  |  |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

|  |  |  |  |  |  |  |  |  |  | RITE | EAD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type |  |  |  |  |  |  |  |  | RO | RO | RO | RO |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7 | BUS_BUSY | SPARE | Reserved |
| 6 | MASTER_W RITE | MASTER_WRITE | For debugging only 1: Current transfer is in the master write dir. |
| 5 | MASTER_R EAD | MASTER_READ | For debugging only 1: Current transfer is in the master read dir. |
| 4:0 | MASTER_ST ATE | MASTER_STATE | (For debugging only) Reads back the current master_state. 0: Idle state 1: I2c master is preparing to send out the start bit, SCL=1, SDA=1. 2: I2C master is sending out the start bit, SCL=1, SDA=0. 3: I2C master/slave is preparing to transmit data bit, SCL=0, SDA=data bit. (Data bit can be changed when SCL=0.) 4: I2C master/slave is transmitting data bit, SCL=1, SDA=data bit. (Data bit is stable when SCL=1.) 5: I2C master/slave is preparing to transmit the ACK bit, SCL=0, SDA=ack. (The ACK bit can be changed when SCL=0.) 6: I2C master/slave is transmitting the ACK bit, SCL=1, SDA=0. (The ACK bit is stable when SCL=1.) 7: I2C master is preparing to send out stop bit or repeated-start bit, SCL=0, SDA=0/1. (0: Stop bit; 1: Repeated-start bit) 8: I2C master is sending out stop bit or repeated-start bit, SCL=1, SDA=1/0. (0: Repeated-start bit; 1: Stop bit) 9: I2C master is in delay start between two transfers, SCL=1, SDA=1. 10: I2C master is in FIFO wait state; For writing transaction, it means FIFO is empty and I2C master is waiting for DMA controller to write data into FIFo. For reading transaction, it means FIFO is full and I2C master is waiting for DMA controller to read data from FIFOo, SCL=0, SDA=don't care. 12: I2C master is preparing to send out data bit of master code. This state is used only in high-speed transaction, SCL=0, SDA=data bit of master code. (Data bit of master code can be changed when SCL=0.) 13: I2C master is sending out data bit of master code. This state is used only in high-speed transaction, SCL=1, SDA=data bit of master code. (Data bit of master code is stable when SCL=1.) 14: I2C master/slave is preparing to transmit the NACK bit, SCL=0, SDA=nack bit. (The NACK bit can be changed when SCL=0.) This state is used only in high-speed transaction. 15: I2C master/slave is transmitting the NACK bit, SCL=1, SDA=1. This state is used only in high-speed transaction. |

A0120068	DEBUGCTRL	Debug Control Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  | APB_ DEBU G_RD | FIFO_ APB_ DEBU G |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | WO | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 1 | APB_DEBU G_RD | APB_DEBUG_RD | Only valid when fifo_apb_debug is set to 1 Writing to this register will generate a 1 pulsed FIFO APB RD signal for reading the FIFO data. |
| 0 | FIFO_APB_ DEBUG | FIFO_APB_DEBUG | Used for trace 32 debugging When using trace 32, and the memory map is shown, turning this bit on will block the normal APB read access. The APB read access to the FIFO will then be enabled by writing to apb_debug_rd. 0: Disable 1: Enable |

A012006C	TRANSFER_LE

N_AUX

Transfer Length Register (Number of Bytes per Transfer)

0001

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | TRANSFER_LEN |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 3:0 | TRANSFER_ LEN | TRANSFER_LEN_ AUX | Only valid when dir_change or transfer_len_change is set to 1. Indicates the number of data bytes to be transferred in 1 transfer unit (excluding slave address byte) for the transfers following the direction change or transfer_len_change If dir_change =1, the first write transfer length will depend on transfer_len, while the second read transfer length will depend on transfer_len_aux. Dir change is always after the first transfer. Similarly, transfer length change is always after the first transfer. Note: The value must be set to be bigger than 1; otherwise no transfer will take place. |

A0120074	TIMEOUT	Timeout Timing Register	FFFF

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | TIMEOUT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:0 |  | TIMEOUT | Indicates the number of steps to count before time-out The time-out counter counts only when the time-out mechanism is enabled and has started a transaction. |

### I2C/SCCB Controller (I2C_SCCB_Controller_V18)

##### General Description

I2C (Inter-IC)/SCCB (Serial Camera Control Bus) is a two-wire serial interface. The two signals are SCL and SDA.	SCL is a clock signal that is driven by the master. SDA is a bi-directional data signal

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

that can be driven by either the master or the slave. This generic controller supports the master role and conforms to the I2C specification.

###### Feature

- I2C compliant master mode operation

- Adjustable clock speed for LS/FS mode operation

- Supports 7-bit/10-bit addressing

- Supports high-speed mode

- Supports slave clock extension

- START/STOP/REPEATED START condition

- Manual transfer mode

- Multi-write per transfer (up to 8 data bytes for non-DMA mode)

- Multi-read per transfer (up to 8 data bytes for non-DMA mode)

- Multi-transfer per transaction

- Combined format transfer with length change capability

- Active drive/wired-and I/O configuration

###### Manual Transfer Mode

The controller offers one transfer mode, the manual mode.

When the manual mode is selected, in addition to the slave address register, the controller has a

built-in 8byte deep FIFO which allows MCU to prepare up to 8 bytes of data for a write transfer, or read up to 8 bytes of data for a read transfer.

###### Transfer Format Support

This controller has been designed to be as generic as possible in order to support a wide range of devices that may utilize different combinations of transfer formats. Here are the transfer format types that can be supported through different software configuration:

Wording convention note

- Transfer = Anything encapsulated within a Start and Stop or Repeated Start.

- Transfer length = Number of bytes within the transfer

- Transaction = This is the top unit. Everything combined equals 1 transaction.

- Transaction length = Number of transfers to be conducted.

Master to slave dir  Slave to master dir

Single-byte access

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Single Byte Write

S

A

Slave Address

A

DATA

P

Single Byte Read

S

A

Slave Address

nA

DATA

P

Multi-byte access

S

A

Slave Address

P

Multi Byte Write

N bytes + ack

DATA

A

Multi Byte Read

S

A

Slave Address

A/

nA

DATA

P

N bytes + ack/nak

Multi-byte transfer + multi-transfer (same direction)

Multi Byte Write + Multi Transfer

Slave

Address

A

DATA	A

+ wait time +

N bytes + ack/nak

X transfers

P

S

Multi Byte Read + Multi Transfer

| DATA | A/ |
| --- | --- |
|  | nA |

+ wait time +

N bytes + ack/nak

X transfers

A

Slave

Address

P

S

Multi-byte transfer + multi-transfer w RS (same direction)

Multi Byte Write + Multi Transfer + Repeated Start

Slave

Address

A

DATA	A

N bytes + ack/nak

X transfers

R

P

S

+

N bytes + ack/nak

X transfers

A

Slave Address

R

Multi Byte Read + Multi Transfer + Repeated Start

P

S

| DATA | A/ |
| --- | --- |
|  | nA |

+	+

Combined write/read with Repeated Start (direction change)

Note: Only supports write and then read sequence. Read and then write is not supported.

Combined Multi Byte Write + Multi Byte Read

S

A

Slave Address

A

DATA

R

A

Slave Address

A

DATA

P

N bytes + ack/nak	M bytes + ack/nak

##### Programming Examples

Common transfer programmable parameters

Programmable Parameters

slave_addr

rs_stop

slave_addr + dir change

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

S

Slave

Address

A

P/

RS

transfer_len

P/

RS

transfer_len / aux transfer_len

| DATA | A | DATA | A |
| --- | --- | --- | --- |

| S | Slave Address | A |
| --- | --- | --- |

| DATA | A | DATA | A |
| --- | --- | --- | --- |

delay_len

transac_len

Output waveform timing programmable parameters

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

##### Register Definition

Module name: I2C_SCCB_Controller_V18 base address: (+A02A0000h)

| Address | Name | Width | Register function |
| --- | --- | --- | --- |
| A02A0000 | DATA_PORT | 16 | Data port register |
| A02A0004 | SLAVE_ADDR | 16 | Slave address register |
| A02A0008 | INTR_MASK | 16 | Interrupt mask register This register provides masks for the corresponding interrupt sources as indicated in the intr_stat register. 1 = Allow interrupt 0 = Disable interrupt Note: While disabled, the corresponding interrupt will not be asserted, however intr_stat will still be updated with the status, i.e. mask does not affect intr_stat register values. |
| A02A000C | INTR_STAT | 16 | Interrupt status register When an interrupt is issued by the I2C controller, this register will need to be read by MCU to determine the cause for the interrupt. After this status has been read and appropriate actions are taken, the corresponding interrupt source will need to be written 1 to clear. |
| A02A0010 | CONTROL | 16 | Control register |
| A02A0014 | TRANSFER_LEN | 16 | Transfer length register (number of bytes per transfer) |
| A02A0018 | TRANSAC_LEN | 16 | Transaction length register (number of transfers per transaction) |
| A02A001C | DELAY_LEN | 16 | Inter delay length register |
| A02A0020 | TIMING | 16 | Timing control register LS/FS only. This register is used to control the output waveform timing. Each half pulse width, i.e. each high or low pulse, is equal to (step_cnt_div+1)*(sample_cnt_div + 1)/13MHz |
| A02A0024 | START | 16 | Start register |
| A02A0030 | FIFO_STAT | 16 | FIFO status register |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Address | Name | Width | Register function |
| --- | --- | --- | --- |
| A02A0038 | FIFO_ADDR_CLR | 16 | FIFO address clear register |
| A02A0040 | IO_CONFIG | 16 | IO config register This register is used to configure the I/O for the SDA and SCL lines to select between normal I/O mode, or open-drain mode to support wired-and bus. |
| A02A0048 | HS | 16 | High speed mode register This register contains options for supporting high speed operation features. Each HS half pulse width, i.e. each high or low pulse, is equal to (step_cnt_div+1)*(sample_cnt_div + 1)/13MHz |
| A02A0050 | SOFTRESET | 16 | Soft reset register |
| A02A0060 | SPARE | 16 | SPARE |
| A02A0064 | DEBUGSTAT | 16 | Debug status register |
| A02A0068 | DEBUGCTRL | 16 | Debug control register |
| A02A006C | TRANSFER_LEN_A UX | 16 | Transfer length register (number of bytes per transfer) |
| A02A0074 | TIMEOUT | 16 | Timeout timing register |

A02A0000	DATA_PORT	Data Port Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | DATA_PORT |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7:0 | DATA_PORT | DATA_PORT | FIFO access port During master write sequences (slave_addr[0] = 0), this port can be written by APB, and during master read sequences (slave_addr[0] = 1), this port can be read by APB. Note: Slave_addr must be set correctly before accessing FIFO. For debugging only: If the fifo_apb_debug bit is set, FIFO can be read and written by the APB. |

A02A0004	SLAVE_ADDR	Slave Address Register	00BF

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | SLAVE_ADDR |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7:0 | SLAVE_ADD R | SLAVE_ADDR | Specifies the slave address of the device to be accessed Bit 0 is defined by the I2C protocol as a bit that indicates the direction of transfer. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: Master write 1: Master read |

A02A0008	INTR_MASK	Interrupt Mask Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  | MASK _TIME OUT |  | MASK _HS_N ACKE RR | MASK _ACK ERR | MASK _TRA NSAC _COM P |
| Type |  |  |  |  |  |  |  |  |  |  |  | RW |  | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  | 0 |  | 0 | 0 | 0 |

Overview: This register provides masks for the corresponding interrupt sources as indicated in the intr_stat register. 1 = allow interrupt; 0 = disable interrupt Note that while disabled, the corresponding interrupt will not be asserted. However, intr_stat will still be updated with the status, i.e. mask does not affect intr_stat register values.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 4 | MASK_TIME OUT | MASK_TIMEOUT | Setting this value to 0 will mask TIMEOUT interrupt signal. |
| 2 | MASK_HS_ NACKERR | MASK_HS_NACKE RR | Setting this value to 0 will mask HS_NACKERR interrupt signal. |
| 1 | MASK_ACK ERR | MASK_ACKERR | Setting this value to 0 will mask ACK_ERR interrupt signal. |
| 0 | MASK_TRA NSAC_COM P | MASK_TRANSAC_ COMP | Setting this value to 0 will mask TRANSAC_COMP interrupt signal. |

A02A000C	INTR_STAT	Interrupt Status Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  | TIMEO UT | ARB_ LOST | HS_N ACKE RR | ACKE RR | TRAN SAC_ COMP |
| Type |  |  |  |  |  |  |  |  |  |  |  | W1C | W1C | W1C | W1C | W1C |
| Reset |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 |

Overview: When an interrupt is issued by the I2C controller, this register will need to be read by MCU to determine the cause for the interrupt. After this status has been read and appropriate actions are taken, the corresponding interrupt source will need to be written 1 to clear.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 4 | TIMEOUT | TIMEOUT_IRQ | This status is asserted if time-out is enabled and the timer expires. The internal master state machine will stop, and MCU will need to manually clear the state machine by either issuing software reset by disabling the transact_en bit. Time-out can be used to detect PCB or I2C malfunction. |
| 3 | ARB_LOST | SPARE | Reserved |
| 2 | HS_NACKE | HS_NACKERR | This status is asserted if HS master code NACK error detection is |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  | RR |  | enabled. If enabled, HS master code NACK err will cause transaction to end, and stop will be issued. |
| 1 | ACKERR | ACKERR | This status is asserted if ACK error detection is enabled. If enabled, ACKERR will cause transaction to end, and stop will be issued. |
| 0 | TRANSAC_ COMP | TRANSAC_COMP | This status is asserted when a transaction is completed successfully. |

A02A0010	CONTROL	Control Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  | TIMEO UT_E N | RESE T_BU S_BU SY_E N | TRAN SFER _LEN_ CHAN GE | ACKE RR_D ET_E N | DIR_C HANG E | CLK_ EXT_E N |  | RS_S TOP |  |
| Type |  |  |  |  |  |  |  | RW | RW | RW | RW | RW | RW |  | RW |  |
| Reset |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 8 | TIMEOUT_E N | TIMEOUT_EN | Enables time-out mechanism When enabled, if SCL stays at 0 for too long period of time due to I2C slave holds SCL at 0 for too long or SCL stucks at 0 due to PCB issue, the master shall terminate the transaction, stop the internal state machine and assert time-out interrupt. MCU shall handle this case appropriately and reset the master and FIFO address before reissuing the transaction again. If this option is disabled, the HW timer will not count and nor expire forever. 0: Disable 1: Enable |
| 7 | RESET_BUS _BUSY_EN | SPARE | Reserved |
| 6 | TRANSFER_ LEN_CHAN GE | TRANSFER_LEN_ CHANGE | Specifies whether or not to change the transfer length after the fist transfer is completed If enabled, the transfers after the first transfer will use the transfer_len_aux parameter. 0: Disable 1: Enable |
| 5 | ACKERR_D ET_EN | ACKERR_DET_EN | Enables slave ACK error detection When enabled, if slave ACK error is detected, the master shall terminate the transaction by issuing a STOP condition and then assert the ACKERR interrupt. MCU handles this case appropriately then resets the FIFO address before reissuing transaction. If this option is disabled, the controller will ignore slave ACK error and keep on scheduled transaction. 0: Disable 1: Enable |
| 4 | DIR_CHANG E | DIR_CHANGE | Combined transfer format, where the direction of transfer is to be changed from write to read after the FIRST RS condition Note: When set to 1, the transfers after the direction change will be based on the transfer_len_aux parameter. 0: Disable 1: Enable |
| 3 | CLK_EXT_E | CLK_EXT_EN | I2C spec allows slaves to hold the SCL line low if it is not yet |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  | N |  | ready for further processing Therefore, if this bit is set to 1, the master controller will enter a high wait state until the slave releases the SCL line. |
| 1 | RS_STOP | RS_STOP | In LS/FS mode, this bit affects multi-transfer transaction only. It controls whether or not the REPEATED-START condition is used between transfers. The last ending transfer always ends with a STOP. In HS mode, this bit must be set to 1. 0: Use STOP 1: Use REPEATED-START |

A02A0014	TRANSFER_LE N

Transfer Length Register (Number of Bytes per Transfer)

0001

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | TRANSFER_LEN_AUX |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 3:0 | TRANSFER_ LEN_AUX | TRANSFER_LEN | Indicates the number of data bytes to be transferred in 1 transfer unit (excluding slave address byte) Note: The value must be set to be bigger than 1; otherwise no transfer will take place. |

A02A0018	TRANSAC_LEN Transaction Length Register (Number of Transfers per

Transaction)

0001

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | TRANSAC_LEN |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7:0 | TRANSAC_ LEN | TRANSAC_LEN | Indicates the number of transfers to be transferred in 1 transaction Note: The value must be set to be bigger than 1; otherwise no transfer will take place. |

A02A001C	DELAY_LEN	Inter Delay Length Register	0002

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | DELAY_LEN |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7:0 | DELAY_LEN | DELAY_LEN | Sets up wait delay between consecutive transfers when RS_STOP bit is set to 0 Unit: Half the pulse width |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A02A0020	TIMING	Timing Control Register	1303

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | DATA _REA D_AD J | DATA_READ_TIME |  |  |  | SAMPLE_CNT_DIV |  |  |  |  | STEP_CNT_DIV |  |  |  |  |  |
| Type | RW | RW |  |  |  | RW |  |  |  |  | RW |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 1 |  | 0 | 1 | 1 |  |  | 0 | 0 | 0 | 0 | 1 | 1 |

Overview: LS/FS only. This register is used to control the output waveform timing. Each half pulse width, i.e. each high or low pulse, is equal to (step_cnt_div+1)*(sample_cnt_div + 1)/13MHz

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | DATA_REA D_ADJ | DATA_READ_ADJ | When set to 1, data latch in sampling time during master reads are adjusted according to the DATA_READ_TIME value. Otherwise, by default, the data are latched in at half of the high pulse width point. This value must be set to be smaller than or equal to half the high pulse width. |
| 14:12 | DATA_REA D_TIME | DATA_READ_TIME | This value is valid only when DATA_READ_ADJ is set to 1. This can be used to adjust so that the data are latched in at earlier sampling points (assuming data are settled by then). |
| 10:8 | SAMPLE_C NT_DIV | SAMPLE_CNT_DIV | Used for LS/FS only. This adjusts the width of each sample. Sample width = (sample_cnt_div +1)/13MHz |
| 5:0 | STEP_CNT_ DIV | STEP_CNT_DIV | Specifies the number of samples per half pulse width, i.e. each high or low pulse |

A02A0024	START	Start Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | STAR T |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 0 | START | START | Starts the transaction on the bus It is auto de-asserted at the end of the transaction. |

A02A0030	FIFO_STAT	FIFO Status Register	0001

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | RD_ADDR |  |  |  | WR_ADDR |  |  |  | FIFO_OFFSET |  |  |  |  |  | WR_F ULL | RD_E MPTY |
| Type | RO |  |  |  | RU |  |  |  | RU |  |  |  |  |  | RO | RO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |  | 0 | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:12 | RD_ADDR | RD_ADDR | Current RD address pointer |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | Only bit [2:0] have physical meanings. |
| 11:8 | WR_ADDR | WR_ADDR | Current WR address pointer Only bit [2:0] have physical meanings. |
| 7:4 | FIFO_OFFS ET | FIFO_OFFSET | wr_addr[3:0] - rd_addr[3:0] |
| 1 | WR_FULL | WR_FULL | Indicates FIFO is full |
| 0 | RD_EMPTY | RD_EMPTY | Indicates FIFO is empty |

A02A0038	FIFO_ADDR_CLR FIFO Address Clear Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FIFO_ ADDR _CLR |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | WO |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 0 | FIFO_ADDR _CLR | FIFO_ADDR_CLR | When written with 1'b1, a 1 pulse fifo_addr_clr is generated to clear the FIFO address back to 0. |

A02A0040	IO_CONFIG	IO Config Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | IDLE_ OE_E N |  | SDA_I O_CO NFIG | SCL_I O_CO NFIG |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW |  | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  | 0 | 0 |

Overview: This register is used to configure the I/O for the SDA and SCL lines to select between normal I/O mode or open-drain mode to support wired-and bus.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 3 | IDLE_OE_E N | IDLE_OE_EN | 0: Does not drive bus in idle state 1: Drive bus in idle state |
| 1 | SDA_IO_CO NFIG | SDA_IO_CONFIG | 0: Normal tristate I/O mode 1: Open-drain mode |
| 0 | SCL_IO_CO NFIG | SCL_IO_CONFIG | 0: Normal tristate I/O mode 1: Open-drain mode |

A02A0048	HS	High Speed Mode Register	0102

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | HS_SAMPLE_CNT_ DIV |  |  |  | HS_STEP_CNT_DIV |  |  |  | MASTER_CODE |  |  |  |  | HS_N ACKE RR_D ET_E N | HS_E N |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Type |  | RW |  |  |  | RW |  |  |  | RW |  |  |  |  | RW | RW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reset |  | 0 | 0 | 0 |  | 0 | 0 | 1 |  | 0 | 0 | 0 |  |  | 1 | 0 |

Overview: This register contains options for supporting high speed operation features Each HS half pulse width,

i.e. each high or low pulse, is equal to (step_cnt_div+1)*(sample_cnt_div + 1)/13MHz.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 14:12 | HS_SAMPL E_CNT_DIV | HS_SAMPLE_CNT _DIV | When the high-speed mode is entered after the master code transfer is completed, the sample width will become dependent on this parameter. |
| 10:8 | HS_STEP_C NT_DIV | HS_STEP_CNT_DI V | When the high-speed mode is entered after the master code transfer is completed, the number of samples per half pulse width will become dependent on this value. |
| 6:4 | MASTER_C ODE | MASTER_CODE | This is the 3 bit programmable value for the master code to be transmitted. |
| 1 | HS_NACKE RR_DET_EN | HS_NACKERR_DE T_EN | Enables NACKERR detection during the master code transmission When enabled, if NACK is not received after the master code is transmitted, the transaction will be terminated with a STOP condition. |
| 0 | HS_EN | HS_EN | Enables high-speed transaction Note: rs_stop must be set to 1. |

A02A0050	SOFTRESET	Soft Reset Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SOFT _RES ET |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | WO |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 0 | SOFT_RESE T | SOFT_RESET | When written with 1'b1, a 1 pulse soft reset is used as synchronous reset to reset the I2C internal hardware circuits. |

A02A0060	SPARE	SPARE	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | SPARE |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 3:0 | SPARE | SPARE | Reserved for future use |

A02A0064	DEBUGSTAT	Debug Status Register	0020

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | BUS_ BUSY | MAST ER_W | MAST ER_R | MASTER_STATE |  |  |  |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

|  |  |  |  |  |  |  |  |  |  | RITE | EAD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type |  |  |  |  |  |  |  |  | RO | RO | RO | RO |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7 | BUS_BUSY | SPARE | Reserved |
| 6 | MASTER_W RITE | MASTER_WRITE | For debugging only 1: Current transfer is in the master write dir. |
| 5 | MASTER_R EAD | MASTER_READ | For debugging only 1: Current transfer is in the master read dir. |
| 4:0 | MASTER_ST ATE | MASTER_STATE | (For debugging only) Reads back the current master_state. 0: Idle state 1: I2c master is preparing to send out the start bit, SCL=1, SDA=1. 2: I2C master is sending out the start bit, SCL=1, SDA=0. 3: I2C master/slave is preparing to transmit data bit, SCL=0, SDA=data bit. (Data bit can be changed when SCL=0.) 4: I2C master/slave is transmitting data bit, SCL=1, SDA=data bit. (Data bit is stable when SCL=1.) 5: I2C master/slave is preparing to transmit the ACK bit, SCL=0, SDA=ack. (The ACK bit can be changed when SCL=0.) 6: I2C master/slave is transmitting the ACK bit, SCL=1, SDA=0. (The ACK bit is stable when SCL=1.) 7: I2C master is preparing to send out stop bit or repeated-start bit, SCL=0, SDA=0/1. (0: Stop bit; 1: Repeated-start bit) 8: I2C master is sending out stop bit or repeated-start bit, SCL=1, SDA=1/0. (0: Repeated-start bit; 1: Stop bit) 9: I2C master is in delay start between two transfers, SCL=1, SDA=1. 10: I2C master is in FIFO wait state; For writing transaction, it means FIFO is empty and I2C master is waiting for DMA controller to write data into FIFo. For reading transaction, it means FIFO is full and I2C master is waiting for DMA controller to read data from FIFOo, SCL=0, SDA=don't care. 12: I2C master is preparing to send out data bit of master code. This state is used only in high-speed transaction, SCL=0, SDA=data bit of master code. (Data bit of master code can be changed when SCL=0.) 13: I2C master is sending out data bit of master code. This state is used only in high-speed transaction, SCL=1, SDA=data bit of master code. (Data bit of master code is stable when SCL=1.) 14: I2C master/slave is preparing to transmit the NACK bit, SCL=0, SDA=nack bit. (The NACK bit can be changed when SCL=0.) This state is used only in high-speed transaction. 15: I2C master/slave is transmitting the NACK bit, SCL=1, SDA=1. This state is used only in high-speed transaction. |

A02A0068	DEBUGCTRL	Debug Control Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  | APB_ DEBU G_RD | FIFO_ APB_ DEBU G |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | WO | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 1 | APB_DEBU G_RD | APB_DEBUG_RD | Only valid when fifo_apb_debug is set to 1 Writing to this register will generate a 1 pulsed FIFO APB RD signal for reading the FIFO data. |
| 0 | FIFO_APB_ DEBUG | FIFO_APB_DEBUG | Used for trace 32 debugging When using trace 32, and the memory map is shown, turning this bit on will block the normal APB read access. The APB read access to the FIFO will then be enabled by writing to apb_debug_rd. 0: Disable 1: Enable |

A02A006C	TRANSFER_LE

N_AUX

Transfer Length Register (Number of Bytes per Transfer)

0001

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | TRANSFER_LEN |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 3:0 | TRANSFER_ LEN | TRANSFER_LEN_ AUX | Only valid when dir_change or transfer_len_change is set to 1. Indicates the number of data bytes to be transferred in 1 transfer unit (excluding slave address byte) for the transfers following the direction change or transfer_len_change If dir_change =1, the first write transfer length will depend on transfer_len, while the second read transfer length will depend on transfer_len_aux. Dir change is always after the first transfer. Similarly, transfer length change is always after the first transfer. Note: The value must be set to be bigger than 1; otherwise no transfer will take place. |

A02A0074	TIMEOUT	Timeout Timing Register	FFFF

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | TIMEOUT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:0 |  | TIMEOUT | Indicates the number of steps to count before time-out The time-out counter counts only when the time-out mechanism is enabled and has started a transaction. |

### Real Time Clock

##### General Descriptions

The Real-Time Clock (RTC) module provides time and data information, as well as 32.768kHz clock. By configuring pin XOSC32_ENB, the use of the 32k crystal can be determined, i.e. using a 32k crystal,

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

or not to use a 32k crystal. The RTC block has an independent power supply. When the mobile handset is powered off, a dedicated regulator supplies the RTC block. If the main battery is not present, a backup supply such as a small mercury cell battery or a large capacitor is used. In addition to providing timing data, an alarm interrupt is generated and can be used to power up the baseband core. Regulator interrupts corresponding to seconds, minutes, hours and days can be generated whenever the time counter value reaches a maximum value (e.g., 59 for seconds and minutes, 23 for hours, etc.). The year span is supported up to 2127. The maximum day-of-month values, which depend on the leap year condition, are stored in the RTC block.

##### Register Definition

Module name: RTC Base address: (+A0710000h)

| Address | Name | Width | Register function |
| --- | --- | --- | --- |
| A0710000 | RTC_BBPU | 16 | Baseband power up |
| A071 0004 | RTC_IRQ_STA | 16 | RTC IRQ status This register is fixed to 0 when RTC_POWERKEY1 & RTC_POWERKEY2 unmatch the correct values. |
| A071 0008 | RTC_IRQ_EN | 16 | RTC IRQ enable This register is fixed to 0 when RTC_POWERKEY1 & RTC_POWERKEY2 unmatch the correct values. |
| A071 000C | RTC_CII_EN | 16 | Counter increment IRQ enable This register activates or de-activates the IRQ generation when the TC counter reaches its maximum value. |
| A071 0010 | RTC_AL_MAS K | 16 | RTC alarm mask The alarm condition for alarm IRQ generation depends on whether or not the corresponding bit in this register is masked. Warning: If you set all bits to 1 in RTC_AL_MASK (i.e. RTC_AL_MASK=0x7f) and PWREN=1 in RTC_BBPU, it means the alarm will come every second, not disabled. |
| A071 0014 | RTC_TC_SEC | 16 | RTC seconds time counter register |
| A071 0018 | RTC_TC_MIN | 16 | RTC minutes time counter register |
| A071001C | RTC_TC_HOU | 16 | RTC hours time counter register |
| A0710020 | RTC_TC_DOM | 16 | RTC day-of-month time counter register |
| A0710024 | RTC_TC_DOW | 16 | RTC day-of-week time counter register |
| A0710028 | RTC_TC_MTH | 16 | RTC month time counter register |
| A071002C | RTC_TC_YEA | 16 | RTC year time counter register |
| A0710030 | RTC_AL_SEC | 16 | RTC second alarm setting register |
| A0710034 | RTC_AL_MIN | 16 | RTC minute alarm setting register |
| A0710038 | RTC_AL_HOU | 16 | RTC hour alarm setting register |
| A071003C | RTC_AL_DOM | 16 | RTC day-of-month alarm setting register |
| A0710040 | RTC_AL_DOW | 16 | RTC day-of-week alarm setting register |
| A0710044 | RTC_AL_MTH | 16 | RTC month alarm setting register |
| A0710048 | RTC_AL_YEA | 16 | RTC year alarm setting register |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Address | Name | Width | Register function |
| --- | --- | --- | --- |
| A071004C | RTC_OSC32C ON | 16 | OSC32 control The function is only active when RTC_POWERKEY1 & RTC_POWERKEY2 match the correct values. |
| A0710050 | RTC_POWERK EY1 | 16 | RTC_POWERKEY1 register |
| A0710054 | RTC_POWERK EY2 | 16 | RTC_POWERKEY2 register |
| A0710058 | RTC_PDN1 | 16 | PDN1 |
| A071005C | RTC_PDN2 | 16 | PDN2 |
| A0710060 | RTC_SPAR0 | 16 | Spare register for specific purpose |
| A0710064 | RTC_SPAR1 | 16 | Spare register for specific purpose |
| A0710068 | RTC_PROT | 16 | Lock/unlock scheme to prevent RTC miswriting |
| A071006C | RTC_DIFF | 16 | One-time calibration offset This register is fixed to 0 when RTC_POWERKEY1 & RTC_POWERKEY2 unmatch the correct values. |
| A0710070 | RTC_CALI | 16 | Repeat calibration offset This register is fixed to 0 when RTC_POWERKEY1 & RTC_POWERKEY2 unmatch the correct values. |
| A0710074 | RTC_WRTGR | 16 | Enable transfers from core to RTC in queue |
| A0710078 | RTC_CON | 16 | Other RTC control registers Note: LPRST and LPEN are tied to 0 internally when RTC_POWERKEY1 & RTC_POWERKEY2 do not match the correct values. After changing RTC_CON, write WRTGR = 1 to make it take effect. |

A0710000	RTC_BBPU	Baseband Power Up	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne | KEY_BBPU |  |  |  |  |  |  |  |  | CBUS Y | RELO AD | CLRP KY | AUTO | BBPU |  | PWRE N |
| Type | WO |  |  |  |  |  |  |  |  | RO | WO | WO | RW | RW |  | RW |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 |  | 0 |

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:8 | KEY_BBPU | KEY_BBPU | A bus write is acceptable only when KEY_BBPU=0x43. |
| 6 | CBUSY | CBUSY | The read/write channels between RTC/Core is busy. This bit indicates high after the software program sequence to anyone of RTC data registers and enables the transfer by RTC_WRTGR = 1. In addition, it is high after the reset from low to high due to RTC reload process. |
| 5 | RELOAD | RELOAD | Reloads the values from RTC domain to core domain Generally the RTC will reload and synchronize the data from RTC to core when being reset from 0 to 1. This bit can be treated as a debugging bit. |
| 4 | CLRPKY | CLRPKY | Clears powerkey1 and powerkey2 at the same time In some cases, the software may clear powerkey1 & |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | powerkey2. BBWAKEUP depends on the matching specific patterns of powerkey1 and powerkey2. If any one of powerkey1 or powerkey2 or BBPU is cleared, BBWAKEUP will go low immediately. The software cannot program the other control bits without power. By programming RTC_BBPU with CLRPKY = 1 and BBPU = 0 condition, RTC can clear powerkey1, powerkey2 and BBPU at the same time. |
| 3 | AUTO | AUTO | Controls if BBWAKEUP is automatically in the low state when SYSRST transitions from high to low. 0: BBWAKEUP is not automatically in the low state when SYSRST# transitions are from high to low. 1: BBWAKEUP is automatically in the low state when SYSRST# transitions are from high to low. The function is only active when RTC_POWERKEY1 & RTC_POWERKEY2 match the correct values. |
| 2 | BBPU | BBPU | Controls the power of PMU If powerkey1 = A357h and powerkey2 = 67D2h, PMU takes on the value programmed by software; otherwise PMU is low. 0: Power down 1: Power on |
| 0 | PWREN | PWREN | 0: RTC alarm has no action on power switch. 1: When an RTC alarm occurs, BBPU is set to 1 and the system is powered on by RTC alarm wakeup. |

A0710004	RTC_IRQ_STA	RTC IRQ Status	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  | LPSTA |  | TCSTA | ALSTA |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RC |  | RC | RC |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  | 0 | 0 |

Overview: This register is read-cleared and is fixed to 0 when RTC_POWERKEY1 and RTC_POWERKEY2 unmatch the correct values.

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 3 | LPSTA | LPSTA | Indicates the IRQ status and whether or not the LPD is asserted (LPD function is either provided by XOSC32 or EOSC326, depending on XOSC32_ENB) 0: No IRQ occurred; the 32K clock is good. 1: IRQ occurred; the 32K clock stops. This can be masked by LP_EN or cleared by initializing LPD. |
| 1 | TCSTA | TCSTA | Indicates the IRQ status and whether or not the tick condition has been met. 0: No IRQ occurred; the tick condition has not been met. 1: IRQ occurred; the tick condition has been met. |
| 0 | ALSTA | ALSTA | Indicates the IRQ status and whether or not the alarm condition has been met. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: No IRQ occurred; the alarm condition has not been met. 1: IRQ occurred; the alarm condition has been met. |

A0710008	RTC_IRQ_EN	RTC IRQ Enable	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  | LP_EN | ONES HOT | TC_EN | AL_EN |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |

Overview: This register is fixed to 0 when RTC_POWERKEY1 and RTC_POWERKEY2 unmatch the correct values.

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 3 | LP_EN | LP_EN | Enables the control bit for IRQ generation if low power detected (32k clock off). 0: Disable IRQ generations 1: Enable LPD |
| 2 | ONESHOT | ONESHOT | Controls automatic reset of AL_EN and TC_EN |
| 1 | TC_EN | TC_EN | Enables the control bit for IRQ generation if the tick condition has been met. 0: Disable IRQ generations 1: Enable the tick time match interrupt. Clear the interrupt when ONESHOT is high upon generation of the corresponding IRQ. |
| 0 | AL_EN | AL_EN | Enables the control bit for IRQ generation if the alarm condition has been met. 0: Disable IRQ generations 1: Enable the alarm time match interrupt. Clear the interrupt when ONESHOT is high upon generation of the corresponding IRQ. |

A071000C	RTC_CII_EN	Counter Increment IRQ Enable	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  | 1/8SE CCII | 1/4SE CCII | 1/2SE CCII | YEACII | MTHCI I | DOWC II | DOMCI I | HOUCI I | MINCII | SECCII |
| Type |  |  |  |  |  |  | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW |
| Reset |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: This register activates or de-activates the IRQ generation when the TC counter reaches its maximum value.

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 9 | 1/8SECCII | SECCII_1_8 | Set the bit to 1 to activate the IRQ at each 1/8 of a second update |
| 8 | 1/4SECCII | SECCII_1_4 | Set the bit to 1 to activate the IRQ at each 1/4 of a second update |
| 7 | 1/2SECCII | SECCII_1_2 | Set the bit to 1 to activate the IRQ at each 1/2 of a second |

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | update |
| 6 | YEACII | YEACII | Set the bit to 1 to activate the IRQ at each year update |
| 5 | MTHCII | MTHCII | Set the bit to 1 to activate the IRQ at each month update |
| 4 | DOWCII | DOWCII | Set the bit to 1 to activate the IRQ at each day-of-week update |
| 3 | DOMCII | DOMCII | Set the bit to 1 to activate the IRQ at each day-of-month update |
| 2 | HOUCII | HOUCII | Set the bit to 1 to activate the IRQ at each hour update |
| 1 | MINCII | MINCII | Set the bit to 1 to activate the IRQ at each minute update |
| 0 | SECCII | SECCII | Set this bit to 1 to activate the IRQ at each second update |

A0710010	RTC_AL_MAS K

RTC Alarm Mask	0000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  | YEA_ MSK | MTH_ MSK | DOW_ MSK | DOM_ MSK | HOU_ MSK | MIN_M SK | SEC_ MSK |
| Type |  |  |  |  |  |  |  |  |  | RW | RW | RW | RW | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: The alarm condition for alarm IRQ generation depends on whether or not the corresponding bit in this register is masked. Note that if all bits 1 in RTC_AL_MASK are set (i.e. RTC_AL_MASK = 0x7f) and PWREN = 1 in RTC_BBPU, it means the alarm will come every second, not disabled.

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 6 | YEA_MSK | YEA_MSK | 0: Condition (RTC_TC_YEA = RTC_AL_YEA) is checked to generate the alarm signal. 1: Condition (RTC_TC_YEA = RTC_AL_YEA) is masked, i.e. the value of RTC_TC_YEA does not affect the alarm IRQ generation. |
| 5 | MTH_MSK | MTH_MSK | 0: Condition (RTC_TC_MTH = RTC_AL_MTH) is checked to generate the alarm signal. 1: Condition (RTC_TC_MTH = RTC_AL_MTH) is masked, i.e. the value of RTC_TC_MTH does not affect the alarm IRQ generation. |
| 4 | DOW_MSK | DOW_MSK | 0: Condition (RTC_TC_DOW = RTC_AL_DOW) is checked to generate the alarm signal. 1: Condition (RTC_TC_DOW = RTC_AL_DOW) is masked, i.e. the value of RTC_TC_DOW does not affect the alarm IRQ generation. |
| 3 | DOM_MSK | DOM_MSK | 0: Condition (RTC_TC_DOM = RTC_AL_DOM) is checked to generate the alarm signal. 1: Condition (RTC_TC_DOM = RTC_AL_DOM) is masked, i.e. the value of RTC_TC_DOM does not affect the alarm IRQ generation. |
| 2 | HOU_MSK | HOU_MSK | 0: Condition (RTC_TC_HOU = RTC_AL_HOU) is checked to generate the alarm signal. 1: Condition (RTC_TC_HOU = RTC_AL_HOU) is masked, i.e. the value of RTC_TC_HOU does not affect the alarm IRQ generation. |
| 1 | MIN_MSK | MIN_MSK | 0: Condition (RTC_TC_MIN = RTC_AL_MIN) is checked to generate the alarm signal. 1: Condition (RTC_TC_MIN = RTC_AL_MIN) is masked, i.e. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | the value of RTC_TC_MIN does not affect the alarm IRQ generation. |
| 0 | SEC_MSK | SEC_MSK | 0: Condition (RTC_TC_SEC = RTC_AL_SEC) is checked to generate the alarm signal. 1: Condition (RTC_TC_SEC = RTC_AL_SEC) is masked, i.e. the value of RTC_TC_SEC does not affect the alarm IRQ generation. |

A0710014	RTC_TC_SEC	RTC Seconds Time Counter Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  | TC_SECOND |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 5:0 | TC_SECOND | TC_SECOND | Second initial value for the time counter Range: 0 ~ 59 |

A0710018	RTC_TC_MIN	RTC Minutes Time Counter Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  | TC_MINUTE |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 5:0 | TC_MINUTE | TC_MINUTE | Minute initial value for the time counter Range: 0 ~ 59 |

A071001C	RTC_TC_HOU	RTC Hours Time Counter Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  |  | TC_HOUR |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 |

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 4:0 | TC_HOUR | TC_HOUR | Hour initial value for the time counter Range: 0 ~ 23 |

A0710020	RTC_TC_DOM	RTC Day-of-month Time Counter Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Mne |  |  |  |  |  |  |  |  |  |  |  | TC_DOM |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 |

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 4:0 | TC_DOM | TC_DOM | Day-of-month initial value for the time counter The day-of-month maximum value depends on the leap year condition, i.e. 2 LSB of year time counter are zeros. |

A0710024	RTC_TC_DO

W

RTC Day-of-week Time Counter Register	0000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  | TC_DOW |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 |

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 2:0 | TC_DOW | TC_DOW | Day-of-week initial value for the time counter Range: 1 ~ 7 |

A0710028	RTC_TC_MTH	RTC Month Time Counter Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  | TC_MONTH |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 3:0 | TC_MONTH | TC_MONTH | Month initial value for the time counter Range: 1 ~ 12 |

A071002C	RTC_TC_YEA	RTC Year Time Counter Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  | TC_YEAR |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 6:0 | TC_YEAR | TC_YEAR | Year initial value for the time counter Range: 0 ~ 127 (2000-2127). The software can bias the year as multiples of 4 for the internal leap-year formula. Here are 3 examples: 2000 ~ 2127, 1972 ~ 2099 and 1904 ~ 2031. To simplify the process, the RTC hardware treats all 4-multiple as leap years. If the range you |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | define includes non-leap 4-multiple year (e.g. 2100), please adjust it to the correct date by yourselves. (e.g. change Feb. 29th, 2100 to Mar. 1st, 2100). It is suggested to bias the range to be bigger than 1900 and smaller than 2100 to evade the manual adjustment, i.e. the bias values are suggested to be in the range of [-28,-96], that are (1972~ 2099) ~ (1904~ 2031). The formal leap formula: If year modulo 400 is 0 then leap Else if year modulo 100 is 0 then no_leap Else if year modulo 4 is 0 then leap Else no_leap |

A0710030	RTC_AL_SEC	RTC Second Alarm Setting Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  | AL_SECOND |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 5:0 | AL_SECOND | AL_SECOND | Second value of the alarm counter setting Range: 0 ~ 59 |

A0710034	RTC_AL_MIN	RTC Minute Alarm Setting Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  | AL_MINUTE |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 5:0 | AL_MINUTE | AL_MINUTE | Minute value of the alarm counter setting Range: 0 ~ 59 |

A0710038	RTC_AL_HOU	RTC Hour Alarm Setting Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne | NEW_SPARE0 |  |  |  |  |  |  |  |  |  |  | AL_HOUR |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  | 0 | 0 | 0 | 0 | 0 |

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:8 | NEW_SPARE 0 | NEW_SPARE 0 | New spare-register 0 |
| 4:0 | AL_HOUR | AL_HOUR | Hour value of the alarm counter setting |

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | Range: 0 ~ 23 |

A071003C	RTC_AL_DOM	RTC Day-of-month Alarm Setting Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne | NEW_SPARE1 |  |  |  |  |  |  |  |  |  |  | AL_DOM |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  | 0 | 0 | 0 | 0 | 0 |

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:8 | NEW_SPARE 1 | NEW_SPARE 1 | New spare-register 1 |
| 4:0 | AL_DOM | AL_DOM | Day-of-month value of the alarm counter setting The day-of-month maximum value depends on the leap year condition, i.e. 2 LSB of year time counter are zeros. |

A0710040	RTC_AL_DO

W

RTC Day-of-week Alarm Setting Register	0000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne | NEW_SPARE2 |  |  |  |  |  |  |  |  |  |  |  |  | AL_DOW |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  | 0 | 0 | 0 |

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:8 | NEW_SPARE 2 | NEW_SPARE 2 | New spare-register 2 |
| 2:0 | AL_DOW | AL_DOW | Day-of-week value of the alarm counter setting Range: 1 ~ 7 |

A0710044	RTC_AL_MTH	RTC Month Alarm Setting Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne | NEW_SPARE3 |  |  |  |  |  |  |  |  |  |  |  | AL_MONTH |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |  | 0 | 0 | 0 | 0 |

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:8 | NEW_SPARE 3 | NEW_SPARE 3 | New spare-register 3. |
| 3:0 | AL_MONTH | AL_MONTH | Month value of the alarm counter setting. Range: 1 ~ 12 |

| A0710048 | RTC_AL_YEA | RTC Year Alarm Setting Register | 0000 |
| --- | --- | --- | --- |

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne | NEW_SPARE4 |  |  |  |  |  |  |  |  | AL_YEAR |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:8 | NEW_SPARE 4 | NEW_SPARE 4 | New spare-register 4 |
| 6:0 | AL_YEAR | AL_YEAR | Year value of the alarm counter setting Range: 0 ~ 127 (2000-2127) |

A071004C	RTC_OSC32C

ON

OSC32 Control	0000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne | EOSC32_RSV |  |  |  |  |  |  | EMBCK_SEL_MODE |  |  | XOSC3 2_ENB | XOSCCALI |  |  |  |  |
| Type | RW |  |  |  |  |  |  | RW |  |  | RO | RW |  |  |  |  |
| Reset | - | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | - | 0 | 1 | 1 | 1 | 1 |

Overview: The function is only active when RTC_POWERKEY1 & RTC_POWERKEY2 match the correct values.

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:9 | OSC32_RSV | OSC32_RSV | OSC32 reserved bits Keep it at 0x2. |
| 8:6 | EMB_MODE | EMBCK_SEL | Mode setting for crystal removal case |
| 5 | XOSC32_ENB | XOSC32_ENB | XOSC32_ENB Reads pin XOSC32_ENB configuration to know the 32k crystal usage. 0: Use 32k crystal 1: Does not use 32k crystal. |
| 4:0 | XOSCCALI | XOSCCALI | Controls XOSC32/EOSC32 caliration If XOSC32_ENB(RTC_OSC32CON[5]) = 0, XOSCCALI controls the bias current for 32k xtal in XOSC32. When powerkeys do not match, the default value is 0x7. If XOSC32_ENB(RTC_OSC32CON[5]) = 1, XOSCCALI is the trimming value for EOSC32. SW needs to find the best trimming value for EOSC32 when the 1st power-on by frequency meter. When powerkeys do not match, the default value is 0xf. |

OSC32CON is not protected by RTC_PROT because RTC_PROT needs 32.768kHz clock to unlock. Similarly, to modify RTC_OSC32CON, writing RTC_WRTGR is not needed, neither. To protect the OSC32 control bits, follow the update sequence to update RTC_OSC32CON. After the updating sequence is completed, reload to acquire the internal RTC_OSC32CON. This register needs to be initialized before writing powerkeys match.

Update sequence:

Step 1: Write RTC_OSC32CON = 0x1a57 and wait until CBUSY=0 Step 2: Write RTC_OSC32CON = 0x2b68 and wait until CBUSY=0

Step 3: Write the real value you would like to write RTC_OSC32CON and wait until CBUSY=0

Step 4: Return to step 1 if you need to modify RTC_OSC32CON again.

Note: RTC_OSC32CON should be set before writing POWERKEY1 and POWERKEY2 to the correct value.

A0710050	RTC_POWERKEY

1

RTC_POWERKEY1 Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne | RTC_POWERKEY1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:0 | RTC_POWER KEY1 | RTC_POWER KEY1 |  |

A0710054	RTC_POWERKE Y2

RTC_POWERKEY2 Register	0000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne | RTC_POWERKEY2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:0 | RTC_POWER KEY2 | RTC_POWER KEY2 |  |

These register sets are used to determine if the real-time clock has been programmed by the software, i.e. the time value in real time clock is correct. When the real-time clock is first powered on, the register contents are all undefined, and therefore the time values shown are incorrect. The software needs to know if the real-time clock has been programmed. Hence, the two registers are defined to solve this power-on issue. After the software programs the correct value, the two register sets will not need to be updated. In addition to programming the correct time value, when the contents of the register sets are wrong, the interrupt will not be generated. Therefore, the-real time clock will not generate the interrupts before the software programs the registers, and unwanted interrupt due to wrong time value will not occur. The correct values of these two register sets are:

RTC_POWERKEY1	A357h

RTC_POWERKEY2	67D2h

A0710058	RTC_PDN1	PDN1	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne | RTC_PDN1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:0 | RTC_PDN1 | RTC_PDN1 | Spare registers for software to keep the power-on and power-off state information |

A071005C	RTC_PDN2	PDN2	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne | RTC_PDN2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:0 | RTC_PDN2 | RTC_PDN2 | Spare registers for software to keep power-on and power-off state information |

A0710060	RTC_SPAR0	Spare Register For Specific Purpose	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne | RTC_SPAR0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:0 | RTC_SPAR0 | RTC_SPAR0 | Reserved for specific purposes |

A0710064	RTC_SPAR1	Spare Register For Specific Purpose	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne | RTC_SPAR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:0 | RTC_SPAR1 | RTC_SPAR1 | Reserved for specific purposes |

A0710068	RTC_PROT	Lock/Unlock Scheme to Prevent RTC Miswriting	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne | RTC_PROT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:0 | RTC_PROT | RTC_PROT | Protects RTC write interface by RTC_PROT Whether the RTC writing interface is enabled or not is decided by RTC_PROT contents. When RTC_POWERKEY1 & RTC_POWERKEY2 are not equal to the correct values, the RTC writing interface will always be enabled. However, when they match, the user has to perform the unlock flow to enable the writing interface. Unlock flow: Step1: *RTC_PROT=0x586a; Step2: *RTC_WRTGR=1; Step3: while(*RTC_BBPU & 0x40) {}; // Timeout period: 120usec Step4: *RTC_PROT=0x9136; Step5: *RTC_WRTGR=1; Step6: while(*RTC_BBPU & 0x40) {}; // Timeout period: 120usec Note: Always keep RTC in the unlock state in the power-on mode. Once the normal RTC content writing is completed, DO NOT modify the RTC_PROT content to lock RTC. The RTC_PROT contents will be cleared automatically when being powered off immediately. |

A071006C	RTC_DIFF	One-time Calibration Offset	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne | RESE RVED | RESE RVED |  |  | RTC_DIFF |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW | RO |  |  | RW |  |  |  |  |  |  |  |  |  |  |  |
| Reset | - | - |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: This register is fixed to 0 when RTC_POWERKEY1 and RTC_POWERKEY2 unmatch the correct values.

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 11:0 | RTC_DIFF | RTC_DIFF | Adjusts internal counter of RTC It takes effect once and returns to 0 when done. In some cases, RTC is faster or slower than the standard. To change RTC_TC_SEC being coarse may cause alarm problem. RTC_DIFF provides a finer time unit. An internal 15-bit counter accumulates in each 32,768Hz clock. Entering a non-0 value to RTC_DIFF will cause the internal RTC counter to increase or decrease RTC_DIFF when RTC_DIFF changes to 0 again. RTC_DIFF represents 2's completement. For example, if you fill in 0xfff into RTC_DIFF, the internal counter will decrease by 1 when RTC_DIFF returns to 0. In other words, you can only use RTC_DIFF continuously if RTC_DIFF is equal to 0 now. Note: RTC_DIFF ranges from 0x800 (-2048) to 0x7fd (2045). 0x7ff & 0x7fe are forbidden. |

| A0710070 | RTC_CALI | Repeat Calibration Offset | 0000 |
| --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne | RESE RVED | RESE RVED | RTC_CALI |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RO | RW | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | - | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: This register is fixed to 0 when RTC_POWERKEY1 & RTC_POWERKEY2 unmatch the correct values.

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 13:0 | RTC_CALI | RTC_CALI | Provides a repeated calibration scheme RTC_CALI provides 7-bit calibration capability in 8-second duration, i.e. 5-bit calibration capability in each second. RTC_CALI represents 2's complement form for the user to adjust RTC increase or decrease Due to RTC_CALI is revealed in 8 seconds, the resolution is less than a 1/32768 clock. Avg. resolution: 1/32768/8 = 3.81us Avg. adjust range: -0.244~0.240ms/sec in 2's complement: -0x2000 ~ 0x1fff (-8192 ~ 8191) |

A0710074	RTC_WRTGR	Enable Transfers From Core to RTC in Queue	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | WRTG R |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | WO |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 0 | WRTGR | WRTGR | Enables the transfers from core to RTC After you modify all the RTC registers and would like to change it, write 1 to RTC_WRTGR to trigger the transfer. The prior writing operation is queued in the core power domain. The pending data will not be transferred to the RTC domain until WRTGR = 1. After WRTGR=1, the pending data will be transferred to the RTC domain sequentially in order of register addresses, from low to high. For example: RTC_BBPU -> RTC_IRQ_EN -> RTC_CII_EN -> RTC_AL_MASK -> RTC_TC_SEC -> etc. CBUSY in RTC_BBPU is equal to 1 in the writing process. Observe CBUSY to determine when the transmission is completed. |

A0710078	RTC_CON	Other RTC Control Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne | LPST A_RA W | RESE RVED | RESE RVED | POW EROF F_SE Q_EN | RESE RVED | RESE RVED | RESE RVED | RESE RVED | RESE RVED | RESE RVED | RESE RVED | RESE RVED | LPRS T | LPEN |  | VBAT _LPS TA_R AW |
| Type | W1 C | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW | RW |  | RW |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Bit (s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | LPSTA_RAW | LPSTA_RAW | Raw status of LP_STA Re-initialize LPD to clear this bit. Note: This bit is always high before LPD initialization sequence after the first power-on. |
| 3 | LPRST | LPRST | Resets LPDETB Only takes effect when LPEN = 1. |
| 2:1 | LPEN | LPEN | Enables LPDETB LP initialization sequence: - Write LPEN = 1, LPRST = 0. Write RTC_WRTGR = 1. wait cbusy down. - Write LPEN = 1, LPRST = 1. Write RTC_WRTGR = 1. wait cbusy down. - Write LPEN = 1, LPRST = 0. Write RTC_WRTGR = 1. wait cbusy down. |
| 0 | VBAT_LPSTA_ RAW | VBAT_LPSTA_ RAW | Indicates the battery has been in LP state Software needs to clear this bit for the next time use 0: VBAT has not been in LP state. 1: VBAT has been in LP state. Note: VBAT LP state = VBAT < 2.5V |

When Vcore always exists, the software can trust the registers and wait for the LP interrupt from RTC if the 32.768 kHz clock stopped or has been stopped.

However, nothing can be trusted after the battery is off (Vrtc may drop). In every boot time, the software checks if LPSTA_RAW = 1. (LP_STA = LPSTA_RAW & LP_IRQ_EN) If true, the RTC contents will no longer be trusted just like powerkeys do not match. You have to initialize the RTC contents in this case.

### Auxiliary ADC Unit

The auxiliary ADC unit is used to monitor the status of the battery and charger, to identify the plugged peripheral, and to perform temperature measurement.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

ADC_SFS

Vin

APB BUS

SPLD

ADC_RDY

DAC

ADC_SDATA

F13M_EN

TDMA_AUXADCSTR

QBIT_EN

PDN_AUXADC	QBIT_EN

AUTOSET_STR IMMEDIATE_STR SYNCHRONOUS_MODE

PUWAIT_EN

AUXADC_REG

COMP

AUXADC_CORE

CLR_SYNCMODE [1:0]

SEL_LATCH [7:0] ADC_STATE

AUXADC_SIF	ADC_SCKO	SADC_SIF

ADC

ADC_LATCH

TS_CSB TS_CIN

ADC_ST

Touch Panel Signals

Latch_Data

ADC_PDN

ADC_SEL_IN

ADC_SEL_OUT

Figure 38. AUXADC architecture

Each channel operates in one of the two modes: immediate mode or timer-triggered mode. The mode of each channel can be individually selected through register AUXADC_CON0. For example, if the flag SYN0 in register AUXADC_CON0 is set, channel 0 will be set in the timer-triggered mode. Otherwise, the channel will operate in the immediate mode.

In the immediate mode, the A/D converter samples the value once only when the flag in the AUXADC_CON1 register is set. For example, if the flag IMM0 in AUXADC_CON1 is set, the A/D converter will sample the data for channel 0. The IMM flags must be cleared and set again to initialize another sampling.

The value sampled for channel 0 is stored in register AUXADC_DAT0, and the value for channel 1 is stored in register AUXADC_DAT1, and so on.

If the AUTOSET flag in register AUXADC_CON3 is set, the auto-sample function will be enabled. The A/D converter samples the data for the channel in which the corresponding data register is read. For example, in the case where the SYN1 flag is not set, the AUTOSET flag is set. When the data register AUXADC_DAT0 is read, the A/D converter will sample the next value for channel 1 immediately.

If multiple channels are selected at the same time, the task will be performed sequentially on every selected channel. For example, if AUXADC_CON1 is set to 0x3f, i.e. 6 channels are selected, the state machine in the unit will start sampling from channel 6 to channel 0 and save the values of each input channel in respective registers. The same process also applies to the timer-triggered mode.

In the timer-triggered mode, the A/D converter samples the value for the channels in which the corresponding SYN flags are set when the TDMA timer counts to the value specified in register TDMA_AUXEV1 placed in the TDMA timer. For example, if AUXADC_CON0 is set to 0x3f, the 6 channels will be selected to be in the timer-triggered mode. The state machine will sample the 6

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

channels sequentially and save the values in registers from AUXADC_DAT0 to AUXADC_DAT5, as it does in the immediate mode.

AUTOCLRn in register AUXADC_CON3 is set when it is intended to sample only once after setting up the timer-triggered mode. If the AUTOCLR1 flag is set, after the data for the channels in the

timer-triggered mode are stored, the SYNn flags in register AUXADC_CON0 will be cleared.

The uses of the immediate mode and timer-triggered mode are mutually exclusive in terms of individual channels.

There are only two external pins (channel 4 ~ 5) for voltage detection. Other channels (0 ~ 3) are for battery voltage, battery current, charger and battery temperature respectively. Channel 9 is used for audio.

Touch panel

PAD_XP

PAD_YP

S/H

PAD_XM

PAD_YM

VBAT_ADC ISENSE_ADC

CHRIN_ADC TSENSE_ADC

VRB VRT

SAR ADC

DO<11:0>

PAD_AUX_IN8~6

3

Digital C	r

ontrolle

PAD_VREF

Pen Interrupt

AVDD	AVDD

CLK DIN CSB

BUSY DOUT

Figure 39. Touch panel circuit structure

Besides the normal sampling of external input voltage, the AUXADC includes the sampling of the touch panel function. For specified axis, the software should program AUX_TS_CMD first then trigger the sample of touch panel in register AUX_TS_CON. The touch panel sampling waveform is shown as the following. After the software polls the status bit in register AUXADC_CON3 to know that the touch panel sample is finished, the software can read back the specified axis value from register AUX_TS_DAT0.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Ib0

A2

S

Ib1

PD0

PD1

SE/DF

MODE

A0

A1

CSB DCLK DIN

SPL

Busy

D4

D5

D6

D7

D8

D9

DOUT	D3

Figure 40. Touch panel sampling waveform

S: Start bit

A2 ~ A0: Addressing bits Mode: 10-bit or 8-bit

SE/DF: Single end or differential mode PD1 ~ 0: Power down command

These values are defined in register AUX_TS_CMD. The table below shows the relationship between AUX_TS_CMD and touch panel control signals.

Table 49. Relationship between commands and touch panel control signals

| A2 | A1 | A0 | SE/DFB | CHN_SEL | ADC In | X switches | Y switches | +REF | -REF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 0 | C | X+ | OFF | ON | Y+ | Y- |
| 0 | 1 | 0 | 0 | F | X- | OFF | ON | Y+ | Y- |
| 0 | 1 | 1 | 0 | C | X+ | X+ OFF X- ON | Y+ ON Y- OFF | Y+ | X- |
| 1 | 0 | 0 | 0 | E | Y- | X+ OFF X- ON | Y+ ON Y- OFF | Y+ | X- |
| 1 | 0 | 1 | 0 | D | Y+ | ON | OFF | X+ | X- |
| 1 | 1 | 0 | 0 | E | Y- | ON | OFF | X+ | X- |

Table 2. AUXADC channel description

| AuxADC Channel ID | Description |
| --- | --- |
| Channel 0 | VBAT |
| Channel 1 | ISENSE |
| Channel 2 | CHRIN |
| Channel 3 | BATON (BATtemp) |
| Channel 4 | AUXIN4 (external) |
| Channel 5 | ACCDET (external) |
| Channel 9 | ClassAB |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| AuxADC Channel ID | Description |
| --- | --- |
| Channel 12 | XP / External |
| Channel 13 | YP / External |
| Channel 14 | YM / External |
| Channel 15 | XM / External |

##### Register Definition

Module name: AUXADC Base address: (+A0790000h)

| Address | Name | Width | Register Function |
| --- | --- | --- | --- |
| A0790000 | AUXADC_CON0 | 16 | AuxiliaryADC Control Register 0 These bits define whether the corresponding channel is sampled or not in the timer-triggered mode. It is associated with the timing offset register TDMA_AUXEV1. It supports multiple flags. The flags can be automatically cleared after those channels are sampled if AUTOCLR1 in register AUXADC_CON3 is set. |
| A0790004 | AUXADC_CON1 | 16 | AuxiliaryADC Control Register 1 These bits are set individually to sample the data for the corresponding channel. It supports multiple flags. |
| A079000C | AUXADC_CON3 | 16 | AuxiliaryADC Control Register 3 |
| A0790010 | AUXADC_DAT0 | 16 | AuxiliaryADC Channel 0 Register (VBAT) |
| A0790014 | AUXADC_DAT1 | 16 | AuxiliaryADC Channel 1 Register (ISENSE) |
| A0790018 | AUXADC_DAT2 | 16 | AuxiliaryADC Channel 2 Register (CHRIN) |
| A079001C | AUXADC_DAT3 | 16 | AuxiliaryADC Channel 3 Register (VBATTMP) |
| A0790020 | AUXADC_DAT4 | 16 | AuxiliaryADC Channel 4 Register (External) |
| A0790024 | AUXADC_DAT5 | 16 | AuxiliaryADC Channel 5 Register (External/ACCDET) |
| A0790034 | AUXADC_DAT9 | 16 | AuxiliaryADC Channel 9 Register (ClassAB) |
| A0790040 | AUXADC_DAT12 | 16 | AuxiliaryADC Channel 12 Register (External) |
| A0790044 | AUXADC_DAT13 | 16 | AuxiliaryADC Channel 13 Register (External) |
| A0790048 | AUXADC_DAT14 | 16 | AuxiliaryADC Channel 14 Register (External) |
| A079004C | AUXADC_DAT15 | 16 | AuxiliaryADC Channel 15 Register (External) |
| A0790054 | AUX_TS_CMD0 | 16 | Touch Screen Sample Command 0 |
| A0790058 | AUX_TS_CON | 16 | Touch Screen Control |
| A079005C | AUX_TS_DAT0 | 16 | Touch Screen Sample DATA 0 |
| A0790070 | AUXADC_DAT_ZCV | 16 | AuxiliaryADC ZCV Sample DATA |
| A07900D0 | AUXADC_CON4 | 16 | AuxiliaryADC Control Register 4 |
| A07900D4 | AUX_TS_CMD1 | 16 | Touch Screen Sample Command 1 |
| A07900D8 | AUX_TS_DAT1 | 16 | Touch Screen Sample DATA 1 |

A0790000	AUXADC_CON0 AuxiliaryADC Control Register 0	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | SYN1 5 | SYN14 | SYN13 | SYN12 |  |  | SYN9 |  |  |  | SYN5 | SYN4 | SYN3 | SYN2 | SYN1 | SYN0 |
| Type | R/W | R/W | R/W | R/W |  |  | R/W |  |  |  | R/W | R/W | R/W | R/W | R/W | R/W |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Reset | 0 | 0 | 0 | 0 |  |  | 0 |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | SYN15 | SYN15 | Channel 15 sync mode 0: The channel is not selected. 1: The channel is selected. |
| 14 | SYN14 | SYN14 | Channel 14 sync mode 0: The channel is not selected. 1: The channel is selected. |
| 13 | SYN13 | SYN13 | Channel 13 sync mode 0: The channel is not selected. 1: The channel is selected. |
| 12 | SYN12 | SYN12 | Channel 12 sync mode 0: The channel is not selected. 1: The channel is selected. |
| 9 | SYN9 | SYN9 | Channel 9 sync mode 0: The channel is not selected. 1: The channel is selected. |
| 5 | SYN5 | SYN5 | Channel 5 sync mode 0: The channel is not selected. 1: The channel is selected. |
| 4 | SYN4 | SYN4 | Channel 4 sync mode 0: The channel is not selected. 1: The channel is selected. |
| 3 | SYN3 | SYN3 | Channel 3 sync mode 0: The channel is not selected. 1: The channel is selected. |
| 2 | SYN2 | SYN2 | Channel 2 sync mode 0: The channel is not selected. 1: The channel is selected. |
| 1 | SYN1 | SYN1 | Channel 1 sync mode 0: The channel is not selected. 1: The channel is selected. |
| 0 | SYN0 | SYN0 | Channel 0 sync mode 0: The channel is not selected. 1: The channel is selected. |

A0790004	AUXADC_CON1 AuxiliaryADC Control Register 1	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | IMM15 | IMM14 | IMM13 | IMM12 |  |  | IMM9 |  |  |  | IMM5 | IMM4 | IMM3 | IMM2 | IMM1 | IMM0 |
| Type | R/W | R/W | R/W | R/W |  |  | R/W |  |  |  | R/W | R/W | R/W | R/W | R/W | R/W |
| Reset | 0 | 0 | 0 | 0 |  |  | 0 |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | IMM15 | IMM15 | Channel 15 immediate mode 0: The channel is not selected. 1: The channel is selected. |
| 14 | IMM14 | IMM14 | Channel 14 immediate mode 0: The channel is not selected. 1: The channel is selected. |
| 13 | IMM13 | IMM13 | Channel 13 immediate mode 0: The channel is not selected. 1: The channel is selected. |
| 12 | IMM12 | IMM12 | Channel 12 immediate mode 0: The channel is not selected. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

|  |  |  | 1: The channel is selected. |
| --- | --- | --- | --- |
| 9 | IMM9 | IMM9 | Channel 9 immediate mode 0: The channel is not selected. 1: The channel is selected. |
| 5 | IMM5 | IMM5 | Channel 5 immediate mode 0: The channel is not selected. 1: The channel is selected. |
| 4 | IMM4 | IMM4 | Channel 4 immediate mode 0: The channel is not selected. 1: The channel is selected. |
| 3 | IMM3 | IMM3 | Channel 3 immediate mode 0: The channel is not selected. 1: The channel is selected. |
| 2 | IMM2 | IMM2 | Channel 2 immediate mode 0: The channel is not selected. 1: The channel is selected. |
| 1 | IMM1 | IMM1 | Channel 1 immediate mode 0: The channel is not selected. 1: The channel is selected. |
| 0 | IMM0 | IMM0 | Channel 0 immediate mode 0: The channel is not selected. 1: The channel is selected. |

A079000C	AUXADC_CON3 AuxiliaryADC Control Register 3	0010

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | AUTO SET |  |  |  | RSV |  | AUTO CLR1 |  | SOFT _RST |  |  | BYPA SS_SL P_ZC V_TRI GGER |  |  |  | AUXA DC_S TA |
| Type | R/W |  |  |  | R/W |  | R/W |  | R/W |  |  | R/W |  |  |  | RO |
| Reset | 0 |  |  |  | 0 |  | 0 |  | 0 |  |  | 1 |  |  |  | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | AUTOSET | AUTOSET | Defines the auto-sample mode of the module. In auto-sample mode, each channel with its sample register being read can start sampling immediately without configuring the control register AUXADC_CON1 again. |
| 11 | RSV | RSV | Please keep 1'b0 |
| 9 | AUTOCLR1 | AUTOCLR1 | Defines the auto-clear mode of the module for event 1. In the auto-clear mode, each timer-triggered channel acquires samples of specified channels once the SYNn bit in register AUXADC_CON0 is set. The SYNn bits are automatically cleared and the channel is not enabled again by the timer event except when the SYNn flags are set again. 0: The automatic clear mode is not enabled. 1: The automatic clear mode is enabled. |
| 7 | SOFT_RST | SOFT_RST | Software reset AUXADC state machine 0: Normal function 1: Reset AUXADC state machine |
| 4 | BYPASS_SL P_ZCV_TRI GGER | BYPASS_SLP_ZCV _TRIGGER | Bypass zcv triggering after sleep mode setting 0: trigger zcv measuring after sleep mode 1: bypass zcv measuring trigger after sleep mode |
| 0 | AUXADC_S TA | AUXADC_STA | Defines the state of the module 0: This module is idle. 1: This module is busy. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A0790010	AUXADC_DAT0 AuxiliaryADC Channel 0 Register (VBAT)	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  | DAT0 |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  | RO |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 9:0 | DAT0 | DAT0 | Sampled data for channel0 |

A0790014	AUXADC_DAT1 AuxiliaryADC Channel 1 Register (ISENSE)	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  | DAT1 |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  | RO |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 9:0 | DAT1 | DAT1 | Sampled data for channel1 |

A0790018	AUXADC_DAT2 AuxiliaryADC Channel 2 Register (CHRIN)	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  | DAT2 |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  | RO |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 9:0 | DAT2 | DAT2 | Sampled data for channel2 |

A079001C	AUXADC_DAT3 AuxiliaryADC Channel 3 Register (VBATTMP)	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  | DAT3 |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  | RO |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 9:0 | DAT3 | DAT3 | Sampled data for channel3 |

A0790020	AUXADC_DAT4 AuxiliaryADC Channel 4 Register (External)	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  | DAT4 |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  | RO |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 9:0 | DAT4 | DAT4 | Sampled data for channel4 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A0790024	AUXADC_DAT5 AuxiliaryADC Channel 5 Register (External/ACCDET)	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  | DAT5 |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  | RO |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 9:0 | DAT5 | DAT5 | Sampled data for channel5 |

A0790034	AUXADC_DAT9 AuxiliaryADC Channel 9 Register (ClassAB)	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  | DAT9 |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  | RO |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 9:0 | DAT9 | DAT9 | Sampled data for channel9 |

A0790040	AUXADC_DAT1 AuxiliaryADC Channel 12 Register (External)	0000

2

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  | DAT12 |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  | RO |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 9:0 | DAT12 | DAT12 | Sampled data for channel12 |

A0790044	AUXADC_DAT1 AuxiliaryADC Channel 13 Register (External)	0000

3

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  | DAT13 |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  | RO |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 9:0 | DAT13 | DAT13 | Sampled data for channel13 |

A0790048	AUXADC_DAT1 AuxiliaryADC Channel 14 Register (External)	0000

4

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  | DAT14 |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  | RO |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| 9:0 | DAT14 | DAT14 | Sampled data for channel14 |
| --- | --- | --- | --- |

A079004C	AUXADC_DAT1 AuxiliaryADC Channel 15 Register (External)	0000

5

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  | DAT15 |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  | RO |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 9:0 | DAT15 | DAT15 | Sampled data for channel15 |

A0790054	AUX_TS_CMD0 Touch Screen Sample Command 0	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | TS_S PL | TS_MAGIC_KEY |  |  |  |  |  |  |  | ADDRESS |  |  | MODE | SEDF | PD |  |
| Type | R/W | WO |  |  |  |  |  |  |  | R/W |  |  | R/W | R/W | R/W |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | TS_SPL | TS_SPL | Touch screen sample trigger For the touch screen status, please refer to PMU datasheet : "AUX_CON6" register bit 14 "ts_status" 0: No action 1: When software writes 1'b1, AUXADC will trigger the touch screen process. After the sampling process of touch screen is finished, this bit will be disserted. |
| 14:7 | TS_MAGIC_ KEY | TS_MAGIC_KEY | The TS commands in AUX_TS_CMD0 can only take effect when the TS_MAGIC_KEY matches the correct value. TS_MAGIC_KEY=0xaa . |
| 6:4 | ADDRESS | ADDRESS | Defines which x or y or z data will be sampled 001: Y position 011: Z1 position 100: Z2 position 101: X position Others: Reserved |
| 3 | MODE | MODE | Selects sample resolution 0: 10-bit 1: 8-bit |
| 2 | SEDF | SEDF | Selects mode 0: Differential mode 1: Single-end mode |
| 1:0 | PD | PD | Power-down control for analog IRQ signal and touch screen sample control signal 00: Turn on Y-_drive signal and PDN_sh_ref 01: Turn on PDN_IRQ and PDN_sh_ref 10: Reserved 11: Turn on PDN_IRQ |

A0790058	AUX_TS_CON	Touch Screen Control	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TS_S PL |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 0 | TS_SPL | TS_SPL | Touch screen sample trigger For the touch screen status, please refer to PMU datasheet : "AUX_CON6" register bit 14 "ts_status" 0: No action 1: When software writes 1'b1, AUXADC will trigger the touch screen process. After the sampling process of touch screen is finished, this bit will be disserted. |

A079005C	AUX_TS_DAT0 Touch Screen Sample DATA 0	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | TS_S PL |  |  |  |  |  | TS_DAT |  |  |  |  |  |  |  |  |  |
| Type | RO |  |  |  |  |  | RO |  |  |  |  |  |  |  |  |  |
| Reset | 0 |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | TS_SPL | TS_SPL | Touch screen sample trigger For the touch screen status, please refer to PMU datasheet : "AUX_CON6" register bit 14 "ts_status" |
| 9:0 | TS_DAT | TS_DAT | Touch screen sample result data |

A0790070	AUXADC_DAT_ AuxiliaryADC ZCV Sample DATA	0000

ZCV

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  | DAT_ZCV |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  | RO |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 9:0 | DAT_ZCV | DAT_ZCV | Sampled data for ZCV |

A07900D0	AUXADC_CON4 AuxiliaryADC Control Register 4	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | TS_S PL | AUXA DC_S TA |  |  |  |  | SYN9 |  |  |  | SYN5 | SYN4 | SYN3 | SYN2 | SYN1 | SYN0 |
| Type | RO | RO |  |  |  |  | RO |  |  |  | RO | RO | RO | RO | RO | RO |
| Reset | 0 | 0 |  |  |  |  | 0 |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | TS_SPL | TS_SPL | Touch screen sample trigger For the touch screen status, please refer to PMU datasheet : "AUX_CON6" register bit 14 "ts_status" 0: No action 1: When software writes 1'b1, AUXADC will trigger the touch screen process. After the sampling process of touch screen is finished, this bit will be disserted. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| 14 | AUXADC_S TA | AUXADC_STA | Defines the state of the module 0: This module is idle. 1: This module is busy. |
| --- | --- | --- | --- |
| 9 | SYN9 | SYN9 | Channel 9 sync mode 0: The channel is not selected. 1: The channel is selected. |
| 5 | SYN5 | SYN5 | Channel 5 sync mode 0: The channel is not selected. 1: The channel is selected. |
| 4 | SYN4 | SYN4 | Channel 4 sync mode 0: The channel is not selected. 1: The channel is selected. |
| 3 | SYN3 | SYN3 | Channel 3 sync mode 0: The channel is not selected. 1: The channel is selected. |
| 2 | SYN2 | SYN2 | Channel 2 sync mode 0: The channel is not selected. 1: The channel is selected. |
| 1 | SYN1 | SYN1 | Channel 1 sync mode 0: The channel is not selected. 1: The channel is selected. |
| 0 | SYN0 | SYN0 | Channel 0 sync mode 0: The channel is not selected. 1: The channel is selected. |

A07900D4	AUX_TS_CMD1 Touch Screen Sample Command 1	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  | ADDRESS |  |  | MODE | SEDF | PD |  |
| Type |  |  |  |  |  |  |  |  |  | R/W |  |  | R/W | R/W | R/W |  |
| Reset |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 6:4 | ADDRESS | ADDRESS | Defines which x or y or z data will be sampled 001: Y position 011: Z1 position 100: Z2 position 101: X position Others: Reserved |
| 3 | MODE | MODE | Selects sample resolution 0: 10-bit 1: 8-bit |
| 2 | SEDF | SEDF | Selects mode 0: Differential mode 1: Single-end mode |
| 1:0 | PD | PD | Power-down control for analog IRQ signal and touch screen sample control signal 00: Turn on Y-_drive signal and PDN_sh_ref 01: Turn on PDN_IRQ and PDN_sh_ref 10: Reserved 11: Turn on PDN_IRQ |

A07900D8	AUX_TS_DAT1 Touch Screen Sample DATA 1	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  | TS_DAT |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  | RO |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 9:0 | TS_DAT | TS_DAT | Touch screen sample result data |

※ Please refer to “PMU” datasheet for other AuxADC related settings:

- Sampling cycle control: Please see “AUX_CON4” and “AUX_CON5”

##### General Programming Guide

All register writes will occur in sequence.	However, due to synchronization time, any reads after writes need to be delayed by three dummy reads.

##### Usage Programming Guide

There are two modes to program the AUXADC to sample: immediate mode and synchronous mode. The following are notes on programming each mode:

Immediate mode sampling is accomplished by programming AUXADC_CON1 with the channels to be sampled.	After programming, it is necessary to perform three dummy reads on AUXADC_CON3.

After the dummy reads, the next read of AUXADC_CON3 will be valid.	After sampling is done, it is necessary to program AUXADC_CON1 back to zero before sampling again.

Synchronous mode sampling is accomplished by programming AUXADC_CON0 with the channels to be sampled.	Then it is necessary to program TDMA_EVTENA7 to 0x2. After the sample is done, TDMA_EVTENA7 must be programmed to 0x0 with waiting of two frames before sampling again.

##### Performance Programming Guide

For details on adjusting the performance of ADC sampling, please refer to registers AUX_CON4, AUX_CON5 and AUX_CON6 in the PMU datasheet.

##### AUXADC PDN

AUXADC is located in A-die.	Due to limitation, one of UART1,2,3 clocks must be turned on in order to use AUXADC.	Please clear the corresponding PDN bits to enable the AUXADC clock (bit 2 or 3 of ACFG_CLK_CG).

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Notice

The 4 TP pins – PAD_XP, PAD_XM, PAD_YP, PAD_YM, are used as EINT pins when no R-touch is used in the system. Therefore, please make sure the pin settings are correct.

### USB Device Controller

##### General Description

This chip provides a USB function interface which complies with Universal Serial Bus Specification Rev

  - The USB device controller supports only full-speed (12Mbps) operation. The chip can make use of this widely available USB interface to transmit/receive data with USB hosts, typically PC/laptop.

There are 6 endpoints in the USB device controller besides the mandatory control endpoint, 4 of which are for IN transactions and 2 endpoints are for OUT transactions. Word, half-word, and byte access are all allowed for loading and unloading the FIFO. The controller features 4 DMA channels to accelerate the data transfer for ACL and SCO data streams. The features of the endpoints are:

- Endpoint 0: The control endpoint feature 64 bytes FIFO and accommodates maximum packet size of up to 64 bytes. DMA transfer is not supported.

- IN endpoint 1: It features 64 bytes FIFO and accommodates maximum packet size of up to 64 bytes. GDMA Channel 4 Write Transfer is supported.

- IN endpoint 2: It features 64 bytes FIFO and accommodates maximum packet size of up to 64 bytes. GDMA Channel 6 Write Transfer is supported.

- IN endpoint 3: It features 16-byte FIFO and accommodates maximum packet size of 16 bytes. GDMA Transfer is not supported.

- IN endpoint 4: It features 16-byte FIFO and accommodates maximum packet size of 16 bytes. GDMA Transfer is not supported.

- OUT endpoint 1: It features 64 bytes FIFO and accommodates maximum packet size of 64 bytes. GDMA Channel 5 Read Transfer is supported.

- OUT endpoint 2: It features 64 bytes FIFO and accommodates maximum packet size of 64 bytes. GDMA Channel 7 Read Transfer is supported.

For each endpoint except for the control endpoint, when the packet size is smaller than half the size of the FIFO, at most 2 packets can be buffered.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

This unit is highly software configurable. All endpoints except for the control endpoint can be configured to be a bulk, interrupt or isochronous endpoints. Composite devices are also supported. IN endpoint 1 and OUT endpoint 1 share the same endpoint number but they can be used separately. So is the situation for the endpoint 2.

The USB device uses the cable-powered feature for the transceiver but only drains little current. An internal pull-up resistor is integrated across Vbus and D+ signal. The switch on/off of the pull-up resistor can be configured through the internal register. Two additional external serial resistors might be needed to place on the output of D+ and D- signals to make the output impedance equivalent to 28~44Ohm.

2 layers AHB bus

usb.all.v

FS transceiver

SPRAM288X8M8_ BIST_CON

musbfsfc_ahb

slv_mux

Figure 1. USB11 controller system diagram

##### Terminology

RW: Writable, Readable.

RO: Read-only. Value never changes. WO: Write-only.

W1: Write-once. Readable.

RU: Read-only but value updated by the design. W1C: Readable. Write 1 to bitwise-clear.

RC: Clear on read.

A1: Auto-set by the design. Can be read and write 0 to clear. A0: Auto-cleared by the design. Can be read and write 1 to set. DC: Don't care.

OTHER: Others. Mixed attribute. Refer to bit description.

RSV: Reserved. The read/write behavior to this bit is undefined.

##### Register Definition

- USB_FADDR	USB Function Address Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Name |  |  |  |  |  |  |  |  | UPD | FADDR |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type |  |  |  |  |  |  |  |  | RW | RW |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7 | UPD | This is an 8bit register that should be written with the functions 7-bit address (received through a SET_ADDRESS description). It is then used for decoding the function address in subsequent token packets. When set by the MCU, the core will wait for an SOF token from the time INPKTRDY is set before sending the packet |
| 6:0 | FADDR | Function address of device |

- USB_POWER	USB Power Control Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | ISOU PDAT E |  |  | SWR STEN AB | RESE T | RESU ME | SUSP MOD E | SUSP ENA B |
| Type |  |  |  |  |  |  |  |  | RW |  |  | RU | RU | RW | RU | RW |
| Reset |  |  |  |  |  |  |  |  | 0 |  |  | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7 | ISOUPDATE | When set by the MCU, the core will wait for an SOF token from the time INPKTRDY is set before sending the packet |
| 4 | SWRSTENAB | Set by the MCU to enable the mode in which the device can only be reset by the software after reset signals are detected on the bus. In case the software is delayed by other high priority processes and cannot make it to read the command from the buffer before the hardware reset the device after the reset signal is detected on the bus, the command will be lost. That is why the software reset mode is effective. When the flag is enabled, the hardware state machine cannot reset itself but by the software. In that sense, the software and hardware can keep synchronous detecting the reset signal. |
| 3 | RESET | The read-only bit is set when Reset signaling is present on the bus |
| 2 | RESUME | Set by the MCU to generate Resume signaling when the function is in the suspend mode. The MCU should clear this bit after 10ms (maximum 15ms) to end Resume signaling |
| 1 | SUSPMODE | Set by the USB core when the Suspend mode is entered Cleared when the Resume bit of this register is set. |
| 0 | SUSPENAB | Set by the MCU to enable device into the Suspend mode when Suspend signaling is received on the bus |

0002	USB_INTRIN	USB IN Endpoints Interrupt Register	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  | EP4_I N | EP3_I N | EP2_I N | EP1_I N | EP0 |
| Type |  |  |  |  |  |  |  |  |  |  |  | RC | RC | RC | RC | RC |
|  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Name | Description |
| --- | --- | --- |
| 4 | EP4_IN | IN Endpoint 4 interrupt event |
| 3 | EP3_IN | IN Endpoint 3 interrupt event |
| 2 | EP2_IN | IN Endpoint 2 interrupt event |
| 1 | EP1_IN | IN Endpoint 1 interrupt event |
| 0 | EP0 | Endpoint 0 interrupt event |

0004	USB_INTROUT	USB OUT Endpoints InterruptRegister	0000

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  | EP2_ OUT | EP1_ OUT |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | RC | RC |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 |  |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 2 | EP2_OUT | OUT Endpoint 2 interrupt event |
| 1 | EP1_OUT | OUT Endpoint 1 interrupt event |

06	USB_INTRUSB	USB General Interrupt Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  | POW ERD WN | SOF | RESE T | RESU ME | SUSP END |
| Type |  |  |  |  |  |  |  |  |  |  |  | RC | RC | RC | RC | RC |
| Reset |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 4 | POWERDWN | Set at SUSPMODE and LineState is JState. The programmer should have de bounce scheme in SW code when using this interrupt. |
| 3 | SOF | Set at the start of each frame |
| 2 | RESET | Set when Reset signaling is detected on the bus |
| 1 | RESUME | Set when Resume signaling is detected on the bus while the USB core is in suspend mode |
| 0 | SUSPEND | Set when Suspend signaling is detected on the bus |

0007	USB_INTRINE	USB IN Endpoints Interrupt Enable Register	00FF

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  | EP4_I NE | EP3_I NE | EP2_I NE | EP1_I NE | EP0_ E |
| Type |  |  |  |  |  |  |  |  |  |  |  | RW | RW | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 | 1 | 1 | 1 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 4 | EP4_INE | 1b0: Disable IN Endpoint 4 interrupt event 1b1: Enable IN Endpoint 4 interrupt event |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Name | Description |
| --- | --- | --- |
| 3 | EP3_INE | 1b0: Disable IN Endpoint 3 interrupt event 1b1: Enable IN Endpoint 3 interrupt event |
| 2 | EP2_INE | 1b0: Disable IN Endpoint 2 interrupt event 1b1: Enable IN Endpoint 2 interrupt event |
| 1 | EP1_INE | 1b0: Disable IN Endpoint 1 interrupt event 1b1: Enable IN Endpoint 1 interrupt event |
| 0 | EP0_E | 1b0: Disable IN Endpoint 0 interrupt event 1b1: Enable IN Endpoint 0 interrupt event |

0009	USB_INTROUTE USB OUT Endpoints Interrupt Enable Register	00FE

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  | EP2_ OUTE | EP1_ OUTE |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | RW | RW |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |  |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 2 | EP2_OUTE | 1b0: Disable OUT Endpoint 2 interrupt event 1b1: Enable OUT Endpoint 2 interrupt event |
| 1 | EP1_OUTE | 1b0: Disable OUT Endpoint 1 interrupt event 1b1: Enable OUT Endpoint 1 interrupt event |

0B	INTRUSBE	USB General Interrupt Enable Register	06

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  | POW ERD WN_ E | SOF_ E | RESE T_E | RESE UM_E | SUSP END_ E |
| Type |  |  |  |  |  |  |  |  |  |  |  | RW | RW | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 1 | 1 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 4 | POWERDWN_E | Enables power-down interrupt |
| 3 | SOF_E | Enables SOF interrupt |
| 2 | RESET_E | Enables reset/babble interrupt |
| 1 | RESEUM_E | Enables resume interrupt |
| 0 | SUSPEND_E | Enables suspend interrupt |

0C	USB_FRAME1	USB Frame Count #1 Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | NUML |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RU |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | NUML | The lower 8 bits of the frame number |

0D	USB_FRAME2	USB Frame Count #2 Register	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  | NUMH |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | RU |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 2:0 | NUMH | The upper 3 bits of the frame number |

0E	USB_INDEX	USB Endpoint Register Index	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | INDEX |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RW |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |

The register determines which endpoint control/status registers are to be accessed at addresses USB+10h to USB+17h. Each IN endpoint and OUT endpoint has its own set of control/status registers. Only one set of IN control/status and one set of OUT control/status registers appear in the memory map at a time. Before accessing the control/status registers of an endpoint, the endpoint number should be written to the USB_INDEX register to ensure that the correct control/status registers appear in the memory map.

| Bit(s) | Name | Description |
| --- | --- | --- |
| 3:0 | INDEX | Index of the endpoint |

0F	USB_RSTCTRL	USB Reset Control	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | SWR ST |  |  |  | RSTCNTR |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  | RW |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 |  |  |  | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7 | SWRST | If the flag SWRSTENAB in register USB_POWER is set to 1, the software enable mode will be enabled, and the device can be reset by writing 1 to this flag. |
| 3:0 | RSTCNTR | Signifies the duration of the reset operation after reset signal is detected on the bus. It is only enabled when software reset is not enabled. If the value is 0, the duration will be 2.5us. Otherwise, the duration will be this value multiplied by 341 then added by 2.5 in us unit. The range consequently starts from 2.5us to 5122.5us. |

| 11 | USB_EP0_CSR | USB Control/Status Register for Endpoint 0 | 00 |
| --- | --- | --- | --- |

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | SSET UPEN D | SOUT PKTR DY | SEND STAL L | SETU PEND | DATA END | SENT STAL L | INPK TRDY | OUT PKTR DY |
| Type |  |  |  |  |  |  |  |  | A0 | A0 | A0 | RU | A0 | A1 | A0 | RU |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7 | SSETUPEND | The MCU writes 1 to this bit to clear the SETUPEND bit. It is cleared automatically. Only active when a transaction is started. |
| 6 | SOUTPKTRDY | The MCU writes 1 to this bit to clear the OUTPKTRDY bit. It is cleared automatically. Only active when an OUT transaction is started |
| 5 | SENDSTALL | The MCU writes 1 to this bit to terminate the current transaction. The STALL handshake will be transmitted, and this bit will be cleared automatically. |
| 4 | SETUPEND | This bit will be set when a control transaction ends before the DATAEND bit is set. An interrupt will be generated and FIFO flushed at this time. The bit is cleared by the MCU writing 1 to the SSETUPEND bit. |
| 3 | DATAEND | The MCU sets this bit: - When setting INPKTRDY for the last data packet. - When clearing OUTPKTRDY after unloading the last data packet. - When setting INPKTRDY for a zero length data packet. It is cleared automatically. |
| 2 | SENTSTALL | This bit is set when a STALL handshake is transmitted. The MCU should clear this bit by writing 0 to it. |
| 1 | INPKTRDY | The MCU sets up this bit after loading a data packet into the FIFO. It is cleared automatically when the data packet is transmitted. An interrupt is generated when this bit is set |
| 0 | OUTPKTRDY | This bit is set when a data packet is received. An interrupt is generated when this bit is set. The MCU clears this bit by setting up the SOUTPKTRDY bit. |

16	USB_EP0_COUN T

EP0 Received Bytes Count Register	00

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  | EP0_COUNT |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  | RU |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

The register indicates the number of received data bytes in the endpoint 0. The value returned is valid while the OUTPKTRDY bit of the USB_EP0_CSR register is set. The register is active when the USB_INDEX register is set to 0.

| Bit(s) | Name | Description |
| --- | --- | --- |
| 6:0 | EP0_COUNT | Number of received data bytes in the endpoint 0 |

- USB_EP_INMAXP USB Maximum Packet Size Register for IN Endpoint	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Name |  |  |  |  |  |  |  |  | MAXP |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

The register holds the maximum packet size for transactions through the currently selected IN endpoint - in units of one byte. When setting up the value, the programmer should note the constraints placed by the USB Specification on the packet size for bulk interrupt and isochronous transactions in full speed operation. There is an INMAXP register for each IN endpoint except for endpoint 0. The registers are active when the USB_INDEX register is set to 1, 2, 3, and 4 respectively.

The value written to this register should match the wMaxPacketSize field of the standard endpoint descriptor for the associated endpoint. A mismatch may cause unexpected results. If a value is bigger than the configured IN FIFO size for the endpoint written to the register, the value will be automatically changed to the IN FIFO size. If the value written to the register is smaller than or equal to half the IN FIFO size, two IN packets can be buffered. The configured IN FIFO size for the endpoint 1, 2, and 3, are 16 bytes, 64 bytes, and 64 bytes respectively.

The register is reset to 0. If the register is changed after the packets are sent from the endpoint, the endpoint IN FIFO should be completely flushed after writing the new value to the register.

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | MAXP | Maximum packet size (unit: byte) |

- USB_EP_INCSR1 USB Control/Status Register #1 for IN Endpoint	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | ABO RTPK T_EN | CLRD ATAT OG | SENT STAL L | SEND STAL L | FLUS HFIF O | UNDE RRU N | FIFO NOTE MPTY | INPK TRDY |
| Type |  |  |  |  |  |  |  |  | RW | A0 | A1 | RW | A0 | A1 | RU | A0 |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7 | ABORTPKT_EN | When MCU writes 1 to ABORTPKT_EN, FLUSHFIFO switches to abort the packet function. This bit should be enabled before FLUSHFIFO is set. If FLUSHFIFO is set and ABORTPKT_EN is enabled the data loaded into FIFO will be discarded. After the packet is aborted, EP will issue an interrupt. The programmer should wait for this interrupt to make sure the packet is aborted |
| 6 | CLRDATATOG | The MCU writes 1 to this bit to reset the endpoint IN data toggle to 0 |
| 5 | SENTSTALL | The bit is set when a STALL handshake is transmitted. The FIFO is flushed and the INPKTRDY bit is cleared. The MCU should clear this bit by writing 0 to this bit. |
| 4 | SENDSTALL | The MCU writes 1 to this bit to issue a STALL handshake to an IN token. The MCU clears this bit to terminate the stall condition |
| 3 | FLUSHFIFO | The MCU writes 1 to this bit to flush the next packet to be transmitted from the endpoint IN FIFO. The FIFO pointer is reset and the INPKTRDY bit is cleared. If the FIFO contains two packets, FLUSHFIFO will need to be set twice to completely clear the FIFO. FLUSHFIFO should only be used when INPKTRDY is set. At other times, it may cause data corruption. If ABORTPKT_EN is enabled and this bit is set , the function of this bit will become ABORTPKT to abort the next packet to be transmitted from the endpoint IN FIFO and does not need to set INPKTRDY. It is the same with the |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Name | Description |
| --- | --- | --- |
|  |  | FLUSHFIFO function. This bit is only active when the endpoint is idle. |
| 2 | UNDERRUN | In isochronous mode, this bit is set when a zero length data packet is sent after receiving an IN token with the INPKTRDY bit not set. In Bulk/Interrupt mode, this bit is set when a NAK is returned in response to an IN token. The MCU should clear this bit by writing a 0 to this bit |
| 1 | FIFONOTEMPTY | This bit is set when there is at least 1 packet in the IN FIFO |
| 0 | INPKTRDY | The MCU sets up this bit after loading a data packet into the FIFO. Only active when an IN transaction is started. It is cleared automatically when a data packet is transmitted. An interrupt will be generated (if enabled) when the bit is cleared |

- USB_EP_INCSR2 USB Control/Status Register #2 for IN Endpoint	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | AUTO SET | ISO | MOD E | DMA ENA B | FRCD ATAT OG |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW | RW | RW | RW | RW |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 |  |  |  |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7 | AUTOSET | If the MCU sets up the bit, INPKTRDY will be automatically set when the data of the maximum packet size (value in INMAXP) is loaded into the IN FIFO. If a packet of smaller than the maximum packet size is loaded, INPKTRDY will have to be set manually. When 2 packets are in the IN FIFO, INPKTRDY will also be automatically set when the first packet is sent if the second packet is the maximum packet size |
| 6 | ISO | The MCU sets up this bit to enable the IN endpoint for isochronous transfer and clears it to enable the IN endpoint for bulk/interrupt transfers |
| 5 | MODE | The MCU sets up this bit to enable the endpoint direction as IN and clears it to enable the endpoint direction as OUT. It is valid only when the same endpoint FIFO is used for both IN and OUT transactions. |
| 4 | DMAENAB | The MCU sets up this bit to enable the DMA request for the IN endpoint. |
| 3 | FRCDATATOG | The MCU sets up this bit to force the endpoints IN data toggle to switch after each data packet is sent regardless of whether an ACK has been received. This can be used by interrupt IN endpoints which are used to communicate rate feedback for isochronous endpoints |

- USB_EP_OUTMA USB Maximum Racket Size Register for OUT Endpoint	00

XP

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | MAXP |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

This register holds the maximum packet size for transactions through the currently selected OUT endpoint (unit: byte). When setting up this value, the programmer should note the constraints placed by the USB specification on packet sizes for bulk, interrupt, and isochronous transactions in full speed operations. There is an OUTMAXP register for each OUT endpoint except for endpoint 0. The registers are active when the USB_INDEX register is set to 1 and 2 respectively.

The value written to this register should match the wMaxPacketSize field of the standard endpoint descriptor for the associated endpoint. A mismatch may cause unexpected results. The total amount of data represented by the value written to this register must not exceed the FIFO size for the OUT endpoint and should not exceed half the FIFO size if double buffering is required. If a value bigger than the configured OUT FIFO size for the endpoint is written to the register, the value will be automatically changed to the OUT FIFO size. If the value written to the register is smaller than or equal to half the OUT FIFO size, two OUT packets can be buffered. The configured IN FIFO size for the endpoint 1, 2, and 3 are both 16, 64, and 64 bytes, respectively.

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | MAXP | Maximum packet size (unit: byte) |

- USB_EP_OUTCS R1

USB Control/Status Register #1 for OUT Endpoint	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | CLRD TATO G | SENT STAL L | SEND STAL L | FLUS HFIF O | DATA ERR OR | OVER RUN | FIFO FULL | RXP KTRD Y |
| Type |  |  |  |  |  |  |  |  | A0 | A1 | RW | A0 | RU | A1 | RU | A1 |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7 | CLRDTATOG | The MCU writes 1 to this bit to reset the endpoint data toggle to 0. |
| 6 | SENTSTALL | The bit is set when a STALL handshake is transmitted. The MCU should clear this bit by writing 0 to it. |
| 5 | SENDSTALL | The MCU writes 1 to this bit to issue a STALL handshake. The MCU clears this bit to terminate the stall condition. This bit will have no effect if the OUT endpoint is in the isochronous mode |
| 4 | FLUSHFIFO | The MCU writes 1 to this bit to flush the next packet to be read from the endpoint OUT FIFO. If the FIFO contains two packets, FLUSHFIFO will need to be set twice to completely clear the FIFO. FLUSHFIFO should only be used when OUTPKTRDY is set. At other times, it may cause data corruption. |
| 3 | DATAERROR | The bit is set when OUTPKTRDY is set if the data packet has a CRC or bit stuff error. It is cleared when OUTPKTRDY is cleared. This bit is only valid in the isochronous mode. |
| 2 | OVERRUN | The bit will be set if an OUT packet cannot be loaded into the OUT FIFO. The MCU should clear the bit by writing 0 to it. This bit is only valid in the isochronous mode. |
| 1 | FIFOFULL | This bit is set when no more packets can be loaded into the OUT FIFO |
| 0 | RXPKTRDY | The bit is set when a data packet has been received. The MCU should clear (write 0 to it) the bit when the packet is unloaded from the OUT FIFO. An interrupt will be generated when the bit is set. When the receiving null packet is received, OUTPKTRDY will be set after USB_INTROUT1 is high. |

- USB_EP_OUTCS R2

USB Control/Status Register #2 for OUT Endpoint	00

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Name |  |  |  |  |  |  |  |  | AUTO CLEA R | ISO | DMA ENA B | DMA MOD E |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type |  |  |  |  |  |  |  |  | RW | RW | RW | RW |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7 | AUTOCLEAR | If the MCU sets up this bit, the OUTPKTRDY bit will be automatically cleared when a packet of OUTMAXP bytes is unloaded from the OUT FIFO. When packets of smaller than the maximum packet size are unloaded, OUTPKTRDY will have to be cleared manually. |
| 6 | ISO | The MCU sets up this bit to enable the OUT endpoint for isochronous transfers and clears it to enable the OUT endpoint for bulk/interrupt transfers |
| 5 | DMAENAB | The MCU sets up this bit to enable the DMA request for the OUT endpoint |
| 4 | DMAMODE | Two modes of DMA operation are supported: DMA mode 0 in which a DMA request is generated for all received packets, together with an interrupt (if enabled). DMA mode 1 in which a DMA request (but no interrupt) is generated for OUT packets of size OUTMAXP bytes and an interrupt (but no DMA request) is generated for OUT packets of any other size. The MCU sets up the bit to select DMA mode 1 and clears this bit to select DMA mode 0. |

- USB_EP_COUNT USB OUT Endpoint Byte Counter Register LSB Part for	00

1	Endpoint

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | NUML |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RU |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

The register holds the lower 8 bits of the number of received data bytes in the packet in the FIFO associated with the currently selected OUT endpoint. The value returned is valid while OUTPKTRDY in register USB_OUTCSR1 is set. The registers are active when the USB_INDEX register is set to 1 and 2 respectively.

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | NUML | Lower 8 bits of the number of received data bytes for the OUT endpoint |

- USB_EP_COUNT

2

USB OUT Endpoint Byte Counter Register MSB Part for

Endpoint	00

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  | NUMH |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | RU |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 |

The register holds the upper 3 bits of the number of received data bytes in the packet in the FIFO associated with the currently selected OUT endpoint. The value returned is valid while OUTPKTRDY in register USB_EP_OUTCSR1 is set. The registers are active when the USB_INDEX register is set to 1 and 2 respectively.

| Bit(s) | Name | Description |
| --- | --- | --- |
| 2:0 | NUMH | Upper 8 bits of the number of received data bytes for the OUT endpoint. |

- USB_EP0_FIFO_ DB0

USB Endpoint 0 FIFO Register DB0	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FIFO0_DB0 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

The register provides MCU access to the FIFO for the endpoint 0. Writing to this register will load data to the FIFO for the endpoint 0. Reading this register unloads data from the FIFO for the endpoint 0.

The register provides word, half word, and byte mode accesses. If word or half word accesses are performed, the less significant byte will correspond to the prior byte to load to or unload from the FIFO.

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | FIFO0_DB0 | The first byte to be loaded to or unloaded from the FIFO. |

- USB_EP0_FIFO_ DB1

USB Endpoint 0 FIFO Register DB1	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FIFO0_DB1 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | FIFO0_DB1 | The second byte to be loaded to or unloaded from the FIFO. |

- USB_EP0_FIFO_ DB2

USB Endpoint 0 FIFO Register DB2	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FIFO0_DB2 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | FIFO0_DB2 | The third byte to be loaded to or unloaded from the FIFO. |

- USB_EP0_FIFO_ DB3

USB Endpoint 0 FIFO Register DB3	00

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FIFO0_DB3 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | FIFO0_DB3 | The forth byte to be loaded to or unloaded from the FIFO. |

- USB_EP1_FIFO_ DB0

USB Endpoint 1 FIFO Register DB0	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FIFO1_DB0 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

The register provides MCU access to the FIFO for the endpoint 1. Writing to this register will load data to the FIFO for the endpoint 1. Reading this register will unload data from the FIFO for the endpoint 1.

The register provides word, half word, and byte mode accesses. If word or half word accesses are performed, the less significant byte will correspond to the prior byte to load to or unload from the FIFO.

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | FIFO1_DB0 | The first byte to be loaded to or unloaded from the FIFO. |

- USB_EP1_FIFO_ DB1

USB Endpoint 1 FIFO Register DB1	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FIFO1_DB1 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | FIFO1_DB1 | The second byte to be loaded to or unloaded from the FIFO. |

- USB_EP1_FIFO_ DB2

USB Endpoint 1 FIFO Register DB2	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FIFO1_DB2 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | FIFO1_DB2 | The third byte to be loaded to or unloaded from the FIFO. |

- USB_EP1_FIFO_ DB3

USB Endpoint 1 FIFO Register DB3	00

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FIFO1_DB3 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | FIFO1_DB3 | The forth byte to be loaded to or unloaded from the FIFO. |

- USB_EP2_FIFO_ DB0

USB Endpoint 2 FIFO Register DB0	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FIFO2_DB0 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

The register provides MCU access to the FIFO for the endpoint 2. Writing to this register will load data to the FIFO for the endpoint 2. Reading this register will

unloads data from the FIFO for the endpoint 2.unloads data from the FIFO for the endpoint 2.

The register provides word, half word, and byte mode accesses. If word or half word accesses are performed, the less significant byte will correspond to the prior byte to load to or unload from the FIFO.

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | FIFO2_DB0 | The first byte to be loaded to or unloaded from the FIFO. |

- USB_EP2_FIFO_ DB1

USB Endpoint 2 FIFO Register DB1	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FIFO2_DB1 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | FIFO2_DB1 | The second byte to be loaded to or unloaded from the FIFO. |

2A	USB_EP2_FIFO_ DB2

USB Endpoint 2 FIFO Register DB2	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FIFO2_DB2 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | FIFO2_DB2 | The third byte to be loaded to or unloaded from the FIFO. |

2B	USB_EP2_FIFO_ DB3

USB Endpoint 2 FIFO Register DB3	00

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FIFO2_DB3 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | FIFO2_DB3 | The forth byte to be loaded to or unloaded from the FIFO. |

2C	USB_EP3_FIFO_ DB0

USB Endpoint 3 FIFO Register DB0	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FIFO3_DB0 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

The register provides MCU access to the FIFO for the endpoint 3. Writing to this register will load data to the FIFO for the endpoint 3. Reading this register will unload data from the FIFO for the endpoint 3.

The register provides word, half word, and byte mode accesses. If word or half word accesses are performed, the less significant byte will correspond to the prior byte to load to or unload from the FIFO.

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | FIFO3_DB0 | The first byte to be loaded to or unloaded from the FIFO. |

2D	USB_EP3_FIFO_ DB1

USB Endpoint 3 FIFO Register DB1	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FIFO3_DB1 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | FIFO3_DB1 | The second byte to be loaded to or unloaded from the FIFO. |

2E	USB_EP3_FIFO_ DB2

USB Endpoint 3 FIFO Register DB2	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FIFO3_DB2 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | FIFO3_DB2 | The third byte to be loaded to or unloaded from the FIFO. |

2F	USB_EP3_FIFO_ DB3

USB Endpoint 3 FIFO Register DB3	00

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FIFO3_DB3 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | FIFO3_DB3 | The forth byte to be loaded to or unloaded from the FIFO. |

- USB_EP4_FIFO_ DB0

USB Endpoint 4 FIFO Register DB0	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FIFO4_DB0 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

The register provides MCU access to the FIFO for the endpoint 4. Writing to this register will load data to the FIFO for the endpoint 4. Reading this register will unload data from the FIFO for the endpoint 4.

The register provides word, half word, and byte mode accesses. If word or half word accesses are performed, the less significant byte will correspond to the prior byte to load to or unload from the FIFO.

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | FIFO4_DB0 | The first byte to be loaded to or unloaded from the FIFO. |

- USB_EP4_FIFO_ DB1

USB Endpoint 4 FIFO Register DB1	00

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FIFO4_DB1 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | FIFO4_DB1 | The second byte to be loaded to or unloaded from the FIFO. |

- USB_EP4_FIFO_ DB2

USB Endpoint 4 FIFO Register DB2	00

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FIFO4_DB2 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | FIFO4_DB2 | The third byte to be loaded to or unloaded from the FIFO. |

| 33 | USB_EP4_FIFO_ USB Endpoint 4 FIFO Register DB3 | 00 |
| --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| DB3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  | FIFO4_DB3 |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 7:0 | FIFO4_DB3 | The forth byte to be loaded to or unloaded from the FIFO. |

240	USB_CON	USB PHY Control	20

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  | NULL PKT_ FIX |  |  |  | DMP ULLU P | DPP ULLU P |
| Type |  |  |  |  |  |  |  |  |  |  | RW |  |  |  | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  | 0 | 0 |

| Bit(s) | Name | Description |
| --- | --- | --- |
| 5 | NULLPKT_FIX | If NULLPKT_FIX is set to 1, the USB controller will not issue a DMAreq when a null packet is received. |
| 1 | DMPULLUP | Pull-up enabling pin. Enables the pull-up 1.5KOhm pull-up on D pin as a full speed device by setting it to high |
| 0 | DPPULLUP | Pull-up enabling pin. Enables the pull-up 1.5KOhm pull-up on D+ pin as a full speed device by setting it to high |

##### System Integration Guide

###### USB device configuration

The target audience of this section is the software engineer.

The USB device controller features one control endpoint and 6 other endpoints. The configuration of interfaces and endpoints can be accommodated by software for specific functions, basically supporting Bluetooth HCI, and device firmware upgrade.

Bluetooth HCI transport layer defines the configuration of endpoints and interfaces.

- One voice channel with 16-bit encoding.

| Endpoint | Endpoint type | Max. packet size (bytes) | Max. bandwidth (bytes/ms) | Min. bandwidth (bytes/ms) | Double buffer in controller | Generic DMA |
| --- | --- | --- | --- | --- | --- | --- |
| Endpoint 0 (command) | Control | 64 |  |  | No | No |
| Endpoint 1 IN | Bulk (IN) | 64 |  | 1024 | No | Yes |
| Endpoint 2 OUT | Bulk (OUT) | 64 |  | 1024 | No | Yes |
| Endpoint 2 IN | Bulk (IN) | 64 | 1024 |  | No | Yes |
| Endpoint 2 OUT | Bulk (OUT) | 64 | 1024 |  | No | Yes |
| Endpoint 3 IN | Interrupt (IN) | 16 |  | 16 | No | No |
| Endpoint 4 IN | Interrupt (IN) | 16 |  | 16 | No | No |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

*When the maximum packet size is smaller than one half of the device FIFO size (64 bytes), the double buffer will be automatically enabled by hardware.

The pull-up resister on the USB transceiver is initially disconnected when boot-up. No external resistor is required. The software should enable it after performing the configuration of the USB device controller.

###### System Infrastructure Configuration

The clock, interrupt and DMA are defined as the system infrastructure. It requires several steps to bring up the USB. Those steps should be done in sequence to prevent from malfunction.

Power-on

- Enable USB PLL.

- Enable USB clock after USB PLL is settled.

- Unmask the USB interrupt in the interrupt controller.

- Enable the pull-up resistor.

The USB device controller can generate the interrupt when conditions are met as defined in USB_INTRINE, USB_INTROUTE, and USB_INTRUSBE.

The generic DMA controller is used to move data from or to the USB device controller. The USB device controller will use at most 4 DMA channels for ACL and SCO. The user should use the half-channel DMA since only the half channel DMA has the hardware flow control.

The USB FIFO provides byte and word accesses to the read/write port of USB_EP0_FIFO, USB_EP1_FIFO, USB_EP2_FIFO, USB_EP3_FIFO and USB_EP4_FIFO. If the data buffer allocated in memory is word aligned, the user can enable word transfer in the DMA controller. If the data buffer allocated in memory is not word aligned, the user should set to byte aligned and set B2W in DMAx_CON to 1 to enable fast byte-to-word transfer.

###### Power On/Off USB PHY and Controller Sequence

Power-on sequence after plug-in

- Turn on Vusb(PHY 3.3v power) – The control register is in PMIC document.

- Turn on USB AHB clock(78MHz) – The control register is in config document.

- Turn on internal 48MHz PLL – the control register is in clock document.

- Wait for 50 usec. (PHY 3.3v power stable time)

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

- Turn on USB PHY BIAS current control  reg[USB+08C1h] bit3 = 1. (RG_USB11_FSLS_ENBGRI).

- Wait for 10 usec.

- Set up D+ pull up register for connecting Host  reg[USB+0240h] bit 0 =1(PUB)

Power-off sequence after plug-out

- Release D+ pull up register for disconnecting Host  Setting reg[USB+0240h] bit 0 =0 (PUB)

- Turn off USB PHY BIAS current control  reg[USB+08C1h] bit3 = 0. (RG_USB11_FSLS_ENBGRI).

- Turn off Vusb (PHY 3.3v power) – The control register is in PMIC document

### Accessory Detector

##### General Description

The hardware accessory detector (ACCDET) detects plug-in/out of multiple types of external components. Based on the suggested circuit (see Figure 41), this design supports 3 types of external components, which are microphone, hook-switch and TV-OUT line. This design uses the internal 2-bit comparator to separate external components. The de-bounce scheme is also supported to resist uncertain input noises. When the plug-in/out state is stable, the PWM unit of ACCDET will enable the comparator, MBIAS and threshold voltage of the comparator periodically for the plugging detection.

With suitable PWM settings, very low-power consumption can be achieved when the detection feature is enabled. In order to compensate the delay between the detection login and comparator, the delay enabling scheme is adopted. Given the suitable delay number compared to the rising edge of PWM high pulse, the stable plugging state can be prorogated to digital detection logic. Then the correct plugging state can be detected and reported.

Figure 42 shows the state machine without TV-OUT mode. The state machine is executed by the software to control the ACCDET design. The ACCDET design will send one interrupt to acknowledge the software after the ACCDET input state is changed and the duration of the state is longer than

de-bounce time. The software needs to read out the memorized ACCDET input state and follow the recommend state machine to program the register in it.

B

TVOUT

75

MBias_clk

47p

37.5Ohm

AccDetection Logic

RG_AnaSwCtrl

540k

1.2V =32mA  Itvdac

10k

100n

10k

Brown: Register Red: Digital Signal

10k

VIN1_N

RG_TV_Det

2k

Vth_clk

2k

MICBIASP

1.9V

110k

2k

47p

1.77V

0.4V

+

-

2k

1220k

CMP_clk

350k

Pploy w=0.5u

AnaSwitch

VIN1_P RG_VPWDB_MBIAS

1

100n

+

-

0

A

AccDetect

JackIn

1.8u

Figure 41. Suggested Accessory Detection Circuit.

RG_VPWDB_MBIAS=0

means Microphone activated; RG_VPWDB_MBIAS=1

means not activated

A=1 B=1

Standby

(not plugged)

RG_VPWDB_MBAIS=

0 or 1 MicBias_clk=on/off Vth_clk=on/off CMP_clk=on/off

Always: MicBias_clk=on/off Vth_MB_clk=on/off CMP_clk=on/off

A=0 B=1

MIC

RG_VPWDB_MBAIS=

0 or 1 MicBias_clk=on/off Vth_clk=on/off CMP_clk=on/off

A=0

A=0,B=0

B=1

A=0 B=0

A=1 B=1

Hook Switched

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

RG_VPWDB_MBAIS=

0 or 1 MicBias_clk=on/off Vth_clk=on/off CMP_clk=on/off

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Figure 42. The Sate machine between Microphone and Hook-Switch plug-in/out change.

##### Pulse Width Modulation

The ACCDET design also provides one Pulse-Width-Modulation (PWM) to enable the comparator, microphone’s bias current and the threshold voltage of the comparator periodically. With suitable PWM and settings for delayed enabling, the ACCDET can achieve very low power consumption and accurate plug-in/out detection. Figure 43 shows a timing diagram example of such PWM design. The output from PWM keeps being at “0” until the value of the counter is smaller than the programmed threshold.

PWM_WIDTH

Internal Counter Threshold

PWM Signal

Figure 43. PWM waveform.

##### Register Definition

Module name: ACCDET base address: (+A0750000h)

| Address | Name | Width | Register function |
| --- | --- | --- | --- |
| A0750000 | ACCDET_RSTB | 32 | ACCDET software reset register |
| A0750004 | ACCDET_CTRL | 32 | ACCDET control register |
| A0750008 | ACCDET_STATE_SWCTRL | 32 | ACCDET state switch control register |
| A075000C | ACCDET_PWM_WIDTH | 32 | ACCDET PWM width register |
| A0750010 | ACCDET_PWM_THRESH | 32 | ACCDET PWM threshold register |
| A0750024 | ACCDET_EN_DELAY_NUM | 32 | ACCDET enable delay number register |
| A0750028 | ACCDET_PWM_IDLE_VALU E | 32 | ACCDET PWM IDLE value register |
| A075002C | ACCDET_DEBOUNCE0 | 32 | ACCDET debounce0 register |
| A0750030 | ACCDET_DEBOUNCE1 | 32 | ACCDET debounce1 register |
| A0750038 | ACCDET_DEBOUNCE3 | 32 | ACCDET debounce3 register |
| A075003C | ACCDET_IRQ_STS | 32 | ACCDET interrupt status register |
| A0750040 | ACCDET_CURR_IN | 32 | ACCDET current input status register |
| A0750044 | ACCDET_SAMPLE_IN | 32 | ACCDET sampled input status register |
| A0750048 | ACCDET_MEMOIZED_IN | 32 | ACCDET memorized input status register |
| A075004C | ACCDET_LAST_MEMOIZED _IN | 32 | ACCDET last memorized input status register |
| A0750050 | ACCDET_FSM_STATE | 32 | ACCDET FSM status register |
| A0750054 | ACCDET_CURR_DEBOUNC E | 32 | ACCDET current de-bounce status register |

| Address | Name | Width | Register function |
| --- | --- | --- | --- |
| A0750058 | ACCDET_VERSION | 32 | ACCDET version code |
| A075005C | ACCDET_IN_DEFAULT | 32 | default value of accdet_in |

A0750000	ACCDET_RSTB ACCDET Software Reset Register	00000001

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RSTB |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |

Overview: After applying the setting to register, software reset is necessary for state initialization. Without this process, ACCDET may detect incorrect plug state.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 0 | RSTB | RSTB | Set to 0 to reset the ACCDET unit and set to 1 after the reset process is finished. This software reset will clear ACCDET's enable signal but keep all ACCDET's settings. After the reset process, ACCDET will return to the IDLE state. |

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ACCD ET_E N |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |

A0750004	ACCDET_CTRL ACCDET Control Register	00000001

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 0 | ACCDET_E N | EN | Set to 1 to enable the ACCDET unit. |

A0750008	ACCDET_STAT

E_SWCTRL

ACCDET State Switch Control Register	00000001

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  |  | MBIA S_PW M_EN | VTH_ PWM_ EN | CMP_ PWM_ EN |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  | RW | RW | RW |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 |  |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 4 | MBIAS_PW M_EN | MBIAS_PWM_EN | Enables PWM of ACCDET MBIAS unit |
| 3 | VTH_PWM_ EN | VTH_PWM_EN | Enables PWM of ACCDET voltage threshold unit |
| 2 | CMP_PWM_ EN | CMP_PWM_EN | Enables PWM of ACCDET comparator |

A075000C	ACCDET_PWM_

WIDTH

ACCDET PWM Width Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Mne | PWM_WIDTH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:0 | PWM_WIDT H | PWM_WIDTH | ACCDET PWM width It is PWM max. counter value. It will be the initial value for the internal counter. The PWM internal counter always counts down to zero to finish one compete period, and the value of the internal counter will return to the value of PWM_WIDTH. PWM output frequency = (32k/PWM_WIDTH) Hz. |

A0750010	ACCDET_PWM_

THRESH

ACCDET PWM Threshold Register	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Mne | PWM_THRESH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:0 | PWM_THRE SH | PWM_THRESH | ACCDET PWM threshold When the internal counter value is bigger than or equal to PWM_THRESH, the PWM output signal will be "0". When the internal |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | counter is smaller than PWM_THRESH, the PWM output signal will be "1". PWM output duty cycle = (PWM_THRESH)x(1/32) ms. |

Figure 44 shows the PWM waveform with register value present.

Stable Detection Sate

PWM WIDTH =10

PWM THRES =6

DELAY_NUM =2

Stable Detection Sate

Stable Detection Sate

Figure 44. PWM waveform with register value present

A0750024	ACCDET_EN_D

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Mne | FALL_ DELA Y_NU M | RISE_DELAY_NUM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |

ELAY_NUM

ACCDET Enable Delay Number Register	00000101

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | FALL_DELA Y_NUM | FALL_DELAY_NU M | Falling delay cycle compared to CMP PWM waveform In order to make sure the plug state is stable after disabling ACCDET, the suitable delay cycle is necessary. This number indicates the clock cycle number between the point when the digital part of ACCDET stops receiving accdet_in and the point when the analog part of ACCDET stops working. |
| 14:0 | RISE_DELA Y_NUM | RISE_DELAY_NU M | Rising delay cycle compared to PWM waveform In order to make sure the plug state is stable before activating ACCDET, the suitable delay cycle is necessary. This number indicates the clock cycle number between the point when the analog part of ACCDET starts working and the point when the digital part of ACCDET starts receiving stable accdet_in. This number should be fine tuned depending on different project requirements. |

A0750028	ACCDET_PWM_

IDLE_VALUE

ACCDET PWM IDLE Value Register	00000001

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  | MBIAS | VTH | CMP |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | RW | RW | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 2 | MBIAS | MBIAS | IDLE value of MBIAS PWM |
| 1 | VTH | VTH | IDLE value of VTH PWM |
| 0 | CMP | CMP | IDLE value of CMP PWM |

A075002C	ACCDET_DEBO

UNCE0

ACCDET Debounce0 Register	00000010

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Mne | DEBOUNCE0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |

Overview: This register defines the waiting period before plug-in/out or release events are considered stable. If the de-bounce setting is too small, the plug-in/out will be too sensitive and detect too many unexpected

plug-ins/outs. The suitable de-bounce time setting must be adjusted for the user's demand.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:0 | DEBOUNCE 0 | DEBOUNCE0 | De-bounce time control of the next state = 2'b00 De-bounce time = DEBOUNCE/32 ms |

A0750030	ACCDET_DEBO

UNCE1

ACCDET Debounce1 Register	00000010

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Mne | DEBOUNCE1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |

Overview: This register defines the waiting period before plug-in/out or release events are considered stable. If the de-bounce setting is too small, the plug-in/out will be too sensitive and detect too many unexpected

plug-ins/outs. The suitable de-bounce time setting must be adjusted for the user's demand.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:0 | DEBOUNCE 1 | DEBOUNCE1 | De-bounce time control of the next state = 2'b01 De-bounce time = DEBOUNCE/32 ms |

A0750034	ACCDET_DEBO

UNCE2

ACCDET Debounce2 Register	00000010

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Mne | DEBOUNCE2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |

Overview: This register defines the waiting period before plug-in/out or release events are considered stable. If the de-bounce setting is too small, the plug-in/out will be too sensitive and detect too many unexpected

plug-ins/outs. The suitable de-bounce time setting must be adjusted for the user's demand.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:0 | DEBOUNCE 2 | DEBOUNCE2 | De-bounce time control of the next state = 2'b10 De-bounce time = DEBOUNCE/32 ms |

A0750038	ACCDET_DEBO

UNCE3

ACCDET Debounce3 Register	00000010

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Mne | DEBOUNCE3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |

Overview: This register defines the waiting period before plug-in/out or release events are considered stable. If the de-bounce setting is too small, the plug-in/out will be too sensitive and detect too many unexpected

plug-ins/outs. The suitable de-bounce time setting must be adjusted for the user's demand.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:0 | DEBOUNCE 3 | DEBOUNCE3 | De-bounce time control of the next state = 2'b11 De-bounce time = DEBOUNCE/32 ms |

A075003C	ACCDET_IRQ_STS	ACCDET Interrupt Status Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Mne |  |  |  |  |  |  |  | IRQ_C LR |  |  |  |  |  |  |  | IRQ |
| Type |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |  | RW |
| Reset |  |  |  |  |  |  |  | 0 |  |  |  |  |  |  |  | 0 |

Overview: When the interrupt of ACCDET is asserted, IRQ_CLR must be set to 1 to clear the interrupt status. This bit will pause all activities in the ACCDET design until both interrupt status and IRQ_CLR are cleared. The software should write 1 to IRQ_CLR first to clear the interrupt (IRQ). After that, if pclk gating is enabled, the software should read ACCDET_IRQ_STS again to make IRQ_CLR self-reset to 0.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 8 | IRQ_CLR | IRQ_CLR | Clears interrupt status of ACCDET unit |
| 0 | IRQ | IRQ | Interrupt status of ACCDET unit Because this register will be cleared by hardware, the interrupt edge-sensitive scheme should be adopted for this design. |

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CURR_IN |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RO |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |

A0750040	ACCDET_CURR_IN ACCDET Current Input Status Register	00000003

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 1:0 | CURR_IN | CURR_IN | Current input status of ACCDET unit |

A0750044	ACCDET_SAMP

LE_IN

ACCDET Sampled Input Status Register	00000003

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SAMPLE_IN |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RO |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 1:0 | SAMPLE_IN | SAMPLE_IN | Samples input status of ACCDET unit When the plug-in/out state is changed, the ACCDET unit will do sampling. |

A0750048	ACCDET_MEM

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MEMORIZED _IN |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RO |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |

OIZED_IN

ACCDET Memorized Input Status Register	00000003

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 1:0 | MEMORIZE D_IN | MEMORIZED_IN | Memorized input status of ACCDET unit When the plug-in/out states is changed and held longer than the de-bounce time, the ACCDET unit will save the sampled input state to the memorized state. The interrupt will also be asserted to acknowledge the software. |

A075004C	ACCDET_LAST

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  |  | LAST_MEMO RIZED_IN |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RO |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |

_MEMOIZED_IN

ACCDET Last Memorized Input Status Register	00000003

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 1:0 | LAST_MEM ORIZED_IN | LAST_MEMORIZE D_IN | Last memorized input status of ACCDET unit |

A0750050	ACCDET_FSM_

STATE

ACCDET FSM Status Register	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  | FSM_STATE |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | RO |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 2:0 | FSM_STATE | FSM_SATE | State of ACCDET unit finite-state-machine 0: ACCDET_IDLE 1: ACCDET_SAMPLE 2: ACCDET_DEBOUNCE 3: ACCDET_CHECK 4: ACCDET_MEMORIZED 5: ACCDET_IRQ |

A0750054	ACCDET_CURR

_DEBOUNCE

ACCDET Current De-bounce Status Register	00000004

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mne |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Mne | CURR_DEBOUNCE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15:0 | CURR_DEB OUNCE | CURR_DEBOUNC E | Currently used de-bounce time setting |

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ACCDET_ VERSION |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RO |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |

A0750058	ACCDET_VERSION	ACCDET Version Code	000000 03

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 1:0 | ACCDET_ VERSION | ACCDET_VERSI ON | Version code for ACCDET |

A075005C	ACCDET_IN_

DEFAULT

Default Value of accdet_in	00000000

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  |  |  |  | ACC |  |  | ACCDET_IN |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

|  |  |  |  |  |  |  |  |  |  |  |  | DET_ IN_D EFA ULT_ REF RES H_E N |  |  | _DEFAULT |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type |  |  |  |  |  |  |  |  |  |  |  | RW |  |  | RW |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  | 0 |  |  | 0 | 0 |

Overview: The default value of sample_accdet_in and memorised_accdet_in can be set by software instead of using the default value set by hardware(i.e. 3). ACCDET_DEFAULT_REFRESH_EN is the enable bit controlling whether to use this additional function. The value of sample_accdet_in and memorized_accdet_in will change when accdet_en rises from low to high. Note that if software reset is applied when accdet_en is high, the default value of sample_accdet_in and memorized_accdet_in will also be loaded when the software reset is de-asserted.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 4 | ACCDET_I N_DEFAU LT_REFR ESH_EN | ACCDET_IN_DE FAULT_REFRES H_EN | Enable signal for whether to load accdet_in_default 0: accdet_in_default will not be loaded. 1: accdet_in_default will be loaded. |
| 1:0 | ACCDET_I N_DEFAU LT | ACCDET_IN_DE FAULT | Default value of accdet_in set by software |

### SD Memory Card Controller (MSDC0)

##### Introduction

The controller fully supports the SD memory card bus protocol as defined in SD Memory Card Specification Part 1 Physical Layer Specification version 2.0.

Furthermore, the controller also partially supports the SDIO card specification version 2.0. However, the controller can only be configured as the host of the SD memory card. Hereafter, the controller is also abbreviated as the SD controller. The following are the main features of the controller.

- Interface with MCU by APB bus

- 16/32-bit access on APB bus

- 16/32-bit access for control registers

- 32-bit access for FIFO

- Built-in 32 bytes FIFO buffers for transmit and receive, FIFO is shared for transmit and receive

- Built-in CRC circuit

- CRC generation can be disabled

- DMA supported

- Interrupt capabilities

- Data rate up to 48 Mbps in serial mode, 48x4 Mbps in parallel model, the module is targeted at 48 MHz operating clock

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Serial clock rate on SD bus is programmable

- Card detection capabilities during sleep mode

- Controllability of power for memory card

- Does not support SPI mode for SD memory card

- Does not support multiple SD memory cards

##### Overview

###### Pin Assignment

The following lists pins required for the SD memory card. Table 50 shows how the pins are shared. Note that all I/O pads have embedded both pull-up and pull-down resistors because they are shared by the SD memory card. The pull-down resistors for these pins can be used for power saving. If optimal pull-up or pull-down resistors are required on the system board, all embedded pull-up and pull-down resistors can be disabled by programming the corresponding control registers. The VDDPD pin is used for power saving. Power for the SD memory card can be shut down by programming the corresponding control register. The WP (Write Protection) pin is used to detect the status of the Write Protection Switch on the SD memory card.

Table 50. Sharing of pins for SD memory card controller

| No. | Name | Type | MMC | SD | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | SD_CLK | O | CLK | CLK | Clock |
| 2 | SD_DAT3 | I/O/PP |  | CD/DAT3 | Data Line [Bit 3] |
| 3 | SD_DAT0 | I/O/PP | DAT0 | DAT0 | Data Line [Bit 0] |
| 4 | SD_DAT1 | I/O/PP |  | DAT1 | Data Line [Bit 1] |
| 5 | SD_DAT2 | I/O/PP |  | DAT2 | Data Line [Bit 2] |
| 6 | SD_CMD | I/O/PP | CMD | CMD | Command Or Bus State |
| 7 | SD_PWRON | O |  |  | VDD ON/OFF |
| 8 | SD_WP | I |  |  | Write Protection Switch in SD |
| 9 | SD_INS | I | VSS2 | VSS2 | Card Detection |

###### Card Detection

For SD memory card, detection of card insertion/removal by hardware is supported, and a dedicated pin “INS” is used to perform card insertion and removal for SD. The pin “INS” will be connected to the pin “VSS2” of a SD connector (see Figure 45).

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

HOST

CARD

RPU

SW1

PAD

INS

RPD

SW2

INS_IN

Figure 45. Card detection for SD memory card

##### Register Definition

Module name: MSDC0 base address: (+A0130000h)

| Address | Name | Width | Register function |
| --- | --- | --- | --- |
| A0130000 | MSDC_CFG | 32 | SD memory card controller configuration register For general configuration of the SD controller. Note: MSDC_CFG[31:16] can be accessed by 16-bit APB bus access. |
| A0130004 | MSDC_STA | 32 | SD memory card controller status register Contains the status of FIFO, interrupts and data requests. |
| A0130008 | MSDC_INT | 32 | SD memory card controller interrupt register Contains the status of interrupts. Note that the register still shows the status of interrupt even though the interrupt is disabled, that is, register bit INTEN of register MSDC_CFG is set to 0. It implies that software interrupt can be implemented by polling register bit INT of register MSDC_STA and this register. However, if hardware interrupt is desired, be sure to clear the register before setting up register bit INTEN of register MSDC_CFG to 1, or undesired hardware interrupt arisen from the previous interrupt status may take place. |
| A013000C | MSDC_PS | 32 | SD memory card pin status register Used for card detection. When the memory card |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Address | Name | Width | Register function |
| --- | --- | --- | --- |
|  |  |  | controller and system are powered on, the power for the memory card will still be off unless the power is supplied by the PMIC. Meanwhile, the pad for card detection defaults to pull down when the system is powered on. The scheme of card detection for MS is the same as that for SD. For detecting card insertion, first pull up the INS pin and then enable card detection and the input pin at the same time. After 32 cycles of controller clock, the status of pin changes will emerge. To detect card removal, simply keep enabling card detection and the input pin. |
| A0130010 | MSDC_DAT | 32 | SD memory card controller data register Reads/Writes data from/to FIFO inside SD controller. Data access unit: 32 bits |
| A0130014 | MSDC_IOCON | 32 | SD memory card controller IO control register Specifies output driving capability and slew rate of IO pads for MSDC. The reset value is suggested setting. If the output driving capability of pins DAT0, DAT1, DAT2 and DAT3 is too large, it is possible to arise ground bounce and thus result in glitch on SCLK. The actual driving current will depend on the PAD type selected for the chip. |
| A0130018 | MSDC_IOCON1 | 32 | SD memory card controller IO control register 1 |
| A0130020 | SDC_CFG | 32 | SD memory card controller configuration register Configures the SD memory card controller when it is configured as the host of SD. If the controller is configured as the host of memory stick, the contents of the register will have no impact on the operation of the controller. Note: SDC_CFG[31:16] can be accessed by 16-bit APB bus access. |
| A0130024 | SDC_CMD | 32 | SD memory card controller command register Defines a SD memory card command and its attribute. Before the SD controller issues a transaction onto the SD bus, application shall specify other relative settings such as argument for command. After application writes the register, the SD controller will issue the corresponding transaction onto SD serial bus. If the command is GO_IDLE_STATE, the controller will have serial clock on SD bus run 128 cycles before issuing the command. |
| A0130028 | SDC_ARG | 32 | SD memory card controller argument register Contains argument of the SD memory card command. |
| A013002C | SDC_STA | 32 | SD memory card controller status register Contains various statuses of SD controller as the controller is configured as the host of SD memory card. |
| A0130030 | SDC_RESP0 | 32 | SD memory card controller response register 0 Contains parts of the last SD memory card bus response. See descriptions of register field |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Address | Name | Width | Register function |
| --- | --- | --- | --- |
|  |  |  | SDC_RESP3. |
| A0130034 | SDC_RESP1 | 32 | SD memory card controller response register 1 Contains parts of the last SD memory card bus response. See descriptions of register field SDC_RESP3. |
| A0130038 | SDC_RESP2 | 32 | SD memory card controller response register 2 Contains parts of the last SD memory card bus response. See descriptions of register field SDC_RESP3. |
| A013003C | SDC_RESP3 | 32 | SD memory card controller response register 3 Contains parts of the last SD memory card bus response. Register fields SDC_RESP0, SDC_RESP1, SDC_RESP2 and SDC_RESP3 compose the last SD memory card bus response. For response of type R2, that is, response of the command ALL_SEND_CID, SEND_CSD and SEND_CID, only bit 127 to 0 of the response token are stored in register field SDC_RESP0, SDC_RESP1, SDC_RESP2 and SDC_RESP3. For responses of other types, only bit 39 to 8 of the response token are stored in register field SDC_RESP0. |
| A0130040 | SDC_CMDSTA | 32 | SD memory card controller command status register Contains the status of SD controller during command execution and that of SD bus protocol after command execution when the SD controller is configured as the host of SD memory card. The register will also be used as the interrupt source. The register is cleared when being read. Meanwhile, if interrupt is enabled and thus interrupt caused by the register is generated, reading the register will de-assert the interrupt. |
| A0130044 | SDC_DATSTA | 32 | SD memory card controller data status register Contains the status of SD controller during data transfer on DAT line(s) when the SD controller is configured as the host of SD memory card. The register is also used as the interrupt source. The register is cleared when being read. Meanwhile, if interrupt is enabled and thus interrupt caused by the register is generated, reading the register will de-assert the interrupt. |
| A0130048 | SDC_CSTA | 32 | SD memory card status register After commands with R1 and R1b response this register containing the status of the SD card, it will be used as the response interrupt source. In all register fields, logic high indicates error, and logic low indicates there is no error. The register is cleared when being read. Meanwhile, if interrupt is enabled and thus interrupt caused by the register is generated, reading the register will de-assert the interrupt. |
| A013004C | SDC_IRQMASK0 | 32 | SD memory card IRQ mask register 0 Contains parts of SD memory card interrupt mask |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Address | Name | Width | Register function |
| --- | --- | --- | --- |
|  |  |  | register. See the descriptions of register SDC_IRQMASK1 for reference. The register masks interrupt sources from register SDC_CMDSTA and SDC_DATSTA. IRQMASK[3:0] is for SDC_CMDSTA, and IRQMASK[18:16] for SDC_DATSTA. Note that IRQMASK[18] masks SDC_DATSTA[9:2] together. ‘1’ in some bits of the register masks the corresponding interrupt source with the same bit position. For example, if IRQMASK[0] is 1, then the interrupt source from register field CMDRDY of register SDC_CMDSTA will be masked. '0' in some bits does not cause interrupt mask on the corresponding interrupt source from register SDC_CMDSTA and SDC_DATSTA. |
| A0130050 | SDC_IRQMASK1 | 32 | SD memory card IRQ mask register 1 Contains parts of SD memory card interrupt mask register. Registers SDC_IRQMASK1 and SDC_IRQMASK0 compose the SD memory card interrupt mask register. The register masks interrupt sources from register SDC_CSTA. '1' in some bits of the register masks the corresponding interrupt source with the same bit position. For example, if IRQMASK[63] is 1, then interrupt source from register field OUT_OF_RANGE of register SDC_ CSTA will be masked. '0' in some bit does not cause interrupt mask on the corresponding interrupt source from register SDC_CSTA. |
| A0130054 | SDIO_CFG | 32 | SDIO configuration register Configures functions for SDIO. |
| A0130058 | SDIO_STA | 32 | SDIO status register Identifies if thers is SDIO interrupt during the interrupt period on data line. |
| A0130080 | CLK_RED | 32 | CLK latch configuration register Configures the MSDC sample data/response clock. Note: When MSDC_IOCON[19] = 1, the host will latch response; otherwise MSDC FSM will handle the response from PAD directly. |
| A0130098 | DAT_CHECKSUM | 32 | MSDC Rx data checksum register Competes the checksum value of Rx read data |

A0130000	MSDC_CFG	SD Memory Card Controller Configuration Register	04000020

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  | FIFOTHD |  |  |  |  |  | VDDP D | RCDE N | DIRQE N | PINEN | DMAE N | INTEN |
| Type |  |  |  |  | RW |  |  |  |  |  | RW | RW | RW | RW | RW | RW |
| Reset |  |  |  |  | 0 |  | 0 | 0 |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | SCLKF |  |  |  |  |  |  |  | SCLK ON | CRED | STDB Y | CLKSRC |  | NOCR C | RST | MSDC |
| Type | RW |  |  |  |  |  |  |  | RW | RW | RW | RW |  | RW | W1C | RW |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Overview: For general configuration of the SD controller. Note that MSDC_CFG[31:16] can be accessed by 16-bit APB bus access.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 27:24 | FIFOTHD | FIFOTHD | FIFO threshold The register field determines when to issue a DMA request. For write transactions, DMA requests will be asserted if the number of free entries in FIFO are bigger than or equal to the value in the register field. For read transactions, DMA requests will be asserted if the number of valid entries in FIFO are bigger than or equal to the value in the register field. The register field must be set according to the setting of data transfer count in DMA burst mode. If single mode for DMA transfer is used, the register field shall be set to 0b0001. 0000: Invalid 0001: Threshold value is 1. 0010: Threshold value is 2. 0011~01111: ... 1000: Threshold value is 8. Others: Invalid |
| 21 | VDDPD | VDDPD | Controls output pin VDDPD used for power saving Output pin VDDPD controls the power for memory card. 0: Output pin VDDPD outputs logic low. The power for memory card will be turned off. 1: Output pin VDDPD outputs logic high. The power for memory card will be turned on. |
| 20 | RCDEN | RCDEN | Controls output pin RCDEN used for card identification process when the controller is for SD memory card Its output controls the pull-down resistor on the system board to connect to or disconnect from signal CD/DAT3. 0: The output pin RCDEN outputs logic low. 1: The output pin RCDEN outputs logic high. |
| 19 | DIRQEN | DIRQEN | Enables data request interrupt The register bit is used to control if data request is used as an interrupt source. 0: Data request is not used as an interrupt source. 1: Data request is used as an interrupt source. |
| 18 | PINEN | PINEN | Enables pin interrupt The register bit is used to control if the pin for card detection is used as an interrupt source. 0: The pin for card detection is not used as an interrupt source. 1: The pin for card detection is used as an interrupt source. |
| 17 | DMAEN | DMAEN | Enables DMA Note: If DMA capability is disabled, the application software must poll the status of register MSDC_STA to check on any data transfer request. If DMA is desired, the register bit must be set up before command register is written. 0: DMA request induced by various conditions is disabled, no matter the controller is configured as the host of either SD memory card or memory stick. 1: DMA request induced by various conditions is enabled, no matter the controller is configured as the host of either SD memory card or memory stick. |
| 16 | INTEN | INTEN | Enables interrupt Note: If interrupt capability is disabled, the application software must |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | poll the status of register MSDC_STA to check on any interrupt request. 0: Interrupt induced by various conditions is disabled, no matter the controller is configured as the host of either SD memory card or memory stick. 1: Interrupt induced by various conditions is enabled, no matter the controller is configured as the host of either SD memory card or memory stick. |
| 15:8 | SCLKF | SCLKF | Controls clock frequency of serial clock on SD bus and denotes clock frequency of SD bus serial clock as fslave and clock frequency of the SD controller as fhost which is 98.3 or 96.2 MHz Note: The allowed maximum frequency of fslave is 49.15MHz. While changing the clock rate, "1T clock period before change + 1T clock period after change" is required for HW signal to re-synchronize. 00000000b: fslave =(1/2)*fhost 00000001b: fslave = [1/(4*1)]*fhost 00000010b: fslave = [1/(4*2)]*fhost 00000011b: fslave = [1/(4*3)]*fhost 00000100b~111111110b: ... 11111111b: fslave = [1/(4*255)]*fhost |
| 7 | SCLKON | SCLKON | Serial clock always on For debugging. 0: Serial clock not always on 1: Serial clock always on |
| 6 | CRED | CRED | Rising edge data The register bit is used to determine the serial data input is latched at the falling edge or rising edge of the serial clock. The default setting is at the rising edge. If the serial data have bad timing, set the register bit to 1. When the memory card has bad timing on returned read data, set the register bit to 1. 0: Serial data input is latched at the rising edge of serial clock. 1: Serial data input is latched at the falling edge of serial clock. |
| 5 | STDBY | STDBY | Standby mode If the module is powered down, operating clock to the module will be stopped. At the same time, the clock to card detection circuitry will also be stopped. If detection on memory card insertion and removal is desired, write 1 to the register bit. If interrupt for detection on memory card insertion and removal is enabled, the interrupt will take place whenever the memory is inserted or removed. 0: Standby mode is disabled. 1: Standby mode is enabled. |
| 4:3 | CLKSRC | CLKSRC | Specifies which clock is used as source clock of memory card 00 : MPLL/5.5MHz clock 01 : MPLL/7MHz clock (this divider is default off, before switch controller source clock to this clock, you should switch controller source clock to 26Mhz first, and then enable divider MPLL/7, after a moment you can switch controller source clock to MPLL/ 7 to void source clock glitch. For detail setting please refer to confgsys document) - : MPLL/8MHz clock - : MPLL/10MHz clock For phone 00 : 94.5MHz clock Need to keep BT_APP_DIV_EN= 1'b0 in CLK_CONDA[15]. 01 : 74.3MHz clock NOTE: Need to set POWERFUL_DIV_EN1 = 1'b1 first in CLK_CONDA[10]. - : 65MHz clock - : Forbidden |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | For BT app. 00 : Forbidden 01 : 89.1MHz clock NOTE: Need to set POWERFUL_DIV_EN1 = 1'b1 first in CLK_CONDA[10]. - 78MHz clock - : 62.4MHz clock NOTE: Need to set POWERFUL_DIV_EN2 = 1'b1 first in CLK_CONDA[9]. |
| 2 | NOCRC | NOCRC | Disable CRC '1' indicates data transfer without CRC is desired. For write data block, the data are transmitted without CRC. For read data block, CRC will not be checked. For testing purpose. 0: Data transfer with CRC is desired. 1: Data transfer without CRC is desired. |
| 1 | RST | RST | Software reset Writing 1 to the register bit will cause internal synchronous reset of SD controller but will not reset register settings, RST should only be set when RST equal to 0. 0: Read 0 stands for the reset process is finished. 1: Write 1 to reset SD controller. |
| 0 | MSDC | MSDC | Configures the controller as SD memory card mode CLK/CMD/DAT line is pulled low when SD memory card mode is disable. 0: Disable SD memory card 1: Enable SD memory card |

A0130004	MSDC_STA	SD Memory Card Controller Status Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | BUSY | FIFOC LR |  |  |  |  |  |  | FIFOCNT |  |  |  | INT | DRQ | BE | BF |
| Type | R | W1C |  |  |  |  |  |  | RO |  |  |  | RO | RO | RO | RO |
| Reset | 0 | 0 |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: The register contains the status of FIFO, interrupts and data requests.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | BUSY | BUSY | Status of the controller If the controller is in busy state, the register bit will be 1; otherwise 0. 0: The controller is in busy state. 1: The controller is in idle state. |
| 14 | FIFOCLR | FIFOCLR | Clears FIFO Writing 1 to the register bit will cause the content of FIFO clear and reset the status of FIFO controller. 0: Read 0 stands for the FIFO clear process is finished. 1: Write 1 to clear the content of FIFO clear and reset the status of FIFO controller. |
| 7:4 | FIFOCNT | FIFOCNT | FIFO count The register field shows how many valid entries are there in FIFO. 0000: 0 valid entry in FIFO |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0001: 1 valid entry in FIFO 0010: 2 valid entries in FIFO 0011~0111: ... 1000: 8 valid entries in FIFO |
| 3 | INT | INT | Indicates if there is any interrupt existing When there is interrupt existing, the register bit will still be active even if register bit INTEN in register MSDC_CFG is disabled. The SD controller can interrupt MCU by issuing interrupt request to the interrupt controller, or the software/application will poll the register endlessly to check if there is any interrupt request existing in the SD controller. When register bit INTEN in register MSDC_CFG is disabled, the second method is used. For read commands, it is possible that time-out error takes place. The software can read the status register to check if the time-out error takes place without OS time tick support or data request asserted. Note: The register bit will be cleared when register MSDC_INT is read. 0: No interrupt request existing. 1: Interrupt request exists. |
| 2 | DRQ | DRQ | Indicates if any data transfer is required When a data transfer is required, the register bit will still be active even if register bit DIRQEN in register MSDC_CFG is disabled. Data transfer can be achieved by DMA channel alleviating MCU loading, or by polling the register bit to check if any data transfer is requested. When register bit DIRQEN in register MSDC_CFG is disabled, the second method is used. 0: No DMA request existing. 1: DMA request exists. |
| 1 | BE | BE | Indicates if FIFO in SD controller is empty 0: FIFO in SD controller is not empty. 1: FIFO in SD controller is empty. |
| 0 | BF | BF | Indicates if FIFO in SD controller is full 0: FIFO in SD controller is not full. 1: FIFO in SD controller is full. |

A0130008	MSDC_INT	SD Memory Card Controller Interrupt Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  | SDIOI RQ | SDR1 BIRQ |  | SDMC IRQ | SDDA TIRQ | SDCM DIRQ | PINIR Q | DIRQ |
| Type |  |  |  |  |  |  |  |  | RC | RC |  | RC | RC | RC | RC | RC |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 |

Overview: The register contains the status of interrupts. Note that the register still show status of interrupt even though interrupt is disabled, that is, register bit INTEN of register MSDC_CFG is set to 0. It implies that software interrupt can be implemented by polling register bit INT of register MSDC_STA and this register. However, if hardware interrupt is desired, be sure to clear the register before setting register bit INTEN of register MSDC_CFG to 1, or undesired hardware interrupt arisen from the previous interrupt status may take place.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7 | SDIOIRQ | SDIOIRQ | SDIO interrupt The register bit indicates if there is any interrupt for SDIO existing. Whenever an interrupt for SDIO exists, the register bit will be set to 1 if the interrupt is enabled. It will be reset when the register is read. 0: No SDIO interrupt 1: Interrupt for SDIO exists. |
| 6 | SDR1BIRQ | SDR1BIRQ | SD R1b response interrupt The register bit will be active when a SD command with R1b response is finished and the DAT0 line is transited from busy to idle state. Single block write commands with R1b response will cause interrupt when the command is completed either successfully or with CRC error. However, multi-block write commands with R1b response do not cause interrupt because multi-block write commands are always stopped by STOP_TRANS commands. STOP_TRANS commands (with R1b response) behind multi-block write commands will cause interrupt. Single block read command with R1b response will cause interrupt when the command is completed, but multi-block read commands do not. Note: STOP_TRANS commands (with R1b response) behind multi-block read commands will cause interrupt. 0: No interrupt for SD R1b response. 1: Interrupt for SD R1b response exists. |
| 4 | SDMCIRQ | SDMCIRQ | SD memory card interrupt The register bit indicates if there is any interrupt for SD memory card existing. Whenever an interrupt for SD memory card exists, i.e. any bit in register SDC_CSTA is active, the register bit will be set to 1 if interrupt is enabled. It will be reset when the register is read. Note: This bit will not trigger MSDC hardware interrupt. 0: No SD memory card interrupt 1: SD memory card interrupt exists. |
| 3 | SDDATIRQ | SDDATIRQ | SD bus DAT interrupt The register bit indicates if there is any interrupt for SD DAT line existing. Whenever interrupt for SD DAT line exists, i.e. any bit in register SDC_ DATSTA is active, the register bit will be set to 1 if interrupt is enabled. It will be reset when the register is read. 0: No SD DAT line interrupt 1: SD DAT line interrupt exists. |
| 2 | SDCMDIRQ | SDCMDIRQ | SD bus CMD interrupt The register bit indicates if there is any interrupt for SD CMD line existing. Whenever interrupt for SD CMD line exists, i.e. any bit in the register SDC_CMDSTA is active, the register bit will be set to 1 if interrupt is enabled. It will be reset when the register is read. 0: No SD CMD line interrupt 1: SD CMD line interrupt exists. |
| 1 | PINIRQ | PINIRQ | Pin change interrupt The register bit indicates if there is any interrupt for memory card insertion/removal existing. Whenever the memory card is inserted or removed and card detection interrupt is enabled, i.e. register bit PINEN in register MSDC_CFG is set to 1, the register bit will be set to 1. It will be reset when the register is read. 0: Otherwise 1: Card is inserted or removed. |
| 0 | DIRQ | DIRQ | Data request interrupt The register bit indicates if there is any interrupt for data request existing. Whenever data request exists and data request as an |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | interrupt source is enabled, i.e. register bit DIRQEN in register MSDC_CFG is set to 1, the register bit will be active. It will be reset when being read. For software, data requests can be recognized by polling register bit DRQ or by data request interrupt. Data request interrupts will be generated every FIFOTHD data transfers. 0: No data request interrupt 1: Data request interrupt occurs. |

A013000C	MSDC_PS	SD Memory Card Pin Status Register	00000008

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  | CMD | DAT |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  | RO | RO |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | CDDEBOUNCE |  |  |  |  |  |  |  |  |  |  | PINCH G | PIN0 | POEN 0 | PIEN0 | CDEN |
| Type | RW |  |  |  |  |  |  |  |  |  |  | RC | RO | RW | RW | RW |
| Reset | 0 | 0 | 0 | 0 |  |  |  |  |  |  |  | 0 | 1 | 0 | 0 | 0 |

Overview: The register is used for card detection. When the memory card controller and system are powered on, , the power for the memory card will still be off unless the power is supplied by the PMIC. Meanwhile, the pad for card detection defaults to pull-down when the system is powered on. The scheme of card detection for MS is the same as that for SD. To detect card insertion, first pull up the INS pin and then enable card detection and the input pin at the same time. After 32 cycles of controller clock, the status of pin changes will emerge. To detect card removal, simply keep enabling card detection and the input pin.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 24 | CMD | CMD | Memory card/SDIO card/MMC card command lines |
| 23:16 | DAT | DAT | Memory card/SDIO card/MMC card data lines |
| 15:12 | CDDEBOUN CE | CDDEBOUNCE | Specifies the time interval for card detection de-bounce Default value: 0. It means the de-bounce interval is 32-cycle time at 32kHz. The interval can extend one cycle time at 32kHz by increasing the counter by 1. |
| 4 | PINCHG | PINCHG | Pin change The register bit indicates the status of card insertion/removal. If the memory card is inserted or removed, the register bit will be set to 1 no matter pin change interrupt is enabled or not. It will be cleared when the register is read. 0: Otherwise 1: Card is inserted or removed. |
| 3 | PIN0 | PIN0 | Shows the value of input pin for card detection 0: The value of input pin for card detection is logic low. 1: The value of input pin for card detection is logic high. |
| 2 | POEN0 | POEN0 | Controls output of input pin for card detection 0: Output of input pin for card detection is disabled. 1: Output of input pin for card detection is enabled. |
| 1 | PIEN0 | PIEN0 | Controls input pin for card detection 0: Input pin for card detection is disabled. 1: Input pin for card detection is enabled. |
| 0 | CDEN | CDEN | Enables card detection |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | The register bit is used to enable or disable card detection. 0: Card detection is disabled. 1: Card detection is enabled. |

A0130010	MSDC_DAT	SD Memory Card Controller Data Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | DATA[31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | DATA[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: The register is used to read/write data from/to FIFO inside the SD controller. Data access unit: 32 bits.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31:0 | DATA | DATA | Reads/Writes data from/to FIFO inside SD controller Data access unit: 32 bits |

A0130014	MSDC_IOCON	SD Memory Card Controller IO Control Register	010000C3

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  | SAMPLEDLY |  | FIXDLY |  | SAMP ON | CRCDI S | CMDS EL | INTLH |  | DSW |
| Type |  |  |  |  |  |  | RW |  | RW |  | RW | RW | RW | RW |  | RW |
| Reset |  |  |  |  |  |  | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | CMDR E |  |  |  |  | HIGH_ SPEE D | DMABURST |  | SRCF G1 | SRCF G0 | ODCCFG1 |  |  | ODCCFG0 |  |  |
| Type | RW |  |  |  |  | RW | RW |  | RW | RW | RW |  |  | RW |  |  |
| Reset | 0 |  |  |  |  | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 |

Overview: The register specifies the output driving capability and slew rate of IO pads for MSDC. The reset value is suggested setting. If the output driving capability of pins DAT0, DAT1, DAT2 and DAT3 is too large, it is possible to arise ground bounce and thus result in glitch on SCLK. The actual driving current depends on the PAD type selected for the chip.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 25:24 | SAMPLEDL Y | SAMPLEDLY | Used for SW to select the turn-around delay cycle between write data end bit and CRC status for SD card 00: 0-T delay 01: 1-T delay 10: 2-T delay 11: 3-T delay |
| 23:22 | FIXDLY | FIXDLY | Used for SW to select the delay cycle after clock fix high for the host controller to SD card 00: 0-T delay 01: 1-T delay |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 10: 2-T delay 11: 3-T delay |
| 21 | SAMPON | SAMPON | Data sample enabling always on The bit is suggested to be set to 1 when the feedback clock is used and to 0 when the multiple phase clock is used. 0: Data sample enabling not always on 1: Data sample enabling always on |
| 20 | CRCDIS | CRCDIS | Switches off data CRC check for SD read data 0: CRC check is on. 1: CRC check is off. |
| 19 | CMDSEL | CMDSEL | Determines whether the host should delay 1-T to latch response from card 0: Host latches response without 1-T delay 1: Host latches response with 1-T delay. |
| 18:17 | INTLH | INTLH | Selects latch timing for SDIO multi-block read interrupt 00: Host latches INT at the second backend clock after the end bit of the current data block from card is received. (Default) 01: Host latches INT at the first backend clock after the end bit of the current data block from card is received. 10: Host latches INT at the second backend clock after the end bit of the current data block from card is received. 11: Host latches INT at the third backend clock after the end bit of the current data block from card is received. |
| 16 | DSW | DSW | Determines whether the host should latch data with 1-T delay or not For SD card, this bit is suggested to be 0. For MSPRO cards, it is suggested to be 1. 0: Host latches the data with 1-T delay. 1: Host latches the data without 1-T delay. |
| 15 | CMDRE | CMDRE | Determines whether the host should latch response token (sent from card on CMD line ) at rising edge or falling edge of serial clock (T.B.D this bit is un-useful) 0: Host latches response at rising edge of serial clock. 1: Host latches response at falling edge of serial clock. |
| 10 | HIGH_SPEE D | HIGH_SPEED | For high-speed mode when internal sample clock is used High-speed mode means the SD/MMC serial bus clock rate is bigger than 25MHz. The default speed mode means that the SD/MMC serial bus clock rate is bigger than 25MHz. 0: Default speed 1: High speed |
| 9:8 | DMABURST | DMABURST | Used for SW to select burst type when data are transferred by DMA Note: Only single mode can support non-4N bytes data transfer in read operation. 00: Single mode 01: 4-beat incrementing burst 10: 8-beat incrementing burst 11: Reserved |
| 7 | SRCFG1 | SRCFG1 | Output driving capability for pins DAT0, DAT1, DAT2 and DAT3 0: Fast slew rate 1: Slow slew rate |
| 6 | SRCFG0 | SRCFG0 | Output driving capability for pins CMD/BS and SCLK 0: Fast slew rate |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: Slow slew rate |
| 5:3 | ODCCFG1 | ODCCFG1 | Output driving capability for pins DAT0, DAT1, DAT2 and DAT3 000: 4mA 001: 8mA 010: 12mA 011: 16mA |
| 2:0 | ODCCFG0 | ODCCFG0 | Output driving capability for pins CMD/BS and SCLK 000: 4mA 001: 8mA 010: 12mA 011: 16mA |

A0130018	MSDC_IOCON1 SD Memory Card Controller IO Control Register 1	00022022

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  | PRCF G_CK | PRVAL_CK |  |  | PRCF G_CM | PRVAL_CM |  |  | PRCF G_DA | PRVAL_DA |  |  |  |  |  |
| Type |  | RW | RW |  |  | RW | RW |  |  | RW | RW |  |  |  |  |  |
| Reset |  | 0 | 1 | 0 |  | 0 | 0 | 0 |  | 0 | 1 | 0 |  |  |  |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 14 | PRCFG_CK | PRCFG_CK | Pull-up/down register configuration for pin CK Default value: 0 0: Pull-up resistor in the I/O pad of pin CK is enabled. 1: Pull-down resistor in the I/O pad of pin CK is enabled. |
| 13:12 | PRVAL_CK | PRVAL_CK | Pull-up/down register value for pin CLK Default value: 10 00: Pull-up/down resistor in the I/O pad of pin CLK are all disabled. 01: Pull-up/down resistor in the I/O pad of pin CLK value is 47k. 10: Pull-up/down resistor in the I/O pad of pin CLK value is 47k. 11: Pull-up/down resistor in the I/O pad of pin CLK value is 23.5k. |
| 10 | PRCFG_CM | PRCFG_CM | Pull-up/down register configuration for pin CM Default value is 0. 0: Pull-up resistor in the I/O pad of pin CM is enabled. 1: Pull-down resistor in the I/O pad of pin CM is enabled. |
| 9:8 | PRVAL_CM | PRVAL_CM | Pull-up/down register value for pin CMD/BS Default value: 00 00: Pull-up/down resistor in the I/O pad of pin CMD/BS are all disabled. 01: Pull-up/down resistor in the I/O pad of pin CMD/BS value is 47k. 10: Pull-up/down resistor in the I/O pad of pin CMD/BS value is 47k. 11: Pull-up/down resistor in the I/O pad of pin CMD/BS value is 23.5k. |
| 6 | PRCFG_DA | PRCFG_DA | Pull-up/down register configuration for pin DAT0, DAT1, DAT2 and DAT3 Default value: 0 0: Pull-up resistor in the I/O pad of pin DAT is enabled. 1: Pull-down resistor in the I/O pad of pin DAT is enabled. |
| 5:4 | PRVAL_DA | PRVAL_DA | Pull-up/down register value for pin DAT0, DAT1, DAT2 and DAT3 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | Default value: 10 00: Pull-up/ down resistor in the I/O pad of pin DAT are all disabled. 01: Pull-up/down resistor in the I/O pad of pin DAT value is 47k. 10: Pull-up/down resistor in the I/O pad of pin DAT value is 47k. 11: Pull-up/down resistor in the I/O pad of pin DAT value is 23.5k. |

A0130020	SDC_CFG	SD Memory Card Controller Configuration Register	00008000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | DTOC |  |  |  |  |  |  |  | WDOD |  |  |  | SDIO |  | MDLE N | SIEN |
| Type | RW |  |  |  |  |  |  |  | RW |  |  |  | RW |  | RW | RW |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | BSYDLY |  |  |  | BLKLEN |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  | RW |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: The register is used to configure the SD memory card controller when it is configured as the host of SD. If the controller is configured as the host of memory stick, the contents of the register will have no impact on the operation of the controller. Note that SDC_CFG[31:16] can be accessed by 16-bit APB bus access.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31:24 | DTOC | DTOC | Data time-out counter The period from finish of the initial host read command or the last read data block in a multiple block read operation to the start bit of the next read data block requires at least two serial clock cycles. The counter is used to extend the period (Read Data Access Time) in unit of 65,536 serial clock. See the register field descriptions of register bit RDINT for reference. 00000000: Extend 65,536 more serial clock cycles 00000001: Extend 65,536x2 more serial clock cycles 00000010: Extend 65,536x3 more serial clock cycles 00000011~11111110: ... 11111111: Extend 65,536x 256 more serial clock cycles |
| 23:20 | WDOD | WDOD | Write data output delay The period from finish of the response for the initial host write command or the last write data block in a multiple block write operation to the start bit of the next write data block requires at least two serial clock cycles. The register field is used to extend the period (Write Data Output Delay) in unit of one serial clock. 0000: No extension 0001: Extend 1 more serial clock cycle 0010: Extend 2 more serial clock cycles 0011~1110: ... 1111: Extend 15 more serial clock cycle |
| 19 | SDIO | SDIO | Enables SDIO 0: Disable SDIO mode 1: Enable SDIO mode |
| 17 | MDLEN | MDLEN | Enables multiple data line The register can be enabled only when SD memory card is applied and detected by software application. It is the responsibility of the application to program the bit correctly when an multi-media card is applied. If an multi-media card is applied and 4-bit data line is |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | enabled, the 4 bits will be output every serial clock. Therefore, data integrity will fail. 0: Disable 4-bit data line 1: Enable 4-bit data line |
| 16 | SIEN | SIEN | Enables serial interface It should be enabled as soon as possible before any command. 0: Disable serial interface for SD 1: Enable serial interface for SD |
| 15:12 | BSYDLY | BSYDLY | Only valid for the commands with R1b response If the command has a response of R1b type, the SD controller must monitor the data line 0 for card busy status from the bit time that is two serial clock cycles after the command end bit to check if the operation in SD memory card have finished. The register field is used to expand the time between the command end bit and end of detection period to detect card busy status. If time is up and there is no card busy status on data line 0, the controller will abandon the detection. 0000: No extension 0001: Extend 1 more serial clock cycle 0010: Extend 2 more serial clock cycles 0011~1110: ... 1111: Extend 15 more serial clock cycle |
| 11:0 | BLKLEN | BLKLEN | Block length The register field is used to define the length of one block in unit of byte in a data transaction. The maximum value of block length is 2,048 bytes. 000000000000: Reserved 000000000001: Block length is 1 byte. 000000000010: Block length is 2 bytes. 000000000011~011111111110: ... 011111111111: Block length is 2,047 bytes. 100000000000: Block length is 2,048 bytes. |

A0130024	SDC_CMD	SD Memory Card Controller Command Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CMDF AIL |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | INTC | STOP | RW | DTYPE |  | IDRT | RSPTYP |  |  | BREA K | CMD |  |  |  |  |  |
| Type | RW | RW | RW | RW |  | RW | RW |  |  | RW | RW |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: The register defines a SD memory card command and its attribute. Before the SD controller issues a transaction onto the SD bus, application shall specify other relative settings such as argument for command. After application writes the register, the SD controller will issue the corresponding transaction onto the SD serial bus. If the command is GO_IDLE_STATE, the controller will have serial clock on SD bus run 128 cycles before issuing the command.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 16 | CMDFAIL | CMDFAIL | If 4-bit SDIO mode is enabled and when CMD/DAT error occurrs, set this bit to select whether to "wait stop command" or "wait data state machine idle". 0: Wait stop command 1: Wait data state machine idle |
| 15 | INTC | INTC | Indicates if the command is GO_IRQ_STATE If the command is GO_IRQ_STATE, the period between command token and response token will not be limited. 0: The command is not GO_IRQ_STATE. 1: The command is GO_IRQ_STATE. |
| 14 | STOP | STOP | Indicates if the command is a stop transmission command 0: The command is not a stop transmission command. 1: The command is a stop transmission command. |
| 13 | RW | RW | Defines the command is a read command or write command The register bit is valid only when the command causes a transaction with data token. 0: The command is a read command. 1: The command is a write command. |
| 12:11 | DTYPE | DTYPE | Defines data token type for the command 00: No data token for the command 01: Single block transaction 10: Multiple block transaction, i.e. the command is a multiple block read or write command. 11: Stream operation. It can only be used when an multi-media card is applied. |
| 10 | IDRT | IDRT | Identification response time The register bit indicates if the command has a response with NID (i.e. 5 serial clock cycles as defined in SD Memory Card Specification Part 1 Physical Layer Specification version 1.0) response time. The register bit is valid only when the command has a response token. Thus the register bit must be set to 1 for CMD2 (ALL_SEND_CID) and ACMD41 (SD_APP_OP_CMD). 0: Otherwise 1: The command has a response with NID response time. |
| 9:7 | RSPTYP | RSPTYP | Defines response type for the command For commands with R1 and R1b response, register SDC_CSTA (not SDC_STA) updates after response token is received. This register SDC_CSTA contains the status of the SD, and it can be used as a response interrupt source. Note: If CMD7 is used with all 0's RCA, then RSPTYP must be "000". Command "GO_TO_IDLE" also has RSPTYP='000'. 000: There is no response for the command, e.g. broadcast command without response and GO_INACTIVE_STATE command. 001: The command has R1 response. R1 response token is 48-bit. 010: The command has R2 response. R2 response token is 136-bit. 011: The command has R3 response. Even though R3 is 48-bit response, it does not contain CRC checksum. 100: The command has R4 response. R4 response token is 48-bit. (only for MMC) 101: The command has R5 response. R5 response token is 48-bit. (only for MMC) 110: The command has R6 response. R6 response token is 48-bit. 111: The command has R1b response. If the command has a response of R1b type, SD controller must monitor the data line 0 for card busy status from the bit time that is 2 or 4 serial clock cycles |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | after the command end bit to check if the operation in SD memory card has finished. There are two cases for detection of card busy status. The first case is that the host stops the data transmission during an active write data transfer. The card will assert busy signal after the stop transmission command end bit followed by 4 serial clock cycles. The second case is that the card is in idle state or receiving a stop transmission command between data blocks when multiple block write command is in progress. The register bit will be valid only when the command has a response token. |
| 6 | BREAK | BREAK | Aborts pending MMC GO_IRQ_MODE command It is only valid for a pending GO_IRQ_MODE command waiting for MMC interrupt response. 0: Other fields are valid. 1: Break a pending MMC GO_IRQ_MODE command in the controller. Other fields are invalid. |
| 5:0 | CMD | CMD | SD memory card command Total 6 bits. |

A0130028	SDC_ARG	SD Memory Card Controller Argument Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | ARG[31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | ARG[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: The register contains the argument of the SD memory card command.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31:0 | ARG | ARG | Contains argument of the SD memory card command. |

A013002C	SDC_STA	SD Memory Card Controller Status Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | WP |  |  |  |  |  |  |  |  |  |  | FEDA TBUS Y | FECM DBUS Y | BEDA TBUS Y | BECM DBUS Y | BESD CBUS Y |
| Type | RO |  |  |  |  |  |  |  |  |  |  | RO | RO | RO | RO | RO |
| Reset | 0 |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 |

Overview: The register contains various statuses of SD controller as the controller is configured as the host of SD memory card.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | WP | WP | Detects the status of write protection switch on SD memory card The register bit shows the status of write protection switch on SD memory card. There is no default reset value. Pin WP (Write Protection) is only useful when the controller is configured for SD memory card. 1: Write protection switch on, i.e. memory card is desired to be write-protected. 0: Write protection switch off, i.e. memory card is writable. |
| 4 | FEDATBUS Y | FEDATBUSY | Indicates if there is any transmission going on DAT line on SD bus This bit indicates directly the CMD line at card clock domain. For those commands without data but still involving DAT line, the register bit is useless. For example, if an erase command is issued, checking if the register bit is 0 before issuing the next command with data will not guarantee that the controller is idle. In this case, use register bit BESDCBUSY. 0: No transmission is going on DAT line on SD bus. 1: There is transmission going on DAT line on SD bus. |
| 3 | FECMDBUS Y | FECMDBUSY | Indicates if there is any transmission going on CMD line on SD bus This bit indicates directly the CMD line at card clock domain. 0: No transmission is going on CMD line on SD bus. 1: There is transmission going on CMD line on SD bus. |
| 2 | BEDATBUS Y | BEDATBUSY | Indicates if there is any transmission going on DAT line on SD bus 0: Backend SDC controller gets the info that no transmission is going on DAT line on SD bus. 1: Backend SDC controller gets the info that there is transmission going on DAT line on SD bus. |
| 1 | BECMDBUS Y | BECMDBUSY | Indicates if there is any transmission going on CMD line on SD bus This bit shows backend controller's CMD busy state. The busy state is synced from card clock domain to bus clock domain. 0: Backend SDC controller gets the info that no transmission is going on CMD line on SD bus. 1: Backend SDC controller gets the info that there is transmission going on CMD line on SD bus. |
| 0 | BESDCBUS Y | BESDCBUSY | Indicates if SD controller is busy, i.e. is there any transmission going on CMD or DAT line on SD bus This bit shows backend controller's SDC busy state. The busy state is synced from card clock domain to bus clock domain. 0: Backend SD controller is idle. 1: Backend SD controller is busy. |

A0130030	SDC_RESP0	SD Memory Card Controller Response Register 0	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | RESP[31:0][31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | RESP[31:0][15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Overview: The register contains parts of the last SD memory card bus response. See descriptions of register field SDC_RESP3.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31:0 | RESP[31:0] | RESP_31_0 |  |

A0130034	SDC_RESP1	SD Memory Card Controller Response Register 1	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | RESP[63:32][31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | RESP[63:32][15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: The register contains parts of the last SD memory card bus response. See descriptions of register field SDC_RESP3.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31:0 | RESP[63:32] | RESP_63_32 |  |

A0130038	SDC_RESP2	SD Memory Card Controller Response Register 2	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | RESP[95:64][31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | RESP[95:64][15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: The register contains parts of the last SD memory card bus response. See descriptions of register field SDC_RESP3.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31:0 | RESP[95:64] | RESP_95_64 |  |

A013003C	SDC_RESP3	SD Memory Card Controller Response Register 3	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | RESP[127:96][31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | RESP[127:96][15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Overview: The register contains parts of the last SD memory card bus response. Register fields SDC_RESP0, SDC_RESP1, SDC_RESP2 and SDC_RESP3 compose the last SD memory card bus response. For response of type R2, i.e. response of commands ALL_SEND_CID, SEND_CSD and SEND_CID, only bit 127 to 0 of the response token are stored in register fields SDC_RESP0, SDC_RESP1, SDC_RESP2 and SDC_RESP3. For response of other types, only bit 39 to 8 of the response token are stored in register field SDC_RESP0.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31:0 | RESP[127:9 6] | RESP_127_96 |  |

A0130040	SDC_CMDSTA  SD Memory Card Controller Command Status Register 00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  | RSPC RCER R | CMDT O | CMDR DY |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | RC | RC | RC |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 |

Overview: The register contains the status of SD controller during command execution and that of SD bus protocol after command execution when the SD controller is configured as the host of SD memory card. The register can also be used as an interrupt source. The register is cleared when being read. Meanwhile, if the interrupt is enabled and thus interrupt caused by the register is generated, reading the register will de-assert the interrupt.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 2 | RSPCRCER R | RSPCRCERR | CRC error on CMD detected '1' indicates the SD controller detects a CRC error after reading a response from the CMD line. 0: Otherwise 1: SD controller detects a CRC error after reading a response from the CMD line. |
| 1 | CMDTO | CMDTO | Time-out on CMD detected '1' indicates the SD controller detects a time-out condition while waiting for a response on the CMD line. 0: Otherwise 1: SD controller detects a timeout condition while waiting for a response on the CMD line. |
| 0 | CMDRDY | CMDRDY | For command without response, the register bit will be 1 once the command is completed on SD bus For command with response, the register bit will be 1 whenever the command is issued onto the SD bus and its corresponding response is received without CRC error. 0: Otherwise 1: Command with/without response is finished successfully without CRC error. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A0130044	SDC_DATSTA	SD Memory Card Controller Data Status Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  | DATCRCERR |  |  |  |  |  |  |  | DATT O | BLKD ONE |
| Type |  |  |  |  |  |  | RC |  |  |  |  |  |  |  | RC | RC |
| Reset |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: The register contains the status of SD controller during data transfer on DAT line(s) when the SD controller is configured as the host of SD memory card. The register can be used as an interrupt source. The register is cleared when being read. Meanwhile, if the interrupt is enabled and thus interrupt caused by the register is generated, reading the register will de-assert the interrupt.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 9:2 | DATCRCER R | DATCRCERR | CRC error on DAT detected '1' indicates that the SD controller detects a CRC error for bit n after reading a block of data from the DAT line or SD signals a CRC error after writing a block of data to the DAT line. 0: Otherwise 1: SD controller detects a CRC error after reading a block of data from the DAT line or SD signals a CRC error after writing a block of data to the DAT line. Note: n is 7 ~ 1 for 8-bits mode. Each bit is read and cleared individually. |
| 1 | DATTO | DATTO | Time-out on DAT detected A '1' indicates that the SD controller detects a time-out condition while waiting for data token on the DAT line. 0: Otherwise 1: SD controller detects a time-out condition while waiting for data token on the DAT line. |
| 0 | BLKDONE | BLKDONE | Indicates the status of data block transfer 0: Otherwise 1: A data block is successfully transferred. |

A0130048	SDC_CSTA	SD Memory Card Status Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | CSTA [31:0][31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | CSTA [31:0][15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: After commands with R1 and R1b respond this register containing the status of the SD card, it will be used as a response interrupt source. In all register fields, logic high indicates error, and logic low indicates no error.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

The register is cleared when being read. Meanwhile, if the interrupt is enabled and thus interrupt caused by the register is generated, reading the register will de-assert the interrupt.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31:0 | CSTA [31:0] | CSTA_31_0 | CSTA31: OUT_OF_RANGE. The command's argument is out of the allowed range for this card. CSTA30: ADDRESS_ERROR. A misaligned address that does not match the block length is used in the command. CSTA29: BLOCK_LEN_ERROR. The transferred block length is not allowed for this card, or the number of transferred bytes does not match the block length. CSTA28: ERASE_SEQ_ERROR. An error in the sequence of erase commands occurs. CSTA27: ERASE_PARAM. An invalid selection of write-blocks for erase occurs. CSTA26: WP_VIOLATION. Attempt to program a write-protected block. CSTA25: Reserved. Return to 0. CSTA24: LOCK_UNLOCK_FAILED. Set when a sequence or password error is detected in lock/unlock card command or if there is an attempt to access a locked card. CSTA23: COM_CRC_ERROR. The CRC check of the previous command fails. CSTA22: ILLEGAL_COMMAND. Command not legal for the card state. CSTA21: CARD_ECC_FAILED. Card internal ECC is applied but fails to correct the data. CSTA20: CC_ERROR. Internal card controller error. CSTA19: ERROR. A general or unknown error occurs during the operation. CSTA18: UNDERRUN. The card cannot sustain data transfer in stream read mode. CSTA17: OVERRUN. The card cannot sustain data programming in stream write mode. CSTA16: CID/CSD_OVERWRITE. It can be either one of the following errors: 1) The CID register has been already written and cannot be overwritten; 2) The read-only section of the CSD does not match the card; 3) An attempt to reverse the copy (set as the original) or permanent WP (unprotected) bits is made. CSTA[15: 4]: Reserved. Return to 0. CSTA3: AKE_SEQ_ERROR. Error in the sequence of authentication process CSTA[2: 0]: Reserved. Return to 0. |

A013004C	SDC_IRQMASK0	SD Memory Card IRQ Mask Register 0	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | IRQMASK [31:0][31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | IRQMASK [31:0][15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Overview: The register contains parts of SD memory card interrupt mask register. See the register descriptions of register SDC_IRQMASK1 for reference. The register masks interrupt sources from register SDC_CMDSTA and SDC_DATSTA. IRQMASK[3:0] is for SDC_CMDSTA, and IRQMASK[18:16] for SDC_DATSTA. Note that

IRQMASK[18] masks SDC_DATSTA[9:2] together. '1' in some bits of the register masks the corresponding interrupt source with the same bit position. For example, if IRQMASK[0] is 1, then the interrupt source from register field CMDRDY of register SDC_CMDSTA will be masked. '0' in some bits does not cause interrupt mask on the corresponding interrupt source from registers SDC_CMDSTA and SDC_DATSTA.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31:0 | IRQMASK [31:0] | IRQMASK_31_0 |  |

A0130050	SDC_IRQMASK1	SD Memory Card IRQ Mask Register 1	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | IRQMASK [63:32][31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | IRQMASK [63:32][15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: The register contains parts of SD memory card interrupt mask register. Registers SDC_IRQMASK1 and SDC_IRQMASK0 compose the SD memory card interrupt mask register. The register masks interrupt sources from register SDC_CSTA. '1' in some bits of the register masks the corresponding interrupt source with the same bit position. For example, if IRQMASK[63] is 1, then the interrupt source from register field OUT_OF_RANGE of register SDC_ CSTA will be masked. '0' in some bits does not cause interrupt mask on the corresponding interrupt source from register SDC_CSTA.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31:0 | IRQMASK [63:32] | IRQMASK_63_32 |  |

A0130054	SDIO_CFG	SDIO Configuration Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  |  |  | DISSE L |  | INTCS EL | DSBS EL |  | INTEN |
| Type |  |  |  |  |  |  |  |  |  |  | RW |  | RW | RW |  | RW |
| Reset |  |  |  |  |  |  |  |  |  |  | 0 |  | 0 | 0 |  | 0 |

Overview: The register is used to configure functions for SDIO.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 5 | DISSEL | DISSEL | Selects data block interrupt source 0: The host detects SDIO interrupt during interrupt period between two data blocks of multiple block data access. 1: The host ignores SDIO interrupt during interrupt period between two data blocks of multiple block data access. |
| 3 | INTCSEL | INTCSEL | Selects interrupt control 0: The host detects DAT1 low as SDIO interrupt. 1: The host detects DAT3/DAT2/DAT1/DAT0 4'b1101 as SDIO interrupt. |
| 2 | DSBSEL | DSBSEL | Selects data block start bit 0: Use data line 0 as start bit of data block. Other data lines are ignored. 1: Start bit of a data block is received only when all data line 0-3 become low. |
| 0 | INTEN | INTEN | Enables interrupt for SDIO 0: Disable 1: Enable |

A0130058	SDIO_STA	SDIO Status Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | IRQ |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RO |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |

Overview: This register is used to identify if there is SDIO interrupt during the interrupt period on data line.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 0 | IRQ | IRQ | SDIO interrupt exists on the data line. For example, when in the interrupt period or the 1 -bit data line mode and DAT1/5 goes low from high, this bit will become 1 from 0. If DAT1/5 goes high from low, this bit will become 0 from 1. 0: There is no SDIO interrupt existing on the data line. 1: There is SDIO interrupt existing on the data line. |

A0130080	CLK_RED	CLK Latch Configuration Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  | CMD_ RED |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  | DAT_ RED |  |  |  |  |  | CLKP AD_R ED | CLK_ LATC H |  |  |  |  |  |  |
| Type |  |  | RW |  |  |  |  |  | RW | RW |  |  |  |  |  |  |
| Reset |  |  | 0 |  |  |  |  |  | 0 | 0 |  |  |  |  |  |  |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

Overview: The register is used to configure the MSDC sample data/response clock. Note that only when MSDC_IOCON[19] = 1 will the host latch response; otherwise MSDC FSM will handle the response from PAD directly.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 29 | CMD_RED | CMD_RED | Determines the command response from card output is latched at falling edge or rising edge of internal clock Only effective when CLK_LATCH = 1 0: Internal clock rising edge to latch response 1: Internal clock falling edge to latch response |
| 13 | DAT_RED | DAT_RED | Determines the input data from card output is latched at falling edge or rising edge of internal sample clock Only effective when CLK_LATCH = 1 0: Internal clock rising edge to latch data 1: Internal clock falling edge to latch data |
| 7 | CLKPAD_R ED | CLKPAD_RED | Determines the input data from card is latched at falling edge or rising edge of the feedback clock from pad The suggested setting is 0 when SD serial clock is lower than 25MHz. The suggestion setting is 1 when SD serial clock is higher than 25MHz. The suggestion setting is 0 for MMC card no matter the serial clock rate is high speed or default speed. Only effective when CLK_LATCH = 0. 0: Internal feedback clock rising edge to latch data/response 1: Internal feedback clock falling edge to latch data/response |
| 6 | CLK_LATCH | CLK_LATCH | Determines which clock to latch data from card The suggested setting is 1 if SCLKF in register field MSDC_CFG is 0x0. Otherwise, the suggested setting is 0. 0: Internal feedback clock is used to latch data/response from card. 1: Internal clock is used to latch data/response from card. |

A0130098	DAT_CHECKSUM  MSDC Rx Data Checksum Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | DAT_CHECKSUM[31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | DAT_CHECKSUM[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: The register is used to compete the checksum value of Rx read data

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31:0 | DAT_CHEC KSUM | DAT_CHECKSUM | The checksum algorithm is 32-bit XOR. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

SD Memory Card Controller (MSDC1)

##### Introduction

The controller fully supports the SD memory card bus protocol as defined in SD Memory Card Specification Part 1 Physical Layer Specification version 2.0.

Furthermore, the controller also partially supports the SDIO card specification version 2.0. However, the controller can only be configured as the host of the SD memory card. Hereafter, the controller is also abbreviated as the SD controller. The following are the main features of the controller.

- Interface with MCU by APB bus

- 16/32-bit access on APB bus

- 16/32-bit access for control registers

- 32-bit access for FIFO

- Built-in 32 bytes FIFO buffers for transmit and receive, FIFO is shared for transmit and receive

- Built-in CRC circuit

- CRC generation can be disabled

- DMA supported

- Interrupt capabilities

- Data rate up to 48 Mbps in serial mode, 48x4 Mbps in parallel model, the module is targeted at 48 MHz operating clock

- Serial clock rate on SD bus is programmable

- Card detection capabilities during sleep mode

- Controllability of power for memory card

- Does not support SPI mode for SD memory card

- Does not support multiple SD memory cards

##### Overview

###### Pin Assignment

The following lists pins required for the SD memory card. Table 51 shows how the pins are shared. Note that all I/O pads have embedded both pull-up and pull-down resistors because they are shared by the SD memory card. The pull-down resistors for these pins can be used for power saving. If optimal pull-up or pull-down resistors are required on the system board, all embedded pull-up and pull-down resistors can be disabled by programming the corresponding control registers. The VDDPD pin is used for power saving. Power for the SD memory card can be shut down by programming the corresponding control register. The WP (Write Protection) pin is used to detect the status of the Write Protection Switch on the SD memory card.

Table 51. Sharing of pins for SD memory card controller

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| No. | Name | Type | MMC | SD | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | SD_CLK | O | CLK | CLK | Clock |
| 2 | SD_DAT3 | I/O/PP |  | CD/DAT3 | Data Line [Bit 3] |
| 3 | SD_DAT0 | I/O/PP | DAT0 | DAT0 | Data Line [Bit 0] |
| 4 | SD_DAT1 | I/O/PP |  | DAT1 | Data Line [Bit 1] |
| 5 | SD_DAT2 | I/O/PP |  | DAT2 | Data Line [Bit 2] |
| 6 | SD_CMD | I/O/PP | CMD | CMD | Command Or Bus State |
| 7 | SD_PWRON | O |  |  | VDD ON/OFF |
| 8 | SD_WP | I |  |  | Write Protection Switch in SD |
| 9 | SD_INS | I | VSS2 | VSS2 | Card Detection |

###### Card Detection

For SD memory card, detection of card insertion/removal by hardware is supported, and a dedicated pin “INS” is used to perform card insertion and removal for SD. The pin “INS” will be connected to the pin “VSS2” of a SD connector (see Figure 45).

HOST

CARD

RPU

SW1

INS_IN

PAD

INS

RPD

SW2

Figure 46. Card detection for SD memory card

output enable

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

RCDEN

SW2

CD/DAT3 IN

470 Kohm

RPD

PAD

PAD

DAT3 OUT

SW3

SW1

10-90 K

RPU

CARD

HOST

output enable

Figure 47. Card detection for SD memory card (Scheme 2)

##### Register Definition

Module name: MSDC1 base address: (+A0270000h)

| Address | Name | Width | Register function |
| --- | --- | --- | --- |
| A0270000 | MSDC_CFG | 32 | SD memory card controller configuration register For general configuration of the SD controller. Note: MSDC_CFG[31:16] can be accessed by 16-bit APB bus access. |
| A0270004 | MSDC_STA | 32 | SD memory card controller status register Contains the status of FIFO, interrupts and data requests. |
| A0270008 | MSDC_INT | 32 | SD memory card controller interrupt register Contains the status of interrupts. Note that the register still shows the status of interrupt even though the interrupt is disabled, that is, register bit INTEN of register MSDC_CFG is set to 0. It implies that software interrupt can be implemented by polling register bit INT of register MSDC_STA and this register. However, if hardware interrupt is desired, be sure to clear the register before setting up register bit INTEN of register MSDC_CFG to 1, or undesired hardware interrupt arisen from the previous interrupt status may take place. |
| A027000C | MSDC_PS | 32 | SD memory card pin status register Used for card detection. When the memory card controller and system are powered on, the power for the memory card will still be off unless the power is supplied by the PMIC. Meanwhile, the pad for card detection defaults to pull down when the system is powered on. The scheme of card detection for MS is the same as that for SD. For detecting card insertion, first pull up the INS pin and |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Address | Name | Width | Register function |
| --- | --- | --- | --- |
|  |  |  | then enable card detection and the input pin at the same time. After 32 cycles of controller clock, the status of pin changes will emerge. To detect card removal, simply keep enabling card detection and the input pin. |
| A0270010 | MSDC_DAT | 32 | SD memory card controller data register Reads/Writes data from/to FIFO inside SD controller. Data access unit: 32 bits |
| A0270014 | MSDC_IOCON | 32 | SD memory card controller IO control register Specifies output driving capability and slew rate of IO pads for MSDC. The reset value is suggested setting. If the output driving capability of pins DAT0, DAT1, DAT2 and DAT3 is too large, it is possible to arise ground bounce and thus result in glitch on SCLK. The actual driving current will depend on the PAD type selected for the chip. |
| A0270018 | MSDC_IOCON1 | 32 | SD memory card controller IO control register 1 |
| A0270020 | SDC_CFG | 32 | SD memory card controller configuration register Configures the SD memory card controller when it is configured as the host of SD. The register is used to configure the SD memory card controller when it is configured as the host of SD memory card. If the controller is configured as the host of memory stick, the contents of the register will have no impact on the operation of the controller. Note: SDC_CFG[31:16] can be accessed by 16-bit APB bus access. |
| A0270024 | SDC_CMD | 32 | SD memory card controller command register Defines a SD memory card command and its attribute. Before the SD controller issues a transaction onto the SD bus, application shall specify other relative settings such as argument for command. After application writes the register, the SD controller will issue the corresponding transaction onto SD serial bus. If the command is GO_IDLE_STATE, the controller will have serial clock on SD bus run 128 cycles before issuing the command. |
| A0270028 | SDC_ARG | 32 | SD memory card controller argument register Contains argument of the SD memory card command. |
| A027002C | SDC_STA | 32 | SD memory card controller status register Contains various statuses of SD controller as the controller is configured as the host of SD memory card. |
| A0270030 | SDC_RESP0 | 32 | SD memory card controller response register 0 Contains parts of the last SD memory card bus response. See descriptions of register field SDC_RESP3. |
| A0270034 | SDC_RESP1 | 32 | SD memory card controller response register 1 Contains parts of the last SD memory card bus response. See descriptions of register field SDC_RESP3. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Address | Name | Width | Register function |
| --- | --- | --- | --- |
| A0270038 | SDC_RESP2 | 32 | SD memory card controller response register 2 Contains parts of the last SD memory card bus response. See descriptions of register field SDC_RESP3. |
| A027003C | SDC_RESP3 | 32 | SD memory card controller response register 3 Contains parts of the last SD memory card bus response. Register fields SDC_RESP0, SDC_RESP1, SDC_RESP2 and SDC_RESP3 compose the last SD memory card bus response. For response of type R2, that is, response of the command ALL_SEND_CID, SEND_CSD and SEND_CID, only bit 127 to 0 of the response token are stored in register field SDC_RESP0, SDC_RESP1, SDC_RESP2 and SDC_RESP3. For responses of other types, only bit 39 to 8 of the response token are stored in register field SDC_RESP0. |
| A0270040 | SDC_CMDSTA | 32 | SD memory card controller command status register Contains the status of SD controller during command execution and that of SD bus protocol after command execution when the SD controller is configured as the host of SD memory card. The register will also be used as the interrupt source. The register is cleared when being read. Meanwhile, if interrupt is enabled and thus interrupt caused by the register is generated, reading the register will de-assert the interrupt. |
| A0270044 | SDC_DATSTA | 32 | SD memory card controller data status register Contains the status of SD controller during data transfer on DAT line(s) when the SD controller is configured as the host of SD memory card. The register is also used as the interrupt source. The register is cleared when being read. Meanwhile, if interrupt is enabled and thus interrupt caused by the register is generated, reading the register will de-assert the interrupt. |
| A0270048 | SDC_CSTA | 32 | SD memory card status register After commands with R1 and R1b response this register containing the status of the SD card, it will be used as the response interrupt source. In all register fields, logic high indicates error, and logic low indicates there is no error. The register is cleared when being read. Meanwhile, if interrupt is enabled and thus interrupt caused by the register is generated, reading the register will de-assert the interrupt. |
| A027004C | SDC_IRQMASK0 | 32 | SD memory card IRQ mask register 0 Contains parts of SD memory card interrupt mask register. See the descriptions of register SDC_IRQMASK1 for reference. The register masks interrupt sources from register SDC_CMDSTA and SDC_DATSTA. IRQMASK[3:0] is for SDC_CMDSTA, and IRQMASK[18:16] for SDC_DATSTA. Note that IRQMASK[18] masks SDC_DATSTA[9:2] together. ‘1’ in |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Address | Name | Width | Register function |
| --- | --- | --- | --- |
|  |  |  | some bits of the register masks the corresponding interrupt source with the same bit position. For example, if IRQMASK[0] is 1, then the interrupt source from register field CMDRDY of register SDC_CMDSTA will be masked. '0' in some bits does not cause interrupt mask on the corresponding interrupt source from register SDC_CMDSTA and SDC_DATSTA. |
| A0270050 | SDC_IRQMASK1 | 32 | SD memory card IRQ mask register 1 Contains parts of SD memory card interrupt mask register. Registers SDC_IRQMASK1 and SDC_IRQMASK0 compose the SD memory card interrupt mask register. The register masks interrupt sources from register SDC_CSTA. '1' in some bits of the register masks the corresponding interrupt source with the same bit position. For example, if IRQMASK[63] is 1, then interrupt source from register field OUT_OF_RANGE of register SDC_ CSTA will be masked. '0' in some bit does not cause interrupt mask on the corresponding interrupt source from register SDC_CSTA. |
| A0270054 | SDIO_CFG | 32 | SDIO configuration register Configures functions for SDIO. |
| A0270058 | SDIO_STA | 32 | SDIO status register Identifies if thers is SDIO interrupt during the interrupt period on data line. |
| A0270080 | CLK_RED | 32 | CLK latch configuration register Configures the MSDC sample data/response clock. Note: When MSDC_IOCON[19] = 1, the host will latch response; otherwise MSDC FSM will handle the response from PAD directly. |
| A0270098 | DAT_CHECKSUM | 32 | MSDC Rx data checksum register Competes the checksum value of Rx read data |

A0270000	MSDC_CFG	SD Memory Card Controller Configuration Register	04000020

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  | FIFOTHD |  |  |  |  |  | VDDP D | RCDE N | DIRQE N | PINEN | DMAE N | INTEN |
| Type |  |  |  |  | RW |  |  |  |  |  | RW | RW | RW | RW | RW | RW |
| Reset |  |  |  |  | 0 | 1 | 0 | 0 |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | SCLKF |  |  |  |  |  |  |  | SCLK ON | CRED | STDB Y | CLKSRC |  | NOCR C | RST | MSDC |
| Type | RW |  |  |  |  |  |  |  | RW | RW | RW | RW |  | RW | W1C | RW |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |

Overview: For general configuration of the SD controller. Note that MSDC_CFG[31:16] can be accessed by 16-bit APB bus access.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 27:24 | FIFOTHD | FIFOTHD | FIFO threshold The register field determines when to issue a DMA request. For write transactions, DMA requests will be asserted if the number of free entries in FIFO are bigger than or equal to the value in the register field. For read transactions, DMA requests will be asserted if the number of valid entries in FIFO are bigger than or equal to the value in the register field. The register field must be set according to the setting of data transfer count in DMA burst mode. If single mode for DMA transfer is used, the register field shall be set to 0b0001. 0000: Invalid 0001: Threshold value is 1. 0010: Threshold value is 2. 0011~01111: ... 1000: Threshold value is 8. Others: Invalid |
| 21 | VDDPD | VDDPD | Controls output pin VDDPD used for power saving Output pin VDDPD controls the power for memory card. 0: Output pin VDDPD outputs logic low. The power for memory card will be turned off. 1: Output pin VDDPD outputs logic high. The power for memory card will be turned on. |
| 20 | RCDEN | RCDEN | Controls output pin RCDEN used for card identification process when the controller is for SD memory card Its output controls the pull-down resistor on the system board to connect to or disconnect from signal CD/DAT3. 0: The output pin RCDEN outputs logic low. 1: The output pin RCDEN outputs logic high. |
| 19 | DIRQEN | DIRQEN | Enables data request interrupt The register bit is used to control if data request is used as an interrupt source. 0: Data request is not used as an interrupt source. 1: Data request is used as an interrupt source. |
| 18 | PINEN | PINEN | Enables pin interrupt The register bit is used to control if the pin for card detection is used as an interrupt source. 0: The pin for card detection is not used as an interrupt source. 1: The pin for card detection is used as an interrupt source. |
| 17 | DMAEN | DMAEN | Enables DMA Note: If DMA capability is disabled, the application software must poll the status of register MSDC_STA to check on any data transfer request. If DMA is desired, the register bit must be set up before command register is written. 0: DMA request induced by various conditions is disabled, no matter the controller is configured as the host of either SD memory card or memory stick. 1: DMA request induced by various conditions is enabled, no matter the controller is configured as the host of either SD memory card or memory stick. |
| 16 | INTEN | INTEN | Enables interrupt Note: If interrupt capability is disabled, the application software must poll the status of register MSDC_STA to check on any interrupt request. 0: Interrupt induced by various conditions is disabled, no matter the controller is configured as the host of either SD memory card or memory stick. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: Interrupt induced by various conditions is enabled, no matter the controller is configured as the host of either SD memory card or memory stick. |
| 15:8 | SCLKF | SCLKF | Controls clock frequency of serial clock on SD bus and denotes clock frequency of SD bus serial clock as fslave and clock frequency of the SD controller as fhost which is 98.3 or 96.2 MHz Note: The allowed maximum frequency of fslave is 49.15MHz. While changing the clock rate, "1T clock period before change + 1T clock period after change" is required for HW signal to re-synchronize. 00000000b: fslave =(1/2)*fhost 00000001b: fslave = [1/(4*1)]*fhost 00000010b: fslave = [1/(4*2)]*fhost 00000011b: fslave = [1/(4*3)]*fhost 00000100b~111111110b: ... 11111111b: fslave = [1/(4*255)]*fhost |
| 7 | SCLKON | SCLKON | Serial clock always on For debugging. 0: Serial clock not always on 1: Serial clock always on |
| 6 | CRED | CRED | Rising edge data The register bit is used to determine the serial data input is latched at the falling edge or rising edge of the serial clock. The default setting is at the rising edge. If the serial data have bad timing, set the register bit to 1. When the memory card has bad timing on returned read data, set the register bit to 1. 0: Serial data input is latched at the rising edge of serial clock. 1: Serial data input is latched at the falling edge of serial clock. |
| 5 | STDBY | STDBY | Standby mode If the module is powered down, operating clock to the module will be stopped. At the same time, the clock to card detection circuitry will also be stopped. If detection on memory card insertion and removal is desired, write 1 to the register bit. If interrupt for detection on memory card insertion and removal is enabled, the interrupt will take place whenever the memory is inserted or removed. 0: Standby mode is disabled. 1: Standby mode is enabled. |
| 4:3 | CLKSRC | CLKSRC | Specifies which clock is used as source clock of memory card 00 : MPLL/5.5MHz clock 01 : MPLL/7MHz clock (this divider is default off, before switch controller source clock to this clock, you should switch controller source clock to 26Mhz first, and then enable divider MPLL/7, after a moment you can switch controller source clock to MPLL/ 7 to void source clock glitch. For detail setting please refer to confgsys document) - : MPLL/8MHz clock - : MPLL/10MHz clock For phone 00 : 94.5MHz clock Need to keep BT_APP_DIV_EN= 1'b0 in CLK_CONDA[15]. 01 : 74.3MHz clock NOTE: Need to set POWERFUL_DIV_EN1 = 1'b1 first in CLK_CONDA[10]. - : 65MHz clock - : Forbidden For BT app. 00 : Forbidden 01 : 89.1MHz clock NOTE: Need to set POWERFUL_DIV_EN1 = 1'b1 first in CLK_CONDA[10]. - 78MHz clock - : 62.4MHz clock |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | NOTE: Need to set POWERFUL_DIV_EN2 = 1'b1 first in CLK_CONDA[9]. |
| 2 | NOCRC | NOCRC | Disable CRC '1' indicates data transfer without CRC is desired. For write data block, the data are transmitted without CRC. For read data block, CRC will not be checked. For testing purpose. 0: Data transfer with CRC is desired. 1: Data transfer without CRC is desired. |
| 1 | RST | RST | Software reset Writing 1 to the register bit will cause internal synchronous reset of SD controller but will not reset register settings, RST should only be set when RST equal to 0. 0: Read 0 stands for the reset process is finished. 1: Write 1 to reset SD controller. |
| 0 | MSDC | MSDC | Configures the controller as SD memory card mode CLK/CMD/DAT line is pulled low when SD memory card mode is disable. 0: Disable SD memory card 1: Enable SD memory card |

A0270004	MSDC_STA	SD Memory Card Controller Status Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | BUSY | FIFOC LR |  |  |  |  |  |  | FIFOCNT |  |  |  | INT | DRQ | BE | BF |
| Type | R | W1C |  |  |  |  |  |  | RO |  |  |  | RO | RO | RO | RO |
| Reset | 0 | 0 |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: The register contains the status of FIFO, interrupts and data requests.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | BUSY | BUSY | Status of the controller If the controller is in busy state, the register bit will be 1; otherwise 0. 0: The controller is in busy state. 1: The controller is in idle state. |
| 14 | FIFOCLR | FIFOCLR | Clears FIFO Writing 1 to the register bit will cause the content of FIFO clear and reset the status of FIFO controller. 0: Read 0 stands for the FIFO clear process is finished. 1: Write 1 to clear the content of FIFO clear and reset the status of FIFO controller. |
| 7:4 | FIFOCNT | FIFOCNT | FIFO count The register field shows how many valid entries are there in FIFO. 0000: 0 valid entry in FIFO 0001: 1 valid entry in FIFO 0010: 2 valid entries in FIFO 0011~0111: ... 1000: 8 valid entries in FIFO |
| 3 | INT | INT | Indicates if there is any interrupt existing |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | When there is interrupt existing, the register bit will still be active even if register bit INTEN in register MSDC_CFG is disabled. The SD controller can interrupt MCU by issuing interrupt request to the interrupt controller, or the software/application will poll the register endlessly to check if there is any interrupt request existing in the SD controller. When register bit INTEN in register MSDC_CFG is disabled, the second method is used. For read commands, it is possible that time-out error takes place. The software can read the status register to check if the time-out error takes place without OS time tick support or data request asserted. Note: The register bit will be cleared when register MSDC_INT is read. 0: No interrupt request existing. 1: Interrupt request exists. |
| 2 | DRQ | DRQ | Indicates if any data transfer is required When a data transfer is required, the register bit will still be active even if register bit DIRQEN in register MSDC_CFG is disabled. Data transfer can be achieved by DMA channel alleviating MCU loading, or by polling the register bit to check if any data transfer is requested. When register bit DIRQEN in register MSDC_CFG is disabled, the second method is used. 0: No DMA request existing. 1: DMA request exists. |
| 1 | BE | BE | Indicates if FIFO in SD controller is empty 0: FIFO in SD controller is not empty. 1: FIFO in SD controller is empty. |
| 0 | BF | BF | Indicates if FIFO in SD controller is full 0: FIFO in SD controller is not full. 1: FIFO in SD controller is full. |

A0270008	MSDC_INT	SD Memory Card Controller Interrupt Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  | SDIOI RQ | SDR1 BIRQ |  | SDMC IRQ | SDDA TIRQ | SDCM DIRQ | PINIR Q | DIRQ |
| Type |  |  |  |  |  |  |  |  | RC | RC |  | RC | RC | RC | RC | RC |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 |

Overview: The register contains the status of interrupts. Note that the register still show status of interrupt even though interrupt is disabled, that is, register bit INTEN of register MSDC_CFG is set to 0. It implies that software interrupt can be implemented by polling register bit INT of register MSDC_STA and this register. However, if hardware interrupt is desired, be sure to clear the register before setting register bit INTEN of register MSDC_CFG to 1, or undesired hardware interrupt arisen from the previous interrupt status may take place.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 7 | SDIOIRQ | SDIOIRQ | SDIO interrupt The register bit indicates if there is any interrupt for SDIO existing. Whenever an interrupt for SDIO exists, the register bit will be set to 1 if the interrupt is enabled. It will be reset when the register is read. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 0: No SDIO interrupt 1: Interrupt for SDIO exists. |
| 6 | SDR1BIRQ | SDR1BIRQ | SD R1b response interrupt The register bit will be active when a SD command with R1b response is finished and the DAT0 line is transited from busy to idle state. Single block write commands with R1b response will cause interrupt when the command is completed either successfully or with CRC error. However, multi-block write commands with R1b response do not cause interrupt because multi-block write commands are always stopped by STOP_TRANS commands. STOP_TRANS commands (with R1b response) behind multi-block write commands will cause interrupt. Single block read command with R1b response will cause interrupt when the command is completed, but multi-block read commands do not. Note: STOP_TRANS commands (with R1b response) behind multi-block read commands will cause interrupt. 0: No interrupt for SD R1b response. 1: Interrupt for SD R1b response exists. |
| 4 | SDMCIRQ | SDMCIRQ | SD memory card interrupt The register bit indicates if there is any interrupt for SD memory card existing. Whenever an interrupt for SD memory card exists, i.e. any bit in register SDC_CSTA is active, the register bit will be set to 1 if interrupt is enabled. It will be reset when the register is read. Note: This bit will not trigger MSDC hardware interrupt. 0: No SD memory card interrupt 1: SD memory card interrupt exists. |
| 3 | SDDATIRQ | SDDATIRQ | SD bus DAT interrupt The register bit indicates if there is any interrupt for SD DAT line existing. Whenever interrupt for SD DAT line exists, i.e. any bit in register SDC_ DATSTA is active, the register bit will be set to 1 if interrupt is enabled. It will be reset when the register is read. 0: No SD DAT line interrupt 1: SD DAT line interrupt exists. |
| 2 | SDCMDIRQ | SDCMDIRQ | SD bus CMD interrupt The register bit indicates if there is any interrupt for SD CMD line existing. Whenever interrupt for SD CMD line exists, i.e. any bit in the register SDC_CMDSTA is active, the register bit will be set to 1 if interrupt is enabled. It will be reset when the register is read. 0: No SD CMD line interrupt 1: SD CMD line interrupt exists. |
| 1 | PINIRQ | PINIRQ | Pin change interrupt The register bit indicates if there is any interrupt for memory card insertion/removal existing. Whenever the memory card is inserted or removed and card detection interrupt is enabled, i.e. register bit PINEN in register MSDC_CFG is set to 1, the register bit will be set to 1. It will be reset when the register is read. 0: Otherwise 1: Card is inserted or removed. |
| 0 | DIRQ | DIRQ | Data request interrupt The register bit indicates if there is any interrupt for data request existing. Whenever data request exists and data request as an interrupt source is enabled, i.e. register bit DIRQEN in register MSDC_CFG is set to 1, the register bit will be active. It will be reset when being read. For software, data requests can be recognized by polling register bit DRQ or by data request interrupt. Data request |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | interrupts will be generated every FIFOTHD data transfers. 0: No data request interrupt 1: Data request interrupt occurs. |

A027000C	MSDC_PS	SD Memory Card Pin Status Register	00000008

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  | CMD | DAT |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  | RO | RO |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | CDDEBOUNCE |  |  |  |  |  |  |  |  |  |  | PINCH G | PIN0 | POEN 0 | PIEN0 | CDEN |
| Type | RW |  |  |  |  |  |  |  |  |  |  | RC | RO | RW | RW | RW |
| Reset | 0 | 0 | 0 | 0 |  |  |  |  |  |  |  | 0 | 1 | 0 | 0 | 0 |

Overview: The register is used for card detection. When the memory card controller and system are powered on, , the power for the memory card will still be off unless the power is supplied by the PMIC. Meanwhile, the pad for card detection defaults to pull-down when the system is powered on. The scheme of card detection for MS is the same as that for SD. To detect card insertion, first pull up the INS pin and then enable card detection and the input pin at the same time. After 32 cycles of controller clock, the status of pin changes will emerge. To detect card removal, simply keep enabling card detection and the input pin.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 24 | CMD | CMD | Memory card/SDIO card/MMC card command lines |
| 23:16 | DAT | DAT | Memory card/SDIO card/MMC card data lines |
| 15:12 | CDDEBOUN CE | CDDEBOUNCE | Specifies the time interval for card detection de-bounce Default value: 0. It means the de-bounce interval is 32-cycle time at 32kHz. The interval can extend one cycle time at 32kHz by increasing the counter by 1. |
| 4 | PINCHG | PINCHG | Pin change The register bit indicates the status of card insertion/removal. If the memory card is inserted or removed, the register bit will be set to 1 no matter pin change interrupt is enabled or not. It will be cleared when the register is read. 0: Otherwise 1: Card is inserted or removed. |
| 3 | PIN0 | PIN0 | Shows the value of input pin for card detection 0: The value of input pin for card detection is logic low. 1: The value of input pin for card detection is logic high. |
| 2 | POEN0 | POEN0 | Controls output of input pin for card detection 0: Output of input pin for card detection is disabled. 1: Output of input pin for card detection is enabled. |
| 1 | PIEN0 | PIEN0 | Controls input pin for card detection 0: Input pin for card detection is disabled. 1: Input pin for card detection is enabled. |
| 0 | CDEN | CDEN | Enables card detection The register bit is used to enable or disable card detection. 0: Card detection is disabled. 1: Card detection is enabled. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A0270010	MSDC_DAT	SD Memory Card Controller Data Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | DATA[31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | DATA[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: The register is used to read/write data from/to FIFO inside the SD controller. Data access unit: 32 bits.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31:0 | DATA | DATA | Reads/Writes data from/to FIFO inside SD controller Data access unit: 32 bits |

A0270014	MSDC_IOCON	SD Memory Card Controller IO Control Register	010000C3

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  | SAMPLEDLY |  | FIXDLY |  | SAMP ON | CRCDI S | CMDS EL | INTLH |  | DSW |
| Type |  |  |  |  |  |  | RW |  | RW |  | RW | RW | RW | RW |  | RW |
| Reset |  |  |  |  |  |  | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | CMDR E |  |  |  |  | HIGH_ SPEE D | DMABURST |  | SRCF G1 | SRCF G0 | ODCCFG1 |  |  | ODCCFG0 |  |  |
| Type | RW |  |  |  |  | RW | RW |  | RW | RW | RW |  |  | RW |  |  |
| Reset | 0 |  |  |  |  | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 |

Overview: The register specifies the output driving capability and slew rate of IO pads for MSDC. The reset value is suggested setting. If the output driving capability of pins DAT0, DAT1, DAT2 and DAT3 is too large, it is possible to arise ground bounce and thus result in glitch on SCLK. The actual driving current depends on the PAD type selected for the chip.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 25:24 | SAMPLEDL Y | SAMPLEDLY | Used for SW to select the turn-around delay cycle between write data end bit and CRC status for SD card 00: 0-T delay 01: 1-T delay 10: 2-T delay 11: 3-T delay |
| 23:22 | FIXDLY | FIXDLY | Used for SW to select the delay cycle after clock fix high for the host controller to SD card 00: 0-T delay 01: 1-T delay 10: 2-T delay 11: 3-T delay |
| 21 | SAMPON | SAMPON | Data sample enabling always on The bit is suggested to be set to 1 when the feedback clock is used and to 0 when the multiple phase clock is used. 0: Data sample enabling not always on |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | 1: Data sample enabling always on |
| 20 | CRCDIS | CRCDIS | Switches off data CRC check for SD read data 0: CRC check is on. 1: CRC check is off. |
| 19 | CMDSEL | CMDSEL | Determines whether the host should delay 1-T to latch response from card 0: Host latches response without 1-T delay 1: Host latches response with 1-T delay. |
| 18:17 | INTLH | INTLH | Selects latch timing for SDIO multi-block read interrupt 00: Host latches INT at the second backend clock after the end bit of the current data block from card is received. (Default) 01: Host latches INT at the first backend clock after the end bit of the current data block from card is received. 10: Host latches INT at the second backend clock after the end bit of the current data block from card is received. 11: Host latches INT at the third backend clock after the end bit of the current data block from card is received. |
| 16 | DSW | DSW | Determines whether the host should latch data with 1-T delay or not For SD card, this bit is suggested to be 0. For MSPRO cards, it is suggested to be 1. 0: Host latches the data with 1-T delay. 1: Host latches the data without 1-T delay. |
| 15 | CMDRE | CMDRE | Determines whether the host should latch response token (sent from card on CMD line ) at rising edge or falling edge of serial clock (T.B.D this bit is un-useful) 0: Host latches response at rising edge of serial clock. 1: Host latches response at falling edge of serial clock. |
| 10 | HIGH_SPEE D | HIGH_SPEED | For high-speed mode when internal sample clock is used High-speed mode means the SD/MMC serial bus clock rate is bigger than 25MHz. The default speed mode means that the SD/MMC serial bus clock rate is bigger than 25MHz. 0: Default speed 1: High speed |
| 9:8 | DMABURST | DMABURST | Used for SW to select burst type when data are transferred by DMA Note: Only single mode can support non-4N bytes data transfer in read operation. 00: Single mode 01: 4-beat incrementing burst 10: 8-beat incrementing burst 11: Reserved |
| 7 | SRCFG1 | SRCFG1 | Output driving capability for pins DAT0, DAT1, DAT2 and DAT3 0: Fast slew rate 1: Slow slew rate |
| 6 | SRCFG0 | SRCFG0 | Output driving capability for pins CMD/BS and SCLK 0: Fast slew rate 1: Slow slew rate |
| 5:3 | ODCCFG1 | ODCCFG1 | Output driving capability for pins DAT0, DAT1, DAT2 and DAT3 000: 4mA 001: 8mA 010: 12mA 011: 16mA |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 2:0 | ODCCFG0 | ODCCFG0 | Output driving capability for pins CMD/BS and SCLK 000: 4mA 001: 8mA 010: 12mA 011: 16mA |

A0270018	MSDC_IOCON1 SD Memory Card Controller IO Control Register 1	00022022

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  | PRCF G_CK | PRVAL_CK |  |  | PRCF G_CM | PRVAL_CM |  |  | PRCF G_DA | PRVAL_DA |  |  |  |  |  |
| Type |  | RW | RW |  |  | RW | RW |  |  | RW | RW |  |  |  |  |  |
| Reset |  | 0 | 1 | 0 |  | 0 | 0 | 0 |  | 0 | 1 | 0 |  |  |  |  |

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 14 | PRCFG_CK | PRCFG_CK | Pull-up/down register configuration for pin CK Default value: 0 0: Pull-up resistor in the I/O pad of pin CK is enabled. 1: Pull-down resistor in the I/O pad of pin CK is enabled. |
| 13:12 | PRVAL_CK | PRVAL_CK | Pull-up/down register value for pin CLK Default value: 10 00: Pull-up/down resistor in the I/O pad of pin CLK are all disabled. 01: Pull-up/down resistor in the I/O pad of pin CLK value is 47k. 10: Pull-up/down resistor in the I/O pad of pin CLK value is 47k. 11: Pull-up/down resistor in the I/O pad of pin CLK value is 23.5k. |
| 10 | PRCFG_CM | PRCFG_CM | Pull-up/down register configuration for pin CM Default value is 0. 0: Pull-up resistor in the I/O pad of pin CM is enabled. 1: Pull-down resistor in the I/O pad of pin CM is enabled. |
| 9:8 | PRVAL_CM | PRVAL_CM | Pull-up/down register value for pin CMD/BS Default value: 00 00: Pull-up/down resistor in the I/O pad of pin CMD/BS are all disabled. 01: Pull-up/down resistor in the I/O pad of pin CMD/BS value is 47k. 10: Pull-up/down resistor in the I/O pad of pin CMD/BS value is 47k. 11: Pull-up/down resistor in the I/O pad of pin CMD/BS value is 23.5k. |
| 6 | PRCFG_DA | PRCFG_DA | Pull-up/down register configuration for pin DAT0, DAT1, DAT2 and DAT3 Default value: 0 0: Pull-up resistor in the I/O pad of pin DAT is enabled. 1: Pull-down resistor in the I/O pad of pin DAT is enabled. |
| 5:4 | PRVAL_DA | PRVAL_DA | Pull-up/down register value for pin DAT0, DAT1, DAT2 and DAT3 Default value: 10 00: Pull-up/ down resistor in the I/O pad of pin DAT are all disabled. 01: Pull-up/down resistor in the I/O pad of pin DAT value is 47k. 10: Pull-up/down resistor in the I/O pad of pin DAT value is 47k. 11: Pull-up/down resistor in the I/O pad of pin DAT value is 23.5k. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

A0270020	SDC_CFG	SD Memory Card Controller Configuration Register	00008000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | DTOC |  |  |  |  |  |  |  | WDOD |  |  |  | SDIO |  | MDLE N | SIEN |
| Type | RW |  |  |  |  |  |  |  | RW |  |  |  | RW |  | RW | RW |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | BSYDLY |  |  |  | BLKLEN |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  | RW |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: The register is used to configure the SD memory card controller when it is configured as the host of SD. The register is used to configure the SD memory card controller when it is configured as the host of SD memory card. If the controller is configured as the host of memory stick, the contents of the register will have no impact on the operation of the controller. Note that SDC_CFG[31:16] can be accessed by 16-bit APB bus access.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31:24 | DTOC | DTOC | Data time-out counter The period from finish of the initial host read command or the last read data block in a multiple block read operation to the start bit of the next read data block requires at least two serial clock cycles. The counter is used to extend the period (Read Data Access Time) in unit of 65,536 serial clock. See the register field descriptions of register bit RDINT for reference. 00000000: Extend 65,536 more serial clock cycles 00000001: Extend 65,536x2 more serial clock cycles 00000010: Extend 65,536x3 more serial clock cycles 00000011~11111110: ... 11111111: Extend 65,536x 256 more serial clock cycles |
| 23:20 | WDOD | WDOD | Write data output delay The period from finish of the response for the initial host write command or the last write data block in a multiple block write operation to the start bit of the next write data block requires at least two serial clock cycles. The register field is used to extend the period (Write Data Output Delay) in unit of one serial clock. 0000: No extension 0001: Extend 1 more serial clock cycle 0010: Extend 2 more serial clock cycles 0011~1110: ... 1111: Extend 15 more serial clock cycle |
| 19 | SDIO | SDIO | Enables SDIO 0: Disable SDIO mode 1: Enable SDIO mode |
| 17 | MDLEN | MDLEN | Enables multiple data line The register can be enabled only when SD memory card is applied and detected by software application. It is the responsibility of the application to program the bit correctly when an multi-media card is applied. If an multi-media card is applied and 4-bit data line is enabled, the 4 bits will be output every serial clock. Therefore, data integrity will fail. 0: Disable 4-bit data line 1: Enable 4-bit data line |
| 16 | SIEN | SIEN | Enables serial interface |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | It should be enabled as soon as possible before any command. 0: Disable serial interface for SD 1: Enable serial interface for SD |
| 15:12 | BSYDLY | BSYDLY | Only valid for the commands with R1b response If the command has a response of R1b type, the SD controller must monitor the data line 0 for card busy status from the bit time that is two serial clock cycles after the command end bit to check if the operation in SD memory card have finished. The register field is used to expand the time between the command end bit and end of detection period to detect card busy status. If time is up and there is no card busy status on data line 0, the controller will abandon the detection. 0000: No extension 0001: Extend 1 more serial clock cycle 0010: Extend 2 more serial clock cycles 0011~1110: ... 1111: Extend 15 more serial clock cycle |
| 11:0 | BLKLEN | BLKLEN | Block length The register field is used to define the length of one block in unit of byte in a data transaction. The maximum value of block length is 2,048 bytes. 000000000000: Reserved 000000000001: Block length is 1 byte. 000000000010: Block length is 2 bytes. 000000000011~011111111110: ... 011111111111: Block length is 2,047 bytes. 100000000000: Block length is 2,048 bytes. |

A0270024	SDC_CMD	SD Memory Card Controller Command Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CMDF AIL |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RW |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | INTC | STOP | RW | DTYPE |  | IDRT | RSPTYP |  |  | BREA K | CMD |  |  |  |  |  |
| Type | RW | RW | RW | RW |  | RW | RW |  |  | RW | RW |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: The register defines a SD memory card command and its attribute. Before the SD controller issues a transaction onto the SD bus, application shall specify other relative settings such as argument for command. After application writes the register, the SD controller will issue the corresponding transaction onto the SD serial bus. If the command is GO_IDLE_STATE, the controller will have serial clock on SD bus run 128 cycles before issuing the command.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 16 | CMDFAIL | CMDFAIL | If 4-bit SDIO mode is enabled and when CMD/DAT error occurrs, set this bit to select whether to "wait stop command" or "wait data state machine idle". 0: Wait stop command 1: Wait data state machine idle |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | INTC | INTC | Indicates if the command is GO_IRQ_STATE If the command is GO_IRQ_STATE, the period between command token and response token will not be limited. 0: The command is not GO_IRQ_STATE. 1: The command is GO_IRQ_STATE. |
| 14 | STOP | STOP | Indicates if the command is a stop transmission command 0: The command is not a stop transmission command. 1: The command is a stop transmission command. |
| 13 | RW | RW | Defines the command is a read command or write command The register bit is valid only when the command causes a transaction with data token. 0: The command is a read command. 1: The command is a write command. |
| 12:11 | DTYPE | DTYPE | Defines data token type for the command 00: No data token for the command 01: Single block transaction 10: Multiple block transaction, i.e. the command is a multiple block read or write command. 11: Stream operation. It can only be used when an multi-media card is applied. |
| 10 | IDRT | IDRT | Identification response time The register bit indicates if the command has a response with NID (i.e. 5 serial clock cycles as defined in SD Memory Card Specification Part 1 Physical Layer Specification version 1.0) response time. The register bit is valid only when the command has a response token. Thus the register bit must be set to 1 for CMD2 (ALL_SEND_CID) and ACMD41 (SD_APP_OP_CMD). 0: Otherwise 1: The command has a response with NID response time. |
| 9:7 | RSPTYP | RSPTYP | Defines response type for the command For commands with R1 and R1b response, register SDC_CSTA (not SDC_STA) updates after response token is received. This register SDC_CSTA contains the status of the SD, and it can be used as a response interrupt source. Note: If CMD7 is used with all 0's RCA, then RSPTYP must be "000". Command "GO_TO_IDLE" also has RSPTYP='000'. 000: There is no response for the command, e.g. broadcast command without response and GO_INACTIVE_STATE command. 001: The command has R1 response. R1 response token is 48-bit. 010: The command has R2 response. R2 response token is 136-bit. 011: The command has R3 response. Even though R3 is 48-bit response, it does not contain CRC checksum. 100: The command has R4 response. R4 response token is 48-bit. (only for MMC) 101: The command has R5 response. R5 response token is 48-bit. (only for MMC) 110: The command has R6 response. R6 response token is 48-bit. 111: The command has R1b response. If the command has a response of R1b type, SD controller must monitor the data line 0 for card busy status from the bit time that is 2 or 4 serial clock cycles after the command end bit to check if the operation in SD memory card has finished. There are two cases for detection of card busy status. The first case is that the host stops the data transmission during an active write data transfer. The card will assert busy signal after the stop transmission command end bit followed by 4 serial clock cycles. The second case is that the card is in idle state or |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | receiving a stop transmission command between data blocks when multiple block write command is in progress. The register bit will be valid only when the command has a response token. |
| 6 | BREAK | BREAK | Aborts pending MMC GO_IRQ_MODE command It is only valid for a pending GO_IRQ_MODE command waiting for MMC interrupt response. 0: Other fields are valid. 1: Break a pending MMC GO_IRQ_MODE command in the controller. Other fields are invalid. |
| 5:0 | CMD | CMD | SD memory card command Total 6 bits. |

A0270028	SDC_ARG	SD Memory Card Controller Argument Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | ARG[31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | ARG[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: The register contains the argument of the SD memory card command.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31:0 | ARG | ARG | Contains argument of the SD memory card command. |

A027002C	SDC_STA	SD Memory Card Controller Status Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | WP |  |  |  |  |  |  |  |  |  |  | FEDA TBUS Y | FECM DBUS Y | BEDA TBUS Y | BECM DBUS Y | BESD CBUS Y |
| Type | RO |  |  |  |  |  |  |  |  |  |  | RO | RO | RO | RO | RO |
| Reset | 0 |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 |

Overview: The register contains various statuses of SD controller as the controller is configured as the host of SD memory card.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 15 | WP | WP | Detects the status of write protection switch on SD memory card The register bit shows the status of write protection switch on SD memory card. There is no default reset value. Pin WP (Write Protection) is only useful when the controller is configured for SD memory card. 1: Write protection switch on, i.e. memory card is desired to be |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
|  |  |  | write-protected. 0: Write protection switch off, i.e. memory card is writable. |
| 4 | FEDATBUS Y | FEDATBUSY | Indicates if there is any transmission going on DAT line on SD bus This bit indicates directly the CMD line at card clock domain. For those commands without data but still involving DAT line, the register bit is useless. For example, if an erase command is issued, checking if the register bit is 0 before issuing the next command with data will not guarantee that the controller is idle. In this case, use register bit BESDCBUSY. 0: No transmission is going on DAT line on SD bus. 1: There is transmission going on DAT line on SD bus. |
| 3 | FECMDBUS Y | FECMDBUSY | Indicates if there is any transmission going on CMD line on SD bus This bit indicates directly the CMD line at card clock domain. 0: No transmission is going on CMD line on SD bus. 1: There is transmission going on CMD line on SD bus. |
| 2 | BEDATBUS Y | BEDATBUSY | Indicates if there is any transmission going on DAT line on SD bus 0: Backend SDC controller gets the info that no transmission is going on DAT line on SD bus. 1: Backend SDC controller gets the info that there is transmission going on DAT line on SD bus. |
| 1 | BECMDBUS Y | BECMDBUSY | Indicates if there is any transmission going on CMD line on SD bus This bit shows backend controller's CMD busy state. The busy state is synced from card clock domain to bus clock domain. 0: Backend SDC controller gets the info that no transmission is going on CMD line on SD bus. 1: Backend SDC controller gets the info that there is transmission going on CMD line on SD bus. |
| 0 | BESDCBUS Y | BESDCBUSY | Indicates if SD controller is busy, i.e. is there any transmission going on CMD or DAT line on SD bus This bit shows backend controller's SDC busy state. The busy state is synced from card clock domain to bus clock domain. 0: Backend SD controller is idle. 1: Backend SD controller is busy. |

A0270030	SDC_RESP0	SD Memory Card Controller Response Register 0	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | RESP[31:0][31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | RESP[31:0][15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: The register contains parts of the last SD memory card bus response. See descriptions of register field SDC_RESP3.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31:0 | RESP[31:0] | RESP_31_0 |  |

A0270034	SDC_RESP1	SD Memory Card Controller Response Register 1	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | RESP[63:32][31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | RESP[63:32][15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: The register contains parts of the last SD memory card bus response. See descriptions of register field SDC_RESP3.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31:0 | RESP[63:32] | RESP_63_32 |  |

A0270038	SDC_RESP2	SD Memory Card Controller Response Register 2	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | RESP[95:64][31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | RESP[95:64][15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: The register contains parts of the last SD memory card bus response. See descriptions of register field SDC_RESP3.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31:0 | RESP[95:64] | RESP_95_64 |  |

A027003C	SDC_RESP3	SD Memory Card Controller Response Register 3	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | RESP[127:96][31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | RESP[127:96][15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: The register contains parts of the last SD memory card bus response. Register fields SDC_RESP0, SDC_RESP1, SDC_RESP2 and SDC_RESP3 compose the last SD memory card bus response. For response of

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

type R2, i.e. response of commands ALL_SEND_CID, SEND_CSD and SEND_CID, only bit 127 to 0 of the response token are stored in register fields SDC_RESP0, SDC_RESP1, SDC_RESP2 and SDC_RESP3. For response of other types, only bit 39 to 8 of the response token are stored in register field SDC_RESP0.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31:0 | RESP[127:9 6] | RESP_127_96 |  |

A0270040	SDC_CMDSTA  SD Memory Card Controller Command Status Register 00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  | RSPC RCER R | CMDT O | CMDR DY |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | RC | RC | RC |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 |

Overview: The register contains the status of SD controller during command execution and that of SD bus protocol after command execution when the SD controller is configured as the host of SD memory card. The register can also be used as an interrupt source. The register is cleared when being read. Meanwhile, if the interrupt is enabled and thus interrupt caused by the register is generated, reading the register will de-assert the interrupt.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 2 | RSPCRCER R | RSPCRCERR | CRC error on CMD detected '1' indicates the SD controller detects a CRC error after reading a response from the CMD line. 0: Otherwise 1: SD controller detects a CRC error after reading a response from the CMD line. |
| 1 | CMDTO | CMDTO | Time-out on CMD detected '1' indicates the SD controller detects a time-out condition while waiting for a response on the CMD line. 0: Otherwise 1: SD controller detects a timeout condition while waiting for a response on the CMD line. |
| 0 | CMDRDY | CMDRDY | For command without response, the register bit will be 1 once the command is completed on SD bus For command with response, the register bit will be 1 whenever the command is issued onto the SD bus and its corresponding response is received without CRC error. 0: Otherwise 1: Command with/without response is finished successfully without CRC error. |

| A0270044 | SDC_DATSTA | SD Memory Card Controller Data Status Register | 00000000 |
| --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  | DATCRCERR |  |  |  |  |  |  |  | DATT O | BLKD ONE |
| Type |  |  |  |  |  |  | RC |  |  |  |  |  |  |  | RC | RC |
| Reset |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: The register contains the status of SD controller during data transfer on DAT line(s) when the SD controller is configured as the host of SD memory card. The register can be used as an interrupt source. The register is cleared when being read. Meanwhile, if the interrupt is enabled and thus interrupt caused by the register is generated, reading the register will de-assert the interrupt.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 9:2 | DATCRCER R | DATCRCERR | CRC error on DAT detected '1' indicates that the SD controller detects a CRC error for bit n after reading a block of data from the DAT line or SD signals a CRC error after writing a block of data to the DAT line. 0: Otherwise 1: SD controller detects a CRC error after reading a block of data from the DAT line or SD signals a CRC error after writing a block of data to the DAT line. Note: n is 7 ~ 1 for 8-bits mode. Each bit is read and cleared individually. |
| 1 | DATTO | DATTO | Time-out on DAT detected A '1' indicates that the SD controller detects a time-out condition while waiting for data token on the DAT line. 0: Otherwise 1: SD controller detects a time-out condition while waiting for data token on the DAT line. |
| 0 | BLKDONE | BLKDONE | Indicates the status of data block transfer 0: Otherwise 1: A data block is successfully transferred. |

A0270048	SDC_CSTA	SD Memory Card Status Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | CSTA [31:0][31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | CSTA [31:0][15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: After commands with R1 and R1b respond this register containing the status of the SD card, it will be used as a response interrupt source. In all register fields, logic high indicates error, and logic low indicates no error. The register is cleared when being read. Meanwhile, if the interrupt is enabled and thus interrupt caused by the register is generated, reading the register will de-assert the interrupt.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| 31:0 | CSTA [31:0] | CSTA_31_0 | CSTA31: OUT_OF_RANGE. The command's argument is out of the allowed range for this card. CSTA30: ADDRESS_ERROR. A misaligned address that does not match the block length is used in the command. CSTA29: BLOCK_LEN_ERROR. The transferred block length is not allowed for this card, or the number of transferred bytes does not match the block length. CSTA28: ERASE_SEQ_ERROR. An error in the sequence of erase commands occurs. CSTA27: ERASE_PARAM. An invalid selection of write-blocks for erase occurs. CSTA26: WP_VIOLATION. Attempt to program a write-protected block. CSTA25: Reserved. Return to 0. CSTA24: LOCK_UNLOCK_FAILED. Set when a sequence or password error is detected in lock/unlock card command or if there is an attempt to access a locked card. CSTA23: COM_CRC_ERROR. The CRC check of the previous command fails. CSTA22: ILLEGAL_COMMAND. Command not legal for the card state. CSTA21: CARD_ECC_FAILED. Card internal ECC is applied but fails to correct the data. CSTA20: CC_ERROR. Internal card controller error. CSTA19: ERROR. A general or unknown error occurs during the operation. CSTA18: UNDERRUN. The card cannot sustain data transfer in stream read mode. CSTA17: OVERRUN. The card cannot sustain data programming in stream write mode. CSTA16: CID/CSD_OVERWRITE. It can be either one of the following errors: 1) The CID register has been already written and cannot be overwritten; 2) The read-only section of the CSD does not match the card; 3) An attempt to reverse the copy (set as the original) or permanent WP (unprotected) bits is made. CSTA[15: 4]: Reserved. Return to 0. CSTA3: AKE_SEQ_ERROR. Error in the sequence of authentication process CSTA[2: 0]: Reserved. Return to 0. |
| --- | --- | --- | --- |

A027004C	SDC_IRQMASK0	SD Memory Card IRQ Mask Register 0	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | IRQMASK [31:0][31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | IRQMASK [31:0][15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: The register contains parts of SD memory card interrupt mask register. See the register descriptions of register SDC_IRQMASK1 for reference. The register masks interrupt sources from register SDC_CMDSTA and SDC_DATSTA. IRQMASK[3:0] is for SDC_CMDSTA, and IRQMASK[18:16] for SDC_DATSTA. Note that

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

IRQMASK[18] masks SDC_DATSTA[9:2] together. '1' in some bits of the register masks the corresponding interrupt source with the same bit position. For example, if IRQMASK[0] is 1, then the interrupt source from register field CMDRDY of register SDC_CMDSTA will be masked. '0' in some bits does not cause interrupt mask on the corresponding interrupt source from registers SDC_CMDSTA and SDC_DATSTA.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31:0 | IRQMASK [31:0] | IRQMASK_31_0 |  |

A0270050	SDC_IRQMASK1	SD Memory Card IRQ Mask Register 1	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | IRQMASK [63:32][31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | IRQMASK [63:32][15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: The register contains parts of SD memory card interrupt mask register. Registers SDC_IRQMASK1 and SDC_IRQMASK0 compose the SD memory card interrupt mask register. The register masks interrupt sources from register SDC_CSTA. '1' in some bits of the register masks the corresponding interrupt source with the same bit position. For example, if IRQMASK[63] is 1, then the interrupt source from register field OUT_OF_RANGE of register SDC_ CSTA will be masked. '0' in some bits does not cause interrupt mask on the corresponding interrupt source from register SDC_CSTA.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31:0 | IRQMASK [63:32] | IRQMASK_63_32 |  |

A0270054	SDIO_CFG	SDIO Configuration Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  |  |  | DISSE L |  | INTCS EL | DSBS EL |  | INTEN |
| Type |  |  |  |  |  |  |  |  |  |  | RW |  | RW | RW |  | RW |
| Reset |  |  |  |  |  |  |  |  |  |  | 0 |  | 0 | 0 |  | 0 |

Overview: The register is used to configure functions for SDIO.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 5 | DISSEL | DISSEL | Selects data block interrupt source 0: The host detects SDIO interrupt during interrupt period between two data blocks of multiple block data access. 1: The host ignores SDIO interrupt during interrupt period between two data blocks of multiple block data access. |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 3 | INTCSEL | INTCSEL | Selects interrupt control 0: The host detects DAT1 low as SDIO interrupt. 1: The host detects DAT3/DAT2/DAT1/DAT0 4'b1101 as SDIO interrupt. |
| 2 | DSBSEL | DSBSEL | Selects data block start bit 0: Use data line 0 as start bit of data block. Other data lines are ignored. 1: Start bit of a data block is received only when all data line 0-3 become low. |
| 0 | INTEN | INTEN | Enables interrupt for SDIO 0: Disable 1: Enable |

A0270058	SDIO_STA	SDIO Status Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | IRQ |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RO |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |

Overview: This register is used to identify if there is SDIO interrupt during the interrupt period on data line.

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 0 | IRQ | IRQ | SDIO interrupt exists on the data line. For example, when in the interrupt period or the 1 -bit data line mode and DAT1/5 goes low from high, this bit will become 1 from 0. If DAT1/5 goes high from low, this bit will become 0 from 1. 0: There is no SDIO interrupt existing on the data line. 1: There is SDIO interrupt existing on the data line. |

A0270080	CLK_RED	CLK Latch Configuration Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  | CMD_ RED |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name |  |  | DAT_ RED |  |  |  |  |  | CLKP AD_R ED | CLK_ LATC H |  |  |  |  |  |  |
| Type |  |  | RW |  |  |  |  |  | RW | RW |  |  |  |  |  |  |
| Reset |  |  | 0 |  |  |  |  |  | 0 | 0 |  |  |  |  |  |  |

Overview: The register is used to configure the MSDC sample data/response clock. Note that only when MSDC_IOCON[19] = 1 will the host latch response; otherwise MSDC FSM will handle the response from PAD directly.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 29 | CMD_RED | CMD_RED | Determines the command response from card output is latched at falling edge or rising edge of internal clock Only effective when CLK_LATCH = 1 0: Internal clock rising edge to latch response 1: Internal clock falling edge to latch response |
| 13 | DAT_RED | DAT_RED | Determines the input data from card output is latched at falling edge or rising edge of internal sample clock Only effective when CLK_LATCH = 1 0: Internal clock rising edge to latch data 1: Internal clock falling edge to latch data |
| 7 | CLKPAD_R ED | CLKPAD_RED | Determines the input data from card is latched at falling edge or rising edge of the feedback clock from pad The suggested setting is 0 when SD serial clock is lower than 25MHz. The suggestion setting is 1 when SD serial clock is higher than 25MHz. The suggestion setting is 0 for MMC card no matter the serial clock rate is high speed or default speed. Only effective when CLK_LATCH = 0. 0: Internal feedback clock rising edge to latch data/response 1: Internal feedback clock falling edge to latch data/response |
| 6 | CLK_LATCH | CLK_LATCH | Determines which clock to latch data from card The suggested setting is 1 if SCLKF in register field MSDC_CFG is 0x0. Otherwise, the suggested setting is 0. 0: Internal feedback clock is used to latch data/response from card. 1: Internal clock is used to latch data/response from card. |

A0270098  DAT_CHECKSUM  MSDC Rx Data Checksum Register	00000000

| Bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | DAT_CHECKSUM[31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Name | DAT_CHECKSUM[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Overview: The register is used to compete the checksum value of Rx read data

| Bit(s) | Mnemonic | Name | Description |
| --- | --- | --- | --- |
| 31:0 | DAT_CHEC KSUM | DAT_CHECKSUM | The checksum algorithm is 32-bit XOR. |

## BTIF

##### General Description

Bluetooth Interface (BTIF) is designed in SOC (BT+GSM) as the UART interface between the BT chip and baseband chip. As in the UART design, BTIF is an APB slave which transmits or receives data by MCU access or through DMA/VFIFO.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

arm9_btif_sleep_wakeup_out

arm7_btif_tx_buf_empty_out

arm9_btif_tx_buf_empty_out

arm7_btif_rx_rd_en_out

arm9_btif_rx_rd_en_out

arm7_btif_tx_bus_data

8

arm9_btif_tx_bus_data

arm7_btif_tx_buf_empty_in

arm9_btif_tx_buf_empty_in

arm7_btif_rx_rd_en_in

arm9_btif_rx_rd_en_in

arm7_btif_rx_bus_data

8

arm9_btif_rx_bus_data

Bluesys

ap_mcusys

BTIF

BTIF

arm7_btif_sleep_wakeup_out

ext_intr

ext_intr

Figure 48. Interface connection between BT and baseband system

##### Register Definition

BTIF+0000h	Rx Buffer Register	BTIF_RBR

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | RBR[7:0] |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | RO |  |  |  |  |  |  |  |

RBR	Rx buffer register. A read-only register. The received data can be read by accessing this register. This register is valid only when BTIF_FAKELCR[7] (0x0C) is 0.

##### BTIF+0000h	Tx Holding Register	BTIF_THR

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | THR[7:0] |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | WO |  |  |  |  |  |  |  |

THR  Tx holding register. A write-only register. The data to be transmitted are written to this register and sent to the Bluetooth via BTIF. This register is valid only when BTIF_FAKELCR[7] (0x0C) is 0.

##### BTIF+0004h	Interrupt Enable Register	BTIF_IER

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TXEE N | RXFE N |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | W/R | W/R |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

This register is valid only when BTIF_FAKELCR[7] is 0.

TXEEN	Enables Tx empty interrupt. When set to 1, an interrupt will be generated if the Tx holding register is empty.

- No interrupt will be generated if the Tx holding register is empty.

- An interrupt will be generated if the Tx holding register is empty

RXFEN	Enables Rx full interrupt. When set to 1, an interrupt will be generated if the Rx buffer contains data.

- No interrupt will be generated if the Rx buffer contains data.

- An interrupt will be generated if the Rx buffer contains data.

##### BTIF+0008h	Interrupt Identification Register	BTIF_IIR

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  | ID2 | ID1 | ID0 | NINT |
| Type |  |  |  |  |  |  |  |  |  |  |  |  | RO | RO | RO | RO |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 1 |

This register is valid only when BTIF_FAKELCR (0x0C) is not 0xBF.

IIR	Identifies if there are pending interrupts. The following table lists the IIR[5:0] codes associated with the possible interrupts:

Table 52. IIR[5:0] codes associated with the possible interrupts

| IIR[3:0] | Priority level | Interrupt | Source |
| --- | --- | --- | --- |
| 0001 | - | No pending interrupt |  |
| 0100 | 1 | Rx data received | Rx data received |
| 1100 | 2 | Rx data time-out | Time-out on character in Rx buffer |
| 0010 | 3 | Tx holding register empty | Tx holding register empty. |

Rx data received interrupt

A Rx received interrupt (IIR[3:0] = 0x04) is generated when RXFEN (IER[0]) is set and Rx data are placed in the Rx buffer register. The interrupt is cleared by reading the Rx buffer register.

Rx data time-out interrupt

The Rx data time-out interrupt will be generated if all of the following conditions are applied:

- Rx buffer is empty.

- The most recent character is received longer than (RTOCNT*bclk period*4).

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

- RXFEN (IER[0]) is set to 1.

The time-out timer is restarted upon receipt of a new byte from the Rx shift register. This interrupt is only valid while VFIFO is used. This register is cleared by reading the VFIFO status register (0x4C).

Tx holding register empty

A Tx holding register empty interrupt (IIR[3:0] = 0x02) is generated when TXEEN(IER[1]) is set and no data are placed in the Tx holding register. This interrupt is cleared by writing data into BTIF_THR (0x00).

##### BTIF+0008h	FIFO_CTRL	BTIF_FIFOCT RL

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  | CLRT | CLRR |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | WO | WO |  |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 |  |

This register is valid only when BTIF_FAKELCR (0x0C) is not 0xBF.

CLRT	Clears transmit FIFO. This bit is self-clearing.

  - Leave Tx FIFO intact.

  - Clear all the bytes in Tx FIFO.

CLRR	Clears receive FIFO. This bit is self-clearing.

- Leave Rx FIFO intact.

- Clear all the bytes in Rx FIFO.

##### BTIF+000Ch	FAKE LCR	BTIF_FAKEL

CR

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | FAKELCR[7:0] |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

FAKELCR	This register is added to synchronize the software control method of UART. When FAKELCR[7] is 1, RBR(0x00), THR(0x00) and IER(0x04) will not be readable/writable. When FAKELCR is 0xBF, RBR(0x00), THR(0x00), IER(0x04), IIR(0x08) and LSR(0x14)

will not be readable/writable.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

##### BTIF+0014h	Line Status Register	BTIF_LSR

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  | TEMT | THRE |  |  |  |  | DR |
| Type |  |  |  |  |  |  |  |  |  | RO | RO |  |  |  |  | RO |
| Reset |  |  |  |  |  |  |  |  |  | 1 | 1 |  |  |  |  | 0 |

LSR	Line status register. Readable when LCR != 0xBF.

TEMT	Tx holding register is empty.

  - Empty conditions are not met.

  - This bit is set when the Tx holding register is empty.

THRE	Indicates if Tx FIFO is reduced to its trigger level

- Reset whenever the contents of Tx FIFO are more than its trigger level (FIFOs are enabled)

- Set whenever the contents of Tx FIFO are reduced to its trigger level (FIFOs are enabled)

DR	Data Ready

- Cleared by reading the Rx buffer.

- Set by the Rx buffer becoming full.

##### BTIF+0048h Sleep Enable Register	BTIF_SLEEP_EN

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SLEE P_EN |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |

SLEEP_EN	For sleep mode issue

  - Does not deal with sleep mode indication signal

  - Activate flow control according to software initial settings when the chip enters the sleep mode. Release hardware flow when the chip wakes up.

##### BTIFn+004C DMA Enable Register	BTIF_DMA_EN h

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  | TO_C NT_A UTOR ST | TX_D MA_E N | RX_D MA_E N |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W |

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

RX_DMA_EN	RX_DMA mechanism enabling signal

    - Does not use DMA in Rx.

    - Use DMA in Rx. When this register is enabled, the flow control is based on the DMA threshold and generates a time-out interrupt

TX_DMA_EN	TX_DMA mechanism enabling signal

- Does not use DMA in Tx.

- Use DMA in Tx. When this register is enabled, the flow control is based on the DMA threshold and generates a time-out interrupt for DMA.

TO_CNT_AUTORST	Time-out counter auto reset register

- After Rx time-out takes place, the software shall reset the interrupt by reading BTIF 0x4C.

- The time-out counter will be auto reset.

##### BTIF+0054h	Rx Time-out Count	BTIF_RTOCNT

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | RTOCNT[7:0] |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0x40 |  |  |  |  |  |  |  |

RTOCNT	Used for Rx time-out interrupt. The Rx time-out interrupt will be generated when:

- RXFEN (0x04[0]) is set to 1.

- Rx buffer is empty.

- The most recent character is received longer than (RTOCNT*bclk period*4).

##### BTIF+0060h TRX_TRIGGER_LEVEL	BTIF_TRI_LVL

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  | BTIF_LOOP | RX_TRI_LVL |  |  | TX_TRI_LVL |  |  |  |
| Type |  |  |  |  |  |  |  |  | R/W | R/W |  |  | R/W |  |  |  |
| Reset |  |  |  |  |  |  |  |  | 0x0 | 0x5 |  |  | 0x0a |  |  |  |

TX_TRI_LVL	Used for Tx FIFO trigger threshold. THRE(0x14[5]) will be set if the data in the TXFIFO are less than TX_TRI_LVL.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

RX_TRI_LVL  Used for Rx FIFO trigger threshold. A Rx trigger interrupt (IIR(0x08]) = 4) might be set if the data in the RXFIFO are more than RX_TRI_LVL. The output flow control signal will also be set if the data in the RXFIFO are more than RX_TRI_LVL.

BTIF_LOOP	Enables BTIF loop back mode. The data output from Tx will be received by Rx.

##### BTIF+0064h SLEEP_WAKEUP	BTIF_WAK

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SLEE P_WA KE |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | WO |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |

SLEEP_WAKE	ARM9 side btif_sleep_wakeup_in_b is connected to eint[16] (ARM9 has eint[19:0].

ARM7 side btif_sleep_wakeup_in_b is connected to eint[0] (ARM7 has eint[3:0])

##### BTIF+0068h ASYNC_WAIT_TIME	BTIF_WAT_TIME

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  | WAT_TIME_2 |  |  | WAT_TIME_1 |  |  |
| Type |  |  |  |  |  |  |  |  |  |  | R/W |  |  | R/W |  |  |
| Reset |  |  |  |  |  |  |  |  |  |  | 0X2 |  |  | 0x2 |  |  |

ASYNC_WAIT_TIME	Sets up waiting time of RX read-out.

WAT_TIME_1	The first level of wait time.

WAT_TIME_2	The second level of wait time.

Notes: The value of WAT_TIME_1/ WAT_TIME_1 cannot be smaller than 0x2.

##### BTIF+006C NEW_HANDSHAKE	BTIF_HANDSHAKE

h

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  |  |  |  |  |  |  |  |  |  |  |  |  | RTO_ EXT | HIGH_ SPEE D_EN | HAND SHAK E_EN |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W |
| Reset |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 1 |

NEW_HANDSHAKE	The default value of handshake is 0. The function of handshake is disabled. The function of BTIF has limitation. The ratio of bclk cannot be bigger than 2; otherwise, two system data transmissions will be wrong. If the value of handshake is 1, the ratio of bclk will be free.

HANDSHAKE_EN	Enables handshake mode.

high_speed_en	Enables high speed mode. Reserved.

MEDIATEK CONFIDENTIAL

FOR wangq@ szsimtech.com USE ONLY

RTO_EXT	Extends the value of RX time-out counter (16*rto_time).